3GPP TSG-RAN ad hoc	RPa160078

Barcelona, Spain, January 28-29 2016

Source:	Telecom Italia

Title: 	Requirements on RAN architecture

Agenda Item:	5

Document for:	Approval

# Introduction

The following proposal is based on the discussion in RPa160003, RPa160056 and proposes inclusion of requirements on RAN Architecture in Section 8 of TR38.913.

# Text Proposal

-------------------------------------------------- BEGIN TEXT PROPOSAL ----------------------------------------------------------

### X.Y	Requirements for architecture and migration of Next Generation Radio Access Technologies

The RAN design for the Next Generation Radio Access Technologies shall be designed to fulfil the following requirements:

- The RAN architecture shall support tight interworking between the new RAT and LTE

  - Including high performing inter-RAT mobility and multi-RAT aggregation/connectivity. This shall be supported for both collocated and non-collocated site deployments.

- The RAN architecture shall support connectivity through multiple transmission points, either collocated or non-collocated.

  - The RAN architecture shall enable a separation of control plane signalling and user plane data from different sites.

  - The RAN architecture shall support interfaces supporting effective inter-site scheduling coordination

- [The design of the RAN architecture shall allow a flexible split of RAN functions]

- The design of the RAN architecture shall allow the deployment of new services rapidly and efficiently

- The design of the RAN architecture shall allow the support of 3GPP defined service classes (e.g. interactive, background, streaming and conversational)

- The design of the RAN architecture shall enable lower CAPEX/OPEX with respect to current networks to achieve the same level of services

- RAN-CN interfaces and RAN internal interfaces (both between new RAT functions and between new RAT and LTE functions) shall be open for multi-vendor interoperability

-------------------------------------------------- END TEXT PROPOSAL ----------------------------------------------------------

The following requirements were agreed but it was concluded to move them to a different Section in the TR

-------------------------------------------------- BEGIN TEXT PROPOSAL ----------------------------------------------------------

### X.Z	Operational requirements

- RF requirements for multistandard base stations shall be supported also for the new RAT

- The RAN nodes shall be designed to allow upgrade by software as much as possible

-------------------------------------------------- END TEXT PROPOSAL ----------------------------------------------------------

The following proposed requirements are FOR EMAIL DISCUSSION

- FOR EMAIL DISCUSSION All control plane functions should be separated out from user plane functions via standardized open interfaces.

- FOR EMAIL DISCUSSION The RAN design shall be designed to cater for different performance requirements on interfaces between RAN nodes (e.g. synchronization, latency, jitter), taking into account costs

- FOR EMAIL DISCUSSION synchronization requirements

- FOR EMAIL DISCUSSION Radio node resources (e.g. computational capacity) shall be software reconfigurable