3GPP TSG RAN Rel-18 workshop                                            RWS-210456

Electronic Meeting, June 28 - July 2, 2021

Source: 	Huawei, HiSilicon

Title: 	mmWave multi-band BS

Agenda Item:	4.1

Document for:	Discussion and Decision

# Introduction

BS requirements for mmWave frequency range were introduced initially from Rel-15 [1]. With the release evolution, more FR2 bands are specified after Rel-15. Now the FR2 frequency range covers 26 GHz, 28 GHz, 39 GHz, 40 GHz, 47 GHz bands and also extends to 52.6-71GHz.

Requirements for FR2 BS are full OTA based, but so far the mmWave BS requirements are only applicable for single FR2 operating band. However, inter-band CA band combinations are already supported for FR2. There are lots of discussion for band groups for UE side in Rel-16 and Rel-17.

This contribution provides our consideration on mmWave multi-band BS.

# Discussion

Table 1 lists the specified operating bands for FR2 [2].

Table 1: NR operating bands in FR2

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |
| n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |
| n259 | 39500 MHz | – | 43500 MHz | 39500 MHz | – | 43500 MHz | TDD |
| n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |
| n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |
| n262 | 47200 MHz | – | 48200 MHz | 47200 MHz | – | 48200 MHz | TDD |

According to operators’ deployment demand, requirements for both inter-band DL CA and inter-band UL CA are studied in UE RF FR2 WI [3][4]. Specifically, common beam management (CBM) and independent beam management (IBM) are discussed under inter-band DL CA enhancements:

- Inter-band DL CA enhancements [RAN4 RF/RRM] - Agree a method how applicable CBM/IBM information is captured into specification for a particular CA configuration. Agree how it is decided whether a certain CA configuration is assuming CBM or IBM based requirements (for-example is applicability based on operator request or some general rule or are all CA configurations applicable for both CBM and IBM). - Study and if feasible define UE requirements for CBM between different freq. groups (e.g. 28GHz + 37GHz). - Define requirements for CA_n258A-n260A and CA_n257A-n259A based on IBM (Note these CA configurations will be moved to Basket WI in RAN#90 and more combinations may be added to Basket WI later). - Define UE requirements for inter-band CA within the same freq. group (e.g. 28GHz + 28GHz) for common beam management (CBM) based on requested band combinations. Evaluate performance impact based on deployment conditions and design constraints, including outcome of MRTD requirement if any. - Study and if feasible define UE RF requirements for inter-band CA within the same freq. group (e.g. 28GHz + 28GHz) for (IBM) based on explicitly requested band combinations. - Both RF and RRM requirement aspects are in scope for DL interband CA.

The definitions of CBM and IBM were further clarified in RAN4, i.e.

CBM (Common Beam Management): A UE that supports inter-band CA with CBM selects its DL Rx beam(s) for all CCs in all configured bands based on DL measurements made in the only CC configured with the reference signal for beam management.

IBM (Independent Beam management): A UE that supports inter-band CA with IBM selects its DL Rx beam(s) for all CCs in each configured band based on DL reference signals measurements made in that band.

The capability of beam management as well as corresponding requirements would have impact to network deployment scenario.

In the existing RRM specification TS 38.133, MRTD for inter-band CA is defined as below:

For inter-band NR carrier aggregation, the UE shall be capable of handling at least a relative receive timing difference between slot timing of all pairs of carriers to be aggregated at the UE receiver as shown in Table 7.6.4-2 below. Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation Frequency Range of the pair of carriers; Maximum receive timing difference (µs). FR1; 33. FR2; 8 note1. Between FR1 and FR2; 25. Note1: This requirement applies to the UE capable of independent beam management for FR2 inter-band CA..

According the Note, 8us MRTD is applicable for IBM in Rel-16, but there is no conclusion on MRTD for CBM. During the discussion in Rel-16, some companies proposed to reduce the MRTD requirement for CBM to 260ns, or at least the value should be reduced to some extent in order to guarantee the system performance. The figures below are cited from [5].

Figure 1. When UE has to sweep a common Rx beam of 2 CCs, but the CP of the 2 CC are non-overlapped.

With analysis in [5], if the CP of 2 CCs are not overlapped, UE cannot find a duration to switch its Rx beam for both cells without causing interruption to data reception.

Figure 2. MRTD = TAE + difference in propagation delay

Thus it is proposed by some companies to reduce the MRTD to 260ns. The OTA TAE requirements for FR2 BS are defined as below:

#### 9.6.3.3 Minimum requirement for BS type 2-O For MIMO transmission, at each carrier frequency, OTA TAE shall not exceed 65 ns. For intra-band contiguous carrier aggregation, with or without MIMO, OTA TAE shall not exceed 130 ns. For intra-band non-contiguous carrier aggregation, with or without MIMO, OTA TAE shall not exceed 260 ns. For inter-band carrier aggregation, with or without MIMO, OTA TAE shall not exceed 3 µs.

The proposal of smaller MRTD actually limits the deployment to co-location scenario only to support CBM. We don’t think that the UE capability should have any impact to NW deployment, but it is one factor worthy of consideration for BS evolution. Even for co-location scenario, if different modules are implemented for different operating bands, whether smaller TAE can be guaranteed also depends on BS implementation. Obviously, a multi-band RF module can easily fulfill the requested TAE requirement for co-location scenario.

On the other hand, with increasingly many inter-band CA combinations proposed for FR2 and evolution of the frequency range to E-band and even higher, it is expected that the RF component ability as well as antenna performance will be enhanced for wider RF bandwidth. Retrospect to the history of FR1 MB-MSR, it was not popular at the time the WI proposed for multi-band BS, but nowadays, MB BS is widely deployed around the world. We think MB-BS for FR2 could drive the enhancement of wide band implementation for mmWave BS, and it can also be helpful to guarantee the performance for co-location scenario.

Proposal: It is proposed to have a new WI in Rel-18 for mmWave multi-band BS.

Figure 3 mmWave Multi-band BS supporting IBM and CBM UE

The proposed mmWave multi-band BS supports both IBM and CBM UEs, and same band group can be considered firstly.

# Conclusion

This contribution provides consideration on mmWave multi-band BS.

For evolution of single band FR2 BS to multi-band BS, it is foreseen wide band capability for RF components and antenna design would be further enhanced also it is beneficial to guarantee the system performance for multi-band co-location scenario if CBM UE capability is taken into account.

Proposal: It is proposed to have a new WI in Rel-18 for mmWave multi-band BS.

# References

[1] TS 38.104

[2] R4-2107791, Introduction of n262 UE RF requirements, Nokia, Nokia Shanghai Bell

[3] RP-192653, Revised WID on NR RF Requirement Enhancements for FR2, Nokia, Nokia Shanghai Bell

[4] RP-210914, Revised WID Further enhancements of NR RF requirements for frequency range 2 (FR2), Nokia, Nokia Shanghai Bell

[5] R4-2006571, MRTD for inter band CA, MediaTek