3GPP TSG RAN Rel-18 workshop                                                                   RP-210466
Electronic Meeting, June 28 – July 2,2021

Agenda Item:	4.1

Source:	ZTE

Title:	Further enhancement of data collection for SON&MDT in NR and MR-DC

Document for:		Discussion and Decision

1. Introduction

The data collection for SON&MDT support for NG-RAN is specified in Rel-16 [1] and the normative work continues for Rel-17[2]. In this contribution, the motivation of new work item towards Rel-18 is elaborated.

2. Discussion

2.1 Enhancement of Rel-18 topics

- NR multicast and broadcast services

In Rel-16 & Rel-17, NR MDT aiming to scenarios of coverage and QoS verification. It is also notes in Rel-12, an enhancement of logged MDT for MBSFN has been conducted and specified.  Given the fact that NR multicast and broadcast services is going to be specified in Rel-17 [3], it is benefit to consider how to support NR multicast and broadcast services into NR MDT.

NR multicast and broadcast services for MDT can be normative in Rel-18.

- MDT for NPN

In Rel-16 & Rel-17, NG-RAN node is not aware of  CAG ID during the accessing and the mobility. After the user is authorized to access a PNI-NPN network by the core network, the RAN network cannot know the CAG information that the user is currently accessing, thus the operator is unable to calculate the user load of different CAG networks and optimize  the network capacity according to different NPN network loads. It is benefit that the MDT can record the accessing CAG information.

MDT supports recording accessing NPN information e.g, CAG ID can be normative in Rel-18.

- Left issue in Rel-16 or Rel-17 of NR SON /MDT

Due to short of time, several issues raised in Rel-16 or in Rel-17 has decided to be postpone to next release,the left issues listed below:

- SON Enhancements for CPC with waiting for the progress of R17 CPC enhancements and SON enhancements for CHO.

- For CHO/DAPS, SON related Data forwarding enhancements on HO to wrong cell.

- Enhancements for RACH Report retrieval from DU to CU.

- For Mobility Load Balancing, enhancements on Partial Failure mechanism.

Left issue in Rel-16 or Rel-17 of NR SON /MDT can be normative in Rel-18.

3. Conclusion

In this document, the motivation and the potential topics for data collection for SON&MDT in NR and MR-DC towards Rel-18 are described and the proposals were as follows:

- NR multicast and broadcast services for MDT can be normative in Rel-18.

- MDT supports recording accessing NPN information e.g, CAG ID can be normative in Rel-18.

Left issue in Rel-16 or Rel-17 of NR SON /MDT can be normative in Rel-18.

4. Reference

[1]  RP-192610, WID on SON/MDT support for NR

[2]  RP-201281, New WID on enhancement of data collection for SON/MDT in NR

[3] RP-193248,  New Work Item on NR support of Multicast and Broadcast Service