3GPP TSG RAN WG1 Meeting #100-e	R1-2000217

February 24 - March 6, 2020

Agenda Item:	6.2.2.3

Source:	Huawei, HiSilicon

Title:	Corrections on scheduling of multiple DL/UL transport blocks

Document for:	Discussion and Decision

- Introduction

In NB-IoT Rel-16, the agreements related to multiple DL/UL transport blocks are summarized in [1]. In this contribution, we provide our views on the remaining issues.

- SC-PTM related issues

  - Scheduling gap

In RAN1#99, the following agreement about a RRC configured scheduling gap after each TB was made. As discussed in [2], the gap could be used for backward compatibility purposes if there are legacy UEs deployment together with Rel-16 UE.

Agreement For multicast, a scheduling gap can be inserted after each TB, where the gap length is configurable between {0, 16, 32, 64, 128} subframes. The configuration is per cell.

And the following agreement about a 20ms gap inserted every two TBs was also made. The purpose is for UE processing needed at receiver.

Agreement If RRC configured scheduling gap is zero, for multicast, for UE processing at receiver, - If R>=12, no gap is inserted - R is the number of subframes of each of the NPDSCH - Otherwise, a 20msec gap is inserted every 2 TBs

The agreements above have not been captured in RAN1 specification yet. And based on the agreement above, the scheduling gap is inserted after the end of each TB even for last TB. As shown in Figure 1, one DCI schedules four TBs. After the end of last TB, i.e. TB4, there is also a gap inserted. It is not the intention and it will delay the subsequent transmission for the last gap after TB4.

Figure 1 An example of RRC configured scheduling gap

Proposal 1: For SC-MTCH multiple TBs scheduling, a RRC configured scheduling gap is not inserted after the end of the last TB.

Proposal 2: Adopt the following text proposals in Annex A for TS 36.213.

  - Collision handling

As discussed in [2], the RRC configured scheduling gap is used for backward compatibility purpose. Figure 2 is an example. In network perspective, the same SC-MTCH for Rel-16 UE and legacy UE is sent only once. The scheduling gap is used for DCI transmission for legacy UE.

Figure 2 An example of scheduling gap for backward compatibility purpose

Observation 1: For SC-MTCH multiple TBs scheduling, the configurable gap inserted after each TB may be used for backward compatibility purposes.

In Rel-13, DL gap is supported. If the condition Rmax≥Ngap,threshold is satisfied, DL gap is activated during NPDSCH transmission. For backward compatibility purpose, DL gap for Rel-16 UE and legacy UE should be activated at the same time for the same SC-MTCH. If DL gap is activated, scheduling gap may collide with DL gap. Then how to handle the collision between scheduling gap and DL gap should be clarified.

- Option 1：Overlapped part between scheduling gap and DL gap is counted as part of NPDSCH, so the overlapped part is postponed together with NPDSCH. An example is shown in Figure 3.

- Option 2：Overlapped part between scheduling gap and DL gap is counted as part of DL gap, so the overlapped part is not postponed. An example is shown in Figure 4.

Figure 3 An example of option 1

(i.e. overlapped part is postponed)

Figure 4 An example of option 2

(i.e. overlapped part is counted as part of DL gap, i.e. not postponed)

For option 1, the overlapped part is postponed, thus legacy DCI can be sent in the postponed scheduling gap. The same SC-MTCH TB2 is sent only once. For option 2, the overlapped scheduling gap is counted as part of DL gap. There is no room to send legacy DCI to schedule SC-MTCH TB2. So the same SC-MTCH TB2 is sent twice. It is a waste of network resource. And it also makes the carrier crowded. It will impact legacy SC-MTCH scheduling in the same carrier. So option 1 is preferred.

Observation 2: For the case that RRC configured scheduling gap collides with DL gap, postpone the overlapped scheduling gap together with NPDSCH transmission (i.e. option 1) does not increase network resource consumption, but the case that overlapped scheduling gap counted as part of DL gap (i.e. option 2) does.

Proposal 3: For SC-MTCH multiple TBs scheduling, if Rmax≥Ngap,threshold and RRC configured scheduling gap collides with DL gap, overlapped scheduling gap is postponed with NPDSCH transmission.

If the RRC configured scheduling gap is zero, 20ms gap maybe inserted every two TBs. The motivation is for UE processing at receiver. In this case, every two TBs are transmitted back to back. It is not practical to send same SC-MTCH TBs only once for both Rel-16 UE and legacy UE in this case. So with respect to the 20ms gap, no need to consider backward compatibility issue. If the 20ms gap collides with DL gap, only overlapped NPDSCH transmission needs to be postponed. No need to consider the legacy DCI. The overlapped scheduling gap is counted as part of DL gap. An example is shown in Figure 5.

Figure 5 An example of 20ms gap inserted every two TBs collision with DL gap

Observation 3: For SC-MTCH multiple TBs scheduling, the 20ms gap inserted every two TBs is used for UE processing needed at receiver and not for backward compatibility purposes.

Proposal 4: For SC-MTCH multiple TBs scheduling, if Rmax≥Ngap,threshold and 20ms gap inserted every two TBs collides with DL gap, overlapped part of the 20ms gap is counted as part of DL gap.

Proposal 5: Adopt the following text proposals in Annex B for TS 36.211.

- Unicast related issues

In RAN1#99, the following agreements were made:

Agreement For interleaving case, the symbol counter is reset at the start of the first TB transmission and incremented for each symbol during the whole transmission of the two TBs. - l=0,1,…,NMrepNPUSCHNRUNslotsULNsymbUL-1, where N is the number of scheduled TBs.

The agreements above have not been captured in RAN1 specification accurately yet. For the following description in current specification:

If NTB >1 and interleaving between codewords is applied according to clause 16.5.1 of 3GPP TS 36.213 [4], then the symbol counter  is reset at the start of the transmission and incremented for each symbol during the transmission.

“The transmission” would introduce ambiguity as it may be interpreted as the transmission of each TB or the transmission of NTB TBs. Then it may lead to two different implementations for symbol counter reset. One implementation is that the symbol counter  is reset at the start of the transmission of each TB, and the other implementation is that the symbol counter  is reset at the start of the transmission of NTB TBs. Based on the agreement, the symbol counter  should be reset at the start of the transmission of NTB TBs. Thus “the transmission” should be clarified as “the transmission of NTB NPUSCH codewords”.

Proposal 6: Adopt the following text proposals in Annex C for TS 36.211.

- Conclusion

In this contribution, our views on the remaining issues of multiple TB scheduling are provided. The following proposals are made.

Proposal 1: For SC-MTCH multiple TBs scheduling, a RRC configured scheduling gap is not inserted after the end of the last TB.

Proposal 2: Adopt the following text proposals in Annex A for TS 36.213.

Observation 1: For SC-MTCH multiple TBs scheduling, the configurable gap inserted after each TB may be used for backward compatibility purposes.

Observation 2: For the case that RRC configured scheduling gap collides with DL gap, postpone the overlapped scheduling gap together with NPDSCH transmission (i.e. option 1) does not increase network resource consumption, but the case that overlapped scheduling gap counted as part of DL gap (i.e. option 2) does.

Proposal 3: For SC-MTCH multiple TBs scheduling, if Rmax≥Ngap,threshold and RRC configured scheduling gap collides with DL gap, overlapped scheduling gap is postponed with NPDSCH transmission.

Observation 3: For SC-MTCH multiple TBs scheduling, the 20ms gap inserted every two TBs is used for UE processing needed at receiver and not for backward compatibility purposes.

Proposal 4: For SC-MTCH multiple TBs scheduling, if Rmax≥Ngap,threshold and 20ms gap inserted every two TBs collides with DL gap, overlapped part of the 20ms gap is counted as part of DL gap.

Proposal 5: Adopt the following text proposals in Annex B for TS 36.211.

Proposal 6: Adopt the following text proposals in Annex C for TS 36.211.

- References

R1-1913595, “RAN1 agreements for Rel-16 Additional Enhancements for NB-IoT”, Futurewei, RAN1#99, Reno, USA, 18th – 22nd November 2019.

R1-1912418, “Consideration on scheduling enhancement for NB-IoT”, ZTE, RAN1#99, Reno, USA, 18th – 22nd November 2019.

- Annex A. TP for scheduling gaps in SC-PTM in TS 36.213

---------------------------------------------- Start of Text Proposal ------------------------------------------

-------------------------------------------- Unchanged parts omitted -----------------------------------------

### 16.4.1	UE procedure for receiving the narrowband physical downlink shared channel

-------------------------------------------- Unchanged parts omitted -----------------------------------------

-	for ,

-	if the UE is configured with higher layer parameter multi-TB-DL-Unicast-Interleaving-config, and NPDSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI, and

-	NB-IoT DL subframes  with  are associated with TBr+1 ,

-	otherwise,

-	NB-IoT DL subframes  with  are associated with TBr+1 ,

-	For the UE configured with higher layer parameter multi-TB-SC-MTCH-config and the NPDSCH corresponding to a NPDCCH with DCI CRC scrambled by G-RNTI and ,

-	NB-IoT DL subframes  with  are associated with TBr+1 ,  and,

-	Ngap, 1 TB subframes,which is configured by higher layer, are inserted from the end of DL subframe nr+1∙NRepNSF-1 to the start of DL subframe nr+1∙NRepNSF, r = 0, 1, …, NTB-2 and,

-	Ngap, 2 TBs subframes, if Ngap, 1 TB is zero,are inserted from the end of DL subframe n2r+2∙NRepNSF-1 to the start of DL subframe n2r+2∙NRepNSF , r = 0, 1…,NTB2-1, where Ngap, 2 TBs = 20 if NRepNSF<12, and Ngap, 2 TBs = 0 otherwise.

-------------------------------------------- Unchanged parts omitted -----------------------------------------

----------------------------------------------- End of Text Proposal ------------------------------------------

- Annex B. TP for collision handling in SC-PTM in TS 36.211

---------------------------------------------- Start of Text Proposal ------------------------------------------

-------------------------------------------- Unchanged parts omitted -----------------------------------------

#### 10.2.3.4	Mapping to resource elements

-------------------------------------------- Unchanged parts omitted -----------------------------------------

The NPDSCH transmission can be configured by higher layers with transmission gaps where the NPDSCH transmission is postponed. There are no gaps in the NPDSCH transmission if where  is given by the higher layer parameter dl-GapThreshold and  is given by [4]. The gap starting frame and subframe is given by  where the gap periodicity,, is given by the higher layer parameter dl-GapPeriodicity. The gap duration in number of subframes is given by , where  is given by the higher layer parameter dl-GapDurationCoeff. For NPDSCH carrying the BCCH there are no gaps in the transmission.

If Rmax≥Ngap,threshold and Ngap,1 TB subframes defined in 16.4.1 of TS 36.213 overlaps with the transmission gaps, the overlapped part of the Ngap,1 TB subframes are postponed until not overlapping with the transmission gaps. There are no gaps in the Ngap,1 TB subframes if .

If Rmax≥Ngap,threshold and Ngap, 2 TBs subframes defined in 16.4.1 of TS 36.213 overlaps with the transmission gaps, the overlapped part of the Ngap, 2 TBs subframes are counted as part of the transmission gaps. There are no gaps in the Ngap, 2 TBs subframes if .

-------------------------------------------- Unchanged parts omitted -----------------------------------------

----------------------------------------------- End of Text Proposal ------------------------------------------

- Annex C. TP for unicast in TS 36.211

---------------------------------------------- Start of Text Proposal ------------------------------------------

-------------------------------------------- Unchanged parts omitted -----------------------------------------

### 10.1.5	SC-FDMA baseband signal generation

-------------------------------------------- Unchanged parts omitted -----------------------------------------

For , the time-continuous signal  for sub-carrier index in SC-FDMA symbol  in an uplink slot is defined by

for  where parameters for  and  are given in Table 10.1.5-1,  is the modulation value of symbol , and the phase rotation  is defined by

ϕk,j=ρl mod 2+φkl

ρ=π2        for BPSKπ4        for QPSK

φkl=0                                                                                 l=0φkl-1+2πΔfk+1/2N+NCP,lTs    l>0

l=0,1,…,NTBMrepNPUSCHNRUNslotsULNsymbUL-1

l= l mod NsymbUL

where NTB  is the number of transport blocks defined in 16.5.1 of 3GPP TS 36.213 [4]. If NTB >1 and interleaving between codewords is applied according to clause 16.5.1 of 3GPP TS 36.213 [4], then the symbol counter  is reset at the start of the transmission of NTB NPUSCH codewords and incremented for each symbol during the transmission. For other cases, the symbol counter  is reset to 0 at the start of each NPUSCH codeword transmission and incremented for each symbol during the transmission of the NPUSCH codeword.

-------------------------------------------- Unchanged parts omitted -----------------------------------------

----------------------------------------------- End of Text Proposal ------------------------------------------