3GPP TSG RAN WG1 Meeting #100	R1-2000405

e-Meeting, February 24th – March 6th, 2020

Agenda item:		7.2.5.2

Source:		Nokia, Nokia Shanghai Bell

Title:	Maintenance of Rel-16 URLLC UCI enhancements

Document for:		Discussion and Decision

# 1	Introduction

In this contribution, we summarize our input to the maintenance discussions on UCI enhancements on Rel-16 URLLC & IIoT enhancements.

Section 2 includes identified issues related to HARQ-ACK enhancements, which need to be clarified during RAN1#100 and Section 3 includes identified issues related to intra-UE prioritization/multiplexing. Identified issues are sorted by criticality within each section.

# 2	Identified issues related to HARQ-ACK enhancements

## 2.1 The association between the HARQ-ACK codebook and the priority

The priority index of a HARQ-ACK codebook is given by the following text in TS 38.213:

## 9.1 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-Codebook-List, the UE can be indicated by pdsch-HARQ-ACK-Codebook-List to generate one or two HARQ-ACK codebooks - a first HARQ-ACK codebook is associated with a PUCCH of priority index either 0 or 1 and a second HARQ-ACK codebook is associated with a PUCCH of priority index either 1 or 0, respectively - the UE is provided first and second for each of {PUCCH-Config, UCI-OnPUSCH, PDSCH-codeBlockGroupTransmission} by {PUCCHConfigurationList, UCI-OnPUSCH-List, PDSCH-CodeBlockGroupTransmission-List}, respectively, for use with the first and second HARQ-ACK codebooks, respectively

The current text implies that there is a certain way to indicate whether the first HARQ-ACK codebook is associated with priority index of 0 or 1, without specifying exactly how. However, we have an agreement from RAN1#98:

Agreements:

When at least two HARQ-ACK codebooks are simultaneously constructed for supporting different service types for a UE, the PHY identification of HARQ-ACK codebook is also used to determine the priority of the HARQ-ACK codebook for collision handling.

This means that there is a fixed association between a HARQ-ACK codebook and its priority index. Therefore, we propose a TP to align with this agreement.

Proposal 2.1: Adopt the following TP to TS 38.213 Sec. 9.1 so that there is a fixed relationship between a HARQ-ACK codebook and its priority index:

TP to TS 38.213 Sec. 9.1 ## 9.1 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-Codebook-List, the UE can be indicated by pdsch-HARQ-ACK-Codebook-List to generate one or two HARQ-ACK codebooks - a first HARQ-ACK codebook is associated with a PUCCH of priority index either 0 or 1 and a second HARQ-ACK codebook is associated with a PUCCH of priority index either 1 or 0, respectively - the UE is provided first and second for each of {PUCCH-Config, UCI-OnPUSCH, PDSCH-codeBlockGroupTransmission} by {PUCCHConfigurationList, UCI-OnPUSCH-List, PDSCH-CodeBlockGroupTransmission-List}, respectively, for use with the first and second HARQ-ACK codebooks, respectively <Unchanged text is omitted>

## 2.2 UE behavior when both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP

We have made the following agreements in RAN1#99:

Agreement

When both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP, a DCI format (from the formats 0_1/1_1/0_2/1_2) can be used to schedule PDSCH with different HARQ-ACK priorities or PUSCH with different priorities.

- This feature is UE optional

Our understanding of the agreement is that a UE can always be configured to monitor both DCI format 0_1/1_1 and DCI format 0_2/1_2 in a BWP (if it supports DCI format 0_2/1_2). What is optional is, when both sets of DCI formats are configured, whether the priority indicator field can still be present in the DCI formats to dynamically indicate the priority or not. However, currently TS 38.213, as highlighted below, defines whether both set of formats can be configured in a BWP as a UE capability, which does not align with our understanding. Therefore, changes are necessary.

A PUSCH or a PUCCH, including repetitions if any, can be of priority index 0 or of priority index 1. If a priority index is not provided for a PUSCH or a PUCCH, the priority index is 0. If in an active DL BWP a UE monitors PDCCH either for detection of DCI format 0_1 and DCI format 1_1 or for detection of DCI format 0_2 and DCI format 1_2, a priority index can be provided by a priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format 0_1 and DCI format 1_1 and for detection of DCI format 0_2 and DCI format 1_2, a DCI format 0_1 or a DCI format 0_2 can schedule a PUSCH transmission of any priority and a DCI format 1_1 or a DCI format 1_2 can schedule a PDSCH reception and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority.

In addition, the agreements above define the behavior when UE supports this optional feature. However, the UE behavior is not defined when the UE does not support this optional feature. To be more specific, for a UE not supporting this feature, when both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP, how to define the HARQ-ACK or PUSCH priority for each of the formats (given that one format cannot schedule different priorities). There can be two options:

- Option 1: Each format has a pre-defined priority. E.g. DCI formats 0_1/1_1 always corresponds to low priority, and DCI formats 0_2/1_2 always corresponds to high priority.

- Option 2: The priority of each DCI format is RRC configured. This would require additional RRC parameters.

Option 1 mandates that the two DL (or UL) formats correspond to two priorities, while Option 2 still allows the flexibility of mapping the two formats to the same priority. For example, for HARQ-ACK priority, even though PDSCH scheduled by DCI format 1_2 may be for URLLC, the HARQ-ACK priority does not necessarily need to have high priority, especially if the latency requirement is too stringent and does not allow HARQ retransmissions. In this case, Option 2 would still allow both DCI format 1_1 and 1_2 to be associated with low HARQ-ACK priority, while for PUSCH, DCI format 0_1 can be configured with low priority, and DCI format 0_2 can be configured with high priority. In our view, this function can also be extended to the UEs who support the optional feature of dynamic priority indication in this case.

This can be done by adding a new RRC parameter for each DCI format, which is what is assumed in the TP below. Alternatively, we can update the agreed RRC parameter PriorityIndicator-ForDCIFormatx_x (which takes the value of {enabled}) to Priority-ForDCIFormatx_x with the value set {0, 1, ‘dynamic’}, where ‘dynamic’ means the priority indicator field is present in the DCI format.

Proposal 2.2: When both DCI format 0_1/1_1 and 0_2/1_2 are configured to be monitored in a BWP, in case the HARQ-ACK/PUSCH priority is not dynamically indicated in the DCI, each of the DCI formats 0_1/1_1/0_2/1_2 is RRC configured with either low or high priority for HARQ-ACK/PUSCH (RRC impact). Adopt the following TP to TS 38.213 Sec. 9:

TP to TS 38.213 Sec. 9 # 9 UE procedure for reporting control information ## <Unchanged text is omitted> A PUSCH or a PUCCH, including repetitions if any, can be of priority index 0 or of priority index 1. - If a priority index is not provided for a PUSCH or a PUCCH, the priority index is 0. - If [Priority-ForDCIFormat0_1] is configured, the priority index of a PUSCH scheduled by DCI format 0_1 is given by [Priority-ForDCIFormat0_1]. - If [Priority-ForDCIFormat0_2] is configured, the priority index of a PUSCH scheduled by DCI format 0_2 is given by [Priority-ForDCIFormat0_2]. - If [Priority-ForDCIFormat1_1] is configured, the priority index of a PUCCH with HARQ-ACK for a PDSCH scheduled by DCI format 1_1 is given by [Priority-ForDCIFormat1_1]. - If [Priority-ForDCIFormat1_2] is configured, the priority index of a PUCCH with HARQ-ACK for a PDSCH scheduled by DCI format 1_2 is given by [Priority-ForDCIFormat1_2]. - If the priority indicator field is present in DCI format 0_1 or DCI format 0_2, the priority index of a PUSCH scheduled by DCI format 0_1 or DCI format 0_2 is given by the priority indication field. - If the priority indicator field is present in DCI format 1_1 or DCI format 1_2, the priority index of a PUCCH with HARQ-ACK for a PDSCH scheduled by DCI format 1_1 or DCI format 1_2 is given by the priority indication field. - A UE does not expect the priority indicator field to be present in DCI format 0_1, DCI format 0_2, DCI format 1_1, or DCI format 1_2 if the UE is configured to monitor PDCCH for detection of DCI format 0_1, DCI format 0_2, DCI format 1_1 and DCI format 1_2 in an active BWP and the UE does not indicate a capability [priorityIndicatorInMultiFormats]. If in an active DL BWP a UE monitors PDCCH either for detection of DCI format 0_1 and DCI format 1_1 or for detection of DCI format 0_2 and DCI format 1_2, a priority index can be provided by a priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format 0_1 and DCI format 1_1 and for detection of DCI format 0_2 and DCI format 1_2, a DCI format 0_1 or a DCI format 0_2 can schedule a PUSCH transmission of any priority and a DCI format 1_1 or a DCI format 1_2 can schedule a PDSCH reception and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority. If, after resolving overlapping for PUCCH and/or PUSCH transmissions of a same priority index, a UE determines to transmit - a first PUCCH of larger priority index, a PUSCH or a second PUCCH of smaller priority index, and a transmission of the first PUCCH would overlap in time with a transmission of the PUSCH or the second PUCCH, the UE does not transmit the PUSCH or the second PUCCH - a PUSCH of larger priority index, a PUCCH of smaller priority index, and a transmission of the PUSCH would overlap in time with a transmission of the PUCCH, the UE does not transmit the PUCCH - a first PUSCH of larger priority index on a serving cell, a second PUSCH of smaller priority index on the serving cell, and a transmission of the first PUSCH would overlap in time with a transmission of the second PUSCH, the UE does not transmit the second PUSCH, where at least one of the two PUSCH is not scheduled by a DCI format ## <Unchanged text is omitted>

## 2.3 Type 1 HARQ-ACK CB generation using dl-DataToUL-ACK-ForDCIFormat1_2 and pdsch-TimeDomainAllocationList-ForDCIformat1_2 (38.213, Sec. 9.1.2.1, 9.1.2.2)

With the new DCI format 1_2, the K1 and the TDRA table can be separately configured through dl-DataToUL-ACK-ForDCIFormat1_2, and pdsch-TimeDomainAllocationList-ForDCIformat1_2. The behavior described in TS 38.213 [2] does not account for these parameters in the Type-1 HARQ-ACK codebook construction.

Following the logic from Release 15, there is no need to construct a codebook, based on options (K1 sets and entries from TDRA tables) that cannot have been used to indicate the indicated PUCCH (sub-)slot. Therefore, the determination of the K1 set to determine the HARQ-ACK window needs to consider which DCI formats the UE is configured to monitor, and the configured K1 parameters and TDRA tables associated with the DCI formats.

Further, as each HARQ-ACK codebook has an associated priority, it makes sense to only consider the DCI formats which can indicate the HARQ-ACK codebook priority associated with the Type-1 HARQ-ACK codebook being constructed. The intention here is that for (i) the HARQ-Ack codebook of high priority (i.e. priority index 1) and (ii) for the HARQ-Ack codebook of priority index 0 if the new DCI format 1_2 is configured, i.e. the new applicable Rel-16 cases, we try to keep the Type-1 HARQ-Ack codebook small without including any unnecessary entries as partially has been done in Rel-15. This is seen especially of importance for the high-priority codebook.

In the TP below, the cases under the first-level bullet a) captures the cases related to determining the K1 set to be used for the HARQ-ACK codebook construction. For K1 set determination, there are 5 cases:

- Case a) is the case where the UE is configured to monitor only for DCI format 1_0 (among DL DCI formats). In this case the default slot timing values provides the K1 set. This case is a legacy case and follows the legacy behavior, and the condition that DCI format 1_2 is not configured is added.

- Case b) is the case where the UE is configured to monitor for DCI format 1_1 and possibly DCI format 1_0, but not DCI format 1_2. This case is also a legacy case and follows legacy behavior and the K1 set is provided by the parameter dl-DataToUL-ACK.

- Case c) This case is new and is the case where the UE is configured to monitor for DCI format 1_2 and possibly DCI format 1_0, but not DCI format 1_1. In this case the K1 set is provided by dl-DataToUL-ACK-ForDCIFormat1_2.

- Case d) This case is new and covers the case where the UE is configured to monitor for DCI format 1_1 and DCI format 1_2, and possibly DCI format 1_0. In this case, the K1 set is found as the union of dl-DataToUL-ACK for DCI format 1_1 and dl-DataToUL-ACK-ForDCIFormat1_2 as the PUCCH slot can be indicated by K1 values from both sets.

- Case e) considers the case when no DCI format is configured to be monitored for the HARQ-ACK priority associated with the codebook in the serving cell. This can happen, for example, when the DL DCI formats on a serving cell is configured to schedule low priority HARQ-ACK only, in which case there would be no DL DCI formats being able to schedule high priority HARQ-ACK on this serving cell.

The second part of the algorithm (the first-level bullet b))is determining the TDRA entries, for which the current specification does not capture the parameters associated with the DCI format 1_2 and the fact that only low priority HARQ-ACK priority can be indicated with DCI format 1_0. In the TP below, 4 cases are defined separately:

- Case a) the UE is not configured to monitor for DCI format 1_2. Here we have two sub-cases, one for low HARQ-ACK priority and one for high HARQ-ACK priority.

  - Case a1) the HARQ-ACK codebook is low priority. This is the legacy case and the legacy behavior is reused.

  - Case a2) the HARQ-ACK codebook is high priority. Since DCI format 1_0 can only schedule low priority HARQ-ACK, the UE can only have been configured with DCI format 1_1 for high priority HARQ-ACK (as DCI format 1_2 is not configured in Case a)). Therefore, the TDRA table for DCI format 1_1 is used (i.e. determined according to 38.214 Table 5.1.2.1.1-1).

- Case b) is new and covers the case where the UE is configured to monitor only for DCI format 1_2 (among the DL DCI formats). In this case the TDRA table for DCI format 1_2 is used (i.e. given by 38.214 Table 5.1.2.1.1-1A).

- Case c) is new and covers the case where the UE is configured to monitor for both DCI format 1_1 and DCI format 1_2, but not DCI format 1_0. In this case, the union of the TDRA tables for DCI format 1_1 and DCI format 1_2 (38.214 Table 5.1.2.1.1-1 and Table 5.1.2.1.1-1A) is used.

- Case d) is also new and is the case of the low-priority HARQ-Ack codebook where the UE is configured to monitor for DCI format 1_0 and DCI format 1_2, and possibly DCI format 1_1. In this case the TDRA entries are found as a union of the parameters feasible for each format, which is logically a union of case a) (legacy where the UE can be configured with DCI format 1_0 and/or DCI format 1_1 and b) (the UE is configured to monitor for DCI format 1_2 only). Therefore, this case is to be considered as the logical extension of the legacy case of a1 including the applicable TDRA table of DCI format 1_2.

In the current version of 38.213, it is defined that the UE is not expected to be indicated by a slot timing value with DCI format 1_0, which is not an intersection between the default set {1,2,3,4,5,6,7,8} and the set of slot timing values provided in dl-DataToUL-ACK. In the TP, this logic is extended to cover the case where the dl-DataToUL-ACK-ForDCIFormat1_2 is configured and dl-DataToUL-ACK is not, and the case where both dl-DataToUL-ACK-ForDCIFormat1_2 and dl-DataToUL-ACK is configured. In the latter case, the UE is not expecting to be indicated by a slot timing value which is not an intersection between the default values and the union of the values provided in dl-DataToUL-ACK-ForDCIFormat1_2 and dl-DataToUL-ACK.

Proposal 2.3: Introduce the interaction of Type 1 HARQ-ACK CB generation with the new RRC parameters dl-DataToUL-ACK-ForDCIFormat1_2 & pdsch-TimeDomainAllocationList-ForDCIformat1_2 in Sec. 9.1.2.1 & 9.1.2.2 of TS 38.213 by agreeing to the following TP with changes in red:

TP to TS 38.213, 9.1.2.1 on Type 1 HARQ-Ack CB with dl-DataToUL-ACK-ForDCIFormat1_2 & pdsch-TimeDomainAllocationList-ForDCIformat1_2 #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel In this subclause, the condition on which DCI format(s) a UE is configured to monitor PDCCH for refers only to the DCI format(s) that can schedule a PDSCH with the same HARQ-ACK priority index as the HARQ-ACK codebook. For a serving cell , an active DL BWP, and an active UL BWP, as described in Clause 12, the UE determines a set of occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot . If serving cell is deactivated, the UE uses as the active DL BWP for determining the set of occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based: a) on a set of slot timing values associated with the active UL BWP a) If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 on serving cell , is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} for DCI format 1_0 b) If the UE is configured to monitor PDCCH for DCI format 1_1 and is not configured to monitor PDCCH for DCI format 1_2 for serving cell , is provided by dl-DataToUL-ACK for DCI format 1_1 c) If the UE is configured to monitor PDCCH for DCI format 1_2 and is not configured to monitor PDCCH for DCI format 1_1 for serving cell , is provided by dl-DataToUL-ACK-ForDCIFormat1_2 for DCI format 1_2 d) If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 for serving cell , is provided by the union of dl-DataToUL-ACK for DCI format 1_1 and dl-DataToUL-ACK-ForDCIFormat1_2 for DCI format 1_2 e) If the UE is not configured to monitor PDCCH for any DCI format scheduling PDSCH for serving cell , is an empty set b) on a set of row indexes of a table that is provided either by a first set of row indexes of a table that is provided by pdsch-TimeDomainAllocationList in pdsch-ConfigCommon or by Default PDSCH time domain resource allocation A [6, TS 38.214], or by the union of the first set of row indexes and a second set of row indexes, if provided by pdsch-TimeDomainAllocationList in pdsch-Config, associated with the active DL BWP and defining respective sets of slot offsets , start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214], where the row indexes R of the table are provided by a) If the UE is not configured to monitor PDCCH for DCI format 1_2 on a serving cell c, a1) If the HARQ-ACK codebook is associated with priority index 0, the row indexes of the table are provided either by a first set of row indexes of a table that is provided by pdsch-TimeDomainAllocationList in pdsch-ConfigCommon or by Default PDSCH time domain resource allocation A [6, TS 38.214], or by the union of the first set of row indexes and a second set of row indexes, if provided by pdsch-TimeDomainAllocationList in pdsch-Config. a2) If the HARQ-ACK codebook is associated with priority index 1, the row indexes of the table are provided the applicable time domain resource allocation tables for DCI format 1_1 according to Table 5.1.2.1.1-1 of [6, TS 38.214]. b) If the UE is configured to monitor PDCCH for DCI format 1_2 but is not configured to monitor PDCCH for DCI format 1_0 and DCI format 1_1 on a serving cell c, the row indexes of the table are provided by the applicable time domain resource allocation table according to Table 5.1.2.1.1-1A of [6, TS 38.214]. c) If the UE is configured to monitor PDCCH for DCI format 1_1 and DCI format 1_2 but is not configured to monitor PDCCH for DCI format 1_0 on a serving cell c, the row indexes of the table are provided by the union of row indexes of the applicable time domain resource allocation tables for DCI format 1_1 according to Table 5.1.2.1.1-1 and for DCI format 1_2 according to Table 5.1.2.1.1-1A of [6, TS 38.214]. d) If the UE is configured to monitor PDCCH for DCI format 1_0 and DCI format 1_2 on a serving cell c, the row indexes of the table are provided by the union of a first set of row indexes that is provided by pdsch-TimeDomainAllocationList in pdsch-ConfigCommon or by Default PDSCH time domain resource allocation A [6, TS 38.214] and a second set of row indexes that is provided by the the applicable time domain resource allocation tables for DCI format 1_2 according to Table 5.1.2.1.1-1A of [6, TS 38.214], or by the union of the first set of row indexes, the second set of row indexes and a third set of row indexes, if provided by pdsch-TimeDomainAllocationList in pdsch-Config. ## <Unchanged text is omitted> If a UE is provided dl-DataToUL-ACK, the A UE does not expect to be indicated by DCI format 1_0 a slot timing value for transmission of HARQ-ACK information that does not belong to the intersection of the set of slot timing values {1, 2, 3, 4, 5, 6, 7, 8} and - the set of slot timing values provided by dl-DataToUL-ACK for the active DL BWP of a corresponding serving cell, if the UE is provided dl-DataToUL-ACK but is not provided dl-DataToUL-ACK-ForDCIFormat1_2, or - the set of slot timing values provided by dl-DataToUL-ACK-ForDCIFormat1_2 for the active DL BWP of a corresponding serving cell, if the UE is provided dl-DataToUL-ACK-ForDCIFormat1_2 but is not provided dl-DataToUL-ACK, or - the set of slot timing values provided by the union of dl-DataToUL-ACK and dl-DataToUL-ACK-ForDCIFormat1_2 for the active DL BWP of a corresponding serving cell, if the UE is provided dl-DataToUL-ACK and dl-DataToUL-ACK-ForDCIFormat1_2. ## <Unchanged text is omitted> TP to TS 38.213, 9.1.2.2 on Type 1 HARQ-Ack CB with dl-DataToUL-ACK-ForDCIFormat1_2 #### 9.1.2.2 Type-1 HARQ-ACK codebook in physical uplink shared channel If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by DCI format 0_0, then - if the UE has not received any PDSCH or SPS PDSCH release that the UE transmits corresponding HARQ-ACK information in the PUSCH, based on a value of a respective PDSCH-to-HARQ_feedback timing indicator field in a DCI format scheduling the PDSCH reception or the SPS PDSCH release or on the value of dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format 0_1 or on the value of dl-DataToUL-ACK-ForDCIFormat1_2 if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format 0_2, in any of the occasions for candidate PDSCH receptions by DCI format 1_0 or DCI format 1_1 or SPS PDSCH on any serving cell , as described in Clause 9.1.2.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission; - else the UE generates the HARQ-ACK codebook as described in Clause 9.1.2.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH, unless the UE receives only a SPS PDSCH release, or only SPS PDSCH reception, or only a PDSCH that is scheduled by DCI format 1_0 with a counter DAI field value of 1 on the PCell in the occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the PDSCH reception as described in Clause 9.1.2. ## <Unchanged text is omitted>

## 2.4 Type-1 HARQ-ACK codebook algorithm compliance with PUCCH sub-slots (TS 38.213, Sec. 9.1.2.1)

The Type-1 HARQ-ACK codebook algorithm in the TS 38.213 [2] needs to be modified to accommodate operation with sub-slot based PUCCH spanning either 2 or 7 OS, possibly in combination with the cross-numerology case where the PDSCH and PUCCH SCSs are different. TS 38.213 has not been updated to specifically take into account sub-slot based HARQ-ACK feedback. For example, the following cases (illustrated in Figure 2.1) are not handled properly in the current specification [2]:

- The start and/or the end of the HARQ-ACK window does not have to be aligned with the DL slot boundary.

- A PUCCH sub-slot may cross the DL slot boundary.

Figure 2.1. Illustration of issues related to Type-1 HARQ-ACK codebook construction algorithm on sub-slot PUCCH. The examples use an HARQ-ACK window with dl_dataToUL_ACK =0,1,2,3, when HARQ-ACK is indicated in UL sub-slot nU.

The algorithm is therefore changed to accommodate these cases which is done in three high level steps (and implemented in detail in the TP below).

- Aggregate TDRA rows of PDSCH slots, where multiple PDSCH slots may overlap with one PUCCH slot in cross-numerology case. The rows are collected in a RUL set of rows. The number of PDSCH slots overlapping with a PUCCH slot is indexed by nD, starting at 0. The start symbol index is offset by nD⋅14 symbols.

- Split RUL into sets per PUCCH (sub-)slot, based on the ending symbol index (S+L-1). For example;

  - If subslotLength-ForPUCCH = 7, 2 sets R0 and R1 is defined

  - If subslotLength-ForPUCCH = 2, 7 sets R0, R1, …, R6 is defined.

  - If subslotLength-ForPUCCH is not provided, only R0 is defined and subslotLength-ForPUCCH=14 is assumed.

- The rest of the existing algorithm is reused, and with the pre-processing of sets Ri, the loop per PDSCH slot in a PUCCH slot (counted by nD is not needed) is removed. Further the check that the codebook only starts at the beginning of a DL slot is removed, as this might not be the case for an PUCCH slot defined as a sub-slot. In the iteration per PUCCH (sub-)slot nu-K1,k the corresponding set Ri is determined and used as R in the existing algorithm.

Proposal 2.4: Adopt the following TP for Type-1 HARQ-ACK codebook algorithm compliance with PUCCH sub-slots to Sec. 9.1.2.1 of TS 38.213 with the changes marked in red:

TP to TS 38.213, Sec. 9.1.2.1 #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel For a serving cell , an active DL BWP, and an active UL BWP, as described in Clause 12, the UE determines a set of occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot . If serving cell is deactivated, the UE uses as the active DL BWP for determining the set of occasions for candidate PDSCH receptions a DL BWP provided by firstActiveDownlinkBWP-Id. The determination is based: a) on a set of slot timing values associated with the active UL BWP a) If the UE is configured to monitor PDCCH for DCI format 1_0 and is not configured to monitor PDCCH for DCI format 1_1 on serving cell , is provided by the slot timing values {1, 2, 3, 4, 5, 6, 7, 8} for DCI format 1_0 b) If the UE is configured to monitor PDCCH for DCI format 1_1 for serving cell , is provided by dl-DataToUL-ACK for DCI format 1_1 b) on a set of row indexes RDL of a table that is provided either by a first set of row indexes of a table that is provided by pdsch-TimeDomainAllocationList in pdsch-ConfigCommon or by Default PDSCH time domain resource allocation A [6, TS 38.214], or by the union of the first set of row indexes and a second set of row indexes, if provided by pdsch-TimeDomainAllocationList in pdsch-Config, associated with the active DL BWP and defining respective sets of slot offsets , start and length indicators SLIV, and PDSCH mapping types for PDSCH reception as described in [6, TS 38.214] c) on the ratio between the downlink SCS configuration and the uplink SCS configuration provided by subcarrierSpacing in BWP-Downlink and BWP-Uplink for the active DL BWP and the active UL BWP, respectively d) if provided, on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated as described in Clause 11.1. e) s indicated by subslotLength-ForPUCCH in symbols, if provided, otherwise set to 14, resulting into Ns=14s PUCCH slots in a UL slot. If a UE - is not provided CORESETPoolIndex or is provided CORESETPoolIndex with a value of 0 for first CORESETs on active DL BWPs of serving cells, and - is provided CORESETPoolIndex with a value of 1 for second CORESETs on active DL BWPs of the serving cells, and - is provided ACKNACKFeedbackMode = JointFeedback where - a serving cell is placed in a first set S0 of NcellsDL,0 serving cells if the serving cell includes a first CORESET, and - a serving cell is placed in a second set S1 of NcellsDL,1 serving cells if the serving cell includes a second CORESET, and - serving cells are placed in a set according to an ascending order of a serving cell index the UE generates a Type-1 HARQ-ACK codebook for a set S0∪S1 of NcellsDL=NcellsDL,0+ NcellsDL,1 serving cells. For the set of slot timing values , the UE determines a set of occasions for candidate PDSCH receptions or SPS PDSCH releases according to the following pseudo-code. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to a single SPS PDSCH release is same as for a corresponding SPS PDSCH reception. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to multiple SPS PDSCH releases by a single DCI format is same as for a corresponding SPS PDSCH reception with the lowest SPS configuration index among the multiple SPS PDSCH releases. RUL=∅ Set nD=0 – index of a DL slot within an UL slot while nD<max2μDL-μUL,1 For each row on RDL, for which the PDSCH end symbol S+L-1 overlaps with the PUCCH slot nU-K1,k modify start OFDM symbol index S to S+14nD to obtain RDL Set RUL=RUL ∪ RDL nD=nD+1; end while i=0 while i <Ns r=0 while r< CRUL Ri=∅ if i∙s∙max⁡(2μDL-μUL,1)<S+L-1 ≤i+1∙s∙max⁡(2μDL-μUL,1) Ri= Ri ∪r end r=r+1 end end Set - index of occasion for candidate PDSCH reception or SPS PDSCH release Set Set Set to the cardinality of set Set k =0 – index of slot timing values , in descending order of the slot timing values, in set for serving cell while if modnU-K1,k+1,max2µUL-µDL,1=0 Set nD=0 index of a DL slot within an UL slot while nD< max2µUL-µDL,1 Set R=Ri to the set of rows, where i= modnU-K1,k , Ns Set to the cardinality of Set – index of row in set if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell or an active UL BWP change on the PCell and corresponding DL slot to PUCCH slot nU-K1,k slot nU-K1,k⋅2μDL-μUL+nD is before the slot for the active DL BWP change on serving cell or the active UL BWP change on the PCell k=k+1; else while if the UE is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot nU-K1,k⋅2μDL-μUL+nD-NPDSCHrepeat+1 to slot nU-K1,k⋅2μDL-μUL+nD , at least one symbol of the PDSCH time resource derived by row in each of the corresponding DL slot and the preceding NPDSCHrepeat-1 DL slots is configured as UL where is the k-th slot timing value in set , ; else ; end if end while if the UE does not indicate a capability to receive more than one unicast PDSCH per slot and , ; ; The UE does not expect to receive SPS PDSCH release and unicast PDSCH in a same slot; else Set to the cardinality of Set to the smallest last OFDM symbol index, as determined by the SLIV, among all rows of while Set while if for start OFDM symbol index for row br,k=j br,k,nD=j; - index of occasion for candidate PDSCH reception or SPS PDSCH release associated with row ; B=B∪br,k,nD; B=B∪br,k; else ; end if end while ; Set to the smallest last OFDM symbol index among all rows of ; end while end if nD=nD+1; k=k+1; end if end while end if k=k+1; end while If the UE indicates a capability to receive more than one PDSCH per slot, for occasions of candidate PDSCH receptions corresponding to rows of R associated with a same value of br,kbr,k,nD , where br,k∈B br,k,nD∈B, the UE does not expect to receive more than one PDSCH in a same DL slot. ## <Unchanged text is omitted>

## 2.5 HARQ-ACK CB with the new DCI formats 0_2 and 1_2 (38.213, Sec. 9.1.2, 9.1.2.1, 9.1.2.2, 9.1.3.2)

In the CR phase, we introduced in some places DCI formats 0_2 and 1_2 by e.g. specifically mentioning 1_2 or 0_2 where applicable, or only referring to ‘a DCI format’ (if all UL grant / DL assignment formats are applicable). But it seems such changes are still missing in various places in Sec. 9.1.X.X – and the following logic is used here in the changes:

- 0_2 with / without DAI: DCI format 0_1  a DCI format that includes / does not include a DAI field (wording used in Sec. 9)

- 1_2 with /without total DAI: DCI format 1_1  a DCI format that includes / does not include a total DAI field (modification from the wording above for DL assignment)

- 1_2 & CBG: DCI format 1_0  a DCI format that does not support CBG-based PDSCH receptions (wording reused from Sec. 9.1.3.1)

Proposal 2.5: Introduce the interaction of HARQ-Ack CB generation and the new DCI formats 0_2 & 1_2 in sections 9.1.2, 9.1.2.1, 9.1.2.2 and 9.1.3.2 of TS 38.213 by adopting the following TPs with changes in red:

TP to TS 38.213, Sec. 9.1.2, 9.1.2.1, 9.1.2.2 and 9.1.3.2 on DCI formats 0_2 & 1_2 and HARQ-Ack procedures ### 9.1.2 Type-1 HARQ-ACK codebook determination This clause applies if the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static. A UE reports HARQ-ACK information for a corresponding PDSCH reception or SPS PDSCH release only in a HARQ-ACK codebook that the UE transmits in a slot indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format 1_0 or DCI format 1_1. The UE reports NACK value(s) for HARQ-ACK information bit(s) in a HARQ-ACK codebook that the UE transmits in a slot not indicated by a value of a PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format 1_0 or DCI format 1_1. ## <Unchanged text is omitted> TP to TS 38.213, Sec. 9.1.2.1 on DCI formats 0_2 & 1_2 and HARQ-Ack procedures #### 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel ## <Unchanged text is omitted> If a UE receives a SPS PDSCH, or a SPS PDSCH release, or a PDSCH that is scheduled by a DCI format 1_0 that does not support CBG-based PDSCH receptions and if - the UE is configured with one serving cell, and - , and - PDSCH-CodeBlockGroupTransmission is provided to the UE the UE generates HARQ-ACK information only for the transport block in the PDSCH or only for the SPS PDSCH release. If a UE receives a SPS PDSCH, or a SPS PDSCH release, or a PDSCH that is scheduled by a DCI format 1_0 that does not support CBG-based PDSCH receptions and if - the UE is configured with more than one serving cells, or - , and - PDSCH-CodeBlockGroupTransmission is provided to the UE the UE repeats times the HARQ-ACK information for the transport block in the PDSCH or for the SPS PDSCH release. ## <Unchanged text is omitted> A UE determines HARQ-ACK information bits, for a total number of HARQ-ACK information bits, of a HARQ-ACK codebook for transmission in a PUCCH according to the following pseudo-code. In the following pseudo-code, if the UE does not receive a transport block or a CBG, due to the UE not detecting a corresponding DCI format 1_0 or DCI format 1_1, the UE generates a NACK value for the transport block or the CBG. The cardinality of the set defines a total number of occasions for PDSCH reception or SPS PDSCH release for serving cell corresponding to the HARQ-ACK information bits. ## <Unchanged text is omitted> If , the UE determines a number of HARQ-ACK information bits for obtaining a transmission power for a PUCCH, as described in Clause 7.2.1, as where - is the number of transport blocks the UE receives in PDSCH reception occasion for serving cell if harq-ACK-SpatialBundlingPUCCH and PDSCH-CodeBlockGroupTransmission are not provided, or the number of transport blocks the UE receives in PDSCH reception occasion for serving cell if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format 1_0 that does not support CBG-based PDSCH receptions, or the number of PDSCH receptions if harq-ACK-SpatialBundlingPUCCH is provided or SPS PDSCH release in PDSCH reception occasion for serving cell and the UE reports corresponding HARQ-ACK information in the PUCCH. - is the number of CBGs the UE receives in a PDSCH reception occasion for serving cell if PDSCH-CodeBlockGroupTransmission is provided and the PDSCH reception is scheduled by a DCI format 1_1 that supports CBG-based PDSCH reception and the UE reports corresponding HARQ-ACK information in the PUCCH. ## <Unchanged text is omitted> TP to TS 38.213, Sec. 9.1.2.2 on DCI formats 0_2 & 1_2 and HARQ-Ack procedures #### 9.1.2.2 Type-1 HARQ-ACK codebook in physical uplink shared channel If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format 0_0 that does not include a DAI field, then - if the UE has not received any PDSCH or SPS PDSCH release that the UE transmits corresponding HARQ-ACK information in the PUSCH, based on a value of a respective PDSCH-to-HARQ_feedback timing indicator field in a DCI format scheduling the PDSCH reception or the SPS PDSCH release or on the value of dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format, in any of the occasions for candidate PDSCH receptions by a DCI format 1_0 or DCI format 1_1 or SPS PDSCH on any serving cell , as described in Clause 9.1.2.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission; - else the UE generates the HARQ-ACK codebook as described in Clause 9.1.2.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH, unless the UE receives only a SPS PDSCH release, or only SPS PDSCH reception, or only a PDSCH that is scheduled by DCI format 1_0 with a counter DAI field value of 1 on the PCell in the occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the PDSCH reception as described in Clause 9.1.2. A UE sets to NACK value in the HARQ-ACK codebook any HARQ-ACK information corresponding to PDSCH reception or SPS PDSCH release that the UE detects in a PDCCH monitoring occasion that starts after a PDCCH monitoring occasion where the UE detects a DCI format 0_0 or a DCI format 0_1 scheduling the PUSCH transmission. A UE does not expect to detect a DCI format switching a DL BWP within symbols prior to a first symbol of a PUSCH transmission where the UE multiplexes HARQ-ACK information, where is defined in [6, TS 38.214]. If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by DCI format 0_1 that includes a DAI field, the UE generates the HARQ-ACK codebook as described in Clause 9.1.2.1 when a value of the DAI field in the DCI format 0_1 is except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. The UE does not generate a HARQ-ACK codebook for multiplexing in the PUSCH transmission when unless the UE receives only a SPS PDSCH release, or only a SPS PDSCH, or only a PDSCH that is scheduled by DCI format 1_0 with a counter DAI field value of 1 on the PCell in the occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the PDSCH reception as described in Clause 9.1.2. if the PUSCH is scheduled by a DCI format that includes a DAI field and the DAI field in the DCI format 0_1 is set to '0'; otherwise, . ## <Unchanged text is omitted> TP to TS 38.213, Sec. 9.1.3.2 on DCI formats 0_2 & 1_2 and HARQ-Ack procedures #### 9.1.3.2 Type-2 HARQ-ACK codebook in physical uplink shared channel If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DAI field, then - if the UE has not received any PDCCH within the monitoring occasions for DCI formats scheduling PDSCH receptions or SPS PDSCH release on any serving cell and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission; - else, the UE generates the HARQ-ACK codebook as described in Clause 9.1.3.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by DCI format 0_1 that includes a DAI field, the UE generates the HARQ-ACK codebook as described in Clause 9.1.3.1, with the following modifications: - For the pseudo-code for the HARQ-ACK codebook generation in Clause 9.1.3.1, after the completion of the and loops, the UE sets where is the value of the DAI field in the DCI format 0_1 scheduling the PUSCH transmission according to Table 9.1.3-2 - For the case of first and second HARQ-ACK sub-codebooks, DCI format 0_1 and DCI format 0_2 includes a first DAI field corresponding to the first HARQ-ACK sub-codebook and a second DAI field corresponding to the second HARQ-ACK sub-codebook - harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. If a UE is not provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format 0_1 that includes a DAI field with DAI field value and the UE has not received any PDCCH within the monitoring occasions for PDCCH with a DCI format 1_0 or DCI format 1_1 for scheduling PDSCH receptions or SPS PDSCH release on any serving cell and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission. If a UE is provided PDSCH-CodeBlockGroupTransmission and the UE is scheduled for a PUSCH transmission by DCI format 0_1 that includes a DAI field with first DAI field value or with second DAI field value and the UE has not received any PDCCH within the monitoring occasions for PDCCH with a DCI format 1_0 or with DCI format 1_1, respectively, for scheduling PDSCH receptions or SPS PDSCH release on any serving cell and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in Clause 9.1.3.1, the UE does not multiplex HARQ-ACK information for the first sub-codebook or for the second sub-codebook, respectively, in the PUSCH transmission. ## <Unchanged text is omitted>

## 2.6 More than one PUCCH carrying HARQ-ACK overlapping with a PUSCH or another PUCCH with the same priority (38.213, Section 9.2.5)

In contrast to Rel-15 NR where a UE does not expect to transmit more than one PUCCH with HARQ-ACK information in a slot, with the current Rel-16 specs, there could be more than one PUCCH resource carrying HARQ-ACK information in a slot. This results in new scenarios of overlapping UL channels that need to be handled, such as more than one PUCCH resource carrying HARQ-ACK overlapping with a PUSCH or with a PUCCH resource carrying SR or CSI, as shown in Figures 2.2 and 2.3. We here focus on the cases where at least the first PUCCH carrying HARQ-ACK satisfies the conditions for multiplexing with the other UL channel(s).

Figure 2.2: Scenario where two PUCCH resources carrying HARQ-ACK overlap with a PUSCH.

Figure 2.3: Scenarios where two PUCCH resources carrying HARQ-ACKs overlap with i) a PUCCH resource carrying SR, ii) a PUCCH resource carrying CSI.

To cover all these scenarios, we propose the following simple and general handling rule. The rule consists in allowing, among the one or more PUCCH resource carrying HARQ-ACK that satisfy the conditions for multiplexing, at most one PUCCH resource carrying HARQ-ACK to be multiplexed with another UL channel. In fact, allowing more than one PUCCH resource carrying HARQ-ACK to be multiplexed on a PUSCH or another PUCCH resource would require decisions and substantial specification changes, which is not desirable given that we are already in Rel-16 URLLC CR / maintenance phase. On the decision of which HARQ-ACK to choose for multiplexing on PUSCH or PUCCH, we propose to use the following simple approach. If the earliest PUCCH resource carrying HARQ-ACK fulfills the multiplexing timeline conditions, then this HARQ-ACK is prioritized and multiplexed, and the other overlapping PUCCH resources carrying HARQ-ACKs are dropped. On the other hand, the case where none of the PUCCHs carrying HARQ-ACK fulfills the multiplexing conditions should be treated as an error case.

Proposal 2.6: For the scenario where a PUSCH or a PUCCH carrying SR and/or CSI would overlap with more than one PUCCHs carrying HARQ-ACK in a slot:

- The UE expects the earliest PUCCH carrying HARQ-ACK to satisfy the timeline conditions for multiplexing with the PUSCH or the PUCCH carrying SR and/or CSI.

- Only the HARQ-ACK information of the earliest PUCCH is multiplexed with the PUSCH or the PUCCH carrying SR and/or CSI, and the HARQ-ACK information of the remaining PUCCH resources is dropped.

Proposal 2.7: Adopt the following text proposal for the scenario where more than one PUCCH resource carrying HARQ-ACK overlap with a PUSCH or another PUCCH in TS 38.213, Section 9.2.5 with changes marked in red:

TP to TS 38.213, Section 9.2.5 ##### 9.2.5 UE procedure for reporting multiple UCI types ## <Unchanged text is omitted> If a UE would transmit multiple overlapping PUCCHs in a slot or overlapping PUCCH(s) and PUSCH(s) in a slot, one of the PUCCHs includes HARQ-ACK information in response to an SPS PDSCH reception, and any PUSCH is not in response to a DCI format detection, the UE expects that the first symbol of the earliest PUCCH or PUSCH satisfies the first of the previous timeline conditions with the exception that components associated to a SCS configuration for a PDCCH scheduling a PDSCH or a PUSCH are absent from the timeline conditions. If a UE would transmit multiple overlapping PUCCHs or overlapping PUCCH(s) and PUSCH(s) in a slot and there are more than one PUCCH carrying HARQ-ACK among these overlapping channels, the UE drops the PUCCH(s) carrying HARQ-ACK other than the earliest one, and the UE follows the behaviour defined in this subclause for the remaining PUCCH(s) and PUSCH(s). A UE does not expect a PUCCH or a PUSCH that is in response to a DCI format detection to overlap with any other PUCCH or PUSCH that does not satisfy the above timing conditions. ## <Unchanged text is omitted>

# 3	Identified issues related to intra-UE prioritization/multiplexing

## 3.1 Cancellation and scheduling restriction for the scenarios      PUSCH/PUCCH vs. PUSCH/PUCCH (38.213 Section 9, 38.214 Section 6.1)

In RAN1#99 meeting, the following was agreed regarding the cancellation of low priority UL transmission that overlaps with high priority UL transmission:

Agreement (RAN1#99 Reno)

When a high-priority UL transmission overlaps with a low-priority UL transmission in a slot,

- The UE is expected to cancel the low-priority UL transmission starting from Tproc,2 +d1 after the end of PDCCH scheduling the high-priority transmission, where

  - Tproc,2 is correponding to UE processing time capability for the carrier.

  - Value d1 is the time duration corresponding to 0,1,2 symbols reported by UE capability

  - Note: d_2,1=0 is for cancellation

- The minimum processing time of the high priority channel is extended by d2 symbols

  - Value d2 is the time duration corresponding to 0,1,2 symbols reported by UE capability

The overlapping condition is per repetition of the uplink transmission

As noticed in some related email discussions, this agreement has been causing some confusion among the companies regarding which scenarios it covers and the necessity to introduce both d1 and d2. We next provide our view on this agreement for the scenarios with a low-priority UL transmission overlapping with a high-priority UL transmission, where an UL transmission could correspond to a PUCCH/PUSCH.

First, we do not really see the need to introduce two timeline extensions, namely d1 and d2, in case of cancellation of a low priority UL transmission due to an overlap with a high priority UL transmission. From specification point of view, it could be sufficient to have the UE cancel the low priority transmission no later than the start of the high priority transmission. It can be left to UE implementation if the UE chooses to cancel earlier.

Proposal 3.1: In case a high priority UL transmission overlaps with a low priority UL transmission, the UE is expected to cancel the low priority UL transmission no later than the start of the high priority UL transmission.

Then, what is really important is for which cases the processing time of the high priority channel needs to be extended (d2 in the agreement), so that the UE can have sufficient time to cancel one transmission and start another. Here, we term this timeline extension as d2.

High-priority DG PUSCH vs. low-priority CG PUSCH:

For this scenario, it is important to stay consistent with the corresponding Rel-15 NR timeline condition. Specifically, in Rel-15, when a DG PUSCH overlaps with a CG PUSCH, the DG PUSCH is prioritized and the last symbol of the PDCCH scheduling the DG PUSCH should be at least Tproc,2 from the first symbol of the CG PUSCH. This timeline condition leaves enough time for MAC to do the prioritization, and basically only the PDU corresponding to DG PUSCH will be delivered to PHY in this case. Following this reasoning for Rel-16, if the time from the last symbol of the PDCCH scheduling the high-priority DG PUSCH to the first symbol of the low-priority CG PUSCH is at least Tproc,2, similarly there is no need to have the timeline extension d2 since there will be no UL transmission to be cancelled at PHY and only one PDU would be delivered to PHY in this case. An illustration for this case is given in Figure 3.1.

Figure 3.1: Timeline condition under which no timeline extension is needed for the scenario HP DG PUSCH vs. LP CG PUSCH, which is consistent with Rel-15 NR.

Otherwise, d2 is needed since the UE would have to cancel an ongoing LP CG PUSCH transmission. Consequently, we have the following scheduling restriction in this case: the UE is not expected to be scheduled with the start of the HP DG PUSCH to be earlier than Tproc,2 +d2 after the end of the last symbol of the scheduling PDCCH. An illustration of this case is given in Figure 3.2.

Figure 3.2: Timeline condition under which the timeline extension d2 is needed for the scenario HP DG PUSCH vs. LP CG PUSCH.

Proposal 3.2: For the scenario of high-priority DG PUSCH vs. low-priority CG PUSCH:

- If the time from the last symbol of the PDCCH scheduling the high-priority DG PUSCH to the first symbol of an overlapping low-priority CG PUSCH is shorter than Tproc,2, the UE is not expected to be scheduled with the start of the high-priority PUSCH to be earlier than Tproc,2 +d2 after the end of the last symbol of the scheduling PDCCH.

- If the time from the last symbol of the PDCCH scheduling the high-priority DG PUSCH to the first symbol of the low-priority CG PUSCH is greater than or equal to Tproc,2, there is no need for a timeline extension.

High-priority (HP) CG PUSCH vs. low-priority (LP) DG/CG PUSCH:

Since no timeline is defined for CG PUSCH, which is the channel with the higher priority here, then the handling should be left up to UE implementation. Specifically, when MAC decides to deliver the MAC PDU corresponding to the high-priority CG PUSCH, a reasonable UE implementation should leave enough time for PHY to cancel (if needed) the low priority transmission (DG PUSCH or CG PUSCH) and prepare for the high priority CG PUSCH transmission.

Figure 3.3: illustration of the scenario HP CG PUSCH vs. LP DG PUSCH.

Proposal 3.3: For the scenario of high-priority CG PUSCH vs. low-priority DG/CG PUSCH, it is up to UE implementation to make sure that the low-priority DG/CG PUSCH transmission can be cancelled before the start of the high-priority CG PUSCH.

High-priority DG PUSCH vs. low-priority PUCCH (i.e. HARQ-ACK, SR, CSI):

The scenarios where a high-priority DG PUSCH overlaps with a low-priority PUCCH (i.e. CSI, HARQ-ACK, SR) has some similarities with the scenario of high-priority DG PUSCH vs. low-priority CG PUSCH in the previous section. If the time from the last symbol of the PDCCH scheduling the high-priority DG PUSCH to the first symbol of the low-priority PUCCH resource is greater than or equal to Tproc,2, then no need to introduce any timeline extension since PHY will not have an ongoing low-priority PUCCH transmission to cancel. Otherwise, PHY may already start the low-priority transmission before it knows that this transmission needs to be cancelled, in which case the timeline extension d2 is needed. Therefore, as a general rule, if the time from the last symbol of the PDCCH scheduling the high-priority PUSCH to the first symbol of the low-priority PUCCH resource is shorter than Tproc,2, the UE is not expected to be scheduled with the start of the PUSCH to be earlier than Tproc,2 +d2 after the end of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH.

Figure 3.4: illustration for the case where the scenario HP DG PUSCH vs. LP PUCCH would require timeline extension.

Proposal 3.4: For the scenario of high-priority DG PUSCH vs. low-priority PUCCH (i.e. HARQ-ACK, SR, CSI):

- If the time from the last symbol of the PDCCH scheduling the PUSCH to the first symbol of the low-priority PUCCH resource is shorter than Tproc,2, the UE is not expected to be scheduled with the start of the high-priority PUSCH to be earlier than Tproc,2 +d2 after the end of the last symbol of the PDCCH carrying the DCI scheduling the high priority DG PUSCH.

- If the time from the last symbol of the PDCCH scheduling the high priority DG PUSCH to the first symbol of the low-priority PUCCH resource is greater than or equal to Tproc,2, there is no need for a timeline extension.

High-priority CG PUSCH vs. low-priority PUCCH (i.e. HARQ-ACK, SR, CSI):

The scenario here is similar to the scenario where a high-priority CG PUSCH overlaps with a low-priority DG/CG PUSCH, and the discussion provided earlier for this latter scenario holds here as well. Hence, for the scenario where a high-priority CG PUSCH overlaps with a low-priority PUCCH, there is no need to define a timeline extension, and it should be up to UE implementation to make sure that the low-priority PUCCH can be cancelled before the start of the high-priority CG PUSCH.

Proposal 3.5: For the scenario of high-priority CG PUSCH vs. low-priority PUCCH (i.e. HARQ-ACK, SR, CSI), it is up to UE implementation to make sure that the low-priority PUCCH can be cancelled before the start of the high-priority CG PUSCH.

High-priority SR vs. low-priority DG/CG PUSCH or HARQ-ACK or CSI:

The scenarios where a high-priority SR overlaps with a low-priority UL transmission (such as CG/DG PUSCH, HARQ-ACK, CSI) is similar to the scenario of high-priority CG PUSCH vs. low-priority DG/CG PUSCH since both CG PUSCH and SR are of high priority and they do not have timelines defined. Hence, the discussion provided earlier for the scenario of high-priority CG PUSCH vs. low-priority DG/CG PUSCH holds for the scenario of high-priority SR vs. low-priority UL transmission. Therefore, for the scenario here, it should be up to UE implementation to make sure that the low-priority transmission can be cancelled before the start of the high-priority SR transmission.

Proposal 3.6: For the scenario of high-priority SR vs. low-priority DG/CG PUSCH or HARQ-ACK or CSI, it is up to UE implementation to make sure that the low-priority transmission can be cancelled before the start of the high-priority SR transmission.

High-priority HARQ-ACK vs. low-priority DG/CG PUSCH or SR or CSI:

For the high-priority HARQ-ACK corresponds to dynamically scheduled PDSCH or to SPS PDSCH, HARQ-ACK resource index is indicated in DCI or RRC configured. The PUCCH resource may still be overridden by a later DCI that schedules another PDSCH pointing to the same slot/sub-slot for HARQ-ACK feedback. However, a DCI overriding the PUCCH resource should not come later than Tproc,2 before the start of the original indicated PUCCH resource. Once the UE decodes the last DCI scheduling PDSCH with HARQ-ACK feedback in the slot/sub-slot, it knows the PUCCH resource to be used for high-priority HARQ-ACK, then the UE could start the cancellation of the low-priority transmission. This includes the case where the UE has started the low-priority transmission and the cancellation is done for instance in parallel with the decoding of the PDSCH; this is possible since these two events are independent. Based on this observation and given that from the time the UE gets aware of the presence of high-priority HARQ-ACK (and thus of the overlapping channels) to the time the actual HARQ-ACK transmission starts is at least a couple of symbols, there is no need to introduce any timeline extension for the scenario here.

Proposal 3.7: For the scenario of high-priority HARQ-ACK vs. low-priority DG/CG PUSCH or SR or CSI, there is no need for a timeline extension.

Scheduling restrictions on canceled PUSCH resources for CG & DG PUSCH collision:

At RAN1#99, the following has been agreed:

Agreement

When a high-priority UL transmission overlaps with a low-priority UL transmission in a slot,

- The UE is not expected to be scheduled to transmit in the non-overlapping canceled symbols

First, in case the HP CG PUSCH cancels the LP DG PUSCH, such specific restriction is not needed as the UE is not expected to be scheduled with two overlapping DG PUSCHs (i.e. as DG PUSCH prioritization is not supported). In the case the HP DG PUSCH cancels HP/LP CG PUSCH, this cancelation behavior is basically the same as supported in Rel-15 CG/DG PUSCH collision and in Rel-15 there is no scheduling restriction for back-to-back PUSCH transmissions overlapping with a CG PUSCH defined. Therefore, we do not see a need to introduce this restriction in Rel-16 for HP DG to HP/LP CG PUSCH collision either.

Observation 3.1: For CG/DG PUSCH collision, the scheduling restriction ‘The UE is not expected to be scheduled to transmit in the non-overlapping canceled symbols’ is either not applicable (HP CG canceling LP DG PUSCH) or needs to be considered as an additional Rel-16 restriction which was not required in Rel-15 (i.e. DG PUSCH canceling CG PUSCH). It is not clear which cases this scheduling restriction is intended for.

Proposal 3.8: Revert the following agreement from RAN1#99:

Agreement When a high-priority UL transmission overlaps with a low-priority UL transmission in a slot, - The UE is not expected to be scheduled to transmit in the non-overlapping canceled symbols

On the required changes in the specifications, it is preferable to cover the intra-UE prioritization aspects discussed above only in TS 38.213, instead of covering the CG PUSCH vs. DG/CG PUSCH scenario in TS 38.214 and the other scenarios in TS 38.213. Consequently, the related text in brackets in TS 38.214, Section 6.1 should be removed and all aspects of CG PUSCH vs. DG PUSCH collision to be handled in 38.213.

Proposal 3.9: Adopt the following text proposal for the intra-UE multiplexing/prioritization to TS 38.213, Section 9 with the changes marked in red:

TP to TS 38.213, Section 9 ##### 9 UE procedure for reporting control information ## <Unchanged text is omitted> A PUSCH or a PUCCH, including repetitions if any, can be of priority index 0 or of priority index 1. If a priority index is not provided for a PUSCH or a PUCCH, the priority index is 0. If in an active DL BWP a UE monitors PDCCH either for detection of DCI format 0_1 and DCI format 1_1 or for detection of DCI format 0_2 and DCI format 1_2, a priority index can be provided by a priority indicator field. If a UE indicates a capability to monitor, in an active DL BWP, PDCCH for detection of DCI format 0_1 and DCI format 1_1 and for detection of DCI format 0_2 and DCI format 1_2, a DCI format 0_1 or a DCI format 0_2 can schedule a PUSCH transmission of any priority and a DCI format 1_1 or a DCI format 1_2 can schedule a PDSCH reception and trigger a PUCCH transmission with corresponding HARQ-ACK information of any priority. If, after resolving overlapping for PUCCH and/or PUSCH transmissions of a same priority index, a UE determines to transmit - a first PUCCH of larger priority index, a PUSCH or a second PUCCH of smaller priority index, and a transmission of the first PUCCH would overlap in time with a transmission of the PUSCH or the second PUCCH, the UE does not transmit the PUSCH or the second PUCCH, or cancel the PUSCH or the second PUCCH transmission at latest before the starting symbol of the first PUCCH transmission. - a PUSCH of larger priority index, a PUCCH of smaller priority index, and a transmission of the PUSCH would overlap in time with a transmission of the PUCCH, the UE does not transmit the PUCCH, or cancel the PUCCH transmission at latest before the starting symbol of the PUSCH transmission. - If the PUSCH is scheduled by a DCI and the time from the end of the last symbol of the PDCCH with the DCI scheduling the PUSCH to the start of the first symbol of the PUCCH is shorter than Tproc,2, the UE is not expected to be scheduled with the PUSCH starting earlier than Tproc,2 +d2 after the end of the last symbol of the PDCCH carrying the DCI, where Tproc,2 is given in [6, TS 38.214] for the corresponding PUSCH timing capability and d2 is determined by the reported UE capability [XXXXX]. - a first PUSCH of larger priority index on a serving cell, a second PUSCH of smaller priority index on the serving cell, and a transmission of the first PUSCH would overlap in time with a transmission of the second PUSCH, where at least one of the two PUSCHs is not scheduled by a DCI format, the UE does not transmit the second PUSCH, where at least one of the two PUSCH is not scheduled by a DCI format or cancel the second PUSCH transmission at latest before the starting symbol of the first PUSCH transmission. - If the first PUSCH is scheduled by a DCI format and the time from the end of the last symbol of the PDCCH with the DCI scheduling the first PUSCH to the start of the first symbol of the second PUSCH is shorter than Tproc,2, the UE is not expected to be scheduled with the first PUSCH starting earlier than Tproc,2 +d2 after the end of the last symbol of the PDCCH carrying the DCI, where Tproc,2 is given in [6, TS 38.214] for the corresponding PUSCH timing capability and d2 is determined by the reported UE capability [XXXXX]. In case of PUSCH repetitions, the overlapping handling is performed for each PUSCH repetition separately. In the remaining of this Clause, a UE multiplexes UCIs with same priority index in a PUCCH or a PUSCH. A PUCCH or a PUSCH is assumed to have a same priority index as a priority index of UCIs a UE multiplexes in the PUCCH or the PUSCH. ## <Unchanged text is omitted>

Proposal 3.10: Adopt the following text proposal for the intra-UE multiplexing/prioritization to TS 38.214, Section 6.1 with the changes marked in red:

TP to TS 38.214, Section 6.1 ##### 6.1 UE procedure for transmitting the physical uplink shared channel ## <Unchanged text is omitted> A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a PUSCH on a given serving cell overlapping in time with a transmission occasion, where the UE is allowed to transmit a PUSCH with configured grant according to [10, TS38.321], starting in a symbol j on the same serving cell if the end of symbol i is not at least N2 symbols before the beginning of symbol j. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH. [If [a UE reports the capability of intra-UE prioritization], and if a PUSCH corresponding to a configured grant and a PUSCH scheduled by a PDCCH on a serving cell are partially or fully overlapping in time, - If the PUSCH corresponding to the configured grant has priority in configuredGrantConfig set to 1 (i.e., high priority), and the PUSCH scheduled by the PDCCH is indicated as low priority by having the [priority indicator] field in the scheduling DCI set to 0 or by not having the [priority indicator] field present in the scheduling DCI, the UE is expected to transmit the PUSCH corresponding to the configured grant, and cancel the PUSCH transmission scheduled by the PDCCH at latest starting at the first symbol of the PUSCH corresponding to the configured grant. - Otherwise, the UE shall cancel the PUSCH transmission corresponding to the configured grant at latest starting M symbols after the end of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, and transmit the PUSCH scheduled by the PDCCH, where - M = Tproc,2 +d1, where Tproc,2 is given by clause 6.4 for the corresponding PUSCH timing capability assuming d2,1 = 0 and d1 is determined by the reported UE capability [XXXXX], - In this case, the UE is not expected to be scheduled for the PUSCH by the PDCCH where the PUSCH starts earlier than N symbols after the end of the last symbol of the PDCCH, where - N = Tproc,2 + d2, where Tproc,2 is the PUSCH preparation time of the PUSCH scheduled by the PDCCH using the associated PUSCH timing capability according to clause 6.4 and d2 is determined by the reported UE capability [YYYYY]. - In case of PUSCH repetitions, the overlapping handling is performed for each PUSCH repetition separately. - The UE is not expected to be scheduled for another PUSCH by a PDCCH where this PUSCH starts no earlier than the end of the prioritized transmitted PUSCH and before the end of the time domain allocation of the cancelled PUSCH.] A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a PUSCH on a given serving cell for a given HARQ process, if there is a transmission occasion where the UE is allowed to transmit a PUSCH with configured grant according to [10, TS38.321] with the same HARQ process on the same serving cell starting in a symbol j after symbol i, and if the gap between the end of PDCCH and the beginning of symbol j is less than N2 symbols. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH. ## <Unchanged text is omitted>

## 3.2 SRS handling

In TS 38.214, we have included the following text for SRS handling to accommodate Rel-16 behavior:

If a UE is not configured with [intraUEPrioritization] and PUSCH and SRS are transmitted in the same slot on a serving cell, the UE may only be configured to transmit SRS after the transmission of the PUSCH and the corresponding DM-RS. If a UE is configured with [intraUEPrioritization] and a PUSCH transmission would overlap in time with an SRS transmission on a serving cell, the UE does not transmit the SRS in the overlapping symbol(s).

There has been very little discussion on this. The current text requires the introduction of a new RRC parameter [intraUEPrioritization], which has not been included in the RRC parameter list yet. If we keep the text, we will need to add a new RRC parameter. Alternatively, we can define the behavior based on the UE capability instead of RRC configuration, thus avoiding the need for a new RRC parameter. We propose to use UE capability so that the RRC parameter can be saved.

Proposal 3.11: Adopt the following TP to TS 38.214 Sec. 6.2.1 so that Rel-16 SRS vs PUSCH collision handling is based on UE capability instead of the RRC configuration:

TP to TS 38.214, Section 6.2.1 6.2.1 UE sounding procedure <Unchanged text is omitted> If a UE does not report the capability of [intraUEPrioritization] is not configured with [intraUEPrioritization] and PUSCH and SRS are transmitted in the same slot on a serving cell, the UE may only be configured to transmit SRS after the transmission of the PUSCH and the corresponding DM-RS. If a UE reports the capability of [intraUEPrioritization] is configured with [intraUEPrioritization] and a PUSCH transmission would overlap in time with an SRS transmission on a serving cell, the UE does not transmit the SRS in the overlapping symbol(s). <Unchanged text is omitted>

# 4	Conclusions

In this contribution, we discuss the identified clarifications and additions on UCI enhancements with respect to the Rel-16 URLLC & IIoT WIs.

The discussions in Sec. 2 on HARQ-ACK related issues can be summarized as:

Proposal 2.1: Adopt the proposed TP to TS 38.213 Sec. 9.1 so that there is a fixed relationship between a HARQ-ACK codebook and its priority index.

Proposal 2.2: When both DCI format 0_1/1_1 and 0_2/1_2 are configured to be monitored in a BWP, in case the HARQ-ACK/PUSCH priority is not dynamically indicated in the DCI, each of the DCI formats 0_1/1_1/0_2/1_2 is RRC configured with either low or high priority for HARQ-ACK/PUSCH (RRC impact). Adopt the proposed TP to TS 38.213 Sec. 9.

Proposal 2.3: Introduce the interaction of Type 1 HARQ-ACK CB generation with the new RRC parameters dl-DataToUL-ACK-ForDCIFormat1_2 & pdsch-TimeDomainAllocationList-ForDCIformat1_2 in Sec. 9.1.2.1 & 9.1.2.2 of TS 38.213 by agreeing to the proposed TP.

Proposal 2.4: Adopt the proposed TP for Type-1 HARQ-ACK codebook algorithm compliance with PUCCH sub-slots to Sec. 9.1.2.1 of TS 38.213.

Proposal 2.5: Introduce the interaction of HARQ-Ack CB generation and the new DCI formats 0_2 & 1_2 in sections 9.1.2, 9.1.2.1, 9.1.2.2 and 9.1.3.2 of TS 38.213 by adopting the proposed TPs.

Proposal 2.6: For the scenario where a PUSCH or a PUCCH carrying SR and/or CSI would overlap with more than one PUCCHs carrying HARQ-ACK in a slot:

- The UE expects the earliest PUCCH carrying HARQ-ACK to satisfy the timeline conditions for multiplexing with the PUSCH or the PUCCH carrying SR and/or CSI.

- Only the HARQ-ACK information of the earliest PUCCH is multiplexed with the PUSCH or the PUCCH carrying SR and/or CSI, and the HARQ-ACK information of the remaining PUCCH resources is dropped.

Proposal 2.7: Adopt the proposed text proposal for the scenario where more than one PUCCH resource carrying HARQ-ACK overlap with a PUSCH or another PUCCH in TS 38.213, Section 9.2.5.

The discussions in Sec. 3 on Intra-UE prioritization/multiplexing related issues can be summarized as:

Proposal 3.1: In case a high priority UL transmission overlaps with a low priority UL transmission, the UE is expected to cancel the low priority UL transmission no later than the start of the high priority UL transmission.

Proposal 3.2: For the scenario of high-priority DG PUSCH vs. low-priority CG PUSCH:

- If the time from the last symbol of the PDCCH scheduling the high-priority DG PUSCH to the first symbol of an overlapping low-priority CG PUSCH is shorter than Tproc,2, the UE is not expected to be scheduled with the start of the high-priority PUSCH to be earlier than Tproc,2 +d2 after the end of the last symbol of the scheduling PDCCH.

- If the time from the last symbol of the PDCCH scheduling the high-priority DG PUSCH to the first symbol of the low-priority CG PUSCH is greater than or equal to Tproc,2, there is no need for a timeline extension.

Proposal 3.3: For the scenario of high-priority CG PUSCH vs. low-priority DG/CG PUSCH, it is up to UE implementation to make sure that the low-priority DG/CG PUSCH transmission can be cancelled before the start of the high-priority CG PUSCH.

Proposal 3.4: For the scenario of high-priority DG PUSCH vs. low-priority PUCCH (i.e. HARQ-ACK, SR, CSI):

- If the time from the last symbol of the PDCCH scheduling the PUSCH to the first symbol of the low-priority PUCCH resource is shorter than Tproc,2, the UE is not expected to be scheduled with the start of the high-priority PUSCH to be earlier than Tproc,2 +d2 after the end of the last symbol of the PDCCH carrying the DCI scheduling the high priority DG PUSCH.

- If the time from the last symbol of the PDCCH scheduling the high priority DG PUSCH to the first symbol of the low-priority PUCCH resource is greater than or equal to Tproc,2, there is no need for a timeline extension.

Proposal 3.5: For the scenario of high-priority CG PUSCH vs. low-priority PUCCH (i.e. HARQ-ACK, SR, CSI), it is up to UE implementation to make sure that the low-priority PUCCH can be cancelled before the start of the high-priority CG PUSCH.

Proposal 3.6: For the scenario of high-priority SR vs. low-priority DG/CG PUSCH or HARQ-ACK or CSI, it is up to UE implementation to make sure that the low-priority transmission can be cancelled before the start of the high-priority SR transmission.

Proposal 3.7: For the scenario of high-priority HARQ-ACK vs. low-priority DG/CG PUSCH or SR or CSI, there is no need for a timeline extension.

Observation 3.1: For CG/DG PUSCH collision, the scheduling restriction ‘The UE is not expected to be scheduled to transmit in the non-overlapping canceled symbols’ is either not applicable (HP CG canceling LP DG PUSCH) or needs to be considered as an additional Rel-16 restriction which was not required in Rel-15 (i.e. DG PUSCH canceling CG PUSCH). It is not clear which cases this scheduling restriction is intended for.

Proposal 3.8: Revert the following agreement from RAN1#99:

Agreement When a high-priority UL transmission overlaps with a low-priority UL transmission in a slot, - The UE is not expected to be scheduled to transmit in the non-overlapping canceled symbols

Proposal 3.9: Adopt the proposed text proposal for the intra-UE multiplexing/prioritization to TS 38.213, Section 9.

Proposal 3.10: Adopt the proposed text proposal for the intra-UE multiplexing/prioritization to TS 38.214, Section 6.1.

Proposal 3.11: Adopt the proposed TP to TS 38.214 Sec. 6.2.1 so that Rel-16 SRS vs PUSCH collision handling is based on UE capability instead of the RRC configuration.

# References

[1]	TS 38.212, v16.0.0 (i.e. ‘g00’)

[2]	TS 38.213, v16.0.0 (i.e. ‘g00’)

[3]	TS 38.214, v16.0.0 (i.e. ‘g00’)