3GPP TSG RAN WG1 #100			        R1-2000439

e-Meeting, February 24th – March 6th, 2020

Agenda Item:	7.2.2.2.3

Source: 	MediaTek Inc.

Title:	Remaining issues on enhancements to HARQ for NR-U operation

Document for: 	Discussion and Decision

# Introduction

In this contribution, we discuss remaining issues on enhancements to HARQ for NR-U operation and corrections to the latest version of the R16 NR-U specifications.

# Discussion

## Remaining issues on enhanced dynamic HARQ-ACK codebook

Agreement in RAN1#99:

For enhanced dynamic HARQ-ACK codebook, when more than one PDSCH group exists in a HARQ-ACK feedback report, the placement of HARQ-ACK feedback for the two groups is ordered based on increasing group index.

- FFS: Further enhancement for the case when only T-DAI is configured only for one group.

Agreement in RAN1#99:

For enhanced dynamic HARQ-ACK codebook, the number of requested groups is indicated in DCI 1_1 by one bit, indicating “0” for only the scheduled group, and “1” for both groups.

When enhanced dynamic HARQ-ACK codebook is configured, one DCI with format 1_1 can request HARQ-ACK feedbacks for two PDSCH groups in a PUCCH occasion according the request field (RQ = 1) in the DCI, and the placement of HARQ-ACK feedbacks for the two PDSCH groups is ordered based on increasing group index. Endorsed CR of TS 38.213 for R16 NR-U [1] already captures corresponding UE behaviour. However, even the DCI doesn't request feedback for two PDSCH (RQ = 0), a single PUCCH occasion is still possible to include HARQ-ACK feedbacks for the two PDSCH groups. As the example shown in Figure 1, the DCIs scheduling the two PDSCH groups indicate the same PUCCH occasion in slot #n+6 according to respective K1 values, and UE has to include the HARQ-ACK information respectively generated for the two PDSCH groups in the same PUCCH occasion. In this case, the agreement in RAN1#99 meeting made for the placement of HARQ-ACK feedbacks for two groups is still applicable. We believe that the corresponding UE behaviour should be also captured in 9.1.3.3 Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission in TS 38.213.

Figure 1. UE detects DCIs scheduling two PDSCH groups with respective K1 values indicating the same PUCCH occasion

Proposal 1: When enhanced dynamic HARQ-ACK codebook is configured, RAN1 specification should cover the following case:

- If UE detects DCIs scheduling two PDSCH groups with respective K1 values indicating the same PUCCH occasion, two PDSCH groups will exist in the same PUCCH occasion.

Agreement in RAN1#99:

If enhanced dynamic codebook is configured, for a PDSCH scheduled by DL DCI 1_0:

- NFI for group #0 is not signalled in DCI 1_0

- If the UE detects a DCI that indicates an NFI corresponding to group #0 since the last scheduled PUCCH that includes feedback for group #0 and before the PUCCH occasion that includes feedback corresponding to PDSCH scheduled with the DCI 1_0

  - UE follows the indicated NFI (in DCI 1_1) to determine the codebook.

- If the UE does not detect a DCI that indicates an NFI corresponding to group #0 since the last scheduled PUCCH that includes feedback for group #0 and before the PUCCH occasion that includes feedback corresponding to PDSCH scheduled with the DCI 1_0

  - UE assumes that the PDSCHs scheduled by DCI 1_0 since the latest PUCCH occasion do not belong to any group, and the UE only reports HARQ-ACK feedback for the PDSCHs scheduled by DCI 1_0 since the latest PUCCH occasion (as in Rel-15)

  - If the C-DAI received in the first DCI format 1_0 received after the latest PUCCH occasion is not equal to 1, and if the UE didn’t detect a DCI format 1_1 since the latest PUCCH occasion, the UE assumes that missed DCIs are DCI formats 1_0

In RAN1#99, it was agreed that if UE detects DCIs with respective K1 values indicating a same PUCCH occasion and none of these DCIs includes a NFI corresponding to PDSCH group #0, i.e., all of the detected DCIs have format 1_0, UE should assume that the PDSCHs scheduled by the DCIs since the latest PUCCH occasion do not belong to any group and only report HARQ-ACK feedback for the PDSCHs scheduled by the DCIs since the latest PUCCH occasion.

According to the agreement, RAN1 doesn’t preclude that multiplexing the HARQ-ACK feedback for PDSCHs not belong to any group and the HARQ-ACK feedback for PDSCH group #1 (with PGI = 1) in the same PUCCH occasion, as the example shown in Figure 2. However, current RAN1 agreements don't specify how to perform placement of the two HARQ-ACK feedbacks. In order to avoid unnecessary discussion since we don't see essential use cases, we propose to add a paragraph in 9.1.3.3 Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission in TS 38.213 to preclude the multiplexing of the HARQ-ACK feedback for PDSCHs not belong to any group and the HARQ-ACK feedback for PDSCH groups #1 in the same PUCCH occasion.

Proposal 2: When enhanced dynamic HARQ-ACK codebook is configured, UE does not expect to multiplex the HARQ-ACK feedbacks for PDSCHs not belonging to any group with the HARQ-ACK feedbacks for group #1 to the same PUCCH occasion.

Figure 2. An example of HARQ-ACK feedbacks for PDSCHs not belonging to any group and HARQ-ACK feedbacks for PDSCH group #1 (with PGI = 1) being scheduled to the same PUCCH occasion

Based on proposals 1 and 2 in this contribution, the following text proposal is provided for 9.1.3.3 Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission in TS 38.213:

===================================Text proposal starts========================================

#### 9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

*** Unchanged text is omitted ***

If q=0, the UE

includes only the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

elseif q=1 or ig=i(g+1mod2)

if g = 1

appends the first HARQ-ACK information to the second HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

else

append the second HARQ-ACK information to the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

end if

end if

*** Unchanged text is omitted ***

If a UE detects DCI formats with respective PDSCH-to-HARQ_feedback timing field values indicating a same PUCCH transmission occasion and none of the DCI formats includes a New_Feedback indicator field, the UE generates HARQ-ACK information as described in Subclause 9.1.3.1 or 9.1.3.2 for multiplexing in the PUCCH transmission occasion. The UE doesn't expect to multiplex the HARQ-ACK information and HARQ-ACK information corresponding to any detection of DCI format providing a value of g = 1 in the same PUCCH transmission occasion.

*** Unchanged text is omitted ***

===================================== Text Proposal Ends=====================================

## Corrections of enhanced dynamic HARQ-ACK codebook in TS 38.213

As the agreement made in RAN1#99, if UE does not detect a DCI that indicates an NFI corresponding to group #0 since the last scheduled PUCCH that includes feedback for group #0 and before the PUCCH occasion that includes feedback corresponding to PDSCH scheduled with the DCI 1_0, UE assumes that the PDSCHs scheduled by DCI 1_0 since the latest PUCCH occasion do not belong to any group, and the UE only reports HARQ-ACK feedback for the PDSCHs scheduled by DCI 1_0 since the latest PUCCH occasion (as in Rel-15). However, this agreement is not well captured in the endorsed CR of TS 38.213 for R16 NR-U [1].

Extract from Section 9.1.3.3 in the endorsed CR of TS 38.213 for R16 NR-U [1]:

If a UE detects DCI formats with respective PDSCH-to-HARQ_feedback timing field values indicating a same PUCCH transmission occasion and none of the DCI formats includes a New_Feedback indicator field, the UE generates HARQ-ACK information as described in Subclause 9.1.3.1 or 9.1.3.2 for multiplexing in the PUCCH transmission occasion.

From the extract above, “none of the DCI formats includes a New_Feedback indicator field” should be changed to “none of the DCI formats includes a New_Feedback indicator field and provides a value of g=0” to align with the agreement.

Also, there is a missing paragraph in the following extract from Section 9.1.3.3:

Extract from Section 9.1.3.3 in the endorsed CR of TS 38.213 for R16 NR-U [1]:

If hg+1mod2g=∅ or hg+1mod2g=h(g+1mod2), generate second HARQ-ACK information for PUCCH transmission occasion i(g+1mod2) in a slot, as described in Subclause 9.1.3.1, where

-	the second HARQ-ACK information corresponds to detections of DCI formats each providing a same value of g+1mod2, of h(g+1mod2), if any

-	at least one of the DCI formats provides a h(g+1mod2) value

-	m=0 corresponds to a PDCCH monitoring occasion, where the UE detects a DCI format that provides a value of g+1mod2, that is the first PDCCH monitoring occasion after a PDCCH monitoring occasion where the UE detects another DCI format that provides a value different than h(g+1mod2)

-	the PUCCH transmission occasion i(g+1mod2) is a last one for multiplexing second HARQ-ACK information and it is not after PUCCH transmission occasion i(g)

-	if VDAIg+1mod2≠∅, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Subclause 9.1.3.1, set Vtemp2=VDAIg+1mod2 for both sub-codebooks, if any.

When UE generates the second HARQ-ACK information corresponds to detections of DCI formats each providing a same value of g+1mod2, of h(g+1mod2), we see that at least one of the DCI formats should provide a K1 value indicating the slot of PUCCH transmission occasion i(g+1mod2) and thus this description should be added in the specification.

Based on the two corrections we mentioned above, the following text proposal is provided for 9.1.3.3 Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission in TS 38.213:

===================================Text proposal starts========================================

#### 9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

*** Unchanged text is omitted ***

Set k(g) to the value of a PDSCH-to-HARQ_feedback timing field, if any, in a DCI format providing a value of g. If the DCI format does not include a PDSCH-to-HARQ_feedback timing field, set k to the value provided by dl-DataToUL-ACK

*** Unchanged text is omitted ***

Generate first HARQ-ACK information for PUCCH transmission occasion i(g) in a slot, as described in Subclause 9.1.3.1, where

-	the first HARQ-ACK information corresponds only to detections of DCI formats each providing a same value of g, of h(g), if any, and at least one of the DCI formats providing a value of k(g) indicating the slot

*** Unchanged text is omitted ***

If hg+1mod2g=∅ or hg+1mod2g=h(g+1mod2), generate second HARQ-ACK information for PUCCH transmission occasion i(g+1mod2) in a slot, as described in Subclause 9.1.3.1, where

-	the second HARQ-ACK information corresponds to detections of DCI formats each providing a same value of g+1mod2, of h(g+1mod2), if any, and at least one of the DCI formats providing a value of k(g+1mod2) indicating the slot

*** Unchanged text is omitted ***

If a UE detects DCI formats with respective PDSCH-to-HARQ_feedback timing field values indicating a same PUCCH transmission occasion and none of the DCI formats includes a New_Feedback indicator field and provides a value of g=0, the UE generates HARQ-ACK information as described in Subclause 9.1.3.1 or 9.1.3.2 for multiplexing in the PUCCH transmission occasion.

*** Unchanged text is omitted ***

===================================== Text Proposal Ends=====================================

Proposal 3: Adopt the above text proposal to correct the specification for enhanced dynamic HARQ-ACK codebook in TS 38.213.

# Conclusion

In this contribution, HARQ operation and multi-TTI PUSCH scheduling for NR-U operation were discussed. Based on the discussion in the previous sections, we made the following proposals:

Proposal 1: When enhanced dynamic HARQ-ACK codebook is configured, RAN1 specification should cover the following case:

- If UE detects DCIs scheduling two PDSCH groups with respective K1 values indicating the same PUCCH occasion, two PDSCH groups will exist in the same PUCCH occasion.

Proposal 2: When enhanced dynamic HARQ-ACK codebook is configured, UE does not expect to multiplex the HARQ-ACK feedbacks for PDSCHs not belonging to any group with the HARQ-ACK feedbacks for group #1 to the same PUCCH occasion.

Proposal 3: Adopt the above text proposal to correct the specification for enhanced dynamic HARQ-ACK codebook in TS 38.213.

# Reference

- 3GPP RAN1 #99 R1-1913632, “Introduction of shared spectrum channel access”, Reno, US, November 2019.