3GPP TSG RAN WG1 #100                              	R1-2000520

e-Meeting, February 24th –March 6th, 2020

Source:	CATT

Title:	Remaining issues on physical layer structure for NR sidelink

Agenda Item:	7.2.4.1

Document for:	Discussion and Decision

Introduction

In this contribution we discuss remaining issues on physical layer structure for NR sidelink.

Discussion

TBS determination

For the TBS determination, there are two main remaining issues. The first issue is the TBS determination for CSI-only transmission.  The second issue is the TBS determination impacts due to 2nd stage SCI.

According to the agreements in RAN2#108 meeing, the sidelink CSI Reporting MAC CE is defined as follows [1]:

- RI: This field indicates the derived value of the Rand indicator for sidelink CSI reporting, 1bit;

- CQI: This field indicates the derived value of the Channel Quality Indicator for sidelink CSI reporting, 4bits;

- R: Reserved bit, 3bits;

The payload size of CSI feedback in MAC layer can be obtained as follows:

SL-SCH subheader + R/LCID subheader + MAC CE + Padding = 24/32 + 8 + 8 + padding bits

In case of CSI-only transmission, UE doesn’t transmit data and only transmits CSI on PSSCH, the minimum frequency granularity for PSSCH is sub-channel. Therefore, at least one sub-channel should be used for the PSSCH transmission in case of CSI-only transmission. However, the payload size of the CSI in MAC CE is so little that only the low-spectral efficiency 64QAM MCS table can be used for the TBS determination. Otherwise, the size of padding will be too much. This will waste the radio resources. Thus, an information field used for indicating the CSI-only transmission or overwriting the MCS table should be introduced in the SCI.

Proposal 1: An information field used for indicating the CSI-only transmission or overwriting the MCS table should be introduced in the SCI.

The second issue is about the impacts of 2nd stage SCI on the TBS determination for PSSCH. Since 2nd stage SCI is always appeared on PSSCH region.  The REs used for 2nd stage SCI should be excluded from the REs(NRE)  used for PSSCH transmission.

Proposal 2: Modify the transport block size determination according to the following TP.

-----------------------------------------------------Start of TP for 38.214--------------------------------------------------------

- 8.1.3.2	Transport block size determination

The UE shall first determine the number of REs (NRE) within the slot.

- A UE first determines the number of REs allocated for PSSCH within a PRB  by

- N'RE=NSCRB∙Nsymbsh-NDMRSPRB-NohPRB, where NSCRB=12 is the number of subcarriers in the frequency domain in a physical resource block, Nsymbsh Nsymbslotis the number of symbols L of the PSSCH allocation according to Clause 8.1.2.1 for scheduled PSSCH, NDMRSPRB is the number of REs for DM-RS per PRB in the allocated duration including the overhead of the DM-RS CDM groups without data, as described for PSSCH in Clause 8.2.2 or as indicated by SCI format 0_1, and NohPRB is the overhead configured by higher layer parameter xOverhead in PSSCH-ServingCellConfig. If the NohPRB is not configured (a value from 6, 12, or 18), the NohPRB is assumed to be 0.

- A UE determines the total number of REs allocated for PSSCH (NRE) by NRE=min(156,  N'RE)∙nPRB- Q'SCI2, where  nPRB is the total number of allocated PRBs for the UE, Q'SCI2 is the total number of REs for the 2nd  stage SCI as given by Clause 8.4.4 in TS 38.212.

- Next, proceed with steps 2-4 as defined in Clause 5.1.3.2

<Unchanged part omitted>

--------------------------------------------------------------End of TP -------------------------------------------------------------

Scrambling sequence generators

The scrambling sequence for Rel-16 sidelink physical channel are not well discussed, the remaining details about the scrambling sequence generators for sidelink physcal channels are discussed in following sections.

Scrambling sequence initialization for PSCCH

Because the 1st stage SCI transmitted on PSCCH is transparent for all UEs, the initialization of the scrambling sequence generator for PSCCH should be a fixed value. The scrambling sequence generator can be initialised with cinit=1024 at the start of every PSCCH slot, which can avoid potential scrambling sequence collision with PUSCH and PDSCH.

Proposal 3: The scrambling sequence generator can be initialised with cinit=1024 at the start of every PSCCH slot

-----------------------------------------------------Start of TP for 38.211--------------------------------------------------------

- 8.3.2.1	Scrambling

The block of bits b0,…,b(Mbit-1), where Mbit is the number of bits transmitted on the physical channel, shall be scrambled prior to modulation, resulting in a block of scrambled bits b0,…,b(Mbit-1) according to

bi=bi+cimod 2

where the scrambling sequence ci is given by clause 5.2.1. The scrambling sequence generator shall be initialized with

cinit=1024

<Unchanged part omitted>

--------------------------------------------------------------End of TP -------------------------------------------------------------

Scrambling sequence initialization for 2nd stage SCI and PSSCH

It has been agreed that scrambling operation for the 2nd stage SCI is applied separately with PSSCH. In clause 8.3.1.1 of TS38.211 for the scrambling of 2nd stage SCI and data, following scrambling operation is defined:

bqi=bqi+cqi-M(q)mod 2

Where M(q) is different for 2nd stage SCI and data.

For the initialization of the scrambling sequence for 2nd stage SCI and data, we can use the same initialization computational formula by reference to the scrambling operation for NR PUSCH:

cinit=nRNTI∙215+nID

For PSSCH, nRNTI can be replaced with NID, and nID should be equal to 1024, where NID=NIDX mod 216 where the quantity NIDX equals the decimal representation of CRC on the PSCCH associated with the PSSCH according to NIDX=i=0L-1pi∙2L-1-i with p and L given by clause 7.3.2 in TS 38.212

Proposal 4: Modify the initialization of the scrambling sequence for 2nd stage SCI and data transmitted on PSSCH according to the following TP.

-----------------------------------------------------Start of TP for 38.211--------------------------------------------------------

- 8.3.1.1	Scrambling

For the single codeword q=0, the block of bits bq0,…,bqMbit(q)-1, where Mbit(q)=Mbit,SCI2(q)+Mbit,data(q) is the number of bits in codeword q transmitted on the physical channel as defined in [4, TS 38.212], shall be scrambled prior to modulation.

Scrambling shall be done according to the following pseudo code

set i=0

while i<Mbitq

if b(q)i=x		// SCI placeholder bits

b(q)i=b(q)i-2

else

b(q)i=b(q)i+cq(i-M(q)) mod 2

end if

i = i + 1

end while

where the scrambling sequence cq(i) is given by clause 5.2.1 and

-    The scrambling sequence generator shall be initialized with

cinit=NID∙215+1024

-	for 0≤i<Mbit,SCI2(q)

-	M(q)=0

cinit

-	for Mbit,SCI2(q)≤i< Mbit(q)

-	M(q)=Mbit,SCI2q

cinit

where NID=NIDX mod 216 where the quantity NIDX equals the decimal representation of CRC on the PSCCH associated with the PSSCH according to NIDX=i=0L-1pi∙2L-1-i with p and L given by clause 7.3.2 in TS 38.212

<Unchanged part omitted>

--------------------------------------------------------------End of TP -------------------------------------------------------------

scrambling sequence initialization for PSBCH

The scrambling sequence generator shall be initialised at the start of every PSBCH subframe with as LTE V2X, where NIDSL is given by clause 8.4.2.1 in TS 38.212

Proposal 5: Modify the initialization of the scrambling sequence for the sidelink master information transmitted on PSBCH according to the following TP.

-----------------------------------------------------Start of TP for 38.211--------------------------------------------------------

- 8.3.3.1	Scrambling

The block of bits b0, …, b(Mbit-1), where Mbit is the number of bits transmitted on the physical sidelink broadcast channel, shall be scrambled prior to modulation, resulting in a block of scrambled bits  b0, …, b(Mbit-1) according to

bi=bi+c(i) mod 2

where the scrambling sequence ci is given by clause 5.2.1. The scrambling sequence generator shall be initialized with

cinit=NIDSL

where NIDSL is given by clause 8.4.2.1 in TS 38.211

<Unchanged part omitted>

--------------------------------------------------------------End of TP -------------------------------------------------------------

PSSCH DMRS pattern in frequency domain and multiplexing of 2 ports

DMRS frequency-domain pattern: Type-1 vs Type-2

In NR Uu, the structure of Type-1 and Type-2 DMRS is illustrated in Figure 1. It can be observed that Type-2 DMRS uses the less overhead of reference signal resource element than Type-1 DMRS. With the same data payload size, Type-2 DMRS can provide lower coding rate to improve the BLER performance comparing to that with Type-1 DMRS in highway-LOS channel as shown in Figure 2. However, urban-NLOS scenario will slightly deteriorate  the performance of Type-2 DMRS. The main reason is that the delay spread of NLOS model in urban scenario is much higher than that of LOS model in highway scenario, which means the coherence bandwidth of channel is smaller. The urban-NLOS channel changes quickly in the frequency domain. So we can see the performance of Type-2 becomes worse and worse with the increase of SCS.

The DMRS frequency-domain pattern should be determined by a new field in the SCI or higher layer indication in RRC signaling.

Proposal 6: Both Type-1 and Type-2 should be supported, and the frequency-domain pattern of PSSCH DMRS will be determined by a new field in the SCI or higher layer indication in RRC signaling.

Figure 1: NR Uu DMRS frequency pattern Type-1 and Type2

a)120km/h, Highway-LOS                                            b)500km/h, Highway-LOS

c)120km/h, Urban-NLOS

Figure 2: The bler performance of type-1 and type-2 DMRS pattern [2]

2-ports CDMed DMRS vs 2-ports FDMed DMRS

In this subsection, the performance of 2-ports CDMed DMRS and FDMed DMRS patterns is evaluated. The structure of CDM and FDM is provided in Figure 4. We can see from the Figure 3 that CDMed DMRS pattern use less overhead than FDMed pattern. In theory, CDMed pattern can bring better performance than FDMed pattern because of the lower coding rate under the same payload size. Besides, FMDed pattern with the puncture DMRS sequence is used for the channel estimation of each antenna port, it will cause degraded channel estimation performance because the punctured DMRS sequence has worse CM property compared with legacy full DMRS sequence and may lose orthogonality.

Figure 4 shows the evaluation results of CDMed and FDMed pattern. It can be observed that CDMed pattern has a better BLER performance compared to that with FDMed. As the CDMed pattern cannot keep working well, it is better to let sidelink UE itself determine the multiplexing. So the multiplexing of different ports should be determined by the SCI.

Proposal 7: For 2-ports DMRS, both CDMed pattern and FDMed pattern should be considered, and the multiplexing of different ports should be determined by the SCI.

Figure 3: CDMed DMRS vs FDMed DMRS

Speed: 120km/h                                                           Speed: 500km/h

Figure 4: BLER performance of CDMed pattern and FDMed pattern [2]

-----------------------------------------------------Start of TP for 38.211--------------------------------------------------------

- 8.4.1.1.2	    Mapping to physical resources

The sequence rm shall be mapped to the intermediate quantity ak,l(pj,μ) according to

ak,lpj,μ=wfk'r2n+k'

k=4n+2k'+∆      Configuration type 16n+k'+∆        Configuration type 2

k'=0,1

l=l+l'

n=0,1,…

j=0,1,…,v-1

where wfk' and Δ are given by Tables 8.4.1.1.2-1 and 8.4.1.1.2-2 and the configuration type is given by the higher-layer parameter or SCI [TBD], and both k' and Δ correspond to p0, …, pν-1.

The time-domain patterns used for the PSSCH DM-RS is indicated in the SCI as described in clause XXX of [TS 38.212].

The intermediate quantity ak,l(pj,μ) shall be precoded, multiplied with the amplitude scaling factor βPSSCHDMRS  in order to conform to the transmit power specified in [6, TS 38.214], and mapped to physical resources according to

ak,l(p0,μ)⋮ak,l(pρ-1,μ)=βPSSCHDMRSWak,l(p0,μ)⋮ak,l(pυ-1,μ)

where

-	the precoding matrix W is given by clause 8.3.1.4,

-	the set of antenna ports p0,…,pρ-1 is given by clause 8.3.1.4, and

-	the set of antenna ports p0,…,pυ-1 is given by [6, TS 38.214];

and the following conditions are fulfilled:

-	the resource elements ak,l(pj,μ) are within the common resource blocks allocated for PSSCH transmission.

The quantity k is defined relative to subcarrier 0 in common resource block 0 and the quantity l is defined relative to the start of the scheduled resources for transmission of PSSCH and the associated PSCCH, including the OFDM symbol duplicated as described in clauses 8.3.1.5 and 8.3.2.3.

The position(s) of the DM-RS symbols is given by l according to Table 8.4.1.1.2-3 where ld is the duration of the scheduled resources for transmission of PSSCH and the associated PSCCH, including the OFDM symbol duplicated as described in clauses 8.3.1.5 and 8.3.2.3.

able 8.4.1.1.2-1: Parameters for PSSCH DM-RS configuration type 1.

|  | CDM group λ | Δ |  |  |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | +1 | +1 |
| 1(CDM) | 0 | 0 | +1 | -1 |
| 1(FDM) | 1 | 1 | +1 | +1 |

Table 8.4.1.1.2-2: Parameters for PSSCH DM-RS configuration type 2.

|  | CDM group λ | Δ |  |  |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | +1 | +1 |
| 1(CDM) | 0 | 0 | +1 | -1 |
| 1(FDM) | 1 | 2 | +1 | +1 |

Table 8.4.1.1.2-3: PSSCH DM-RS time-domain location.

| ld in symbols | DM-RS position l |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PSCCH duration 2 symbols |  |  | PSCCH duration 3 symbols |  |  |
|  | Number of PSSCH DM-RS |  |  | Number of PSSCH DM-RS |  |  |
|  | 2 | 3 | 4 | 2 | 3 | 4 |
| 6 | 1, 5 |  |  | 1, 5 |  |  |
| 7 | 1, 5 |  |  | 1, 5 |  |  |
| 8 | 1, 5 |  |  | 1, 5 |  |  |
| 9 | 3, 8 | 1, 4, 7 |  | 4, 8 | 1, 4, 7 |  |
| 10 | 3, 8 | 1, 4, 7 |  | 4, 8 | 1, 4, 7 |  |
| 11 | 3, 10 | 1, 5, 9 | 1, 4, 7, 10 | 4, 10 | 1, 5, 9 | 1, 4, 7, 10 |
| 12 | 3, 10 | 1, 5, 9 | 1, 4, 7, 10 | 4, 10 | 1, 5, 9 | 1, 4, 7, 10 |
| 13 | 3, 10 | 1, 6, 11 | 1, 4, 7, 10 | 4, 10 | 1, 6, 11 | 1, 4, 7, 10 |

<Unchanged part omitted>

--------------------------------------------------------------End of TP -------------------------------------------------------------

The sequence generation of PSFCH

In RAN1#99, it has been agreed that:

- CDM between PSFCH transmissions from different UEs in the same PRB is supported as follows:

  - Base sequence is

    - (Pre-)configured per resource pool

The parameter hoppingID_PSFCH is introduced to generate the base sequence of the PSFCH used in the resource pool.

Proposal 8: Modify the sequence generation of PSFCH according to the following TP

-----------------------------------------------------Start of TP for 38.211--------------------------------------------------------

- 8.3.4.2.1	Sequence generation

The sequence xn shall be generated according to

xn=ru,vα,δn

n=0,1,…,NscRB-1

where ru,vα,δ(n) is given by clause 6.3.2.2 with the following exceptions:

-	mcs is given by clause XXX of [5, TS 38.213];

-	l is the OFDM symbol number in the PSFCH transmission where l=0 corresponds to the first OFDM symbol of the PSFCH transmission;

-	l' is the index of the OFDM symbol in the slot that corresponds to the first OFDM symbol of the PSFCH transmission in the slot given by [5, TS 38.213]

The function nCS(nc,l) is given by

ncsns,fμ,l=m=072mc8Nsymbslotns,fμ+8l+m

where the pseudo-random sequence c(i) is defined by clause 5.2.1. The pseudo-random sequence generator shall be initialized with  .

<Unchanged part omitted>

--------------------------------------------------------------End of TP -------------------------------------------------------------

Details on SCI design

For HARQ feedback in groupcast and unicast, when PSFCH resource is (pre-)configured in the resource pool, SCI explicitly indicates whether HARQ feedback is used or not for the corresponding PSSCH transmission [3]. This working assumption should be confirmed, and 1st stage SCI should introduce 1 bit information to indicate HARQ feedback is enable or disable.

There are some remaining details of resource indication in 1st SCI. In RAN1 #99, when NMAX = 2, the joint-coding of LTE-V2X principle can be reused to indicate the time-frequency resource of the second transmission.  5 bits are utilized to indicate the time position of the second transmission. 1 bit needs to be added in SCI content to indicate the relative timing position of the second transmission.

When NMAX = 3, it was agreed that 9 bits are applied to indicate the time-domain information of the second and the third transmission, there are possible three conditions for the relative timing position of the second and the third transmission corresponding to the current transmission: 1) Both are in the past; 2) One is in the past and the other is in the future; 3) Both are in the future. In order to indicate the relative timing position of the second and the third transmission accurately in SCI content, 2 bits should be added in SCI content to indicate the relative timing position of the second transmission and the third transmission [4].

In NR Uu, CA polar code is used for the channel coding of UL control information.  The CRC length for UCI encoded by Polar code is up to the payload size of UCI. If the payload size 12≤A≤19, the CRC length will be 6 bits. If the payload size A≥20, the CRC length will be 11 bits. If the payload size , payload needs to be padded to 12 bits. For the 2nd stage SCI on PSSCH, it is very similar to the UCI on the PUSCH. Thus,  the CA polar code is used for 2nd stage SCI with CRC length 11 bits to support the payload size of 2nd stage SCI larger than 20.

Besides, there are some issues on the Rate Matching of 2nd stage SCI in clause 8.4.4 of TS 38.211. Some suggestions are provided in the following TP.

Proposal 9: Modify the 1st stage SCI and 2nd stage SCI formats and contents according to the following TP.

-----------------------------------------------------Start of TP for 38.212--------------------------------------------------------

- 8.3.1.1	SCI format 0-1

SCI format 0-1 is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH

The following information is transmitted by means of the SCI format 0-1:

-	   Priority – 3 bits as defined in subclause x.x.x of [6, TS 38.214].

-	Frequency resource assignment – log2(N subChannel SLN subChannel SL + 12) bits when the value of the higher layer parameter maxNumResource is configured to 2; otherwise log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16) bits when the value of the higher layer parameter maxNumResource is configured to 3, as defined in subclause x.x.x of [6, TS 38.214].

-	Time resource assignment – 5 bits when the value of the higher layer parameter maxNumResource is configured to 2; otherwise 9 bits when the value of the higher layer parameter maxNumResource is configured to 3, as defined in subclause x.x.x of [6, TS 38.214].

-    Time resource relative position –  1 bit is supposed to be added in SCI content when the value of the higher layer parameter maxNumResource is configured to 2; Otherwise 2 bits when the value of the higher layer parameter maxNumResource is configured to 3.

-	Resource reservation period – log2(NreservPeriod) bits as defined in subclause x.x.x of [6, TS 38.214], if higher parameter reserveResourceDifferentTB is configured; 0 bit otherwise.

-    HARQ feedback enable/disable – [1] bit as defined in subclause x.x.x of [6, TS 38.214].

-	DMRS pattern – [x] bits as defined in subclause x.x.x of [6, TS 38.214], if more than one DMRS patterns are configured by higher layer parameter TimePatternPsschDmrs; 0 bit otherwise.

-	   2nd-stage SCI format – [x] bits as defined in subclause x.x.x of [6, TS 38.214].

-	   Beta_offset indicator – [2] bits as defined in subclause x.x.x of [6, TS 38.214].

-	Number of DMRS port – 1 bit as defined in subclause x.x.x of [6, TS 38.214].

-	   Modulation and coding scheme – 5 bits as defined in subclause x.x.x of [6, TS 38.214].

-     Reserved – [2 - 4] bits as determined by higher layer parameter [XXX], with value set to zero.

<Unchanged part omitted>

- 8.3.2		CRC attachment

CRC attachement is performed according to subclause 6.3.1.2.1 except that scrambling is not performed.

<Unchanged part omitted>

- 8.4.4	           Rate Matching

For 2nd-stage SCI transmission on PSSCH with SL-SCH, the number of coded modulation symbols for 2nd-stage SCI transmission, decoted as QSCI2', is determined as follows:

QSCI2'=minOSCI2+LSCI2∙βoffsetSCI2∙l=0NsymbolPSSCH-1MscSCI2(l)∙QmPSSCHr=0CSL-SCH-1Kr∙∙QmSCI2, αl=0NsymbolPSSCH-1MscSCI2(l)+γ

where

-	OSCI2 is the number of the SCI format 0-2 bits

-	LSCI2 is the number of CRC bits for SCI format 0-2, which is [xxx] bits.

-	βoffsetSCI2 is indicated in the corresponding SCI format 0-1.

-	CSL-SCH is the number of code blocks for SL-SCH of the PSSCH transmission.

-	MscPSSCH(l) is the scheduled bandwidth of PSSCH transmission, expressed as a number of subcarriers;

-	MscDMRS(l) is the number of subcarriers in OFDM symbol l that carries DMRS, in the PSSCH transmission.

-	MscPT-RS(l) is the number of subcarriers in OFDM symbol l that carries PT-RS, in the PSSCH transmission.

-	MscCSI-RS(l) is the number of subcarriers in OFDM symbol l that carries CSI-RS, in the PSSCH transmission.

-	MscSCI2(l) is the number of resource elements that can be used for transmission of the SCI format 0-2 in OFDM symbol l, for l=0,1,2⋯,NsymbolPSSCH, in PSSCH transmission and NsymbolPSSCH is the number of allocated symbols for the PSSCH except AGC symbol as defined in [6, TS 38.214]:

-	MscSCI2(l) = MscPSSCH(l) - MscDMRS(l) - MscPT-RS(l) -  MscCSI-RS(l)

-	γ is the number of otherwise vacant resource elements in the resource block to which the last coded symbol of the SCI format 0-2 belongs.

-	Kr is the r-th code block size for SL-SCH of the PSSCH transmission.

-    α is configured by higher layer parameter [SL-scaling].

-	QmPSSCH is the modulation order of the PSSCH;

-	QmSCI2 is the modulation order of the 2nd stage SCI;

<Unchanged part omitted>

--------------------------------------------------------------End of TP -------------------------------------------------------------

Frequency domain OCC design for PSCCH DMRS

In the email discussion after RAN1#99 [5], it has been agreed that NR PDCCH DMRS sequence is the baseline for PSCCH DMRS sequence at least with the following modification:

- n_ID is determined by a (pre-)configured value per resource pool

- Frequency-domain OCC is applied, one of the [2 or 3 or 4] OCCs is randomly selected by the Tx UE.

  - Note: there is no (pre-)configuration on the number of OCCs.

The increase in the number of OCCs can improve the performance of avoiding collision but also add complexity. Besides, the higher the number of OCCs is, the more likely the channel estimation performance is to be affected by frequency selection channel with high selectivity. We have to weigh the performance against the complexity.

When an odd number of PRB is (pre)configured for PSCCH, because there are 3 DMRS REs per RB in a symbol.  The total number of DMRS RE in a symbol also is odd. The frequency-domain OCC with length 2 or 4 will create some extra DMRS REs in this condition. There are no special circumstances for the FD-OCC with length 3. For the FD-OCC with length 3, it is difficult to achieve the orthogonality between different OCCs of UEs. So all of the kinds of OCCs have disadvantages.

Given all the evidence above it appears that the frequency-domain OCC with length 2 or 4 is better than that with length 3. In LTE V2X, one of 4 cyclic shifts is adopted randomly to alleviate the influence of collision. In NR V2X, the frequency-domain OCC with length 4 is also a good choice like LTE V2X. For the frequency-domain OCC with length 2, the performance of avoiding collision maybe hardly meet the requirement of system.

Proposal 10: Frequency-domain OCC is applied, one of the 4 OCCs is randomly selected by the Tx UE.

Besides, the candidate number of PRB for PSCCH should be a multiple of 4 to make sure the frequency-domain OCC with length 4 will not leave extra DMRS REs. Therefore, it is necessary to revise the working assumption about candidate numbers of PRBs for 2-symbol and 3-symbol PSCCH.

Proposal 11: The candidate number of PRBs for 2-symbol and 3-symbol PSCCH should be modified to a multiple of 4, such as { 8, 12, 16, 20, 24}.

Proposal 12: Modify the DMRS for PSCCH according to the following TP.

-----------------------------------------------------Start of TP for 38.211--------------------------------------------------------

- 8.4.1.3.2	Mapping to physical resources

The sequence rlm shall be multiplied with the amplitude scaling factor βPSCCH in order to conform to the transmit power specified in [5, 38.213] and mapped in sequence starting with rl0 to resource elements k,lp,μ in a slot on antenna port p=2000 according to

k=nNscRB+4k'+1
k'=0,1,2
n=0,1,…

where the following conditions are fulfilled

-	they are within the resource elements constituting the PSCCH

The reference point for k is subcarrier 0 in common resource block 0.

The quantity l is the OFDM symbol number within the slot.

is given by Table 8.4.1.3.2-1.

Table 8.4.1.3.2-1: Parameters for PSCCH DM-RS

| OCC group |  |  |  |  |
| --- | --- | --- | --- | --- |
| 0 | +1 | +1 | +1 | +1 |
| 1 | +1 | -1 | +1 | +1 |
| 2 | +1 | +1 | +1 | -1 |
| 3 | +1 | -1 | +1 | -1 |

<Unchanged part omitted>

--------------------------------------------------------------End of TP -------------------------------------------------------------

Others

In this section, some minor typos in clause 8.4.1.2.2 of TS38.211 are corrected as the following TP.

-----------------------------------------------------Start of TP for 38.211--------------------------------------------------------

- 8.4.1.2.2	Mapping to physical resources

The UE shall transmit phase-tracking reference signals only in the resource blocks used for the PSSCH, and only if the procedure in [6, TS 38.214] indicates that phase-tracking reference signals are being used.

The PSSCH PT-RS shall be mapped to resource elements according to

ak,lpo,μ⋮ak,lpρ-1,μ=βPT-RSWr(p0)(2n+k')⋮r(pυ-1)(2n+k')

k=4n+2k'+Δconfiguration type 16n+k'+Δ[configuration type 2]

when all the following conditions are fulfilled

-	l is within the OFDM symbols allocated for the PSSCH transmission;

-	resource element k,l is not used for sidelink CSI-RS, PSCCH, nor DM-RS associated with PSSCH;

-	k' and Δ correspond to p0, …, pυ-1

The precoding matrix W is given by clause 8.3.1.4. The quantity βPTRS is an amplitude scaling factor to conform with the transmit power specified in clause XXX of [6, TS 38.214].

The set of time indices l defined relative to the start of the PSSCH allocation is defined by

1. set i=0 and lref=0

2. if any symbol in the interval max lref+i-1LPT-RS+1, lref,…,lref+iLPT-RS overlaps with a symbol used for DM-RS according to clause 8.4.1.1.2

-	set i=1

-	set lref to the symbol index of the DM-RS symbol

-	repeat from step 2 as long as lref+iLPT-RS is inside the PSSCH allocation

3. add lref+iLPT-RS to the set of time indices for PT-RS

4. increment i by one

5. repeat from step 2 above as long as lref+iLPT-RS is inside the PSSCH allocation

where LPT-RS∈1,2,4 is defined in Table XXXX of [6, TS 38.214].

<Unchanged part omitted>

--------------------------------------------------------------End of TP -------------------------------------------------------------

Conclusion

In this contribution, we provide some proposals on the NR V2X physical structure as follow:

Proposal 1: An information field used for indicating the CSI-only transmission or overwriting the MCS table should be introduced in the SCI.

Proposal 2: Modify the transport block size determination according to the following TP.

Proposal 3: The scrambling sequence generator can be initialised with cinit=1024 at the start of every PSCCH slot

Proposal 4: Modify the initialization of the scrambling sequence for 2nd stage SCI and data transmitted on PSSCH according to the following TP.

Proposal 5: Modify the initialization of the scrambling sequence for the sidelink master information transmitted on PSBCH according to the following TP.

Proposal 6: Both Type-1 and Type-2 should be supported, and the frequency-domain pattern of PSSCH DMRS will be determined by a new field in the SCI or higher layer indication in RRC signaling.

Proposal 7: For 2-ports DMRS, both CDMed pattern and FDMed pattern should be considered, and the multiplexing of different ports should be determined by the SCI.

Proposal 8: Modify the sequence generation of PSFCH according to the following TP

Proposal 9: Modify the 1st stage SCI and 2nd stage SCI formats and contents according to the following TP.

Proposal 10: Frequency-domain OCC is applied, one of the 4 OCCs is randomly selected by the Tx UE.

Proposal 11: The candidate number of PRBs for 2-symbol and 3-symbol PSCCH should be modified to a multiple of 4, such as { 8, 12, 16, 20, 24}.

Proposal 12: Modify the DMRS for PSCCH according to the following TP.

References

  - RAN2#108 Chairman’s Notes, Reno, USA, 18th – 22th November 2019.

  - R1-1912153, “Physical layer structure for NR sidelink”, CATT.

  - 3GPP TSG RAN WG1 Meeting RAN1 #98bis chairman notes, October 2019.

  - R1-2000522, “Discussion on resource allocation mechanism for sidelink Mode 2 in NR V2X”, CATT.

  - R1-1913601, “Agreement of 5G V2X”, November 2019

  - 3GPP TS 38.211, “Physical channels and modulation”, V16.0.0.

  - 3GPP TS 38.212, “Multiplexing and channel coding”, V16.0.0.

  - 3GPP TS 38.213, “Physical layer procedures for control”, V16.0.0.

  - 3GPP TS 38.214, “Physical layer procedures for data”, V16.0.0.

- Appendix

In this section, we provide link level evaluation assumptions used for analysis of NR sidelink physical layer.

Table 7. Link level evaluation assumption

| Parameters | Assumptions |
| --- | --- |
| Carrier frequency | 5.9 GHz |
| Waveform | CP-OFDM, |
| Subcarrier spacing | 15 kHz, 30 kHz, 60 kHz |
| DS | 100ns |
| PSSCH length | 12 symbols |
| DMRS pattern | 4 columns:<2,5,8,11> 3 columns:<3,6,9> 2 columns:<3,9> Type-1,Type-2 |
| Channel estimation | No-Ideal |
| Channel model | Sidelink: Highway-LOS, Urban-NLOS |
| Channel coding | LDPC |
| Antenna configuration | (Tx, Rx) = (2, 4) 1/2 ports |
| UE speed (Relative ) | 120 km/h, 280km/h, 500km/h |
| Modulation | 16QAM |
| TBS | 800/200 bytes |
| PRB | 25 RBs |