3GPP TSG RAN WG1 #100	R1-2000544

e-Meeting, February 24th – March 6th, 2020

Agenda Item:	7.2.2.2.1

Source: 	Fujitsu

Title:	Remaining issues on channel access procedures

Document for:	Discussion/decision

- Introduction

In this contribution, we would like to discuss some remaining issues with respect to CWS adjustment and CP extension.

- Discussion

  - CWS adjustment

Issue #1

In the last RAN1 meeting, the following agreement was achieved. As implied by the highlight part, this agreement also applies to UE maintaining CWS. However, it seems not captured in the current specification yet. A text proposal is provided in Appendix I.

Agreement: For CWS adjustment for an LBT sub-band when a single contention window is maintained per LBT subband, all CBGs (if any are present) and TBs that partially or fully overlap with that LBT sub-band are taken into account. - CW is reset if “ACK” is received for at least 10 % of the CBGs or for at least one TB in the reference duration - Note: Other procedures for contention window adjustment within an LBT subband are also applicable - A UE can choose to apply feedback only based on TBs for CW adjustment

Proposal 1: Capture the agreement below for UE maintaining CWS.

- A text proposal is provided in Appendix I.

Agreement: For CWS adjustment for an LBT sub-band when a single contention window is maintained per LBT subband, all CBGs (if any are present) and TBs that partially or fully overlap with that LBT sub-band are taken into account. - CW is reset if “ACK” is received for at least 10 % of the CBGs or for at least one TB in the reference duration - Note: Other procedures for contention window adjustment within an LBT subband are also applicable - A UE can choose to apply feedback only based on TBs for CW adjustment

Issue #2

According to the agreement in issue #1, a UE can maintain a CWS per LBT sub-band. On the other hand, for UL transmission, UE may switch BWP. However, how to maintain a CWS for a LBT sub-band in case of BWP switching has not been discussed. And it is ambiguous in current specification. For sake of fair co-existence of UEs, it is necessary to be specified/clarified. One simple way is, a UE maintains a CWS per LBT sub-band included in configured UL BWP(s). Then BWP switching would have no impact on CWS value.

Observation 1: For sake of fair co-existence of UEs, CWS maintenance/adjustment in case of BWP switching is necessary to be specified/clarified.

Proposal 2: A UE maintains a CWS per LBT sub-band included in configured UL BWP(s).

  - CP extension

In the last meeting, the following relevant agreements were made.

Agreement: At least for LBE operation: - For signaling of LBT type & CP extension for both Fallback DL assignment and Fallback UL Grant, the following table is used: LBT Type; CP extension. Cat1 16 µs; C2*symbol length – 16 us – TA. Cat2 25 µs; C3*symbol length – 25 us – TA. Cat2 25 µs; C1*symbol length – 25 us. Cat4; 0. Agreement: - For RAR, we use the same LBT type and CP extension tables and CAPC selection mechanism as used for the Fallback DCI UL grant. Agreement: C2 &C3 (from prior agreement on CP extension) are UE specifically RRC configured - Note: this does not restrict the maximum TA that can be used in a cell - FFS: Agree the values of C2 and C3 in RAN1#99 Agreement: The supported range of values of both C2 and C3 (from prior agreement on CP extension) that can be configured by RRC is - 1, 2, …, 28 for 15 and 30 kHz SCS, and - 2, 3, …, 28 for 60 kHz SCS Note: Other agreements are not superseded by this agreement

Issue #1

In current specification (also according to the agreements above), length of CP extension is calculated based on Cx*symbol length.

However, since symbols in a slot have different length due to different CP length for NCP case, the “symbol length” here is ambiguous. Therefore, it is necessary to modify the previous agreements. Considering the intension of introducing CP extension, it could be calculated according to the length of Cx symbols before the first symbol of a PUSCH/PUCCH. A text proposal is provided in Appendix II.

Observation 2: According to the previous agreements, the symbol length in the formula for CP extension calculation is ambiguous for NCP case. It is necessary to modify the relevant agreement.

Proposal 3: CP extension should be calculated according to the length of Cx symbols before the first symbol of a PUSCH/PUCCH.

- A text proposal is provided in Appendix II.

Issue #2

In current specification (also according to the agreements above), DCI format 0_0 with CRC scrambled based on TC-RNTI and RAR can indicate UE to use CP extension based on C2/C3 for msg.3 transmission. Note that DCI format 0_0 with CRC scrambled based on TC-RNTI is used for msg.3 retransmission, which is only needed for CBRA. It implies UL transmissions in CBRA can use CP extension based on C2/C3.

However, according to the agreement above, C2 &C3 are UE specifically RRC configured. In case of CBRA, after receiving a preamble, gNB cannot identify the UE transmitting the preamble, thus it cannon know whether the UE is configured with C2&C3. If gNB indicates UE to use CP extension based on C2/C3, while UE has no C2/C3 configuration, the UE could not calculate CP extension.

Considering the case, two alternatives are as below.

- Alt.1: Clarify that CP extension based on C2/C3 is not supported for CBRA.

- Alt.2: In case of CBRA, C2/C3 is determined based on TA, such that the CP extension is shorter than 1 symbol.

Observation 3: UL transmissions in CBRA can use CP extension based on C2/C3 according to the current specification/agreements. But it does not work.

Proposal 4: Consider two alternatives:

- Alt.1: Clarify that CP extension based on C2/C3 is not supported for CBRA.

- Alt.2: In case of CBRA, C2/C3 is determined based on TA, such that the CP extension is shorter than 1 symbol.

- Conclusion

According to the discussions above, we have the following proposals.

Proposal 1: Capture the agreement below for UE maintaining CWS.

- A text proposal is provided in Appendix I.

Agreement: For CWS adjustment for an LBT sub-band when a single contention window is maintained per LBT subband, all CBGs (if any are present) and TBs that partially or fully overlap with that LBT sub-band are taken into account. - CW is reset if “ACK” is received for at least 10 % of the CBGs or for at least one TB in the reference duration - Note: Other procedures for contention window adjustment within an LBT subband are also applicable - A UE can choose to apply feedback only based on TBs for CW adjustment

Observation 1: For sake of fair co-existence of UEs, CWS maintenance/adjustment in case of BWP switching is necessary to be specified/clarified.

Proposal 2: A UE maintains a CWS per LBT sub-band included in configured UL BWP(s).

Observation 2: According to the previous agreements, the symbol length in the formula for CP extension calculation is ambiguous for NCP case. It is necessary to modify the relevant agreement.

Proposal 3: CP extension should be calculated according to the length of Cx symbols before the first symbol of a PUSCH/PUCCH.

- A text proposal is provided in Appendix II.

Observation 3: UL transmissions in CBRA can use CP extension based on C2/C3 according to the current specification/agreements. But it does not work.

Proposal 4: Consider two alternatives:

- Alt.1: Clarify that CP extension based on C2/C3 is not supported for CBRA.

- Alt.2: In case of CBRA, C2/C3 is determined based on TA, such that the CP extension is shorter than 1 symbol.

Appendix I: Text proposal for Proposal 1

37.213 g00

4.2.1.0.4	Channel access procedures for UL multi-channel transmission(s)

If a UE

-	is scheduled to transmit on a set of channels C, and if Type 1 channel access procedure is indicated by the UL scheduling grants for the UL transmissions on the set of channels C, and if the UL transmissions are scheduled to start transmissions at the same time on all channels in the set of channels C , or

-	intends to perform an uplink transmission on configured resources on the set of channels C with Type 1 channel access procedure, and if UL transmissions are configured to start transmissions on the same time all channels in the set of channels C, and

if the channel frequencies of set of channels C is a subset of one of the sets of channel frequencies defined in subclause 5.7.4 in [2]

-	the UE may transmit on channel ci∈C using Type 2 channel access procedure as described in subclause 4.2.1.2,

-	if Type 2 channel access procedure is performed on channel ci immediately before the UE transmission on channel cj∈C, i≠j, and

-	if the UE has accessed channel cj using Type 1 channel access procedure as described in subclause 4.2.1.1,

-	where channel cj is selected by the UE uniformly randomly from the set of channels C before performing Type 1 channel access procedure on any channel in the set of channels C.

-	the UE may not transmit on channel ci∈C within the bandwidth of a carrier, if the UE fails to access any of the channels, of the carrier bandwidth, on which the UE is scheduled or configured by UL resources.

For determining CWp for channel ci, any PUSCH that fully or partially overlaps with channel ci, is used in the procedure described in subclause 4.2.2.2.

Appendix II: Text proposal for Proposal 3

38.212 g00

Table 7.3.1.1.1-4: Channel access type & CP extension for DCI format 0_0 and DCI format 1_0

| Bit field mapped to index | Channel Access Type | CP extension |
| --- | --- | --- |
| 0 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | length of C2 symbols before the first symbol of a PUSCH/PUCCH – 16 us – TA |
| 1 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | length of C3 symbols before the first symbol of a PUSCH/PUCCH – 25 us – TA |
| 2 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | length of C1 symbols before the first symbol of a PUSCH/PUCCH – 25 us |
| 3 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 |

Table 7.3.1.1.2-35: Allowed entries for DCI format 0_1, configured by high layer parameter ULDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16

| Entry index | Channel Access Type | CP extension | CAPC |
| --- | --- | --- | --- |
| 0 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 1 |
| 1 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 2 |
| 2 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 3 |
| 3 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 4 |
| 4 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | length of C2 symbols before the first symbol of a PUSCH– 16 us – TA | 1 |
| 5 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | length of C2 symbols before the first symbol of a PUSCH – 16 us – TA | 2 |
| 6 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | length of C2 symbols before the first symbol of a PUSCH – 16 us – TA | 3 |
| 7 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | length of C2 symbols before the first symbol of a PUSCH – 16 us – TA | 4 |
| 8 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 1 |
| 9 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 2 |
| 10 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 3 |
| 11 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 | 4 |
| 12 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | length of C2 symbols before the first symbol of a PUSCH – 16 us – TA | 1 |
| 13 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | length of C2 symbols before the first symbol of a PUSCH – 16 us – TA | 2 |
| 14 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | length of C2 symbols before the first symbol of a PUSCH – 16 us – TA | 3 |
| 15 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | length of C2 symbols before the first symbol of a PUSCH – 16 us – TA | 4 |
| 16 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 | 1 |
| 17 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 | 2 |
| 18 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 | 3 |
| 19 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 | 4 |
| 20 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | length of C1 symbols before the first symbol of a PUSCH – 25 us | 1 |
| 21 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | length of C1 symbols before the first symbol of a PUSCH – 25 us | 2 |
| 22 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | length of C1 symbols before the first symbol of a PUSCH – 25 us | 3 |
| 23 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | length of C1 symbols before the first symbol of a PUSCH – 25 us | 4 |
| 24 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | length of C3 symbols before the first symbol of a PUSCH – 25 us – TA | 1 |
| 25 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | length of C3 symbols before the first symbol of a PUSCH– 25 us – TA | 2 |
| 26 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | length of C3 symbols before the first symbol of a PUSCH – 25 us – TA | 3 |
| 27 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | length of C3 symbols before the first symbol of a PUSCH– 25 us – TA | 4 |
| 28 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 | 1 |
| 29 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 | 2 |
| 30 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 | 3 |
| 31 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 | 4 |
| 32 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | length of C1 symbols before the first symbol of a PUSCH – 25 us | 1 |
| 33 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | length of C1 symbols before the first symbol of a PUSCH – 25 us | 2 |
| 34 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | length of C1 symbols before the first symbol of a PUSCH – 25 us | 3 |
| 35 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | length of C1 symbols before the first symbol of a PUSCH – 25 us | 4 |
| 36 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | length of C2 symbols before the first symbol of a PUSCH – 16 us – TA | 1 |
| 37 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | length of C2 symbols before the first symbol of a PUSCH – 16 us – TA | 2 |
| 38 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | length of C2 symbols before the first symbol of a PUSCH– 16 us – TA | 3 |
| 39 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | length of C2 symbols before the first symbol of a PUSCH – 16 us – TA | 4 |
| 40 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | length of C3 symbols before the first symbol of a PUSCH – 25 us – TA | 1 |
| 41 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | length of C3 symbols before the first symbol of a PUSCH – 25 us – TA | 2 |
| 42 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | length of C3 symbols before the first symbol of a PUSCH – 25 us – TA | 3 |
| 43 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | length of C3 symbols before the first symbol of a PUSCH – 25 us – TA | 4 |

Table 7.3.1.2.2-6: Allowed entries for DCI format 1_1, configured by high layer parameter DLDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16

| Entry index | Channel Access Type | CP extension |
| --- | --- | --- |
| 0 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 |
| 1 | Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | length of C2 symbols before the first symbol of a PUCCH – 16 us – TA |
| 2 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | 0 |
| 3 | Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213] | length of C2 symbols before the first symbol of a PUCCH – 16 us – TA |
| 4 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | 0 |
| 5 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | length of C1 symbols before the first symbol of a PUCCH – 25 us |
| 6 | Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213] | length of C3 symbols before the first symbol of a PUCCH – 25 us – TA |
| 7 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | 0 |
| 8 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | length of C1 symbols before the first symbol of a PUCCH – 25 us |
| 9 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | length of C2 symbols before the first symbol of a PUCCH – 16 us – TA |
| 10 | Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213] | length of C3 symbols before the first symbol of a PUCCH – 25 us – TA |

38.211 g00

5.3.1	OFDM baseband signal generation for all channels except PRACH and RIM-RS

The time-continuous signal  on antenna port p and subcarrier spacing configuration μ for OFDM symbol  in a subframe for any physical channel or signal except PRACH is defined by

sl(p,μ)t=sl(p,μ)ttstart,lμ≤t<tstart,lμ+Tsymb,lμ0otherwise
sl(p,μ)t=k=0Ngrid,xsize,μNscRB-1ak,l(p,μ)ej2πk+k0μ-Ngrid,xsize,μNscRB2Δft-NCP,lμTc-tstart,lμ
k0μ=Ngrid,xstart,μ+Ngrid,xsize,μ2NscRB-Ngrid,xstart,μ0+Ngrid,xsize,μ02NscRB2μ0-μ
Tsymb,lμ=Nuμ+NCP,lμTc

where t=0 at the start of the subframe,

and

-	 is given by clause 4.2;

-	 is the subcarrier spacing configuration;

-	μ0 is the largest μ value among the subcarrier spacing configurations by the higher-layer parameter scs-SpecificCarrierList.

In case of cyclic extension of the first OFDM symbol l allocated for PUSCH/PUCCH transmission, the time-continuous signal sext(p,μ)t for the interval tstart,lμ-Text≤t<tstart,lμ preceding the first OFDM symbol for PUSCH/PUCCH is given by

sext(p,μ)t=sl(p,μ)t

where t<0 refers to the signal in the previous subframe and Text is given by

-	Table 5.3.1-1 with the index, C2 and C3 given by the higher-layer parameters CP-ExtensionC2-r16 and CP-ExtensionC3-r16, respectively, and TTA given by clause 4.3.1 for dynamically scheduled PUSCH/PUCCH transmissions; l0 is the index of the first OFDM symbol for PUSCH/PUCCH.

-	the procedure in [6, TS 38.214] for a PUSCH transmission using configured grant.

The starting position of OFDM symbol  for subcarrier spacing configuration in a subframe is given by

Table 5.3.1-1: The cyclic extension Text.

| Index | μ=0 | μ=1 | μ=2 |
| --- | --- | --- | --- |
| 0 | - | - | - |
| 1 | Tsymb, mod(l0-1，Nslotsubframe,μNsymbslot)ll0-1μ-25∙10-6 | Tsymb, mod(l0-1，Nslotsubframe,μNsymbslot)l0-1lμ-25∙10-6 | l=mod(l0-2，Nslotsubframe,μNsymbslot)mod(l0-1，Nslotsubframe,μNsymbslot)Tsymb,lμ2Tsymb,lμ-25∙10-6 |
| 2 | l=mod(l0-C2，Nslotsubframe,μNsymbslot)mod(l0-1，Nslotsubframe,μNsymbslot)Tsymb,lμC2Tsymb,lμ-16∙10-6-TTA | l=mod(l0-C2，Nslotsubframe,μNsymbslot)mod(l0-1，Nslotsubframe,μNsymbslot)Tsymb,lμC2Tsymb,lμ-16∙10-6-TTA | l=mod(l0-C2，Nslotsubframe,μNsymbslot)mod(l0-1，Nslotsubframe,μNsymbslot)Tsymb,lμC2Tsymb,lμ-16∙10-6-TTA |
| 3 | l=mod(l0-C3，Nslotsubframe,μNsymbslot)mod(l0-1，Nslotsubframe,μNsymbslot)Tsymb,lμC3Tsymb,lμ-25∙10-6-TTA | l=mod(l0-C3，Nslotsubframe,μNsymbslot)mod(l0-1，Nslotsubframe,μNsymbslot)Tsymb,lμC3Tsymb,lμ-25∙10-6-TTA | l=mod(l0-C3，Nslotsubframe,μNsymbslot)mod(l0-1，Nslotsubframe,μNsymbslot)Tsymb,lμC3Tsymb,lμ-25∙10-6-TTA |

References

[1] 3GPP TSG RAN WG1 Meeting #99, RAN1 Chairman’s Notes, Reno, USA, Nov. 18th – 22nd, 2019.