3GPP TSG RAN WG1 Meeting #100e R1-2000603 e-Meeting, February 24th – March 6th, 2020 CR-Form-v11.2. [DRAFT] CHANGE REQUEST. 38.213; CR; rev; Current version:; 15.8.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on SSB-RO association. Source to WG:; Samsung. Source to TSG:. Work item code:; NR_newRAT-Core; Date:; 2019-02-24. Category:; F; Release:; Rel-15. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; For SSB-RO association, there will be a case that multiple SSB mapped to a RO, so when determining the SSB-RO mapping cycle, there will be some preambles left unmapped instead of the ROs, however the spec only captures the RO left umapped will be unused, there is no description on the preambles, so there is an ambigulity on whether those leftover preambles will be mapped or not. Following the same logic that leftover ROs are not used, the leftover preambles should not be used in the SSB-RO mapping as well.. Summary of change:; Add the “or preambles” in the place where the leftover ROs are not mapped.. Consequences if not approved:; UE(s) and/or gNB have ambigulity of specifications.. Clauses affected:; 8.1. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: If gNB implements this change but UE doesn’t: UE may select some preambles these are not exptected to be detected by gNB; and also UE may receive some preamble index in the PDCCH ordered CFRA which other UE consdier these preamble index as CBRA preambles, so to jerpodize the performace of CFRA preambles. If UE implements this change but gNB doesn’t: No impact..

8.1	Random access preamble

*** Unchanged text is omitted ***

An association period, starting from frame 0, for mapping SS/PBCH blocks to PRACH occasions is the smallest value in the set determined by the PRACH configuration period according Table 8.1-1 such that  SS/PBCH blocks are mapped at least once to the PRACH occasions within the association period, where a UE obtains  from the value of ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon. If after an integer number of SS/PBCH blocks to PRACH occasions mapping cycles within the association period there is a set of PRACH occasions or PRACH preambles that are not mapped to  SS/PBCH blocks, no SS/PBCH blocks are mapped to the set of PRACH occasions or PRACH preambles. An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH blocks repeats at most every 160 msec. PRACH occasions not associated with SS/PBCH blocks after an integer number of association periods, if any, are not used for PRACH transmissions.

*** Unchanged text is omitted ***