3GPP TSG RAN WG1 #100-e	R1-2000637

e-Meeting, February 24th – March 6th, 2020

Agenda item:	7.2.9.1

Source:	Samsung

Title:	Remaining issues for PDCCH-based power saving signal/channel

Document for:	Discussion and Decision

# Introduction

In RAN1#99 [1], the following were agreed:

Agreements:

- PS_offset is set with the unit of milliseconds.   The value range of the PS_offset is from [0.125, 0.25, 0.5, 1, 2, …, [15]]ms

  - FFS whether or not to add extra values

Agreements:

The minimum time gap between the end of the slot of last DCI format 3_0 monitoring occasion and the start of the DRX ON is a UE capability based on subcarrier spacing.

- The reporting is per SCS in units of slots of the respective SCS

  - The reported value for a SCS is taken from two possible values per SCS

  - The largest value of minimum time gap in UE capability is no more than the number of slots equal to [3]ms

- FFS impact of dormancy/non-dormancy transition

Agreements:

UE follows legacy DRX operation when DCI format 3_0 monitoring occasion(s) is invalid or there are no DCI format 3_0 monitoring occasions outside Active Time.

- FFS: Additional cases in addition to Rel-15 invalid PDCCH monitoring,

- FFS: whether it is all monitoring occasions or some monitoring occasions

Agreements:

The WUS indication in the DCI format 3_0 is only associated with the next occurrence of drx_OnDuration

Agreements:

When drx_OnDurationTimer does not start, RAN1 agrees the following report(s) are impacted by the WUS indication

- SP L1-RSRP reporting

- SP-CSI

- SRS

Except:

- by configuration, whether or not for periodic L1-RSRP reporting

- by configuration, whether or not for periodic CSI

- By default, both the above two are also impacted by the WUS indication

Note: for the above two bullets (under Except), no additional RAN1 impact is expected in Rel-16

Agreements:

Higher layer configuration of DCI format 3_0 with UE specific indication includes,

- Number of information bits of DCI format 3_0

- A starting position for 1-bit WUS, and immediately followed by X-bit additional power saving information (when configured)

- FFS whether the value X is implicitly derived from other configurations or explicitly configured

Agreements:

- DCI format 3_0 is not monitored in USS

Agreements:

Confirm the working assumption,

The range of monitoring is derived based on existing parameters, e.g., duration and  monitoringSymbolsWithinSlot, in SearchSpace IE without any change

Agreements:

The parameters of monitoringSlotPeriodicityAndOffset, duration and monitoringSymbolsWithinSlot in the SearchSpace IE are used for the search space in the indication of DCI format 3_0 monitoring.

Agreements:

For each SearchSpace set, UE monitors DCI format 3_0 only in the 1st full “duration” at or after the PS_offset, but before the DRX on-duration

Agreements:

UE is not required to monitor DCI format 3_0 during the minimum time gap before DRX ON.

This contribution discusses remaining issues for PDCCH-based power saving signal/channel.

# Remaining issues for PDCCH-based power saving signal/channel

# 2.1 Minimum time gap for DCI format 2_6

Currently the minimum time gap between the last DCI format 2_6 monitoring occasions and the start of the DRX ON is based on a UE capability. UE does not monitor DCI format 2_6 during the minimum time gap before DRX ON. It is FFS whether the time gap is affected by the dormancy/non-dormancy transition. Regarding dormancy/non-dormancy transition delay, following conclusion was made in RAN1#99 as below:

Conclusion:

- From RAN1 perspective,

  - Application delay for transitions between dormant BWP and non-dormant BWP will be specified by RAN4.

  - Until further RAN4 input is received, current DCI based BWP switching time that is supported by the UE is assumed as the application delay.

If a UE is configured with the bitmap for dormancy indication for DCI format 2_6, it requires additional time gap for the potential dormant/non-dormant BWP switching. Therefore, it would be desirable to consider larger minimum time gap when dormancy indication is supported for DCI format 2_6.

Proposed TP for TS 38.213 Section 10.3

============================= Text omitted ============================= If a UE reports for an active DL BWP a requirement for a number of slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer, - if a UE is configured with the bitmap, the UE is not required to monitor PDCCH for detection of DCI format 2_6 during the number of slots + Tbwp slots where Tbwp is a delay required by the UE for an active DL BWP change [10, TS 38.133]. - otherwise, the UE is not required to monitor PDCCH for detection of DCI format 2_6 during the number of slots.

# 2.3 Wake-up indication in DCI format 2_6

According to the following agreement, it is not confirmed yet how UE is provided with the starting position for wake-up indication in DCI format 2_6.

Agreements:

The new DCI of PDCCH-based power saving signal/channel outside Active Time is designed with UE-specific configured power saving information for one or more UEs.

- The new DCI format supports multiplexing of one or more UEs

- FFS: Whether the starting position and the location of the indication in the new DCI format for a specific UE is based on higher layer configuration

- FFS: Details of the DCI format design for a specific UE

  - Whether new DCI format is similar to that of DCI format 2_x

For the benefit of simplicity, the starting position can be provided to UE explicit through RRC configuration.

Proposed TP for TS 38.213 Section 10.3

============================= Text omitted ============================= A UE configured with DRX mode operation [11, TS 38.321] on the PCell or on the SpCell can be provided [12, TS 38.331] - a PS-RNTI for DCI format 2_6 by ps-RNTI - a number of search space sets, by dci-Format2-6, to monitor PDCCH for detection of DCI format 2_6 on the active DL BWP of the PCell or of the SpCell according to a common search space as described in Subclause 10.1 - a payload size for DCI format 2_6 by SizeDCI_2-6 - a location in DCI format 2_6 of a Wake-up indication bit provided by PSPositionDCI2-6, for the UE [12, TS 38.331], where

For the bit for indicating whether or not a UE wake up for DRX ON duration, uses a consistent terminology across the specifications as “Wake-up indication”.

Proposed TP for TS 38.213 Section 10.3

============================= Text omitted ============================= - a location in DCI format 2_6 of a Wake-up indication bit by PSPositionDCI2-6, where - the UE may not start the drx-onDurationTimer for the next long DRX cycle when a value of the ‘PDCCH monitoring’ bit is ‘0’, and - the UE starts the drx-onDurationTimer for the next long DRX cycle when a value of the ‘PDCCH monitoring’ bit is ‘1’ - a bitmap, when the UE is provided a number of groups of configured SCells by Scell-groups-for-dormancy-outside-active-time, where - the bitmap location is immediately after the ‘PDCCH monitoring’ Wake-up indication bit location

# Conclusion

This contribution discussed remaining issues for PDCCH-based power saving signal/channel and proposed 3 TPs as described in Section 2.

# References

- RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #99

- 3GPP TS 38.213 v16.0.0 (2019-12), “Physical layer procedures for control (Release 16)”