3GPP TSG RAN WG1 #100		R1-2000712

e-Meeting, February 24th – March 6th, 2020

Agenda item:	6.2.4.2

Source: 	Qualcomm Incorporated

Title: 	Corrections for CAS reception

Document for:	Discussion and Decision

# Issue #1: Semistatic CFI

The current specification text states that, when the UE is configured with semistatic CFI, it “shall” assume that the CFI value is indicated by MIB. We propose it to change to “may”, since the support of semistatic CFI is optional.

Proposal 1: Endorse TP1

<TP1 36.213, 9.1.3>

For a MBMS-dedicated cell, if a UE is configured with higher layer parameter semistaticCFI included in MasterInformationBlock-MBMS, the UE may assume the CFI is equal to the value of the higher layer parameter semistaticCFI for non-MBSFN subframes.

</TP1 36.213>

# Issue #2: AL16 in non-MBSFN subframes

In TS 36.211, the support of AL16 is captured as follows: “where PDCCH format 4 is supported only for the CAS subframe in conjunction with MBMS”. In our view, AL16 should not only apply to the CAS, but to all non-MBSFN subframes in an MBMS-dedicated carrier. Thus, we propose to modify the text as follows

<TP2 36.211, 6.8.1>

The physical downlink control channel carries scheduling assignments and other control information. A physical control channel is transmitted on an aggregation of one or several consecutive control channel elements (CCEs), where a control channel element corresponds to 9 resource element groups. The number of resource-element groups not assigned to PCFICH or PHICH is . The CCEs available in the system are numbered from 0 to, where . The PDCCH supports multiple formats as listed in Table 6.8.1-1 where PDCCH format 4 is supported only for non-MBSFN subframes in an MBMS-dedicated cell. A PDCCH consisting of  consecutive CCEs may only start on a CCE fulfilling, where  is the CCE number.

Multiple PDCCHs can be transmitted in a subframe.

Table 6.8.1-1: Supported PDCCH formats

| PDCCH format | Number of CCEs | Number of resource-element groups | Number of PDCCH bits |
| --- | --- | --- | --- |
| 0 | 1 | 9 | 72 |
| 1 | 2 | 18 | 144 |
| 2 | 4 | 36 | 288 |
| 3 | 8 | 72 | 576 |
| 4 | 16 | 144 | 1152 |

</TP2 36.211>

Proposal 2: Endorse TP2