3GPP TSG RAN WG1 #100-E                                                         	   R1-2000724

e-Meeting, February 24th – March 6th, 2020

Agenda Item:	7.2.2.2.1

Source: 	Intel Corporation

Title:	Channel Access Mechanism for NR-unlicensed

Document for: 	Discussion/Decision

# Introduction

In 3GPP TSG RAN Meeting #82, a new WID on NR-based access to Unlicensed Spectrum (RP-182878) [1] was approved. As part of the objectives of the work item, the following physical layer aspects related to the design of initial access signals and channels for unlicensed operation were suggested to be specified as per the outcome of the study item (TR38.889) [2]:

Physical layer procedure(s) including [RAN1, RAN2]: - For LBE, channel access mechanism in line with agreements from the NR-U study item (TR 38.889, Section 7.2.1.3.1). Specification work to be performed by RAN1. - For FBE, channel access mechanism in line with agreements from the NR-U study item (TR 38.889, Section 7.2.1.3.1). FBE is intended for environments where the absence of Wi-Fi is guaranteed (e.g., by level of regulations, private premises policies, etc. Further, the targeted scenario is limited to a single NR-U network (i.e. single operator) in the operating band and geographic area. Specification work to be performed by RAN1.

In this contribution, in the context of Channel Access Procedures for NR-unlicensed, and the agreements made along this WI [3-9], the following aspects are addressed:

- Clarification of FBE operation with single 9 microsecond measurement CAT2 25us LBT

- Correction of ED Threshold formula for 10 MHz transmit bandwidth case in the case that the higher layer parameter absenceofAnyOtherTechnology-r14 and absenceofAnyOtherTechnology-r16 are not provided

- Clarification on gNB and eNB behavior on CWS adjustment for multi-channel access

- Clarification on CP extension for long CP case

- Correction on supported durations for CP extension values C2 and C3 values at the UE

- Clarification on CWS adjustment in the Configured Grant case with CBG transmissions

- Indication of uplink LBT failure to higher layer

# Clarification for FBE Operation

In previous RAN1 #99 meeting [9], the following agreement was made:

Agreement: At least for LBE operation: - For signaling of LBT type & CP extension for both Fallback DL assignment and Fallback UL Grant, the following table is used: LBT Type; CP extension. Cat1 16 µs; C2*symbol length – 16 us – TA. Cat2 25 µs; C3*symbol length – 25 us – TA. Cat2 25 µs; C1*symbol length – 25 us. Cat4; 0. - CAPC is not indicated explicitly: - For the UL grants - The UE assumes CAPC=4 was used by the gNB to acquire the CO, - For UE initiated COTs (Cat4 case) the UE may select the CAPC by itself. - Note: The mapping between priority classes and traffic classes follows the same mechanism as defined for UL CG transmissions. - Note: For PUCCH associated with DL assignments may use the highest priority CAPC when CAT4 LBT is used, as agreed earlier. - If the network indicates FBE operation, for an indication of LBT type of Cat 2 25 us or Cat4 the UE follows the mechanism whereby one 9 microsecond slot is measured within a 25 microsecond interval as in 37.213.

However, in TS 38.212 , TS 38.213, and TS 37.213, the highlighted part of this agreement related to the FBE operation has not been captured.

In particular, in TS 37.213 the description of the CAT-2 LBT always links to two observations periods, while in the agreement above a single 9 us observation window is defined for FBE only operation. For this purpose, the following text should be implemented in TS 37.213 :

*** Unchanged text is omitted *** ##### 4.1.2.4 Type 2D DL channel access procedure An eNB/gNB may transmit a DL transmission immediately after sensing the channel to be idle for at least a sensing interval Tshort_dl=25us. Tshort_dl consists of a duration Tf=16us immediately followed by one sensing slot. The channel is considered to be idle for Tshort_dl if the sensing slot sensed to be idle. *** Unchanged text is omitted *** ##### 4.2.1.2.4 Type 2D UL channel access procedure If a UE is indicated to perform Type 2D UL channel access procedures, the UE uses Type 2D UL channel access procedures for a UL transmission. The UE may transmit the transmission immediately after sensing the channel to be idle for at least a sensing interval Tshort_ul=25us. Tshort_ul consists of a duration Tf=16us immediately followed by one slot sensing slot. The channel is considered to be idle for Tshort_ul if the sensing slot of Tshort_ul.is sensed to be idle. *** Unchanged text is omitted ***

In TS 38.212, for both fallback and non-fallback DCIs the signalling related to the CP extension for FBE operation has not been properly implemented. For example, for DCI 0_0, a table describing the LBT type and CP extension to use when FBE is configured was not captured. Furthermore, no signalling is provided in DCI 0_1 related to the FBE operation, which is not correct since at least CAT-1 LBT and 25usec CAT-2 LBT have to be supported. For this purpose, the following text should be implemented in TS 38.212:

*** Unchanged text is omitted *** ##### 7.3.1.1.1 Format 0_0 *** Unchanged text is omitted *** The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Frequency domain resource assignment – bits where is defined in subclause 7.3.1.0- For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Subclause 6.3 of [6, TS 38.214], where if the higher layer parameter frequencyHoppingOffsetLists contains two offset values and if the higher layer parameter frequencyHoppingOffsetLists contains four offset values - bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214] - Time domain resource assignment – 4 bits as defined in Subclause 6.1.2.1 of [6, TS 38.214] - Frequency hopping flag – 1 bit according to Table 7.3.1.1.1-3, as defined in Subclause 6.3 of [6, TS 38.214] - Modulation and coding scheme – 5 bits as defined in Subclause 6.1.4.1 of [6, TS 38.214] - New data indicator – 1 bit - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 - HARQ process number – 4 bits - TPC command for scheduled PUSCH – 2 bits as defined in Subclause 7.1.1 of [5, TS 38.213] - Padding bits, if required. - UL/SUL indicator – 1 bit for UEs configured with supplementaryUplink in ServingCellConfig in the cell as defined in Table 7.3.1.1.1-1 and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s). - If the UL/SUL indicator is present in DCI format 0_0 and the higher layer parameter pusch-Config is not configured on both UL and SUL the UE ignores the UL/SUL indicator field in DCI format 0_0, and the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured; - If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config is configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the UL or SUL for which high layer parameter pucch-Config is configured. - If the UL/SUL indicator is not present in DCI format 0_0 and pucch-Config is not configured, the corresponding PUSCH scheduled by the DCI format 0_0 is for the uplink on which the latest PRACH is transmitted. - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4a when ChannelAccessMode-r16 = "dynamic" and Table 7.3.1.1.1-4b when ChannelAccessMode-r16 = "semistatic" for operation in a cell with shared spectrum channel access; 0 bit otherwise. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: Identifier for DCI formats – 1 bit - The value of this bit field is always set to 0, indicating an UL DCI format - Frequency domain resource assignment – number of bits determined by the following: - bits if the higher layer parameter useInterlacePUSCH-Common-r16 is not configured, where - is the size of the initial UL bandwidth part. - For PUSCH hopping with resource allocation type 1: - MSB bits are used to indicate the frequency offset according to Table 8.3-1 in Subclause 8.3 of [5, TS 38.213], where if and otherwise - bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214] - For non-PUSCH hopping with resource allocation type 1: - bits provides the frequency domain resource allocation according to Subclause 6.1.2.2.2 of [6, TS 38.214] - if the higher layer parameter useInterlacePUSCH-Common-r16 is configured - 5 bits provide the frequency domain resource allocation according to Subclause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz - 6 bits provide the frequency domain resource allocation according to Subclause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz - Time domain resource assignment – 4 bits as defined in Subclause 6.1.2.1 of [6, TS 38.214] - Frequency hopping flag – 1 bit according to Table 7.3.1.1.1-3, as defined in Subclause 6.3 of [6, TS 38.214] - Modulation and coding scheme – 5 bits as defined in Subclause 6.1.4.1 of [6, TS 38.214] - New data indicator – 1 bit, reserved - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 - HARQ process number – 4 bits, reserved - TPC command for scheduled PUSCH – 2 bits as defined in Subclause 7.1.1 of [5, TS 38.213] - Padding bits, if required. - UL/SUL indicator – 1 bit if the cell has two ULs and the number of bits for DCI format 1_0 before padding is larger than the number of bits for DCI format 0_0 before padding; 0 bit otherwise. The UL/SUL indicator, if present, locates in the last bit position of DCI format 0_0, after the padding bit(s). - If 1 bit, reserved, and the corresponding PUSCH is always on the same UL carrier as the previous transmission of the same TB - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 a when ChannelAccessMode-r16 = "dynamic" and Table 7.3.1.1.1-4b when ChannelAccessMode-r16 = "semistatic" for operation in a cell with shared spectrum channel access; 0 bit otherwise. *** Unchanged text is omitted *** Table 7.3.1.1.1-4a: Channel access type & CP extension for DCI format 0_0 and DCI format 1_0 when ChannelAccessMode-r16 = "dynamic" Bit field mapped to index; Channel Access Type; CP extension. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; C2*symbol length – 16 us – TA. 1; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; C3*symbol length – 25 us – TA. 2; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; C1*symbol length – 25 us. 3; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0. Table 7.3.1.1.1-4b: Channel access type & CP extension for DCI format 0_0 and DCI format 1_0 when ChannelAccessMode-r16 = "semistatic" Bit field mapped to index; Channel Access Type; CP extension. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; C2*symbol length – 16 us – TA. 1; Type2D-ULChannelAccess defined in [clause 4.2.1.2.4 in 37.213]; C3*symbol length – 25 us – TA. 2; Type2D-ULChannelAccess defined in [clause 4.2.1.2.4 in 37.213]; C1*symbol length – 25 us. 3; Type2D-ULChannelAccess defined in [clause 4.2.1.4 in 37.213]; 0. *** Unchanged text is omitted *** ##### 7.3.1.1.2 Format 0_1 *** Unchanged text is omitted *** - DMRS sequence initialization – 0 bit if transform precoder is enabled; 1 bit if transform precoder is disabled. - UL-SCH indicator – 0 or 1 bit as follows - 0 bit if the number of scheduled PUSCH indicated by the Time domain resource assignment field is larger than 1; - 1 bit otherwise. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. Except for DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, a UE is not expected to receive a DCI format 0_1 with UL-SCH indicator of "0" and CSI request of all zero(s). - ChannelAccess-CPext-CAPC – 0, 1, 2, 3, 4, 5 or 6 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter ULDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16 for operation in a cell with shared spectrum. One or more entries from Table 7.3.1.1.2-35 are configured by the higher layer parameter ULDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16. *** Unchanged text is omitted *** Table 7.3.1.1.2-35: Allowed entries for DCI format 0_1, configured by high layer parameter ULDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16 Entry index; Channel Access Type; CP extension; CAPC. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0; 1. 1; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0; 2. 2; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0; 3. 3; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0; 4. 4; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; C2*symbol length – 16 us – TA; 1. 5; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; C2*symbol length – 16 us – TA; 2. 6; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; C2*symbol length – 16 us – TA; 3. 7; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; C2*symbol length – 16 us – TA; 4. 8; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0; 1. 9; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0; 2. 10; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0; 3. 11; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0; 4. 12; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; C2*symbol length – 16 us – TA; 1. 13; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; C2*symbol length – 16 us – TA; 2. 14; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; C2*symbol length – 16 us – TA; 3. 15; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; C2*symbol length – 16 us – TA; 4. 16; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 0; 1. 17; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 0; 2. 18; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 0; 3. 19; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 0; 4. 20; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; C1*symbol length – 25 us; 1. 21; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; C1*symbol length – 25 us; 2. 22; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; C1*symbol length – 25 us; 3. 23; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; C1*symbol length – 25 us; 4. 24; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; C3*symbol length – 25 us – TA; 1. 25; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; C3*symbol length – 25 us – TA; 2. 26; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; C3*symbol length – 25 us – TA; 3. 27; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; C3*symbol length – 25 us – TA; 4. 28; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0; 1. 29; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0; 2. 30; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0; 3. 31; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0; 4. 32; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; C1*symbol length – 25 us; 1. 33; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; C1*symbol length – 25 us; 2. 34; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; C1*symbol length – 25 us; 3. 35; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; C1*symbol length – 25 us; 4. 36; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; C2*symbol length – 16 us – TA; 1. 37; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; C2*symbol length – 16 us – TA; 2. 38; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; C2*symbol length – 16 us – TA; 3. 39; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; C2*symbol length – 16 us – TA; 4. 40; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; C3*symbol length – 25 us – TA; 1. 41; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; C3*symbol length – 25 us – TA; 2. 42; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; C3*symbol length – 25 us – TA; 3. 43; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; C3*symbol length – 25 us – TA; 4. 44; Type2D-ULChannelAccess defined in [clause 4.2.1.2.4 in 37.213]; 0; 1. 45; Type2D-ULChannelAccess defined in [clause 4.2.1.2.4 in 37.213]; 0; 2. 46; Type2D-ULChannelAccess defined in [clause 4.2.1.2.4 in 37.213]; 0; 3. 47; Type2D-ULChannelAccess defined in [clause 4.2.1.2.4 in 37.213]; 0; 4. 48; Type2D-ULChannelAccess defined in [clause 4.2.1.2.4 in 37.213]; C1*symbol length – 25 us; 1. 49; Type2D-ULChannelAccess defined in [clause 4.2.1.2.4 in 37.213]; C1*symbol length – 25 us; 2. 50; Type2D-ULChannelAccess defined in [clause 4.2.1.2.4 in 37.213]; C1*symbol length – 25 us; 3. 51; Type2D-ULChannelAccess defined in [clause 4.2.1.2.4 in 37.213]; C1*symbol length – 25 us; 4. 52; Type2D-ULChannelAccess defined in [clause 4.2.1.2.4 in 37.213]; C3*symbol length – 25 us – TA; 1. 53; Type2D-ULChannelAccess defined in [clause 4.2.1.2.4 in 37.213]; C3*symbol length – 25 us – TA; 2. 54; Type2D-ULChannelAccess defined in [clause 4.2.1.2.4 in 37.213]; C3*symbol length – 25 us – TA; 3. 55; Type2D-ULChannelAccess defined in [clause 4.2.1.2.4 in 37.213]; C3*symbol length – 25 us – TA; 4. *** Unchanged text is omitted *** ##### 7.3.1.2.1 Format 1_0 DCI format 1_0 is used for the scheduling of PDSCH in one DL cell. The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bits - The value of this bit field is always set to 1, indicating a DL DCI format - Frequency domain resource assignment – bits where is given by subclause 7.3.1.0 If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows: - Random Access Preamble index – 6 bits according to ra-PreambleIndex in Subclause 5.1.2 of [8, TS38.321] - UL/SUL indicator – 1 bit. If the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; otherwise, this field is reserved - SS/PBCH index – 6 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved. - PRACH Mask index – 4 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission, according to Subclause 5.1.1 of [8, TS38.321]; otherwise, this field is reserved - Reserved bits – 12 bits for operation in a cell with shared spectrum channel access; otherwise 10 bits Otherwise, all remaining fields are set as follows: - Time domain resource assignment – 4 bits as defined in Subclause 5.1.2.1 of [6, TS 38.214] - VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5 - Modulation and coding scheme – 5 bits as defined in Subclause 5.1.3 of [6, TS 38.214] - New data indicator – 1 bit - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 - HARQ process number – 4 bits - Downlink assignment index – 2 bits as defined in Subclause 9.1.3 of [5, TS 38.213], as counter DAI - TPC command for scheduled PUCCH – 2 bits as defined in Subclause 7.2.1 of [5, TS 38.213] - PUCCH resource indicator – 3 bits as defined in Subclause 9.2.3 of [5, TS 38.213] - PDSCH-to-HARQ_feedback timing indicator – 3 bits as defined in Subclause 9.2.3 of [5, TS38.213] - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 a when ChannelAccessMode-r16 = "dynamic" and Table 7.3.1.1.1-4b when ChannelAccessMode-r16 = "semistatic" for operation in a cell with shared spectrum channel access; 0 bit otherwise. *** Unchanged text is omitted *** The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by TC-RNTI: - Identifier for DCI formats – 1 bit - The value of this bit field is always set to 1, indicating a DL DCI format - Frequency domain resource assignment – bits - is the size of CORESET 0 - Time domain resource assignment – 4 bits as defined in Subclause 5.1.2.1 of [6, TS38.214] - VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5 - Modulation and coding scheme – 5 bits as defined in Subclause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1 - New data indicator – 1 bit - Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2 - HARQ process number – 4 bits - Downlink assignment index – 2 bits, reserved - TPC command for scheduled PUCCH – 2 bits as defined in Subclause 7.2.1 of [5, TS38.213] - PUCCH resource indicator – 3 bits as defined in Subclause 9.2.3 of [5, TS38.213] - PDSCH-to-HARQ_feedback timing indicator – 3 bits as defined in Subclause 9.2.3 of [5, TS38.213] - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4 a when ChannelAccessMode-r16 = "dynamic" and Table 7.3.1.1.1-4b when ChannelAccessMode-r16 = "semistatic" for operation in a cell with shared spectrum channel access; 0 bit otherwise. *** Unchanged text is omitted *** ##### 7.3.1.2.2 Format 1_1 *** Unchanged text is omitted *** - DMRS sequence initialization – 1 bit. - Priority indicator – 0 bit if higher layer parameter PriorityIndicator-ForDCIFormat1_1 is not configured; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213]. - ChannelAccess-CPext – 0, 1, 2, 3 or 4 bits. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter DLDCI-trigerred-UL-ChannelAccess-CPext-List-r16 for operation in a cell with shared spectrum. One or more entries from Table 7.3.1.2.2-6 are configured by the higher layer parameter DLDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16. *** Unchanged text is omitted *** Table 7.3.1.2.2-6: Allowed entries for DCI format 1_1, configured by high layer parameter DLDCI-trigerred-UL-ChannelAccess-CPext-CAPC-List-r16 Entry index; Channel Access Type; CP extension. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0. 1; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; C2*symbol length – 16 us – TA. 2; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 0. 3; Type2B-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; C2*symbol length – 16 us – TA. 4; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 0. 5; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; C1*symbol length – 25 us. 6; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; C3*symbol length – 25 us – TA. 7; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0. 8; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; C1*symbol length – 25 us. 9; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; C2*symbol length – 16 us – TA. 10; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; C3*symbol length – 25 us – TA. 11; Type2D-ULChannelAccess defined in [clause 4.2.1.2.4 in 37.213]; 0. 12; Type2D-ULChannelAccess defined in [clause 4.2.1.2.4 in 37.213]; C1*symbol length – 25 us. 13; Type2D-ULChannelAccess defined in [clause 4.2.1.2.4 in 37.213]; C3*symbol length – 25 us – TA. *** Unchanged text is omitted ***

In TS 38.213, the text refers to the RRC parameter “ChannelAccessType-r16”, which is a typo of “ChannelAccessMode-r16”. For this purpose, the following text should be implemented in TS 38.213:

*** Unchanged text is omitted *** For unpaired spectrum, - if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if it does not precede a SS/PBCH block in the PRACH slot and starts at least symbols after a last SS/PBCH block reception symbol, where is provided in Table 8.1-2. - the index of the SS/PBCH block is provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon - If a UE is provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if - it is within UL symbols, or - it does not precede a SS/PBCH block in the PRACH slot and starts at least symbols after a last downlink symbol and at least symbols after a last SS/PBCH block symbol, where is provided in Table 8.1-2, and if ChannelAccessMode-r16 = “semistatic” is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213] *** Unchanged text is omitted ***

# LBT Procedure For 10 MHz Channel Bandwidth

During RAN #84 [10], it was agreed that the following physical layer aspect should be captured in the NR-U specifications:

- 10MHz operation for 5GHz band via NR-U/NR-U CA or NR/NR-U CA without air-interface optimizations specific to 10MHz.

NOTE:

- 10MHz Pcell or SpCell is not supported in NR-U.

- The absence of WiFi channels should be guaranteed.

The purpose of this agreement is to enable NR-U to be deployed and operated in Band n46, where 10 MHz operation is allowed. For this band, whose regulatory requirements were made available by the Indian ministry of communications on October 2018, the maximum PSD is constrained not to exceed 30 dBm [11]. In order to capture the fact that the LBT procedure for this deployment must be performed over a 10 MHz band, and that the maximum PSD for this band is constrained to 30 dBm, the value of the energy detection (ED) threshold should account for both the increase in PSD limitation (i.e., from 23 dBm to 30 dBm) and the bandwidth (BW) reduction (i.e., from 20 MHz to 10 MHz).

In Rel. 13 and 14, the following equation was captured in TS 37.213 in Section 4.1.5 (for downlink) and Section 4.2.3 (for uplink) in order to apply a constant adjustment across all power levels (which is proportional to the channel BW) to the ED threshold when the channel BW is set to different values (e.g., 10, 15 or 20 MHz) in the specific case when the absence of any incumbent technology cannot be guaranteed:

-	XThresh_max=max&-72+10⋅log10(BWMHz /20MHz) dBm,&min&Tmax,&Tmax-TA+PH+10⋅log10(BWMHz /20MHz)-PTX (1)

where

= 10dB for transmission(s) including PDSCH or PUSCH;

-	= 5dB for transmissions including discovery signal transmission(s) and not including PDSCH;

-	 = 23 dBm;

-	;

When (1) is applied to a 10 MHz transmission, there should be a 3dB shift lower to the ED threshold from its 20 MHz counterpart. In Fig. 1, a 3 dB shift exists in the low output and high output power regions, as can be seen by the red curve.  This shift can be as large as 6 dB in the middle output power region (e.g. 15 dBm).  ETSI BRAN guidelines [12] define the ED threshold using -85dBm/MHz PSD and have transition regions ranging from 13 to 23 dBm.   For 10 MHz transmissions, the current formula in 37.213 has a transition region from 10 to 20 dBm, which is not consistent with [12].   The proposed change has a transition region from 13 to 23 dBm.

Fig. 1 – ED threshold as function of the output power for a PUSCH transmission.

Furthermore, in RAN1#83 the following agreement was made, where the formula in question has been highlighted in yellow:

Agreements: For the category 4 LBT procedure used by LAA: - If the absence of any other technology sharing the carrier can be guaranteed on a long term basis (e.g. by level of regulation), the maximum energy detection threshold used by LAA is - Min(X, Y) - X = -75 dBm/MHz+ 10*log10(BWMHz) + 10 dB - Y = - Maximum energy detection threshold defined by regulatory requirements when such requirements are defined - Y = X, otherwise - Otherwise, LAA uses the maximum energy detection threshold of TH; - TH = max(-72 dBm (20MHz), min(Tmax, Tmax – 10 dB + (PH – PTX))) - If an LAA eNB uses channel access signals of other technologies for the purpose of LAA channel access, it shall continue to meet the LAA maximum energy detection threshold requirement. (Captured in 36.300) - The parameters used in the above proposal are defined as the following: - PH is a reference power equaling 23 dBm, - PTX is the configured maximum transmit power for the carrier in dBm - Uses the configured maximum transmission power over a single carrier irrespective of whether single carrier or multi-carrier transmission is employed - Tmax is defined as the following: - Tmax = -75 dBm/MHz+ 10*log10(BWMHz). - BWMHz is the channel bandwidth in MHz

When the absence of any other technology sharing the carrier cannot be guaranteed, the RAN1 #83 agreement states that for PTX =20 dBm, the threshold should be -72 dBm.  When applied to the case when PTX =20 dBm, the current formula in 37.213 leads to an ED threshold of -75 dBm, which is not consistent with the agreement above. Therefore, it is proposed to modify (1) by removing the term 10⋅log10(BWMHz /20MHz) as shown below.

(2)

In order to address the aforementioned concern, the following text should be implemented in TS 37.213:

*** Unchanged text is omitted *** ### 4.1.5 Energy detection threshold adaptation procedures An eNB/gNB accessing a channel on which transmission(s) are performed, shall set the energy detection threshold (XThresh) to be less than or equal to the maximum energy detection threshold XThresh_max. XThresh_max is determined as follows: - If the absence of any other technology sharing the channel can be guaranteed on a long-term basis (e.g. by level of regulation) then: - XThresh_max=min&Tmax+10dB,&Xr - Xr is Maximum energy detection threshold defined by regulatory requirements in dBm when such requirements are defined, otherwise Xr=Tmax+10dB - Otherwise, - XThresh_max=max&-72+10⋅log10(BWMHz /20MHz) dBm,&min&Tmax,&Tmax-TA+PH+10⋅log10(BWMHz /20MHz)-PTX - Where: - TA=10dB for transmission(s) including PDSCH; - TA=5dB for transmissions including discovery burst(s) as described in subclause 4.1.2; - PH=23dBm dBm; - PTX is the set maximum eNB/gNB output power in dBm for the channel; - eNB/gNB uses the set maximum transmission power over a single channel irrespective of whether single channel or multi-channel transmission is employed - Tmax(dBm)=10⋅log103.16228⋅10-8(mW/MHz) ⋅ BWMHz (MHz); - BWMHz is the single channel bandwidth in MHz. *** Unchanged text is omitted *** 4.2.3.1 Default maximum energy detection threshold computation procedure If the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided - X'Thresh_max=min&Tmax+10dB&Xr where - Xr is Maximum energy detection threshold defined by regulatory requirements in dBm when such requirements are defined, otherwise Xr=Tmax+10dB otherwise - X'Thresh_max=max&-72+10⋅log10(BWMHz /20MHz) dBm,&min&Tmax,&Tmax-TA+PH+10⋅log10(BWMHz /20MHz)-PTX where - TA=10dB; - PH=23dBm; - PTX is the set to the value of PCMAX_H,c as defined in [3]; - Tmax(dBm)=10⋅log13.16228⋅10-8(mW/MHz) ⋅ BWMHz (MHz) ; - BWMHz is the single channel bandwidth in MHz. *** Unchanged text is omitted ***

# CWS Adjustment For Multi-Carrier

In Sec. 4.1.6.2.1 and 4.1.6.2.2 of TS 37.213, the CWS adjustment for wideband operation for NR-U is described in parallel with that of LAA. However, it is unclear which text applies for LAA and which text applies to NR-U even though the procedures are different from each other. Therefore, the text in TS 37.213 should be revised as follows:

*** Unchanged text is omitted *** ##### 4.1.6.2.1 Type B1 multi-channel access procedure A single CWp value is maintained for the set of channels C. For determining CWp at the eNB for channel access on channel cj, step 2 of the procedure described in subclause 4.1.4.1 is modified as follows - if at least Z=80% of HARQ-ACK values corresponding to PDSCH transmission(s) in reference subframe k of all channels ci∈C are determined as NACK, increase CWp for each priority class p∈1,2,3,4 to the next higher allowed value; otherwise, go to step 1. For determining CWp at the gNB for a set of channels C, any PDSCH that fully or partially overlaps with any channel ci∈C, is used in the procedure described in subclause 4.1.4.2. ##### 4.1.6.2.2 Type B2 multi-channel access procedure For the eNB, a CWp value is maintained independently for each channel ci∈C using the procedure described in subclause 4.1.4.1. For the gNB, a CWp value is maintained independently for each channel ci∈C using the procedure described in subclause 4.1.4.2. For determining CWp for channel ci at the gNB, any PDSCH that fully or partially overlaps with channel ci, is used in the procedure described in subclause 4.1.4.2. For determining Ninit for channel cj, CWp value of channel cj1∈C is used, where cj1 is the channel with largest CWp among all channels in set C. *** Unchanged text is omitted ***

# CP Extension Modification

In Sec. 5.3.1. of TS 38.211, additional text has been added to capture the agreements made during this WI [3-9] related to the CP extension. However, the way how the CP extension is computed, which is summarized in Table 5.3.1-1, currently does not account for the fact that the first OFDM symbol in each half of a subframe is always slightly longer than the other OFDM symbols. Therefore, the following proposed text for TS 38.211 should be implemented:

*** Unchanged text is omitted *** ### 5.3.1 OFDM baseband signal generation for all channels except PRACH and RIM-RS *** Unchanged text is omitted *** Table 5.3.1-1: The cyclic extension Text. Index; μ=0; μ=1; μ=2. 0; -; -; -. 1; Tsymb,lμ-25∙10-6; Tsymb,lμ-25∙10-6; k=12Tsymb,mod(l-k,7∙2μ)μ2Tsymb,lμ-25∙10-6. 2; k=1C2Tsymb,mod(l-k,7∙2μ)μC2Tsymb,lμ-16∙10-6 -TTA; k=1C2Tsymb,mod(l-k,7∙2μ)μC2Tsymb,lμ-16∙10-6-TTA; k=1C2Tsymb,mod(l-k,7∙2μ)μC2Tsymb,lμ-16∙10-6-TTA. 3; k=1C3Tsymb,mod(l-k,7∙2μ)μC3Tsymb,lμ-25∙10-6-TTA; k=1C3Tsymb,mod(l-k,7∙2μ)μC3Tsymb,lμ-25∙10-6-TTA; k=1C3Tsymb,mod(l-k,7∙2μ)μC3Tsymb,lμ-25∙10-6-TTA. *** Unchanged text is omitted ***

In previous RAN1 meetings [3-9], the following agreements were made regarding the CP extension:

Agreement: - For the CP extension prior to at least a dynamically scheduled PUSCH transmission, the CP extension is located in the symbol(s) immediately preceding the PUSCH allocation indicated by SLIV. The supported durations for CP extension at the UE are: - 0 (i.e. no CP extension) - C1*symbol length – 25 us - C2*symbol length – 16 us - TA - C3*symbol length – 25 us – TA - C1=1 for 15 and 30 kHz SCS, C1=2 for 60 kHz SCS - FFS: Whether C2/C3 is fixed or implicitly derived based on TA for each subcarrier spacing - The N2 timeline (UL grant to PUSCH delay) needs to be relaxed to take the CP extension into account - FFS: Whether the limit as per the previous agreement bounding the resulting CP extension to be less than or equal to one symbol for the given subcarrier spacing should be relaxed - FFS: Applicability of this to other UL transmissions - FFS: Whether the number of durations for CP extension that the UE can be signalled dynamically can be configured Agreement: C2 &C3 (from prior agreement on CP extension) are UE specifically RRC configured - Note: this does not restrict the maximum TA that can be used in a cell - FFS: Agree the values of C2 and C3 in RAN1#99 Agreement: The supported range of values of both C2 and C3 (from prior agreement on CP extension) that can be configured by RRC is - 1, 2, …, 28 for 15 and 30 kHz SCS, and - 2, 3, …, 28 for 60 kHz SCS Note: Other agreements are not superseded by this agreement

The minimum value of C2 is 2 for 60KHz SCS in the current agreement. However, if TA is smaller than 1.83854us (e.g. timing advance command TA values less than 14), C2 could assume the value of 1 for 60 kHz SCS.

In addition, the maximum values for C2 and C3 were determined based on the maximum possible value of TA for each subcarrier spacing.  However, for large TA values (e.g. timing advance command TA value equal to 3846, which is equivalent to 2.003125ms/1.0015625ms/0.50078125ms in 15/30/60 kHz SCS), maximum values for C2 can be as large as 29/29/29 for 15/30/60 kHz SCS.  Similarly, maximum values for C3 can be as large as 29/29/30 for 15/30/60 kHz SCS.  In summary, the supported range for C2 should be updated to support values in the range of 1, 2, …, 29 for 15, 30 kHz, and 60 kHz SCS. C3 can assume value in the range of 1, 2, …, 29 for 15 and 30 kHz SCS, and 2, 3, …, 30 for 60 kHz SCS.

Proposal 1: The supported range of values of C2 that can be configured by RRC is

- 1, 2, …, 29 for 15, 30 kHz, and 60 kHz SCS

Proposal 2: The supported range of values of C3 that can be configured by RRC is

- 1, 2, …, 29 for 15 and 30 kHz SCS, and

- 2, 3, …, 30 for 60 kHz SCS

# Clarification for CWS Adjustment

In previous RAN1 #99 meeting [9], the following agreements were made regarding the CWS adjustment mechanism when both a TB or a CBG based transmission is performed:

Agreement: For TB based HARQ feedback within a single LBT subband, CW is reset if at least one “ACK” is received, or at least one NDI is toggled for the TB(s) transmitted in the reference duration - Note: HARQ feedback includes any implicit methods of HARQ feedback determination. Agreement: For CBG based HARQ feedback within a single LBT subband, and when all CBGs are confined within the LBT subband, CW is reset if “ACK” is received for at least 10 % of the CBGs in the reference duration - For purpose of CWS adjustment, a CBG TI set to 0 is assumed to be an ACK - Note: HARQ feedback includes any implicit methods of HARQ feedback determination.

When scheduled transmissions are configured per CBG and the feedback information is provided within a DFI, it is not clear how CWS adjustment should be done.   In this case, since the HARQ-ACK information in DFI is provided per TB, the CWS adjustment should be based on the feedback information and not the transmission configuration.  This change has the advantage that the CWS will be reset in the case that DFI contains at least one Ack in the HARQ-ACK feedback even though the transmission was CBG based, independent of all other feedback information.  To address this concern, the following text is proposed in TS 37.213.

*** Unchanged text is omitted *** #### 4.1.4.2 Contention window adjustment procedures for DL transmissions by gNB If a gNB transmits transmissions including PDSCH that are associated with channel access priority class p on a channel, the gNB maintains the contention window value CWp and adjusts CWp before step 1 of the procedure described in subclause 4.1.1 for those transmissions using the following steps: 1) For every priority class p∈1,2,3,4, set CWp=CWmin,p. 2) If HARQ-ACK feedback is available after the last update of CWp , go to step 3. Otherwise, if the gNB transmission after procedure described in subclause 4.1.1 does not include a retransmission or is transmitted within a duration Tw from the end of the reference duration corresponding to the earliest DL transmission burst after the last update of CWp transmitted after the procedures described in subclause 4.1.1, go to step 5; otherwise go to step 4. 3) The HARQ-ACK feedback(s) corresponding to PDSCH(s) in the reference duration for the latest DL transmission burst for which HARQ-ACK feedback is available is used as follows: a. If at least one HARQ-ACK feedback is 'ACK' for PDSCH(s) with transport block based feedback or at least 10% of HARQ-ACK feedbacks is 'ACK' for PDSCH(s) with code block group based feedback go to step 1; otherwise go to step 4. 4) Increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value. 5) For every priority class p∈1,2,3,4, maintain CWp as it is; go to step 2. *** Unchanged text is omitted *** #### 4.2.2.2 Contention window adjustment procedures for UL transmissions scheduled/configured by gNB If a UE transmits transmissions using Type 1 channel access procedures that are associated with channel access priority class p on a channel, the UE maintains the contention window value CWp and adjusts CWp for those transmissions before step 1 of the procedure described in subclause 4.2.1.1, using the following steps: 1) For every priority class p∈1,2,3,4, set CWp=CWmin,p; 2) If HARQ-ACK feedback is available after the last update of CWp, go to step 3. Otherwise, if the UE transmission after procedure described in subclause 4.2.1.1 does not include a retransmission or is transmitted within a duration Tw from the end of the reference duration corresponding to the earliest UL transmission burst after the last update of CWp transmitted after the procedures described in subclause 4.1.1, go to step 5; otherwise go to step 4. 3) The HARQ-ACK feedback(s) corresponding to PUSCH(s) in the reference duration for the latest UL transmission burst for which HARQ-ACK feedback is available is used as follows: a. If at least one HARQ-ACK feedback is 'ACK' for PUSCH(s) with transport block (TB) based feedback or at least 10% of HARQ-ACK feedbacks is 'ACK' for PUSCH(s) with code block group (CBG) based feedback go to step 1; otherwise go to step 4. 4) Increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value; 5) For every priority class p∈1,2,3,4, maintain CWp as it is; go to step 2. *** Unchanged text is omitted ***

# Indication of uplink LBT failure to higher layer

RAN1 received an LS from RAN2 requesting to introduce uplink LBT failure reporting to the higher layer as given in [13]. In order to address the request from RAN2, the following text proposal is proposed in TS 37.213:

*** Unchanged text is omitted *** ## 4.2 Uplink channel access procedures A UE performing transmission(s) on LAA Scell(s), an eNB scheduling or configuring UL transmission(s) for a UE performing transmission(s) on LAA Scell(s), and a UE performing transmission(s) on channel(s) and a gNB scheduling or configuring UL transmission(s) for a UE performing transmissions on channel(s) shall perform the procedures described in this subclause for the UE to access the channel(s) on which the transmission(s) are performed. In this subclause, transmissions from a UE are considered as separate UL transmissions, irrespective of having a gap between transmissions or not, and XThresh for sensing is adjusted as described in subclause 4.2.3 when applicable. A UE performs channel access procedures in this subclause unless the higher layer parameter ChannelAccessMode-r16 is provided and ChannelAccessMode-r16 =' semistatic'. If a UE fails to access the channel, this event shall be reported to higher layers. *** Unchanged text is omitted *** ## 4.3 Channel access procedures for semi-static channel occupancy *** Unchanged text is omitted *** - A UE may transmit UL transmission burst(s) after DL transmission burst(s) within the channel occupancy time as follows: - If the gap between the UL and DL transmission bursts is at most 16us, the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time without sensing the channel. - If the gap between the UL and DL transmission bursts is more than 16us, the UE may transmit UL transmission burst(s) after a DL transmission burst(s) within the channel occupancy time after sensing the channel to be idle for at least a sensing slot duration Tsl=9us before transmission. If a UE fails to access the channel, this event shall be reported to higher layers. *** Unchanged text is omitted ***

# Conclusion

In this contribution, we discussed several aspects related to the channel access mechanism, and made the following proposals:

Proposal 1: The supported range of values of C2 that can be configured by RRC is

- 1, 2, …, 29 for 15, 30 kHz, and 60 kHz SCS

Proposal 2: The supported range of values of C3 that can be configured by RRC is

- 1, 2, …, 29 for 15 and 30 kHz SCS, and

- 2, 3, …, 30 for 60 kHz SCS

# References

[1] RP-182878, “New WID on NR-based Access to Unlicensed Spectrum”, Qualcomm Inc, 3GPP TSG RAN Meeting #82, Dec. 2018.

[2] 3GPP TR 38.889, “Study on NR-based Access to Unlicensed Spectrum (Rel-16),” V 16.0.0, Nov. 2018.

[3] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #AH 1901, January, 2019.

[4] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #96, February, 2019.

[5] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #96b, April, 2019.

[6] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #97, May, 2019.

[7] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #98, August, 2019.

[8] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #98bis, October, 2019.

[9] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #99, November, 2019.

[10] RAN #84 Meeting Chairman’s Notes – RP-191575 “Revised NR-U WID”.

[11]http://wpc.dot.gov.in/WriteReadData/userfiles/License%20Exemption%20in%205%20GHz%20G_S_R_1048(E)%20dated%2022nd%20October,%202018.pdf

[12] ETSI EN 301 893 V2.1.1, “5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”, (2017-05).

[13] R1-2000008, “LS on consistent Uplink LBT failure detection mechanism”, RAN2, February, 2020