3GPP TSG RAN WG1 Meeting #100-E															R1-2000729

e-Meeting, February 24th – March 6th, 2020

Source:	Intel Corporation

Title:	Remaining opens of sidelink physical structure for NR V2X design

Agenda item:	7.2.4.1

Document for:	Discussion and Decision

Introduction

At the RAN#83 meeting, the work item on NR V2X was approved [1]. The specification of technical solutions for NR sidelink is one of the major work item objectives:

- 1. NR sidelink: Specify NR sidelink solutions necessary to support sidelink unicast, sidelink groupcast, and sidelink broadcast for V2X services, considering in-network coverage, out-of-network coverage, and partial network coverage. Support of sidelink signals, channels, bandwidth part, and resource pools [RAN1, RAN2]

In this contribution, we continue discussion on sidelink physical layer structure for NR V2X. Our views on other NR-V2X design aspects are summarized in our companion contributions [7]-[11]. The discussion is currently mainly focusing on Frequency Range 1 (FR1), since according to the WID description [1], “NR sidelink in FR2 is supported by applying the design for FR1”.

Sidelink Slot Structure

According to the RAN4 WG reply in [2], the following AGC settling time is required for sidelink in FR1:

For single CC using CP-OFDM waveform and at least 10 RB allocation

  - 35 usec for 15 kHz SCS

  - 35 usec for 30 kHz SCS

  - 18 usec for 60 kHz SCS

RAN4 has not studied the multicarrier cases.

In the last meeting the duration of time gaps before and after PSFCH was discussed. The option of 1-symbol gap was agreed, and the option of half-symbol gap is open for the case of 15 and 30 kHz. Overall, the option of half-symbol gap can save a whole symbol for useful PSSCH transmission, that is a huge portion considering other overhead such as PSCCH and DMRS. For example, in the case of 3 symbol DMRS, 3 symbol PSCCH, 10 PRB allocation, the number of REs left for PSSCH (including 2nd stage SCI) is equivalent to 14-2(PSFCH)-3(DMRS)-3(PSCCH)-1(AGC)-2(GAP) = 3 symbols. When the switching gap overhead is reduced from 2 symbols to 1, resulting in 4 symbols for PSSCH, the gain is 33% in coverage/throughput!

Furthermore, implementation of such a feature is identical to TA (timing advance) between DL reception and UL transmission in Uu case, specifically in TDD. Thus, no real technical issue is expected if such feature is adopted. Since current agreements already allow configuration of 1-symbol gaps, a new RRC parameter has to be introduced for the purpose of reconfiguring the length of the gap.

  - For 15 and 30 kHz SCS, based on (pre-)configuration in addition to the 1-symbol gap support the half-symbol gap is also supported:

    - Half symbol is used for gap right after PSSCH transmission

    - Half symbol is used for gap right after PSFCH transmission

PSCCH

1st Stage PSCCH scrambling

In order to avoid same scrambling sequence of the 1st stage PSCCH from colliding transmissions using different OCC for the 1st stage PSCCH DMRS, the scrambling of the 1st stage PSCCH must be initialized with the value that is dependent on the used OCC. This can be achieved by initializing the sequence generator with the following initial value:

cinit=(NOCC+1)211

where NOCC is the OCC index used for the 1st stage PSCCH DMRS. As the used OCC value needs to be blindly detected it is known after the DMRS detection.

  - Initialize the 1st stage SCI scrambling based on the OCC ID used for the 1st stage PSCCH DMRS

1st Stage PSCCH DMRS OCC

With the currently agreed values for the 1st stage PSCCH allocation in frequency direction in combination with the allocation of 3 DMRS per OFDM symbol for the OCC values 2 and 4 the total amount of available DMRS is not for all cases an integer multiple of the 2 or 4. As an implementation using OCC with some remaining DMRS not having an orthogonal OCC would lead to non-orthogonal DMRS at one end of the allocated 1st stage PSCCH resource, this leaves an OCC value of 3 as the only option.

  - Use OCC 3 for PSCCH DMRS

In a unicast or groupcast scenario it is beneficial if the receiver knows the OCC in advance as well as the scrambling that is used by transmitter. This can be achieved if OCC is a function that is dependent on the transmitter ID or group ID and slot index. This needs to be agreed during the unicast/groupcast connection setup.

  - For unicast and groupcast communication, selection of OCC value is based on predefined function of the transmitter ID and slot index

As the 1st stage PSCCH DMRS need to be received by each device, the scrambling should be initialized with a static value a priory known to each device.

  - Sidelink resource pool (pre)-configuration includes initialization seed used for PSCCH DMRS generation

1st Stage PSCCH resource mapping in allocation

Furthermore, it is FFS whether to support mapping of PSCCH starting from the highest PRB of PSSCH. In our view, it should be supported in order to solve the highly non-uniform PSCCH resource utilization in the system. As illustrated in Figure 1, the mapping from lowest PRBs only leads to an increased utilization of PSCCH in the lower part of spectrum while underutilized the PSCCH in the higher part of spectrum.

Figure 1. Mapping of PSCCH only from lowest PRB of PSSCH.

The problem can be easily solved by configuring the first half of the sub-channels to be used for the mapping of PSCCH from the lowest PRB of PSSCH and the second half of sub-channels to be used for the mapping of PSCCH from the highest PRB of PSSCH, as illustrated in Figure 2.

Figure 2. Mapping of PSCCH from both lowest PRB of PSSCH and highest PRB of PSSCH, in non-interlaced manner.

The proposed modification preserves the agreement on contiguous mapping of PSSCH as well as greatly relaxes the PSCCH collision/utilization problem, as discussed in [98b-NR-10] email discussion. In that case, when a UE selects a PSSCH resource where both mapping directions are available, it should randomly select which one to use. Note, it is not needed to signal which direction is used since it is unambiguously derived from the PSCCH sub-channel where the SCI was decoded.

  - Support the mapping of PSCCH from both highest and lowest PRBs of the corresponding PSSCH

    - First half of the sub-channels are dedicated to the mapping of PSCCH from the lowest PRB

    - Second half of the sub-channels are dedicated to the mapping of PSCCH from the highest PRB

    - When a UE selects a PSSCH resource where both mapping directions are available, it should randomly select which one to use

PSCCH SCI content (1st Stage)

To enable the signaling of additional resource transmitting the same TB in a chain fashion (see mode-2 contribution [8]) a transmission index needs to be introduced in addition to the time frequency resource signaling.

  - The first stage SCI content includes index of time resource within SCI signaling window to support backward signaling

CRC Size for the 2nd Stage PSCCH

The downlink NR DL polar is using an interleaver according to [4] section 5.3.1.1. This interleaver has the purpose to place some of the CRC bits within the control information buts, thus enabling early termination of blocks in the case of blind detection. For the second stage PSCCH no blind detection is performed and thus early termination is not required. As this interleaver is designed for 24 bit CRC, if it is used with a reduced CRC length in many cases no CRC bits are mapped in between the control information. There are possible ways to handle this situation:

- Bypassing the D-CRC interleaver

- Use the DL interleaver with the same size as SCI + reduced length CRC. This results in no CRC bits being interleaved within the SCI information bits.

- Use the DL interleaver with additional dummy CRC bits

Bypassing the D-CRC interleaver

This is the simplest solution. In this case the interleaver is not used.

D-CRC interleaver unchanged

In general the D-CRC can be described as a procedure mapping the input bits (c0 to cK-1) to output bits (c’0 to c’K-1) as shown in Figure 3. In this case the number of inputs is composed out of A information bits and L CRC bits (K = A + L). The CRC bits are assumed to be the last L input buts (cK-1-L to cK-1). As for the purpose of the 2nd stage SCI we do not necessarily need CRC bits inside the information bits using this D-CRC interleave with a reduced CRC size would lead to the CRC bits remaining at the end of the output block (c’K-1-L to c’K-1).

Figure 3: Usage of D CRC Interleaver with reduced CRC size.

D-CRC interleaver with dummy input insertion

It is also possible to add dummy bits to the input of the D-CRC interleaver. This solution requires the following usage of the interleaver shown in Figure 4. The input bits f0 to f24+L-1 are additional dummy input bits. As for many sizes if the interleaver in [4] section 5.3.1.1 is used it can be guaranteed that these dummy bits are mapped to last bits after the interleaver (c’K to c’K+24-L-1), thus they can be directly discarded.

Dependent on the final CRC size L a reduced set of SCI information bits can be used. It needs to be guaranteed that all dummy bits are mapped to the end of information block.  A table showing all possible SCI sizes without CRC (equal to K – L) is shown in Table 1. We also assume that only the CRC sizes already defined in [4] are used. For L=6 if a higher number of SCI information bits than 98 should be enable it is necessary to have an SCI size dependent mapping to the input bits of the interleaver.

In the decoder this is connected to the swapping of the CRC Gen Matrix. This needs also to be adapted to reflect the reduced CRC size. For the case of L = 6 and a number of information bits larger than 98 (A > 98) which input bits are used need to be defined separately for each possible configuration of A. This means the dummy CRC bits cannot be just mapped to the last input bits but out of the 24 CRC input bit specific ones need to be selected. However, with this mapping it can be guaranteed that the maximum of 18 CRC dummy bits are mapped to last 18 CRC bits.

Table 1: Usable SCI sizes with workaround.

| CRC size | Usable SCI information bit size with workaround |
| --- | --- |
| 6 | 1-98 (With systematic handling as in Figure 3) 1-140 (With SCI size dependent handling) |
| 11 | 1-140 |
| 16 | 1-140 |

Figure 4: Usage of D CRC Interleaver with reduced CRC size with dummy input bits.

Text proposal for Section 8.4.2 in [4] (38.212):

CRC attachment is performed according to clause 7.3.2 except that scrambling is not performed and L=6 and thus gCRC6(D) is used in Clause 5.1. Section 8.4.3 in [4] (38.212): Chanel coding is performed according to clause 7.3.3 except IIL=0.

  - Reduce the CRC length used for the 2nd stage PSCCH to 6 bits

  - Bypass D-CRC interleaver for the 2nd stage PSCCH

The following simulations illustrate the performance difference (see Figure 5). As the current SCI size of the second stage for broadcast is 32 bits, going from 24 bits CRC to 6 bits CRC decreases the overall information bits by 36 %. The simulation parameters are provided in Table 2.

Figure 5: Performance comparison with different CRC size.

2nd Stage PSCCH scrambling initialization

The scrambling sequence of the PSSCH as well as the 2nd stage PSCCH can be initialized with the following parameter:

cinit=NIDX 27+ns,fμ

where ns,fμ is the slot number within a frame, and NIDX equals the decimal representation of CRC on the 1st Stage PSCCH in the same slot associated with the PSSCH according to NIDX=i=0L-1pi2L-1-i with p and L given by clause 8.3.2 in [4] (38.212).

  - Use the CRC bits of the 1st stage PSCCH in combination with the slot number within a frame to initialize the scrambling sequence for the 2nd stage PSCCH

2nd Stage PSCCH details

CSI-RS & 2nd Stage PSCCH

The calculation of the 2nd stage PSCCH symbols in 8.4.4 of [4] (38.212) assumes that the presence of CSI-RS is known. As this information is transmitted in the 2nd stage PSCCH it cannot be assumed to be known for the calculation for the number of used symbols of the 2nd stage PSCCH.

Reminder PRBs & 2nd Stage PSCCH

Reminder PRBs occur if the PRBs in a resource pool cannot be divided into equal size sub-channels. If these resources are included, the resulting REs used for the 2nd stage PSSCH depend on whether selected sub-channels have additional PRBs or not.

MCS, Number of layers & 2nd Stage PSCCH

The formula currently in place in TS 38.212 section 8.4.4 [4] is based on the UCI version. In contrast to the UCI on PUSCH, the 2nd stage PSCCH does not use the same number of spatial layers and modulation order. This means this also needs to be factored into this equation.

Considering these problems with the current formula this leaves two options for solving the problem. First, it is possible to fix some of the problems of the current formula, but this will not solve the cross dependency of the PSSCH TBS determination and the 2nd stage resource calculation. A second option is to make the 2nd stage resource calculation independent of the TBS determination.

Identified issues must be resolved and therefore, we have following proposals:

  - For 2nd stage SCI resource identification

    - Alternative 1: Keep dependency on PSSCH TBS and update the formula as follows

      - Remove CSI-RS overhead from the 2nd stage PSCCH resource calculation

      - Remove REs in reminder PRBs from the calculation of the 2nd stage PSCCH resources

      - Add consideration about different modulation order of the 2nd stage PSCCH and PSSCH into the calculation of the 2nd stage PSCCH resource

      - Add consideration about different number of spatial layers of the 2nd stage PSCCH and PSSCH into the calculation of the 2nd stage PSCCH resources

    - Alternative 2: Remove dependency on PSSCH TBS and update the formula as follows

      - Calculate the number of the 2nd stage PSCCH resources based on the PSSCH spectral efficiency given by MCS index, PSSCH modulation, and number of spatial layers

FDM between the 2nd stage PSCCH and PSSCH DMRS should not be supported as in future extensions it is possible that a higher number of spatial layers, or a different frequency direction DMRS density is defined. Combining this with the possibility of the case that in colliding transmissions this would result in PSSCH DMRS to 2nd stage PSCCH interference, it is of benefit to avoid such situations.

In the case that FDM between 1st stage PSCCH and 2nd stage PSCCH is allowed there is the possibility of 1st stage PSCCH transmissions of other devices colliding with the 2nd stage PSCCH transmission. In addition, for single sub-channel allocations, dependent on the sub-channel size, there might not be enough resource available in this region. Thus, it is better to not allow FDM of 1st and 2nd stage PSCCH.

In the TS 38.212 section 8.4.4, the number of allocated symbols QSCI2' is defined. The number of allocated symbols are mapped to OFDM symbols directly adjacent to the ones that contain PSSCH DMRS, starting from the one with the lowest OFDM symbol index in a frequency first order. If all symbols directly adjacent to PSSCH DMRS are completely allocated with the 2nd stage PSCCH OFDM symbols with distance of 2 OFDM symbols are used in the same fashion. This procedure continues till all 2nd stage PSCCH symbols are allocated.

  - FDM of the 2nd stage PSCCH and PSSCH DMRS is not supported

  - FDM of the 1st stage PSCCH and the 2nd stage PSCCH is not supported

  - Map the 2nd stage PSCCH symbols first to OFDM symbols adjacent to OFDM symbols with PSSCH DMRS starting from the OFDM symbol fulfilling this criterion with the lowest index

    - If after this procedure, there are still 2nd stage PSCCH symbols not mapped to REs, continue this procedure with OFDM symbols with a distance of 2 symbols to ones with PSSCH DMRS

    - This procedure is continued until all symbols are mapped

PSSCH

TBS determination

Calculation of the TBS in DL assumes that the number of PSSCH REs per PRB is constant across the whole allocation of the PDCCH. Afterwards the TBS is determined by multiplying the number of REs with the spectral efficiency defined by the MCS. However, for SL option 3 multiplexing of PSCCH and PSSCH the assumption of constant PSSCH REs across all allocated PRBs is not valid. In contrast to the DL, the SL also must handle the effect of symbols possibly being not usable due to the AGC adaptation.

Figure 6: Example sub-channel configuration with 10 PRB per sub-channel and a specific PSCCH allocation.

To make the determined TBS the same as, for the case the CSI-RS, PT-RS or other dynamic allocated resources which reduce the resources available for PSSCH are allocated or not, this overhead is excluded from the calculation.

For the case that the PSFCH is either present in every TTI or not present at all, no special handling of the TBS determination regarding this aspect is required. For the case when the PSFCH is only present every Nth slot special handling needs to be introduced. From system level perspective this can be solved by overwriting the RE values used for the PSSCH RE determination with (pre)-defined values. To handle different sub-channel allocations this does also need to be considered for this pre-configuration. With the same solution we can also handle the case of different slot length in the case of sidelink transmission within a licensed carrier.

In addition, as dynamic DMRS pattern were agreed, the DMRS pattern used for sidelink transmission of a TB can change over different slots. This also needs to be considered. We take this into account assuming that the densest pattern (among configured DMRS patterns) is used.

As there is currently a cross dependency between the TBS determination and the 2nd stage resource calculation we show two alternative formulations of the TBS determination. The first is assuming (as in the 2nd stage PSCCH resource section) that the 2nd stage PSCCH resource calculation is dependent on the calculated TBS, and the second one is assuming this not the case and the amount of REs reserved for the 2nd stage PSCCH in know at the time the TBS size is calculated.

The text for TBS determination for alternative 1 can be as follows:

nRE=l=1NsymbolPSSCH-1MSCPSSCHl-MSCDMRSl-MSCPT-RSl-NohPRBnPRB

where:

-	MscPSSCH(l) is the scheduled bandwidth of PSSCH transmission without reminder PRBs. in subcarriers (REs)

-	MscDMRS(l) is the number of subcarriers (REs) in OFDM symbol l that carries DMRS, in the PSSCH transmission assuming DMRS configuration with the most amount of REs (pre)-configured for the resource pool.  assuming that the PSFCH is present if configured, in all PRBs except reminder PRBs.

-	MscPT-RS(l) is the number of subcarriers (REs) in OFDM symbol l that carries PT-RS, in the PSSCH transmission in all PRBs except reminder PRBs

(REs) as reminder PRBs to the allocated sub-channels-	NsymbolPSSCH is the number of allocated symbols for the PSSCH. I, that it is allocated in every sidelink slot, if PSFCH is configured it is assumed to be present in all slots.

-	NohPRB is a per PRB overhead value preconfigured.

- 	nPRB is the number of PRBs allocated to PSSCH excluding reminder PRBs

Further proceed with the step 2) of clause 5.1.3.2 in 38.214.

The text for TBS determination for alternative 2 can be as follows:

nRE=l=1NsymbolPSSCH-1MSCPSSCHl-MSCDMRSl-MSCPT-RSl-MSCSCI-2(l)-NohPRBnPRB

where:

-	MscPSSCH(l) is the scheduled bandwidth of PSSCH transmission without reminder PRBs.

-	MscDMRS(l) is the number of subcarriers in OFDM symbol l that carries DMRS, in the PSSCH transmission assuming DMRS configuration with the most amount of REs (pre)-configured for the resource pool assuming that the PSFCH is present if configured, in all PRBs except reminder PRBs.

-	MscPT-RS(l) is the number of subcarriers in OFDM symbol l that carries PT-RS, in the PSSCH transmission in all PRBs except reminder PRBs.

-	MSCSCI-2(l) is the number of subcarriers in OFDM symbol l that carrier 2nd stage PSCCH symbols.

-	NsymbolPSSCH is the number of allocated symbols for the PSSCH, if PSFCH is configured it is assumed to be present in all slots.

-	NohPRB is a per PRB overhead value preconfigured.

- 	nPRB is the number of PRBs allocated to PSSCH.

Further proceed with step 2) of clause 5.1.3.2 in 38.214.

  - Alternative 1: 2nd stage PSCCH resource allocation dependent on TBS size

    - Calculate the TBS assuming knowledge of CSI-RS and 2nd stage PSCCH allocation is not available

  - Alternative 2: 2nd stage PSCCH resource allocation is independent of TBS size

    - Calculate the TBS with knowledge of the presence of the 2nd stage PSCCH allocation

PSSCH scrambling

The PSSCH and the 2nd stage PSCCH are transmitted in the same resources based on the same DMRS. It is not necessary to have PSSCH having a different scrambling initialization compared to the PSSCH.

  - Use the CRC bits of the 1st stage PSCCH in combination with the slot number within a frame to initialize the scrambling sequence for the PSSCH

PSSCH DMRS type 1 and/or type 2 Rel. 16 NR CP-OFDM DMRS

To reduce the implementation complexity only type 1 should be supported, especially since for Rel. 16 NR V2X only up to 2 spatial layers need to be supported.

  - Only type 1 DMRS are used for the PSSCH

CSI-RS

As the presence of the CSI-RS is signaled in the 2nd stage PSCCH, in case that CSI-RS would be allocated in the resource used for the 2nd stage PSCCH assume that the CSI-RS are punctured.

  - Puncture CSI-RS if allocation collide with the 2nd stage PSCCH

PSFCH

In our understanding, the issue of cyclic shift assignment should be discussed in procedures. Please refer to our tdoc on NR V2X physical layer procedures [10].

Sidelink Pool Configuration and Spectrum Underutilization

The current agreement for configuration of resource pool based on number of subchannels and sub-channel sizes is suboptimal since may lead to spectrum underutilization in many scenarios. Therefore, we propose to fix this issue as described in this section.

In order to address this problem, we propose to define total number of PRBs N for allocation of resource pool in frequency domain and divide total number of PRBs N on sub-channel size M. Two alternatives how to handle reminder PRBs are possible:

Alt.1 Remainder of division P can be added to the first or last sub-channel so that its size is always larger by P comparing to other subchannels, i.e. {M, M+P} partitioning

Alt.2 Remainder of PRBs P can be distributed equally across P sub-channels (e.g. P first or last subchannels) so that their size is equal to (N+1) comparing to other subchannels, i.e. {M, M+1} partitioning

It is obvious that having uniform subchannel sizes is more desirable from system perspective and therefore alternative 2 should be preferred by RAN1. Therefore, we recommend the following approach to configure number of subchannels NS per resource pool and derive subchannel size as follows

S = floor(N/Ns) – basic sub-channel size in PRBs

P = mod(N, NS) – number of subchannels with size in PRBs (S+1)

(NS-P) number of subchannels of size S PRBs

A numerical example with the currently agreed sub-channel size would be a 10 MHz channel with 30 kHz SCS and 10 PRB sub-channel size. This means that the resulting sub-channels with the here shown alternative 1 would (LTE legacy procedure) result in one sub-channel with 10 PRBs and one with 14 PRBs. As these 4 additional PRBs result in a huge performance difference between the two subchannels we don’t think this is a good solution. Especially, considering that since for multi-TTI transmission the presence cannot be considered for single sub-channel transmissions not always the same one will be selected.

  - For sidelink resource pool configuration,

    - Define total number of PRBs N for allocation of resource pool in frequency domain and divide total number of PRBs N on sub-channel size M

    - Remainder of PRBs P are distributed equally across P sub-channels (e.g. P first or last subchannels) so that their size is equal to (M+1) comparing to other subchannels

Conclusions

In this contribution, we provided our views on sidelink physical structure for NR V2X communication. In summary, we have following proposals:

  - For 15 and 30 kHz SCS, based on (pre-)configuration in addition to the 1-symbol gap support, the half-symbol gap is also supported:

    - Half symbol is used for gap right after PSSCH transmission

    - Half symbol is used for gap right after PSFCH transmission

  - Initialize the 1st stage SCI scrambling based on the OCC ID used for the 1st stage PSCCH DMRS

  - Use OCC 3 for PSCCH DMRS

  - For unicast and groupcast communication, selection of OCC value is based on predefined function of the transmitter ID and slot index

  - Sidelink resource pool (pre)-configuration includes initialization seed used for PSCCH DMRS generation

  - Support the mapping of PSCCH from both highest and lowest PRBs of the corresponding PSSCH

    - First half of the sub-channels are dedicated to the mapping of PSCCH from the lowest PRB

    - Second half of the sub-channels are dedicated to the mapping of PSCCH from the highest PRB

    - When a UE selects a PSSCH resource where both mapping directions are available, it should randomly select which one to use

  - The first stage SCI content includes index of time resource within SCI signaling window to support backward signaling

  - Reduce the CRC length used for the 2nd stage PSCCH to 6 bits

  - Bypass D-CRC interleaver for the 2nd stage PSCCH

  - Use the CRC bits of the 1st stage PSCCH in combination with the slot number within a frame to initialize the scrambling sequence for the 2nd stage PSCCH

  - For 2nd stage SCI resource identification

    - Alternative 1: Keep dependency on PSSCH TBS and update the formula as follows

      - Remove CSI-RS overhead from the 2nd stage PSCCH resource calculation

      - Remove REs in reminder PRBs from the calculation of the 2nd stage PSCCH resources

      - Add consideration about different modulation order of the 2nd stage PSCCH and PSSCH into the calculation of the 2nd stage PSCCH resource

      - Add consideration about different number of spatial layers of the 2nd stage PSCCH and PSSCH into the calculation of the 2nd stage PSCCH resources

    - Alternative 2: Remove dependency on PSSCH TBS and update the formula as follows

      - Calculate the number of the 2nd stage PSCCH resources based on the PSSCH spectral efficiency given by MCS index, PSSCH modulation, and number of spatial layers

  - FDM of the 2nd stage PSCCH and PSSCH DMRS is not supported

  - FDM of the 1st stage PSCCH and the 2nd stage PSCCH is not supported

  - Map the 2nd stage PSCCH symbols first to OFDM symbols adjacent to OFDM symbols with PSSCH DMRS starting from the OFDM symbol fulfilling this criterion with the lowest index

    - If after this procedure, there are still 2nd stage PSCCH symbols not mapped to REs, continue this procedure with OFDM symbols with a distance of 2 symbols to ones with PSSCH DMRS

    - This procedure is continued until all symbols are mapped

  - Alternative 1: 2nd stage PSCCH resource allocation dependent on TBS size

    - Calculate the TBS assuming knowledge of CSI-RS and 2nd stage PSCCH allocation is not available

  - Alternative 2: 2nd stage PSCCH resource allocation is independent of TBS size

    - Calculate the TBS with knowledge of the presence of the 2nd stage PSCCH allocation

  - Use the CRC bits of the 1st stage PSCCH in combination with the slot number within a frame to initialize the scrambling sequence for the PSSCH

  - Only type 1 DMRS are used for the PSSCH

  - Puncture CSI-RS if allocation collide with the 2nd stage PSCCH

  - For sidelink resource pool configuration,

    - Define total number of PRBs N for allocation of resource pool in frequency domain and divide total number of PRBs N on sub-channel size M

    - Remainder of PRBs P are distributed equally across P sub-channels (e.g. P first or last subchannels) so that their size is equal to (M+1) comparing to other subchannels

References

- 3GPP RP-190766, “New WID on 5G V2X with NR sidelink”, Shenzhen, China, 2019.

- 3GPP R4-1905241, “Reply LS on NR V2X UE RF parameters for NR V2X service”, RAN4 WG, Xian, China, 8th – 12th April 2019.

- 3GPP TS 38.211, “Physical channels and modulations (DRAFT)”, version 16.0.0, December 2019.

- 3GPP TS 38.212, “Multiplexing and channel coding (DRAFT)”, version 16.0.0, December 2019.

- 3GPP TS 38.213, “Physical layer procedures for control (DRAFT)”, version 16.0.0, December 2019.

- 3GPP TS 38.214, “Physical layer procedures for data (DRAFT)”, version 16.0.0, December 2019.

- R1-2000730 “Remaining Issues in Mode-1 Resource Allocation”, e-Meeting, February 2020

- R1-2000731 “Remaining opens of resource allocation mode-2 for NR V2X design”, e-Meeting, February 2020

- R1-2000732 “Remaining opens of sidelink synchronization for NR V2X design”, e-Meeting, February 2020

- R1-2000733 “Remaining Issues and Corrections in Sidelink Procedures”, e-Meeting, February 2020

- R1-2000734 “Remaining details of QoS and congestion control for NR V2X design”, e-Meeting, February 2020

Annex

LLS simulations assumptions

Table 2: stage PSCCH simulations assumptions

| Parameter | Value |
| --- | --- |
| SCI Size | 32 bit SCI for the second stage in the two stage design |
| PSCCH FEC | DL Polar code |
| PSCCH allocation | Two stage, second stage: 1 OFDM Symbols 10 PRBs |
| Channel model | CDL Urban LOS with 120 km/h according |
| Carrier frequency | 5.9 GHz |
| Modulation format | QPSK |
| SCS | 30 kHz |
| Channel Estimation | MMSE based on ideal knowledge of Doppler-delay statistics |
| Tx antenna configuration | 2 cross polarized antennas |
| Rx antenna configuration | 4 antennas with 2 polarizations |
| EVM | No EVM is applied |
| UE receiver algorithm | MMSE |
| AGC settling time | 1 OFDM symbol (Current LLS assumption, currently waiting till final guidance from RAN4 is available) |
| Tx/Rx switching gap | 1 OFDM symbol |