3GPP TSG RAN WG1 #100		                                                      R1-2000760

e-Meeting, February 24th – March 6th, 2020

Agenda item:	7.2.8.1

Source: 	CMCC

Title: 	Corrections on DL PRS resource set parameters

Document for:	Discussion and Decision

Introduction

After RAN#86 meeting, the Rel-16 specifications are available for check. However, in TS 38.214 [1], section 5.1.6.5 on PRS reception procedures, the values of the DL PRS resource set parameter DL-PRS-periodicity are wrongly captured as “TperPRS∈2μ4, 8, 16, 32, 64, 5, 10, 20, 40, 80, 160, 320, 640, 1280, 2560, 5120, 10240, 20480 slots, where μ=0, 1, 2, 3 for DL-PRS-SubcarrierSpacing = 15, 30, 60 and 120kHz respectively. TperPRS=2μ⋅20480 is not supported for μ=0.” The parameters in the current version were agreements of RAN1#98 meeting [2], however, in RAN1#99 meeting [3], it was updated as follows:

Agreement: The following periodicity values of DL PRS resource allocation are supported depending on SCS - {4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240} slots, µ = 0, 1, 2, 3 for SCS 15, 30, 60 and 120kHz respectively

In this contribution, text proposal to the TS 38.214 is provided to correct the DL PRS resource set parameters that are mistakenly captured.

- References

- TS 38.214, Physical procedures for data, v16.0.0

- Final Report of 3GPP TSG RAN WG1 #98 v2.0.0, Prague, Czech Republic.

- Draft Report of 3GPP TSG RAN WG1 #99 v0.1.0, Reno, USA.

Text proposal

--------------------------<Start of text proposal for TS 38.214>--------------------------

- 5.1.6 5 PRS reception procedure

The UE can be configured with one or more DL PRS resource set configuration(s) as indicated by the higher layer parameters DL-PRS-ResourceSet and DL-PRS-Resource. Each DL PRS resource set consists of K≥1 DL PRS resource(s) where each has an associated spatial transmission filter. The UE can be configured with one or more DL PRS Positioning Frequency Layer configuration(s) as indicated by the higher layer parameter DL-PRS-PositioningFrequencyLayer. A DL PRS Positioning Frequency Layer is defined as a collection of DL PRS Resource Sets which have common parameters configured by DL-PRS-PositioningFrequencyLayer.

The UE assumes that the following parameters for each DL PRS resource(s) are configured via higher layer parameters DL-PRS-PositioningFrequencyLayer, DL-PRS-ResourceSet and DL-PRS-Resource.

A positioning frequency layer consists of one or more PRS resource sets and it is defined by:

-	DL-PRS-SubcarrierSpacing defines the subcarrier spacing for the DL PRS resource. All DL PRS Resources and DL PRS Resource sets in the same DL-PRS-PositioningFrequencyLayer have the same value of DL-PRS-SubcarrierSpacing. The supported values of DL-PRS-SubcarrierSpacing are given in Table 4.2-1 of [4, TS38.211].

-	DL-PRS-CyclicPrefix defines the cyclic prefix for the DL PRS resource. All DL PRS Resources and DL PRS Resource sets in the same DL-PRS-PositioningFrequencyLayer have the same value of DL-PRS-CyclicPrefix. The supported values of DL-PRS-CyclicPrefix are given in Table 4.2-1 of [4, TS38.211].

-	DL-PRS-PointA defines the absolute frequency of the reference resource block. Its lowest subcarrier is also known as Point A. All DL PRS resources belonging to the same DL PRS Resource Set have common Point A and all DL PRS Resources sets belonging to the same DL-PRS-PositioningFrequencyLayer have a common Point A.

The UE expects that it will be configured with [IDs] each of which is defined such that it is associated with multiple DL PRS Resource Sets from the same cell. The UE expects that one of these [IDs] along with a DL-PRS-ResourceSetId and a DL-PRS-ResourceId can be used to uniquely identify a DL PRS Resource.

A PRS resource set consists of one or more PRS resources and it is defined by:

-	DL-PRS-ResourceSetId defines the identity of the DL PRS resource set configuration.

-	DL-PRS-Periodicity defines the DL PRS resource periodicity and takes values 
TperPRS∈2μ4, 8, 16, 32, 64, 5, 10, 20, 40, 80, 160, 320, 640, 1280, 2560, 5120, 10240, 20480 slots, where μ=0, 1, 2, 3 for DL-PRS-SubcarrierSpacing=15, 30, 60 and 120kHz respectively.  TperPRS=2μ⋅20480 is not supported for μ=0. All the DL PRS resources within one resource set are configured with the same periodicity.

-	DL-PRS-ResourceRepetitionFactor defines how many times each DL-PRS resource is repeated for a single instance of the DL-PRS resource set and takes values TrepPRS∈1,2,4,6,8,16,32,. All the DL PRS resources within one resource set have the same ResourceRepetitionFactor

-	DL-PRS-ResourceTimeGap defines the offset in number of slots between two repeated instances of a DL PRS resource with the same DL-PRS-ResourceID within a single instance of the DL PRS resource set and takes values TgapPRS∈1,2,4,8,16,32. The UE only expects to be configured with DL-PRS-ResourceTimeGap if DL-PRS-ResourceRepetitionFactor is configured with value greater than 1. The time duration spanned by one instance of a DL-PRS-ResourceSet is not expected to exceed the configured value of DL-PRS-Periodicity. All the DL PRS resources within one resource set have the same DL-PRS-ResourceTimeGap.

-	DL-PRS-MutingPattern defines a bitmap of the time locations where the DL PRS resource is expected to not be transmitted for a DL PRS resource set. The bitmap size can be {2, 4, 8, 16, 32} bits long. The bitmap has two options for applicability. In the first option each bit in the bitmap corresponds to a configurable number of consecutive instances of a DL-PRS-ResourceSet where all the DL-PRS-Resources within the set are muted for the instance that is indicated to be muted. In the second option each bit in the bitmap corresponds to a single repetition index for each of the DL-PRS-Resources within each instance of a DL-PRS-ResourceSet and the length of the bitmap is equal to DL-PRS-ResourceRepetitionFactor. Both options may be configured at the same time in which case the logical AND operation is applied to the bit maps as described in clause 7.4.1.7.4 of [4, TS 38.211].

-	DL-PRS-SFN0-Offset defines the time offset of the SFN0 slot 0 for the transmitting cell with respect to SFN0 slot 0 of [FFS in RAN2].

-	DL-PRS-ResourceSetSlotOffset defines the slot offset with respect to SFN0 slot 0 and takes values ToffsetPRS∈0,1,…,TperPRS-1.

-	DL-PRS-CombSizeN defines the comb size of a DL PRS resource where the allowable values are given in Clause 7.4.1.7.1 of [TS38.211]. All DL PRS resource sets belonging to the same positioning frequency layer have the same value of DL-PRS-combSizeN.

-    DL-PRS-ResourceBandwidth defines the number of resource blocks configured for PRS transmission. The parameter has a granularity of 4 PRBs with a minimum of 24 PRBs and a maximum of 272 PRBs. All DL PRS resources sets within a positioning frequency layer have the same value of DL-PRS-ResourceBandwidth.