3GPP TSG-RAN WG1 Meeting #100-e	R1-2000780

Online, February 24th – March 6th, 2020

Agenda Item:	7.2.6.5

Source:	Ericsson

Title:	Feature lead summary of low PAPR RS

Document for:	Discussion

# 1	Introduction

This document contains the feature lead summary of critical issues related to maintenance of the low PAPR RS topic under Rel-16 eMIMO WI.

The chairman’s guideline to the feature lead regarding this summary, which is the summary of the preparation phase (2/18-2/21) is:

      - For the summary, due to limited time for preparation, each feature lead is not required to provide a detailed summary of companies positions and their proposals, but rather based on the contributions, to provide a list of issues/proposals and an indication of priority

From Tuesday 2/18, three email threads will be started to finalize the set of critical issues from those listed in this document. The titles of these email list will be

      - [100e-Prep-eMIMO-lowPAPR-A]

      - [100e-Prep-eMIMO-lowPAPR-B]

      - [100e-Prep-eMIMO-lowPAPR-C]

By the end of Friday 2/21, once all critical issues have been identified and the group agrees the topics to be discussed over email a revised feature lead summary will be submitted with Tdoc#.

# 2 Low PAPR RS maintenance issues and priority

To organize the email discussion, the issues have been grouped into maximum three threads according to the chairman’s guidance. The issues in thread A may be less controversial and are lumped together while issue 6 and 7 may need more discussion and are thus given separate threads.

## 2.1 Issues with high priority

| Issue # | Description | Tdoc | Email discussion thread |
| --- | --- | --- | --- |
| 1 | If pi/2-BPSK PUSCH is scheduled without n_SCID bit in the DCI format, then n_SCID=0. This description is missing from 38.211, section 6.4.1.1.1.2 | R1-2000242 | A |
| 2 | The restriction that the low PAPR RS for pi/2 BPSK PUSCH does not apply when scheduled from DCI format 0_0 is missing | R1-2000460, R1-2000991 | A |
| 3 | In TS 38.212, the low PAPR RS for pi/2 BPSK may be configured but the new tables should not be used if not pi/2 BPSK is not scheduled | R1-2000991 | A |
| 4 | Whether the low PAPR RS enhancements for pi/2-BSPK PUSCH modulation applies also to DCI format 0_2 | R1-2000242, R1-2000991 | A |
| 5 | For PDSCH DMRS for CP-OFDM, λ is not introduced as the new variable as in the PUSCH case and fallback to Rel-15 DMRS is not possible. | R1-2000460, R1-2000757, R1-2000768, R1-2000991, R1-2001042 | A |
| 6 | Whether nIDPUSCH=NIDnSCID (as in current spec 6.4.1.1.2 of 38.211) or nIDRS= NIDnSCID is the correct interpretatation of the agreement for pi/2-BSPK PUSCH. Needs to be clarified. | R1-2000460, R1-2000768 | B |
| 7 | For low PAPR pi/2-BPSK PUCCH, format 3 and 4, whether and if so how the DMRS depends on the cyclic shift value α is unclear | R1-2000460, R1-2000991, R1-2001042 | C |

## 2.2 Editorial issues

| Issue # | Description | Tdoc | Email discussion thread |
| --- | --- | --- | --- |
| 8 | The sentence defining the symbol index l appears twice in 6.4.1.1.1.2 of TS 38.211 | R1-2000242 | A |
| 9 | For PUSCH DMRS for CP-OFDM, nSCIDλ should be corrected to nSCIDλ in 6.4.1.1.1.1 of TS 38.211 | R1-2000460, R1-2000768, R1-2000991, R1-2001042, R1-2001076 | A |
| 10 | Alignment of parameter name dmrs-Downlink-r16 with RRC specifications | R1-2000991 | A |

## 2.3 Issue with low priority

This issue has been discussed in previous RAN1 meetings without consensus and is thus given lowest priority in this e-meeting.

| Issue # | Description | Tdoc |
| --- | --- | --- |
| 11 | Whether to specify the relation between PUSCH data spectrum shaping filter and PUSCH RS spectrum shaping filter in 38.211 for pi/2-BPSK PUSCH | R1- 2000945 |