3GPP TSG RAN WG1 Meeting #100	R1- 2000797

e-Meeting, February 24th – March 06th, 2020

Agenda item:		7.2.3.3

Source:		Nokia, Nokia Shanghai Bell

Title:	Maintenance of Rel-16 IAB Resource Multiplexing

Document for:		Discussion and Decision

# 1	Introduction

In this contribution, we summarize our input to the maintenance discussions on IAB resource multiplexing among access and backhaul links.

# 2	Discussion

Suggested changes on 38.213 are highlighted as follows,

### Correction 1: Use per-DU resource configuration

When implementing IAB resource configuration and related signaling details, the “serving cell” is used in several places and which seems to be inconsistence with the RAN1 agreements. We mark this issue in yellow below from the current specification.

“For each serving cell of an IAB-node DU, the IAB-node DU can be provided an indication for a slot format over a number of slots by IAB-DU-Resource-Configuration. For each serving cell, an IAB-node MT can be provided an indication for a slot format over a number of slots by tdd-UL-DL-ConfigDedicated-IAB-MT. If the IAB-node MT is provided tdd-UL-DL-ConfigDedicated-IAB-MT, the parameter tdd-UL-DL-ConfigDedicated-IAB-MT overrides only flexible symbols over the number of slots as provided by TDD-UL-DL-ConfigurationCommon.”

“Serving cell of an IAB-node DU” may mean that “per-link” resource configuration where serving cells that are configured for child nodes/access UEs and supported by the IAB-node DU may be configured with separate IAB-DU-Resource-Configuration. This leads to the discussion in Rel-16 on “per-link” or “per-DU” resource configuration, and “per-DU” was agreed in RAN1 #97 meeting,

Agreements:

For the semi-static DU resource configuration, the following is supported:

- The resources are configured on a per DU (cell) basis

  - FFS: indication of additional supplemental per-link resource configurations of child DUs

In other words, the DU resource configuration provided by the CU, and the DU resource configuration should be applicable for all UE/child nodes supported by the IAB DU. We highlight the changes required to correct this in proposal 1.

### Correction 2: H/S/NA attributes per resource type in a slot

The current spec does not reflect the idea of configuring H/S/NA attributes per resource type in a slot, as it mentioned to be a valid per symbol in a slot. We mark this issue in yellow below from the current specification.

“With reference to slots of an IAB-node DU serving cell, a symbol in a slot of an IAB-node DU serving cell can be configured to be of hard, soft, or unavailable type. When a downlink, uplink, or flexible symbol is configured as hard, the IAB-node DU serving cell can respectively transmit, receive, or either transmit or receive in the symbol.”

However, the RAN1 #98bis meeting agreement on this is as follows,

Agreements:

H/S/NA attributes for the per-cell DU resource configuration are explicitly indicated per-resource type (D/U/F) in each slot.

We address the both issues in proposal 1.

Proposal 1: Adopt the following text proposal to correct to resource configuration for DU:

- Correcting the mentions of “serving cell of an IAB-node DU” (in Red).

- Correction of configuring H/S/NA attributes per resource type in a slot (in Green).

TP to TS 38.213, Sec. 14: ## <Unchanged text is omitted> For each serving cell of an IAB-node DU, the IAB-node DU can be provided an indication for a slot format over a number of slots by IAB-DU-Resource-Configuration-TDD-Config. For each serving cell, an IAB-node MT can be provided an indication for a slot format over a number of slots by tdd-UL-DL-ConfigDedicated-IAB-MT. If the IAB-node MT is provided tdd-UL-DL-ConfigDedicated-IAB-MT, the parameter tdd-UL-DL-ConfigDedicated-IAB-MT overrides only flexible symbols over the number of slots as provided by TDD-UL-DL-ConfigurationCommon. ## <Unchanged text is omitted> With reference to slots of an IAB-node DU serving cell, all downlink, uplink, or flexible symbols in a slot of an IAB-node DU serving cell can be configured to be of hard, soft, or unavailable type by IAB-DU-Resource-Configuration-H/S/NA-Config. When a downlink, uplink, or flexible symbol is configured as hard, the IAB-node DU serving cell can respectively transmit, receive, or either transmit or receive in the symbol. ## <Unchanged text is omitted> For each serving cell of an IAB-node DU in a set of serving cells of the IAB-node DU, the IAB-node DU can be provided: - an identity of the IAB-node DU serving cell by iabDuCellId-AI - a location of an availability indicator (AI) index field in DCI format 2_5 by positionInDCI-AI - a set of availability combinations by availabilityCombinations, where each availability combination in the set of availability combinations includes - resourceAvailability indicating availability of soft symbols in one or more slots for the IAB-node DU serving cell, and - a mapping for the soft symbol availability combinations provided by AvailabilityCombination to a corresponding AI index field value in DCI format 2_5 provided by availabilityCombinationId ## <Unchanged text is omitted>

### Correction 3: Entries provided in Table 14.2

RAN1 #98 and #99 meetings had following agreements on the explicit indication of the availability of the soft resources.

Agreements:

A DCI following the DCI Format 2_0 structure is used to indicate DU-IA to an IAB node using a new IA-RNTI different from SFI-RNTI.

This DCI contains one or multiple fields (similar to SFI-index fields in DCI Format 2_0), each field value is used as the index in a RRC configured AI (Availability Indicator) AvailabilityCombination table (similar to the SFI SlotFormatCombination table).

Each entry in the AI AvailabilityCombination table indicates the resource availability for a set of consecutive slots.

Each element of each entry in the AI AvailabilityCombination table indicates the resource availability in a slot.

The resource availability can take 8 values:

| Value | Meaning |
| --- | --- |
| 0 | No resources available |
| 1 | D resources available |
| 2 | U resources available |
| 3 | D and U resources available |
| 4 | F resources available |
| 5 | D and F resources available |
| 6 | U and F resources available |
| 7 | All resources available |

The maximum number of entries in the AI AvailabilityCombination table is 512.

Agreements:

Update the agreement from RAN1#98bis as the following:

The resource availability can take 8 values:

| Value | Meaning |
| --- | --- |
| 0 | D soft resources: no indication of availability U soft resources: no indication of availability F soft resources: no indication of availability |
| 1 | D soft resources: indicated available U soft resources: no indication of availability F soft resources: no indication of availability |
| 2 | D soft resources: no indication of availability U soft resources: indicated available F soft resources: no indication of availability |
| 3 | D soft resources: indicated available U soft resources: indicated available F soft resources: no indication of availability |
| 4 | D soft resources: no indication of availability U soft resources: no indication of availability F soft resources: indicated available |
| 5 | D soft resources: indicated available U soft resources: no indication of availability F soft resources: indicated available |
| 6 | D soft resources: no indication of availability U soft resources: indicated available F soft resources: indicated available |
| 7 | D soft resources: indicated available U soft resources: indicated available F soft resources: indicated available |

However, 38.213 is not capturing the agreement clearly and there can be a misunderstanding that of indication of not available can also be explicitly indicated toward the IAB-node DU.  We mark the issue in yellow using the current specification text.

“An AI index field value in a DCI format 2_5 indicates to an IAB-node DU a soft symbol availability in each slot for a number of slots for each DL BWP or each UL BWP starting from a slot where the IAB-node detects the DCI format 2_5. The number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 2_5 as provided by SearchSpace-IAB. The AI index field includes  bits where maxAIindex is the maximum of the values provided by corresponding availabilityCombinationId. An availability for a soft symbol in a slot is identified by a corresponding value resourceAvailability as provided in Table 14.2.

Table 14.2: Mapping between values of resourceAvailability elements and types of soft symbol availability in a slot

| Value | Indication |
| --- | --- |
| 0 | No soft symbols are indicated available |
| 1 | Only DL soft symbols are indicated available |
| 2 | Only UL soft symbols are indicated available |
| 3 | Only DL and UL soft symbols are indicated available |
| 4 | Only Flexible soft symbols are indicated available |
| 5 | Only DL and Flexible soft symbols are indicated available |
| 6 | Only UL and Flexible soft symbols are indicated available |
| 7 | DL, UL, and Flexible soft symbols are indicated available |

If a PDCCH monitoring periodicity for DCI format 2_5 is smaller than a duration of an availability combination of soft symbols over a number of slots that the UE obtains at a PDCCH monitoring occasion for DCI format 2_5 by a corresponding AI index field value, and the UE detects more than one DCI formats 2_5 indicating an availability combination of soft symbols in a slot, the UE expects that each of the more than one DCI formats 2_5 indicates a same value for the availability combination of the soft symbols in the slot.”

Proposal 2: Adopt the following text proposal to correct to resource configuration for DU:

- Correcting Table 14.2 to reflect the agreed entries (in Red).

TP to TS 38.213, Sec. 14: ## <Unchanged text is omitted> An AI index field value in a DCI format 2_5 indicates to an IAB-node DU a soft symbol availability in each slot for a number of slots for each DL BWP or each UL BWP starting from a slot where the IAB-node detects the DCI format 2_5. The number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 2_5 as provided by SearchSpace-IAB. The AI index field includes bits where maxAIindex is the maximum of the values provided by corresponding availabilityCombinationId. An availability for a soft symbol in a slot is identified by a corresponding value resourceAvailability as provided in Table 14.2. Table 14.2: Mapping between values of resourceAvailability elements and types of soft symbol availability in a slot 0; No indication of availability for soft symbols are indicated available. 1; Only DL soft symbols are indicated available No indication of availability for UL and Flexible soft symbols. 2; Only UL soft symbols are indicated available No indication of availability for DL and Flexible soft symbols. 3; Only DL and UL soft symbols are indicated available No indication of availability for Flexible soft symbols. 4; Only Flexible soft symbols are indicated available No indication of availability for DL and UL soft symbols. 5; Only DL and Flexible soft symbols are indicated available No indication of availability for UL soft symbols. 6; Only UL and Flexible soft symbols are indicated available No indication of availability for DL soft symbols. 7; DL, UL, and Flexible soft symbols are indicated available. If a PDCCH monitoring periodicity for DCI format 2_5 is smaller than a duration of an availability combination of soft symbols over a number of slots that the UE obtains at a PDCCH monitoring occasion for DCI format 2_5 by a corresponding AI index field value, and the UE detects more than one DCI formats 2_5 indicating an availability combination of soft symbols in a slot, the UE expects that each of the more than one DCI formats 2_5 indicates a same value for the availability combination of the soft symbols in the slot. Adopt the following regarding the IAB behaviour as it relates to soft DU resources: The IAB node may use a soft DU resource for transmission and/or reception in line with D/U/F configuration of the resource, if and only if - the use of the resource does not change the IAB-MT behavior relative to the IAB-MT behavior if the resource was instead configured as unavailable type or - the IAB node has received a DCI with DCI format 4_0 indicating that the resource is available ## <Unchanged text is omitted>

### Correction 4: Use of soft resources

The following agreement was made for using soft resources at the IAB DU.

Agreements:

Adopt the following regarding the IAB behaviour as it relates to soft DU resources:

The IAB node may use a soft DU resource for transmission and/or reception in line with D/U/F configuration of the resource, if and only if

- the use of the resource does not change the IAB-MT behavior relative to the IAB-MT behavior if the resource was instead configured as unavailable type

or

- the IAB node has received a DCI with DCI format 4_0 indicating that the resource is available

However, the agreement above is not clearly reflected in 38.213. We mark this issue in yellow below from the current specification.

“When a downlink, uplink, or flexible symbol is configured as soft, the IAB-node DU can respectively transmit, receive or either transmit or receive in the symbol only if

- for the IAB-node MT, ability to transmit or receive by the IAB-node DU in the soft symbol is equivalent to a configuration of the soft symbol as unavailable, or

- the IAB-node DU detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available for transmission or reception

When a symbol is configured as unavailable, the IAB-node DU neither transmits nor receives in the symbol.

A symbol of a slot is equivalent to being configured as hard if an IAB-node DU would either transmit a SS/PBCH block or a periodic CSI-RS in the symbol of the slot, or would receive a PRACH or a SR in the symbol of the slot.”

Proposal 3: Adopt the following text proposal to correct to resource configuration for DU:

- simplifying the text describing the use of the soft resource at the IAB-node DU (in Red).

TP to TS 38.213, Sec. 14: ## <Unchanged text is omitted> When a downlink, uplink, or flexible symbol is configured as soft, the IAB-node DU can respectively transmit, receive or either transmit or receive in the symbol only if - for the IAB-node MT, ability to transmit or receive by the IAB-node DU in the soft symbol is not changed due to the use of equivalent to a configuration of the soft symbol as unavailable, or - the IAB-node DU detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available for transmission or reception ## <Unchanged text is omitted>

# 3	Conclusions

In this contribution, we provide corrections on the IAB CR on 38.213, and our proposals are as follows,

Proposal 1: Adopt the following text proposal to correct to resource configuration for DU:

- Correcting the mentions of “serving cell of an IAB-node DU” (in Red).

- Correction of configuring H/S/NA attributes per resource type in a slot (in Green).

TP to TS 38.213, Sec. 14: ## <Unchanged text is omitted> For each serving cell of an IAB-node DU, the IAB-node DU can be provided an indication for a slot format over a number of slots by IAB-DU-Resource-Configuration-TDD-Config. For each serving cell, an IAB-node MT can be provided an indication for a slot format over a number of slots by tdd-UL-DL-ConfigDedicated-IAB-MT. If the IAB-node MT is provided tdd-UL-DL-ConfigDedicated-IAB-MT, the parameter tdd-UL-DL-ConfigDedicated-IAB-MT overrides only flexible symbols over the number of slots as provided by TDD-UL-DL-ConfigurationCommon. ## <Unchanged text is omitted> With reference to slots of an IAB-node DU serving cell, all downlink, uplink, or flexible symbols in a slot of an IAB-node DU serving cell can be configured to be of hard, soft, or unavailable type by IAB-DU-Resource-Configuration-H/S/NA-Config. When a downlink, uplink, or flexible symbol is configured as hard, the IAB-node DU serving cell can respectively transmit, receive, or either transmit or receive in the symbol. ## <Unchanged text is omitted> For each serving cell of an IAB-node DU in a set of serving cells of the IAB-node DU, the IAB-node DU can be provided: - an identity of the IAB-node DU serving cell by iabDuCellId-AI - a location of an availability indicator (AI) index field in DCI format 2_5 by positionInDCI-AI - a set of availability combinations by availabilityCombinations, where each availability combination in the set of availability combinations includes - resourceAvailability indicating availability of soft symbols in one or more slots for the IAB-node DU serving cell, and - a mapping for the soft symbol availability combinations provided by AvailabilityCombination to a corresponding AI index field value in DCI format 2_5 provided by availabilityCombinationId ## <Unchanged text is omitted>

Proposal 2: Adopt the following text proposal to correct to resource configuration for DU:

- Correcting Table 14.2 to reflect the agreed entries (in Red).

TP to TS 38.213, Sec. 14: ## <Unchanged text is omitted> An AI index field value in a DCI format 2_5 indicates to an IAB-node DU a soft symbol availability in each slot for a number of slots for each DL BWP or each UL BWP starting from a slot where the IAB-node detects the DCI format 2_5. The number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 2_5 as provided by SearchSpace-IAB. The AI index field includes bits where maxAIindex is the maximum of the values provided by corresponding availabilityCombinationId. An availability for a soft symbol in a slot is identified by a corresponding value resourceAvailability as provided in Table 14.2. Table 14.2: Mapping between values of resourceAvailability elements and types of soft symbol availability in a slot 0; No indication of availability for soft symbols are indicated available. 1; Only DL soft symbols are indicated available No indication of availability for UL and Flexible soft symbols. 2; Only UL soft symbols are indicated available No indication of availability for DL and Flexible soft symbols. 3; Only DL and UL soft symbols are indicated available No indication of availability for Flexible soft symbols. 4; Only Flexible soft symbols are indicated available No indication of availability for DL and UL soft symbols. 5; Only DL and Flexible soft symbols are indicated available No indication of availability for UL soft symbols. 6; Only UL and Flexible soft symbols are indicated available No indication of availability for DL soft symbols. 7; DL, UL, and Flexible soft symbols are indicated available. If a PDCCH monitoring periodicity for DCI format 2_5 is smaller than a duration of an availability combination of soft symbols over a number of slots that the UE obtains at a PDCCH monitoring occasion for DCI format 2_5 by a corresponding AI index field value, and the UE detects more than one DCI formats 2_5 indicating an availability combination of soft symbols in a slot, the UE expects that each of the more than one DCI formats 2_5 indicates a same value for the availability combination of the soft symbols in the slot. Adopt the following regarding the IAB behaviour as it relates to soft DU resources: The IAB node may use a soft DU resource for transmission and/or reception in line with D/U/F configuration of the resource, if and only if - the use of the resource does not change the IAB-MT behavior relative to the IAB-MT behavior if the resource was instead configured as unavailable type or - the IAB node has received a DCI with DCI format 4_0 indicating that the resource is available ## <Unchanged text is omitted>

Proposal 3: Adopt the following text proposal to correct to resource configuration for DU:

- simplifying the text describing the use of the soft resource at the IAB-node DU (in Red).

TP to TS 38.213, Sec. 14: ## <Unchanged text is omitted> When a downlink, uplink, or flexible symbol is configured as soft, the IAB-node DU can respectively transmit, receive or either transmit or receive in the symbol only if - for the IAB-node MT, ability to transmit or receive by the IAB-node DU in the soft symbol is not changed due to the use of equivalent to a configuration of the soft symbol as unavailable, or - the IAB-node DU detects a DCI format 2_5 with an AI index field value indicating the soft symbol as available for transmission or reception ## <Unchanged text is omitted>

# References

[1]     RP-193123, “Introduction of integrated access and backhaul” CR TS 38.213.