3GPP TSG-RAN WG1 #100-e		R1-2000888

e-Meeting, February 24th – March 6th, 2020

Source:	Ericsson

Title:	Remaining issues for NR-DC UL Power Control

Agenda Item:	7.2.10.1

Document for:	Discussion and Decision

# Introduction

In this document we discuss remaining issues related specification of uplink power control for NR-DC and propose some corrections to TS 38.213.

# Discussion

The following was agreed for semi-static power sharing

## Semi-static Power Sharing

Agreements: Slide 3 of R1-1909864 is agreed:

- [Offline consensus] Considering the following two alternatives for semi-static power sharing with PMCG+ PSCG≤PTotalNN- DC

    - Alt.1: For the uplink transmission in MCG, the UE checks the semi-statically configured direction of the overlapping symbols of all serving cells of SCG, and vice versa.

      - If such overlapping with UL transmission on the SCG is possible (i.e. collides with semi-static ‘UL’ and ‘flexible’ symbols on some CCs of SCG), UE limits its actual transmission power pMCGin MCG such that pMCG≤PMCG;

      - Otherwise (i.e. collides with only semi-static ‘DL’ symbols on all CCs of SCG), pMCG can be up to PMCG' and  pSCG can be up to  PSCG'  .

        - Alt.1-1:  PMCG' and PSCG'  are configured by RRC signaling.

        - Alt.1-2:  PMCG' and PSCG'  are determined by RAN4 requirement.

    - Alt.2: For the uplink transmission in MCG and in SCG, UE limits its actual transmission power pMCG to be up to PMCGand  pSCG to be up to PSCG

Agreements:

- Adopt Alt.1-2 and Alt.2 for semi-static power sharing for NR-NR DC.

  - Alt.1-2 is only subject to configured maximum transmission power defined by RAN4

  - Configuration between Alt.1-2 and Alt.2 is supported.

  - FFS: add more clarification

  - FFS: applied for synchronous DC only or applied for both synchronous and asynchronous DC (which may be the same or different for Alt.1-2 and Alt. 2)

However, in current specification text, the condition PMCG+ PSCG≤PTotalNN- DC is missing. This can be corrected using the TP below

Proposal 1

- Agree to Text proposal 1 below for semi-static power sharing

---------------------------- start TP1 to sub clause 7.6.2 of 38.213v16.0.0 ---------------------------------

7.6.2	NR-DC

If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5.

If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2 and with an inter-CG power sharing mode by NR-DC-PC-mode for FR1 and/or by NR-DC-PC-mode for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range.

If a UE is provided NR-DC-PC-mode = Semi-static-mode1 or  NR-DC-PC-mode = Semi-static-mode2, the UE does not expect PMCG  and  PSCG to be configured such that PMCG+PSCG>PTotalNR-DC

If a UE is provided NR-DC-PC-mode = Semi-static-mode1, the UE determines a transmission power for the MCG or for the SCG as described in Clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively.

If a UE is provided NR-DC-PC-mode = Semi-static-mode2

-	if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission in slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG in slot i2 as described in Clauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power

-	otherwise, the UE determines a transmission power for the MCG or the SCG, as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively

The UE expects to be provided NR-DC-PC-mode = Semi-static-mode2 only for synchronous NR-DC operation [10, TS 38.133].

<.unchanged text omitted >

---------------------------- end TP1 -----------------------------------------------------------------------------------

## Dynamic Power Sharing

The following was agreed in RAN1#99 regarding dynamic power sharing

Agreements:

- For NR-DC dynamic power sharing, to compute the transmit power for SCG UL transmission starting at time T0,

- UE checks for PDCCH(s) received before time T0-T_offset that trigger an overlapping MCG UL transmission, and

  - If such PDCCH(s) are detected, UE sets it’s transmit power in SCG (pwr_SCG) such that pwr_SCG <= min{PSCG, Ptotal – MCG tx power} where ‘MCG tx power’ is the actual transmission power of MCG

  - Otherwise, pwr_SCG <= Ptotal;

- UE does not expect to be scheduled by PDCCH(s) received on MCG after T0-[T_offset] that trigger(s) MCG UL transmission(s) that overlaps with the SCG transmission.

  - (working assumption) No new RRC signaling is introduced for T_offset:

    - Alt.1: T_offset <= T_proc,2

    - Alt.2: T_offset <= 2*T_proc,2

    - Alt.3: T_offset reasonbly larger than Alt 1. & Alt 2 but <=4ms

    - To be addressed in the CR stage

Regarding T_offset, for a UE configured with dynamic power sharing, the gNB is essentially constrained to set k2, k1 values for MCG transmissions such that they are not smaller than  T_offset. This negatively impacts latency achievable via NR-DC. Considering this, a small value should be used for this parameter at least for synchronous NR-DC case. For async NR-DC, a larger value (e.g. 0.5ms+ value for sync case) can be considered to account for the possible timing difference between MCG and SCG. Also, as discussed in RAN1#99 [1], if T_offset is made a UE capability to account for different UE implementations, it is preferable from NW perspective to provide the option of configuring one offset to all the UEs in a cell via RRC. Hence the WA on no RRC signalling for T_offset should be revisited if it is made a UE capability.

Considering the above, we propose T_offset = N2 + 4syms (i.e., outer limit of T_proc,2). N2 is as defined in sub clause 6.4 of 38.214, for the CC with largest SCS configured on the MCG. Alternately, a scaled value of N2 (e.g. 1.25*N2) can be used.

Proposal 2

- For dynamic power sharing

  - At least for sync NR-DC case, T_offset = N2+4sym

  - For async NR-DC case, T_offset = 0.5ms+ N2+4sym

  - Note: For above two cases, N2 is as defined in sub clause 6.4 of 38.214, and numerology of the  CC with largest SCS configured on the MCG should be used.

  - In case a UE capability related to T_offset is introduced, also introduce new RRC signaling for the NW to configure appropriate T_offset for the UE.

# Conclusions

In this document, we discuss the framework for NR-NR DC power control and propose the following.

Proposal 1

- Agree to Text proposal 1 below for semi-static power sharing

---------------------------- start TP1 to sub clause 7.6.2 of 38.213v16.0.0 ---------------------------------

7.6.2	NR-DC

If a UE is configured with an MCG using NR radio access in FR1 or in FR2 and with a SCG using NR radio access in FR2 or in FR1, respectively, the UE performs transmission power control independently per cell group as described in Clauses 7.1 through 7.5.

If a UE is configured with an MCG and a SCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the MCG by p-NR-FR1 and/or by p-NR-FR2 and a maximum power PSCG for transmissions on the SCG by p-NR-FR1 and/or by p-NR-FR2 and with an inter-CG power sharing mode by NR-DC-PC-mode for FR1 and/or by NR-DC-PC-mode for FR2. The UE determines a transmission power on the MCG and a transmission power on the SCG per frequency range.

If a UE is provided NR-DC-PC-mode = Semi-static-mode1 or  NR-DC-PC-mode = Semi-static-mode2, the UE does not expect PMCG  and  PSCG to be configured such that PMCG+PSCG>PTotalNR-DC

If a UE is provided NR-DC-PC-mode = Semi-static-mode1, the UE determines a transmission power for the MCG or for the SCG as described in Clauses 7.1 through 7.5 using PMCG or PSCG as the maximum transmission power, respectively.

If a UE is provided NR-DC-PC-mode = Semi-static-mode2

-	if at least one symbol of slot i1 of the MCG or of the SCG that is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission in slot i2 of the SCG or of the MCG, respectively, the UE determines a power for the transmission on the SCG or the MCG in slot i2 as described in Clauses 7.1 through 7.5 using PSCG or PMCG, respectively, as the maximum transmission power

-	otherwise, the UE determines a transmission power for the MCG or the SCG, as described in [8-3, TS 38.101-3] and in Clauses 7.1 through 7.5 without considering PMCG or PSCG, respectively

The UE expects to be provided NR-DC-PC-mode = Semi-static-mode2 only for synchronous NR-DC operation [10, TS 38.133].

<.. unchanged text omitted >

---------------------------- end TP1 -----------------------------------------------------------------------------------

Proposal 2

- For dynamic power sharing

  - At least for sync NR-DC case, T_offset = N2+4sym

  - For async NR-DC case, T_offset = 0.5ms+ N2+4sym

  - Note: For above two cases, N2 is as defined in sub clause 6.4 of 38.214, and numerology of the CC with largest SCS configured on the MCG should be used.

  - In case a UE capability related to T_offset is introduced, also introduce new RRC signaling for the NW to configure appropriate T_offset for the UE.

# References

[1] R1-1912784 – “Power Control for NR-NR DC”, Ericsson, RAN1#99, Reno, USA, November 2019.