3GPP TSG RAN WG1 #100-e			R1-2000947

February 24th – March 6th, 2020

Title:	Draft reply LS on clarification of PUCCH configuration

Response to:

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Qualcomm, [RAN1]

To:	RAN2

CC:

Contact Person:

#### Name:	Yi Huang

Email Address:	yihuang@qti.qualcomm.com

Attachments:

1. Overall Description:

In LS R2-1916481, RAN2 mentioned the following restriction for PUCCH configuration for EN-DC.

In EN-DC, The NW configures at most one serving cell per frequency range with PUCCH. And in EN-DC, if two PUCCH groups are configured, the serving cells of the NR PUCCH group in FR2 use the same numerology.

Furthermore, in the same LS, RAN2 asked the following question to RAN1:

- “whether this restriction should be applied to NE-DC, NR-DC and NR standalone also”?

RAN1 would like to thank RAN2 for the question and list the response to the above question as below:

- For NE-DC, it is appropriate to use the same restriction as for EN-DC.

- For NR-DC, there is understood to be a much stricter restriction, i.e. there is no support of multiple PUCCH groups within a CG.

- For NR-CA, no restriction is needed, except that a similar UE capability structure as expected to be used for the cell group capability signalling in NR-DC is recommended. That is, for the signalling in the UE capabilities of which bands can form PUCCH groups for NR-CA, reuse the mechanism expected to be introduced in 38.331 for NR-DC indicating which subset of bands in a DC band combination can form cell groups supported by the UE.

2. Actions:

To RAN2: 	RAN1 respectfully requests RAN2 to take the above information into account.

3. Date of Next TSG-RAN WG1 Meetings:

RAN1#100bis		20-24 April 2020				             Busan, KR

RAN1#101		25-29 May 2020					Athens, GR