3GPP TSG RAN WG1 #100-e	R1-2001045

# e-Meeting, February 24th –March 6th, 2020

Agenda item:	7.2.4.5

Source: 	CAICT

Title: 	On PSFCH of NR Sidelink

Document for:	Discussion and Decision

- Introduction

In this contribution, we present our consideration on the remaining issues about PSFCH based on the agreements.

- Remaining issues for PSFCH

There are some critical FFS issues in determination of the PSFCH resources according to the agreements of RAN1#99 as follows:

- In determining PSFCH candidate resources for a PSFCH format from the starting sub-channel index and the slot index used for the corresponding PSSCH for actual transmission, - Notation - S: the number of sub-channels in a slot - N: the number of PSSCH slots associated with a single PSFCH slot - NF: the number of PRBs in the set (pre-)configured for the actual PSFCH resources. - Within the set of PRBs (pre-)configured for the actual PSFCH resources, the first Z PRBs are associated with the first sub-channel in the first slot associated with the PSFCH slot, the second Z PRBs are associated with the first sub-channel in the second slot associated with the PSFCH slot, and so on. - FFS when NF is not a multiple of S*N - For a PSSCH, the candidate PSFCH resource is the set of PRBs associated with - Option 1: the starting sub-channel and slot used for that PSSCH. - Option 2: the sub-channel(s) and slot used for that PSSCH - PSFCH TX/TX conflict case 2 and 3 will be discussed separately.

In the specification, the problem caused by NF is not a multiple of S*N has not been solved. And the candidate Ntype PSFCH⋅Msubch, slotPSFCH PSFCH PRBs based on the Option1 and Option2 has not been clarified.

For issue that NF is not a multiple of S*N, we think that the condition NF /(S*N)<1 and NF /(S*N)>1 while  not an integer shall be solved in the specification. NF /(S*N)<1 means that a number of sub-channels for PSSCH transmissions will map to one candidate PSFCH PRB.

Proposal1:  The condition NF /(S*N)<1 and NF /(S*N)>1 while not an integer shall be solved in the specification, where S is the number of sub-channels in a slot, N is the number of PSSCH slots associated with a single PSFCH slot, and NF is the number of PRBs in the set (pre-)configured for the actual PSFCH resources.

The text proposal for proposal1 is provided in the following:

Text proposal for Proposal 1: In 38.214[1] Section 16.3 describing UE procedure for reporting HARQ-ACK on sidelink, the affected text will be: A UE is provided, by rbSetPSFCH, a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission in a PRB of the resource pool. For a number of Nsubch sub-channels for the resource pool, provided by numSubchannel, and a number of NPSSCHPSFCH PSSCH slots associated with a PSFCH slot, provided by periodPSFCHresource, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH, i+1+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH-1 PRBs from the MPRB, setPSFCH PRBs to slot i and sub-channel j, where Msubch, slotPSFCH=MPRB, setPSFCHNsubch⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubch, and the allocation starts in an ascending order of i and continues in an ascending order of j. If MPRB, setPSFCHNsubch⋅NPSSCHPSFCH<1, then Msubch, slotPSFCH=1, and the UE allocates the i+j⋅NPSSCHPSFCH/Q th PRB to slot i and sub-channel . If MPRB, setPSFCHNsubch⋅NPSSCHPSFCH>1and it not an integer, then Msubch, slotPSFCH=(MPRB, setPSFCH)Nsubch⋅NPSSCHPSFCH.

For issue that the candidate PSFCH resource is the set of PRBs associated with Option 1(the starting sub-channel and slot used for that PSSCH) or Option 2 (the sub-channel(s) and slot used for that PSSCH). We suggest to use different options according to the cast mode. Since that in the groupcast mode with ACK/NACK feedback, the number of candidate PSFCH PRBs might not enough to allocate each member UE one PSFCH resources for ACK and NACK feedback. In this case, the candidate PSFCH resource is the set of PRBs associated with Option 2 is preferred. In addition, the factor Ntype PSFCH to determine the number of candidate PSFCH PRBs can be configured from high layer to achieve more flexibility.

Proposal2: It is suggest that he factor Ntype PSFCH is determined by the cast mode or configured from high layer to achieve more flexibility.

The text proposal for proposal2 is provided in the following:

Text proposal for Proposal 2: In 38.213[1] Section 16.3 describing UE procedure for reporting HARQ-ACK on sidelink, the affected text will be: A UE determines a number of PSFCH resources available for multiplexing HARQ-ACK information in a PSFCH transmission as RPRB, CSPSFCH=Ntype PSFCH⋅Msubch, slotPSFCH⋅NCSPSFCH where NCSPSFCH is a number of cyclic shift pairs for the resource pool and, based on an indication by higher layers, - Ntype PSFCH=1 and the Msubch, slotPSFCH PRBs are determined by the slot index and starting sub-channel index of the PSSCH transmission. The case is used for unicast scenario and groupcast scenario with NACK only feedback. - Ntype PSFCH=Nsubch PSSCH , where Nsubch PSSCH is the number of sub-channels of the PSSCH occupied and the Nsubch PSSCH⋅Msubch, slotPSFCH PRBs are determined by the slot index and sub-channels index of the PSSCH transmission. The case is used for groupcast scenario with ACK and NACK feedback. If a value of Ntype PSFCH is provided from high layer, the Ntype PSFCH is set to be the value from high layer. And the candidate PSFCH PRBs are associate with the indexes of first Ntype PSFCH sub-channels and the slot index of the PSSCH transmission. Nsubch PSSCH

In addition, the PSFCH transmission can be enabled or disabled. If a UE receives an indication to enable or disable PSFCH transmission, the timing to enable or disable PSFCH transmission is not clarified in the specification, especially in the case that the PSFCH transmission disable indication is between a PSSCH transmission and the corresponding PSFCH transmission. If the receiver UE stops the HARQ feedback right after receiving the HARQ feedback disable indication, then the detection result of the already transmitted PSSCH will not be known at the transmitter UE and its reliability cannot be ensured. In general, the reliability of PSSCH transmission after the HARQ feedback disable indication can be ensured through repetition or MCS adjustment. The reliability of PSSCH transmission before the HARQ feedback disable indication however depends on the HARQ feedback. Therefore, it shall clarify whether the HARQ feedback of PSSCHs before the HARQ feedback disable indication needs to be transmitted or not. Similarly, as for the HARQ feedback enabling, it shall further study whether the HARQ feedback of PSSCHs before the HARQ feedback enable indication needs to be transmitted or not.

Proposal3: Clarify the timing of enable and disable PSFCH transmission.

The text proposal for proposal3 is provided in the following:

Text proposal for Proposal3: In 38.213[1] Section 16.3 describing UE procedure for reporting HARQ-ACK on sidelink, the affected text will be: A UE may be indicated by higher layers to not transmit a PSFCH in response to a PSSCH reception [11, TS 38.321]. After receiving a PSFCH disable indication, a UE starts to disable the PSFCH transmission in the first slot configured with PSFCH resource in which no PSFCH transmission is associated with the PSSCH transmissions before the PSFCH disable indication. A UE may be indicated by higher layers to enable the PSFCH transmission in response to a PSSCH reception [11, TS 38.321] after the PSFCH disable indication. A UE starts to enable the PSFCH transmission in the first slot configured with PSFCH resource in which a PSFCH transmission is associated with a PSSCH transmission after the PSFCH enable indication.

- Conclusions

In this contribution, we have present our considerations on the issues for maintenance of PSFCH of NR sidelink and put forward the following proposes:

Proposal1:  The condition NF /(S*N)<1 and NF /(S*N)>1 while not an integer shall be solved in the specification, where S is the number of sub-channels in a slot, N is the number of PSSCH slots associated with a single PSFCH slot, and NF is the number of PRBs in the set (pre-)configured for the actual PSFCH resources.

Proposal2: It is suggest that he factor Ntype PSFCH is determined by the cast mode or configured from high layer to achieve more flexibility.

Proposal3: Clarify the timing of enable and disable PSFCH transmission.

References

TS 38.213, “NR; Physical layer procedures for control,” v16.0.0.

R1-1913030, Considerations on procedures of NR Sidelink, CAICT, Nov. 2019.