Source: 	MCC Support

Title:	Final Report of 3GPP TSG RAN WG1 #100-e v2.0.0

(Online meeting, 24th February – 6th March 2020)

Document for:	Approval

Table of contents

1	Opening of the meeting	6

1.1	Call for IPR	6

1.2	Competition law statement	6

1.3	Network usage conditions	6

1.4	Check-in for Registered Delegates	7

1.5	Aspects related to RAN1 Meeting Management	7

2	Approval of Agenda	7

3	Highlights from RAN plenary	7

4	Approval of Minutes from previous meeting	7

5	Incoming Liaison Statements	7

5.1	RAN1 Aspects for RF requirements for NR frequency range 1 (FR1)	10

5.2	RAN1 Aspects for RF Requirement Enhancements for FR2	12

6	E-UTRA	12

6.1	Maintenance of E-UTRA Releases 8 – 15	12

6.2	Maintenance of E-UTRA Release 16	13

6.2.1	Maintenance of Additional MTC Enhancements	14

6.2.1.1	UE-group wake-up signal	14

6.2.1.2	Support for transmission in preconfigured UL resources	15

6.2.1.3	Scheduling of multiple DL/UL transport blocks	17

6.2.1.4	Coexistence of LTE-MTC with NR	18

6.2.1.5	Support of quality report in Msg3 and Connected Mode	18

6.2.1.6	MPDCCH performance improvement	18

6.2.1.7	CE mode A and B improvements for non-BL UEs	19

6.2.1.8	Use of LTE Control Channel Region for DL Transmission	21

6.2.1.9	Use of RSS for measurement improvements	21

6.2.1.10	Other	22

6.2.2	Maintenance of Additional Enhancements for NB-IoT	22

6.2.2.1	UE-group wake-up signal	22

6.2.2.2	Support for transmission in preconfigured UL resources	23

6.2.2.3	Scheduling of multiple DL/UL transport blocks	24

6.2.2.4	Coexistence of NB-IoT with NR	25

6.2.2.5	Support of Quality report in Msg3 and Connected Mode	26

6.2.2.6	Presence of NRS on a non-anchor carrier for paging	26

6.2.2.7	Other	26

6.2.3	Maintenance of DL MIMO efficiency enhancements for LTE	26

6.2.3.1	Support of enhancing SRS capacity and coverage	26

6.2.3.1.1	Additional SRS symbols	26

6.2.3.1.2	Other	27

6.2.3.2	Other	27

6.2.4	Maintenance of LTE-based 5G Terrestrial Broadcast	27

6.2.4.1	New numerology(ies) for PMCH for support of rooftop reception	27

6.2.4.2	Necessity and detailed enhancements to the physical channels and signals in the CAS	27

6.2.4.3	New numerology for support of mobility of up to 250km/h	27

6.2.4.4	Other	27

6.2.5	LTE Rel-16 UE Features	27

6.2.6	Other	28

7	NR Maintenance	28

7.1	Maintenance of Release 15 NR	28

7.1.1	Maintenance for Initial access and mobility	28

7.1.2	Maintenance for MIMO	28

7.1.3	Maintenance for Scheduling/HARQ aspects	28

7.1.4	Maintenance for NR-LTE co-existence	29

7.1.5	Maintenance for UL power control	29

7.1.6	Other	29

7.2	Maintenance of Release 16 NR	29

7.2.1	Maintenance of Two step RACH for NR	31

7.2.1.1	Channel Structure for Two-Step RACH	31

7.2.1.2	Procedure for Two-step RACH	32

7.2.1.3	Other	33

7.2.2	Maintenance of NR-based Access to Unlicensed Spectrum	33

7.2.2.1	Physical Layer Signals and Channels	33

7.2.2.1.1	Initial access signals/channels	33

7.2.2.1.2	DL signals and channels	34

7.2.2.1.3	UL signals and channels	38

7.2.2.2	Physical Layer Procedure	39

7.2.2.2.1	Channel access procedures	39

7.2.2.2.2	Enhancements to initial access procedure	40

7.2.2.2.3	HARQ enhancement	43

7.2.2.2.4	Configured grant enhancement	44

7.2.2.2.5	Wide-band operation	46

7.2.2.3	Other	48

7.2.3	Maintenance of Integrated Access and Backhaul for NR	48

7.2.3.1	Extensions of SSBs for inter-IAB-node discovery and measurements	48

7.2.3.2	Extensions of RACH occasions and periodicities for backhaul RACH resources	48

7.2.3.3	Mechanisms for resource multiplexing among backhaul and access links	48

7.2.3.4	Mechanism to support the “case-1” OTA timing alignment	49

7.2.3.5	Other	49

7.2.4	Maintenance for 5G V2X with NR sidelink	49

7.2.4.1	Physical layer structure for sidelink	49

7.2.4.2	Resource allocation for NR sidelink	51

7.2.4.2.1	Mode 1	51

7.2.4.2.2	Mode 2	53

7.2.4.2.3	Other	56

7.2.4.3	Sidelink synchronization mechanism	56

7.2.4.4	In-device coexistence between LTE and NR sidelinks	58

7.2.4.5	Physical layer procedures for sidelink	58

7.2.4.6	QoS management for sidelink	60

7.2.4.7	Support of NR Uu controlling LTE sidelink	62

7.2.4.8	Other	63

7.2.5	Maintenance of Physical Layer Enhancements for NR URLLC	63

7.2.5.1	PDCCH enhancements	63

7.2.5.2	UCI enhancements	66

7.2.5.3	PUSCH enhancements	67

7.2.5.4	Enhancements to scheduling/HARQ	69

7.2.5.5	Enhanced inter UE Tx prioritization/multiplexing	69

7.2.5.6	Enhanced UL configured grant transmission	71

7.2.5.7	Other	72

7.2.6	Maintenance of Enhancements on MIMO for NR	74

7.2.6.1	CSI Enhancement for MU-MIMO Support	74

7.2.6.2	Enhancements on Multi-TRP/Panel Transmission	75

7.2.6.3	Enhancements on Multi-beam Operation	76

7.2.6.4	Full TX Power UL transmission	81

7.2.6.5	Low PAPR RS	82

7.2.6.6	Other	83

7.2.7	Maintenance of UE Power Saving for NR	84

7.2.7.1	PDCCH-based power saving signal/channel	84

7.2.7.2	Procedure of cross-slot scheduling power saving techniques	85

7.2.7.3	UE adaptation to maximum number of MIMO layers	86

7.2.7.4	Other	86

7.2.8	Maintenance of NR positioning support	86

7.2.8.1	DL Reference Signals for NR Positioning	86

7.2.8.2	UL Reference Signals for NR Positioning	87

7.2.8.3	UE and gNB measurements for NR Positioning	88

7.2.8.4	Physical-layer procedures to support UE/gNB measurements	89

7.2.8.5	Other	89

7.2.9	Maintenance of NR Mobility Enhancements	89

7.2.9.1	Physical Layer Aspects for Mobility Enhancements during HO and SCG Change	90

7.2.9.2	Other	91

7.2.10	Maintenance of Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR)	91

7.2.10.1	Uplink Power Control for Supporting NR-NR Dual-Connectivity	91

7.2.10.2	Potential Enhancements to Single Tx Switched Uplink Solution for EN-DC	91

7.2.10.3	Support of efficient and low latency serving cell configuration/activation/setup	92

7.2.10.4	Support of aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH	93

7.2.10.5	Support of unaligned frame boundary with slot alignment and partial SFN alignment for R16 NR inter-band CA	94

7.2.10.6	Support of cross-carrier scheduling with different numerology	94

7.2.10.7	Other	95

7.2.11	NR Rel-16 UE Features	95

7.2.12	Other	95

8	Release 17	96

9	Closing of the meeting	96

Annex A:	List of Tdocs at RAN1 #100-e	97

Annex B:	List of CRs agreed at RAN1 #100-e	98

Annex C-1:	List of Outgoing LSs from RAN1 #100-e	102

Annex C-2:	List of Incoming LSs from RAN1 #100-e	104

Annex D:	List of Approved updated WIDs	106

Annex E:	List of draft TSs/TRs agreed at RAN1 #100-e	107

Annex F:	List of actions	108

Annex G:	List of participants at RAN1 #100-e	109

Annex H:	TSG RAN WG1 meetings in 2020 – 2021	110

Main facts summary

Due to the worldwide situation with regard to the Coronavirus (COVID-19) outbreak, 3GPP TSG WG RAN1#100 meeting, initially to be hosted by EF3 in Athens (Greece), was cancelled and replaced by an e-meeting (email discussions only).

The e-meeting was triggered by RAN1 chair's email at 00:04:51 UTC on Mon, 24 Feb 2020 and was officially closed at 21:48:59 UTC on Wed, 11 Mar 2020.

The number of attending delegates, is based on the number of registered attendees, namely 367.

The list of action points that require RAN1 post meeting follow-up is listed in Annex F (end of document).

The number of contributions for this meeting was 1276.

Note: The amount of documents includes those discussed during the email discussion session post meeting.

|  | Contribution is agreed/approved |
| --- | --- |
|  | Contribution is not pursued (no consensus) |
|  | Further discussion is required – come-back needed |
|  | Working assumption |
|  | Specific action needed from MCC |
|  | Contribution is for email discussion/approval |

# Opening of the meeting

Mr Wanshi Chen (RAN1 Chairman) kicked off the RAN WG1 #100-e meeting by email posted on Feb. 24th to RAN1 email exploder.

RAN1 chair asked all participants to read carefully the different statements made under agenda 1, as if in a physical meeting.

R1-1911417	Statement Regarding Engagement with Companies Added to the U.S. Export Administration Regulations (EAR) Entity List in 3GPP Activities	RAN1 Chair	(Revision of R1-1909504)

Decision: No comments raised by the group.

## Call for IPR

The attention of the delegates to the meeting of this Technical Specification Group was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of.

The delegates take note that they are hereby invited:

-	to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP.

-	to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms (e.g. see the ETSI IPR forms https://www.etsi.org/intellectual-property-rights/ipr ).

More information is available via the “Legal” page on the web site: https://www.3gpp.org/about-3gpp/legal-matters .

## Competition law statement

The Chairman also drew Member’s attention to the fact that 3GPP activities are subject to antitrust and competition laws and that compliance with said laws is therefore required of any participant of this WG meeting including the Chairman and Vice Chairmen. In case of question, please contact your legal counsel.

The present meeting will be conducted with strict impartiality and in the interests of 3GPP.

Furthermore, the Chairman reminded Members that timely submission of work items/contributions in advance of WG meetings is important to allow for full and fair consideration of such matters.

## Network usage conditions

The PCG has laid down the following network usage conditions:

Users shall not use the network to engage in illegal activities. This includes activities such as copyright violation, hacking, espionage or any other activity that may be prohibited by local laws. Users shall not engage in non-work related activities that consume excessive bandwidth or cause significant degradation of the performance of the network. Since the network is a shared resource, users should exercise some basic etiquette when using the 3GPP network at a meeting. It is understood that high bandwidth applications such as downloading large files or video streaming might be required for business purposes, but delegates should be strongly discouraged in performing these activities for personal use. Downloading a movie or doing something in an interactive environment for personal use essentially wastes bandwidth that others need to make the meeting effective. The meeting chairman should remind end users that the network is a shared resource; the more one user grabs, the less there is for another. Email and its attachments already take up significant bandwidth (certain email programs are not very bandwidth efficient). In case of need the chair can ask the delegates to restrict IT usage to things that are essential for the meeting itself. 1. DON’T place your WiFi device in ad-hoc mode 2. DON’T set up a personal hotspot in the meeting room 3. DO try 802.11a if your WiFi device supports it 4. DON’T manually allocate an IP address 5. DON’T be a bandwidth hog by streaming video, playing online games, or downloading huge files 6. DON’T use packet probing software which clogs the local network (e.g., packet sniffers or port scanners)

## Check-in for Registered Delegates

The attention of the delegates to this meeting was drawn to the fact that it is not permitted to check in other delegates on their behalf. In the event of technical difficulties preventing check-in, delegates should present themselves in person to the Secretary.

## Aspects related to RAN1 Meeting Management

Delegates are encouraged to check R1-1721392 for some thoughts on RAN1 meeting management.

# Approval of Agenda

R1-2000150	Draft Agenda of RAN1#100-e meeting	RAN1 Chair

The document was posted to RAN1 email exploder on Feb. 5th by RAN1 chairman.

- Preparation phase of the e-meeting (Feb. 17th – Feb. 21st):

  - Feature leads to summarize essential/critical issues for each sub-agenda by the end of Feb. 17th

  - From Feb. 18th to Feb. 21st: email discussion to finalize the set of critical issues for each sub-agenda

- e-meeting: email discussion/approval phase (Feb. 24th – March 6th)

  - Feb. 24th – March 3rd: email discussion/approvals for the identified critical issues

    - For Rel-15, individual CRs

    - For Rel-16, the endorsed TPs

  - March 4th – 6th: email approvals of per-WI per-spec Rel-16 editors’ CRs

Decision: The agenda is approved as no objections have been expressed by 2/25/2020.

R1-2000390	RAN1#100-e Meeting Timelines, Scope, Process	RAN1 Chair, MCC

The document was posted to RAN1 email exploder on Feb. 10th by Patrick Merias from MCC (ETSI Mobile Competence Center) and provides guidelines/rules for the overall management of the e-meeting.

Decision: The document is noted for information.

Note: Given the number (180+) of email threads, the following sharing of tasks was proposed for the email thread management:

- Younsun Kim (RAN1 Vice chairman) with full decision power for the email threads below:

  - 7.1 CRs

  - Rel-16: NR MIMO, LTE MIMO, LTE eMTC/NB-IoT

- Havish Koorapaty (RAN1 Vice chairman) with full decision power for the email threads below:

  - 6.1 CRs

  - Rel-16: LTE EnTV, NR NR-U, NR Positioning, NR mobility management

- All other AIs (RAN1 Chairman).

Full decision power means no need for further endorsement of the conclusion/agreements made by Havish/Younsun in the respective email threads.

According to RAN1 chair's email posted on Feb. 26th, as no expressed concerns regarding having Younsun/Havish help for managing some email threads with full decision power, it is agreed to proceed accordingly.

# Highlights from RAN plenary

This section is void.

# Approval of Minutes from previous meeting

R1-2000151	Report of RAN1#99 meeting	ETSI MCC

The document was posted to RAN1 email exploder on Feb. 22nd by Patrick Merias from MCC (ETSI Mobile Competence Center) and provides the report from RAN1#99 meeting in Reno.

Decision: According to RAN1 chair's email posted on Feb. 26th, there are no comments to RAN1#99 report which is approved.

# Incoming Liaison Statements

R1-2001122	RAN1#100-e preparation phase on incoming LS	RAN1 Chair

R1-2001123	RAN1#100-e preparation phase output on incoming LS	RAN1 Chair

[100e-5LS-01] – Frank (Huawei)

Email approval of reply LS to R1-2000162 by 2/28

R1-2000162	LS to RAN1 for clarification of PUCCH configuration	RAN2, Huawei

R1-2000353	[DRAFT] Reply LS on Clarification of PUCCH Configuration	ZTE

R1-2000598	Discussion on PUCCH Configuration	Samsung

R1-2000947	Draft reply LS on clarification of PUCCH configuration	Qualcomm Incorporated

R1-2001116	Draft Reply LS for clarification of PUCCH configuration	Huawei

R1-2001481	Outcome of email thread [100e-5LS-01] Reply LS for clarification of PUCCH configuration	Huawei

Decision: As per email decision posted on feb.29th, final LS is approved in R1-2001306.

Note: The approved LS represents RAN1’s endorsement, and should be taken into account accordingly in future UE feature description.

[100e-5LS-02] – Xueming (vivo)

Email discussion regarding how to conclude RAN1’s understanding of the LS in R1-2000163 and one draft CR in R1-2000303 by 2/28

R1-2000163	LS on Uplink skipping	RAN2, vivo

R1-2000298	On the misalignment between RAN1 and RAN2 about UL skipping	vivo

R1-2000299	Drat reply LS on UL skipping	vivo

R1-2000303	Draft 38.214 CR on UL skipping	vivo

R1-2001293	Summary email discussion [100e-5LS-02]	vivo

Decision: As per email decision posted on Feb. 29th, final LS is approved in R1-2001376.

Note: As RAN1 spec should be corrected, corresponding CR will be discussed after the conclusion of case 2 (dynamic PUSCH skipping with overlapping CSI/HARQ-ACK on PUCCH) is reached.

[100e-5LS-03] Karri (Nokia)

Email approval of reply LS to R1-2000164 by 2/28

R1-2000164	LS on default codebook parameters	RAN2, Nokia

R1-2000246	Draft reply LS on default Type I codebook parameters	ZTE

R1-2000300	Draft Reply LS on  default codebook parameters	vivo

R1-2000551	DRAFT Reply LS on default codebook parameters	Nokia

R1-2000560	Discussion on default codebook parameters	OPPO

R1-2000601	Draft reply to RAN2 LS on default codebook parameters	Samsung

Decision: As per email decision posted on Feb. 29th, final LS is approved in R1-2001307.

[100e-5LS-04] – Claes (Ericsson)

Email approval of reply LS to R1-2000165 by 2/28

R1-2000165	LS on secondary DRX group	RAN2, Ericsson

R1-2000510	Draft reply LS on secondary DRX group	ZTE

R1-2001083	Draft LS response on secondary DRX group	Ericsson

R1-2001084	On secondary DRX group	Ericsson

R1-2000340	Other issues for UE power saving	vivo

Decision: As per email decision posted on Mar. 2nd, no consensus on how to reply the LS. Companies are encouraged to do more analysis before restart of the discussion aiming at some reply LS in the next meeting.

[100e-5LS-05] – Hyunsoo (LG Electronics)

Email approval of reply LS to R1-2000168 by 2/28

R1-2000168	LS on clarification of CLI resource configuration	RAN2, LG Electronics

R1-2000508	Draft reply LS on clarification of CLI resource configuration	ZTE

R1-2000600	[DRAFT] LS response to RAN2 LS on clarification of CLI resource configuration	Samsung

R1-2000650	Discussion on clarification of CLI resource configuration	LG Electronics

R1-2000948	Draft reply LS on CLI resource configuration	Qualcomm Incorporated

R1-2001108	draft Reply LS on clarification of CLI resource configuration	Huawei

Decision: As per email decision posted on Mar. 10th, final reply LS is approved .

R1-2001318	Summary of email discussion on [5LS-05] clarification of CLI resource configuration	LG Electronics

R1-2001319	Reply LS on clarification of CLI resource configuration	LG Electronics

[100e-5LS-06] – Zichao (vivo)

Email approval of reply LS related to R1-1908004 using R1-2000301 as a starting point by 2/28

R1-2000301	Draft Reply LS on NR V2X resource pool configuration and selection	vivo

Revised in R1-2001298.

Decision: As per email decision posted on feb.29th, final LS is approved in R1-2001304.

==================

According to R1-2001122:

// LTE related

R1-2000152	LS reply on PUR transmission for NB-IoT/eMTC	RAN2, Ericsson

Decision: No specific action to RAN1. The LS is noted with no subsequent email discussion.

R1-2000153	Feedback on RAN1 agreements on PUR	RAN2, Qualcomm

Decision: No specific action to RAN1. The LS is noted with no subsequent email discussion.

R1-2000154	Reply LS on support of high peak data rate for Cat-M1 Ues in HD-FDD	RAN2, Qualcomm

Decision: No specific action to RAN1. The LS is noted with no subsequent email discussion.

//NR related

R1-2000155	LS on CLI measurements UE capabilities	RAN2, Qualcomm

Decision: No specific action to RAN1. The LS is noted with no subsequent email discussion. To be discussed using the email thread for UE features after the e-Meeting.

R1-2000509	Draft reply LS on CL measurements UE capabilites	ZTE

R1-2000599	Discussion on CLI Measurements UE Capabilities	Samsung

R1-2000651	Discussion on UE capability for CLI-RSSI and SRS-RSRP measurement	LG Electronics

R1-2000795	Draft reply LS on CLI measurements UE capabilities	Nokia

R1-2000156	LS on consistent Uplink LBT failure detection mechanism	RAN2, Qualcomm

Decision: No specific action to RAN1. The LS is noted, no need to reply. To be discussed under NR-U maintenance (7.2.2.2.1).

R1-2000416	Discussion on the RAN2 LS on Consistent Uplink LBT Failure Detection Mechanism	Nokia, Nokia Shanghai Bell

R1-2000648	Discussion on consistent uplink LBT failure detection mechanism for NR-U	LG Electronics

R1-2000157	Reply LS on signalling of sidelink RSRP and CSI	RAN2, LG Electronics

Decision: The LS is noted as already handled during RAN1#99 email discussion. No further action for now.

R1-2000158	Response LS on additional high layer information for sidelink physical layer operations	RAN2, LG Electronics

Decision: The LS is noted as already handled during RAN1#99 email discussion. No further action for now.

R1-2000159	Response LS on TX resource (re-)selection and MAC related agreements	RAN2, LG Electronics

Decision: The LS is noted as already handled during RAN1#99 email discussion. No further action for now.

R1-2000160	Reply LS on sidelink synchronization under multiple synchronization sources with different timing	RAN2, Qualcomm

Decision: The LS is noted as already handled during RAN1#99 email discussion. No further action for now.

R1-2000161	Reply LS on UL-SL prioritization	RAN2, OPPO

Decision: No specific action to RAN1. The LS is noted with no subsequent email discussion.

R1-2000788	[Draft] Reply LS on UL-SL prioritization	LG Electronics

R1-2000166	LS on RAN2 progress on Scell uplink behaviour of the UE in dormancy	RAN2, Intel

Decision: No specific action to RAN1. The LS is noted with no subsequent email discussion.

R1-2000649	Discussion on LS from RAN2 regarding SCell uplink behaviour of the UE in dormancy	LG Electronics

R1-2000811	Discussion on RAN2 LS on SCell uplink behaviour of the UE in dormancy	Futurewei

R1-2000167	Response LS on SL RLM/RLF	RAN2, InterDigital

Decision: No specific action to RAN1. The LS is noted with no subsequent email discussion.

R1-2000171	Reply LS on Tx DC location	RAN4, Huawei

Decision: No specific action to RAN1. The LS is noted with no subsequent email discussion.

R1-2000172	Response LS on Reference Point for Timing Related Measurements in FR2	RAN4, CATT

Decision: The LS is noted, no need to reply. Suggested changes should be included in 38.215 editor’s CR for email approval.

R1-2000173	Reply LS on applicable timing for pathloss RS activated/updated by MAC-CE	RAN4, Apple

Decision: The LS is noted, no need to reply. Any necessary RAN1 specification update can be done in future meetings.

R1-2000174	Reply LS to RAN2 on UL-SL Prioritization	RAN4, Futurewei

Decision: The LS is noted, no need to reply. Any necessary RAN1 specification update can be done in future meetings.

R1-2000178	LS on clarifying NR PC5 priority level	SA2, LG Electronics

Decision: The LS is noted, no need to reply. Any necessary RAN1 specification update can be done in future meetings.

R1-2000179	Reply LS on Response LS on SL RLM/RLF	SA2, Qualcomm

Decision: The LS is noted, no need to reply. Any necessary RAN1 specification update can be done in future meetings.

The following LSs are Cc to RAN1. They are all noted – no actions from RAN1 unless explicitly requested.

R1-2000169	LS on updates for TS 36.300 and TS 38.300	RAN2, Huawei

R1-2000170	Reply LS on clarification of OAM requirements for RIM	RAN3, Nokia

R1-2000175	Reply to LS on UE capabilities on DAPS HO	RAN4, Qualcomm Incorporated

R1-2000176	LS on UE capabilities and RRC signalling on Tx switching period delay	RAN4, Apple

R1-2000177	LS on channel raster for NR V2X UE	RAN4, CATT

Others

//Related to LS R1-1911551, NR-U

R1-2000202	Draft reply LS on contention window update in Initiating device channel access mechanism of EN301 893	Huawei

Decision: Postponed to RAN1#100b

//Related to LS R1-1909950, NR_SON_MDT

R1-2001096	Discussion on the feasibility of received interference power measurement in NR	Huawei, HiSilicon

R1-2001097	Draft Reply LS on the feasibility of received interference power measurement	Huawei

Decision: Postponed to RAN1#100b

==================

R1-2000414	Discussion on the RAN2 LS on Consistent Uplink LBT Failure Detection Mechanism	Nokia, Nokia Shanghai Bell

R1-2001219	Dicussion on switched UL impact in RAN1	Qualcomm Incorporated	(rev of R1-2000949)

The following LSs were received during RAN1#100-e meeting:

R1-2001236	LS to RAN1 on Support of 2-step CFRA	RAN2, ZTE

Decision: The LS is noted. Actions and potential reply will be handled after the e-Meeting.

R1-2001237	LS to RAN1 on NR-U PRACH root sequence for 2-step RA	RAN2, Ericsson

Decision: The LS is noted. Actions and potential reply will be handled after the e-Meeting.

## RAN1 Aspects for RF requirements for NR frequency range 1 (FR1)

Refer to RP-193266 for details regarding RAN4-led WID on RF requirements for NR frequency range 1 (FR1)

R1-2000882	Summary of Tx switching between two uplink carriers	China Telecommunications

Decision: The document is noted.

R1-2001204	Summary#2 of Tx switching between two uplink carriers	China Telecommunications

Decision: The document is noted.

[100e-5.1LS-TxSwitching-01] – Mihai (Nokia)

Email approval of 38.214 CR0065 (Rel-16, B) as in R1-2000792 by 2/25

R1-2000792	Introduction of switched uplink operation	Nokia, Nokia Shanghai Bell

Discussion: Many agreed that the CR is based on assumptions about future agreements and as such is not ready for RAN1 agreement.

Decision: No consensus for a stable and agreeable version, acc. to email posted on Feb. 27th. R1-2000792 is not pursued at this point.

[100e-5.1LS-TxSwitching-02] – Jianchi (China Telecom)

Email discussion/approvalon remaining issues on PUSCH preparation procedure (e.g., length of additional time, judgement condition on increment of time, etc.) and conclude by 02/28; if there is a spec impact, followed up by endorsing the corresponding TP by 3/3

R1-2001274	[100e-5.1LS-TxSwitching-02] Email discussion/approval on remaining issues on PUSCH preparation procedure	China Telecom

Decision: As per email decision posted on Mar. 2nd,

Agreement:

- If uplink Tx switching is triggered, the length of the additional time for PUSCH preparation procedure equals to the length of UL switching period.

Agreements:

- For SUL, if uplink Tx switching is configured, the state of Tx chains of last UL transmission is assumed in case of no UL transmission.

- Work assumption: For inter-band UL CA, if uplink Tx switching is configured, the state of Tx chains of last UL transmission is assumed in case of no UL transmission. It can be revisited in RAN1#100bis.

Clarify the following agreements with changes in red

Agreements:

- For standalone SUL, if UL switching period is configured by RRC

- The switching period is not always applicable on the carrier configured with switching period.

- The switching period is only applicable when the scheduled UL transmissions are switched between 1Tx carrier 1 and 2Tx carrier 2.

  - For each UL transmission occasion on a carrier, the existence of the switching period is determined one time every occasion.

  - Note: 2Tx carrier 2 refers to an UL carrier capable of 2 Tx chains and both 1-port and 2-port UL transmissions.

[100e-5.1LS-TxSwitching-03] – Jianchi (China Telecom)

Email discussion/approvalon remaining issues on inter-band UL CA, including:

- The related issue and solutions w.r.t. the condition of the presence of the switching period, including mapping between UL transmission ports and Tx chain, switching mechanism, UL phase, support of codebook/non-codebook based PUSCH transmission, etc.

- Clarification on handling of transmission collision between 1Tx transmission on carrier 1 and 2Tx transmission on carrier 2 for configured transmission.

and conclude by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001275	[100e-5.1LS-TxSwitching-03] Email discussion/approval on remaining issues on inter-band UL CA	China Telecom

Decision: As per email decision posted on Mar. 3rd,

Agreement:

- For inter-band UL CA, if uplink Tx switching is configured, UE is not expected to be scheduled or configured with UL transmissions that result in simultaneous 1Tx transmission on carrier 1 and 2Tx transmission on carrier 2.

Working assumption:

- For inter-band UL CA, if option 2 is supported, the following sub-option 2-3 is defined.

  - Minimize RAN1 impact

  - No new RAN4 impact

  - No new TDM pattern

- It can be revisited in future RAN1 meeting with taking into consideration any relevant RAN4 decisions on DL interruption for UL Tx switching.

- Option 2-3

|  | Number of Tx chains in WID (carrier 1 + carrier 2) | Number of antenna ports for UL transmission (carrier 1 + carrier 2) |
| --- | --- | --- |
| Case 1 | 1T+1T | 1P+0P, 1P+1P, 0P+1P |
| Case 2 | 0T+2T | 0P+2P, 0P+1P |

Agreements:

| Case 1 | 1 Tx on carrier 1 and 1 Tx on carrier 2 |
| --- | --- |
| Case 2 | 0 Tx on carrier 1 and 2 Tx on carrier 2 |

- For inter-band UL CA, if UL switching period is configured by RRC

  - The switching period is not always applicable on the carrier configured with switching period.

  - The switching period is at least applicable between 1-port transmission in carrier 1 and 2-port transmission in carrier 2.

Agreements:

- For inter-band UL CA, UE is not expected to [be scheduled or configured to] transmit on any of the two carriers in the switching period.

  - FFS: Whether to handle the case when switching period cannot be ensured by gNB.

[100e-5.1LS-TxSwitching-04] – Jianchi (China Telecom)

Email discussion/approvalon remaining issues on inter-band EN-DC without SUL, including

- The related issue and solutions w.r.t. the condition of the presence of the switching period, including mapping between UL transmission ports and Tx chain, TDM pattern, switching mechanism, etc.

- Handling of transmission collision between 1Tx transmission in LTE and 2Tx transmission in NR.

and conclude by 2/28; if there is a spec impact, followed by endorsing the correspoiding TP by 3/3

R1-2001276	[100e-5.1LS-TxSwitching-04] Email discussion/approval on remaining issues on inter-band EN-DC without SUL	China Telecom

Decision: As per email decision posted on Mar. 2nd, no consensus. Further discuss the relevant issues for EN-DC next meeting.

R1-2000302	Potential RAN1 impact to support TDM switching between the 1Tx and 2Tx UL carriers	vivo

R1-2001132	Remaining Issues on Support of Tx Switching between Two Uplink Carriers	ZTE	(rev. of R1-2000354)

R1-2000515	Remaining issues on UL switching	CATT

R1-2000552	On Tx switching between two uplink carriers	Nokia, Nokia Shanghai Bell

R1-2000597	Remaining issues on support of Tx switching between two uplink carriers	China Telecommunications

R1-2000883	RAN1 aspects of UL Tx switching	Ericsson

R1-2000940	Discussion on Tx Switching between Two Uplink Carriers	OPPO

R1-2001026	Discussion on the remaining issues of supporting Tx switching between two uplink carriers	Huawei, HiSilicon

## RAN1 Aspects for RF Requirement Enhancements for FR2

Refer to RP-192653 for details regarding RAN4-led WID on NR RF Requirement Enhancements for FR2

Note: although contributions are allowed, RAN1’s work is “reactive” as follows:

- RAN4 will provide further details on the RAN4 agreed solution(s) to RAN1/RAN2 before RAN1/RAN2 start their work if RAN1/RAN2 help is needed.

- This objective does not aim to propose the same alternatives which were not agreed (i.e. Alt1, Alt2 and Alt3 not agreed in RAN1#98 under Rel-16 NR eMIMO work item)

R1-2000245	Enhancement on FR2 MPE mitigation	ZTE

# E-UTRA

## Maintenance of E-UTRA Releases 8 – 15

R1-2001126	RAN1#100-e preparation phase on LTE Rel-15 (& earlier) CRs	Ad-hoc chair (Ericsson)

Decision: The document is noted.

R1-2001127	RAN1#100-e preparation phase output on LTE Rel-15 (& earlier) CRs	Ad-hoc chair (Ericsson)

Decision: The document is noted.

[100e-LTE-6.1CRs-A] – Havish (Ericsson)

Email discussion/approval of the following CRs by 2/28

R1-2000222	Corrections on the indication value for the transport blocks in a bundle for HD-FDD eMTC UEs	Huawei, HiSilicon

Decision: Final CRs are agreed in R1-2001330 (36.213, CR1304, Rel-14, F), R1-2001331 (36.213, CR1305, Rel-15, A) and in R1-2001332 (36.213, CR1306, Rel-16, A)

In addition, ZTE suggested adding a “specific mapping between the field value to the TB in a bundle" in 36.212.

Final CRs are agreed in R1-2001333 (36.212, CR0332, Rel-14, F), R1-2001334 (36.212, CR0333, Rel-15, A) and in R1-2001335 (36.212, CR0334, Rel-16, A)

R1-2000223	Correction on Msg3 NPUSCH for NB-IoT EDT	Huawei, HiSilicon

Decision: Final CR are agreed in R1-2001326 (36.213, CR1302, Rel-15, F) and R1-2001336 (36.213, CR1307, Rel-16, A)

R1-2000381	Clarification of UL Gap for Rel-13 eMTC	ZTE

R1-2001378	Clarification on MPDCCH monitoring during UL gaps	ZTE,Sanechips

Decision: Final CR are agreed in R1-2001378 (36.213, CR1308, Rel-13, F), R1-2001379 (36.213, CR1309, Rel-14, A), R1-2001380 (36.213, CR1310, Rel-15, A) , R1-2001381 (36.213, CR1311, Rel-16, A).

R1-2001110	Correction on Rel-15 NWUS/MWUS	Huawei, HiSilicon

Agreement:

The draft CR in R1-2001110 is agreed with a modification on the cover sheet and in the CR to list the clauses affected in ascending numerical order.

- Final CR are agreed in R1-2001327 (36.211, CR0508, Rel-15, F) and R1-2001337 (36.211, CR0509, Rel-16, A)

R1-2001112	Inclusion of NWUS, MWUS, and RSS in TS 36.201	Huawei, HiSilicon

Agreement:

The draft CR in R1-2001112 is agreed with a modification to change the occurrences of “wake up signal” to “wake-up signal”.

- Final CR is agreed in R1-2001328 (36.201, CR0030, Rel-15, F)

[100e-LTE-6.1CRs-B] – Havish (Ericsson)

Email discussion/approval of the following CRs by 2/28

R1-2000933	Correction on conditions for indicating Type 2 channel access procedure	WILUS Inc.

Decision: As per email decision posted on Feb. 29th, TP is endorsed for changes to TS37.213, Rel-15. Final CR is agreed in R1-2001305 (37.213, CR0006, Rel-15, F).

[100e-LTE-6.1CRs-C] – Havish (Ericsson)

Email discussion/approval of the following CRs by 2/28

R1-2000214	Correction on the resource block pair definition for 1.25 kHz subcarrier spacing	Huawei, HiSilicon

Agreement:

The draft CR in R1-2000214 is agreed.

- Final CRs are agreed in R1-2001321 (36.211, CR0505, Rel-14, F), R1-2001322 (36.211, CR0506, Rel-15, A) and in R1-2001323 (36.211, CR0507, Rel-16, A)

R1-2001113	Correction to measurements for inter-RAT handover	Huawei, HiSilicon

Agreement:

The draft CR in R1-2001113 is agreed.

- Final Cris agreed in R1-2001324 (36.201, CR0029, Rel-15, F)

R1-2000867	Correction on priority value in LTE V2X	Sharp

Decision: Final CRs are agreed in R1-2001224 (36.213, CR1300, Rel-14, F), R1-2001325 (36.213, CR1301, Rel-15, A) and in R1-2001329 (36.213, CR1303, Rel-16, A)

## Maintenance of E-UTRA Release 16

Only essential corrections

Only text proposals are allowed (no individual draft CRs, please!). Editors to prepare the consolidated CRs per spec.

Limit to maximum 1 contribution per 1 company/organization/university for any agenda item, including “Others”.

[100e-LTE-Editor-WICode-Spec#] – Editors

Editors’ CRs to consolidate all the endorsed TPs for approval by 3/6 -

- Per Rel-16 work item and per spec

- On the cover page, please list the email thread # for each endorsed TP incorporated

- If there is any editorial update not resulting from any email threads, please duly state them on the CR cover page

Decision: The following CRs are agreed:

LTE_eMTC5-Core

R1-2001427	Miscellaneous corrections for Rel-16 LTE-MTC features in 36.211	Ericsson		36.211	CR0510, F

R1-2001431	Miscellaneous corrections for Rel-16 eMTC features in 36.212	Futurewei	36.212	CR0335, F

R1-2001433	Corrections to Additional MTC Enhancements for LTE	Motorola Mobility	36.213	CR1312, F

NB_IOTenh3-Core

R1-2001428	Miscellaneous corrections for Rel-16 NB-IoT features in 36.211	Ericsson		36.211	CR0511, F

R1-2001432	Miscellaneous corrections for Rel-16 NB-IoT features in 36.212	Futurewei	36.212	CR0336, F

R1-2001434	Corrections to Additional enhancements for NB-IoT	Motorola Mobility	36.213	CR1313, F

LTE_terr_bcast-Core

R1-2001429	Corrections to LTE terrestrial broadcast	Ericsson		36.211	CR0512, F

LTE_DL_MIMO_EE-Core

R1-2001430	Corrections to DL MIMO efficiency enhancements for LTE	Ericsson		36.211	CR0513, F

R1-2001436	Corrections to DL MIMO efficiency enhancements for LTE	Motorola Mobility	36.213	CR1315, F

LTE_NR_DC_CA_enh-Core

R1-2001435	Corrections to single Tx switched uplink solution for EN-DC feature of Rel-16 MR DC/CA	Motorola Mobility		36.213	CR1314, F

[100e-LTE-Rel-16-RRC] – Fred (Qualcomm)

Updated RRC parameters consolidating new/updated RRC parameters (if any) for approval 3/6

- Any new RRC parameters are highly discouraged!

Decision: As per email decision posted on Mar. 10th, the latest draft of parameter list is endorsed.

Futurewei's comment after endorsement: terminology “cell-specific” should be replaced with “carrier-specific”

R1-2001477	Updated consolidated parameter list for Rel-16 LTE	Qualcomm

Decision: The updated list is endorsed. Inform RAN2 in LS R1-2001479 (see AI 7.2)

### Maintenance of Additional MTC Enhancements

#### UE-group wake-up signal

R1-2000815	Feature lead summary for UE-group WUS	Qualcomm Incorporated

R1-2001196	Updated feature lead summary for UE-group WUS	Qualcomm Incorporated

[100e-LTE-eMTC5-GroupWUS-01] – Le (Qualcomm)

Email discussion/approval on definition of 3-FDM WUS resource pattern by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001249	FL Summary of email discussion [100e-LTE-eMTC5-GroupWUS-01]	Qualcomm

Agreement

For 3-FDM WUS resources, the WUS resource ID mapping order is defined as

| freqLocation of WUS resource 0 | n0 | n2 | n4 |
| --- | --- | --- | --- |
| WUS resource locations if Rel-15 WUS is configured | f; 0. 1. 2; t. | f; 1. 0. 2; t. | f; 2. 1. 0; t. |

[100e-LTE-eMTC5-GroupWUS-02] – Le (Qualcomm)

Email discussion/approval on clarification of WUS resource freq location by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001250	FL Summary of email discussion [100e-LTE-eMTC5-GroupWUS-02]	Qualcomm

Agreement: Endorse the TP below on TS 36.211. TP to be included in 36.211 editor’s CR.

--------------------------------------------- Start of Text Proposal ---------------------------------------

------------------------------------------------- Text omitted ----------------------------------------------

6.11B.2	Mapping to resource elements

The same antenna port shall be used for all symbols of the MWUS within a subframe. The UE shall not assume that the MWUS is transmitted on the same antenna port as any of the downlink reference signals or synchronization signals. If only one CRS port is configured by the eNB, the UE may assume the transmission of all MWUS subframes is using the same antenna port; otherwise, the UE may assume the same antenna port is used for MWUS transmission in downlink subframes w0 + 2n and w0 + 2n + 1, where w0 is the first downlink subframe of the MWUS transmission as specified in [4], and n=0, 1.

The MWUS bandwidth is 2 consecutive PRBs, the frequency location of the lowermost PRB with is signaled by higher layers. For both PRB pairs in the frequency domain, for which MWUS is defined, the MWUS sequence shall be mapped to resource elements in sequence, starting with in increasing order of first the index , over the 12 assigned subcarriers and then the index in each subframe in which MWUS is transmitted.

------------------------------------------------- Text omitted ----------------------------------------------

--------------------------------------------- End of Text Proposal ---------------------------------------

[100e-LTE-eMTC5-GroupWUS-03] – Le (Qualcomm)

Email approval Typo of equation for scrambling phase by 2/25

--------------------------------------------- Start of Text Proposal ---------------------------------------

------------------------------------------------- Text omitted ----------------------------------------------

6.11B.1	Sequence generation

The MWUS sequence  in subframe x=0, 1, …, M-1 is defined by

wm= θnf, nsm'e-jπunn+1131ej2πgm132

m=0, 1, …, 131

m'=m+132x

n=m mod 132

θnf,nsm'=1 if cnf,ns2m'=0 and cnf,ns2m'+1=0 -1 if cnf,ns2m'=0 and cnf,ns2m'+1=1j if cnf,ns2m'=1 and cnf,ns2m'+1=0-1j if cnf,ns2m'=1 and cnf,ns2m'+1=1

u=NIDcell mod 126+3

where  is the actual duration of MWUS as defined in [4]. For a UE not configured with group MWUS, g=0. For a UE configured with group MWUS, g=14NgroupWUS+1 for 0≤NgroupWUS≤7, where NgroupWUS is determined by the UE group to which the UE is associated as determined by higher layers [10]. The common MWUS sequence shall be determined by  g=126 unless the resource is shared with non-group MWUS and common MWUS is configured to be non-group MWUS in which case g=0.

------------------------------------------------- Text omitted ----------------------------------------------

--------------------------------------------- End of Text Proposal ---------------------------------------

Decision: Above TP is endorsed, to be included in 36.211 editor's CR.

Furthermore, from email thread [100e-LTE-NB_IOTenh3-GroupWUS-02] regarding the total number of NWUS subframes, and decision posted on Mar. 3rd:

Agreement

The following text proposal for Sec. 17 in TS 36.213 is endorsed. TP to be included in 36.213 editor’s CR.

--------------------------------------------- Start of Text Proposal ---------------------------------------

A BL/CE UE using MWUS can assume the actual duration of MWUS, starting in subframe w0, is one of the values in the set listed in Table 17-1 corresponding to the maximum duration of MWUS, , configured by higher layers. There is a total of BL/CE DL subframes in the maximum duration of MWUS. The maximum duration of MWUS starts in subframe w0, where w0 is the latest subframe such that there are BL/CE DL subframes in the maximum duration and is a total of BL/CE DL subframes in the duration that ends in subframe (g0-1), where g0 is defined by [14], if FDM-only MWUS resource configuration [14], otherwise, and is the MWUS resource that the UE is associated to as defined in [3]. The UE may assume that MWUS and its first associated paging occasion subframes are in the same narrowband.

--------------------------------------------- End of Text Proposal ---------------------------------------

From email thread [100e-LTE-NB_IOTenh3-GroupWUS-01] regarding the detection of two WUS sequences, and decision posted on Mar. 3rd:

Agreement

The following text proposal is endorsed for Sec. 17 in TS 36.213. TP to be included in 36.213 editor’s CR.

--------------------------------------------- Start of Text Proposal ---------------------------------------

“A BL/CE UE can be configured with up to two MWUS [14]. A UE may assume that no more than one MWUS sequence is transmitted per MWUS resource at a given time.”

--------------------------------------------- End of Text Proposal ---------------------------------------

R1-2000227	Corrections on UE-group wake-up signal	Huawei, HiSilicon

R1-2000368	Clarification of group WUS for MTC	ZTE

R1-2000700	UE-group wake-up signal	Qualcomm Incorporated

R1-2001066	Clarification of UE-group wake-up signal for MTC	Nokia, Nokia Shanghai Bell

#### Support for transmission in preconfigured UL resources

R1-2001147	LTE-M Preconfigured UL Resources Summary RAN1 #100-e	Sierra Wireless

R1-2001199	LTE-M Preconfigured UL Resources Summary#2 RAN1 #100-e	Sierra Wireless

[100e-LTE-eMTC5-PUR-01] – Gus (Sierra Wireless)

Email discussion/approval on UL Gap for PUR by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: As per decision posted on Mar. 2nd,

Agreement: UL gaps are supported for PUR transmissions and retransmissions

- ue-CE-NeedULGap is reused for the enabling of UL Gap for PUR

The above agreement doesn't require any TP to RAN1 specs

[100e-LTE-eMTC5-PUR-02] – Gus (Sierra Wireless)

Email discussion/approval on missing starting RV text by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001347	TP on missing starting RV for the initial PUR transmission 	Sierra Wireless

Decision: As per decision posted on Mar. 2nd,

Agreement: The TP in R1-2001347 for TS36.213 is endorsed. TP to be included in 36.213 editor’s CR.

[100e-LTE-eMTC5-PUR-03] – Gus (Sierra Wireless)

Email discussion/approval on missing PUR PUSCH frequency hopping configuration text by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Agreement

1 RRC parameter is used for PUR PUSCH frequency hopping configuration

If pur-PUSCH-frequency-hopping =1

Initial PUR is frequency hopped

FH for retransmissions follows the FH bit in DCI.

Else

No hopping for initial or retransmissions

Agreement

The following text proposal for TS36.211 is endorsed. TP to be included in 36.211 editor’s CR.

--------------------------------------------- Start of Text Proposal ---------------------------------------

5.3.4	Mapping to physical resources

<Unchanged parts are omitted>

- For BL/CE UE in CEModeA,

- If PUSCH is transmitted using preconfigured uplink resources, PUSCH frequency hopping is enabled when the higher layer parameter pur-PUSCH-frequency-hopping is set, otherwise frequency hopping is disabled.

- Else if, PUSCH scheduled by DCI format 6-0A is associated with PUR C-RNTI, PUSCH frequency hopping is enabled when the higher layer parameter pur-PUSCH-frequency-hopping is set and the frequency hopping flag in DCI format 6-0A indicates frequency hopping, otherwise frequency hopping is disabled.

- Else, PUSCH frequency hopping is enabled when the higher-layer parameter pusch-HoppingConfig is set and the frequency hopping flag in DCI format 6-0A indicates frequency hopping, otherwise frequency hopping is disabled.

- For BL/CE UE in CEModeB,

- If PUSCH is transmitted using preconfigured uplink resources, PUSCH frequency hopping is enabled when the higher layer parameter pur-PUSCH-frequency-hopping is set, otherwise frequency hopping is disabled.

- Else if, PUSCH scheduled by DCI format 6-0B is associated with PUR C-RNTI, PUSCH frequency hopping is enabled when the higher layer parameter pur-PUSCH-frequency-hopping is set, otherwise frequency hopping is disabled.

- Else, PUSCH frequency hopping is enabled when the higher-layer parameter pusch-HoppingConfig is set, otherwise frequency hopping is disabled.

--------------------------------------------- End of Text Proposal ---------------------------------------

Agreement: The following changes to the RRC consolidated parameter list is agreed.

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect (see note) | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | TDD /FDD | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE_eMTC5-Core | PUR | 36.2113 |  |  |  | pur-DCI-CEmodeA-frequency-hopping-flag pur-PUSCH-frequency-hopping | Existing | PUR Frequency hopping flag in CE mode A | PUR DCI field for PUSCH frequency hopping flag in CE mode A. 1 bit, where value 0 indicates frequency hopping is not enabled and value 1 indicates frequency hopping is enabled configured Indicates the PUR PUSCH frequency hopping configuration as defined in subclause 5.3.4 of TS 36.211. | 1 bit |  |  | UE specific |  | 36.331 |  |

R1-2000219	Corrections on transmission in preconfigured UL resources	Huawei, HiSilicon

R1-2000369	Remaining issues on transmission in preconfigured UL resources for eMTC	ZTE

R1-2000506	LTE-M Pre-configured UL Resources Design Considerations	Sierra Wireless, S.A.

R1-2000701	Support for transmission in preconfigured UL resources	Qualcomm Incorporated

R1-2000907	Remaining issues on transmission in preconfigured UL resources for eMTC	NTT DOCOMO, INC.

R1-2001054	Corrections for Preconfigured UL resources for LTE-MTC	Ericsson

#### Scheduling of multiple DL/UL transport blocks

R1-2001056	Feature lead summary for Multi-TB scheduling for LTE-MTC	Ericsson

R1-2001185	Feature lead summary#2 for Multi-TB scheduling for LTE-MTC	Ericsson

R1-2001220	Feature lead summary#3 for Multi-TB scheduling for LTE-MTC	Ericsson

[100e-LTE-eMTC5-Multi-TB-01] – Johan (Ericsson)

Email discussion/approval on HARQ/NDI/RV/FH encoding for both FDD and TDD by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Conclusion

For FDD case:

- For 36.212, use Futurewei’s TP in R1-2001086 as a basis, possibly with the clarification “From MSB to LSB” in each section.

- For 36.211 and 36.213, take the provided comments and proposals into account in contributions to the next meeting.

For TDD case:

- There is no consensus in RAN1#100e for optimization (or elimination) of the TDD HARQ process grouping. The 36.212 seems adequate and potential corresponding 36.213 text can be added in the next meeting.

As per email decision posted on Mar. 4th, two companies prefer not to add “From MSB to LSB”, so:

Agreement: The text proposal in R1-2001086 is endorsed for inclusion into TS36.212 editor’s CR.

[100e-LTE-eMTC5-Multi-TB-02] – Johan (Ericsson)

Email discussion/approval on HARQ-ACK bundling for both FDD and TDD by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

As per email decision posted on Mar. 5th,:

Agreement: The TP provided in R1-2001214 for TS36.213 section 10.2 is endorsed. To be included as part of the editor’s CR for TS36.213.

[100e-LTE-eMTC5-Multi-TB-03] – Johan (Ericsson)

Email discussion/approval on scheduling gaps for both unicast and multicast by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Conclusion

For the unicast case

- There is no consensus in RAN1#100e for the proposal to specify explicit unicast scheduling gaps.

- Since unicast scheduling gaps are included in the draft RAN1 UE feature list, there may be a need to update the feature list, and this is something that can be brought up in the email discussion for the feature list.

For the multicast case

- There is no consensus in RAN1#100e for the proposal to insert the scheduling gaps before each TB instead of after each TB.

R1-2000220	Corrections on scheduling of multiple transport blocks	Huawei, HiSilicon

R1-2000370	Remaining issues on scheduling enhancement for MTC	ZTE

R1-2000507	LTE-M Multiple Transport Block Grant Design Considerations	Sierra Wireless, S.A.

R1-2000652	Remaining issues on multiple transport blocks scheduling in MTC	LG Electronics

R1-2001214	Scheduling of multiple DL/UL transport blocks	Qualcomm Incorporated	(rev of R1-2000702)

R1-2001055	Corrections for Multi-TB scheduling for LTE-MTC	Ericsson

R1-2001086	Remaining details on STUF for eMTC multiple TB scheduling	Futurewei

#### Coexistence of LTE-MTC with NR

R1-2001058	Feature lead summary for NR coexistence performance improvements for LTE-MTC	Ericsson

R1-2001186	Feature lead summary#2 for NR coexistence performance improvements for LTE-MTC	Ericsson

R1-2001221	Feature lead summary#3 for NR coexistence performance improvements for LTE-MTC	Ericsson

[100e-LTE-eMTC5-NR-coexistence-01] – Johan (Ericsson)

Email discussion/approval focusing on the following issues (refering to section 2 of R1-2001186):

- UE-specific resource reservation (including configuration aspects and Type0-CSS aspects)

- Clarification of handling of fully and partially reserved subframes

- Resource reservation in special subframes in TDD

- Definition of subcarrier puncturing

by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Mar. 4th,

Agreement

Issue #1: UE-specific resource reservation (including configuration aspects and Type0-CSS aspects)

- The 36.211 TPs in Section 2.1 in FL summary in R1-2001186 are endorsed. TP to be included in 36.211 editor’s CR.

- The 36.212 TPs in Section 2.1 in FL summary in R1-2001186 are endorsed with the following change: the words “given by C-RNTI or SPS C-RNTI” are removed in all four places. TP to be included in 36.212 editor’s CR.

- It should be possible to enable the resource reservation feature using UE-specific signaling, separately for DL and UL. Indicate this in the updated L1 parameter list. The details are up to RAN2.

Issue #2: Clarification of handling of fully and partially reserved subframes

- The 36.213 TP in Section 2 in FL summary in R1-2001221 is endorsed. TP to be included in 36.213 editor’s CR.

- The 36.211 TP in Section 3 in FL summary in R1-2001221 is endorsed. TP to be included in 36.211 editor’s CR.

Issue #4: Definition of subcarrier puncturing

- The 36.211 TP on definition of subcarrier puncturing in Section 2.4 in FL summary in R1-2001186 is endorsed. TP to be included in 36.211 editor’s CR.

R1-2000221	Corrections on eMTC co-existence with NR	Huawei, HiSilicon

R1-2000371	Remaining issues on LTE-MTC resource reservation	ZTE

R1-2000703	Coexistence of LTE-MTC with NR	Qualcomm Incorporated

R1-2001057	Corrections for NR coexistence performance improvements for LTE-MTC	Ericsson

#### Support of quality report in Msg3 and Connected Mode

None.

#### MPDCCH performance improvement

R1-2001020	Feature lead summary #1 on MPDCCH performance improvement	Huawei

R1-2001177	Feature lead summary #2 on MPDCCH performance improvement	Huawei

[100e-LTE-eMTC5-MPDCCH-improvement-01] – Yubo (Huawei)

Email discussion/approval on capturing of the mapping between DMRS and CRS ports and fallback candidates in 36.213 by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001277	Feature lead summary #1 on [100e-LTE-eMTC5-MPDCCH-improvement-01]	Huawei

Decision: As per decision posted on Mar. 2nd,

Agreement: The TP in R1-2001277 for TS36.213 is endorsed. TP to be included in 36.213 editor’s CR.

[100e-LTE-eMTC5-MPDCCH-improvement-02] – Yubo (Huawei)

Email discussion/approval on the TP for mapping between DMRS and CRS for a single CRS port by 2/27

R1-2001278	Feature lead summary #1 on [100e-LTE-eMTC5-MPDCCH-improvement-02]	Huawei

Decision: As per decision posted on Mar. 3rd,

Agreement

The following TP for TS36.211 section 6.8B.5 is endorsed. TP to be included in 36.211 editor’s CR.

--------------------------------------------- Start of Text Proposal ---------------------------------------

<Unchanged parts are omitted>

- If mpdcch-crs-idle-config or mpdcch-crs-config is configured by higher layers, the relation between the MPDCCH and CRS antenna ports is defined as follows:

- When one CRS port is configured by the eNB, the antenna port(s) used for MPDCCH transmission are equivalent to CRS port 0.

- For distributed transmission and when two CRS ports are configured by the eNB, the relation between the symbols transmitted on the antenna ports used for MPDCCH transmission and CRS ports 0 – 1 is defined by the precoder matrix for single-layer transmission in Table 6.3.4.2.3-1 using codebook index 0 for antenna port 107 and codebook index 1 for antenna port 109.

<Unchanged parts are omitted>

--------------------------------------------- End of Text Proposal ---------------------------------------

[100e-LTE-eMTC5-MPDCCH-improvement-03] – Yubo (Huawei)

Email discussion/approval on the TP for the issue of 2+4 MPDCCH PRB set by 2/27

R1-2001279	Feature lead summary #1 on [100e-LTE-eMTC5-MPDCCH-improvement-03]	Huawei

Decision: As per decision posted on Mar. 3rd,

Agreement

- For 2+4 MPDCCH PRB sets, perform precoder cycling based on the PRB index overall 6 PRBs

- The TP in R1-2001279 (section 3) for TS36.211 section 6.8B.5 is endorsed. TP to be included in 36.211 editor’s CR.

R1-2000372	Correction on MPDCCH performance improvement	ZTE

R1-2000704	MPDCCH performance improvement	Qualcomm Incorporated

R1-2001059	Corrections for MPDCCH performance improvement for LTE-MTC	Ericsson

#### CE mode A and B improvements for non-BL UEs

R1-2000654	FL summary of CE mode A and B improvements for non-BL UEs	LG Electronics

R1-2001205	FL summary#2 of CE mode A and B improvements for non-BL UEs	LG Electronics

R1-2001247	FL summary#3 of CE mode A and B improvements for non-BL UEs	LG Electronics

[100e-LTE-eMTC5-non-BL-UEs-01] – Jay (LGE)

Email discussion/approval on the TP related to no RI report in Submode 2 of the periodic CSI reporting Mode 1-1 by 2/27

Decision: As per decision posted on Feb. 28th, no consensus on whether specification change is needed or how such specification change should be done.

[100e-LTE-eMTC5-non-BL-UEs-02] – Jay (LGE)

Email discussion/approval on the TP related to related to the codebook indices to use for the feedback based on CSI-RS by 2/27

Decision: As per decision posted on Mar. 3rd,

Agreement: The following TPs are endorsed for TS36.213. TPs to be included in 36.213 editor’s CR.

- Text Proposal 5 in R1-2001247 for TS 36.213 subclause 7.2 to specify UE procedure to determine the precoder codebook subset for 8 antenna ports in TM 9 for CSI-RS based CSI feedback for non-BL UEs in CE mode A.

- Text Proposal 1 in R1-2001247 for TS36.213 subclause 7.2.2

Agreement: The following details on RRC parameter is agreed.

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect (see note) | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | TDD/FDD | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE_eMTC5-Core | CSI-RS-based feedback for non-BL UE | 36.213 |  |  |  | codebookSubsetRestriction | Existing | Codebook subset restriction for CSI-RS-based CSI feedback. | A UE is restricted to report PMI within a precoder codebook subset specified by a bitmap parameter codebookSubsetRestriction configured by higher layer signaling. | See comment field |  |  | UE specific |  | 36.331 | For a BL/CE UE, when reporting PMI the UE reports a single PMI report. A UE is restricted to report PMI within a precoder codebook subset specified by a bitmap parameter codebookSubsetRestriction configured by higher layer signaling. A Codebook subset restriction for 8Tx is used for CSI-RS-based CSI feedback. |

[100e-LTE-eMTC5-non-BL-UEs-03] – Jay (LGE)

Email discussion/approval on the TP on how the second PMI is calculated in Submode 1 of the periodic CSI reporting Mode 1-1 by 2/27

Decision: As per email decision posted on Feb. 28th,

Agreement: The following TP for TS36.213 subclause is endorsed. TP to be included in 36.213 editor’s CR.

7.2.2 Periodic CSI Reporting using PUCCH <Unchanged parts are omitted> For a BL/CE UE, theperiodic CSI reporting modes are described as following: - Wideband feedback - Mode 1-0 description: - In the subframe where CQI is reported: - A UE shall report a type 4 report consisting of one wideband CQI value which is calculated assuming transmission onall narrowband(s) in the CSI reference resource. The wideband CQI is calculated conditioned on transmission rank 1. - Mode 1-1 description: - In the subframe where a first PMI is reported for transmission modes 9 configured with submode 1 and 8 CSI-RS ports,the UE shall report a type 2a report consisting of a first PMI corresponding to a set of precoding matrices selected from the codebook subset assuming transmissionon all narrowband(s) in the CSI reference resource. The wideband first PMI valueis calculated conditioned on transmission rank 1. - In the subframe where wideband CQI/second PMI is reported for transmission modes 9 with 8 CSI-RS ports and submode 1: - A single precoding matrix is selected from the codebook subset assuming transmission on all narrowband(s) in the CSI reference resource.The PMIis calculated conditioned on transmission rank 1 and thewidebandfirst PMI. - A UE shall report a type 2b report consisting of - A single wideband CQI value which is calculated assuming the use of the single precoding matrix in all narrowband(s) inthe CSI reference resource and transmission on all narrowband(s) in the CSI reference resource.The wideband CQI is calculated conditioned on theselected precoding matrixand transmission rank 1. - Thewideband secondPMI corresponding to the selected single precoding matrix. - The wideband second PMIis calculated conditioned on transmission rank 1 and thelast reportedwideband first PMI. The wideband CQI is calculated conditioned on theselected precoding matrixand transmission rank 1. - In the subframe where wideband CQI/first PMI/second PMI is reported for transmission modes 9 with submode 2 and 8 CSI-RS ports configured: - A single precoding matrix is selected from the codebook subset assuming transmission on all narrowband(s) in the CSI reference resource. - A UE shall report a type 2c report consisting of - A single wideband CQI value which is calculated assuming the use of the single precoding matrix in allnarrowband(s) inthe CSI reference resource and transmission on all narrowband(s) in the CSI reference resource. - The widebandfirstPMI andthewideband secondPMI corresponding to the selected single precoding matrix as defined in Subclause 7.2.4. - The wideband firstPMI, thewideband second PMI andthe widebandCQI are calculated conditioned ontransmission rank 1. <Unchanged parts are omitted>

R1-2000228	Corrections on CE Mode A and B improvements for non-BL UEs	Huawei, HiSilicon

R1-2000373	Correction on CSI-RS based CSI feedback for non-BL UE	ZTE

R1-2001060	Corrections for CE mode improvements for non-BL UEs for LTE-MTC	Ericsson

#### Use of LTE Control Channel Region for DL Transmission

R1-2001052	Feature lead summary on use of LTE control channel region	Nokia, Nokia Shanghai Bell

R1-2001188	Feature lead summary#2 on use of LTE control channel region	Nokia, Nokia Shanghai Bell

FL recommendation: No email discussion needed as both identified issues are not critical:

- Issue 1: TBS determination for PDSCH transmission – whether to scale up TBS due to increased number of PDSCH REs

- Issue 2: Unclear meaning of “for other purposes” wording in 6.4.1 of 36.211 – whether to replace “other purposes” with “cell-specific reference signals”

R1-2000229	Corrections on use of LTE control channel region for DL transmission	Huawei, HiSilicon

R1-2000374	Correction on use of LTE control channel region	ZTE

#### Use of RSS for measurement improvements

R1-2000791	Summary of the use of RSS for measurement improvements	Sony Europe B.V.

Further revised in R1-2001152	Feature Lead Summary of the use of RSS for measurement improvements	Sony

R1-2001175	Issues for email discussion on the use of RSS for measurement improvements	Sony Europe B.V.

[100e-LTE-eMTC5-RSS_Measurement-01] – Shin (SONY)

Email discussion/approval how to signal the RRC parameter rss-measurement-power-bias to the UE in Connected Mode for UE detected cells that are NOT in the Neighbour Cell List by considering the following proposal:

- The RRC parameter rss-measurement-power-bias is signaled to the UE in Connected Mode for detected cells that are NOT in the Neighbour Cell List, where q_offset is assumed to be 0 dB.

By 2/27; if there is spec impact, followed up endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Feb. 28th,

Agreement

- If the rss-measurement-non-NCL-enabled is “ENABLED”, the UE assumes that the RSS power bias is common for all neighbour cells that are NOT in the Neighbour Cell List.

  - This power bias is the same as that used for the serving cell or the camped on cell

The above agreement doesn't require any TP to RAN1 specs

[100e-LTE-eMTC5-RSS_Measurement-02] – Shin (SONY)

Email discussion/approval on potentials corrections made to the RRC parameter description for RSS:

- All RSS RRC parameters should be applicable to both Idle Mode and Connected Mode

- The rss-measurement-narrowbands is only provided if rss-measurement-collocation is NOT configured or is indicated as “NonCollocated”

by 2/27; if there is spec impact, followed up endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Feb. 28th,

Agreement

- Correct the description for the RSS RRC Parameter with the following:

  - for every occurrence of “for CE Mode A/B in RRC_CONNECTED” change to “for CE Mode A/B in RRC_IDLE and RRC_CONNECTED”

Agreement

- Correct the description for the rss-measurement-narrowbands RRC Parameter to:

  - Bitmap containing narrowbands used for RSS deployment in carrier for CE mode A/B in RRC_CONNECTED. Narrowbands including central 6 PRBs are excluded from the bitmap. This parameter is only provided if rss-measurement-collocation is configured as “NonCollocated”.

Agreement

- Correct the description for the rss-measurement-time-offset-granularity RRC Parameter by adding the following sentence at the end of its description:

  - This parameter is only provided if rss-measurement-collocation is configured as “NonCollocated”.

[100e-LTE-eMTC5-RSS_Measurement-03] – Shin (SONY)

Email discussion/approval on whether to add the parameter RSS duration = {8, 16, 32, 40} subframes for each neighbour cell in the Neighbour Cell List, into the list of RRC parameters for using RSS for measurement improvements, by 2/27; if there is spec impact, followed up endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Mar. 2nd,

Agreement

- The UE assumes the RSS Duration of the neighbour cell is the same as that of the serving cell

The above agreement doesn't require any TP to RAN1 specs

R1-2000375	Remaining issues on RSS for measurement improvement	ZTE

R1-2000580	Use of RSS for measurement in Connected Mode	Sony

R1-2000705	Measurements based on RSS	Qualcomm Incorporated

#### Other

None

### Maintenance of Additional Enhancements for NB-IoT

#### UE-group wake-up signal

R1-2000790	Feature lead summary of Group WUS for NB-IoT	Ericsson

R1-2001187	Feature lead summary#2 of Group WUS for NB-IoT	Ericsson

[100e-LTE-NB_IOTenh3-GroupWUS-01] – Magnus (Ericsson)

Email discussion/approval on detecting of up to 2 WUS sequences by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Feb.28th,

Agreement

The following text proposal is endorsed for Sec. 16.9 in TS 36.213. TP to be included in 36.213 editor’s CR.

“A NB-IoT UE can be configured with up to two NWUS [14]. A UE may assume that no more than one NWUS sequence is transmitted per NWUS resource at a given time.”

[100e-LTE-NB_IOTenh3-GroupWUS-02] – Magnus (Ericsson)

Email discussion/approval on total number of WUS subframes (WUS resource time location) by 2/27; if there is a spec impact, followed by endorsing the corresponding TP  by 3/2

Decision: As per email decision posted on Feb.28th,

Agreement

The following text proposal for Sec. 16.9 in TS 36.213 is endorsed. TP to be included in 36.213 editor’s CR.

--------------------------------------------- Start of Text Proposal ---------------------------------------

A NB-IoT UE using NWUS can assume the actual duration of NWUS, starting in subframe w0, is one of the values in the set listed in Table 16.9-1 corresponding to the maximum duration of NWUS, , configured by higher layers. There is a total of NB-IoT DL subframes and subframes #4 carrying SystemInformationBlockType1-NB in the maximum duration of NWUS. The maximum duration of NWUS starts in subframe w0, where w0 is the latest subframe such that there is a total of NB-IoT DL subframes and subframes #4 carrying SystemInformationBlockType1-NB in the maximum duration and a total of  NB-IoT DL subframes and subframes #4 carrying SystemInformationBlockType1-NB in the duration that ends in subframe (g0-1), where g0 is defined by [14] and is the NWUS resource that the UE is associated to as defined in [3]. The UE may assume that NWUS and its associated NB-IoT paging occasion subframes are on the same NB-IoT carrier.

--------------------------------------------- End of Text Proposal ---------------------------------------

R1-2000215	Corrections on UE-group wake-up signal	Huawei, HiSilicon

R1-2000376	Clarification of group WUS for NB-IoT	ZTE

R1-2000706	UE-group wake-up signal	Qualcomm Incorporated

#### Support for transmission in preconfigured UL resources

R1-2001021	Feature lead summary #1 on  transmission in pre-configured UL resources	Huawei

R1-2001165	Feature lead summary #2 on  transmission in pre-configured UL resources	Huawei

R1-2001251	Feature lead summary #3 on  transmission in pre-configured UL resources	Huawei

[100e-LTE-NB_IOTenh3-PUR-01] – Xiang (Huawei)

Email discussion/approval on RV definition for PUR initial transmission, subcarrier spacing for PUR and the start of PUR SS window by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Mar. 4th,

Agreement

The following TPs in section 5.5 of R1-2001251 are endorsed.

- TP#A for TS36.213 section 16.5.1.2. To be included in editor’s CR for TS36.213.

- TP#B for TS36.213 section 16.5.1.1. To be included in editor’s CR for TS36.213.

- TP#C for TS36.213 section 16.6. To be included in editor’s CR for TS36.213.

Conclusion

UL gaps are supported for PUR transmissions and retransmissions.

- Specification change not required.

[100e-LTE-NB_IOTenh3-PUR-02] – Xiang (Huawei)

Email discussion/approval on clarification of UE not transmitting NPUSCH upon receiving DCI format N0 indicating PUR ACK/fallback, PUR initial transmission not scheduled by DCI, and PUR retransmission is scheduled by DCI by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Mar. 2nd,

Agreement

The following text proposal regarding PUR retransmission/scrambling for Section 16.5.1 of TS 36.213 is agreed. TP to be included in 36.213 editor’s CR.

----------------------------------- Start of Text Proposal for TS 36.213----------------------------------

-------------------------------------------- Unchanged parts omitted --------------------------------------

16.5.1	UE procedure for transmitting format 1 narrowband physical uplink shared channel

NPUSCH format 1 transmission can be scheduled by a NPDCCH with DCI format N0, or the transmission can correspond to using preconfigured uplink resource configured by higher layers. Transmission using preconfigured uplink resource is initiated by higher layers as specified in [14], while retransmission of transport blocks transmitted using preconfigured uplink resource are scheduled by a NPDCCH with DCI format N0.

A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N0 ending in NB-IoT DL subframe n scheduling NPUSCH intended for the UE, perform, at the end of

-    n+k0 DL subframe for FDD,

-    k0 NB-IoT UL subframes following the end of n+8 subframe for TDD,

a corresponding NPUSCH transmission using NPUSCH format 1 in N consecutive NB-IoT UL slots ni with i = 0, 1, …, N-1 according to the NPDCCH information where

…

Table 16.5.1-5: NPDCCH and NPUSCH configured by SPS C-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format N0 | UE specific by C-RNTI |

A UE may transmit NPUSCH on preconfigured uplink resources as configured by higher layers. The scrambling initialization of NPUSCH transmission using preconfigured uplink resource is by PUR C-RNTI.

If a UE is configured by higher layers to decode NPDCCHs with the CRC scrambled by the PUR C-RNTI, the UE shall decode the NPDCCH according to the combination defined in Table 16.5.1-6 and in case the indication in the DCI corresponds to the retransmission of a transport block transmitted using preconfigured uplink resource, transmit a corresponding NPUSCH. The scrambling initialization of this NPUSCH corresponding to these NPDCCHs and the NPUSCH retransmission for the same transport block is by PUR C-RNTI.

Table 16.5.1-6: NPDCCH and NPUSCH configured by PUR C-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format N0 | UE specific by PUR C-RNTI |

-------------------------------------------- Unchanged parts omitted --------------------------------------

---------------------------------------------- End of Text Proposal ----------------------------------------

[100e-LTE-NB_IOTenh3-PUR-03] – Xiang (Huawei)

Email discussion/approval on collision between i) PUR tx and Paging CSS/NWUS; ii) PUS SS monitoring and Paging CSS/NWUS by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Mar. 4th,

Agreement

Regarding collision handling between PUR transmission and Paging CSS, the following TP for Section 16.6 of TS 36.213 is endorsed. TP to be included in 36.213 editor’s CR.

----------------------------------- Start of Text Proposal for TS 36.213----------------------------------

16.6	Narrowband physical downlink control channel related procedures

-------------------------------------------- Unchanged parts omitted --------------------------------------

UE is not required to monitor Type1A-NPDCCH common search space in subframes in which the UE receives NPDSCH assigned by NPDCCH with DCI CRC scrambled by SC-RNTI.

UE is not required to monitor Type2A-NPDCCH common search space in subframes in which the UE receives NPDSCH assigned by NPDCCH with DCI CRC scrambled by G-RNTI or SC-RNTI.

Until UE receives higher layer configuration of NPDCCH UE-specific search space, the UE monitors NPDCCH according to the same configuration of NPDCCH search space as that for NPDCCH scheduling Msg4.

A UE is not required to monitor Type1-NPDCCH common search space if the set of subframes comprising the NPDCCH candidates include any subframes in which the UE has initiated an NPUSCH transmission using preconfigured uplink resource on a given serving cell.

-------------------------------------------- Unchanged parts omitted --------------------------------------

---------------------------------------------- End of Text Proposal ----------------------------------------

R1-2000216	Corrections on transmission in preconfigured UL resources	Huawei, HiSilicon

R1-2000377	Remaining issues on transmission in preconfigured UL resources for NB-IoT	ZTE

R1-2000707	Support for transmission in preconfigured UL resources	Qualcomm Incorporated

R1-2000908	Remaining issues on transmission in preconfigured UL resources for NB-IoT	NTT DOCOMO, INC.

R1-2001061	Corrections for Preconfigured UL resources for NB-IoT	Ericsson

R1-2001087	PUR and interactions with legacy features	Futurewei

#### Scheduling of multiple DL/UL transport blocks

R1-2001130	FL summary on Multiple TB scheduling enhancement for NB-IoT	ZTE

R1-2001201	FL summary#2 on Multiple TB scheduling enhancement for NB-IoT	ZTE

[100e-LTE-NB_IOTenh3-Multi-TB-01] – Shupeng (ZTE)

Email discussion/approval on scheduling gap for multicast (multicast scheduling gap is inserted before each TB rather than after each TB in order to facilitate insertion of legacy MPDCCH transmissions before each TB; collision of scheduling gap and DL gap; capture missing agreement in the spec)  by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Mar. 2nd,

Agreement

- Remove the scheduling gap after the last TB.

- For SC-MTCH multiple TBs scheduling, if scheduling gap or UE processing gap collides with DL gap, overlapping part of scheduling gap or UE processing gap is considered as part of DL gap.

- Capture the agreement regarding UE processing gap if RRC configured gap is 0.

R1-2001398	TP for gaps of multicast for multiple TB scheduling of NB-IoT 	ZTE

Decision: As per email decision posted on Mar. 5th, the text proposal for TS36.213 in R1-2001398 is endorsed for editor’s CR.

[100e-LTE-NB_IOTenh3-Multi-TB-02] – Shupeng (ZTE)

Email approval of the TP on symbol counter reset, along with misc editorial corrections (typo in one place & rewording in two places to make spec easier to parse) by 2/27

R1-2001399	TP for miscellaneous changes for multiple TB scheduling of NB-IoT 	ZTE

Decision: As per email decision posted on Mar. 5th, TPs for TS 36.211 (section 10.1.5) and for TS 36.213 (section 16.4.2) in R1-2001399 are endorsed for editor’s CRs.

R1-2000217	Corrections on scheduling of multiple DL/UL transport blocks	Huawei, HiSilicon

R1-2000378	Remaining issues on scheduling enhancement for NB-IoT	ZTE

R1-2000708	Scheduling of multiple DL/UL transport blocks	Qualcomm Incorporated

R1-2001062	Corrections for Multi-TB scheduling for NB-IoT	Ericsson

#### Coexistence of NB-IoT with NR

R1-2001022	Feature lead summary #1 on coexistence of NB-IoT with NR	Huawei

R1-2001178	Feature lead summary #2 on coexistence of NB-IoT with NR	Huawei

[100e-LTE-NB_IOTenh3-NR-Coexistence-01] – Yubo (Huawei)

Email discussion/approval on capturing and clarification of Rel-16 resource reservation agreements by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Mar. 2nd,

Agreement

For issue #1 (NB-IoT downlink/uplink subframe): If Rel-16 resource reservation is configured, the unicast NPDCCH/NPDSCH/NPUSCH transmission follow resource reservation of Rel-16, regardless of the Rel-13 valid/invalid configuration

Conclusion

For issue #2 (Whether to use resources which cannot be indicated by bitmaps in a reserved resource period): The subframes not indicated by bitmaps by R16 resource reservation parameters are considered as not reserved, and are available to UEs configured with resource reservation.

Agreement

For issue #6 (UE-specific configuration and presence of DCI resource reservation field): It should be possible to configure or enable the resource reservation feature using UE-specific signaling for uplink and downlink separately. The details are up to RAN2.

The resource reservation field in DCI formats N0 or N1 is only present if resource reservation is configured and the corresponding DCI is mapped onto the UE specific search space given by the C-RNTI.

Email decision posted on Mar. 4th

R1-2001280	Feature lead summary #1 on [100e-LTE-NB_IOTenh3-NR-Coexistence-01]	Huawei

Agreement

- Text proposal in Annex C of R1-2001280 is endorsed for TS 36.213 section 16.4. TP to be included in 36.213 editor’s CR.

- Text proposal in Annex D of R1-2001280 is endorsed for TS 36.213 section 16.5. TP to be included in 36.213 editor’s CR.

- Text proposal in Annex E of R1-2001280 is endorsed for TS 36.212 sections 6.4.3.1 and 6.4.3.2. TP to be included in 36.212 editor’s CR.

[100e-LTE-NB_IOTenh3-NR-Coexistence-02] – Yubo (Huawei)

Email discussion/approval on capturing via a TP the cases omitted by spec for uplink channels by 2/27

R1-2001339	Feature lead summary #2 on [100e-LTE-NB_IOTenh3-NR-Coexistence-02]	Huawei

Decision: As per email decision posted on Mar. 4th,

Agreement

- Text proposal in proposal 1 of R1-2001339 is endorsed for TS 36.211 section 10.1.3.6, with following change. TP to be included in 36.211 editor’s CR.

“If higher layer parameter valid-subframe-config-UL or slot-reserved-resource-config-UL is configured, then in case of NPUSCH format 1 transmission associated with C-RNTI or SPS C-RNTI with the Resource reservation field in the DCI is set to 1, or in case of NPUSCH format 2 transmission associated with C-RNTI”

- Text proposal in proposal 2 of R1-2001339 is endorsed for TS 36.211 section 10.1.3.6. TP to be included in 36.211 editor’s CR.

[100e-LTE-NB_IOTenh3-NR-Coexistence-03] – Yubo (Huawei)

Email discussion/approval on capturing via a TP the cases omitted by spec for downlink channels by 2/27

spec for downlink channels by 2/27 – Yubo (Huawei)

Decision: As per email decision posted on Mar. 3rd,

R1-2001282	Feature lead summary #1 on [100e-LTE-NB_IOTenh3-NR-Coexistence-03]	Huawei

Agreement

- The resource reservation applies also to NPDCCH with CRC scrambled by SPS C-RNTI

- The text proposal in proposal 1 of R1-2001282 is endorsed for TS36.211 subclause 10.2.5.5. TP to be included in 36.211 editor’s CR.

R1-2000218	Corrections on coexistence of NB-IoT with NR	Huawei, HiSilicon

R1-2000379	Remaining issues on NB-IoT resource reservation	ZTE

R1-2000709	Coexistence of NB-IoT with NR	Qualcomm Incorporated

R1-2001063	Corrections for NR coexistence performance improvements for NB-IoT	Ericsson

#### Support of Quality report in Msg3 and Connected Mode

None.

#### Presence of NRS on a non-anchor carrier for paging

R1-2000814	Feature lead summary for NRS in non-anchor carrier	Qualcomm Incorporated

[100e-LTE-NB_IOTenh3-NRS-01] – Alberto (Qualcomm)

Email discussion/approval on a TP clarifying Subclause 10.2.6 of 36.211 by 2/27

R1-2001271	Outcome of email thread [100e-LTE-NB_IOTenh3-NRS-01] - Issues for NRS in non-anchor	Qualcomm Incorporated

Decision: As per email decision posted on Mar. 2nd,

Agreement: The TP in R1-2001271 for subclause 10.2.6 of TS 35.211 is endorsed. TP to be included in 36.211 editor’s CR.

R1-2000380	Correction of NRS on a non-anchor carrier for paging	ZTE

R1-2001019	Corrections on presence of NRS on a non-anchor carrier for paging	Huawei, HiSilicon

#### Other

R1-2001114	Further considerations on RRC parameter lists for NB-IoT	Huawei, HiSilicon

### Maintenance of DL MIMO efficiency enhancements for LTE

#### Support of enhancing SRS capacity and coverage

R1-2001139	Feature summary on LTE DL MIMO efficiency enhancement	Huawei

R1-2001179	Feature summary#2 on LTE DL MIMO efficiency enhancement	Huawei

##### Additional SRS symbols

[100e-LTE_DL_MIMO_EE-AddSRS-01] – Yubo (Huawei)

Email discussion/approval on a TP capturing, correction and clarification of transmission pattern for additional SRS by 2/27

R1-2001283	Feature summary #1 on [100e-LTE_DL_MIMO_EE-AddSRS-01]	Huawei

Decision: As per email decision posted on Mar. 4th,

Agreement.

- Text proposal in proposal 1 of R1-2001283 is endorsed for TS 36.213 section 8.2. TP to be included in 36.213 editor’s CR.

- Text proposal in proposal 2 of R1-2001283 is endorsed for TS 36.211 section 5.5.3.2.2. TP to be included in 36.211 editor’s CR.

- The equation to derive N_FH is N=RNASNFH+NAS-1GAS+NFH-1GFHNAS+NAS-1(1-GAS)GFH ,

  - The relevant text proposal in proposal 3 of R1-2001283 is endorsed for TS 36.211 section 5.5.3.2.2

- Text proposal in proposal 4 of R1-2001283 is endorsed for TS 36.211 section 5.5.3.2.1. TP to be included in 36.211 editor’s CR.

[100e-LTE_DL_MIMO_EE-AddSRS-02] – Yubo (Huawei)

Email discussion/approval on a TP capturing omitted issues or agreements on power control by 2/27 – Yubo (Huawei)

R1-2001284	Feature summary #1 on [100e-LTE_DL_MIMO_EE-AddSRS-02]	Huawei

Decision: As per email decision posted on Mar. 3rd,

Agreement

- Text proposal in proposal 1 of R1-2001284 is endorsed for TS 36.213 section 5.1.3. TP to be included in 36.213 editor’s CR.

- The power scaling factor is decided by the maximum power of all the overlapped SRS symbol(s) transmitted in the same subframe

  - The relevant text proposal in proposal 2 of R1-2001284 is endorsed for TS 36.213 section 5.1.3. TP to be included in 36.213 editor’s CR.

R1-2000225	Corrections on additional SRS symbols	Huawei, HiSilicon

R1-2000559	Maintenance for additional SRS symbols	Nokia, Nokia Shanghai Bell

R1-2000594	Remaining issues on additional SRS symbols	Lenovo, Motorola Mobility

R1-2000655	Text proposals on additional SRS symbols	LG Electronics

R1-2000710	Additional SRS symbols	Qualcomm Incorporated

R1-2000717	Corrections to SRS	Intel Corporation

R1-2000942	Maintenance for additional SRS symbols	Ericsson

##### Other

R1-2000226	Remaining issues of additional SRS symbols	Huawei, HiSilicon

#### Other

### Maintenance of LTE-based 5G Terrestrial Broadcast

R1-2001151	Summary of issues for LTE-based 5G terrestrial broadcast	Qualcomm Incorporated

R1-2001197	Updated summary of issues for LTE-based 5G terrestrial broadcast	Qualcomm Incorporated

[100e-LTE_TerrBcast-01] – Alberto (Qualcomm)

Email approval on the updated TR 36.976 as in R1-2000713 by 2/26

R1-2000713	Updated TR 36.976 Overall description of LTE-based 5G broadcast	Qualcomm Incorporated

Decision: As per email decision posted on Feb. 27th, latest version is endorsed as v2.0.0 in R1-2001230 for approval by RAN#87.

#### New numerology(ies) for PMCH for support of rooftop reception

[100e-LTE_TerrBcast-02] – Alberto (Qualcomm)

Email discussion/approval on the issues for 300us numerology (staggering of MBSFN-RS; scrambling initialization based on slot; warm-up subframes) by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001272	Outcome of email thread [100e-LTE_TerrBcast-02] - Issues for 300us numerology	Qualcomm Incorporated

Decision: As per email decision posted on Feb. 27th, TP1 and TP2 in R1-2001272 for TS 36.211 are endorsed.

No consensus in adopting TP3 in R1-2001272.

R1-2000213	Corrections to 36.211 and 36.213 for the 0.37kHz subcarrier spacing MBSFN	Huawei, HiSilicon

R1-2000711	Support of longer numerologies for rooftop reception	Qualcomm Incorporated

#### Necessity and detailed enhancements to the physical channels and signals in the CAS

[100e-LTE_TerrBcast-03] – Alberto (Qualcomm)

Email discussion/approval one issue for CAS (AL PDCCH applicability to all non-MBSFN subframes) by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001273	Outcome of email thread [100e-LTE_TerrBcast-03] - Issues for CAS	Qualcomm Incorporated

Decision: As per email decision posted on Feb. 27th, TP1 in R1-2001273 for TS 36.211 is endorsed.

R1-2000712	Corrections for CAS reception	Qualcomm Incorporated

#### New numerology for support of mobility of up to 250km/h

None.

#### Other

R1-2001111	Text proposal for new numerology in TS 36.201	Huawei, HiSilicon

### LTE Rel-16 UE Features

[100e-LTE-Rel-16-UEFeatures] – Hiroki (DCM)/Xiaoyi (ATT)

Email discussion/approval of Rel-16 LTE UE feature till next meeting (starting no earlier than 3/6)

R1-2000224	On LTE UE features in Rel-16	Huawei, HiSilicon

R1-2000574	Discussion on LTE Rel-16 UE Features	ZTE

R1-2000714	Views on UE features	Qualcomm Incorporated

R1-2000909	RAN1 UE features list for Rel-16 LTE	NTT DOCOMO, INC., AT&T

Late submission

### Other

Including any essential TPs related to the endorsed Rel-16 LTE TEIs.

None.

# NR Maintenance

## Maintenance of Release 15 NR

Only essential corrections – a rejected draft CR will be marked in red

R1-2001124	RAN1#100-e preparation phase on NR Rel-15 CRs	Ad-hoc chair (Samsung)

R1-2001125	RAN1#100-e preparation phase output NR Rel-15 CRs	Ad-hoc chair (Samsung)

NR_newRAT-Core

R1-2001474	Corrections to 38.213 including alignment of terminology across specifications in RAN1#100-e	Samsung	38.213	CR0096, F

R1-2001476	CR on UL PTRS density selection	Nokia	38.214	CR0077, F

### Maintenance for Initial access and mobility

[100e-NR-7.1CRs-01] – Younsun (Samsung)

Email discussion/approval on the issue raised in R1-2000603 by 2/27

R1-2000603	Draft CR on SSB-RO association	Samsung

Decision: As per email decision posted on Feb. 29th,

Agreement: The text proposal in R1-2000603 is endorsed for the alignment CR for TS38.213.

R1-2000602	Discussion on SSB-RO association	Samsung

R1-2001098	Correction on ATF measurements for SS RSRPB	Huawei, HiSilicon

### Maintenance for MIMO

[100e-NR-7.1CRs-02] – Younsun (Samsung)

Email discussion/approval on the issue raised in R1-2000842 by 2/27 – Younsun (Samsung)

R1-2000842	Draft CR on UL PTRS density selection	Apple

R1-2001210	Draft CR on UL PTRS density selection	Apple

Decision: As per email decision posted on Feb. 27th,

Agreement: The text proposal in R1-2001210 is endorsed as part of the alignment CR for TS38.214.

R1-2000237	Correction of SRS bandwidth 	Ericsson

R1-2000604	Draft CR on TCI state for PDCCH during RA procedure	Samsung

R1-2000656	Clarification of the maximum number of CSI-RS resources in resource setting	LG Electronics

R1-2000744	Correction on TRS configuration to TS38.214	CMCC

R1-2000778	PUCCH Spatial Relation after CBRA BFR	Nokia, Nokia Shanghai Bell

R1-2000840	CR on QCL Assumption for DL PT-RS and DMRS	Apple

R1-2000841	CR on QCL Indication for CSI-RS	Apple

R1-2000868	Clarification on UE assumptions for deriving the CQI index	Sharp

R1-2000944	Clarification on number of occupied CPUs and delay requirements for CSI reports using PUSCH	Nokia, Nokia Shanghai Bell

R1-2000950	Draft CR on resource and port occupation of duplicate CSI-RS resource	Qualcomm Incorporated

R1-2000951	Discussion on CR on resource and port occupation of duplicate CSI-RS resource	Qualcomm Incorporated

R1-2000998	Clarification of PTRS port association for PUSCH 	Nokia, Nokia Shanghai Bell

R1-2001082	Correction on CSI-RS resource set configuration	Ericsson

### Maintenance for Scheduling/HARQ aspects

[100e-NR-7.1CRs-03] – Younsun (Samsung)

Email discussion/approval on the issue raised in R1-2000553 by 2/27 – Younsun (Samsung)

R1-2000553	Draft 38.211 CR Intra-slot frequency hopping with 1-symbol PUCCH	Nokia, Nokia Shanghai Bell

Decision: As per email decision posted on Feb. 29th, the following is concluded:

CR in R1-2000553 is not needed, as it is RAN1 common understanding that the presence or absence of the intra-slot frequency hopping configuration (a.k.a intraSlotFrequencyHopping and secondHopPRB) does not have any impact on how the 1-symbol PUCCH is transmitted.

- For UEs supporting PUCCH format 0/2 without intra-slot frequency hopping, intra-slot frequency hopping configuration can be provided but will be ignored for 1-symbol PUCCH.

- For UEs NOT supporting PUCCH format 0/2 without intra-slot frequency hopping, to avoid possible different interpretations by the UE, 1-symbol PUCCH can still be configured no matter intra-slot frequency hopping configuration is provided or not.

R1-2000303	Draft 38.214 CR on UL skipping	vivo

Already being handled under agenda item 5.1.

R1-2000415	Correction on PDCCH Blind Detection for NR-DC	ZTE

R1-2000486	Correction on multiplexing HARQ-ACK/SR/CSI in a PUCCH determined based on PRI	OPPO

R1-2000487	Correction on multiplexing HARQ-ACK/SR/CSI in a multi-CSI-PUCCH	OPPO

R1-2000516	Clarification for processing order of UL multiplexing and cancellation	CATT

R1-2000517	Correction on the definition for timeline condition	CATT

R1-2000518	Correction for PUCCH repetition transmission	CATT

R1-2000555	Draft 38.212 CR Correction to dynamic frequency domain resource allocation type selection	Nokia, Nokia Shanghai Bell

R1-2000556	Draft 38.214 CR Corrections to PDSCH PRB bundling	Nokia, Nokia Shanghai Bell

R1-2000558	Draft 38.214 CR Correction to TBS determination when 3824<Ninfo<3825	Nokia, Nokia Shanghai Bell

R1-2000579	Clarification on UE behaviour with no transmission of UL-SCH	Fujitsu Limited

R1-2000718	Clarification on UE features on BWP operations	Intel Corporation

R1-2000869	Correction on rate matching output sequence length	Sharp

R1-2000870	Correction on removal of padding bits in DCI size alignment procedure	Sharp

R1-2000952	Draft CR on clarifying A-CSI multiplexing on PUSCH with slot aggregation	Qualcomm Incorporated

R1-2000994	UL grant overridden between configured grant and RAR grant	ASUSTeK

R1-2001032	Clarification on UCI multiplexing on PUSCH	Huawei, HiSilicon

R1-2001100	Correction on PDCCH candidate colliding with RB-symbol level RMR in TS38.213	Huawei, HiSilicon

R1-2001101	Correction on rate-matching for PDSCH/PDSCH with SPS in TS38.214	Huawei, HiSilicon

R1-2001115	Correction on PUSCH with a Type 1 configured grant	Huawei, HiSilicon

### Maintenance for NR-LTE co-existence

R1-2000554	Draft 36.213 CR Correction on PUSCH and PDSCH scheduling with CSS	Nokia, Nokia Shanghai Bell

### Maintenance for UL power control

[100e-NR-7.1CRs-04] – Younsun (Samsung)

Email discussion/approval on the issue raised in R1-2001099 by 2/27 – Younsun (Samsung)

R1-2001099	Correction on resetting accumulation of closed-loop power control	Huawei, HiSilicon

Decision: As per email decision posted on Feb. 29th,

Agreement: The following text proposal for TS38.213 section 7.1.1 is endorsed as part of the alignment CR

If j>1 and the UE is provided higher SRI-PUSCH-PowerControl, l is any of the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to j

Draft CR revised in R1-2001473.

### Other

R1-2001074	Discussion of need for correction for inconsistent shadow fading parameters in NTN rural scenario	Nokia, Nokia Shanghai Bell

R1-2001075	Correction for inconsistent shadow fading parameters in NTN rural scenario	Nokia, Nokia Shanghai Bell

## Maintenance of Release 16 NR

Only essential corrections

Only text proposals are allowed (no individual draft CRs, please!). Editors to prepare the consolidated CRs per spec.

Limit to maximum 1 contribution per 1 company/organization/university for any agenda item, including “Others”.

[100e-NR-Editor-WICode-Spec#]  – Editors

Editors’ CRs to consolidate all the endorsed TPs for approval by 3/6

- Per Rel-16 work item and per spec

- On the cover page, please list the email thread # for each endorsed TP incorporated

- If there is any editorial update not resulting from any email threads, please duly state them on the CR cover page

Decision: The following CRs are agreed:

5G_V2X_NRSL-Core

R1-2001442	Corrections on NR V2X	Nokia	38.214	CR0069, F

R1-2001458	Corrections on Sidelink	Samsung	38.213	CR0093, F

R1-2001465	Corrections on 5G V2X sidelink features after RAN1#100-e	Huawei	38.212	CR0036, F

R1-2001471	Corrections to V2X	Ericsson	38.211	CR0038, F

R1-2001480	Corrections to V2X measurement definitions	Intel Corporation	38.215	CR0020, F

LTE_NR_DC_CA_enh-Core

R1-2001443	Corrections on Cross-carrier Scheduling with Different Numerologies	Nokia	38.214	CR0070, F

R1-2001451	Corrections on NR-DC and on Cross-carrier Scheduling with Different Numerologies	Samsung	38.213	CR0086, F

R1-2001469	Corrections to cross-carrier scheduling with different numerologies	Ericsson	38.211	CR0036, F

NR_2step_RACH-Core

R1-2001448	Corrections on two-step RACH after RAN1#100-e	Nokia	38.214	CR0075, F

R1-2001450	Corrections on two-step RACH	Samsung	38.213	CR0085, F

R1-2001468	Corrections to two-step RACH	Ericsson	38.211	CR0035, F

NR_CLI_RIM-Core

R1-2001440	Corrections to cross layer interference measurements	Intel Corporation	38.215	CR0019, F

NR_eMIMO-Core

R1-2001438	Corrections to L1-SINR definitions	Intel Corporation	38.215	CR0017, F

R1-2001445	Corrections on NR enhanced MIMO	Nokia	38.214	CR0072, F

R1-2001454	Corrections on MIMO enhancements	Samsung	38.213	CR0089, F

R1-2001462	Corrections for NR MIMO after RAN1#100-e	Huawei	38.212	CR0033, F

R1-2001472	Corrections to MIMO enhancements	Ericsson	38.211	CR0039, F

NR_IAB-Core

R1-2001452	Corrections on integrated access and backhaul	Samsung	38.213	CR0087, F

R1-2001466	Corrections to integrated access and backhaul for NR	Ericsson	38.211	CR0033, F

NR_IIOT-Core

R1-2001446	Corrections on Industrial IoT	Nokia	38.214	CR0073, F

R1-2001453	Corrections on Industrial IoT	Samsung	38.213	CR0088, F

NR_L1enh_URLLC-Core

R1-2001444	Corrections on NR URLLC support	Nokia	38.214	CR0071, F

R1-2001460	Corrections on Ultra Reliable Low Latency Communications Enhancements	Samsung	38.213	CR0095, F

R1-2001463	Corrections for URLLC after RAN1#100-e	Huawei	38.212	CR0034, F

NR_Mob_enh-Core

R1-2001455	Corrections on Mobility Enhancements	Samsung	38.213	CR0090, F

NR_pos-Core

R1-2001439	Corrections to NR positioning support	Intel Corporation	38.215	CR0018, F

R1-2001449	Corrections of NR positioning support	Nokia	38.214	CR0076, F

R1-2001457	Corrections on Positioning	Samsung	38.213	CR0092, F

R1-2001467	Corrections to NR positioning support	Ericsson	38.211	CR0034, F

NR_UE_pow_sav-Core

R1-2001447	Corrections of cross-slot scheduling restriction and CSI/L1-RSRP measurement outside active time	Nokia	38.214	CR0074, F

R1-2001459	Corrections on UE power savings	Samsung	38.213	CR0094, F

R1-2001464	Corrections for power saving after RAN1#100-e	Huawei	38.212	CR0035, F

NR_unlic-Core

R1-2001437	Corrections to NR-based access to unlicensed spectrum	Ericsson	37.213	CR0007, F

R1-2001441	Corrections on NR - U	Nokia	38.214	CR0068, F

R1-2001456	Corrections on shared spectrum channel access	Samsung	38.213	CR0091, F

R1-2001461	Corrections for Rel-16 NR-U after RAN1#100-e	Huawei	38.212	CR0032, F

R1-2001470	Corrections to NR-based access to unlicensed spectrum	Ericsson	38.211	CR0037, F

TEI16

R1-2001475	Introduction of half-duplex operation in CA with unpaired spectrum	Samsung	38.213	CR0097, B

[100e-NR-Rel-16-RRC] – Fred (Qualcomm)

Updated RRC parameters consolidating new/updated RRC parameters (if any) for approval 3/6

- Any new RRC parameters are highly discouraged!

R1-2001478	Updated consolidated parameter list for Rel-16 NR	Qualcomm

Decision: As per email decision posted on Mar. 10th, the final parameter list is endorsed

R1-2001479	LS on updated Rel-16 LTE and NR parameter lists	RAN1, Qualcomm

Decision: The LS is approved.

RAN2 NR stage-2 CR (38.300) for Rel.16 to be submitted to March plenary for approval.

[100e-NR-Stage2CR] – Karri (Nokia)

Email discussion/approval on stage 2 CRs providing update to 38.300 for Rel-16 WIs (except NR-U) by 3/3

- Rapporteurs are encouraged to provide the respective inputs, but not required if no time. Additional refinement can be done in later meetings.

- Focusing on RAN1 parts only.

R1-2001355	RAN1 input on Rel-16 WIs (except NR-U) related changes for 38.300 running CR	Nokia

Decision: As per email decision posted on Mar. 5th, the TPs are endorsed by RAN1.

R1-2001356	LS to RAN2 on Rel-16 WIs (except NR-U) related changes for 38.300 running CR	RAN1, Nokia

Decision: The LS is approved.

R1-2000557	Draft 38.215 CR Introduction of UE UTRAN FDD measurements for SRVCC from NR to UMTS	Nokia, Nokia Shanghai Bell

R1-2001053	Editorial corrections to MIMO enhancements	Ericsson

### Maintenance of Two step RACH for NR

#### Channel Structure for Two-Step RACH

R1-2000809	FL summary on the maintenance of the 2-step RACH channel structure	ZTE

R1-2001206	FL summary#2 on the maintenance of the 2-step RACH channel structure	ZTE

[100e-NR-2step-RACH-ChStructure-01] – Li (ZTE)

Email discussion/approval by 3/1 on a TP capturing the following:

- Definition of PUSCH occasion and PUSCH resource unit in 38.213, and

- Description about the mapping between preambles and PUSCH resource units

- And to add the condition that msgA PRACH and PUSCH should be in different slots in 38.213

R1-2001368	Outcome of email thread [100e-NR-2step-RACH-ChStructure-01]	ZTE

Decision: As per email decision posted on Mar. 4th

Agreement: TP#1 in the Appendix of R1-2001368 to clarify the mapping between msgA PRACH and PUSCH in TS 38.213 Section 8.1A is endorsed.

[100e-NR-2step-RACH-ChStructure-02] – Li (ZTE)

Email discussion/approval by 3/1 on a TP capturing the following:

DMRS configurations: nSCIDnSCID,  msgA-maxLength and msgA-dmrs-AdditionalPosition in 38.211; operations related to msgA-maxLength  and exact DMRS port indexes in 38.214

R1-2001369	Outcome of email thread [100e-NR-2step-RACH-ChStructure-02]	ZTE

Decision: As per email decision posted on Mar. 2nd

Agreement:

- TP #1 and TP #2 in the Appendix of R1-2001369 to correct the operations of DMRS for msgA PUSCH in TS 38.211 Section 6.4.1 and TS 38.213 Section 6.2.2 respectively, are endorsed.

- Note: The following can be further review in the next meeting

  - If MsgA-maxLength is configured, whether the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter msgA-DMRS-AdditionalPosition, to be 'pos2', 'pos3'.

[100e-NR-2step-RACH-ChStructure-03] – Li (ZTE)

Email discussion/approval on a 38.213 TP clarifying shared RO part (msgA-ssb-sharedROmaskIndex) by 3/1

R1-2001370	Outcome of email thread [100e-NR-2step-RACH-ChStructure-03]	ZTE

Decision: As per email decision posted on Mar. 2nd

Agreement: TP#1 in the Appendix of R1-2001370 to capture the subset of RO sharing, and clarify the starting position of 2-step RACH preambles for the shared RO case in TS 38.213 Section 8.1 is endorsed.

R1-2001371	Editorial TPs for 2-step RACH	ZTE

R1-2000304	Remaining issues on channel structure for 2-step RACH	vivo

R1-2000476	Discussion on Channel Structure for Two-Step RACH	OPPO

R1-2000605	Channel Structure for Two-Step RACH	Samsung

R1-2000657	Remaining details of Channel Structure for 2-step RACH	LG Electronics

R1-2000719	Remaining details of channel structure for 2-step RACH	Intel Corporation

R1-2000819	Channel Structure Related Corrections For 2-Step RACH	Ericsson

R1-2000843	Remaining issues on channel structure for 2-step RACH	Apple

R1-2000910	Maintenance for Channel Structure for Two-Step RACH	NTT DOCOMO, INC.

R1-2001029	Corrections on channel structure of 2-step RACH	Huawei, HiSilicon

R1-2001073	RAN1 terminology alignment for two-step RACH	Nokia, Nokia Shanghai Bell

#### Procedure for Two-step RACH

R1-2000810	FL summary on the maintenance of the 2-step RACH procedures	ZTE

R1-2001207	FL summary#2 on the maintenance of the 2-step RACH procedures	ZTE

[100e-NR-2step-RACH-Procedure-01] – Li (ZTE)

Email discussion/approval on a TP capturing the related description of the 2bits LSB of SFN in 38.212 and 213 by 3/1

R1-2001372	Summary of email thread [100e-NR-2step-RACH-Procedure-01]	ZTE

Decision : As per email decision posted on Mar. 3rd, no consensus on how to capture a TP in RAN1 specs. R1-2001372 summarizes the discussion and the raised issues, and can be used as starting point for discussion in the next meeting.

[100e-NR-2step-RACH-Procedure-02] – Li (ZTE)

Email discussion/approval on a TP capturing the operation of power control after receiving msgB in 38.213 by 3/1

R1-2001373	Outcome of email thread [100e-NR-2step-RACH-Procedure-02]	ZTE

Decision : As per email decision posted on Mar. 2nd,

Agreement: TP#1 in the Appendix of R1-2001373 to capture the power control operation for PUCCH and PUSCH in TS 38.213 is endorsed.

[100e-NR-2step-RACH-Procedure-03] – Li (ZTE)

Email discussion/approval on a TP changing of RA-RNTI to msgB-RNTI by 2/26

R1-2001229	Outcome of email thread [100e-NR-2step-RACH-Procedure-03]	ZTE

Decision: According to RAN1 chair's decision posted on Feb. 27th:

- TP#1 in R1-2001229 Appendix to include MsgB-RNTI in TS 38.213 Section 8.2A and Section 10.1 is endorsed.

- TP#2 in R1-2001229 Appendix to include MsgB-RNTI in the title of Table 5.1.3.2-2 in TS38.214 is endorsed.

[100e-NR-2step-RACH-Procedure-04] – Li (ZTE)

Email discussion/approval processing time for MsgB HARQ-ACK by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001374	Outcome of email thread [100e-NR-2step-RACH-Procedure-04]	ZTE

Decision: According to RAN1 chair's decision posted on Feb. 29th:

Agreement:

- To determine the minimum gap between the last symbol of msgB PDSCH and the first symbol of PUCCH transmission carrying HARQ-ACK to msgB PDSCH for a UE

  - t =0.

  - TP #1 for TS 38.213 Section 8.2A in R1-2001374 is endorsed.

R1-2000305	Remaining issues on procedure for 2-step RACH	vivo

R1-2000397	Remaining issues of the 2-step RACH procedures	ZTE, Sanechips

R1-2000477	Remaining issues for procedure of 2-step RACH	OPPO

R1-2000581	Differentiating between MsgB carrying RRC and other messages	Sony

R1-2000606	Procedure for Two-step RACH	Samsung

R1-2000658	Remaining details of Procedure for 2-step RACH	LG Electronics

R1-2000720	Remaining details of procedure for 2-step RACH	Intel Corporation

R1-2000820	Procedure Related Corrections for 2-Step RACH	Ericsson

R1-2000844	Remaining issues on 2-step RACH procedure	Apple

R1-2000911	Maintenance for Procedure for Two-step RACH	NTT DOCOMO, INC.

R1-2001030	Corrections on procedure of 2-step RACH	Huawei, HiSilicon

#### Other

R1-2000659	Remaining details of designing 2-step RACH for NR-U	LG Electronics

R1-2000821	msgA PUSCH Configuration in 2-step CFRA	Ericsson

R1-2001102	Further considerations on 2-step RACH	Huawei, HiSilicon

### Maintenance of NR-based Access to Unlicensed Spectrum

RAN2 NR stage-2 CR (38.300) for Rel.16 to be submitted to March plenary for approval.

[100e-NR-unlic-NRU-38.300] – Jing (Qualcomm)

Email approval of RAN1 related update to 38.300 by 2/27

Decision: As per decision posted on Feb. 28th, RAN1 reviewed the RAN2 running CR for 38.300 [Rel.16] for NR-U related updates and endorsed the proposed changes in:

R1-2001299	RAN1 input on NR-U related changes for 38.300 running CR	Qualcomm (rapporteur)

Final LS to RAN2 is approved in:

R1-2001300	LS to RAN2 on NR-U related changes for 38.300 running CR	RAN1, Qualcomm

#### Physical Layer Signals and Channels

##### Initial access signals/channels

R1-2001150	FL summary of 7.2.2.1.1 NR-U initial access signals and channels	Qualcomm

[100e-NR-unlic-NRU-InitSignalsChannels-01] – Jing (Qualcomm)

Email discussion/approval the issue related to off-sync raster SSB to CORESET#0 offset by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001256	Outcome of email thread [100e-NR-unlic-NRU-InitSignalsChannels-01]	Qualcomm

Decision: As per decision posted on Mar. 3rd,

Agreement: The TP for TS38.213 Section 1.1 in R1-2001256 is endorsed.

[100e-NR-unlic-NRU-InitSignalsChannels-02] – Jing (Qualcomm)

Email discussion/approval the issue related to PBCH scrambling by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001257	Outcome of email thread [100e-NR-unlic-NRU-InitSignalsChannels-02]	Qualcomm

Decision: As per decision posted on Feb. 27th, the TPs in Section 7 of R1-2001257 for TS38.211, TS38.212 and TS38.213 are endorsed.

[100e-NR-unlic-InitSignalsChannels-03] – Jing (Qualcomm)

Email discussion/approval the issue related to CSI-RS transmission in DRS by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001258	Outcome of email thread [100e-NR-unlic-NRU-InitSignalsChannels-03]	Qualcomm

Decision: As per decision posted on Feb. 29th, the discussion was concluded without consensus and is summarized in R1-2001258.

R1-2000306	Remaining issues on initial access signals and channles	vivo

R1-2000393	Maintenance on the initial access signals for NR-U	ZTE, Sanechips

R1-2000418	Remaining issues on initial access signals/channels	Spreadtrum Communications

R1-2000467	Discussion on the remaining issues of initial access signal/channel	OPPO

R1-2000500	Remaining issues on Initial Access Signals and Channels for NR-U	Nokia, Nokia Shanghai Bell

R1-2000607	Initial access signals and channels for NR-U	Samsung

R1-2000660	Remaining issues of initial access signals and channels for NR-U	LG Electronics

R1-2000823	Initial access signals and channels	Ericsson

R1-2000953	TP for Initial access signals and channels for NR-U	Qualcomm Incorporated

R1-2001036	Corrections on the initial access signals and channels	Huawei, HiSilicon

##### DL signals and channels

R1-2001047	Feature lead summary for NR-U DL Signals and Channels	Lenovo, Motorola Mobility

R1-2001193	Feature lead summary#2 for NR-U DL Signals and Channels	Lenovo, Motorola Mobility

[100e-NR-unlic-NRU-DL_Signals_and_Channels-01] – Alex (Lenovo)

Email discussion/approval the issue related to COT duration indication by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001340	Feature Lead Summary: Email discussion [100e-NR-unlic-NRU-DL_Signals_and_Channels-01] on COT duration indication	Lenovo, Motorola Mobility

R1-2001387	Agreements from email discussion [100e-NR-unlic-NRU-DL_Signals_and_Channels-01] on COT duration indication	Lenovo, Motorola Mobility

Decision: Agreements are listed in section 1 of R1-2001387 as follows.

As per email decision posted on Mar. 2nd,

Agreement:

The length of each COT duration indicator is not configured explicitly by RRC, but is derived from the number X  of values configured in the co-DurationList-r16, e.g. length=ceil[log2(X)].

Agreement:

The size of the list of configured channel occupancy durations for a serving cell (in the field co-DurationList-r16) can range from 1 to 64 channel occupancy duration values (co-Duration-r16).

Agreement:

The value for a single channel occupancy duration value (co-Duration-r16) can range from 0 to 20 ms with a granularity of one symbol.

As per email decision posted on Mar. 4th, the following text proposal for TS 38.213 is endorsed:

--------------------------------------------- Start of Text Proposal ---------------------------------------

11.1.1	UE procedure for determining slot format

*** Unchanged text is omitted ***

For a set of symbols of a slot that are indicated by a DCI format 2_0 as being within a remaining channel occupancy duration either by a channel occupancy duration field or an SFI-index field, a UE does not expect to detect later a DCI format 2_0 indicating that any symbol from the set of symbols is not within a remaining channel occupancy duration either by a channel occupancy duration field or an SFI-index field.

*** Unchanged text is omitted ***

--------------------------------------------- End of Text Proposal ---------------------------------------

[100e-NR-unlic-NRU-DL_Signals_and_Channels-02] – Alex (Lenovo)

Email discussion/approval the issue related to available LBT bandwidth/RB set by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001341	Feature Lead Summary: Email discussion [100e-NR-unlic-NRU-DL_Signals_and_Channels-02] on available LBT bandwidth/RB set	Lenovo, Motorola Mobility

R1-2001388	Agreements from email discussion [100e-NR-unlic-NRU-DL_Signals_and_Channels-02] on available LBT bandwidth/RB set	Lenovo, Motorola Mobility

Decision: Agreements are listed in section 1 of R1-2001388 as follows.

As per email decision posted on Mar. 3rd,

Agreement:

- A UE may skip monitoring PDCCH in RB-set(s) indicated as unavailable.

Agreement:

For indicating available RB sets, the gNB configures positionInDCI, servingCellId. The bitmap size corresponds to number of RB-sets on a carrier.

As per email decision posted on Mar. 4th, the following text proposal for TS 38.213 is endorsed:

--------------------------------------------- Start of Text Proposal ---------------------------------------

10.1	UE procedure for determining physical downlink control channel assignment

<omitted text>

For the CSS sets, a UE monitors  PDCCH candidates requiring a total of  non-overlapping CCEs in a slot.

The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration  in a slot if the UE is not provided PDCCHMonitoringCapabilityConfig for the primary cell or if the UE is provided PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring capability for all serving cells, or in a span if the UE is provided PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring capability for the primary cell, according to the following pseudocode. If for the USS sets for scheduling on the primary cell the UE is not provided CORESETPoolIndex for first CORESETs, or is provided value 0 for first CORESETs, and is provided value 1 for second CORESETs, and if minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ>minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ or minγ∙CPDCCHmax,slot,μ,CPDCCHtotal,slot,μ>minCPDCCHmax,slot,μ,CPDCCHtotal,slot,μ, the following pseudocode applies only to USS sets associated with the first CORESETs. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring.

For operation with shared spectrum channel access, if a UE is provided with availableRB-SetPerCell-r16, the UE is not expected to monitor PDCCH candidates that overlap with any RB within a RB set indicated as not available for reception by DCI format 2_0.

--------------------------------------------- End of Text Proposal ---------------------------------------

[100e-NR-unlic-NRU-DL_Signals_and_Channels-03] – Alex (Lenovo)

Email discussion/approval the issue related to search space set monitoring/switching by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001342	Feature Lead Summary: Email discussion [100e-NR-unlic-NRU-DL_Signals_and_Channels-03] on search space set monitoring/switching	Lenovo, Motorola Mobility

R1-2001389	Agreements from email discussion [100e-NR-unlic-NRU-DL_Signals_and_Channels-03] on search space set monitoring/switching	Lenovo, Motorola Mobility

Decision: Agreements are listed in section 1 of R1-2001389 as follows.

Agreement:

Adopt the following TP for TS 38.212:

--------------------------------------------- Start of Text Proposal ---------------------------------------

7.3.1.3.1	Format 2_0

DCI format 2_0 is used for notifying the slot format, COT duration, available RB set, and search space group switching.

The following information is transmitted by means of the DCI format 2_0 with CRC scrambled by SFI-RNTI:

-	Slot format indicator 1, Slot format indicator 2, …, Slot format indicator N.

-	If the higher layer parameter availableRB-SetPerCell-r16 is configured,

-	Available RB set Indicator 1, Available RB set Indicator 2, …, Available RB set Indicator N1,

-	If the higher layer parameter CO-DurationPerCell-r16 is configured

-	COT duration indicator 1, COT duration indicator 2, …, COT duration indicator N2.

-	If the higher layer parameter searchSpaceSwitchTrigger-r16 is configured

-	Monitoring group flag 1, Monitoring group flag 2, …, Monitoring group flag [M].

The size of DCI format 2_0 is configurable by higher layers up to 128 bits, according to Clause 11.1.1 of [5, TS 38.213].

--------------------------------------------- End of Text Proposal ---------------------------------------

Agreement:

For search space switching, limit the switching to USS and Type-3 CSS.

Agreement:

Higher layer parameter searchSpaceSwitchingTimer can take values in the range of 1 to 20 ms.

Agreement:

For processing times for search space switching, P1 and P2 have the same value.

Agreement:

Adopt the following TP for Section 10.4 of TS38.213

--------------------------------------------- Start of Text Proposal ---------------------------------------

10.4	Search space set switching

A UE can be provided a group index for a respective search space set by searchSpaceGroupIdList-r16 for PDCCH monitoring on a serving cell. If the UE is not provided searchSpaceGroupIdList-r16 for a search space set, the following procedures are not applicable for PDCCH monitoring according to the search space set.

When a UE is provided with searchSpaceSwitchingGroupList-r16, indicating one or more groups of serving cells, following procedures apply to the all of serving cells within each group. Otherwise, following procedures apply only to a serving cell on which the UE is configured with searchSpaceGroupIdList-r16.

A UE can be provided, by searchSpaceSwitchingTimer-r16, a timer value. The UE decrements the timer value by one after each slot in the active DL BWP of the serving cell where the UE monitors PDCCH for detection of DCI format 2_0.

If a UE is provided by SearchSpaceSwitchTrigger-r16 a location of a search space set switching field for a serving cell in a DCI format 2_0, as described in Clause 11.1.1, and detects DCI format 2_0 in a slot

-	if the UE is not monitoring PDCCH according to search space sets with group index 0, the UE starts monitoring PDCCH according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, on the serving cell at a first slot that is at least P1 symbols after the slot in the active DL BWP of the serving cell, if a value of the search space set switching field is 0

-	if the UE is not monitoring PDCCH according to search space sets with group index 1, the UE monitors PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, on the serving cell at a first slot that is at least P1 symbols after the slot in the active DL BWP of the serving cell, and the UE sets the timer value to the value provided by searchSpaceSwitchingTimer-r16, if a value of the search space set switching field is 1

-	if the UE monitors PDCCH on a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH on the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, on the serving cell at the beginning of the first slot that is at least P1 symbols after a slot where the timer expires or after a last slot of a remaining channel occupancy duration for the serving cell that is indicated by DCI format 2_0

If a UE is not provided SearchSpaceSwitchTrigger-r16 for a serving cell,

-	if the UE detects a DCI format by monitoring PDCCH according to a search space set with group index 0 in a slot, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, on the serving cell at a first slot that is at least P2 symbols after the slot in the active DL BWP of a serving cell, the UE sets the timer value to the value provided by searchSpaceSwitchingTimer-r16 if the UE detects a DCI format by monitoring PDCCH in any search space set

-	if the UE monitors PDCCH on a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH on the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, on the serving cell at the beginning of the first slot that is at least P2 symbols after a slot where the timer expires or, if the UE is provided a search space set to monitor PDCCH for detecting a DCI format 2_0, after a last slot of a remaining channel occupancy duration for the serving cell that is indicated by DCI format 2_0

--------------------------------------------- End of Text Proposal ---------------------------------------

Agreement:

Adopt the following text proposal for TS38.213 section 11.5.2.

- Note: This text proposal captures the aspects for LBE and further discussion is needed for FBE aspects

--------------------------------------------- Start of Text Proposal ---------------------------------------

11.5.2	Search space set switching

A UE can be provided a group index for a respective search space set by searchSpaceGroupIdList-r16 for PDCCH monitoring on a serving cell indicated by searchSpaceSwitchingGroup-r16. If the UE is not provided searchSpaceGroupIdList-r16 for a search space set, or for PDCCH monitoring on a serving cell that is not indicated by searchSpaceSwitchingGroup-r16, the following procedures are not applicable for PDCCH monitoring according to the search space set.

A UE can be provided, by searchSpaceSwitchingTimer-r16, a timer value. The UE decrements the timer value by one after each slot in the active DL BWP of the serving cell where the UE monitors PDCCH for detection of DCI format 2_0.

If a UE is provided by SearchSpaceSwitchTrigger-r16 a location of a search space set switching field for a serving cell in a DCI format 2_0, as described in Clause 11.1.1, and detects DCI format 2_0 in a slot

-	if the UE is not monitoring PDCCH according to search space sets with group index 0, the UE starts monitoring PDCCH according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, on the serving cell at a first slot that is at least P1 symbols after the last symbol of the PDCCH carrying the detected DCI format 2_0, if a value of the search space set switching field is 0

-	if the UE is not monitoring PDCCH according to search space sets with group index 1, the UE monitors PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, on the serving cell at a first slot that is at least P1 symbols after the last symbol of the PDCCH carrying the detected DCI format 2_0, and the UE sets the timer value to the value provided by searchSpaceSwitchingTimer-r16, if a value of the search space set switching field is 1

-	if the UE monitors PDCCH on a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH on the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, on the serving cell at the beginning of the first slot that is at least P1 symbols after a slot where the timer expires or after a last symbol of a remaining channel occupancy duration for the serving cell that is indicated by DCI format 2_0

If a UE is not provided SearchSpaceSwitchTrigger-r16 for a serving cell,

-	if the UE detects a DCI format by monitoring PDCCH according to a search space set with group index 0, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, on the serving cell at a first slot that is at least P2 symbols after the last symbol of the PDCCH carrying the detected DCI format, the UE sets the timer value to the value provided by searchSpaceSwitchingTimer-r16 if the UE detects a DCI format by monitoring PDCCH in any search space set

-	if the UE monitors PDCCH on a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH on the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, on the serving cell at the beginning of the first slot that is at least P2 symbols after a slot where the timer expires or, if the UE is provided a search space set to monitor PDCCH for detecting a DCI format 2_0, after a last symbol of a remaining channel occupancy duration for the serving cell that is indicated by DCI format 2_0

--------------------------------------------- End of Text Proposal ---------------------------------------

[100e-NR-unlic-NRU-DL_Signals_and_Channels-04] – Alex (Lenovo)

Email discussion/approval the issue related to dropping/shifting of DM-RS by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001343	Feature Lead Summary: Email discussion [100e-NR-unlic-NRU-DL_Signals_and_Channels-04] on dropping/shifting of DM-RS	Lenovo, Motorola Mobility

R1-2001390	Agreements from email discussion [100e-NR-unlic-NRU-DL_Signals_and_Channels-04] on dropping/shifting of DM-RS	Lenovo, Motorola Mobility

Decision: Agreements are listed in section 1 of R1-2001390 as follows.

Agreement:

Adopt the following TP for TS 38.211:

--------------------------------------------- Start of Text Proposal ---------------------------------------

7.4.1.1.2	 Mapping to physical resources

<Unchanged part omitted>

For PDSCH mapping type B

-	if the PDSCH duration ld ∈2,3,4,5,6,7,8,9,10,11,12,13 OFDM symbols for normal cyclic prefix or ld∈2,4,6 OFDM symbols for extended cyclic prefix, and the front-loaded DM-RS of the PDSCH allocation collides with resources reserved for a search space set associated with a CORESET,  shall be incremented such that the first DM-RS symbol occurs immediately after the CORESET and until no collision with any CORESET occurs, and

-	if the PDSCH duration ld is 2 symbols, the UE is not expected to receive a DM-RS symbol beyond the second symbol,

-    if the PDSCH duration ld is 5 symbols, if one additional single-symbol DMRS is configured, the UE expects the additional DM-RS to be transmitted on the fifth symbol,

-	if the PDSCH duration ld is 7 symbols for normal cyclic prefix or 6 symbols for extended cyclic prefix:

-	the UE is not expected to receive the front-loaded DM-RS beyond the fourth symbol, and

-	if one additional single-symbol DM-RS is configured, the UE only expects the additional DM-RS to be transmitted on the 5th or 6th symbol when the front-loaded DM-RS symbol is in the 1st or 2nd symbol, respectively, of the PDSCH duration, otherwise the UE should expect that the additional DM-RS is not transmitted;

-	if the PDSCH duration ld is 12 or 13 symbols, the UE is not expected to receive a DM-RS symbol mapped to symbol 12 or later in the slot;

-	for all values of the PDSCH duration ld other than 2, 5, and 7 symbols, the UE is not expected to receive a DM-RS symbol beyond the (ld-1):th symbol;

-	if the PDSCH duration ld is less than or equal to 4 OFDM symbols, only single-symbol DM-RS is supported.

-	if the higher-layer parameter lte-CRS-ToMatchAround or additionalLTE-CRS-ToMatchAroundList is configured, the PDSCH duration ld=10 symbols for normal cyclic prefix, the subcarrier spacing configuration μ=0, single-symbol DM-RS is configured, and at least one PDSCH DM-RS symbol in the PDSCH allocation collides with a symbol containing resource elements as indicated by the higher-layer parameter lte-CRS-ToMatchAround or additionalLTE-CRS-ToMatchAroundList, then l shall be incremented by one in all slots.

<Unchanged part omitted>

--------------------------------------------- End of Text Proposal ---------------------------------------

R1-2000195	Corrections on PDCCH/GC-PDCCH monitoring procedure in TS38.213	Huawei, HiSilicon

R1-2000307	Remaining issues on physical DL channel design in unlicensed spectrum	vivo

R1-2000394	Maintenance on the DL channels for NR-U	ZTE, Sanechips

R1-2000435	Remaining issues on DL signals and channels for NR-U operation	MediaTek Inc.

R1-2000468	Discussion on the remaining issues of DL signals and channels	OPPO

R1-2000501	Remaining issues on DL signals and channels	Nokia, Nokia Shanghai Bell

R1-2000505	Text proposal on Rel-16 PDSCH DMRS	Panasonic Corporation

R1-2000608	DL signals and channels for NR-U	Samsung

R1-2000661	Remaining issues of DL signals and channels for NR-U	LG Electronics

R1-2000723	DL signals and channels for NR-unlicensed	Intel Corporation

R1-2000769	Remaining issues on DL signals and channels for NR-U	ETRI

R1-2000824	DL signals and channels	Ericsson

R1-2000871	Remaining issues and corrections on DL signals and channels for NR-U	Sharp

R1-2000954	TP for DL signals and channels for NR-U	Qualcomm Incorporated

R1-2001001	Remaining issues on search space set switching	Google Inc.

##### UL signals and channels

R1-2000831	Feature Lead Summary for UL Signals and Channels	Ericsson

R1-2001203	Feature Lead Summary#2 for UL Signals and Channels	Ericsson

[100e-NR-unlic-NRU-ULSignalsChannels-01] – Steve (Ericsson)

Email discussion/approval on the issues related FDRA for DCI 0_0/0_1 by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001308	Outcome of email thread [100e-NR-unlic-NRU-ULSignalsChannels-01]	Ericsson

Agreement:

Modify the agreement from RAN1#99 as follows:

- The UE transmits PUSCH scheduled by fallback DCI in CSS within the initial BWP on the interlaces indicated by the X bits of the FDRA field

- Note: The FDRA field for fallback DCI in CSS does not include Y bits

Agreement:

The following text proposal is agreed:

-------------------------------------- Text Proposal for 38.212, Section 7.3.1.1.2 ---------------------------------------

*** Unchanged text omitted ***

-	If the higher layer parameter useInterlacePUSCH-Dedicated-r16 is configured

-	5 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 30 kHz. The 5 MSBs provide the interlace allocation and the Y LSBs provide the RB set allocation.

-	6 + Y bits provide the frequency domain resource allocation according to Clause 6.1.2.2.3 of [6, TS 38.214] if the subcarrier spacing for the active UL bandwidth part is 15 kHz. The 6 MSBs provide the interlace allocation and the Y LSBs provide the RB set allocation.

The value of Y is determined by log2NN+12 where N is the number of RB sets contained in the BWP as defined in clause x of [x].

*** Unchanged text omitted ***

------------------------------------------------------ End Text Proposal -------------------------------------------------------

Agreement:

For FDRA field for DCI 0_0, update the spec to include at least X bits to indicate the interlace allocation, and leave in square brackets whether or not Y bits are included for indication of RB set.

Agreement:

For DCI 0_1 if the bandwidth indicator field indicates a switch to a new active BWP with a different configuration (# RB sets and/or SCS), the UE applies truncation/padding independently to the X and Y bits of the FDRA field to determine the interlace and RB set allocation for the new active BWP

- X bit field: Truncated/padded to X’ = 5/6 bits if the new active BWP has 30/15 kHz SCS and current BWP has 15/30 kHz SCS. No truncation/padding otherwise.

- Y bit field: Truncated/padded to Y’ bits if the new active BWP has different number of RB sets than the current BWP, where Y’ is determined by log2NN+12 and N is the number of RB sets contained in the new active BWP. No truncation/padding otherwise.

Agreement: (TPs for above two agreements)

The text proposals for TS38.212 and TS38.214 in Section 4 of R1-2001308 are endorsed.

[100e-NR-unlic-NRU-ULSignalsChannels-02] – Steve (Ericsson)

Email discussion/approval on the issue related to interlace allocation for 15kHz SCS and for DFT-s-OFDM by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001309	Outcome of email thread [100e-NR-unlic-NRU-ULSignalsChannels-02]	Ericsson

Agreement:

- For Type 2 UL resource allocation for PUSCH configured with DFT-s-OFDM, if the total number of allocated RBs in the indicated interlaces and indicated RB set(s) is not of the form 2a×3b×5c where a, b, c are non-negative integers, the UE transmits PUSCH only on the lowest indexed N PRBs of the allocation where N is the largest integer of the form 2a×3b×5c which is not greater than the number of allocated RBs.

- TP1-a in Section 4 of R1-2001309 for TS 38.214 is endorsed

[100e-NR-unlic-NRU-ULSignalsChannels-03] – Steve (Ericsson)

Email discussion/approval on the issue related to PUCCH (FDRA & rate matching) by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001310	Outcome of email thread [100e-NR-unlic-NRU-ULSignalsChannels-03]	Ericsson

Agreement:

The text proposals in Section 3 of R1-2001310 for TS 38.211, TS38.212 and TS38.213 are endorsed.

R1-2000196	Corrections on uplink signals and channels	Huawei, HiSilicon

R1-2000308	Remaining issues on physical UL channel design in unlicensed spectrum	vivo

R1-2000410	Remaining Issues on UL Signals and Channels for NR-U	Nokia, Nokia Shanghai Bell

R1-2000436	Remaining issues on UL signals and channels for NR-U operation	MediaTek Inc.

R1-2000469	Discussion on the remaining issues of UL signals and channels	OPPO

R1-2000591	Text proposals for UL signals and channels for NR-U	Lenovo, Motorola Mobility

R1-2000609	UL signals and channels for NR-U	Samsung

R1-2000647	TP for SRS configuration	CATT

R1-2000662	Remaining issues of UL signals and channels for NR-U	LG Electronics

R1-2000770	UL signals and channels	ETRI

R1-2000825	UL signals and channels	Ericsson

R1-2000872	Remaining issues and corrections on UL signals and channels for NR-U	Sharp

R1-2000955	TP for UL signals and channels for NR-U	Qualcomm Incorporated

#### Physical Layer Procedure

##### Channel access procedures

R1-2000799	Feature Lead’s Summary on Channel Access Procedures  	Nokia

R1-2001202	Feature Lead’s Summary#2 on Channel Access Procedures	Nokia

[100e-NR-unlic-NRU-ChAccess-01] – Timo (Nokia)

Email discussion/approval on Value Range for maxEnergyDetectionThreshold-r16 and ULtoDL-CO-SharingED-Threshold-r16 by 2/28

R1-2001311	Outcome of email discussions [100e-NR-unlic-NRU-ChAccess-01]	Nokia

Decision: As per email decision posted on Feb. 29th,

Agreement:

- The value range for RRC parameters maxEnergyDetectionThreshold-r16 and ULtoDL-CO-SharingED-Threshold-r16 is defined as {-85 ... -52} dBm, with a step size of 1 dB

[100e-NR-unlic-NRU-ChAccess-02] – Timo (Nokia)

Email discussion/approval on values for C2 and C3 in CP extension prior to dedicated RRC configuration and in CBRA in connected state, as well as possible update to the value range by 2/28

R1-2001312	Outcome of email discussions [100e-NR-unlic-NRU-ChAccess-02]	Nokia

Further revised as the outcome in R1-2001312 was unfortunately erroneous (some of the contents were duplicated from R1-2001313).

R1-2001482	Outcome of email discussions [100e-NR-unlic-NRU-ChAccess-02]	Nokia

Decision: As per email decision posted on Feb. 29th,

Agreement:

For UL transmissions prior to dedicated RRC configuration or in CBRA, C2 and C3 before RRC configuration are set to the maximum integers which satisfy following:

- C2*symbol length – 16 us – TA < symbol length

- C3*symbol length – 25 us – TA < symbol length

Agreement:

The value ranges for C2 and C3 are updated as follows:

- C2 values

  - 1, 2, 3, .. , 28	for 15/30 kHz SCS

  - 2, 3,  .., 28	for 60 kHz SCS

- C3 values

  - 1, 2, 3, .. , 28	for 15 kHz SCS

  - 2, 3, .. , 28	for 30 kHz SCS

  - 3, 4, .. , 28	for 60 kHz SCS

[100e-NR-unlic-NRU-ChAccess-03] – Timo (Nokia)

Email discussion/approval on issues related tp CP extension (1st symbol of a slot, application of PUCCH) by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001313	Outcome of email discussions [100e-NR-unlic-NRU-ChAccess-03]	Nokia

Decision: As per email decision posted on Feb. 29th,

Agreement:

- For CP extension for the 1st symbol of a slot, the TPs in R1-2000767 are taken at least as a starting point for correcting the length of CP extension in 38.211 and 38.212. Consider also at least the following comment:

  - l-k mod 7∙2μ should be changed to (l-k) mod 7∙2μ to avoid calculating negative l values by misinterpretation.

- Can further consider how to also support CP extension values (to be) agreed for Configured Grant transmissions.

Agreement:

For application of CP extension for PUCCH in TS 38.211, the TP in R1-2000437, Section 2, is approved after with a correction for the typo for “clasue”

As per email decision posted on Mar. 4th,

R1-2001396	TPs for CP extension from email discussion [100e-NR-unlic-NRU-ChAccess-03]	Nokia

Agreement:

For CP extension, text proposal 1 for TS 38.211 and text proposals 2, 3 and 4 for TS 38.212 in R1-2001396 are endorsed.

[100e-NR-unlic-NRU-ChAccess-04] – Timo (Nokia)

Email discussion/approval regarding RAN2 LS on UL LBT failure indication by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001314	Outcome of email discussions [100e-NR-unlic-NRU-ChAccess-04]	Nokia

Decision: As per email decision posted on Mar. 4th, final reply LS on consistent Uplink LBT failure detection mechanism is approved in R1-2001397.

R1-2000197	Corrections on the channel access procedure in TS 37.213	Huawei, HiSilicon

R1-2000309	Remaining issues on the channel access procedures	vivo

R1-2000392	Maintenance on the channel access procedure for NR-U	ZTE, Sanechips

R1-2000411	Remaining Issues on Channel Access Procedures for NR-U	Nokia, Nokia Shanghai Bell

R1-2000437	Remaining issues on channel access for NR-U operation	MediaTek Inc.

R1-2000470	Discussion on the remaining issues of channel access procedure	OPPO

R1-2000544	Remaining issues on channel access procedures	Fujitsu

R1-2000610	Channel access procedures for NR-U	Samsung

R1-2000663	Remaining issues of channel access procedure for NR-U	LG Electronics

R1-2000724	Channel access mechanism for NR-unlicensed	Intel Corporation

R1-2000767	Remaining issues on CP extension	NEC

R1-2000826	Channel access procedures	Ericsson

R1-2000873	Remaining issues and corrections on channel access procedure for NR-U	Sharp

R1-2000912	Channel access procedures for NR-U	NTT DOCOMO, INC.

R1-2000934	Remaining issues on channel access procedure for NR-U	WILUS Inc.

R1-2000956	TP for Channel access procedures for NR unlicensed	Qualcomm Incorporated

R1-2001088	Corrections to channel access procedures	Futurewei

##### Enhancements to initial access procedure

R1-2001144	Feature lead summary for NR-U initial access procedures	Charter Communications

[100e-NR-unlic-NRU-InitAccessProc-01] – Amitav (Charter)

Email discussion/approval regarding SSB position validation for FBE scenario, clarification on interpretation of ssb-PositionsInBurst for UE procedure for receiving control information, and LS to RAN4 on candidate SS/PBCH block index and SS/PBCH block index terminology for alignment by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001315	Outcome of email thread [100e-NR-unlic-NRU-InitAccessProc-01]	Charter Communications

Conclusion:

For semi-static channel access, SSBs that (partially) fall in the idle region of a fixed frame period should be considered as invalid. No PDSCH rate matching and no RLM/RRM measurement will be done for those candidate SSB positions.

R1-2001357	LS on SSB index and candidate SSB index for NR-U	RAN1, Samsung, Charter Communications

Decision: LS to RAN2/4 on SSB index and candidate SSB index for NR-U is approved.

Agreement:

Adopt the following TP for TS 38.213:

--------------------------------------------- Start of Text Proposal ---------------------------------------

10	UE procedure for receiving control information

<Unchanged part omitted>

For monitoring of a PDCCH candidate by a UE in a slot or in a span, if the UE

- has received ssb-PositionsInBurst in SIB1 and has not received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and

- does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and

- at least one RE for a PDCCH candidate overlaps with at least one RE corresponding to candidate SS/PBCH blocks corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in SIB1,

the UE is not required to monitor the PDCCH candidate.

For monitoring of a PDCCH candidate by a UE in a slot, if the UE

- has received ssb-PositionsInBurst in ServingCellConfigCommon for a serving cell, and

- does not monitor PDCCH candidates in a Type0-PDCCH CSS set, and

- at least one RE for a PDCCH candidate overlaps with at least one RE corresponding to candidate SS/PBCH blocks corresponding to a SS/PBCH block index provided by ssb-PositionsInBurst in ServingCellConfigCommon,

the UE is not required to monitor the PDCCH candidate.

<Unchanged part omitted>

--------------------------------------------- End of Text Proposal ---------------------------------------

[100e-NR-unlic-NRU-InitAccessProc-02] – Amitav (Charter)

Email discussion/approval regarding issues related to RA procedure including: HARQ-ACK LBT parameter signaling in success RAR for 2-step RACH procedure and Configuration of RNTI and DCI payload for IDLE UE on FBE cell to permit PRACH within FFP by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001316	Outcome of email thread [100e-NR-unlic-NRU-InitAccessProc-02]	Charter Communications

Agreement:

LBT parameter applied for HARQ-ACK transmission corresponding to Msg. B reception is provided via each success RAR in Msg. B PDSCH.

Agreement:

The following text proposal for TS 38.213 is adopted.

--------------------------------------------- Start of Text Proposal ---------------------------------------

8.2A	Random access response - Type-2 random access procedure

In response to a transmission of a PRACH and a PUSCH, a UE attempts to detect a DCI format 1_0 with CRC scrambled by a corresponding RA-RNTI during a window controlled by higher layers [11, TS 38.321]. The window starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS set, as defined in Clause 10.1, that is at least one symbol, after the last symbol of the PUSCH occasion corresponding to the PUSCH transmission, where the symbol duration corresponds to the SCS for Type1-PDCCH CSS set. The length of the window in number of slots, based on the SCS for Type1-PDCCH CSS set, is provided by ra-ResponseWindow.

If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding RA-RNTI, and a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer

- an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in Clause 8.2 when the UE detects a RAR UL grant, or

- transmission of a PUCCH with HARQ-ACK information having ACK value if the RAR message(s) is for successRAR, where

- a PUCCH resource for the transmission of the PUCCH is indicated by PUCCH resource indicator field of 4 bits in the successRAR from a PUCCH resource set that is provided by pucch-ResourceCommon

- a slot for the PUCCH transmission is indicated by a PDSCH-to-HARQ_feedback timing indicator field of 3 bits in the successRAR having a value k from {1, 2, 3, 4, 5, 6, 7, 8} and, with reference to slots for PUCCH transmission having duration Tslot, the slot is determined as ceiln+k+∆+t∆Tslot where n is a slot of the PDSCH reception, ∆ is as defined for PUSCH transmission in Table 6.1.2.1.1-5 of [6, TS 38.214], and t∆≥0

- the UE does not expect the first symbol of the PUCCH transmission to be after the last symbol of the PDSCH reception by a time smaller than NT,1+0.5+t∆ msec where NT,1 is the PDSCH processing time for UE processing capability 1 [6, TS 38.214]

- a channel access type and CP extension for PUCCH transmission is indicated by ChannelAccess-CPext field of 2 bits in the successRAR for operation with shared spectrum channel access [15, TS 37.213]

- the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission

If the UE detects the DCI format 1_0 with CRC scrambled by a C-RNTI and a transport block in a corresponding PDSCH within the window, the UE transmits a PUCCH with HARQ-ACK information having ACK value if the UE correctly detects the transport block or NACK value if the UE incorrectly detects the transport block and the time alignment timer is running [11, TS 38.321].

---------------------------- Unchanged text omitted---------------------------

--------------------------------------------- End of Text Proposal ---------------------------------------

Agreement:

The following text proposal for TS 37.213 is adopted

--------------------------------------------- Start of Text Proposal ---------------------------------------

4.2.1	Channel access procedures for uplink transmission(s)

[Unchanged text omitted]

A UE shall use Type 1 channel access procedures for PUCCH transmissions unless stated otherwise in this subclause. If a DL grant or a RAR message for successRAR scheduling a PUCCH transmission indicates Type 2 channel access procedures, the UE shall use Type 2 channel access procedures.

[Unchanged text omitted]

--------------------------------------------- End of Text Proposal ---------------------------------------

[100e-NR-unlic-NRU-InitAccessProc-03] – Amitav (Charter)

Email discussion/approval regarding issues related to RRM/RLM including:

- RSSI measurement duration

- RMTC configuration in RRC signalling and FFS points for value ranges compared to LAA LTE

- The need to replace Lmax with L ̅_max and add values 10, 20 for RLM in TS 38.213 subclause 5

by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001317	Outcome of email thread [100e-NR-unlic-NRU-InitAccessProc-03]	Charter Communications

Agreement:

The RMTC-Config can indicate a reference SCS and CP as one of {15 kHz, 30 kHz, 60 kHz-NCP, 60 kHz-ECP}

Agreement:

The L1 averaging duration of RSSI measurements (within a configured measurement duration) is limited to 1 OFDM symbol of a configured reference subcarrier spacing.

Agreement:

Keep value range for rmtc-Period-r16 and rmtc-SubframeOffset-r16 the same as for LTE-LAA.

Agreement:

Keep value range for rssi-Result-r16 and channelOccupancyThreshold-r16 the same as for LTE-LAA and inform RAN2 of this decision.

Agreement:

For RSSI (and channel occupancy measurements) layer 3 filtering is not applied.

R1-2001375	LS on NR-U enhancements to initial access procedures	RAN1, Charter Communications

Decision: LS on NR-U enhancements to initial access procedures is approved.

R1-2000198	Corrections on the RACH procedure in TS 38.213	Huawei, HiSilicon

R1-2000310	Remaining issues on initial access procedure for NR-U	vivo

R1-2000395	Maintenance on the initial access procedure for NR-U	ZTE, Sanechips

R1-2000419	Remaining issues on initial access procedure	Spreadtrum Communications

R1-2000438	Remaining issues on initial access procedure for NR-U operation	MediaTek Inc.

R1-2000471	Discussion on the remaining issues of enhancements to initial access procedure	OPPO

R1-2000499	On Enhancements to Initial Access Procedures	Nokia, Nokia Shanghai Bell

R1-2000611	Initial access procedures for NR-U	Samsung

R1-2000664	Remaining issues of initial access and mobility for NR-U	LG Electronics

R1-2000725	Enhancements to initial access and mobility for NR-unlicensed	Intel Corporation

R1-2000771	Remaining issues on initial access procedure for NR-U	ETRI

R1-2000827	Enhancements to initial access procedures	Ericsson

R1-2000957	TP for Initial access and mobility procedures for NR-U	Qualcomm Incorporated

##### HARQ enhancement

R1-2001043	Feature lead summary#1 on NR-U HARQ and multi-PUSCH scheduling	Huawei

R1-2001191	Feature lead summary#2 on NR-U HARQ and multi-PUSCH scheduling	Huawei

[100e-NR-unlic-NRU-HARQandULscheduling-01] – David (Huawei)

Email discussion/approval on the issues related to enhanced HARQ-ACK codebook, including

- Handling of non-numerical K1 (NNK1) value (TS38.213 clause 9.1.3)

- TS38.213 clause 9.1.3.3: UL DAI for dynamic HARQ-ACK codebook feedback in PUSCH

- TS38.213 clause 9.1.3.3: alignment of 38.213 with 38.212

- Implementation of agreement on handling reception of only DCI format 1_0 is incomplete

- TS38.213 clause 9.1.3.3: when 2 T-DAI values are signaled in DCI 1_1 the second total DAI field should be used to set the T-DAI value for a non-scheduled PDSCH group

by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001269	Feature lead summary of email discussion 100e-NR-unlic-NRU-HARQandULscheduling-01 (enhanced Type-2 HARQ-ACK codebook)	Huawei

Agreement:

Proposals 1-5 in R1-2001269 are agreed. TPs 1-4 for TS38.213 in Section 7 of R1-2001269 are endorsed.

[100e-NR-unlic-NRU-HARQandULscheduling-02] – David (Huawei)

Email discussion/approval on the issues related to Type-3 HARQ-ACK codebook, including

- Problem in the pseudo-code when the number of code blocks of one TB is smaller than the configured maximum number of CBGs per TB (when a TB consists of N code blocks, N<M)

- Corrections on HARQ-ACK state reset in case NDI is not reported in the feedback

by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001268	Feature lead summary of email discussion 100e-NR-unlic-NRU-HARQandULscheduling-02 (Type-3 HARQ-ACK codebook)	Huawei

Agreement:

Proposals 1 and 2 in R1-2001268 are agreed and the corresponding text proposals 1 and 2 for TS 38.213 in Section 4 of R1-2001268 are endorsed.

[100e-NR-unlic-NRU-HARQandULscheduling-03] – David (Huawei)

Email discussion/approval on the issues related to multi-PUSCH scheduling with DCI 0_1, including

- Resolving square brackets on 1-bit RV value

- Missing specification of mapping order of indicated HARQ IDs, NDIs, RVs to the multiple PUSCHs

by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001270	Feature lead summary of email discussion 100e-NR-unlic-NRU-HARQandULscheduling-03 (multi-PUSCH scheduling with DCI 0_1)	Huawei

Agreement:

Specify RV = 2 for the second value in Table 7.3.1.1.2-34 of 38.212. The TP #C1 in R1-2001270 is endorsed.

Agreement:

- TS38.214 and TS38.212 editors to implement the missing RAN1#97 agreement:

  - HARQ process ID signaled in the DCI applies to the first scheduled PUSCH. HARQ process ID is then incremented by 1 for subsequent PUSCHs in the scheduled order (with modulo operation as needed)

- The corresponding ordering of bits in RV and NDI fields also needs to be specified for the multiple PUSCHs, taking into account BWP switching where MSB are added with zeros.

R1-2000199	Corrections on HARQ-ACK feedback	Huawei, HiSilicon

R1-2000311	Remaining issues on HARQ operation for NR-U	vivo

R1-2000391	Maintenance on the HARQ for NR-U	ZTE, Sanechips

R1-2000412	Discussion on remaining issues on HARQ enhancement for NR-U	Beijing Xiaomi Mobile Software

R1-2000439	Remaining issues on Enhancements to HARQ for NR-U operation	MediaTek Inc.

R1-2000472	Discussion on the remaining issues of HARQ enhancements	OPPO

R1-2000503	Remaining issues on NR-U HARQ scheduling and feedback	Nokia, Nokia Shanghai Bell

R1-2000592	Text proposals for HARQ enhancement for NR-U	Lenovo, Motorola Mobility

R1-2000612	HARQ enhancement for NR-U	Samsung

R1-2000665	Remaining issues of HARQ procedure for NR-U	LG Electronics

R1-2000726	Enhancements to HARQ for NR-unlicensed	Intel Corporation

R1-2000828	HARQ enhancement	Ericsson

R1-2000849	Remaining issues on HARQ enhancement	Apple

R1-2000874	Remaining issues and corrections on HARQ enhancement for NR-U	Sharp

R1-2000958	TP for Enhancements to Scheduling and HARQ Operation for NR-U	Qualcomm Incorporated

R1-2000997	Correction on HARQ process	ASUSTeK

R1-2001000	Remaining issues on NR-U HARQ procedures	Google Inc.

##### Configured grant enhancement

R1-2000800	Feature lead summary on NRU configured grant enhancement	vivo

R1-2001166	Summary of 100e-Prep-email discussion on NRU configured grant enhancement	vivo

[100e-NR-unlic-NRU-CG-01] – Rakesh (vivo)

Email discussion/approval to finalize RRC value ranges for multiple RRC parameters related to CG by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001252	FL summary of [100e-NR-unlic-NRU-CG-01] email thread	vivo

Decision: As per email decision posted on Mar. 4th,

Agreement:

The set of Beta-offset values used for CG-UCI is the same as the set of Beta-offset values used for HARQ-ACK

- When CG-UCI is jointly encoded with HARQ, use the Beta-offset values configured for HARQ-ACK

Agreement:

The value range for cg-nrofSlots-r16 is {1, 2, …, 40} slots

Agreement:

Value ranges for cg-minDFIDelay-r16 in symbols with the step size of [14] symbols except for the first two values for different SCS are {[7], [14], [28], …, M} where M is as follows:

- 1 *4 * 14 = 56 (symbols)} for 15 kHz SCS

- 2 * 4 * 14 = 112 (symbols)} for 30 kHz SCS

- 4 * 4 * 14 = 224 (symbols) for 60 kHz SCS

Agreement:

For values of CP extension 7 possible starting positions are introduced

- The indices to 7 possible starting positions will be specified

- A UE is configured with indices to values from the indices to the 7 values.

[100e-NR-unlic-NRU-CG-02] – Rakesh (vivo)

Email discussion/approval on the issues related to PUSCH repetition transmission by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001253	FL summary of [100e-NR-unlic-NRU-CG-02] email thread	vivo

R1-2001411	Outcome of discussion on [100e-NR-unlic-NRU-CG-02] – TP for 38.214	vivo

Decision: As per email decision posted on Mar. 4th,

Agreement:

The following text proposal is endorsed for TS 38.214

------------------------------------------- Begin of TP for 38.214 ----------------------------------------------------

6.1.2.3.1 Transport Block repetition for uplink transmissions of PUSCH repetition Type A with a configured grant

<Unchanged parts omitted>

For any RV sequence, the repetitions shall be terminated after transmitting K repetitions, or at the last transmission occasion among the K repetitions within the period P, or from the starting symbol of the repetition that overlaps with a PUSCH with the same HARQ process scheduled by DCI format 0_0, 0_1 or 0_2, whichever is reached first. In addition, the UE shall terminate the repetition of a transport block in a PUSCH transmission if the UE receives a DCI format 0_1 with DFI flag provided and set to ¨1¨, and if within this DCI the UE detects ACK for the HARQ process corresponding to that transport block.

The UE is not expected to be configured with the time duration for the transmission of K repetitions larger than the time duration derived by the periodicity P. If the UE determines that, for a transmission occasion, the number of symbols available for the PUSCH transmission in a slot is smaller than transmission duration L, the UE does not transmit the PUSCH in the transmission occasion.

<Unchanged parts omitted>

------------------------------------------- End of TP for 38.214 ----------------------------------------------------

R1-2001295	Editorial corrections (NR-U)	vivo

Decision: R1-2001295 includes editorial corrections for 38.212 and 38.214 to be forwarded to editors

[100e-NR-unlic-NRU-CG-03] – Rakesh (vivo)

Email discussion/approval on the issues related to the timing of CG-DFI and termination of PUSCH repetition and PDCCH validation for DL SPS and UL grant Type 2 by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001254	FL summary of [100e-NR-unlic-NRU-CG-03] email thread	vivo

Decision: As per email decision posted on Mar. 4th,

R1-2001412	Outcome of discussion on [100e-NR-unlic-NRU-CG-03] – TP for 38.214	vivo

Agreement:

The following text proposal on timing of CG-DFI for TS 38.214 is approved.

------------------------------------------- Start of Draft TP of 38.214 ------------------------------------------------------

6.1  UE procedure for transmitting the physical uplink shared channel

<Unrelated parts are omitted>

A UE is not expected to be scheduled by a PDCCH ending in symbol i to transmit a PUSCH on a given serving cell for a given HARQ process, if there is a transmission occasion where the UE is allowed to transmit a PUSCH with configured grant according to [10, TS38.321] with the same HARQ process on the same serving cell starting in a symbol j after symbol i, and if the gap between the end of PDCCH and the beginning of symbol j is less than N2 symbols. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH with configured grant and the subcarrier spacing of the PDCCH scheduling the PUSCH.

If a UE receives an ACK for a given HARQ process in CG-DFI in a PDCCH ending in symbol i  to terminate a transport block repetition in a PUSCH transmission on a given serving cell with the same HARQ process after symbol i, the UE is expected to terminate the repetition of athe transport block in a PUSCH transmission starting from a symbol j  if the gap between the end of PDCCH of symbol i and the start of the PUSCH transmission in symbol j is equal to or more than N2 symbols. The value N2 in symbols is determined according to the UE processing capability defined in Clause 6.4, and N2 and the symbol duration are based on the minimum of the subcarrier spacing corresponding to the PUSCH and the subcarrier spacing of the PDCCH indicating CG-DFI.

For PUSCH scheduled by DCI format 0_0 on a cell, the UE shall transmit PUSCH according to the spatial relation, if applicable, corresponding to the dedicated PUCCH resource with the lowest ID within the active UL BWP of the cell, as described in Clause 9.2.1 of [6, TS 38.213].

<Unrelated parts are omitted>

---------------------------------------------- end of Draft TP -------------------------------------------------------------

R1-2001413	Outcome of discussion on [100e-NR-unlic-NRU-CG-03] – TP for 38.213	vivo

Agreement:

The following text proposals on PDCCH validation for TS 38.213 are approved.

--------------------------------- Start of Text Proposal for 38.213------------------------------------------

10.2 PDCCH validation for DL SPS and UL grant Type 2

<Unchanged parts are omitted>

Table 10.2-2: Special fields for single DL SPS or single UL grant Type 2 scheduling release PDCCH validation

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_1/1_2 |
| --- | --- | --- |
| HARQ process number | set to all '0's/0_1/0_2 | set to all '0's |
| Redundancy version | set to all '0's | set to all '0's |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all ‘0’s for FDRA Type 0 or Type 2 with μ=1, set to all '1's for FDRA Type 1 or FDRA Type 2 with μ=0, | set to all '0's for FDRA Type 0 set to all '1's for FDRA Type 1 |

<Unchanged parts are omitted>

Table 10.2-4: Special fields for multiple DL SPS and UL grant Type 2 scheduling release PDCCH validation

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_1/1_2 |
| --- | --- | --- |
| Redundancy version | set to all '0's | set to all '0's |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all ‘0’s for FDRA Type 0 or Type 2 with μ=1, set to all '1's for FDRA Type 1 or FDRA Type 2 with μ=0, | set to all '0's for FDRA Type 0 set to all '1's for FDRA Type 1 |

<Unchanged parts are omitted>

--------------------------------- End of Text Proposal for 38.213------------------------------------------

--------------------------------- Start of Text Proposal for 38.213------------------------------------------

10.2 PDCCH validation for DL SPS and UL grant Type 2

A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or configured UL grant Type 2 PDCCH if

- the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI, and

- the new data indicator field in the DCI format for the enabled transport block is set to '0', and

- the DFI flag field, if present, in the DCI format indicating CG-DFI is set to ‘0’.

< Unchanged parts are omitted >

--------------------------------- End of Text Proposal for 38.213------------------------------------------

R1-2000200	Corrections on the configured grant procedures	Huawei, HiSilicon

R1-2000312	Remaining issues on the enhancements to configured grant	vivo

R1-2000396	Maintenance on the configured grant for NR-U	ZTE, Sanechips

R1-2000413	Remaining Issues on Configured Grant Enhancement for NR-U 	Nokia, Nokia Shanghai Bell

R1-2000473	Discussion on the remaining issues of configured grant enhancements	OPPO

R1-2000593	Text proposals for configured grant enhancement for NR-U	Lenovo, Motorola Mobility

R1-2000613	Configured grant enhancement for NR-U	Samsung

R1-2000666	Remaining issues of configured grant for NR-U	LG Electronics

R1-2000693	Clarification on HARQ-ACK feedback for PUSCH in DFI	Panasonic Corporation

R1-2000727	Enhancements to configured grants for NR-unlicensed	Intel Corporation

R1-2000829	Configured grant enhancement	Ericsson

R1-2000913	Configured grant enhancement for NR-U	NTT DOCOMO, INC.

R1-2000959	TP for Enhancements to configured grants for NR-U	Qualcomm Incorporated

##### Wide-band operation

R1-2000668	Summary on maintenance of wide-band operation for NR-U	LG Electronics

R1-2001163	Summary#2 on maintenance of wide-band operation for NR-U	LG Electronics

[100e-NR-unlic-NRU-WideBand-01] – Seonwook (LGE)

Email discussion/approval on the issues related to CORESET and SS set configuration:

- Description of rb-Offset parameter and clarification on the first PRB of CORESET

- Description of freqMonitorLocations-r16 parameter

by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001358	Summary of email discussion [100e-NR-unlic-NRU-WideBand-01] on CORESET and SS set configuration for NR-U wideband operation	LG Electronics

Decision: As per email decision posted on Mar. 4th,

Agreement:

- If CORESET p is not configured with rb-offset, and is not associated with any search space set configured withfreqMonitorLocation-r16,

  - The bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart, where the first common RB of the first group of 6 consecutive RBs has common RB index 6∙NBWPstart6, i.e., same as in Rel-15.

- If CORESET p is not configured with rb-offset, and is associated with at least one search space set configured with freqMonitorLocation-r16,

  - The bits of the first A bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart , where the first common RB of the first group of 6 consecutive RBs has common RB index NBWPstart+rb-Offset, where the UE assumes the default value rb-Offset = 0.

- If CORESET p is configured with rb-offset, and is not associated with any search space set configured withfreqMonitorLocation-r16,

  - The bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart , where the first common RB of the first group of 6 consecutive RBs has common RB index NBWPstart+rb-Offset.

- If CORESET p is configured with rb-offset, and is associated with at least one search space set configured with freqMonitorLocation-r16,

  - The bits of the first A bits of the 45-bit bitmap frequencyDomainResources of the CORESET p have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP with the starting common RB position NBWPstart , where the first common RB of the first group of 6 consecutive RBs has common RB index NBWPstart+rb-Offset.

- Note: A bits in above bullets is defined as floor({the number of available PRBs in the first RB set (accounting for rb-Offset) for the BWP}/6), as per previous agreement.

- TS 38.213 editor to implement this agreement

Agreement:

- TS 38.213 editor to implement the missing RAN1#99 agreement:

Agreement: For a search space set configuration with multiple monitoring locations in the frequency domain, - Within the SearchSpace IE, the agreed RRC parameter freqMonitorLocations-r16 provides a bitmap (where the first bit in the bitmap corresponds to the first RB set in the BWP, and the second bit corresponds to the second RB set, and so on). For a RB set indicated in the bitmap, the first PRB of the frequency domain monitoring location confined within the RB set is aligned with {the first PRB of the RB set + rb-Offset provided by the associated CORESET configuration}. - The frequency domain resource allocation pattern for each monitoring location is determined based on the first A bits in frequencyDomainResources provided by the associated CORESET configuration, where A = floor({the number of available PRBs in the first RB set (accounting for rb-Offset) for the BWP}/6).

[100e-NR-unlic-NRU-WideBand-02] – Seonwook (LGE)

Email discussion/approval on a TP capturing the msising agreement regarding CSI-RS for wide-band operation by 2/28

R1-2001359	Summary of email discussion [100e-NR-unlic-NRU-WideBand-02] on CSI-RS for NR-U wide-band operation	LG Electronics

Decision: As per email decision posted on Feb. 29th,

Agreement:

The following TP for Section 11.1.1 in 38.213 is adopted.

------------------------------------------ Text Proposal for 38.213, Section 11.1.1 -------------------------------------------

*** Unchanged text omitted ***

If a UE is configured by higher layers to receive a CSI-RS or a PDSCH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as uplink or flexible, or the UE detects a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 indicating to the UE to transmit PUSCH, PUCCH, SRS, or PRACH in at least one symbol in the set of the symbols, the UE cancels the CSI-RS reception in the set of symbols of the slot or cancels the PDSCH reception in the slot.

If a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as downlink or flexible, or the UE detects a DCI format 1_0, DCI format 1_1, or DCI format 0_1 indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then

-	the UE does not expect to cancel the transmission in symbols from the set of symbols that occur, relative to a last symbol of a CORESET where the UE detects the DCI format 2_0 or the DCI format 1_0 or the DCI format 1_1 or the DCI format 0_1, after a number of symbols that is smaller than the PUSCH preparation time  for the corresponding PUSCH processing capability [6, TS 38.214] assuming  and  corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format 2_0, DCI format 1_0, DCI format 1_1 or DCI format 0_1 and the SCS configuration of the SRS, PUCCH, PUSCH or r, where r corresponds to the SCS configuration of the PRACH if it is 15kHz or higher; otherwise r=0

-	the UE cancels the PUCCH, or PUSCH, or PRACH transmission in remaining symbols from the set of symbols and cancels the SRS transmission in remaining symbols from the subset of symbols.

If a UE is configured by higher layers to receive a CSI-RS or detects a DCI format 0_1 indicating to the UE to receive a CSI-RS in a set of symbols of a slot, and the CSI-RS occupies one or more RB sets, and the UE detects DCI format 2_0 with bitmap indicating that one or more of the RB sets occupied by the CSI-RS are not available for reception, the UE cancels the CSI-RS reception in the set of symbols of the slot.

A UE assumes that flexible symbols in a CORESET configured to the UE for PDCCH monitoring are downlink symbols if the UE does not detect an SFI-index field value in DCI format 2_0 indicating the set of symbols of the slot as flexible or uplink and the UE does not detect a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, or DCI format 2_3 indicating to the UE to transmit SRS, PUSCH, PUCCH, or PRACH in the set of symbols.

*** Unchanged text omitted ***

------------------------------------------------------ End Text Proposal -------------------------------------------------------

R1-2000201	Corrections on the wideband operation procedures	Huawei, HiSilicon

R1-2000313	Remaining issues on wideband operation in NR-U	vivo

R1-2000474	Discussion on the remaining issues of wide-band operations	OPPO

R1-2000502	Remaining issues on Wideband operation in NR-U	Nokia, Nokia Shanghai Bell

R1-2000543	Remaining issues on Rel-16 NR-U wideband operations 	PANASONIC R&D Center Germany

Withdrawn

R1-2000614	Wide-band operation for NR-U	Samsung

R1-2000667	Remaining issues of wide-band operation for NR-U	LG Electronics

R1-2000728	Wideband operation for NR-unlicensed	Intel Corporation

R1-2000772	Remaining issues on wideband operation for NR-U	ETRI

R1-2000794	Maintenance on the wideband operation for NR-U	ZTE, Sanechips

R1-2000830	Wideband operation	Ericsson

R1-2000875	Remaining issues on wide-band operation for NR-U	Sharp

R1-2000960	TP for Wideband operation for NR-U operation	Qualcomm Incorporated

R1-2000999	Remaining issues on Rel-16 NR-U wideband operations	Panasonic

#### Other

R1-2000475	Discussion on the other remaining issues	OPPO

R1-2001107	Other corrections for NR-U	Huawei, HiSilicon

### Maintenance of Integrated Access and Backhaul for NR

#### Extensions of SSBs for inter-IAB-node discovery and measurements

R1-2000961	Clarifications on IAB resource management	Qualcomm Incorporated

#### Extensions of RACH occasions and periodicities for backhaul RACH resources

R1-2001135	Summary of 7.2.3.2 - Extensions of RACH occasions and periodicities for backhaul RACH resources	Qualcomm

R1-2001183	Summary#2 of 7.2.3.2 - Extensions of RACH occasions and periodicities for backhaul RACH resources	Qualcomm

[100e-NR-IAB-RACH-01] – Luca (Qualcomm)

Email discussion/approval on the issues as summarized in R1-2001135 by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001297	Outcome of email discussions [100e-NR-IAB-RACH-01]	Qualcomm Incorporated

Decision: As per email decision posted on Feb. 28th, the 3 TPs in R1-2001297 are endorsed (2 TPs to 38.211, Section 6.3.3.2; one TP to 38.213, Section 14).

Conclusion:

The maximum association pattern period for the IAB-node MT remains 640 ms unconditionally as per the current 38.213 specifications.

R1-2000796	Maintenance of Rel-16 IAB RACH enhancement	Nokia, Nokia Shanghai Bell

R1-2001037	Corrections on RACH occasions and periodicities for backhaul RACH resources	Huawei, HiSilicon

#### Mechanisms for resource multiplexing among backhaul and access links

R1-2001136	Summary of 7.2.3.3 – Mechanisms for resource multiplexing among backhaul and access links	AT&T

R1-2001160	Summary#2 of 7.2.3.3 – Mechanisms for resource multiplexing among backhaul and access links	AT&T

R1-2001425	Final summary of 7.2.3.3 – Mechanisms for resource multiplexing among backhaul and access links	AT&T

[100e-NR-IAB-ResourceMux-01] – Thomas (ATT)

Email discussion/approval on the issues including (as in R1-2001160):

- Issue #: {2.1.1, 2.1.3} – Definition of soft resource availability

- Issue #: {2.3.2, 2.3.4} – Guard symbols for MT/DU transitions (including indication per type and identification of transition and/or transition type at parent/child)

- Issue #: {2.3.7} – PDCCH for Type0-PDCCH CSS sets treated as Hard resource

by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Feb. 29th, all latest proposed agreements/conclusions are agreed (see R1-2001425).

R1-2000314	Remaining aspects for resource multiplexing among backhaul and access links	vivo

R1-2000400	Remaining issues in IAB resource multiplexing	ZTE, Sanechips

R1-2000669	Text proposals on IAB resource multiplexing	LG Electronics

R1-2000745	Remaining issues on mechanisms for resource multiplexing among backhaul and access links	CMCC

R1-2000797	Maintenance of Rel-16 IAB Resource Multiplexing	Nokia, Nokia Shanghai Bell

R1-2001038	Corrections on resource multiplexing among backhaul and access links	Huawei, HiSilicon

R1-2001092	Corrections on Mechanisms for resource multiplexing among backhaul and access links	Ericsson

#### Mechanism to support the “case-1” OTA timing alignment

R1-2000806	FL summary on remaining issues in IAB case-1 timing	ZTE, Sanechips

R1-2001195	FL summary#2 on remaining issues in IAB case-1 timing	ZTE, Sanechips

R1-2000798	Maintenance of Rel-16 IAB Case-1 timing	Nokia, Nokia Shanghai Bell

#### Other

R1-2000789	On mandatory features for BS-like IAB-nodes	Ericsson

R1-2001105	Corrections on other aspects of IAB	Huawei, HiSilicon

### Maintenance for 5G V2X with NR sidelink

[100e-NR-5G_V2X_NRSL_37.985] – Matthew (Huawei)

Email approval of the above TR by 2/26

R1-2000352	TR 37.985, “Overall description of Radio Access Network (RAN) aspects for Vehicle-to-everything (V2X) based on LTE and NR”, v1.1.0	Huawei

R1-2001223	TR 37.985, “Overall description of Radio Access Network (RAN) aspects for Vehicle-to-everything (V2X) based on LTE and NR”, v1.1.0	Huawei

Decision: As per email decision posted on Feb. 28th, R1-2001223 is endorsed as v1.1.0 as the basis for future updates.

MCC: No need to send this version to RAN, as TR v1.0.0 was already provided for information at RAN#86 and it is assumed RAN will extend the core part till RAN#88 for full completion.

#### Physical layer structure for sidelink

R1-2001148	Feature lead summary#1 for agenda item 7.2.4.1 Physical layer structure for sidelink	Samsung

R1-2001149	Feature lead summary#2 for agenda item 7.2.4.1 Physical layer structure for sidelink	Samsung

[100e-NR-5G_V2X_NRSL-PHYstructure-01] – Jeongho (Samsung)

Email discussion/approval on issue 4 (TBS determination) in the summary by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

Decision: As per email decisions posted on Mar. 3rd and Mar. 4th,

Agreement:

- For sidelink TBS determination, the procedure steps 2), 3), and 4) in 5.1.3.2 Transport block size determination of TS38.214 are reused.

Agreements:

For sidelink TBS determination, N_RE’ and/or N_RE are calculated based on the procedure step 1) in 5.1.3.2 Transport block size determination of TS38.214 with the following considerations.

- For the number of PSSCH symbols,

  - AGC symbol and GP symbol in the end of slot are excluded.

- PSCCH overhead

  - The exact number of REs for PSCCH (including PSCCH DMRS) is considered

- 2nd SCI overhead

  - FFS: How to consider the 2nd SCI

  - This is not intended to revert the existing agreement on the 2nd SCI mapping FFS: How to handle the relationship between PSSCH TBS determination and 2nd stage SCI modulated symbols determination.

- FFS: how to consider PSFCH, PSSCH DMRS, GP symbols before PSFCH, SL PT-RS, SL CSI-RS

  - FFS: N_oh^PRB is introduced or not (pre-)configured per resource pool.

  - The overhead for SL PT-RS and SL CSI-RS is considered as already included in N_oh^PRB.

  - FFS: The number of candidiate values for N_oh^PRB is 8.

- It is RAN1’s understanding that a UE is not expected to receive a retransmission with a TB size that is different from the last valid TB size signalled for this TB.

  - Note: The design will be such that the TBS is the same between a transmission and its re-transmission(s).

The corresponding TP to be discussed in the next meeting.

[100e-NR-5G_V2X_NRSL-PHYstructure-02] – Jeongho (Samsung)

Email discussion/approval on issue 8 (resource pool configuration) in the summary by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

Decision: As per email decision posted on Mar.5th,

Agreement:

- For resource pool configuration, slots for a resource pool is (pre-)configured with bitmap, which is applied with periodicity.

Agreements:

For derivation of the set of slots to be included in the resource pool, the baseline is the derivation with bitmap and periodicity based on Subclause 14.1.5 of TS36.213 with the following modifications.

- FFS: Periodicity and L_bitmap value

- The slot index is relative to slot#0 of the radio frame corresponding to SFN 0 of the serving cell if serving cell timing reference is in use, or DFN 0 otherwise

- The following procedure is used.

  - The set includes all the slots except the following slots:

    - Slots in which SLSS resource is configured,

    - (Working assumption) slots not having at least Y-th, (Y+1)-th, ....., (Y+X-1)-th symbols in a slot semi-statically for UL as indicated in TDD-UL-DL-ConfigCommon, where

      - X is sl-LengthSymbols

      - Y is sl-StartSymbol

    - (Working assumption) reserved slots which are determined by the similar steps in Subclause 14.1.5 of TS36.213

Working assumption

- For the number of PRBs for resource pool, allow configuration of all  number of PRBs in a SL BWP.

- FFS until RAN1#100bis-e whether/how to deal with remaining PRBs if the configured PRBs for resource pool is not a multiple of subchannel size.

The corresponding TPs to be discussed in the next meeting.

[100e-NR-5G_V2X_NRSL-PHYstructure-03] – Jeongho (Samsung)

Email discussion/approval on issue 9 (determination of PSSCH DMRS frequency domain pattern) in the summary by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

Decision: As per email decisions posted on Mar. 5th,

Agreements

- For frequency domain DMRS pattern for PSSCH, support only DMRS configuration type 1.

- The "Number of DMRS ports" field in 1st stage SCI indicates DMRS port(s) of PSSCH with one bit.

  - "0" means to use a port 1000 and "1" means to use tow ports 1000/1001.

- Both PSSCH antenna ports are defined to be CDM group 0.

  - Note: how to capture this is up to editors.

- PSSCH is rate-matched around PSSCH DMRS REs within a resource block used for transmission of PSSCH

  - Note: The REs not used for DMRS in the DMRS symbol will be used for PSSCH, PSCCH, or 2nd SCI mapping.

R1-2001400	Text Proposal for PSSCH DMRS	Samsung

Decision: As per email decisions posted on Mar. 5th, the TP in R1-2001400 is endorsed.

R1-2000181	Remaining details of sidelink physical layer structure	Huawei, HiSilicon

R1-2000315	Remaining issues on physical layer structure for NR sidelink	vivo

R1-2000382	Remaining details of Physical layer structure for sidelink	Nokia, Nokia Shanghai Bell

R1-2000398	Remaining issues of NR sidelink physical layer structure	ZTE, Sanechips

R1-2000420	Remaining issues of physical layer structure for sidelink	Spreadtrum Communications

R1-2000428	Physical layer structure for sidelink	TCL Communication Ltd.

R1-2000445	Discussion on sidelink physical layer structure	MediaTek Inc.

R1-2000489	Draft text proposals on physical layer structure for NR-V2X	OPPO

R1-2000520	Remaining issues on physical layer structure for NR sidelink	CATT

R1-2000545	Remaining issues on physical layer structure for sidelink	Fujitsu

R1-2000564	Remaining details on physical layer structure for the sidelink	Futurewei

R1-2000582	Physical layer structure for NR sidelink	Sony

R1-2000615	On Physical Layer Structures for NR Sidelink	Samsung

R1-2000729	Remaining opens of sidelink physical structure for NR V2X design	Intel Corporation

R1-2000746	Remaining issues on physical layer structure for sidelink	CMCC

R1-2000764	Remaining issues on physical layer structure	NEC

R1-2000781	Discussion on physical layer structure for NR sidelink	LG Electronics

R1-2000813	Remaining details of V2X PHY layer structure	Mitsubishi Electric RCE

R1-2000834	Remaining Issues on Physical Layer Structure for NR V2X	InterDigital, Inc.

R1-2000850	Remaining details on NR V2X physical layer structure	Apple

R1-2000876	Remaining issues on physical layer structure for NR sidelink	Sharp

R1-2000898	Discussion on SCI content for 5G V2X	Xiaomi Communications

R1-2000903	Remaining details of Physical layer structure design for sidelink	AT&T

R1-2000914	Remaining issues on sidelink physical layer structure	NTT DOCOMO, INC.

R1-2001213	Considerations on Physical Layer aspects of NR V2X	Qualcomm Incorporated	(rev of R1-2000962)

R1-2000992	Remaining issue on physical layer structure for sidelink in NR V2X	Panasonic Corporation

R1-2001007	Physical layer structure for NR sidelink	Ericsson

R1-2001070	Remaining issues in NR Sidelink Structures	Lenovo, Motorola Mobility

#### Resource allocation for NR sidelink

##### Mode 1

R1-2000777	Feature lead summary#1 on Resource allocation for NR sidelink Mode 1	Ericsson

R1-2000779	Feature lead summary#2 on Resource allocation for NR sidelink Mode 1	Ericsson

R1-2001285	Feature lead summary#3 on Resource allocation for NR sidelink Mode 1	Ericsson

R1-2001367	Feature lead summary#4 on Resource allocation for NR sidelink Mode 1	Ericsson

[100e-NR-5G_V2X_NRSL-RA_Mode1-01] – Ricardo (Ericsson)

Email discussion/approval on issue 1.1.1 in the summary (R1-2000779) by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

Decision: As per email decision posted on Mar. 2nd, the proposals for Q1 & Q4 are agreed (copied below).

Agreement (Q1):

- Confirm the working assumption from RAN1#99 with the following update to the formula: TDL-TTA2+m×Tslot

Agreements (Q4):

- The working assumption from RAN1#99 is confirmed.

- To derive the time of SL HARQ-ACK to the gNB the timing of Uu is used (i.e., assuming that the SL and the Uu timing are the same).

R1-2001415	Text proposal for TS 38.214 endorsed in [100e-NR-5G_V2X_NRSL-RA_Mode1-01]	Ericsson

Decision: TP for Q1 is endorsed in R1-2001415. TP for Q4 to next meeting.

[100e-NR-5G_V2X_NRSL-RA_Mode1-02] – Ricardo (Ericsson)

Email discussion/approval on issue 1.2 in the summary (R1-2000779) by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

Decision: As per email decision posted on Mar. 2nd,

Agreements (Q1):

- The mapping between the values of HPN signaled in DCI and HPN signaled in SCI is fixed for a TB, and is up to UE implementation.

- For dynamic grant, the toggling of NDI in DCI is used as the toggling of NDI in SCI for the first SL transmission scheduled by the DCI. The SCI for the remaining transmissions scheduled by the DCI, if any, have the NDI untoggled with respect to the first SL transmission.

- FFS NDI in SCI for PUCCH ACK-NACK error cases

Agreements (Q2):

- The HARQ process ID for each transmission in a resource corresponding to a SL configured grant is determined based on the formula used for UL configured grants.

  - The mapping with the values of HPN in SCI is fixed for a TB, and is up to UE implementation.

Note: This corresponds to the HARQ process ID for the interaction between gNB and UE, if any distinction is made.

Note: if the principle itself is agreeable, we can then discuss any need for modifications, possibly as part of the TP drafting phase.

Agreements (Q3):

- Only one new TB can be transmitted in one period of the configured grant.

  - FFS any issue with retransmission spanning multiple periods

- The DCI scheduling the retransmissions uses the first HARQ process ID corresponding to the first transmission of the TB, as agreed for Q2.

Agreements (Q4):

The specification supports having multiple HARQ ID processes for a given SL configured grant.

Note: this is issue is tightly coupled with Q2, so all configuration and specification details can be discussed together.

R1-2001410	Text proposal for TS 38.213 endorsed in [100e-NR-5G_V2X_NRSL-RA_Mode1-02]	Ericsson

Decision: As per email decision posted on Mar. 4th, the TPs in R1-2001410 are endorsed.

Debate whether an LS informing RAN2 of the agreements is necessary; no consensus.

[100e-NR-5G_V2X_NRSL-RA_Mode1-03] – Ricardo (Ericsson)

Email discussion/approval on issue 1.4.1 in the summary (R1-2000779) by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

Decision: As per email decision posted on Mar. 2nd,

- Proposals to Q1 to Q4 are agreed (see below)

- Proposals to Q5 and Q6 are agreed as a working assumption (see below)

Agreements (Q1):

- The TX UE reports NACK to the gNB in the following cases:

  - When it does not transmit the corresponding PSCCH/PSSCH due to intra-UE prioritization.

  - When it does not receive the corresponding PSFCH due to intra-UE prioritization.

Agreements (Q2):

- For groupcast option 2 in the case where there are multiple PSFCHs corresponding to multiple PSCCH/PSSCH transmissions of a single TB, the TX UE reports ACK to the gNB if it has received ACK at least once from each RX UE. Otherwise, it reports NACK to the gNB.

Agreements (Q3):

- For groupcast option 2, the TX UE reports NACK to the gNB when it does not detect some expected PSFCH.

Agreements (Q4):

- For configured grant, the TX UE reports ACK to the gNB in case no PSCCH/PSSCH is transmitted in a set of resources.

NOTE: The rule in Q1 has precedence over this rule.

Working assumption (Q5):

In case of reaching the maximum number of HARQ re-transmissions for a TB, the UE sends one bit on the UL resources for SL HARQ-ACK reporting. The specification will specify the UE behavior (what the behavior is: FFS), and specify the contents of the report (what the content is: FFS).

Working assumption (Q6):

If the SL transmission does not use SL HARQ feedback (if supported by RAN2), the UE sends one bit on the UL resources for SL HARQ-ACK reporting. The specification will specify the UE behavior (what the behavior is: FFS), and specify the contents of the report (what the content is FFS).

R1-2001414	Text proposal for TS 38.213 endorsed in [100e-NR-5G_V2X_NRSL-RA_Mode1-03]	Ericsson

Decision: As per email decision posted on Mar. 5th, the TP in R1-2001414 is endorsed.

R1-2000182	Remaining details of sidelink resource allocation mode 1	Huawei, HiSilicon

R1-2000316	Remaining issues on mode 1 resource allocation mechanism	vivo

R1-2000383	Remaining details of Resource allocation for sidelink - Mode 1	Nokia, Nokia Shanghai Bell

R1-2000402	Remaining issues in mode 1 resource allocation	ZTE, Sanechips

R1-2000421	Remaining issues in NR sidelink mode 1 resource allocation	Spreadtrum Communications

R1-2000429	Resource allocation for NR sidelink Mode 1	TCL Communication Ltd.

R1-2000446	NR Sidelink Mode-1 resource allocation	MediaTek Inc.

R1-2000490	Remaining issues of mode 1 resource allocation for NR-V2X	OPPO

R1-2000521	Remaining issues on  Mode 1 resource allocation in NR V2X	CATT

R1-2000546	Remaining issues on mode 1 resource allocation for NR V2X	Fujitsu

R1-2000565	Remaining details on mode-1 resource allocation	Futurewei

R1-2000616	On Mode 1 for NR Sidelink	Samsung

R1-2000699	Mode-1 resource allocation for NR V2X	ITL

R1-2000730	Remaining Issues in Mode-1 Resource Allocation	Intel Corporation

R1-2000747	Remaining issues on mode 1 resource allocation mechanism	CMCC

R1-2000765	Remaining issues on resource allocation Mode 1	NEC

R1-2001153	Discussion on resource allocation for Mode 1	LG Electronics	(rev of R1-2000782)

R1-2000835	Remaining Issues on NR Sidelink Mode 1 Resource Allocation	InterDigital, Inc.

R1-2000851	Remaining details on mode 1 resource allocation	Apple

R1-2000877	Remaining issues on resource allocation mode 1 for NR sidelink	Sharp

R1-2000899	On Sidelink HARQ report of V2x communications	Xiaomi Communications

R1-2000904	Remaining details of resource allocation mode-1 for NR sidelink	AT&T

R1-2000915	Remaining issues on resource allocation mechanism mode 1	NTT DOCOMO, INC.

R1-2001008	Resource Allocation Mode 1 for NR SL	Ericsson

R1-2001072	Remaining issues in NR sidelink Mode 1	Lenovo, Motorola Mobility

R1-2001118	On Resource Allocation Mode 1 for NR V2X	Convida Wireless

##### Mode 2

R1-2001138	FL summary of critical issues for 7.2.4.2.2 – V2X Mode 2	Intel

R1-2001170	FL summary#2 of critical issues for 7.2.4.2.2 – V2X Mode 2	Intel

[100e-NR-5G_V2X_NRSL-RA_Mode2-01] – Sergey (Intel)

Email discussion/approval on the following issues related to re-evaluation/pre-emption:

- Condition to change resource(s) in re-evaluation

- Re-valuation triggering condition / restriction

- Pre-emption enabling/disabling RRC signalling (per priority and/or per pool)

- Pre-emption different overlapping conditions: time only, time and partial frequency, time and full frequency

- Timeline for pre-emption triggering

by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

R1-2001344	Outcome of email thread [100e-NR-5G_V2X_NRSL-RA_Mode2-01]	Intel Corporation

Decision: As per email decisions posted on Mar. 3rd and Mar. 4th,

Agreements:

- For re-evaluation of a pre-selected resource contained in a slot ‘k’ to be first time signaled in a slot ‘m’, where k ≥ m,

  - Step 1 of the resource (re-)selection procedure is performed at least at the moment ‘m-T3’, and if the pre-selected resource is not in the identified candidate resource set, Step 2 is triggered for reselection of the resource

    - Re-evaluations before the moment ‘m-T3’ or after ‘m-T3’ but before ‘m’ are not precluded and are up to UE implementation

      - FFS whether to mandate a UE to perform Step 1 checking every slot before ‘m-T3’

    - FFS whether evaluation of Step 2 has to ensure any introduced timing restrictions between pre-selected and re-selected resources when re-evaluation is triggered, and whether it is allowed to change the pre-selected but not reserved resources which are still in the candidate resource set in order to ensure the timing restrictions

- FFS whether for the case of enabled periodic reservation, already reserved resources in upcoming periods can be re-evaluated

Agreements:

- For pre-emption, both full and partial frequency domain overlap in the same slot are considered as the overlapping condition to trigger resource reselection, wherein the whole resource is reselected even if the partial overlap happened

- (Re-)selection procedure for an already reserved but pre-empted resource(s) to be used for transmission and/or reserved in a slot ‘m’ is not required to be triggered at moment > ‘m – T3’

  - T3 here is identical to T3 introduced for the re-evaluation

- FFS whether re-selection of the already-reserved, but pre-empted resource applies only to the resource transmitted in slot ‘m’ or to other already-reserved and pre-empted resource(s) reserved signaled in the SCI transmitted in slot ’m’ as well

[100e-NR-5G_V2X_NRSL-RA_Mode2-02] – Sergey (Intel)

Email discussion/approval on the following issues related to step 2 details:

- Nmax resources within a window of 32 slots (chain integrity). Related to the first proposed agreement in FL summary R1-1913569 for RAN1#99 “Friday session”.

- HARQ RTT ensuring (minimum gap between selected resources when FB enabled)

by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

R1-2001345	Outcome of email thread [100e-NR-5G_V2X_NRSL-RA_Mode2-02]	Intel Corporation

Decision: As per email decision posted on Mar. 5th,

Agreements:

- In Step 2, a UE ensures a minimum time gap Z = a + b between any two selected resources of a TB where a HARQ feedback for the first of these resources is expected

  - ‘a’ is a time gap between the end of the last symbol of the PSSCH transmission of the first resource and the start of the first symbol of the corresponding PSFCH reception determined by resource pool configuration and higher layer parameters of MinTimeGapPSFCH and periodPSFCHresource

  - ‘b’ is a time required for PSFCH reception and processing plus sidelink retransmission preparation including multiplexing of necessary physical channels and any TX-RX/RX-TX switching time and is determined by UE implementation

Decision: As per email decision posted on Mar. 11th, no consensus regarding the following:

Proposal

- Option 1:

  - (Working assumption) For a given resource selection, within a resource selection window, the distance in logical slots between any two selected consecutive resources for SL transmission is less than 32 and larger than 0

    - FFS if there are any exceptions and/or different handling to support backward signaling if it is agreed

Supported: NEC, Intel, Ericsson, Qualcomm, Fujitsu, [vivo, if added FFS: how to handle exceptions such as resource not available due to preemption or congestion control, etc]

Objected: Huawei/HiSilicon

- Option 2:

  - (Working assumption) For a given selection of resources to be signaled in one SCI, within a resource selection window, the distance in logical slots between any two selected consecutive resources for SL transmission is less than 32 and larger than 0

Supported: Huawei/HiSilicon, vivo

- No proposal :

Supported: ZTE/Sanechips, [vivo]

Attempt for proposal majority based (no consensus):

- For a given resource selection, within a resource selection window, the distance in logical slots between any two selected consecutive resources for SL transmission is less than 32 and larger than 0

  - FFS if there are any exceptions and/or different handling to support backward signaling if it is agreed

  - FFS how to handle exceptions such as resource not available due to preemption or congestion control, etc

Attempt for proposal combined by FFS (no consensus):

- For a given resource selection, within a resource selection window, the distance in logical slots between two selected consecutive resources for SL transmission is less than 32 and larger than 0

  - FFS if the two selected consecutive resources are any two resources in a given selection, or the resources to be signaled in one SCI

  - FFS if there are any exceptions and/or different handling to support backward signaling if it is agreed

  - FFS how to handle exceptions such as resource not available due to preemption or congestion control, etc

[100e-NR-5G_V2X_NRSL-RA_Mode2-03] – Sergey (Intel)

Email discussion/approval on the following issues related to SCI indication:

- Time resource signalling

- Frequency resource signalling

- Resource index signalling (backward signalling)

- Period values in the range [1:99]

- Period values used for exclusion once SCI received: actual or all configured

by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

R1-2001346	Outcome of email thread [100e-NR-5G_V2X_NRSL-RA_Mode2-03]	Intel Corporation

Decision: As per email decision posted on Mar. 4th, the following is agreed. Corresponding TPs are for discussion at next meeting.

Agreements:

- Time resource assignment in SCI uses an extended time domain RIV mechanism as follows:

if N=1

TRIV=0

elseif N=2

TRIV=T1

else

if T2-T1-1≤15

TRIV=30T2-T1-1+T1+31

Else

TRIV=3031-T2+T1+62-T1

end if

end if

where

  - N denotes the actual number of resources indicated

  - Ti denotes i-th resource time offset

    - for N=2, 1≤T1≤31

    - for N=3, 1≤T1≤30, T1<T2≤31

Agreements:

- For frequency resource indication, the following resource index calculation is used

  - For Nmax = 2,

    - r=f2+i=1m-1N subchannel SL+1-i

  - For Nmax = 3,

    - r=f2+f3⋅N subchannel SL+1-m+i=1m-1N subchannel SL+1-i2

  - where

    - f2 denotes lowest sub-channel index for the second resource, if any

    - f3 denotes lowest sub-channel index for the third resource, if any

    - m denotes number of sub-channels in a frequency resource allocation

  - If time domain allocation indicates N < Nmax, the decoded lowest sub-channel indexes corresponding to Nmax minus N last resources are not used

Agreements:

- Down-select in the next meeting one of the following options

  - Option 1: There is no separate field in the first stage SCI indicating a resource index for the purpose of backward indication, i.e., backward indication is not supported

  - Option 2: When periodic reservations are enabled in a resource pool, a separate field of 1 bit in the first stage SCI indicates a resource index for the purpose of backward indication

  - Option 3: When periodic reservations are enabled in a resource pool, a separate field of ceil(log2(Nmax)) bit in the first stage SCI indicates a resource index for the purpose of backward indication

Agreements:

- On a per resource pool basis, when reservation of a sidelink resource for an initial transmission of a TB at least by an SCI associated with a different TB is enabled:

  - A set of possible period values additionally includes all integer values from 1 to 99 ms

Conclusion

- Evaluate till the next meeting whether given the agreed set of configurable reservation periodicities, the change to the exclusion procedure is necessary, wherein currently all configured period values are used for exclusion as inherited from LTE

R1-2000183	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

R1-2000317	Remaining issues on mode 2 resource allocation mechanism	vivo

R1-2000384	Remaining details of Resource allocation for sidelink - Mode 2	Nokia, Nokia Shanghai Bell

R1-2000403	Remaining issues in mode 2 resource allocation	ZTE, Sanechips

R1-2000422	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

R1-2000430	Resource allocation for NR sidelink Mode 2	TCL Communication Ltd.

R1-2000447	On sidelink mode-2 resource allocation	MediaTek Inc.

R1-2000493	Discussion on remaining open issue for mode 2	OPPO

R1-2000522	Remaining issues on Mode 2 resource allocation in NR V2X	CATT

R1-2000542	Remaining Issues On Sidelink Mode 2 Resource Allocation	Panasonic Corporation

R1-2000547	Remaining details on mode 2 resource allocation for NR V2X	Fujitsu

R1-2000563	Remaining Issues in Resource Allocation for Mode 2 NR V2X	Fraunhofer HHI, Fraunhofer IIS

R1-2000566	Remaining details on mode-2 resource allocation	Futurewei

R1-2000583	Resource allocation mechanism for NR sidelink Mode 2	Sony

R1-2000617	On Mode 2 for NR Sidelink	Samsung

R1-2000695	Remaining Issues on NR Sidelink Mode 2 Resource Allocation	ITRI

R1-2000731	Remaining opens of resource allocation mode-2 for NR V2X design	Intel Corporation

R1-2000783	Discussion on resource allocation for Mode 2	LG Electronics

R1-2000836	Remaining Issues on NR Sidelink Mode 2 Resource Allocation	InterDigital, Inc.

R1-2000852	Remaining details on mode 2 resource allocation	Apple

R1-2000878	Remaining issues on resource allocation mode 2 for NR sidelink	Sharp

R1-2000900	On resource reservation in Mode 2 resource allocation	Xiaomi Communications

R1-2000905	Remaining details of resource allocation mode-2 for NR sidelink	AT&T

R1-2000916	Remaining issues on resource allocation mechanism mode 2	NTT DOCOMO, INC.

R1-2000963	Sidelink Resource Allocation Mechanism for NR V2X	Qualcomm Incorporated

R1-2000996	Remaining issue for Mode 2 resource allocation in NR V2X	ASUSTeK

R1-2001006	Remain details on mode-2 resource allocation for NR V2X	ITL

R1-2001009	Resource allocation Mode 2 for NR SL	Ericsson

R1-2001090	Remaining issues for Mode 2 resource (re-)selection of periodic transmission	ROBERT BOSCH GmbH

##### Other

R1-2000184	On remaining details of pre-emption mechanism	Huawei, HiSilicon

R1-2000318	Remaining aspects for resource allocation	vivo

R1-2000494	On other aspects of mode 2 resource allocation	OPPO

R1-2000618	Open Issues on Mode 1 and Mode 2 for NR Sidelink	Samsung

R1-2001010	Other outstanding resource allocation issues	Ericsson

R1-2001018	Discussion on simultaneous configuration of Mode 1 and Mode 2 for a UE	ASUSTeK

#### Sidelink synchronization mechanism

R1-2000832	Feature lead summary on AI 7.2.4.3 Sidelink synchronization mechanism	CATT

R1-2000833	Feature lead summary#2 on AI 7.2.4.3 Sidelink synchronization mechanism	CATT

R1-2001363	Feature lead summary#3 on AI 7.2.4.3 Sidelink synchronization mechanism – Email discussions outcomes	CATT

[100e-NR-5G_V2X_NRSL-SYNC-01] – Teng (CATT)

Email discussion/approval on the content of PSBCH: indication of TDD configuration by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Mar. 3rd,

Agreement:

- Slot-level indication is supported in TDD configuration indication.

Agreements:

The TDD configuration indication is done as follows:

- X bits to indicate patterns + Y bits to indicate periodicity + Z bits to indicate UL slots.

  - FFS the values of X, Y and Z.

  - Total Z bits to indicate UL slots in pattern 1 and pattern 2 respectively if two patterns are configured.

If any TP, to be discussed next meeting.

[100e-NR-5G_V2X_NRSL-SYNC-02] – Teng (CATT)

Email discussion/approval on the following issues:

- DM-RS sequence initialization for PSBCH: How to generate Cinit

- Scrambling of PSBCH

- S-PSS/S-SSS frequency position in S-SSB

by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

Decision: As per email decision posted on Mar. 3rd,

Agreement:

- SL SSID is used for DM-RS sequence initialization in PSBCH.

Agreements:

The DM-RS sequence initialization for PSBCH is to be down-selected one from the following Alts:

- Alt 1: cinit=211iS-SSB+1NIDSL+1+26iS-SSB+1, where iS-SSB is 3 LSBs of S-SSB index.

- Alt 2: cinit=NIDSL

Agreement:

- Scrambling for PSBCH is processed after rate matching.

Agreement:

- NR PSBCH scrambling sequence is initialized with Cinit=NIDSL at the start of each PSBCH block.

Agreements:

The following TP on S-PSS/S-SSS frequency position in S-SSB is endorsed.

------------------------------------------------------ Start of Draft TP of 38. 211----------------------------------------------

<Unchanged parts omitted>

Table 8.4.3.1-1: Resources within an S-SS/PSBCH block for S-PSS, S-SSS, PSBCH, and DM-RS.

| Channel or signal | OFDM symbol number l relative to the start of an S-SS/PSBCH block | Subcarrier number k relative to the start of an S-SS/PSBCH block |
| --- | --- | --- |
| S-PSS | 1, 2 | 2-128 |
| S-SSS | 3, 4 | 2-128 |
| Set to 0 | 1,2,3,4 | 0,1, 129,130,131 |
| PSBCH | 0, 5,6, …,NsymbS-SSB-1 | 0,1,…,131 |
| DM-RS for PSBCH | 0, 5,6, …,NsymbS-SSB-1 | 0, 4, 8, …., 128 |

---------------------------------------------------------- End of Draft TP -------------------------------------------------------

Decision: As per email decision posted on Mar. 4th, the TPs in R1-2001363 (section 2.2) are endosed.

[100e-NR-5G_V2X_NRSL-SYNC-03] – Teng (CATT)

Email discussion/approval on the tx power of S-SSB:

- S-SSB power control

- Power difference for S-PSS, S-SSS, PSBCH DM-RS, and PSBCH data

by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Mar. 3rd,

Agreements:

- Downlink pathloss based S-SSB open-loop power control is supported.

- The transmission power is same for S-PSS, S-SSS, and PSBCH symbol.

Decision: As per email decision posted on Mar. 4th, the TPs in R1-2001363 (section 2.3) are endosed.

R1-2000185	Remaining details of sidelink synchronization mechanisms	Huawei, HiSilicon

R1-2000319	Remaining issues on sidelink synchronization mechanism	vivo

R1-2000385	Remaining details of Sidelink synchronization mechanism	Nokia, Nokia Shanghai Bell

R1-2000399	Remaining issues of Synchronization	ZTE, Sanechips

R1-2000423	Remaining issues in synchronization mechanism for NR V2X	Spreadtrum Communications

R1-2000448	Discussion on sidelink based synchronization mechanism	MediaTek Inc.

R1-2000491	Remaining issues of synchronization for NR-V2X	OPPO

R1-2000523	Remaining issues on sidelink synchronization mechanism in NR V2X	CATT

R1-2000548	Corrections on sidelink synchronization procedure	Fujitsu

R1-2000567	Remaining details on synchronization for sidelink	Futurewei

R1-2000619	On Synchronization Mechanisms for NR Sidelink	Samsung

R1-2000696	Remaining Issues on Sidelink Synchronization for NR V2X Communication 	ITRI

R1-2000732	Remaining opens of sidelink synchronization for NR V2X design	Intel Corporation

R1-2000748	Remaining issues on sidelink synchronization mechanisms	CMCC

R1-2000784	Discussion on NR sidelink synchronization mechanism	LG Electronics

R1-2000879	Remaining issues on synchronization mechanism for NR sidelink	Sharp

R1-2000917	Remaining issues on sidelink synchronization mechanism	NTT DOCOMO, INC.

R1-2000964	Synchronization mechanism enhancment for cluster merge	Qualcomm Incorporated

R1-2001011	Synchronization mechanism for NR SL	Ericsson

#### In-device coexistence between LTE and NR sidelinks

R1-2000965	Feature Lead summary of In-device Coexistence Aspects in NR-V2X (AI 7.2.4.4)	Qualcomm Incorporated

R1-2001212	Feature Lead summary#2 of In-device Coexistence Aspects in NR-V2X (AI 7.2.4.4)	Qualcomm Incorporated

R1-2000186	Remaining details of in-device coexistence between LTE and NR sidelinks	Huawei, HiSilicon

R1-2000320	Remaining issues on In-device coexistence between NR and LTE sidelinks	vivo

R1-2000386	Remaining details of In-device coexistence between LTE and NR sidelinks	Nokia, Nokia Shanghai Bell

R1-2000449	In-device coexistence between LTE and NR sidelinks	MediaTek Inc.

R1-2000495	Remaining open issues on in-device coexistence	OPPO

R1-2000569	Remaining details on in-device coexistence between LTE and NR sidelinks	Futurewei

R1-2000620	On In-device Coexistence between LTE and NR Sidelink	Samsung

R1-2001089	In-device coexistence between LTE and NR sidelinks	Ericsson

#### Physical layer procedures for sidelink

R1-2000839	Summary of critical issues for AI 7.2.4.5 Physical layer procedures for sidelink	LG Electronics Inc.

R1-2001134	Summary of critical issues for AI 7.2.4.5 Physical layer procedures for sidelink	LG Electronics Inc.

R1-2001140	Feature lead summary for AI 7.2.4.5 Physical layer procedures for sidelink	LG Electronics

R1-2001338	Feature lead summary#2 for AI 7.2.4.5 Physical layer procedures for sidelink	LG Electronics

[100e-NR-5G_V2X_NRSL-PHY_Procedure-01] – Hanbyul (LGE)

Email discussion/approval on the remaining issues in power control formula for PSCCH/PSSCH/PSFCH

- PSD of PSCCH and PSSCH in the overlapping symbols, e.g., whether to apply power boosting

- Whether to support multiple PSFCH TXs and, if supported, PSD of each PSFCH

- How to derive reference PSSCH DMRS power for SL PL estimation

by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

Decision: As per email decisions posted on Mar. 3rd and Mar. 4th,

Agreements:

- A Tx UE transmit a PSSCH in a slot. PSCCH, SL CSI-RS, SL PT-RS, PSCCH-DMRS, and PSSCH-DMRS are all transmitted with the same PSD during a slot.

- Equal power is used for each antenna port for 2-AP PSSCH transmission.

- Simultaneous PSFCH transmissions in a PSFCH TX occasion is supported from RAN1 specification perspective.

- Power control formula is applied to each transmission (i.e. fixing the PSD of PSSCH DMRS at the TX UE for a time duration is not supported).

Agreements:

- For referenceSignalPower in the SL pathloss calculation, TX UE uses L3-filtered TX power with the coefficients configured for L3-RSRP measurement

Decision: As per email decision posted on Mar. 5th, TPs attached to R1-2001338 are endorsed.

[100e-NR-5G_V2X_NRSL-PHY_Procedure-02] – Hanbyul (LGE)

Email discussion/approval on the remaining issues in details of the PSFCH resources

- Whether candidate PSFCH resource is the set of PRBs associated with the starting sub-channel and slot used for that PSSCH or the sub-channel(s) and slot used for that PSSCH

- Details on how to define the values of m0 and mCS for a PSFCH TX

- Whether/how to handle the case when rbSetPSFCH is not a multiple of numSubchannel*periodPSFCHresource

- Whether or not to introduce restriction on the use of GC HARQ feedback Option 2 according to the number of PSFCH resources.

- How to indicate GC HARQ feedback Option 1/2 to RX UE.

Decision: As per email decisions posted on Mar. 3rd and Mar. 4th,

Agreements:

- For unicast, groupcast HARQ feedback Option 2

  - mCS = 0 for NACK

  - mCS = 6 for ACK

- For groupcast HARQ feedback Option 1

  - mCS = 0 for NACK

  - mCS is not defined for ACK

Agreements:

For PSFCH resource indexing, following values of m0 is sequentially used.

- m0={0} for NCSPSFCH=1

- m0={0,3} for NCSPSFCH=2

- m0={0, 2, 4}for NCSPSFCH=3

- m0={0, 1, 2, 3, 4, 5}for NCSPSFCH=6

- NCSPSFCH=4 is not supported

Agreements:

- Physical layer assumes that rbSetPSFCH is always form of a multiple of numSubchannel*periodPSFCHresource.

Agreements:

- RAN1 assumes that RAN2 will handle selection of appropriate groupcast HARQ feedback option. From RAN1 perspective, a TX UE can use GC HARQ feedback Option 2 only when the group size is not greater than the number of candidate PSFCH resources associated with the selected PSSCH resource.

  - Send an LS to RAN2

Agreement:

- One of the following two options is (pre-)configured per resource pool.

  - Option 1: The set of PRBs for the candidate PSFCH resource is determined by the starting sub-channel and slot used for that PSSCH.

  - Option 2: The set of PRBs for the candidate PSFCH resource is determined by the sub-channel(s) and slot used for that PSSCH.

Decision: As per email decision posted on Mar. 5th, TPs attached to R1-2001338 are endorsed.

R1-2001426	LS on sidelink HARQ	RAN1, LG Electronics

Decision: As per email decision posted on Mar. 5th, the LS to RAN2 is approved.

[100e-NR-5G_V2X_NRSL-PHY_Procedure-03] – Hanbyul (LGE)

Email discussion/approval on the remaining issues in details of the TX-RX distance determination

- Details on length/width of each geographic zone and number of zones configured with respect to longitude/latitude

- Details of calculating TX-RX distance for GC HARQ feedback Option 1, including whether/how to handle the case when the location information is not available at TX UE and/or RX UE

by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

Update on 02/28 (PST): to check on 3/2

Update on 3/2 (PST):

Agreements:

- Zone length and zone width are always the same and configurable among {5m, 10m, 20m, 30m, 40m, 50m} per communication range requirement per resource pool.

- Zone ID bit field size is 12.

Agreements:

- Communication range requirement bit field size is 4.

- (Pre-)configuration selects 16 values for the communication range requirement from {20, 50, 80,100, 120,150, 180, 200, 220, 250, 270, 300, 320, 350, 370, 400, 420, 450, 480, 500, 550, 600, 700, 1000, Spare, Spare, Spare, Spare, Spare, Spare, Spare, Spare} meters. Each value is mapped to each codepoint of the communication range requirement field.

Agreement:

- For TX-RX distance calculation, RX UE uses the distance between the center location of the indicated zone nearest to the RX UE and its own location.

Agreements:

- RAN1 assumes that RAN2 will handle selection of appropriate groupcast HARQ feedback option. From RAN1 perspective, a TX UE can use distance HARQ feedback only when the TX UE’s location is available.

Agreement:

- RAN1 assumes that RAN2 will handle the case the RX UE’s location is not available.

Agreement:

- Send an LS to RAN2 to inform the agreements related to the TX-RX distance determination.

Decision: As per email decision posted on Mar. 5th, TPs (38.212, 38.214)attached to R1-2001338 are endorsed.

R1-2001426	LS on sidelink HARQ	RAN1, LG Electronics

Decision: As per email decision posted on Mar. 5th, the LS to RAN2 is approved.

R1-2000187	Remaining details of physical layer procedures for sidelink	Huawei, HiSilicon

R1-2000321	Remaining issues on physical layer procedure for NR sidelink	vivo

R1-2000387	Remaining details of Physical layer procedures for sidelink	Nokia, Nokia Shanghai Bell

R1-2000401	Remaining issues on PHY procedures for Rel-16 sidelink	ZTE, Sanechips

R1-2000424	Remaining issues in physical layer procedures for sidelink	Spreadtrum Communications

R1-2000431	Physical layer procedures for sidelink	TCL Communication Ltd.

R1-2000492	Remaining issues of physical layer procedure for NR-V2X	OPPO

R1-2000524	Remaining issues on physical layerprocedures for NR V2X	CATT

R1-2000549	Remaining issues on physical layer procedures	Fujitsu

R1-2000562	Remaining Issues in Physical Layer Procedures for NR V2X	Fraunhofer HHI, Fraunhofer IIS, Continental Automotive GmbH

R1-2000568	Remaining details on physical procedures for sidelink	Futurewei

R1-2000584	Physical layer procedures for NR sidelink	Sony

R1-2000621	On Physical Layer Procedures for NR Sidelink	Samsung

R1-2000697	Remaining Issues on Physical Layer Procedures in NR Sidelink	ITRI

R1-2000733	Remaining Issues and Corrections in Sidelink Procedures	Intel Corporation

R1-2000749	Remaining issues on physical layer procedures for sidelink	CMCC

R1-2000766	Remaining issues on physical layer procedure	NEC

R1-2000785	Discussion on physical layer procedure for NR sidelink	LG Electronics

R1-2000837	Remaining Issues on Physical Layer Procedures for NR V2X	InterDigital, Inc.

R1-2000853	Remaining details on physical layer procedures for NR V2X sidelink	Apple

R1-2000880	Remaining issues on physical layer procedures for NR sidelink	Sharp

R1-2000901	On procedures for 5G V2x communications	Xiaomi Communications

R1-2000906	Remaining details of physical layer procedures for sidelink	AT&T

R1-2000918	Remaining issues on sidelink physical layer procedure	NTT DOCOMO, INC.

R1-2000966	Physical layer procedures for sidelink	Qualcomm Incorporated

R1-2000993	Remaining issue on physical layer procedures for sidelink in NR V2X	Panasonic Corporation

R1-2001012	Physical layer procedures for NR sidelink	Ericsson

R1-2001017	Remaining issues on sidelink physical layer procedure on NR V2X	ASUSTeK

R1-2001045	On PSFCH of NR Sidelink	CAICT

R1-2001071	Remaining issues in NR Sidelink Physical Layer Procedures 	Lenovo, Motorola Mobility

#### QoS management for sidelink

R1-2001091	Feature Lead Summary for AI 7.2.4.6 QoS management	Nokia, Nokia Shanghai Bell

R1-2001181	Summary of email discussion 100e-Prep-NR-5G_V2X_NRSL-QoS	Nokia

R1-2001294	Outcome of email discussions [100e-NR-5G_V2X_NRSL-QoS]	Nokia

[100e-NR-5G_V2X_NRSL-QoS-01] – Torsten (Nokia)

Email discussion/approval on constraints on past/future window in CR evaluation by 2/27; if there is a spec impact, endorsing the

Proposal: For the constraints on past/future window in CR evaluation re-use the LTE behaviour; in detail:

- a >= (a+b+1)/2

- b >= 0

- n+b shall not exceed the last transmission opportunity of the grant for the current transmission of the TB

Note:

- in the last bullet point above, LTE’s “should” has been replaced by “shall”

Decision: As per email decision posted Mar. 3rd,

Agreements:

For the constraints on past/future window in CR evaluation:

- n+b shall not exceed the last transmission opportunity of the grant for the current transmission

- b >= 0

- (b is in slots) b < (a+b+1)/2

Note:

- in the first bullet point above, LTE’s “should” has been replaced by “shall”

Decision: As per email decision posted Mar. 4th, TP (38.215) in R1-2001294 is endorsed.

[100e-NR-5G_V2X_NRSL-QoS-02] – Torsten (Nokia)

Email discussion/approval on when CR_limit is applied by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Feb. 29th,

Agreement:

- UE evaluates CR and applies CR_limit for every (re)transmission.

Decision: As per email decision posted Mar. 5th, TPs (38.214, 38.215) in R1-2001294 are endorsed.

[100e-NR-5G_V2X_NRSL-QoS-03] – Torsten (Nokia)

Email discussion/approval on congestion control processing time and physical/logical slots for CBR and CR window by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Feb. 29th,

Agreements:

- The slot index in the definition of CBR is the physical slot index.

- The slot index in the definition of CR is the physical slot index.

Decision: As per email decision posted on Mar.4th,

Agreements:

- The CBR processing time is given by UE capability according to the following table

| µ | Congestion process time 1 (slots) | Congestion processing time 2 (slots) |
| --- | --- | --- |
| 0 | 2 | 2 |
| 1 | 2 | 4 |
| 2 | 4 | 8 |
| 3 | 8 | 16 |

- A UE shall only apply a single CBR/CR processing time capability in SL.

- CR processing time is the same as CBR processing time.

Decision: As per email decision posted Mar. 4th, TPs (38.214, 38.215) in R1-2001294 are endorsed.

R1-2000188	Remaining details of QoS management for NR sidelink	Huawei, HiSilicon

R1-2000322	Remaining issues on QoS management for sidelink	vivo

R1-2000388	Remaining details of QoS management for sidelink	Nokia, Nokia Shanghai Bell

R1-2000404	Remaining issues on NR sidelink congestion control	ZTE, Sanechips

R1-2000496	Remaining open issues on channel occupancy ratio	OPPO

R1-2000525	Remaining issues on congestion control in NR V2X	CATT

R1-2000570	Remaining details of QoS for sidelink	Futurewei

R1-2000622	On QoS Management for NR Sidelink	Samsung

R1-2000694	Aspects of Sidelink Channel Occupancy Ratio Evaluation	Continental Automotive GmbH, Fraunhofer HHI, Fraunhofer IIS

R1-2000698	Remaining issues on congestion control for sidelink QoS management	ITRI

R1-2000734	Remaining details of QoS and congestion control for NR V2X design	Intel Corporation

R1-2000786	Discussion on QoS management for NR sidelink	LG Electronics

R1-2000838	Remaining Issues on Congestion Control and QoS Management for NR-V2X	InterDigital, Inc.

R1-2001013	QoS management for NR sidelink	Ericsson

#### Support of NR Uu controlling LTE sidelink

R1-2000822	Summary of critical issues for NR Uu scheduling LTE sidelink	Futurewei

R1-2001159	Update of summary of critical issues for NR Uu scheduling LTE sidelink	Futurewei

R1-2001245	Update#2 of summary of critical issues for NR Uu scheduling LTE sidelink	Futurewei

[100e-NR-5G_V2X_NRSL-NRUuSchedulingLTESL-01] – Phillippe (FutureWei)

Email discussion/approval on issue 1 in the summary by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2

R1-2001394	TP for TS38.213 to capture the outcome of [100e-NR-5G_V2X_NRSL-NRUuSchedulingLTESL-01]	Futurewei

Decision: As per email decision posted on Mar. 3rd,

Agreements

- For NR Uu scheduling LTE sidelink, the subframe of the first sidelink transmission is the first SL subframe of the corresponding resource pool that starts not earlier than , where TDCI is the start timing of the slot carrying DCI format 3_1, NTA and Tc are defined in TS 38.211, X is the value indicated by Timing offset field in DCI format 3_1, m is the value indicated by SL index in DCI format 3_1 if SL index is present, otherwise m=0.

Decision: As per email decision posted on Mar. 5th, the 38.213 TP in R1-2001394 is endorsed. (copied below)

----------------------------------------------- Start of TP -----------------------------------------------------

16.6  UE procedure for LTE sidelink transmission

If the UE receives in slot n DCI format 3_1 with CRC scrambled by SL-L-CS-RNTI, the UE shall consider the received DCI information as an LTE sidelink semi-persistent activation or release for the SPS configuration indicated by the SL SPS configuration index field. If the received DCI activates an SL SPS configuration, the UE procedure for transmitting the PSCCH and PSSCH is as described in [36.213] with the replacement that the transmission starts no earlier than TDCI-NTA2×TC+X+4+m  ms, where TDCI is the start timing of the slot carrying DCI format 3_1, NTA and TC are defined in [TS 38.211], X is the value indicated by Timing offset field in DCI format 3_1, m is the value indicated by SL index in DCI format 3_1 if SL index is present, otherwise m=0.

----------------------------------------------- End of TP -----------------------------------------------------

[100e-NR-5G_V2X_NRSL-NRUuSchedulingLTESL-02] – Phillippe (FutureWei)Email discussion/approval on issue 2 in the summary by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2

R1-2001395	TP for TS38.212 to capture the outcome of [100e-NR-5G_V2X_NRSL-NRUuSchedulingLTESL-02]	Futurewei

Decision: As per email decision posted on Mar. 3rd,

Agreements

- If UE is configured to monitor DCI 3_0, the sizes of DCI 3_1 and DCI 3_0 are is aligned by zero padding to DCI 3_0 size.

- If UE is not configured to monitor DCI 3_0, the mechanism for size alignment between DCI 3_0 and Uu DCI is reused for size alignment between DCI 3_1 and Uu DCI

Decision: As per email decision posted on Mar. 4th, the 38.212 TP in R1-2001395 is endorsed.

[100e-NR-5G_V2X_NRSL-NRUuSchedulingLTESL-03] – Phillippe (FutureWei)

Email discussion/approval on issue 3 in the summary by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Feb.27th,

Agreements:

The set of possible values for X is: 0ms, 0.25ms, 0.5ms, 0.625ms, 0.75ms, 1ms, 1.25ms, 1.5ms,1.75ms, 2ms, 2.5ms, 3ms, 4ms, 5ms, 6ms, 8ms, 10ms, 20 ms

No TP needed, just an update of RRC parameters

R1-2000189	Remaining details of NR Uu control for LTE sidelink	Huawei, HiSilicon

R1-2000323	Remaining issues on support of NR Uu controlling LTE sidelink	vivo

R1-2000389	Remaining details of NR Uu controlling LTE sidelink	Nokia, Nokia Shanghai Bell

R1-2000450	Support of NR-Uu controlling LTE sidelink	MediaTek Inc.

R1-2000497	On DCI signalling to control LTE sidelink	OPPO

R1-2000526	Remaining issues on NR Uu controlling LTE sidelink	CATT

R1-2000571	Remaining details on the support of NR Uu controlling LTE sidelink	Futurewei

R1-2000623	On NR Uu Controlling LTE Sidelink	Samsung

R1-2000787	Discussion on NR Uu controlling LTE sidelink	LG Electronics

R1-2000854	On size alignment of DCI format 3_1	Apple

R1-2000967	Syncronisation details for NR-V2X	Qualcomm Incorporated

R1-2001014	NR UU controlling LTE sidelink transmissions	Ericsson

#### Other

R1-2000324	Remaining aspects for NR V2X	vivo

R1-2000498	Discussion on UE feature list for Release16 5G-V2X	OPPO

R1-2000572	Other remaining issues on sidelink	Futurewei

R1-2000624	Details on frequency resource indication of PSSCH	Samsung

R1-2001015	Remaining details on resource pool determination	Ericsson

R1-2001095	Remaining details of sidelink RRC Parameter List	Huawei, HiSilicon

### Maintenance of Physical Layer Enhancements for NR URLLC

Including maintenance for IIoT

#### PDCCH enhancements

R1-2001044	Feature lead summary#1 on PDCCH enhancements	Huawei

R1-2001164	Feature lead summary#2 on PDCCH enhancements	Huawei

[100e-NR-L1enh_URLLC_PDCCH-01] – Chengyan (Huawei)

Email discussion/approval on remaining issues on DCI format:

- Remaining issues on DCI size alignment

- Remaining issues on resource allocation type 1 with granularity larger than 1 RB

- Remaining open points on the RRC parameters

by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

R1-2001404	Summary of email discussion [100e-NR-L1enh_URLLC_PDCCH-01] on remaining issues on DCI format	Huawei

Decision: As per email decision posted on Mar. 2nd,

Agreements:

- Introduce independent RRC parameters AntennaPorts-FieldPresence-ForDCIFormat0_2 for DCI format 0_2 and AntennaPorts-FieldPresence-ForDCIFormat1_2 for DCI format 1_2.

- Remove the RRC parameters CSI-AperiodicTriggerStateList-ForDCIFormat0_2 and [CSI-SemiPersistentOnPUSCH-TriggerStateList-ForDCIFormat0_2] from the Rel-16 RRC parameter lists.

- Remove the bracket on formats0-0-And-1-0 in the column of value range for RRC parameter dci-Formats-Rel16.

  - Adopt the following TP in section 2.4 in R1-2000230 for section 10.1 in TS 38.213.

------------------------------------ Start of TP for 38.213 --------------------------------------------

For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with  search space sets where, for each search space set from the  search space sets, the UE is provided the following by SearchSpace:

- a search space set index , , by searchSpaceId

- an association between the search space set  and a CORESET  by controlResourceSetId

- a PDCCH monitoring periodicity of  slots and a PDCCH monitoring offset of  slots, by monitoringSlotPeriodicityAndOffset

< Unchanged parts are omitted >

- if search space set  is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or an indication by dci-Formats-Rel16to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or for DCI format 0_2 and DCI format 1_2, or, if a UE indicates a corresponding capability, for DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1

----------------------------------------------End of TP ----------------------------------------------------

Decision: As per email decision posted on Mar. 4th, the following TPs are endorsed.

R1-2001405	Text proposal on Antenna port(s) for TS 38.212 Section 7.3.1.1.3 & 7.3.1.2.3	Huawei

R1-2001406	Text proposal on resource allocation type 1 for TS 38.212 Section 7.3.1.1.3 & 7.3.1.2.3	Huawei

R1-2001407	Text proposal on resource allocation type 1 for TS 38.214 Section 5.1.2.2.2 & 6.1.2.2.2	Huawei

[100e-NR-L1enh_URLLC_PDCCH-02] – Chengyan (Huawei)

Email discussion/approval on remaining issues on PDCCH candidate overbooking and dropping:

- Remaining issues on span based PDCCH candidate overbooking and dropping

- Correction on span definition

by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

R1-2001408	Summary of email discussion [100e-NR-L1enh_URLLC_PDCCH-02] on remaining issues on PDCCH candidate overbooking and dropping	Huawei

Decision: As per email decision posted on Mar. 2nd,

Agreements:

- The same framework for PDCCH candidate dropping in a span in terms of non-overlapped CCEs per slot for channel estimation, is also used for PDCCH candidate dropping in a span in terms of number of monitored PDCCH candidates per slot.

Agreements:

- Take the following text proposal to reflect the agreement related to how to determine a valid combination (X, Y) in section 10.1 in the TS 38.213.

------------------------------------ Start of TP for 38.213 --------------------------------------------

10.1	 UE procedure for determining physical downlink control channel assignment

< Unchanged parts are omitted >

A UE can indicate a capability to monitor PDCCH according to one or more of the combinations X,Y = (2, 2), (4, 3), and (7, 3) per SCS configuration of μ=0 and μ=1. If the UE indicates a capability to monitor PDCCH according to multiple X,Y combinations and a configuration of search space sets to the UE results  in a span pattern with a separation of any two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for two or more of the multiple  X,Y combinations, the UE is expected to monitor PDCCH according to the combination X,Y associated with the largest maximum number of CPDCCHmax,X,Y,μ and MPDCCHmax,X,Y,μ.

< Unchanged parts are omitted >

------------------------------------- End of TP --------------------------------------------

Decision: As per email decision posted on Mar. 6th,

Agreements:

- Take the following text proposal to correct the description for span pattern and UE reported combination (X, Y) in section 10 and 10.1 in the TS 38.213:

------------------------------------ Start of TP for 38.213 --------------------------------------------

10	UE procedure for receiving control information

< Unchanged parts are omitted >

A UE reports one or more combinations of (X, Y) number of symbols, where X ≥ Y, for PDCCH monitoring. A span is a set of consecutive symbols in a slot in which the UE is configured to monitor PDCCH candidates. The UE supports PDCCH monitoring occasions in any symbol of a slot with minimum time separation of X symbols between the first symbol of two consecutive spans, including the cross-slot boundary case. The duration of a span is max{maximum value of all CORESET durations configured to the UE, minimum value of Y in the UE reported candidate values} except possibly the last span in a slot which can be of shorter duration. X,YX≥Y XYXX

< Unchanged parts are omitted >

10.1 	UE procedure for determining physical downlink control channel assignment

< Unchanged parts are omitted >

Table 10.1-2A provides the maximum number of monitored PDCCH candidates, MPDCCHmax,X,Y,μ, per span for a UE in a DL BWP with SCS configuration μ for operation with a single serving cell.

Table 10.1-2A: Maximum number MPDCCHmax,X,Y,μ of monitored PDCCH candidates in a span for combination (X, Y) for a DL BWP with SCS configuration μ∈0, 1 for a single serving cell

|  | Maximum number MPDCCHmax,X,Y,μ of monitored PDCCH candidates per span for combination X,Y and per serving cell |  |  |
| --- | --- | --- | --- |
| μ | (2, 2) | (4, 3) | (7, 3) |
| 0 | M01 | M02 | M03 |
| 1 | M11 | M12 | M13 |

< Unchanged parts are omitted >

Table 10.1-3A provides the maximum number of non-overlapped CCEs, CPDCCHmax,X,Y,μ, for a DL BWP with SCS configuration μ that a UE is expected to monitor corresponding PDCCH candidates per span for operation with a single serving cell.

Table 10.1-3A: Maximum number CPDCCHmax,X,Y,μ of non-overlapped CCEs in a span for combination (X, Y) for a DL BWP with SCS configuration μ∈0, 1 for a single serving cell

|  | Maximum number CPDCCHmax,X,Y,μ of non-overlapped CCEs per span for combination X,Y and per serving cell |  |  |
| --- | --- | --- | --- |
| μ | (2, 2) | (4, 3) | (7, 3) |
| 0 | C01 | C02 | 56 |
| 1 | C11 | C12 | 56 |

< Unchanged parts are omitted >

------------------------------------- End of TP --------------------------------------------

[100e-NR-L1enh_URLLC_PDCCH-03] – Chengyan (Huawei)

Email discussion/approval on the following remaining issue on enhanced PDCCH monitoring capability:

- Scaling PDCCH monitoring capability if the number of CCs configured is larger than the reported capability

by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

R1-2001409	Summary of email discussion [100e-NR-L1enh_URLLC_PDCCH-03] on remaining issue on enhanced PDCCH monitoring capability	Huawei

Decision: As per email decision posted on Mar. 5th, the following proposals aretaken as working assumption. Other aspects for the next meeting.

Working assumption:

If a UE is configured with Ncells,r16DL,(X,Y),μ downlink cells with Rel-16 PDCCH monitoring capability with an associated combination (X, Y) and SCS configuration µ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, the UE is not required to monitor more than CPDCCHtotal,(X,Y),μ non-overlapping CCEs per span on the active DL BWP(s) of scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells if the spans on all downlink cells from the Ncells,r16DL,(X,Y),μ downlink cells are aligned, where

CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j

  - is the number serving cells configured with Rel-16 PDCCH monitoring capability with SCS configuration j.

  - If a UE is configured with multiple carriers with a mix of Rel-15 and Rel-16 PDCCH monitoring capability, Ncellscap-r16 is replaced by.

  - The associated combination (X, Y) is the combination (X, Y) associated with largest maximum number of CPDCCHmax,X,Y,μ , if the UE indicates a capability to monitor PDCCH according to multiple (X, Y) combinations and a configuration of search space sets to the UE results in a span pattern with a separation of any two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for two or more of the (X, Y) combinations.

If a UE is configured with Ncells,r16DL,(X,Y),μ downlink cells with Rel-16 PDCCH monitoring capability with an associated combination (X, Y) and SCS configuration µ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ PDCCH candidates per span on the active DL BWP(s) of scheduling cell(s) from the Ncells,r16DL,(X,Y),μ serving cells if the spans on all downlink cells from the Ncells,r16DL,(X,Y),μ downlink cells are aligned, where

MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j

  - is the number serving cells configured with Rel-16 PDCCH monitoring capability with SCS configuration j.

  - If a UE is configured with multiple carriers with a mix of Rel-15 and Rel-16 PDCCH monitoring capability, Ncellscap-r16 is replaced by.

The associated combination (X, Y) is the combination (X, Y) associated with largest maximum number of MPDCCHmax,(X,Y),μ, if the UE indicates a capability to monitor PDCCH according to multiple (X, Y) combinations and a configuration of search space sets to the UE results in a span pattern with a separation of any two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for two or more of the (X, Y) combinations

R1-2000230	Remaining Issue of PDCCH Enhancements for NR URLLC	Ericsson

R1-2000325	PDCCH enhancements for URLLC	vivo

R1-2000355	Remaining issues on PDCCH enhancements for NR URLLC	ZTE

R1-2000432	Maintenance of Rel-16 URLLC PDCCH enhancements	Nokia, Nokia Shanghai Bell

R1-2000451	Remaining issues on PDCCH enhancements	MediaTek Inc.

R1-2000479	PDCCH enhancements for URLLC	OPPO

R1-2000527	Remaining issues on PDCCH enhancements	CATT

R1-2000587	PDCCH enhancements for NR URLLC	Panasonic Corporation

R1-2000625	Remaining issues for PDCCH enhancements	Samsung

R1-2000670	Remaining issues of PDCCH enhancements for NR URLLC	LG Electronics

R1-2000735	Remaining aspects on Rel-16 PDCCH enhancements for URLLC	Intel Corporation

R1-2000855	Remaining Details on PDCCH Enhancements	Apple

R1-2000881	Remaining issues on PDCCH enhancements for NR URLLC	Sharp

R1-2000896	Discussion on remaining issues of PDCCH enhancements for URLLC	Spreadtrum Communications

R1-2000919	Remaining issues for PDCCH enhancements for Rel.16 URLLC	NTT DOCOMO, INC.

R1-2000935	Remaining issues on PDCCH enhancement for NR URLLC	WILUS Inc.

R1-2000968	Remaining issues on PDCCH Enhancements for URLLC	Qualcomm Incorporated

R1-2000995	Remaining issue for TCI field	ASUSTeK

R1-2001023	Corrections on PDCCH enhancement for URLLC	Huawei, HiSilicon

#### UCI enhancements

R1-2001016	Summary#1 on UCI enhancements for R16 URLLC	OPPO

[100e-NR-L1enh_URLLC-UCI_Enh-01] – Jia (OPPO)

Email discussion/approval on the priority of A-SRS triggered by other DCI formats and Type 1 HARQ-ACK codebook construction for PDSCHs scheduled by DCI format 1_1 and 1_2 by 2/28; endorsing the corresponding TP by 3/3

R1-2001226	Summary of email thread [100e-NR-L1enh_URLLC-UCI_Enh-01]	OPPO

Proposal (from email posted on Feb. 29th):

- Priority of A-SRS should follow the priority indicator carried in the DCI that triggers the A-SRS.

Decision: As per email decision posted on Mar. 4th, no conclusion.

[100e-NR-L1enh_URLLC-UCI_Enh-02] – Jia (OPPO)

Email discussion/approval on the following:

- Separately configured parameters for different HARQ-ACK codebooks

- Clarifications related to different HARQ-ACK codebooks

by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

R1-2001227	Summary of email thread [100e-NR-L1enh_URLLC-UCI_Enh-02]	OPPO

Decision: As per email decision posted on Mar. 4th,

Agreement:

When two PUCCH-Configs are configured, SchedulingRequestResourceConfig can be configured in both PUCCH-Configs

- If the same PUCCH resource ID can be configured in different PUCCH-Config, a PUCCH-ResourceId in a SchedulingRequestResourceConfig refers to a PUCCH-Resource in the PUCCH-Config containing the SchedulingRequestResourceConfig

Decision: As per email decision posted on Mar. 4th, the TPs to TS 38.213 (subclauses 9.1 and 9.2.5.2) in R1-2001227 are endorsed.

[100e-NR-L1enh_URLLC-UCI_Enh-03] – Jia (OPPO)

Email discussion/approval on the following:

- Potential extension of K1 value range

- Clarification of K1 unit

by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

R1-2001228	Summary of email thread [100e-NR-L1enh_URLLC-UCI_Enh-03]	OPPO

Decision: As per email decision posted on Feb. 29th, it's concluded with no agreements/spec impact necessary to address the issues.

R1-2000231	Remaining Issue of UCI Enhancements for NR URLLC	Ericsson

R1-2000326	UCI enhancements for URLLC	vivo

R1-2000356	Remaining issues on UL control enhancements for NR URLLC	ZTE

R1-2000405	Maintenance of Rel-16 URLLC UCI enhancements	Nokia, Nokia Shanghai Bell

R1-2000409	Discussion on remaining issues on UCI enhancement for URLLC	Beijing Xiaomi Mobile Software

R1-2000452	Remaining issues on UCI enhancements	MediaTek Inc.

R1-2000480	UCI enhancements for URLLC	OPPO

R1-2000528	Remaining issues on UCI enhancements	CATT

R1-2000550	Remaining issues on UCI enhancements for URLLC	Fujitsu

R1-2000588	Remaining issue on UCI enhancement	Panasonic Corporation

R1-2000626	Remaining issues for UCI enhancements	Samsung

R1-2000671	Remaining issues of UCI enhancements for NR URLLC	LG Electronics

R1-2000736	Remaining details on UCI enhancements for URLLC	Intel Corporation

R1-2000750	Remaining issues on UCI enhancements for URLLC	CMCC

R1-2000775	Remaining issues on UCI enhancements for NR URLLC	Asia Pacific Telecom co. Ltd

R1-2000816	UCI enhancements for URLLC	InterDigital, Inc.

R1-2000897	Discussion on remaining issues of UCI enhancements for URLLC	Spreadtrum Communications

R1-2000920	Discussion on the priority index for the HARQ-ACK codebook	NTT DOCOMO, INC.

R1-2000936	Remaining issues on UCI enhancement for NR URLLC	WILUS Inc.

R1-2000969	Remaining issues on UCI Enhancements for URLLC	Qualcomm Incorporated

R1-2001027	Corrections on UCI enhancement	Huawei, HiSilicon

#### PUSCH enhancements

R1-2000408	Summary #1 for maintenance of PUSCH enhancements in Rel-16 NR URLLC	Nokia, Nokia Shanghai Bell

R1-2001162	Summary #2 for maintenance of PUSCH enhancements in Rel-16 NR URLLC	Nokia, Nokia Shanghai Bell

[100e-NR-L1enh_URLLC-PUSCH_Enh-01] – Sigen (Nokia)

Email discussion/approval on the remaining open points on the RRC parameters and UCI multiplexing on PUSCH by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

R1-2001401	Summary of email discussion [100e-NR-L1enh_URLLC-PUSCH_Enh-01]	Nokia

Decision: As per email decision posted on Mar. 3rd,

Agreement:

- For numberofrepetitions for PUSCH repetition type A and type B, {3, 8} are additionally supported. That is, {1, 2, 3, 4, 7, 8, 12, 16} are supported for numberofrepetitions. (RRC impact)

Agreement:

The value range for repK remains the same as in Rel-15.

Agreement:

For PUSCH repetition Type B, S is from 0 to 13, and L is from 1 to 14. (RRC impact)

Agreement:

For PUSCH repetition Type B, PUSCH transmit power is determined based on the nominal repetition duration.

Agreements:

Introduce reportSlotOffsetList-r16-ForDCIFormat0_1and reportSlotOffsetList-r16-ForDCIFormat0_2 and update TS 38.214 accordingly

- FFS whether or not to always assume the number of nominal repetitions is equal to 1 when PUSCH with repetition Type B carries A-CSI/SP-CSI only.

As per email decision posted on Mar. 5th, theTP to TS 38.214 (see 5.1 in R1-2001401)  is endorsed. Other TPs are for next meeting.

[100e-NR-L1enh_URLLC-PUSCH_Enh-02] – Sigen (Nokia)

Email discussion/approval on frequency hopping for PUSCH repetition Type B by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

R1-2001402	Summary of email discussion [100e-NR-L1enh_URLLC-PUSCH_Enh-02]	Nokia

Decision: As per email decision posted on Mar. 3rd,

Conclusion on how FH is enabled/disabled for Type 2 CG with DCI format 0_1 in Rel-15:

For Type 2 CG in Rel-15 activated by DCI format 0_1, if frequencyHopping in configuredGrantConfig is not configured, FH is disabled. If frequencyHopping in configuredGrantConfig is configured, FH for Type 2 CG is enabled if the frequency hopping flag field in the activation DCI is set to 1, and FH is disabled if the frequency hopping flag field in the activation DCI is set to 0.

Agreement:

- For PUSCH with repetition Type B, with inter-repetition FH, frequency hopping occurs for each nominal repetition.

Agreement:

- For PUSCH repetition Type B, intra-PUSCH-repetition frequency hopping is not supported. (RRC impact)

Agreements:

Adopt the following TP to TS 38.212, Sec. 7.3.1.1.2 (changes in red):

7.3.1.1.2 Format 0_1 <Unchanged text is omitted> - Frequency hopping flag – 0 or 1 bit: - 0 bit if only resource allocation type 0 is configured, or ifboth the higher layer parameterfrequencyHopping is not configured and the higher layer parameterfrequencyHopping-ForDCIFormat0_1 pusch-RepTypeIndicatorForDCI-Format0-1-r16 is are not configured to ‘pusch-RepTypeB’, or if the higher layer parameter frequencyHoppingForDCI-Format0-1-r16 is not configured and pusch-RepTypeIndicatorForDCI-Format0-1-r16 is configured to ‘pusch-RepTypeB’, or if only resource allocation type 2 is configured; - 1 bit according to Table 7.3.1.1.1-3 otherwise, only applicable to resource allocation type 1, as defined in Clause 6.3 of [6, TS 38.214]. <Unchanged text is omitted>

Decision: As per email decision posted on Mar. 4th,

Agreements:

- For Type 2 CG PUSCH activated by a DCI format configured with PUSCH repetition Type B, the frequency hopping enabling/disabling and the frequency offset follows the indication in the activation DCI, and the frequency hopping scheme follows the corresponding RRC parameter for the activation DCI format.

Decision: As per email decision posted on Mar. 5th, TPs to TS 38.214 - Sec. 6.3.2 (on Type 2 CG FH and on inter-repetition FH) in R1-2001402 are endorsed

[100e-NR-L1enh_URLLC-PUSCH_Enh-03] – Sigen (Nokia)

Email discussion/approval on remaining issues on interaction with DL/UL directions (referenced by sections 4.1/4.2.4/4.3 in the summary) by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3

R1-2001403	Summary of email discussion [100e-NR-L1enh_URLLC-PUSCH_Enh-03]	Nokia

Decision: As per email decision posted on Mar. 3rd,

Agreements:

- The semi-static and dynamic indication of invalid symbols (related to InvalidSymbolPattern) for DG PUSCH repetition Type B in case dynamic SFI is not configured follows the same behaviour as for DG PUSCH repetition Type B in case dynamic SFI is configured.

- For Type 1 CG PUSCH with repetition Type B, regardless of whether dynamic SFI is configured or not, if InvalidSymbolPattern is configured, the configured pattern is applied (that is, segmentation occurs around semi-static DL symbols and invalid symbols indicated by InvalidSymbolPattern).

- For the first Type 2 CG PUSCH with repetition Type B (including all repetitions) after activation, regardless of whether dynamic SFI is configured or not, if InvalidSymbolPattern is configured, whether the configured pattern is applied follows the same procedure as specified for DG PUSCH according to the activation DCI.

- For Type 2 CG PUSCH with repetition Type B (excluding the first Type 2 CG PUSCH, with all repetitions, after activation), regardless of whether dynamic SFI is configured or not, if InvalidSymbolPattern is configured, whether the configured pattern is applied follows the activation DCI.

Agreements:

- For PUSCH repetition Type B, a UE is not expected to be indicated with an antenna port configuration that is invalid for the duration of any actual repetition.

- For PUSCH with repetition Type B, an actual repetition with a single symbol is not transmitted.

Decision: As per email decision posted on Mar. 6th, TPs to TS 38.214 - 6.1.2.1 (related to InvalidSymbolPattern and a single-symbol repetition) and 6.2.2 TP (on antenna port/DMRS configuration) in R1-2001403 are endorsed

R1-2000232	Remaining Issue of PUSCH Enhancements for NR URLLC	Ericsson

R1-2000327	PUSCH enhancements for URLLC	vivo

R1-2000357	Remaining issues on PUSCH enhancements for NR URLLC	ZTE

R1-2000406	Maintenance of PUSCH enhancements for Rel-16 NR URLLC	Nokia, Nokia Shanghai Bell

R1-2000481	PUSCH enhancements for URLLC	OPPO

R1-2000529	Remaining issues on PUSCH enhancements	CATT

R1-2000585	On DMRS of segmented PUSCH	Sony

R1-2000589	Remaining issues on URLLC PUSCH enhancement	Panasonic Corporation

R1-2000627	Remaining issues for PUSCH enhancements	Samsung

R1-2000672	Remaining issues of PUSCH enhancements for NR URLLC	LG Electronics

R1-2000751	Remaining issues on PUSCH enhancements for URLLC	CMCC

R1-2000856	Remaining details of PUSCH enhancements	Apple

R1-2000902	Frequency hopping in PUSCH repetition	Motorola Mobility, Lenovo

R1-2000921	Remaining issues for PUSCH enhancements for Rel.16 URLLC	NTT DOCOMO, INC.

R1-2000937	Remaining issues on PUSCH repetition type B for NR URLLC	WILUS Inc.

R1-2000970	Remaining issues on PUSCH enhancements for URLLC	Qualcomm Incorporated

R1-2001028	Corrections on PUSCH enhancement	Huawei, HiSilicon

R1-2001046	Remaining Issues of PUSCH Enhancements for NR URLLC	CAICT

R1-2001085	Remaining details on PUSCH enhancements for URLLC	Intel Corporation

#### Enhancements to scheduling/HARQ

R1-2000971	Summary of the remaining issues on HARQ and scheduling enhancements for URLLC	Qualcomm Incorporated

R1-2000233	Remaining Issue of Scheduling/HARQ for NR URLLC	Ericsson

R1-2000328	Enhancement for Scheduling/HARQ	vivo

R1-2000358	Remaining issues on scheduling/HARQ enhancements for NR URLLC	ZTE

R1-2000482	Discussion on Out-of-Order HARQ	OPPO

R1-2000530	Remaining issues on out-of-order scheduling/HARQ	CATT

R1-2000590	Remaining issues on out-of-order HARQ and PUCCH overlapping	Panasonic Corporation

R1-2000628	Remaining issues for scheduliung/HARQ enhancements	Samsung

R1-2000673	Remaining issues of Enhancements to scheduling/HARQ for NR URLLC	LG Electronics

R1-2000737	Remaining details on scheduling and HARQ enhancements for URLLC	Intel Corporation

R1-2000752	Remaining issues on scheduling and HARQ	CMCC

R1-2001031	Corrections on operation of out-of-order	Huawei, HiSilicon

#### Enhanced inter UE Tx prioritization/multiplexing

R1-2000801	Summary of UL inter UE Tx prioritization for URLLC	vivo

R1-2001180	Summary#2 of UL inter UE Tx prioritization for URLLC	vivo

[100e-NR-L1enh_URLLC-Inter_UE-01] – Xueming (vivo)

Email discussion/approval on RRC parameter related issues (issues 1 to 5 as in R1-2000801) by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2

R1-2001290	Summary of email discussion [100e-NR-L1enh_URLLC-Inter_UE-01]	vivo

Decision: As per email decision posted on Feb. 28th,

Agreements:

- Confirm that 14OS can be configured for timedurationforCI (when 1-slot is the configured UL CI monitoring periodicity with more than one monitoring occasions within 1 slot)

- The possible values for CI-PayloadSize, are {1,2,4,5,7,8,10,14,16,20, 28,32,35,42,56,112}

The following TP is endorsed

------------------------------------ Start of TP for 38.213 --------------------------------------------

11.2A	Cancellation indication

< Unchanged parts are omitted >

For a group of symbols,  bits from each set of bits have a one-to-one mapping with  groups of PRBs where each of the first  groups includes  PRBs and each of the remaining  groups includes  PRBs. A UE determines a first PRB index as  and a number of contiguous RBs as  from frequencyRegionforCI that indicates an offset  and a length  as RIV according to [6, TS 38.214], and from offsetToCarrier in FrequencyInfoUL-SIB that indicates  for a SCS configuration of an active DL BWP where the UE monitors PDCCH for DCI format 2_4 detection.

< Unchanged parts are omitted >

------------------------------------ End of TP for 38.213 --------------------------------------------

Decision: As per email decision posted on Feb. 29th,

Agreements:

- The maximum UL CI monitoring periodicity is 10 slots.

- Up to X BDs can be configured per UL CI monitoring occasion, X to be decided between X=1 or X=2 in RAN1#100bis.

No RAN1 spec impact - RRC parameter update only.

[100e-NR-L1enh_URLLC-Inter_UE-02] – Xueming (vivo)

Email discussion/approval on UE behavior related issues (issues #1, #3, #5) as in R1-2000801) by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2

R1-2001291	Summary of email discussion [100e-NR-L1enh_URLLC-Inter_UE-02]	vivo

Decision: As per email decision posted on Mar. 3rd,

Agreements:

- UE derives the RUR start based on “logical time” (i.e. assuming DL timing difference is 0 and TA=0) and the actual cancellation symbol based on “actual time” (i.e. assuming actual DL timing difference, actual TA)

- A new RRC parameter delta_offset d having possible values {0, 1, 2} OFDM symbols is introduced, update the spec as the following

For the serving cell, the UE determines the first symbol of the symbols to be the first symbol that is after + d from the end of a PDCCH reception where the UE detects the DCI format 2_4.corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming with being the smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of an SRS transmission on the serving cell.

- Clarify the following by a RAN1 spec update (see below)

  - UE is not expected to cancel the transmission of SRS or PUSCH before the first symbol that is T_proc,2 after the end of the reception of the last symbol of the PDCCH carrying the ULCI including the effect of the timing advance.

38.213 Text proposal (maybe further refined by spec editor)

An indication by a DCI format 2_4 for a serving cell is applicable to PUSCH or SRS transmissions on the serving cell. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by higher layer parameter [xxxx]. Tproc,2 corresponds to the PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=0 with μ being the smallest SCS configuration between the SCS configurations of the PDCCH and of a PUSCH transmission or of an SRS transmission on the serving cell. UE is not expected to cancel the transmission of SRS or PUSCH before the first symbol that is Tproc,2 after the end of the reception of the last symbol of the PDCCH carrying the ULCI including the effect of the timing advance.

Decision: As per email decision posted on Mar. 3rd,

No conclusion on issues related to the interaction between intra-UE UL prioritization/multiplexing and inter-UE UL cancellation.

Postponed to RAN1#100bis

Last issue: Whether another UL transmission can be scheduled in the cancelled symbols that do not overlap with the resource indicated by UL CI. Also postponed to RAN1#100bis

[100e-NR-L1enh_URLLC-Inter_UE-03] – Xueming (vivo)

Email discussion/approval on UE behavior related issues (issues #6, #7, #8, #9) as in R1-2000801) by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2

R1-2001292	Summary of email discussion [100e-NR-L1enh_URLLC-Inter_UE-03]	vivo

Decision: As per email decision posted on Feb. 28th,

Agreements:

- UE performs the UL cancellation based on any detected UL CI, no additional specification for the case of overlapping reference time region for multiple UL CI occasions.

- A cancelled PUSCH transmission by a UE is counted towards the number of PUSCH that a UE can support per slot

Decision: As per email decision posted on Feb. 28th, the following  is taken as a conclusion with the understanding there is no spec impact

Conclusion:

- It is possible for a UE to indicate both  pa-PhaseDiscontinuityImpacts  (i.e. 6-23) and the support of UL CI for intra-band UL CA

- For a UE indicates a capability to cancel overlapping PUSCHs on different intra-band serving cells (if any), and the capability of pa-PhaseDiscontinuityImpacts, and if the PUSCH on at least one serving cell is cancelled, the UE cancels the (repetition of the) PUSCHs transmission on all other intra-band serving cell(s). The cancellation of the (repetition of the) PUSCH transmission on a the set of intra-band serving cell(s) includes all symbols from the earliest symbol that is overlapping with the first cancelled symbol of the PUSCH on the serving cell for which the DCI format 2_4 is applicable to.

R1-2000234	Remaining Issue of Inter-UE Prioritization and Multiplexing of  UL Transmissions	Ericsson

R1-2000329	UL inter UE Tx prioritization for URLLC	vivo

R1-2000359	Remaining issues on UL inter-UE multiplexing between eMBB and URLLC	ZTE

R1-2000433	Maintenance of Rel-16 URLLC Enhanced inter UE Tx prioritization/multiplexing	Nokia, Nokia Shanghai Bell

R1-2000453	Remaining issues on enhanced inter UE Tx prioritization/multiplexing	MediaTek Inc.

R1-2000483	Inter UE Tx prioritization and multiplexing	OPPO

R1-2000531	Remaining issues on inter-UE UL multiplexing	CATT

R1-2000586	On the granularity of overlapping Reference Region for UL CI	Sony

R1-2000629	Remaining issues for inter-UE multiplexing	Samsung

R1-2000674	Remaining issues of UL inter UE Tx prioritization	LG Electronics

R1-2000738	Remaining details on enhanced inter-UE multiplexing	Intel Corporation

R1-2000753	Remaining issues on inter UE Tx prioritization/multiplexing	CMCC

R1-2000857	Remaining Details on Enhanced Inter-UE Prioritization/Multiplexing	Apple

R1-2000938	Remaining issues on UL cancellation indication for NR URLLC	WILUS Inc.

R1-2000972	Remaining issues on uplink Inter-UE Tx Multiplexing and Prioritization	Qualcomm Incorporated

R1-2001025	Corrections on UL inter-UE multiplexing	Huawei, HiSilicon

R1-2001094	Inter-UE prioritization/multiplexing	InterDigital, Inc.

#### Enhanced UL configured grant transmission

R1-2000923	Summary on remaining issues for Rel.16 enhanced configured grant	NTT DOCOMO, INC.

R1-2001171	Summary#2 on remaining issues for Rel.16 enhanced configured grant	NTT DOCOMO, INC.

[100e-NR-L1enh_URLLC-eCG-01] – Lihui (NTT DOCOMO)

Email discussion/approval on RRC parameter exceptions for Type 2 CG activated by DCI format 0_2 by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2

R1-2001422	Outcome of email discussion on [100e-NR-L1enh_URLLC-eCG-01]	NTT DOCOMO

Decision: As per email decision posted on Feb. 29th,

Agreements:

- Following parameters provided by pusch-Config are not supported as exceptions for a Type 2 configured grant activated by DCI format 0_2

  - resourceAllocation-ForDCIFormat0_2,dynamic-ForDCIFormat0_2 of UCI-OnPUSCH, dmrs-UplinkForPUSCH-MappingTypeA-ForDCIFormat0_2 ,dmrs-UplinkForPUSCH-MappingTypeB-ForDCIFormat0_2

  - Note: Those parameter are replace with resourceAllocation, dynamic of CG-UCI-OnPUSCH, cg-DMRS-Configuration in corresponding configuredGrantConfig for a Type 2 configured grant activated by DCI format 0_2, respectively.

- Following parameters provided by pusch-Config are not supported as exceptions for a Type 2 configured grant activated by DCI format 0_2

  - numberOfBitsForRV-ForDCI-Format0-2-r16, harq-ProcessNumberSizeForDCI-Format0-2-r16, dmrs-SequenceInitializationForDCI-Format0-2-r16

  - Note: Those parameter are applied to both DCI format with C-RNTI and CS-RNTI for determining associated DCI field size.

- Following parameter provided by pusch-Config is supported as exceptions for a Type 2 configured grant activated by DCI format 0_2

  - resourceAllocationType1-granularity-ForDCIFormat0_2

Decision: As per email decision posted on Mar. 3rd, the TP to 38.214 – 6.1 in R1-2001422 is endorsed.

[100e-NR-L1enh_URLLC-eCG-02] – Lihui (NTT DOCOMO)

Email discussion/approval on PDCCH validation of SPS/Type 2 CG release by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2

R1-2001423	Outcome of email discussion on [100e-NR-L1enh_URLLC-eCG-02]	NTT DOCOMO

Decision: As per email decision posted on Feb. 29th,

Agreements:

- For FDRA field for single UL grant Type 2 scheduling release PDCCH validation with DCI format 0_1/DCI format 0_2, keep current spec unchanged, i.e., set the value to all ‘1’s.

- For FDRA field for multiple UL grant Type 2 scheduling release PDCCH validation, keep current spec unchanged, i.e., set the value to all ‘1’s.

- Following are not supported for Rel.16 URLLC Type 2CG and SPS:

  - Use the TPC command for scheduled PUSCH field in DCI format 0_0/0_1/0_2 as additional validation of activation or release for NR Rel-16 Type 2 CG.

  - Use the TPC command for scheduled PUCCH field in DCI format 1_0/1_1/1_2 as additional validation of activation or release for NR Rel-16 DL SPS.

Note: no RRC impact, no TP is needed.

[100e-NR-L1enh_URLLC-eCG-03] – Lihui (NTT DOCOMO)

Email discussion/approval on DMRS configuration for Type 2 CG activated by DCI format 0_2 by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2

R1-2001424	Outcome of email discussion on [100e-NR-L1enh_URLLC-eCG-03]	NTT DOCOMO

Decision: As per email decision posted on Mar. 3rd, as the  discussion is highly related to the discussion in PDCCH (not complete yet), discussion is postponed to next meeting.

R1-2000235	Remaining Issue of Enhancements to UL Configured Grant Transmission for NR URLLC	Ericsson

R1-2000330	Enhanced UL configured grant transmissions  for URLLC	vivo

R1-2000360	Remaining issues on enhancements for UL configured grant transmission	ZTE

R1-2000434	Maintenance of Rel-16 URLLC Configured Grant enhancements	Nokia, Nokia Shanghai Bell

R1-2000484	Configured grant enhancements for URLLC	OPPO

R1-2000532	Remaining issues on enhanced UL configured grant transmission	CATT

R1-2000630	Remaining issues for configued grant	Samsung

R1-2000675	Remaining issues of Enhanced UL configured grant transmission for NR URLLC	LG Electronics

R1-2000922	Remaining issues for enhanced Configured grant for Rel.16 URLLC	NTT DOCOMO, INC.

R1-2001024	Corrections on configured grant transmission	Huawei, HiSilicon

#### Other

R1-2000677	Summary on maintenance of other aspects for URLLC/IIOT	LG Electronics

R1-2001184	Summary#2 on maintenance of other aspects for URLLC/IIOT	LG Electronics

R1-2001225	Summary#3 on maintenance of other aspects for URLLC/IIOT	LG Electronics

[100e-NR-L1enh_URLLC-SPS_enh-01] – Hyunho (LGE)

Email discussion/approval on SPS PDSCH collison (reference issue 2.1 in the summary) by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/3

R1-2001382	Outcome of email thread [100e-NR-L1enh_URLLC-SPS_enh-01]	LG Electronics

Decision: As per email decision posted on Mar. 3rd,

Agreements:

For a UE not indicating a capability to receive more than one unicast PDSCH per slot, in a slot with more than one SPS PDSCHs each without a corresponding PDCCH and no dynamic scheduled PDSCH and/or for SPS PDSCH release, a UE is not required to receive SPS PDSCHs other than the SPS PDSCH with the lowest SPS configuration index among SPS PDSCHs in a slot (regardless of whether SPS PDSCHs are overlapped or not).

- The UE shall report HARQ-ACK feedback only for the SPS PDSCH with the lowest SPS configuration index among SPS PDSCHs in the slot.

Decision: As per email decision posted on Mar. 4th, the TP to TS 38.214, Section 5.1 in R1-2001382 is endorsed.

[100e-NR-L1enh_URLLC-SPS_enh-02] – Hyunho (LGE)

Email discussion/approval on HARQ-ACK feedback for DL SPS (reference issues 2.2/2.4/3.3 in the summary) by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/3

R1-2001383	Outcome of email thread [100e-NR-L1enh_URLLC-SPS_enh-02]	LG Electronics

Decision: As per email decision posted on Mar. 3rd,

Agreements:

- In a slot with more than one SPS PDSCHs each without a corresponding PDCCH, for Type-1 HARQ-ACK codebook and without HARQ-ACK feedback for dynamic scheduled PDSCH and/or for SPS PDSCH release in the slot, or for Type-2 HARQ-ACK codebook, HARQ-ACK feedback for a SPS PDSCH should not be included in the HARQ-ACK codebook if the SPS PDSCH would not be received among overlapping SPS PDSCHs without associated PDCCH.

- For HARQ-ACK of SPS PDSCH (without dynamic scheduled PDSCH), the PUCCH resource is determined based on SPS-PUCCH-AN-List once it is configured, regardless of the number of active SPS configurations.

Decision: As per email decision posted on Mar. 5h, the TP to TS 38.213, Section 9.1.2 and TP to TS 38.213, Section 9.2.1 and Section 9.2.3 in R1-2001383 are endorsed.

[100e-NR-L1enh_URLLC-SPS_enh-03] – Hyunho (LGE)

Email discussion/approval on PDSCH aggregation for DL SPS (reference issues 2.5 and 2.6 in the summary) by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/3

R1-2001384	Outcome of email thread [100e-NR-L1enh_URLLC-SPS_enh-03]	LG Electronics

Decision: As per email decision posted on Mar. 3rd,

Agreements:

Introduce configuration of PDSCH aggregation factor (pdsch-AggregationFactor) per DL SPS configuration with the value range of {1,2,4,8} [RRC impact]

- For PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2, or PDSCH scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled with CS-RNTI with NDI=0

  - PDSCH aggregation factor signaled in sps-Config (newly introduced RRC parameter) is applied if configured; otherwise, PDSCH aggregation factor signaled in pdsch-Config is applied

- For PDSCH scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled with CS-RNTI with NDI=1

  - PDSCH aggregation factor signaled in pdsch-Config is applied

Agreements:

For PDSCH scheduled by DCI format 1_1 or 1_2 in PDCCH with CRC scrambled by CS-RNTI with NDI=0, or PDSCH scheduled without corresponding PDCCH transmission using sps-Config and activated by DCI format 1_1 or 1_2, the UE is not expected to be configured with the time duration for the reception of pdsch-AggregationFactor repetitions in sps-Config (if configured) or in pdsch-config (otherwise) larger than the time duration derived by the periodicity P obtained from the corresponding sps-Config.

Decision: As per email decision posted on Mar. 4th, the TP to TS 38.214, Section 5.1.2.1 in R1-2001384 is endorsed.

R1-2000236	Remaining Issue of DL SPS Enhancements for NR URLLC	Ericsson

R1-2000331	Other issues for URLLC	vivo

R1-2000407	Maintenance of Rel-16 URLLC/IIoT SPS enhancements	Nokia, Nokia Shanghai Bell

R1-2000485	DL SPS enhancement	OPPO

R1-2000533	Remaining issues on DL SPS enhancement	CATT

R1-2000631	Remaining issues for DL SPS and PHR	Samsung

R1-2000676	Remaining issues of other aspects for URLLC/IIOT	LG Electronics

R1-2000776	Remaining issues on DL SPS enhancements	Asia Pacific Telecom co. Ltd

R1-2000924	PUCCH resource determination for HARQ-ACK for SPS PDSCH	NTT DOCOMO, INC.

R1-2000943	Remaining RAN1 issues on IIoT related enhancements	ZTE

R1-2001065	Remaining Issues on DL SPS Enhancements for NR URLLC	CAICT

R1-2001078	Remaining details on enhanced UL CG PUSCH and DL SPS for URLLC	Intel Corporation

R1-2001106	Corrections on other aspects for URLLC/IIOT enhancements	Huawei, HiSilicon

### Maintenance of Enhancements on MIMO for NR

#### CSI Enhancement for MU-MIMO Support

R1-2001129	Feature lead summary for MU-MIMO CSI	Samsung

R1-2001157	Feature lead summary#2 for MU-MIMO CSI	Samsung

[100e-NR-eMIMO-MUCSI-01] – Eko (Samsung)

Email discussion/approval regarding the following:

- (SB size is undefined when NPRB < 24): use the following two alternatives as a starting point and aim to finalize the corresponding TP

  - Alt1. NSB = 1 if csi-ReportingBand is not present (when NPRB < 24)

  - Alt2. UE expects csi-ReportingBand to be configured

    - Note: This implies that when the UE is configured with eType-II CSI, the UE is not expected to be configured with NPRB < 24 (i.e. NPRB is at least 24)

by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001215	Feature lead summary for [100e-NR-eMIMO-MUCSI-01]	Samsung

Agreement

- The following issue is to be finalized in the next RAN1 meeting:

When BWP size < 24 PRBs, should CSI reporting be supported? Rel.15 maintenance issue - If yes, what feature(s) need to be refined (e.g. subband size table, csiReportingBand definition, cqi-/pmi-FormatIndicator definition/ configuration, ...) Rel.15 maintenance issue - If no, what spec clarification is needed (e.g. subband size table, csiReportingBand definition, cqi-/pmi-FormatIndicator definition/ configuration, ...) Rel.15 maintenance issue - If yes, which of the Rel.15 codebooks are supported? Rel.15 maintenance issue - If yes, are codebookType Rel.16 eType-II, eType-II port selection supported? Rel.16 maintenance issue

- Issues a, b, and c are to be discussed under Rel.15 maintenance on NR-MIMO

- Issue d is to be discussed under Rel.16 maintenance on eMIMO MU-CSI informed by the outcome (or parallel with) the Rel.15 discussion for issues a, b, and c

[100e-NR-eMIMO-MUCSI-02] – Eko (Samsung)

Email discussion/approval regarding the following:

- (Necessity and/or configuration for pmi-FormatIndicator):

  - For eType-II CSI and eType-II port selection CSI, the parameter pmi-FormatIndicator is not used/configured

by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001216	Feature lead summary for [100e-NR-eMIMO-MUCSI-02]	Samsung

Agreement

The TP for TS 38.214 in section 3 of R1-2001216 is agreed. TP is to be included as part of the editor’s CR for TS38.214.

[100e-NR-eMIMO-MUCSI-03] – Eko (Samsung)

Email approval for a TP addressing the issues summarized in R1-2001157 (H2/E.1/E.2/E.3) by 2/28

R1-2001217	Feature lead summary for [100e-NR-eMIMO-MUCSI-03]	Samsung

Revised in

R1-2001362	Feature lead summary#2 for [100e-NR-eMIMO-MUCSI-03]	Samsung

Decision: As per email decision posted on Mar. 3rd,

Agreement

The following TPs are endorsed. To be included in editor’s CR.

- First TP in section 3 of R1-2001362 for TS 38.212 section 6.3.2.1.2

- Second TP in section 3 of R1-2001362 for TS 38.214 sections 5.2.2.2.5, 5.2.2.2.6, and 5.2.3.

R1-2000205	Remaining issues on MU-CSI in R16	Huawei, HiSilicon

R1-2000238	Maintenance of CSI enhancement for MU-MIMO support	ZTE

R1-2000332	Discussion on remaining issues on MU-CSI	vivo

R1-2000456	Text proposals for CSI enhancements for MU-MIMO support	OPPO

R1-2000534	Remaining issues on CSI enhancement for MU-MIMO	CATT

R1-2000632	Correction on CSI part 2 payload in 38.212	Samsung

R1-2000678	Text proposals on CSI Enhancement for MU-MIMO Support	LG Electronics

R1-2000858	Remaining issues for Rel-16 Type II CSI enhancement	Apple

R1-2000939	Remaining issues for MU-CSI	Ericsson

R1-2000973	Remaining issues on CSI enhancement for MU-MIMO support	Qualcomm Incorporated

R1-2000987	Maintenance on Rel-16 CSI enhancements	Nokia, Nokia Shanghai Bell

R1-2001119	Text proposals for Rel. 16 Type-II CSI enhancements	Fraunhofer IIS, Fraunhofer HHI

#### Enhancements on Multi-TRP/Panel Transmission

R1-2000865	Summary #1 of Multi-TRP/Panel Transmission	OPPO

R1-2001169	Summary #2 of Multi-TRP/Panel Transmission	OPPO

[100e-NR-eMIMO-multiTRP-01] – Li (OPPO)

Email discussion/approval on configuration of new DMRS table entry {0,2,3} for single-PDCCH based multi-TRP transmission and on the operation of default TCI-state of PDSCH reception in single-PDCCH based systems by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001377	Outcome of email thread [100e-NR-eMIMO-multiTRP-01]	OPPO

Decision: As per email decision posted on Mar. 4th,

Agreement

The following text proposal for TS38.212 is endorsed. To be included in the editor’s CR for TS38.212.

——————————————————— TP start ——————————————————

< Unchanged parts are omitted >

Antenna port(s) – 4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4 and Tables 7.3.1.2.2-1A/2A/3A/4A, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports <image003.png> shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4 or Tables 7.3.1.2.2-1A/2A/3A/4A. When a UE receives an activation command that maps at least one codepoint of DCI field 'Transmission Configuration Indication' to two TCI states, the UE shall use Tables 7.3.1.2.2-1A/2A/3A/4A, otherwise, it shall use Tables 7.3.1.2.2-1/2/3/4. The UE can is not expected to receive an entry with DMRS ports equals to 1000, 1002, 1003 when two TCI states are a single TCI state is indicated in a codepoint of the DCI field ‘Transmission Configuration Indication’ [and subject to UE capability].

< Unchanged parts are omitted >

——————————————————— TP end  ——————————————————

[100e-NR-eMIMO-multiTRP-02] – Li (OPPO)

Email discussion/approval on on HARQ-ACK codebook in multi-PDCCH based multi-TRP system by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Mar. 2nd,

Agreement

For type-2 HARQ-ACK codebook when dynamic joint feedback is configured in multi-PDCCH-based multi-TRP:

- In a serving cell configured multi-PDCCH-based multi-TRP transmission, the CORESETs associated with CORESETPoolIndex = 0 are counted first and then the CORESETs associated with CORESETPoolInde = 1 are counted.

R1-2001348	Text Proposal 1 for [100e-NR-eMIMO-multiTRP-02]	OPPO

R1-2001349	Text Proposal 2 for [100e-NR-eMIMO-multiTRP-02]	OPPO

Decision: As per email decision posted on Mar. 4th,

Agreement

The following TPs are endorsed. To be included in editor’s CR.

- Text proposal in R1-2001348 for TS38.213

- Text proposal in R1-2001349 for TS38.213

[100e-NR-eMIMO-multiTRP-03] – Li (OPPO)

Email discussion/approval on on DAI bits for joint HARQ-ACK feedback and timing determination of HARQ-ACK codebook for scheme4 by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

Decision: As per email decision posted on Mar. 2nd,

Agreement

For the number of DAI bits in DCI format 1_1 when dynamic joint HARQ-ACK codebook is configured in multi-PDCCH based multi-TRP system：

- The number of DAI bits is 4 when one serving cell is configured and dynamic joint HARQ-ACK codebook is configured for multi-PDCCH based multi-TRP system.

For the timeline of Type-1 HARQ-ACK codebook of scheme 4：

- For semi-static codebook determination, N^repeat_PDSCH = 1 if at least one entry in pdsch-TimeDomainAllocationList contains RepNumR16.

- For reporting HARQ-ACK information, the UE reports HARQ-ACK information for a PDSCH reception from slot n - RepNumR16 + 1 to slot n when the scheduling DCI field “Time domain resource assignment’ indicates an entry in pdsch-TimeDomainAllocationList containing RepNumR16

R1-2001360	Text Proposal 1 for [100e-NR-eMIMO-multiTRP-03]	OPPO

R1-2001361	Text Proposal 2 for [100e-NR-eMIMO-multiTRP-03]	OPPO

Decision: As per email decision posted on Mar. 4th,

Agreement

The following TPs are endorsed. To be included in editor’s CR.

- Text proposal in R1-2001360 for TS38.212

- Text proposal in R1-2001361 for TS38.213

R1-2000204	Remaining issues on multi-TRP in R16	Huawei, HiSilicon

R1-2000239	Maintenance of enhancements on multi-TRP transmission	ZTE

R1-2000333	Discussion on remaining issues on M-TRP and text proposals	vivo

R1-2000457	Text proposals for enhancements on multi-TRP and panel Transmission	OPPO

R1-2000535	Remaining issues on Multi-TRP/panel transmission for Rel-16	CATT

R1-2000595	Remaining issues on multi-TRP/panel transmission	Lenovo, Motorola Mobility

R1-2000633	On Rel.16 multi-TRP/panel transmission	Samsung

R1-2000679	Text proposals on enhancements on multi-TRP/panel transmission	LG Electronics

R1-2000739	Correction to multi TRP operation	Intel Corporation

R1-2000754	Maintenance for multi-TRP/panel transmission	CMCC

R1-2000773	UCI enhancements	ETRI

R1-2000812	TP on Multi-TRP/Panel Transmission	Futurewei

R1-2000859	Remaining issues for Multi-TRP enhancement	Apple

R1-2000925	Remaining issues on multi-TRP/panel transmission	NTT DOCOMO, INC.

R1-2000941	Corrections for multi-TRP transmission	Ericsson

R1-2000974	Multi-TRP Enhancements	Qualcomm Incorporated

R1-2000988	Maintenance of Rel-16 Multi-TRP operation	Nokia, Nokia Shanghai Bell

#### Enhancements on Multi-beam Operation

R1-2000681	Summary on Rel-16 Multi-Beam 1 maintenance	LG Electronics

R1-2001161	Revised summary on Rel-16 Multi-Beam 1 maintenance	LG Electronics

[100e-NR-eMIMO-MB-01] – Jiwon (LGE)

Email discussion on PL RS application timing for PUSCH/SRS/PUCCH when updated via MAC CE (issues 1 & 9 in R1-2001161) by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001172	Summary of discussion topic A on Rel-16 MB1	LG Electronics

R1-2001259	Outcome of email thread [100e-NR-eMIMO-MB-01]	LG Electronics

Decision: As per email decision posted on Mar. 4th,

Agreement

The application timing for the newly activated PL RSs is the next slot that is 2ms after the N-th measurement sample, where the 1st measurement sample corresponds to be the 1st instance, 3ms after sending ACK for the MAC CE.

- Note: The value of N can be discussed in UE feature session. If there is no consensus on introducing UE capability for the value of N, N is fixed to 5.

- The application timing is applied to PUSCH, AP/SP-SRS and PUCCH.

- Note: Whether/how to capture above in RAN1 specification or send an LS to other WGs to suggest them to update their specifications accordingly will be decided in the next meeting.

Agreement

The following WA is confirmed with modifications (changes are marked by red):

Pathloss reference RS for PUSCH can be activated/updated via a MAC CE

- The MAC CE message can activate/update the value ofPUSCH-PathlossReferenceRS-Id corresponding to sri-PUSCH-PowerControlId.

- Further signaling details are up to RAN2.

- Reuse higher layer filtered RSRP for pathloss measurement, with defining the applicable timing after the MAC CE.

  - Filtered RSRP value for previous pathloss RS will be used before the application time, which is the next slot that is 2ms after theN5th measurement sample, where the 1st measurement sample corresponds to be the 1st instance, 3ms after sending ACK for the MAC CE.

    - This is only applicable for UEs supporting the number of RRC-configurable pathloss RSs larger than 4, and this is only for the case that the activated PL RS by the MAC CE is not tracked.

    - UE is only required to track the activated PL RS(s) if the configured PL RSs by RRC is greater than 4.

    - It is up to UE whether to update the filtered RSRP value for previous PL RS 3ms after sending ACK for the MAC CE.

    - Note: The value of N can be discussed in UE feature session. If there is no consensus on introducing UE capability for the value of N, N is fixed to 5.

- Send an LS to RAN4 asking opinion on this working assumption. Note: Whether/how to capture above in RAN1 specification or send an LS to other WGs to suggest them to update their specifications accordingly will be decided in the next meeting.

Agreement

The following WA is confirmed with modifications (changes are marked by red):

Pathloss reference RS for AP-SRS/SP-SRS can be activated/updated via a MAC CE.

- A UE can be configured with multiple pathloss RSs by RRC and one of them can be activated/updated via the MAC CE for a SRS resource set.

- Further signaling details are up to RAN2.

- Reuse higher layer filtered RSRP for pathloss measurement, with defining the applicable timing after the MAC CE.

  - Filtered RSRP value for previous pathloss RS will be used before the application time, which is the next slot that is 2ms after theN5th measurement sample, where the 1st measurement sample corresponds to be the 1st instance, 3ms after sending ACK for the MAC CE.

    - This is only applicable for UEs supporting the number of RRC-configurable pathloss RSs larger than 4, and this is only for the case that the activated PL RS by the MAC CE is not tracked.

    - UE is only required to track the activated PL RS(s) if the configured PL RSs by RRC is greater than 4.

    - It is up to UE whether to update the filtered RSRP value for previous PL RS 3ms after sending ACK for the MAC CE.

    - Note: The value of N can be discussed in UE feature session. If there is no consensus on introducing UE capability for the value of N, N is fixed to 5.

- Send an LS to RAN4 asking opinion on this working assumption. Note: Whether/how to capture above in RAN1 specification or send an LS to other WGs to suggest them to update their specifications accordingly will be decided in the next meeting.

[100e-NR-eMIMO-MB-02] – Jiwon (LGE)

Email discussion on how to maintain up to 4 PL RSs for a UE to track when more than 4 PL RSs are configured by RRC (issue 2 in R1-2001161) by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001173	Summary of discussion topic B on Rel-16 MB2	LG Electronics

R1-2001260	Outcome of email thread [100e-NR-eMIMO-MB-02]	LG Electronics

Decision: As per email decision posted on Mar. 3rd,

Agreement:

When the number of RRC configured PL RSs for pathloss estimates for PUCCH, PUSCH and SRS is greater than 4, UE is not required to track the RSs which are not activated by MAC-CE.

- Note: How to capture above into the spec will be discussed at RAN1#100bis

- Note: Further consider the configuration cases when the default PL RS is not enabled or enabled.

Conclusion:

If MAC-CE based PL RS activation/update is not enabled, UE is not expected to be configured with more than 4 PL RS.

[100e-NR-eMIMO-MB-03] – Jiwon (LGE)

Email discussion on remaining details on the features of the default PL RS and the MAC-CE based spatial relation update (issues 4 & 13 in R1-2001161) by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001174	Summary of discussion topic C on Rel-16 MB3	LG Electronics

Agreement

When default PL RS is applied, UE expects to be configured with a periodic RS as QCL-TypeD RS for the default pathloss RS.

R1-2001261	Outcome of email thread [100e-NR-eMIMO-MB-03]	LG Electronics

Decision: As per email decision posted on Mar. 4th,

Agreement

The following TP is endorsed for TS38.313 editor’s CR.

——————————————————— TP start ——————————————————

7.1.1	UE behavior

< unchanged parts omitted >

If

- the PUSCH transmission is scheduled by a DCI format 0_0,

- the UE is not provided PUCCH resources for the active UL BWP, and

- the UE is provided enableDefaultBeamPlForPUSCH0_0

the UE determines a RS resource index  providing a periodic RS resource with 'QCL-TypeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the scheduling cell for the serving cell

If

- the PUSCH transmission is scheduled by a DCI format 0_0,

- the UE is not provided a spatial setting for PUCCH resources on the active UL BWP of the primary cell [11, TS 38.321], and

- the UE is provided enableDefaultBeamPlForPUSCH0_0

the UE determines a RS resource index  providing a periodic RS resource with 'QCL-TypeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the primary cell

- For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig, a RS resource index  is provided by a value of pathlossReferenceIndex included in rrc-ConfiguredUplinkGrant where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

< unchanged parts omitted >

7.2.1	UE behavior

< unchanged parts omitted >

- If the UE

- is not provided pathlossReferenceRSs, and

- is not provided PUCCH-SpatialRelationInfo, and

- is provided enableDefaultBeamPlForPUCCH

the UE determines a RS resource index  providing a periodic RS resource with 'QCL-TypeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the primary cell

< unchanged parts omitted >

7.3.1	UE behavior

< unchanged parts omitted >

- If the UE

- is not provided pathlossReferenceRS or SRS-PathlossReferenceRS,

- is not provided spatialRelationInfo, and

- is provided enableDefaultBeamPlForSRS

the UE determines a RS resource index  providing a periodic RS resource with 'QCL-TypeD' in

- the TCI state or the QCL assumption of a CORESET with the lowest index, if CORESETs are provided in the active DL BWP

- the active PDSCH TCI state with lowest ID [6, TS 38.214], if CORESETs are not provided in the active DL BWP

- For the SRS power control adjustment state for active UL BWP  of carrier  of serving cell  and SRS transmission occasion

——————————————————— TP end ——————————————————

R1-2000861	Feature lead summary on SCell BFR and L1-SINR	Apple

R1-2001182	Feature lead summary#2 on SCell BFR and L1-SINR	Apple

[100e-NR-eMIMO-MB-04] – Yushu (Apple)

Email discussion on remaining details for PDCCH/PUCCH beam/power control after SCell BFR is successful by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001262	Outcome of email thread [100e-NR-eMIMO-MB-04]	Apple

Decision: As per email decision posted on Mar. 3rd,

Agreement:

The following TP is endorsed for the editor’s CR for TS38.213.

————————————— Start of the text proposal on TS 38.213 v16.0.0 section 6 —————————

< Unchanged parts are omitted >

A UE can be provided, by schedulingRequestIDForBFR, a configuration for PUCCH transmission with a link recovery request (LRR) as described in Clause 9.2.4. The UE can transmit in a first PUSCH at least one MAC CE providing one index for at least one corresponding SCell with radio link quality worse than Qout,LR, an index qnew for a periodic CSI-RS configuration or for a SS/PBCH block provided by higher layers, as described in [11, TS 38.321], if any, for a corresponding SCell. After 28 symbols from a last symbol of a PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value, the UE

- receives PDCCH on the at least one SCell with same antenna port quasi-collocation parameters as the ones associated with a corresponding index qnew, if any

- transmits PUCCH on a PUCCH-SCell using a same spatial domain filter as the one corresponding to qnew for periodic CSI-RS or SS/PBCH block reception, as described in Clause 9.2.2,  and a power determined as described in Clause 7.2.1 with qu = 0, qd=qnew, and l=0, if

- the UE is provided PUCCH-SpatialRelationInfo for the PUCCH-SCell,

- the PUCCH transmission with the LRR was on the PCell or the PSCell, and

- the PUCCH-SCell is included in the at least one SCell

where the SCS configuration for the 28 symbols is the smallest of the SCS configurations of the active DL BWP for the PDCCH reception and of the active DL BWP(s) of the at least one SCell.

————————————— End of the text proposal on TS 38.213 v16.0.0 section 6 ————————

Agreement

The following TP is endorsed for the editor’s CR for TS38.213.

————————————— Start of the text proposal on TS 38.213 v16.0.0 section 6 —————————

< Unchanged parts are omitted >

A UE can be provided, by schedulingRequestIDForBFR, a configuration for PUCCH transmission with a link recovery request (LRR) as described in Clause 9.2.4. The UE can transmit in a first PUSCH at least one MAC CE providing one index for at least one corresponding SCell with radio link quality worse than Qout,LR, an index qnew for a periodic CSI-RS configuration or for a SS/PBCH block provided by higher layers, as described in [11, TS 38.321], if any, for a corresponding SCell. After 28 symbols from a last symbol of a PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value, the UE

- receives PDCCH monitors PDCCH in all CORESETs on the at least one SCell(s) indicated by the MAC CE using the with same antenna port quasi-collocation quasi co-location parameters as the ones associated with a the corresponding index qnew(s), if any

- transmits PUCCH on a PUCCH-SCell using a same spatial domain filter as the one corresponding to qnew for periodic CSI-RS or SS/PBCH block reception, as described in Clause 9.2.2, if

- the UE is provided PUCCH-SpatialRelationInfo for the PUCCH-SCell,

- the PUCCH transmission with the LRR, if any, was not on the PUCCH-SCell on the PCell or the PSCell, and

- the PUCCH-SCell is included in the at least one SCell(s) indicated by the MAC CE

where the SCS configuration for the 28 symbols is the smallest of the SCS configurations of the active DL BWP for the PDCCH reception and of the active DL BWP(s) of the at least one SCell.

————————————— End of the text proposal on TS 38.213 v16.0.0 section 6 ————————

[100e-NR-eMIMO-MB-05] – Yushu (Apple)

Email discussion on EPRE assumption and interference measurement behavior for L1-SINR measurement by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001263	Outcome of email thread [100e-NR-eMIMO-MB-05]	Apple

Decision: As per email decision posted on Mar. 3rd,

Agreement

- For L1-SINR, when CSI-RS is configured as CMR and/or IMR, the reported L1-SINR should NOT be compensated by powerControlOffsetSS/powerControlOffset

- The interference measurement mechanism defined in section 5.2.1.4.1 in 38.214 v16.0.0 is also applied to L1-SINR

  - Note: A TP is needed to make it aligned with RAN4’s agreement

Decision: As per email decision posted on Mar. 4th,

Agreement

Text Proposal for 38.214. To be included as part of the editor’s CR for TS38.214.

————————————— start of TP —————————————————

5.2.1.4.1	 Resource Setting configuration

<unrelated part omitted>

For CSI measurement(s) other than L1-SINR, a UE assumes:

- each NZP CSI-RS port configured for interference measurement corresponds to an interference transmission layer.

- all interference transmission layers on NZP CSI-RS ports for interference measurement into account the associated EPRE ratios configured in 5.2.2.3.1;

- other interference signal on REs of NZP CSI-RS resource for channel measurement, NZP CSI-RS resource for interference measurement, or CSI-IM resource for interference measurement

For L1-SINR measurement with dedicated interference measurement resources, a UE assumes:

- the total received power on dedicated NZP CSI-RS resource for interference measurement [and/]or dedicated CSI-IM resource for interference measurement corresponds to interference and noise.

<unrelated part omitted>

————————————— end of TP —————————————————

Agreement

Text Proposal for 38.214. To be included as part of the editor’s CR for TS38.214.

————————————— start of TP —————————————————

5.2.1.4.4	 L1-SINR Reporting

<unrelated part omitted>

For L1-SINR reporting, if the higher layer parameter nrofReportedRSForSINR in CSI-ReportConfig is configured to be one, the reported L1-SINR value is defined by a 7-bit value in the range [-23, 40] dB with 0.5 dB step size, and if the higher layer parameter nrofReportedRSForSINR is configured to be larger than one, the UE shall use differential L1-SINR based reporting, where the largest measured value of L1-SINR is quantized to a 7-bit value in the range [-23, 40] dB with 0.5 dB step size, and the differential L1-SINR is quantized to a 4-bit value. The differential L1-SINR is computed with 1 dB step size with a reference to the largest measured L1-SINR value which is part of the same L1-SINR reporting instance. When NZP CSI-RS is configured for channel measurement and/or interference measurement, the reported L1-SINR value should not be compensated by the power offset(s) given by higher layer parameter powerControlOffsetSS or powerControlOffset.

————————————— end of TP —————————————————

Agreement

Text Proposal for 38.215. To be included as part of the editor’s CR for TS38.215.

————————————— start of TP —————————————————

5.1.5	SS signal-to-noise and interference ratio (SS-SINR)

<unrelated part omitted>

SS signal-to-noise and interference ratio (SS-SINR), is defined as the linear average over the power contribution (in [W]) of the resource elements carrying secondary synchronisation signals divided by the linear average of the noise and interference power contribution (in [W]). If SS-SINR is used for L1-SINR reporting with dedicated interference measurement resources, the interference and noise is measured over resource(s) indicated by higher layers as described in TS 38.214 [6]. Otherwise, the interference and noise are measured over the resource elements carrying secondary synchronisation signals within the same frequency bandwidth. The measurement time resource(s) for SS-SINR are confined within SS/PBCH Block Measurement Time Configuration (SMTC) window duration. If SS-SINR is used for L1-SINR as configured by reporting configurations defined in TS 38.214 [6], the measurement time resources(s) restriction by SMTC window duration is not applicable.

<unrelated part omitted>

5.1.6	CSI signal-to-noise and interference ratio (CSI-SINR)

<unrelated part omitted>

CSI signal-to-noise and interference ratio (CSI-SINR), is defined as the linear average over the power contribution (in [W]) of the resource elements carrying CSI reference signals divided by the linear average of the noise and interference power contribution (in [W]). If CSI-SINR is used for L1-SINR reporting with dedicated interference measurement resources, the interference and noise is measured over resource(s) indicated by higher layers as described in TS 38.214 [6]. Otherwise, the interference and noise are measured over the resource elements carrying CSI reference signals reference signals within the same frequency bandwidth.

————————————— end of TP —————————————————

[100e-NR-eMIMO-MB-06] – Yushu (Apple)

Email discussion on [L1-SINR] Release the constrains for CSI-RS when it is used as CMR and when IMR is configured and L1-SINR measurement with reportQuantity set to none by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001264	Outcome of email thread [100e-NR-eMIMO-MB-06]	Apple

Decision: As per email decision posted on Feb. 29th,

Agreement

The following text proposal is endorsed for alignment CR on TS38.214.

——————————————————— TP start ——————————————————

5.2.1.2 Resource settings

<unrelated part omitted>

For L1-SINR measurement:

- When one Resource Setting is configured, the Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel and interference measurement for L1-SINR computation. UE may assume that same 1 port NZP CSI-RS resource(s) with density 3 REs/RB is used for both channel and interference measurements.

- When two Resource Settings are configured, the first one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement on SSB or 1 port NZP CSI-RS with density 3 REs/RB and the second one (given by either higher layer parameter csi-IM-ResourcesForInterference or higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for interference measurement performed on CSI-IM or on 1 port NZP CSI-RS with density 3 REs/RB, where each SSB or NZP CSI-RS resource for channel measurement is associated with one CSI-IM resource or one NZP CSI-RS resource for interference measurement by the ordering of the SSB or NZP CSI-RS resource for channel measurement and CSI-IM resource or NZP CSI-RS resource for interference measurement in the corresponding resource sets. The number of SSB(s) or CSI-RS resources for channel measurement equals to the number of CSI-IM resources or the number of NZP CSI-RS resource for interference measurement.

———————————————————   TP end ——————————————————

R1-2000203	Remaining issues on multi-beam enhancements in R16	Huawei, HiSilicon

R1-2000240	Maintenance of enhancements on multi-beam operation	ZTE

R1-2000334	Discussion on remaining issues on Multi-Beam and text proposals	vivo

R1-2000458	Remaining issues on Multi-beam Operation Enhancement	OPPO

R1-2000596	Remaining issues on Scell beam failure recovery	Lenovo, Motorola Mobility

R1-2000634	On Rel.16 support for multibeam operation	Samsung

R1-2000680	Text proposals on multi-beam operation	LG Electronics

R1-2000740	Correction to multi beam operation	Intel Corporation

R1-2000755	Maintenance for multi-beam operation	CMCC

R1-2000860	Remaining issues for Multi-beam enhancement	Apple

R1-2000926	Remaining issues on multi-beam operation	NTT DOCOMO, INC.

R1-2000975	Enhancements on Multi-beam Operation	Qualcomm Incorporated

R1-2000989	Maintenance of Rel-16 Beam Management	Nokia, Nokia Shanghai Bell

R1-2001079	On the timing for pathloss RS activation	Ericsson

R1-2001120	Enhancements on UE multi-beam operation	Fraunhofer IIS, Fraunhofer HHI

#### Full TX Power UL transmission

R1-2000802	Feature lead summary on UL full power transmission	vivo

R1-2001167	Summary of 100e-Prep-email discussion on UL full power transmission	vivo

R1-2001209	Feature lead summary#2 on UL full power transmission	vivo

[100e-NR-eMIMO-ULFPTx-01] – Rakesh (vivo)

Email discussion/approval regarding codebook configuration in Mode2 with multiple SRS, one with 4 another with 2 SRS ports by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001255	FL summary of [100e-NR-eMIMO-ULFPTx-01] email thread	vivo

Decision: As per email decision posted on Mar. 5th,

Agreement

- In Mode 2, when at least one SRS resource in a SRS resource set is configured with 4 ports, for a 2 port SRS resource (indicated via SRI) in the same SRS resource set, If maxRank>2, maxRank=2, else use the configured value of maxRank.

- The following text proposal for TS38.214 subclause 6.1.1.1 is endorsed for the editor CR on TS38.214.

UE shall not expect to be configured with the higher layer parametercodebookSubset or the higher layer parameterorcodebookSubset-ForDCIFormat0_2 set to'partialAndNonCoherent' when higher layer parameternrofSRS-Ports in an SRS-ResourceSet with usage set to 'codebook' indicates thattwo SRS antenna ports are configuredthe maximum number of the configured SRS antenna ports in theSRS-ResourceSet is two.

- The following text proposal for TS38.212 is provided as a starting point for implementing this agreement. It is up to TS38.212 editor how to exactly capture this.

7.3.1.1.2	Format 0_1

…….…

For the higher layer parameter txConfig = codebook, if ULFPTxModes is configured to Mode2, if maxRank is configured to be larger than 2, and if SRS resources with 4 ports and 2 ports are configured within an SRS resource set with usage set to ‘codebook’, when the SRS with 2 ports is indicated at least one SRS resource with 4 SRS antenna ports is configured in a SRS resource set with usage set to ‘codebook’ and an SRS resource with 2 SRS antenna ports is indicated via SRI in the same SRS resource set, the field of Precoding information and number of layers is according to Table 7.3.1.1.2-4.
For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in a SRS resource among the configured SRS resources. If the number of ports for a configured SRS resource is less than the maximum number of ports in a SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field.

…….…

Decision: As per email decision posted on Mar. 2nd,

Agreement

The following text proposal for section 6.1.1.1 of TS 38.214 is endorsed for TS38.214 editor’s CR.

————————————— start of TP —————————————————

<unrelated part omitted>

For codebook based transmission, the UE may be configured with a single SRS-ResourceSet with usage set to 'codebook' and only one SRS resource can be indicated based on the SRI from within the SRS resource set. Except when higher layer parameter ULFPTxModes is set to ‘Mode 2’, tThe maximum number of configured SRS resources for codebook based transmission is 2. If aperiodic SRS is configured for a UE, the SRS request field in DCI triggers the transmission of aperiodic SRS resources.

<unrelated part omitted>

Except when higher layer parameter ULFPTxModes is set to ‘Mode 2’, wWhen multiple SRS resources are configured by SRS-ResourceSet with usage set to 'codebook', the UE shall expect that higher layer parameters nrofSRS-Ports in SRS-Resource in SRS-ResourceSet shall be configured with the same value for all these SRS resources.

When higher layer parameter ULFPTxModes is set to ‘Mode 2’A UE can be configured to operate in either Mode 1 or Mode 2 upon reception of the higher layer parameter ULFPTxModes.

-            For UL transmission Mode 1:

-            the UE can be configured with one or two SRS resources with same number of SRS ports within an SRS resource set with usage set to ‘codebook’.

-            For UL transmission Mode 2:

-    the UE can be configured with one SRS resource or multiple SRS resources with same or different number of SRS ports within an SRS resource set with usage set to ‘codebook’.

-    up to 2 different spatial relations (maxNumberConfiguredSpatialRelations) can be configured for all SRS resources with usage set to ‘codebook’.

-            a maximum of 4 SRS resources are supported to in an SRS resource set with usage set to ‘codebook’

<unrelated part omitted>

————————————— end of TP —————————————————

Agreement

The following text proposal for TS38.214 subclause 6.1.1.1 is endorsed for the alignment CR on TS38.214

————————————— start of TP —————————————————

6.1.1.1	Codebook based UL transmission

<unrelated part omitted>

For UL transmission Mode 2:

- the UE can be configured with one SRS resource or multiple SRS resources with same or different number of SRS ports within an SRS resource set with usage set to ‘codebook’.

- up to 2 different spatial relations (maxNumberConfiguredSpatialRelations) can be configured for all SRS resources with usage set to ‘codebook’.

- subject to UE capability, a maximum of 2 or 4 SRS resources are supported to an SRS resource set with usage set to ‘codebook’

<unrelated part omitted>

————————————— end of TP —————————————————

R1-2001296	Editorial corrections (NR-MIMO) to 38.214	vivo

R1-2000241	Maintenance of full power UL transmission	ZTE

R1-2000335	Discussion on remaining issues on ULFP and text proposals	vivo

R1-2000459	Text proposals for full TX power UL transmission	OPPO

R1-2000536	Remaining issues on UL full power transmission	CATT

R1-2000635	Correction on UL full power transmission	Samsung

R1-2000682	Discussion on full Tx power UL transmission	LG Electronics

R1-2000741	Correction to Full Tx Power UL Transmission	Intel Corporation

R1-2000756	Discussion on full TX power UL transmission	CMCC

R1-2000818	Corrections for Full Power UL Transmission	Ericsson

R1-2000862	Remaining issues for UL full power transmission enhancement	Apple

R1-2000927	Remaining issues on full Tx power UL transmission	NTT DOCOMO, INC.

R1-2000976	Remaining issues on full power UL transmission	Qualcomm Incorporated

R1-2000990	Maintenance of Rel-16 Full TX Power UL transmission	Nokia, Nokia Shanghai Bell

R1-2001041	Remaining issues on UL full power transmission in R16	Huawei, HiSilicon

#### Low PAPR RS

R1-2000780	Feature lead summary of low PAPR RS	Ericsson

R1-2001158	Summary of critical issues for low PAPR RS maintenance	Ericsson

[100e-NR-eMIMO-lowPAPR-01] – Mattias (Ericsson)

Email discussion/approval on a TP addressing the following:

- If pi/2-BPSK PUSCH is scheduled without n_SCID bit in the DCI format, then n_SCID=0. This description is missing from 38.211, section 6.4.1.1.1.2

by 2/27

R1-2001238	Output of email thread [100e-NR-eMIMO-lowPAPR-01] - TP for TS 38.211	Ericsson

Agreement

The text proposal in R1-2001238 is endorsed as part of the alignment CR for TS38.211.

[100e-NR-eMIMO-lowPAPR-02] – Mattias (Ericsson)

Email discussion/approval on a TP capturing

- the restriction that the low PAPR RS for pi/2 BPSK PUSCH does not apply when scheduled in CSS  is missing

- In TS 38.212, the low PAPR RS for pi/2 BPSK may be configured but the new tables should not be used if not pi/2 BPSK is not scheduled

- For PDSCH DMRS for CP-OFDM, λ is not introduced as the new variable as in the PUSCH case and fallback to Rel-15 DMRS is not possible.

by 2/27

R1-2001239	Output of email thread [100e-NR-eMIMO-lowPAPR-02] - TP#1 to 38.211	Ericsson

R1-2001240	Output of email thread [100e-NR-eMIMO-lowPAPR-02] - TP#2 to 38.212	Ericsson

R1-2001241	Output of email thread [100e-NR-eMIMO-lowPAPR-02] - TP#3 to 38.211	Ericsson

Agreement

The following text proposals are approved.

- Text proposal in R1-2001239 as part of alignment CR for TS38.211

- Text proposal in R1-2001240 as part of alignment CR for TS38.212

- Text proposal in R1-2001241 as part of alignment CR for TS38.211

[100e-NR-eMIMO-lowPAPR-03]   – Mattias (Ericsson)

Email discussion/approval to clarify the following:

Whether nIDPUSCH=NIDnSCID nIDPUSCH=NIDnSCID(as in current spec 6.4.1.1.2 of 38.211) or nIDRS=NIDnSCID  is the correct interpretatation of the agreement for pi/2-BSPK PUSCH.

by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001242	Output of email thread [100e-NR-eMIMO-lowPAPR-03] - TP for TS 38.211	Ericsson

Agreement

The text proposal in R1-2001242 is endorsed as part of the alignment CR for TS38.211.

[100e-NR-eMIMO-lowPAPR-04]   – Mattias (Ericsson)

Email discussion/approval to clarify the following:

- For low PAPR pi/2-BPSK PUCCH, format 3 and 4, whether and if so how the DMRS depends on the cyclic shift value α

by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: Consensus was not reached during the email discussion. Will be further discussed in future meetings.

R1-2000242	Maintenance of low PAPR RS	ZTE

R1-2000460	Text proposals for low PAPR RS	OPPO

R1-2000757	Maintenance for low PAPR RS	CMCC

R1-2000768	Remaining issues on low PAPR RS	NEC

R1-2000774	Enhanced inter UE Tx prioritization/multiplexing	ETRI

R1-2000945	Corrections in low PAPR RS for pi/2 BPSK PUSCH	Indian Institute of Tech (H)

R1-2000991	Corrections on Low PAPR RS	Nokia, Nokia Shanghai Bell

R1-2001042	Remaining issues on Low PAPR DMRS sequence in R16	Huawei, HiSilicon

R1-2001076	Remaining issue on low PAPR RS	Ericsson

#### Other

R1-2000336	Discussion on Rel-16 MIMO UE features	vivo

R1-2000427	Real life MIMO issue from observations in field	Ericsson

R1-2000461	Remaining issues on multi-TRP and panel Transmission	OPPO

R1-2000636	Issues on utilizing CSI-IM for interference measurement	Samsung

R1-2000683	Clarification of the maximum number of CSI-RS resources in resource setting	LG Electronics

R1-2001103	Remaining issues on R16 MIMO	Huawei, HiSilicon

### Maintenance of UE Power Saving for NR

#### PDCCH-based power saving signal/channel

R1-2001137	Summary of PDCCH-based Power Saving Signal/Channel	CATT

R1-2001190	Summary of preparation phase discussion on AI-7.2.7.1 PDCCH-based power saving signal/channel	CATT

R1-2001248	Summary of Phase 1 Discussion on PDCCH-based Power Saving Signal/Channel	CATT

[100e-NR-UE_pow_sav-WUS-01] – Fangchen (CATT)

E-mail discussion/approval regarding the P-CSI/L1-RSRP aspects which includes both Section 1.1 “P-CSI/L1-RSRP measurements/report with wakeup indication by DCI format 2_6” and Section 1.7 on RRC parameters in R1-2001137 by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: As per email decision posted on mar. 5th,

Agreements:

- P-CSI and L1-RSRP reports are independently configured and to allow UE only to report periodic CSI apart from L1-RSRP.

Corresponding TP to TS 38.214 (subclauses 5.1.6.1, 5.2.2.5) in R1-2001248 is endorsed.

[100e-NR-UE_pow_sav-WUS-02] – Fangchen (CATT)

E-mail discussion/approval regarding the Search Space Configuration for DCI format 2_6 monitoring which includes Section 1.3 PS_offset range and resolutions, Section 1.4 Minimum Time Gap values, and Section 1.6 DCI format 2_6 not counting as DCI size budget in R1-2001137 by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: As per email decision posted on mar. 4th,

Agreements:

- PS_offset range from {0.125ms to 15 ms} for all SCS.

- The PS_offset resolution is 0.125 ms.

Decision: As per email decision posted on mar. 5h,

Agreements:

- Candidate values for the minimum time gap are specified by RAN1 and shared with RAN4

  - Minimum time gap is no more than 3 ms for all SCSs

  - Two values of minimum time gap for each SCS are proposed as

    - 15kHz: {TBD, TBD} slots

    - 30kHz {TBD,  TBD} slots

    - 60kHz {TBD, TBD} slots

    - 120kHz {TBD, TBD} slots

Following updated TP (section 10.3 of TS 38.213) of the UE capability of minimum time gap X is endorsed.

------------------------------------------------ Begin of TP -----------------------------------------------------

- If a UE reports for an active DL BWP a requirement of X slots prior to the beginning of a slot where the UE would start the drx-onDurationTimer, the UE is not required to monitor PDCCH for detection of DCI format 2_6 during the X slots, where X corresponds to the requirement of the SCS of the active DL BWP.

------------------------------------------------ End of TP -----------------------------------------------------

[100e-NR-UE_pow_sav-WUS-03] – Fangchen (CATT)

E-mail discussion/approval regarding the monitoring occasion of DCI format 2_6, which include Sections 1.2.1 and 1.2.2 in R1-2001137 by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: As per email decision posted on mar. 5h,

Agreements:

RAN1 agreed to have additional invalid monitoring occasions of DCI format 2_6 colliding with BWP switching time and measurement gap with the following text proposal (section 10.3 of TS38.213)

------------------------------------------------ Begin of TP -----------------------------------------------------

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 2_6 in the active DL BWP of the PCell or of the SpCell and the UE

- is not required to monitor PDCCH for detection of DCI format 2_6, as described in Clauses 10,and 11.1, 12, and Clause 5.7 of  [14, TS 38.321]for all corresponding PDCCH monitoring occasions outside Active Time prior to a next DRX cycle, or

- does not have any PDCCH monitoring occasions for detection of DCI format 2_6 outside Active Time of a next DRX cycle

the UE shall start the bydrx-onDurationTimer for the next DRX cycle.

------------------------------------------------ End of TP -----------------------------------------------------

Conclusion:

No consensus on the change of “all” to “one or more” invalid monitoring occasions in 10.3 of TS38.213

R1-2000210	Corrections on PDCCH based power saving	Huawei, HiSilicon

R1-2000337	Maintenance of PDCCH-based power saving signal	vivo

R1-2000417	Clarification on power saving signal	Spreadtrum Communications

R1-2000443	Remaining issues on PDCCH-based power saving signal	MediaTek Inc.

R1-2000478	Remaining issues for power saving signal	OPPO

R1-2000511	Remaining issues on WUS PDCCH	ZTE

R1-2000537	Remaining issues on the Power Saving Signals/Channels	CATT

R1-2000637	Remaining issues for PDCCH-based power saving signal/channel	Samsung

R1-2000684	Remaining issues on PDCCH-based power saving signal/channel	LG Electronics

R1-2000742	Remaining details of PDCCH-based power saving signal/channel	Intel Corporation

R1-2000758	Discussion on DCI format 2_6 monitoring	CMCC

R1-2000817	Remaining Issues for PDCCH-based Power Saving Signal/Channel 	InterDigital, Inc.

R1-2000866	Work split between 38.213 and 38.321 on DRX operation with PS-RNTI configuration	NEC

R1-2000884	Remaining issues for WUS	Ericsson

R1-2000928	Maintenance for PDCCH-based power saving signal/channel	NTT DOCOMO, INC.

R1-2001048	On open issues related to DCI format 2_6	Nokia, Nokia Shanghai Bell

#### Procedure of cross-slot scheduling power saving techniques

R1-2001141	Summary#1 for cross-slot scheduling power saving techniques	MediaTek

R1-2001211	Summary#2 for cross-slot scheduling power saving techniques	MediaTek

[100e-NR-UE_pow_sav-Cross_Slot-01] – Weide (MTK)

E-mail discussion/approval regarding the proposals:

- For cross-BWP scheduling, the 1-bit indication, if available, is used to indicate the minimum scheduling offset in the target BWP

by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Feb. 28th

Conclusion:

- If a DCI format 1_1 (or 0_1) indicates a target DL (or UL) BWP different from the active DL (or UL) BWP, the minimum applicable scheduling offset indicator field (if present in the DCI format) indicates the minimum scheduling offset restriction to be applied in the target BWP.

  - Note: No specification change needed

[100e-NR-UE_pow_sav-Cross_Slot-02] – Weide (MTK)

E-mail discussion/approval regarding the factors to be considered for the scheduling offet in cross-BWP schedulingby 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Mar. 3rd, no conclusion. Discuss further. See summary in R1-2001211.

[100e-NR-UE_pow_sav-Cross_Slot-03] – Weide (MTK)

E-mail discussion/approval regarding the proposals:

- UE does not expect to receive another indication of minimum scheduling offset change in a scheduling DCI for the same active BWP before the slot for which a previous indication of minimum scheduling offset change is expected to be applied

by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Mar. 3rd,

Agreements:

UE does not expect to receive another change in the ‘Minimum applicable scheduling offset indicator’ field of DCI format 0_1 or 1_1 for the same active BWP before the slot for which a previous change of applied minimum scheduling offset restriction is expected to take effect

Decision: As per email decision posted on Mar. 4th, the TP (to 1st paragraph of Section 5.3.1 of TS 38.214) is endorsed. See summary in R1-2001211.

R1-2000180	Corrections on cross-slot scheduling based power saving	Huawei, HiSilicon

R1-2000338	Maintenance of procedure of cross-slot scheduling power saving techniques	vivo

R1-2000444	Remaining issues on cross-slot scheduling adaptation	MediaTek Inc.

R1-2000455	Clarification on cross-slot scheduling	Spreadtrum Communications

R1-2000512	Remaining issues on cross-slot scheduling power saving techniques	ZTE

R1-2000538	Remaining issues on Power saving scheme with cross-slot scheduling	CATT

R1-2000638	Remaining issues for corss-slot scheduling power saving techniques	Samsung

R1-2000685	Remaining issues on procedure of cross-slot scheduling power saving techniques	LG Electronics

R1-2000715	Remaining issues for cross-slot scheduling	OPPO

R1-2000743	Remaining details of cross-slot scheduling for power saving	Intel Corporation

R1-2000759	Corrections on cross-slot scheduling procedure	CMCC

R1-2000863	Remaining Issues on Cross Slot Scheduling Technique for UE Power Saving	Apple

R1-2000885	Remaining issues for cross-slot scheduling	Ericsson

R1-2000929	Maintenance for procedure of cross-slot scheduling power saving techniques	NTT DOCOMO, INC.

R1-2000977	Remaining issues in cross-slot scheduling power saving	Qualcomm Incorporated

R1-2001049	Procedure of cross-slot scheduling power saving techniques	Nokia, Nokia Shanghai Bell

#### UE adaptation to maximum number of MIMO layers

R1-2000803	Summary of UE adaptation to maximum number of MIMO layers	vivo

R1-2001189	Summary#2 of UE adaptation to maximum number of MIMO layers	vivo

R1-2000211	Corrections on UE adaptation to maximum MIMO layers	Huawei, HiSilicon

R1-2000339	Maintenance of UE adaptation to maximum number of MIMO layers	vivo

R1-2000886	Maintenance issues for MIMO layer signaling per BWP	Ericsson

R1-2001050	On open issues for per-BWP DL MIMO layers	Nokia, Nokia Shanghai Bell

#### Other

R1-2000340	Other issues for UE power saving	vivo

R1-2000513	Views on power saving enhancement	ZTE

R1-2000887	Discussion on UE features for Rel-16 power savings	Ericsson

R1-2001104	Other Remaining issues for Rel-16 Power saving	Huawei, HiSilicon

### Maintenance of NR positioning support

#### DL Reference Signals for NR Positioning

R1-2001142	Feature lead summary for AI 7.2.8.1 – DL Reference Signals for NR Positioning	Intel

R1-2001176	Feature lead summary#2 for AI 7.2.8.1 – DL Reference Signals for NR Positioning	Intel

[100e-NR-Pos-DL-PRS-01] – Alexey (Intel)

E-mail discussion/approval to resolve issue #1 in R1-2001176 by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001233	Output of email thread [100e-NR-Pos-DL-PRS-01]	Intel

Decision: As per email decision posted on Feb. 28th,

Agreement:

Proposal 1 in R1-2001233 is agreed.

R1-2001234	Text proposal to TS 38.211 based on outcome of [100e-NR-Pos-DL-PRS-01]	Intel

Agreement:

Adopt the following text proposal for TS38.211:

------------------------------------------------ Begin of TP -----------------------------------------------------

7.4.1.7.3	 Mapping to physical resources in a downlink PRS resource

---------------------------------------Text omitted ---------------------------------------

and where

- the antenna port p=5000;

- lstartPRS is the first symbol of the downlink PRS within a slot and given by the higher-layer parameter DL-PRS-ResourceSymbolOffset;

- the size of the downlink PRS resource in the time domain LPRS∈2,4,6,12 is given by the higher-layer parameter DL-PRS-NumSymbols;

- the comb size KcombPRS∈2, 4, 6,12 is given by the higher-layer parameter transmissionComb;

- the resource-element offset koffsetPRS∈0,1,…,KcombPRS-1 is given by the higher-layer parameter combOffset;

- the quantity k' is given by Table 7.4.1.7.3-1.

---------------------------------------Text omitted ---------------------------------------

------------------------------------------------ End of TP -----------------------------------------------------

[100e-NR-Pos-DL-PRS-02] – Alexey (Intel)

E-mail discussion/approval to resolve issue #2 in R1-2001176 by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001235	Output of email thread [100e-NR-Pos-DL-PRS-02]	Intel

Agreement:

- UE DL PRS processing capability is reported for maximum DL PRS bandwidth in MHz, which is supported and reported by UE

  - UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value

  - FFS values of maximum DL PRS bandwidth in MHz for UE DL PRS processing capability report

- FFS if UE DL PRS processing capability is scaled inversely proportional to DL PRS processing bandwidth

- Note: this overrides the 272 RB assumption in the previous RAN1 agreement.

Agreement:

- UE DL PRS processing capability is signaled per band

- FFS if UE DL PRS processing capability is agnostic to the configured SCS settings of DL PRS

- FFS if reported values of T are the same across bands within a FR or across FRs

Agreement:

- UE DL PRS processing capability is defined for a single positioning frequency layer

Agreement:

- UE DL PRS processing capability is agnostic to DL PRS comb factor configuration

R1-2000190	Maintenance of DL PRS for NR positioning	Huawei, HiSilicon

R1-2000243	Maintenance on DL reference signals for NR positioning	ZTE

R1-2000341	Discussion on remaining issues on DL RS for NR positioning	vivo

R1-2000462	Remaining Issues on DL Positioning Reference Signal	OPPO

R1-2000539	Remaining issues on DL PRS for NR Positioning	CATT

R1-2000575	TP on DL Reference Signals	Futurewei

R1-2000639	DL reference signals for NR Positioning	Samsung

R1-2000686	Remaining issues of DL reference signals for NR Positioning	LG Electronics

R1-2000760	Corrections on DL PRS resource set parameters	CMCC

R1-2000804	Correction of DL PRS sequence generation	Nokia, Nokia Shanghai Bell

R1-2001002	Maintenance of rel16 DL reference signals for NR positioning	Ericsson

#### UL Reference Signals for NR Positioning

R1-2001128	FL Summary for UL Reference Signals for NR Positioning	Ericsson

[100e-NR-Pos-ULRS-01] – Florent (Ericsson)

E-mail discussion/approval to resolve the issue of simultaneous SRS transmission (proposal: UE is not expected to transmit SRS for positioning and SRS for other usages on the same symbol.) by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001286	Outcome of email thread [100e-NR-Pos-ULRS-01]	Ericsson

Decision: As per email decision posted on Mar. 3rd,

Agreement:

The TP for TS38.214 in Section 3 of R1-2001286 is agreed.

[100e-NR-Pos-ULRS-02] – Florent (Ericsson)

E-mail discussion/approval to resolve SRS priorities/collision with PUSCH by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001287	Outcome of email thread [100e-NR-Pos-ULRS-02]	Ericsson

Decision: As per email decision posted on Feb. 28th,

Agreement:

The text proposal in Section 3.1 of R1-2001287 for TS38.214 is agreed.

[100e-NR-Pos-ULRS-03] – Florent (Ericsson)

E-mail discussion/approval to resolve the issue of periodic SRS for positioning configuration (proposal: Add the parameter slot offset for an SRS resource when the higher layer parameter resourceType in SRS-Resource is set to 'aperiodic'.) by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001288	Outcome of email thread [100e-NR-Pos-ULRS-03]	Ericsson

Decision: As per email decision posted on Feb. 29th,

Agreement:

Add the parameter slot offset for an SRS resource when SRS is configured with the higher layer parameter [SRS-for-positioning] and when the higher layer parameter resourceType in SRS-Resource is set to 'aperiodic'.

- Send an LS to RAN2 to inform of the update of higher layer parameters

LS to RAN2 approved in R1-2001386

MCC: Approved LS is revised in R1-2001483 (accept all change marks and source as RAN1 only).

Agreement:

Endorse the text proposal for TS38.214 in section 3.1 of the summary of the email discussion output in R1-2001288.

[100e-NR-Pos-ULRS-04] – Florent (Ericsson)

E-mail discussion/approval to resolve the issue of spatial relation information for SRS for positioning (default, MAC CE update) by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001289	Outcome of email thread [100e-NR-Pos-ULRS-04]	Ericsson

Decision: As per email decision posted on Mar. 3rd,

Agreement:

- The feature of default spatial relation info is not applicable to SRS for positioning

- The feature of using one MAC CE to activate one same spatial relation info for multiple SRS resources in multiple CCs is not applicable to SRS for positioning.

- The text proposal for TS38.214 in Section 3.1 in R1-2001289 is endorsed.

R1-2000191	Maintenance of SRS for NR positioning	Huawei, HiSilicon

R1-2000342	Discussion on remaining issues on  UL RS for NR positioning	vivo

R1-2000463	Remaining Issues on UL Positioning Reference Signal	OPPO

R1-2000540	Remaining issues on UL SRS for NR Positioning	CATT

R1-2000640	UL reference signals for NR Positioning	Samsung

R1-2000687	Remaining issues of UL reference signals for NR Positioning	LG Electronics

R1-2000761	Discussions on UL SRS for positioning collision rules	CMCC

R1-2001003	Maintenance of rel16 UL reference signals for NR positioning	Ericsson

#### UE and gNB measurements for NR Positioning

R1-2000932	FL Summary of Remaining issues on NR Positioning Measurements	CATT

R1-2001156	FL Summary#2 of Remaining issues on NR Positioning Measurements	CATT

[100e-NR-Pos-Measurements-01] –  Ren (CATT)

Email approval of a TP on essential corrections to UE/gNB measurements (TS 38.215, section 5.1.6.5) (reference: R1-2001156, issues 2.1/2.2/2.5/2.6/2.7/2.10. Note: some part of 2.1/2.10 are to be discussed under [100e-NR-Pos-PHY procedures-01], to be coordinated by FLs) by 2/28

R1-2001243	Output of email thread [100e-NR-Pos-Measurements-01]	CATT

R1-2001244	Text proposal to TS 38.215 based on outcome of [100e-NR-Pos-Measurements-01]	CATT

Decision: As per email decision posted on Feb. 29th,

Agreement:

Adopt the text proposals for TS38.215 in R1-2001244.

R1-2000192	Maintenance of NR positioning measurements	Huawei, HiSilicon

R1-2000343	Discussion on remaining issues on UE and gNB measurements for NR positioning	vivo

R1-2000464	Remaining Issues on Measurements for NR Positioning	OPPO

R1-2000541	Remaining issues on NR Positioning Measurements	CATT

R1-2000577	TP on Measurements	Futurewei

R1-2000641	UE and gNB measurements for NR Positioning	Samsung

R1-2000688	Remaining issues of UE and gNB measurements for NR Positioning	LG Electronics

R1-2000805	Maintainence of Measurement Definitions	Nokia, Nokia Shanghai Bell

R1-2000978	Maintenace on UE and gNB measurements for NR Positioning	Qualcomm Incorporated

R1-2001004	Maintenance of rel16 Physical-layer procedures to support UE - gNB measurements	Ericsson

#### Physical-layer procedures to support UE/gNB measurements

R1-2001143	Summary of remaining issues on PHY procedures for NR positioning measurements	Qualcomm

R1-2001168	Summary#2 of remaining issues on PHY procedures for NR positioning measurements	Qualcomm

[100e-NR-Pos-PHY procedures-01] – Sven (Qualcomm)

Email approval of a TP on essential corrections to PRS reception procedure (TS 38.214, sub-clause 5.1.6.5) (reference: R1-2001168, issues #1, 2, 4-11; R1-2001176, issue #3; R1-2001156, part of the issues 2.1 and issue 2.10 related to Section 5.1.6.5, 38.214) by 3/1

R1-2001364	Summary of Email Discussion [100e-NR-Pos-PHY procedures-01]: PRS reception procedure	Qualcomm Incorporated

Decision: As per email decision posted on Mar. 4th,

Agreement:

The TP for TS 38.214, clause 5.1.6.5, in Section 3 of R1-2001364 is agreed.

[100e-NR-Pos-PHY procedures-02] – Sven (Qualcomm)

Email discussion/approval on UE procedure for determining slot format (TS 38.213, TS 38.214) (reference: R1-2001168, issue #12) by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001365	Summary of Email Discussion [100e-NR-Pos-PHY procedures-02]: UE procedure for determining slot format	Qualcomm Incorporated

Decision: As per email decision posted on Mar. 4th,

Agreement:

Proposals 1-7 in Section 3 of R1-2001365 including text proposals for TS38.213 and TS38.214 are agreed

[100e-NR-Pos-PHY procedures-03] – Sven (Qualcomm)

Email discussion/approval of a TP on essential corrections to UL power control for SRS-for-positioning (TS 38.213, sub-clause 7.3.1) (reference: R1-2001168, issues#15/16/17) by 2/28

R1-2001366	Summary of Email Discussion [100e-NR-Pos-PHY procedures-03]: UE procedures for SRS for positioning	Qualcomm Incorporated

Decision: As per email decision posted on Mar. 4th,

Agreement:

The text proposal for TS 38.213, clause 7.3.1, in Section 3 of R1-2001366 is agreed.

R1-2000193	Maintenance of physical layer procedures to support positioning measurements	Huawei, HiSilicon

R1-2000244	Maintenance on physical layer procedures for NR positioning	ZTE

R1-2000344	Discussion on remaining issues on on physical-layer procedures for NR positioning	vivo

R1-2000465	Remaining Issues on Physical Layer Procedures for NR Positioning	OPPO

R1-2000578	TP on Measurement Procedures	Futurewei

R1-2000642	Physical-layer procedures to support UE/gNB measurements	Samsung

R1-2000689	Remaining issues of Physical-layer preocedures to support UE/gNB measurements	LG Electronics

R1-2000762	Discussions on TA adjustment on timing measurements	CMCC

R1-2001005	Maintenance of rel16 UE and gNB measurements for NR Positioning	Ericsson

R1-2001077	Remaining opens on NR Positioning physical layer procedures	Intel Corporation

#### Other

R1-2000194	RAN procedures for NR positioning	Huawei, HiSilicon

R1-2000345	Discussion on parameter assumptions and evaluations for NR positioning enhancements	vivo

R1-2000466	Discussion on RE mapping in reference signal for NR positioning	OPPO

### Maintenance of NR Mobility Enhancements

R1-2000946	Issue Summary for NR Mobility Enhancements	Intel Corporation

R1-2001200	Issue Summary#2 for NR Mobility Enhancements	Intel Corporation

#### Physical Layer Aspects for Mobility Enhancements during HO and SCG Change

R1-2001246	Email discussion summary for NR Mobility Enhancements	Intel Corporation

[100e-NR-Mob-Enh-01] – Daewon (Intel)

E-mail discussion/approval to resolve the issue of PRACH and PUSCH/PUCCH/SRS overlap in time domain for single cell or intra-frequency CA operation by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: As per email decision posted on Feb. 29th,

Agreement:

When PRACH in target cell and PUSCH/PUCCH/SRS in source cell overlap (within a gap), prioritize the target cell transmission (drop PUSCH/PUCCH/SRS in source).

- Use part of TP from Nokia’s Tdoc R1-2001051 as basis for final TP.

- The following text proposal for TS 38.213 is agreed.

15 Dual active protocol stack based handover < Unchanged parts are omitted > For DAPS operation in a same frequency band, a UE does not transmit PUSCH/PUCCH/SRS to source MCG in a same [source MCG] slot overlapping in time domain with PRACH transmission to target MCG or when a gap between the first or last symbol of a PRACH transmission to target MCG in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission to source MCG in a second slot. N = 2 for µ=0 or µ=1, N=4 for µ=2 or µ=3, and µ is the SCS configuration of the active UL BWP for PUSCH/PUCCH/SRS transmission to the source MCG.

[100e-NR-Mob-Enh-02] – Daewon (Intel)

E-mail discussion/approval to resolve the misalignment between RAN1 agreement and specification text for uplink power control for DAPS-HO (including possible RRC parameter update) by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: According to email decision posted on Feb. 28th, the following is agreed:

Agreement: Include the following parameters to RRC list

- p-DAPS-FR1

- p-DAPS-FR2

- UplinkPowerSharingDAPS-HO-mode

Agreement: The following text proposal is agreed for 38.213.

15 Dual active protocol stack based handover < Unchanged parts are omitted > If a UE is configured with an target MCG and a source MSCG using NR radio access in FR1 and/or in FR2, the UE is configured a maximum power PMCG for transmissions on the target MCG by p-DAPS-FR1 and/or by p-DAPS-FR2 and a maximum power PSCG for transmissions on the source SMCG by p-DAPS-FR1 and/or by p-DAPS-FR2 and with an inter-CG power sharing mode by UplinkPowerSharingDAPS-HO-mode for FR1 and/or by UplinkPowerSharingDAPS-HO-mode for FR2. The UE determines a transmission power on the target MCG and a transmission power on the source MSCG per frequency range. < Unchanged parts are omitted >

[100e-NR-Mob-Enh-03] – Daewon (Intel)

E-mail discussion/approval to resolve uplink transmission prioritization and dropping/cancellation rules by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

Decision: No consensus in the email discussion.

R1-2000206	Remaining issues on DAPS-HO	Huawei, HiSilicon

R1-2000366	Remaining issues on NR mobility enhancements in physical layer	ZTE

R1-2000442	On DAPS HO for mobility enhancement	MediaTek Inc.

R1-2000643	Remaining issues for NR Mobility Enhancement	Samsung

R1-2000864	On remaining issues on NR mobility enhancements	Apple

R1-2000979	Maintenance for NR mobility enhancements	Qualcomm Incorporated

R1-2001051	Remaining physical layer aspects of dual active protocol stack based HO	Nokia, Nokia Shanghai Bell

R1-2001080	Correction to UL power sharing for DAPS HO	Ericsson

#### Other

R1-2000207	Remaining PHY aspects for CHO	Huawei, HiSilicon

### Maintenance of Multi-RAT Dual-Connectivity and Carrier Aggregation enhancements (LTE, NR)

#### Uplink Power Control for Supporting NR-NR Dual-Connectivity

R1-2000846	FL summary on UL Power Control for NN-DC	Apple

R1-2001208	FL summary#2 on UL Power Control for NN-DC	Apple

[100e-NR-LTE_NR_DC_CA_enh-ULPC-01] – Hong (Apple)

E-mail discussion/approval on the issue 1 in R1-2000846 by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001301	Outcome of email thread [100e-NR-LTE_NR_DC_CA_enh-ULPC-01]	Apple

Decision: As per email decision posted on Feb. 29th,

Agreement:

- Dynamic power sharing can be configured for both synchronous and asynchronous NR-DC scenarios subject to corresponding UE capability of dynamic power sharing

- A UE can report support of dynamic power sharing operation for synchronous NR-DC and/or asynchronous NR-DC. Details are up to UE feature discussion.

Decision: As per email decision posted on Mar. 3rd, no TP is needed.

[100e-NR-LTE_NR_DC_CA_enh-ULPC-02] – Hong (Apple)

E-mail discussion/approval on the issue 2 and issue 4 in R1-2000846 by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001302	Outcome of email thread [100e-NR-LTE_NR_DC_CA_enh-ULPC-02]	Apple

Decision: As per email decision posted on Mar. 6th, based on the discussions and majority views working assumption as in R1-2001302 was proposed.

R1-2001421	LS on uplink power control for NR-NR Dual-Connectivity	RAN1, Apple

Decision: LS to RAN2 is approved.

MediaTek: since this LS is asking about the communication capability between MCG and SCG, cc RAN3 maybe consider

Apple: not really necessary to cc RAN3 since inter-gNB coordination message for dual connectivity is a bit different with normal X2 signaling. It is coordination between two special gNBs that connected with a single UE, which is fully handled by RAN2 scope and named it “RRC container”.

[100e-NR-LTE_NR_DC_CA_enh-ULPC-03] – Hong (Apple)

E-mail discussion/approval on the issue 5 and issue 6 in R1-2000846 by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001303	Outcome of email thread [100e-NR-LTE_NR_DC_CA_enh-ULPC-03]	Apple

Decision: As per email decision posted on Feb. 29th, the two TPs of R1-2001303 are endorsed.

R1-2000346	Remaining issues on uplink power control for NR-NR DC	vivo

R1-2000361	Remaining Issues on NR-DC Power Control	ZTE

R1-2000440	On UL Power Control for NN-DC	MediaTek Inc.

R1-2000561	Text proposals for UL Power Sharing for NR-DC	OPPO

R1-2000644	Power Control for NR-DC	Samsung

R1-2000721	Remaining issues on uplink power control for NN-DC	Intel Corporation

R1-2000845	On remaining issues of UL Power Control for NN-DC	Apple

R1-2000888	Remaining issues for NR-DC UL Power Control	Ericsson

R1-2000980	Remaining issues on uplink power control for NR-DC	Qualcomm Incorporated

R1-2001039	UL power control for NR-NR dual connectivity	Huawei, HiSilicon

R1-2001067	Maintenance of rel-16 DC power control	Nokia, Nokia Shanghai Bell

#### Potential Enhancements to Single Tx Switched Uplink Solution for EN-DC

R1-2000848	FL summary of remaining issues on enhancements for single Tx transmission in EN-DC	Apple

[100e-NR-LTE_NR_DC_CA_enh-SingleTx-01] – Wei (Apple)

E-mail discussion/approval on the issue 1 in R1-2000848 by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001391	Outcome of email thread [100e-NR-LTE_NR_DC_CA_enh-SingleTx-01]	Apple

Decision: As per email decision posted on Feb. 28th, following conclusion is made:

- no need to introduce the R16 RRC parameter “Semi-static-LTE-UL-in-all-subframes”.

To be reflected in an update of the RRC parameters list to RAN2, no additional spec impact.

[100e-NR-LTE_NR_DC_CA_enh-SingleTx-02] – Wei (Apple)

E-mail discussion/approval on the TP as of issue 2 in R1-2000848 by 2/27

R1-2001392	Outcome of email thread [100e-NR-LTE_NR_DC_CA_enh-SingleTx-02]	Apple

Decision: As per email decision posted on Mar. 4th, following conclusion applies:

- Conclude that TP is needed to resolve the issue (as summarized in R1-2001392). Further decide on the wording of TP (strive to down select between option 1 and option 2 if possible).

[100e-NR-LTE_NR_DC_CA_enh-SingleTx-03] – Wei (Apple)

E-mail discussion/approval on the TP as of issue 3 in R1-2000848 by 2/27

R1-2001393	Outcome of email thread [100e-NR-LTE_NR_DC_CA_enh-SingleTx-03]	Apple

Decision: As per email decision posted on Feb. 28th,

Agreements:

- Agree the TP on section 10 of TS36.213.

- For text capturing RAN1 agreement that UL/DL configuration 0/6, wait for RAN2 CRs, then decide whether introduce the change in RAN1 spec. (This part of removed from the TP proposal below)

- For text capturing RAN1 agreement on Scell HARQ timing in EN-DC with TDD PCell, the TP on section 7.3 and section 10 of TS 36.213 are agreed.

- Adopt the TP on section 7.3.2.1 and section 7.3.2.2 of TS36.213.

Decision: As per email decision posted on Mar. 3rd, no consensus to clarify type 2 UE’s assumption on “other than offset-UL subframes”.

R1-2000362	Remaining Issues on Single Tx Switched Uplink Solution for EN-DC	ZTE

R1-2000690	Remaining issue on single TX-switched UL for EN-DC	LG Electronics

R1-2000847	On remaining issues on single Tx transmission EN-DC	Apple

R1-2000889	Remaining issues for single Tx UL enhancements	Ericsson

R1-2000981	Remaining issues for EN-DC Single-Tx TDM Operation	Qualcomm Incorporated

R1-2001040	Enhancements for single UL operation for EN-DC	Huawei, HiSilicon

#### Support of efficient and low latency serving cell configuration/activation/setup

R1-2000891	Summary of efficient and low latency serving cell configuration/activation/setup	Ericsson

R1-2001192	Summary#2 of efficient and low latency serving cell configuration/activation/setup	Ericsson

[100e-NR-LTE_NR_DC_CA_enh-ScellDormancy-01] – Ravi (Ericsson)

E-mail discussion/approval regarding issue a in R1-2001192 by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001416	Summary of email discussion [100e-NR-LTE_NR_DC_CA_enh-ScellDormancy-01]	Ericsson

Decision: As per email decision posted on Mar. 6th,

R1-2001419	Text proposals from email discussion [100e-NR-LTE_NR_DC_CA_enh-ScellDormancy-01]	Ericsson

Agreements:

- Adopt TP1 in R1-2001419

- Adopt TP2 and TP3 in R1-2001419 with below note

  - Note: the set of TPs may not yet fully capture the support of HARQ-ACK feedback with Case 2 Scell dormancy indication in TYPE2 HARQ-ACK CB

- Adopt TP4 in R1-2001419

[100e-NR-LTE_NR_DC_CA_enh-ScellDormancy-02] – Ravi (Ericsson)

E-mail discussion/approval regarding issue b in R1-2001192 by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2 – Ravi

R1-2001417	Summary of email discussion [100e-NR-LTE_NR_DC_CA_enh-ScellDormancy-02]	Ericsson

Decision: As per email decision posted on Feb. 28th,

Agreements:

- When UE is configured with CIF, ‘DCI format 0-1/1-1 on primary cell with CIF≠0’ is not used for Case 1 Scell dormancy indication.

- Case 2 dormancy indication using DCI format 1-1 is not supported for the case when DCI CRC is scrambled by CS-RNTI

R1-2001420	Text proposals from email discussion [100e-NR-LTE_NR_DC_CA_enh-ScellDormancy-02]	Ericsson

Decision: As per email decision posted on Mar. 3rd, the TPs in R1-2001420 are endorsed.

Conclusion:

- Regarding the potential TP for the ‘SCell dormancy indication’ field description in 38.212 for DCI 0-1/1-1:

  - No spec update is necessary

The following proposal is for further discussion till next meeting:

- When UE is configured with CIF, ‘DCI format 1-1 on primary cell with CIF≠0’ is not used for Case 2 Scell dormancy indication

[100e-NR-LTE_NR_DC_CA_enh-ScellDormancy-03] – Ravi (Ericsson)

E-mail discussion/approval regarding issue c in R1-2001192 by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001418	Summary of email discussion [100e-NR-LTE_NR_DC_CA_enh-ScellDormancy-03]	Ericsson

RAN1 chair proposed conclusion (posted on Feb. 29th):

- There is no consensus in supporting dormancy indication using DCI format 0-2/1-2 for both case 1 and case 2 in Rel-16.

Nokia: there is no need for such conclusion. If spec does not have support for dormancy in 1_2 or 0_2 then it is not supported.

R1-2000208	Corrections on efficient and low latency serving cell configuration/activation/setup	Huawei, HiSilicon

R1-2000347	Remaining issues on Scell dormancy like behavior	vivo

R1-2000363	Remaining Issues on Fast SCell Activation	ZTE

R1-2000504	Remaining issues on Efficient CA design	Nokia, Nokia Shanghai Bell

R1-2000645	Remaining issues for support of efficient and low latency serving cell configuration/activation/setup	Samsung

R1-2000716	Discussion on default behavior of Scell dormancy	OPPO

R1-2000722	Remaining issues on SCell dormancy behavior	Intel Corporation

R1-2000890	Remaining issues for reduced latency Scell management for NR CA	Ericsson

R1-2000982	Remaining issues for SCell dormancy	Qualcomm Incorporated

#### Support of aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH

R1-2001145	FL summary on cross-carrier scheduling with different numerology	Nokia, Nokia Shanghai Bell

R1-2001146	FL summary update on cross-carrier scheduling with different numerology	Nokia, Nokia Shanghai Bell

[100e-NR-LTE_NR_DC_CA_enh-X-CC-A-CSI-RS-01] – Karri (Nokia)

E-mail discussion/approval regarding issue #5 in R1-2001146 by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001350	Outcome of email thread [100e-NR-LTE_NR_DC_CA_enh-X-CC-A-CSI-RS-01]	Nokia

Decision: As per email decision posted on Feb. 29th,

Agreements:

- Agree to extend the TEI16 specified on top of Rel-15 A-CSI RS triggering handling to cover also the cross-carrier A-CSI-RS triggering when the SCS of the two carriers are different.

- Adopt the TP of R1-2001081

[100e-NR-LTE_NR_DC_CA_enh-X-CC-A-CSI-RS-02] – Karri (Nokia)

E-mail discussion/approval regarding issue #1/2/3 in R1-2001146 by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001351	Outcome of email thread [100e-NR-LTE_NR_DC_CA_enh-X-CC-A-CSI-RS-02]	Nokia

Decision: As per email decision posted on Mar. 4th, the TPs in R1-2001351 are endorsed.

R1-2000348	Remaining issues on aperiodic CSI-RS triggering	vivo

R1-2001033	Corrections on aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH	Huawei, HiSilicon

R1-2001081	On the handling of beamSwitchTiming values of 224 and 336	Ericsson

#### Support of unaligned frame boundary with slot alignment and partial SFN alignment for R16 NR inter-band CA

R1-2001131	FL summary on support of unaligned frame boundary for R16 NR inter-band CA	CMCC

R1-2001154	FL summary#2 on support of unaligned frame boundary for R16 NR inter-band CA	CMCC

[100e-NR-LTE_NR_DC_CA_enh-Unaligned_CA-01] – Xiaodong (CMCC)

E-mail discussion/approval regarding issue 1 in R1-2001154 by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001265	Outcome of email thread [100e-NR-LTE_NR_DC_CA_enh-Unaligned_CA-01]	CMCC

Decision: As per email decision posted on Mar. 3rd,

Agreement

- Adopt the proposed correction for section 4.5 in TS 38.211 (R1-2001265)

[100e-NR-LTE_NR_DC_CA_enh-Unaligned_CA-02] – Xiaodong (CMCC)

E-mail discussion/approval regarding issue 2 in R1-2001154 by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001266	Outcome of email thread [100e-NR-LTE_NR_DC_CA_enh-Unaligned_CA-02]	CMCC

Decision: As per email decision posted on Feb. 29th,

Agreements:

- To define the UE behavior when higher-layer parameter CA-slot-offset is not configured, it is proposed to use if-else- condition in subclause 5.1.2.1 for PDSCH/subclause 6.1.2.1 for PUSCH, subclause 5.2.1.5.1a for aperiodic CSI-RS, or subclause 6.2.1 for SRS, when slot offset is introduced to PDSCH/PUSCH time domain resource allocation, aperiodic CSI-RS or SRS in TS38.214

- To avoid the ambiguity of the parameters, it is proposed to refining the definition of Nslot, offsetCA  and μoffset in in subclause 5.1.2.1 for PDSCH/subclause 6.1.2.1 for PUSCH, subclause 5.2.1.5.1a for aperiodic CSI-RS, or subclause 6.2.1 for SRS, when they are introduced into PDSCH/PUSCH time domain resource allocation,aperiodic CSI-RS and aperiodic SRS triggering in TS38.214

Decision: As per email decision posted on Mar. 4th, the TP for 5.1.2.1 in TS 38.214 (R1-2001266) is endorsed.

[100e-NR-LTE_NR_DC_CA_enh-Unaligned_CA-03] – Xiaodong (CMCC)

E-mail discussion/approval regarding issue 3 in R1-2001154 by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

Decision: As per email decision posted on Feb. 29th,

R1-2001267	Outcome of email thread [100e-NR-LTE_NR_DC_CA_enh-Unaligned_CA-03]	CMCC

Agreement:

- Introduction of slot offset into pseudo-code for Type-1 HARQ-ACK codebook determination

Decision: As per email decision posted on Mar. 4th, the TP for 5.1.2.1 in TS 38.214 (R1-2001267) is endorsed.

R1-2000349	Remaining issues on unaligned frame boundary for CA	vivo

R1-2000364	Remaining Issues on ASync CA	ZTE

R1-2000441	On inter-band CA with unaligned frame boundary	MediaTek Inc.

R1-2000519	Correction on HARQ-ACK codebook for inter-band CA with unaligned frame boundary	CATT

R1-2000691	Remaining issue on unaligned CA with slot alignment	LG Electronics

R1-2000763	Remaining issues on support of unaligned frame boundary with slot alignment and partial SFN alignment	CMCC

R1-2000892	Remaining issues for inter-band CA with unaligned frame boundary	Ericsson

R1-2001034	Corrections on unaligned frame boundary with slot alignment and partial SFN alignment for R16 NR inter-band CA	Huawei, HiSilicon

#### Support of cross-carrier scheduling with different numerology

R1-2001068	FL summary on cross-carrier scheduling with different numerology	Nokia, Nokia Shanghai Bell

R1-2001069	FL summary update on cross-carrier scheduling with different numerology	Nokia, Nokia Shanghai Bell

[100e-NR-LTE_NR_DC_CA_enh-Cross-CCScheduling-01] – Karri (Nokia)

E-mail discussion/approval regarding issue #4 in R1-2001068 by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

Decision: As per email decision posted on Feb. 29th,

Agreements:

- To extend the cross-carrier scheduling with different SCS to be operable with DCI format 1_2 defined for Rel-16 in eURLLC WI.

  - Note: to be discussed under UE feature discussion regarding whether or not to introduce separate UE capability to indicate the support for cross-carrier scheduling with different SCS to be operable with DCI format 1_1 and DCI format 1_2

R1-2001352	Outcome of email thread [100e-NR-LTE_NR_DC_CA_enh-Cross-CCScheduling-01]	Nokia

Decision: As per email decision posted on Mar. 4th, the TP to TS 38.214 subclause 5.1.5 (R1-2001352) is endorsed.

[100e-NR-LTE_NR_DC_CA_enh-Cross-CCScheduling-02] – Karri (Nokia)

E-mail discussion/approval regarding issue #1/2/3 in R1-2001068 by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001353	Outcome of email thread [100e-NR-LTE_NR_DC_CA_enh-Cross-CCScheduling-02]	Nokia

Decision: As per email decision posted on Mar. 4th, the TPs to TS 38.214 subclause 5.1.5 and to TS 38.213 subclause 10.1 (R1-2001353) are endorsed.

[100e-NR-LTE_NR_DC_CA_enh-Cross-CCScheduling-03] – Karri (Nokia)

E-mail discussion/approval regarding issue #5 in R1-2001068 by 2/28; if there is a spec impact, followed by endorsing the corresponding TP by 3/3

R1-2001354	Outcome of email thread [100e-NR-LTE_NR_DC_CA_enh-Cross-CCScheduling-03]	Nokia

Decision: As per email decision posted on Mar. 4th, no conclusion. For further discussion.

R1-2000350	Remaining issues on cross-carrier scheduling with mix numerologies	vivo

R1-2000365	Remaining Issues on Cross-carrier Scheduling with Mixed Numerologies	ZTE

R1-2000646	Remaining issues for cross-carrier scheduling with different numerology	Samsung

R1-2000893	Remaining issues for cross-carrier scheduling with different numerologies	Ericsson

R1-2000983	CR for default QCL assumption for Rel-16 cross-carrier scheduling	Qualcomm Incorporated

R1-2000985	Discussion on support of cross-carrier scheduling with different numerology	ASUSTeK

R1-2001035	Corrections on cross-carrier scheduling with different numerology	Huawei, HiSilicon

#### Other

R1-2000209	Discussion on Cross-carrier triggering PDCCH order	Huawei, HiSilicon

R1-2000894	Data rate handling for intra-FR NR-DC	Ericsson

### NR Rel-16 UE Features

[100e-NR-Rel-16-UEFeatures] – Hiroki (DCM)/Xiaoyi (ATT)

Email discussion/approval of Rel-16 NR UE feature till next meeting (starting no earlier than 3/6)

R1-2000212	On NR UE features in Rel-16	Huawei, HiSilicon

R1-2000351	On Rel-16 UE features	vivo

R1-2000367	Discussion on NR Rel-16 UE Features	ZTE

R1-2000454	Views on Rel-16 UE feature list	MediaTek Inc.

R1-2000488	Discussion on Rel-16 UE features	OPPO

R1-2000930	RAN1 UE features list for Rel-16 NR	NTT DOCOMO, INC., AT&T

Late submission

R1-2001117	On Rel-16 UE Features	Ericsson LM

### Other

Including any essential TPs related to the endorsed Rel-16 NR TEIs.

R1-2000931	Summary on NR Rel-16 TEI related discussion	NTT DOCOMO, INC.

R1-2001155	Output summary on NR Rel-16 TEI after RAN1#100-e preparation phase	NTT DOCOMO, INC.

[100e-NR-TEICRs-01]  – Hiroki (NTT DOCOMO)

Email discussion/approval on remaining issue for CLI measurement and reporting (using R1-2000692 as a starting point) by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001231	Outcome of email thread [100e-NR-TEICRs-01]	NTT DOCOMO

Decision: As per email decision posted on Feb.29th, LS to RAN4 is approved in R1-2001320, further to:

Agreements:

- Send LS to RAN4 (CC RAN2) to ask their checking and confirmation on following RAN1 assumptions.

  - UE may performs CLI-RSSI measurement with the SCS of the active bandwidth part within the configured CLI-RSSI resource in the active BWP regardless of the reference SCS of the measurement resource.

    - CLI-RSSI measurement is applicable for RRC_CONNECTED intra-frequency only.

  - When SRS-RSRP measurement resource is fully confined within BW of DL active BWP, UE operates SRS-RSRP measurement using the SRS-RSRP measurement resource. Otherwise the UE does not operate SRS-RSRP measurement using the SRS-RSRP measurement resource. UE is not expected to be configured a SRS-RSRP resource outside DL BWP.

    - SRS-RSRP measurement is applicable for RRC_CONNECTED intra-frequency only.

[100e-NR-TEICRs-02]  – Hiroki (NTT DOCOMO)

Email discussion/approval on remaining issue for half-dupelx operation in CA (using R1-2001121 (proposal 2)/2000514 (proposal 4) as a starting point) by 2/27; if there is a spec impact, followed by endorsing the corresponding TP by 3/2

R1-2001232	Outcome of email thread [100e-NR-TEICRs-02]	NTT DOCOMO

Decision: As per email decision posted on Mar. 4th,

Agreements:

Following working assumption is confirmed as agreement with following updates.

Regarding TEI “Half-duplex operation in CA”

- Agreements made during email discussion [95-NR-06] on directional collision handling for half duplex CA operation are adopted in Rel-16.

  - Note: Dropping semi SFI U and D on other cell means that UE drops all higher-layer configured transmissions and reception (both RRC U and RRC D) on the other cell, but may receive Dynamic D and transmit Dynamic U, correspondingly.

- A new UE capability to indicate the support for directional collision handling between reference and other cell(s) is introduced.

- A new RRC parameter to activate directional collision handling between reference and other cell(s).

  - Note that: For Half-duplex UEs, UEs that do not indicate the support of this capability should still be able to operate half-duplex TDD CA if network ensures same transmission direction across all the serving cells.

Conclusion:

It’s understood that for the cases where there is inherent confusion due to , e.g. application order of rules, different numerology, logical/actual timing, it is up to UE implementation

R1-2001385	Endorsed TP in [100e-NR-TEICRs-02]	Nokia, Nokia Shanghai Bell

R1-2000514	On the TBS issue and the TEI issue of half-duplex operation in CA	ZTE

R1-2000573	Features for Rel-16 V2X	Futurewei

R1-2000692	Remaining details of CLI measurement and reporting at a UE	LG Electronics

R1-2000895	Remaining issues for DSS	Samsung, KDDI, Nokia, Nokia Shanghai Bell

R1-2000984	Discussion on Rel-16 NR UE features	Qualcomm Incorporated

R1-2000986	Remaining issues for Rel-16 maintenance and TEI	Ericsson

R1-2001064	On remaining NR TEI issues	Nokia, Nokia Shanghai Bell

R1-2001109	Further details of Rel-16 UE features for MIMO	Huawei, HiSilicon

R1-2001121	On remaining NR TEI issues	Nokia, Nokia Shanghai Bell

Late submission

# Release 17

This section is intentionally left as void

# Closing of the meeting

Meeting was closed on Mar. 12th

# Annex A:	List of Tdocs at RAN1 #100-e

Please see excel file attached to this report

# Annex B:	List of CRs agreed at RAN1 #100-e

| TS/TR | CR | rev | Rel | Title | Cat | Vsn | @ Mtg | TD# | Source to WG | Work Item |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36.213 | 1300 | 0 | Rel-14 | Correction on priority value in LTE V2X | F | 14.13.0 | RAN1#100-e | R1-2001224 | Sharp | LTE_V2X-Core |
| 37.213 | 0006 | 0 | Rel-15 | Correction on conditions for indicating Type 2 channel access procedure | F | 15.2.0 | RAN1#100-e | R1-2001305 | WILUS Inc. | LTE_unlic-Core |
| 36.211 | 0505 | 0 | Rel-14 | Correction on the resource block pair definition for 1.25 kHz subcarrier spacing | F | 14.13.1 | RAN1#100-e | R1-2001321 | Huawei, HiSilicon | MBMS_LTE_enh2-Core |
| 36.211 | 0506 | 0 | Rel-15 | Correction on the resource block pair definition for 1.25 kHz subcarrier spacing | A | 15.8.1 | RAN1#100-e | R1-2001322 | Huawei, HiSilicon | MBMS_LTE_enh2-Core |
| 36.211 | 0507 | 0 | Rel-16 | Correction on the resource block pair definition for 1.25 kHz subcarrier spacing | A | 16.0.0 | RAN1#100-e | R1-2001323 | Huawei, HiSilicon | MBMS_LTE_enh2-Core |
| 36.201 | 0029 | 0 | Rel-15 | Correction to measurements for inter-RAT handover | F | 15.2.0 | RAN1#100-e | R1-2001324 | Huawei, HiSilicon | NR_newRAT-Core |
| 36.213 | 1301 | 0 | Rel-15 | Correction on priority value in LTE V2X | A | 15.8.0 | RAN1#100-e | R1-2001325 | Sharp | LTE_V2X-Core |
| 36.213 | 1302 | 0 | Rel-15 | Correction on Msg3 NPUSCH for NB-IoT EDT | F | 15.8.0 | RAN1#100-e | R1-2001326 | Huawei, HiSilicon | NB_IOTenh2-Core |
| 36.211 | 0508 | 0 | Rel-15 | Correction on Rel-15 NWUS/MWUS | F | 15.8.1 | RAN1#100-e | R1-2001327 | Huawei, HiSilicon | LTE_eMTC4-Core, NB_IOTenh2-Core |
| 36.201 | 0030 | 0 | Rel-15 | Inclusion of NWUS, MWUS, and RSS in TS 36.201 | F | 15.2.0 | RAN1#100-e | R1-2001328 | Huawei, HiSilicon | LTE_eMTC4-Core, NB_IOTenh2-Core |
| 36.213 | 1303 | 0 | Rel-16 | Correction on priority value in LTE V2X | A | 16.0.0 | RAN1#100-e | R1-2001329 | Sharp | LTE_V2X-Core |
| 36.213 | 1304 | 0 | Rel-14 | Corrections on the indication value for the transport blocks in a bundle for HD-FDD eMTC UEs | F | 14.13.0 | RAN1#100-e | R1-2001330 | Huawei, HiSilicon | LTE_feMTC-Core |
| 36.213 | 1305 | 0 | Rel-15 | Corrections on the indication value for the transport blocks in a bundle for HD-FDD eMTC UEs | A | 15.8.0 | RAN1#100-e | R1-2001331 | Huawei, HiSilicon | LTE_feMTC-Core |
| 36.213 | 1306 | 0 | Rel-16 | Corrections on the indication value for the transport blocks in a bundle for HD-FDD eMTC UEs | A | 16.0.0 | RAN1#100-e | R1-2001332 | Huawei, HiSilicon | LTE_feMTC-Core |
| 36.212 | 0332 | 0 | Rel-14 | Corrections on the indication value for the transport blocks in a bundle for HD-FDD eMTC UEs | F | 14.11.0 | RAN1#100-e | R1-2001333 | Huawei, HiSilicon | LTE_feMTC-Core |
| 36.212 | 0333 | 0 | Rel-15 | Corrections on the indication value for the transport blocks in a bundle for HD-FDD eMTC UEs | A | 15.8.0 | RAN1#100-e | R1-2001334 | Huawei, HiSilicon | LTE_feMTC-Core |
| 36.212 | 0334 | 0 | Rel-16 | Corrections on the indication value for the transport blocks in a bundle for HD-FDD eMTC UEs | A | 16.0.0 | RAN1#100-e | R1-2001335 | Huawei, HiSilicon | LTE_feMTC-Core |
| 36.213 | 1307 | 0 | Rel-16 | Correction on Msg3 NPUSCH for NB-IoT EDT | A | 16.0.0 | RAN1#100-e | R1-2001336 | Huawei, HiSilicon | NB_IOTenh2-Core |
| 36.211 | 0509 | 0 | Rel-16 | Correction on Rel-15 NWUS/MWUS | A | 16.0.0 | RAN1#100-e | R1-2001337 | Huawei, HiSilicon | LTE_eMTC4-Core, NB_IOTenh2-Core |
| 36.213 | 1308 | 0 | Rel-13 | Clarification on MPDCCH monitoring during UL gaps | F | 13.15.0 | RAN1#100-e | R1-2001378 | ZTE,Sanechips | LTE_MTCe2_L1-Core |
| 36.213 | 1309 | 0 | Rel-14 | Clarification on MPDCCH monitoring during UL gaps | A | 14.13.0 | RAN1#100-e | R1-2001379 | ZTE,Sanechips | LTE_MTCe2_L1-Core |
| 36.213 | 1310 | 0 | Rel-15 | Clarification on MPDCCH monitoring during UL gaps | A | 15.8.0 | RAN1#100-e | R1-2001380 | ZTE,Sanechips | LTE_MTCe2_L1-Core |
| 36.213 | 1311 | 0 | Rel-16 | Clarification on MPDCCH monitoring during UL gaps | A | 16.0.0 | RAN1#100-e | R1-2001381 | ZTE,Sanechips | LTE_MTCe2_L1-Core |
| 36.211 | 0510 | 0 | Rel-16 | Miscellaneous corrections for Rel-16 LTE-MTC features in 36.211 | F | 16.0.0 | RAN1#100-e | R1-2001427 | Ericsson | LTE_eMTC5-Core |
| 36.211 | 0511 | 0 | Rel-16 | Miscellaneous corrections for Rel-16 NB-IoT features in 36.211 | F | 16.0.0 | RAN1#100-e | R1-2001428 | Ericsson | NB_IOTenh3-Core |
| 36.211 | 0512 | 0 | Rel-16 | Corrections to LTE terrestrial broadcast | F | 16.0.0 | RAN1#100-e | R1-2001429 | Ericsson | LTE_terr_bcast-Core |
| 36.211 | 0513 | 0 | Rel-16 | Corrections to DL MIMO efficiency enhancements for LTE | F | 16.0.0 | RAN1#100-e | R1-2001430 | Ericsson | LTE_DL_MIMO_EE-Core |
| 36.212 | 0335 | 0 | Rel-16 | Miscellaneous corrections for Rel-16 eMTC features in 36.212 | F | 16.0.0 | RAN1#100-e | R1-2001431 | Futurewei | LTE_eMTC5-Core |
| 36.212 | 0336 | 0 | Rel-16 | Miscellaneous corrections for Rel-16 NB-IoT features in 36.212 | F | 16.0.0 | RAN1#100-e | R1-2001432 | Futurewei | NB_IOTenh3-Core |
| 36.213 | 1312 | 0 | Rel-16 | Corrections to Additional MTC Enhancements for LTE | F | 16.0.0 | RAN1#100-e | R1-2001433 | Motorola Mobility | LTE_eMTC5-Core |
| 36.213 | 1313 | 0 | Rel-16 | Corrections to Additional enhancements for NB-IoT | F | 16.0.0 | RAN1#100-e | R1-2001434 | Motorola Mobility | NB_IOTenh3-Core |
| 36.213 | 1314 | 0 | Rel-16 | Corrections to single Tx switched uplink solution for EN-DC feature of Rel-16 MR DC/CA | F | 16.0.0 | RAN1#100-e | R1-2001435 | Motorola Mobility | LTE_NR_DC_CA_enh-Core |
| 36.213 | 1315 | 0 | Rel-16 | Corrections to DL MIMO efficiency enhancements for LTE | F | 16.0.0 | RAN1#100-e | R1-2001436 | Motorola Mobility | LTE_DL_MIMO_EE-Core |
| 37.213 | 0007 | 0 | Rel-16 | Corrections to NR-based access to unlicensed spectrum | F | 16.0.0 | RAN1#100-e | R1-2001437 | Ericsson | NR_unlic-Core |
| 38.215 | 0017 | 0 | Rel-16 | Corrections to L1-SINR definitions | F | 16.0.1 | RAN1#100-e | R1-2001438 | Intel | NR_eMIMO-Core |
| 38.215 | 0018 | 0 | Rel-16 | Corrections to NR positioning support | F | 16.0.1 | RAN1#100-e | R1-2001439 | Intel | NR_pos-Core |
| 38.215 | 0019 | 0 | Rel-16 | Corrections to cross layer interference measurements | F | 16.0.1 | RAN1#100-e | R1-2001440 | Intel | NR_CLI_RIM-Core |
| 38.214 | 0068 | 0 | Rel-16 | Corrections on NR - U | F | 16.0.0 | RAN1#100-e | R1-2001441 | Nokia | NR_unlic-Core |
| 38.214 | 0069 | 0 | Rel-16 | Corrections on NR V2X | F | 16.0.0 | RAN1#100-e | R1-2001442 | Nokia | 5G_V2X_NRSL-Core |
| 38.214 | 0070 | 0 | Rel-16 | Corrections on Cross-carrier Scheduling with Different Numerologies | F | 16.0.0 | RAN1#100-e | R1-2001443 | Nokia | LTE_NR_DC_CA_enh-Core |
| 38.214 | 0071 | 0 | Rel-16 | Corrections on NR URLLC support | F | 16.0.0 | RAN1#100-e | R1-2001444 | Nokia | NR_L1enh_URLLC-Core |
| 38.214 | 0072 | 0 | Rel-16 | Corrections on NR enhanced MIMO | F | 16.0.0 | RAN1#100-e | R1-2001445 | Nokia | NR_eMIMO-Core |
| 38.214 | 0073 | 0 | Rel-16 | Corrections on Industrial IoT | F | 16.0.0 | RAN1#100-e | R1-2001446 | Nokia | NR_IIOT-Core |
| 38.214 | 0074 | 0 | Rel-16 | Corrections of cross-slot scheduling restriction and CSI/L1-RSRP measurement outside active time | F | 16.0.0 | RAN1#100-e | R1-2001447 | Nokia | NR_UE_pow_sav-Core |
| 38.214 | 0075 | 0 | Rel-16 | Corrections on two-step RACH after RAN1#100-e | F | 16.0.0 | RAN1#100-e | R1-2001448 | Nokia | NR_2step_RACH-Core |
| 38.214 | 0076 | 0 | Rel-16 | Corrections of NR positioning support | F | 16.0.0 | RAN1#100-e | R1-2001449 | Nokia | NR_pos-Core |
| 38.213 | 0085 | 0 | Rel-16 | Corrections on two-step RACH | F | 16.0.0 | RAN1#100-e | R1-2001450 | Samsung | NR_2step_RACH-Core |
| 38.213 | 0086 | 0 | Rel-16 | Corrections on NR-DC and on Cross-carrier Scheduling with Different Numerologies | F | 16.0.0 | RAN1#100-e | R1-2001451 | Samsung | LTE_NR_DC_CA_enh-Core |
| 38.213 | 0087 | 0 | Rel-16 | Corrections on integrated access and backhaul | F | 16.0.0 | RAN1#100-e | R1-2001452 | Samsung | NR_IAB-Core |
| 38.213 | 0088 | 0 | Rel-16 | Corrections on Industrial IoT | F | 16.0.0 | RAN1#100-e | R1-2001453 | Samsung | NR_IIOT-Core |
| 38.213 | 0089 | 0 | Rel-16 | Corrections on MIMO enhancements | F | 16.0.0 | RAN1#100-e | R1-2001454 | Samsung | NR_eMIMO-Core |
| 38.213 | 0090 | 0 | Rel-16 | Corrections on Mobility Enhancements | F | 16.0.0 | RAN1#100-e | R1-2001455 | Samsung | NR_Mob_enh-Core |
| 38.213 | 0091 | 0 | Rel-16 | Corrections on shared spectrum channel access | F | 16.0.0 | RAN1#100-e | R1-2001456 | Samsung | NR_unlic-Core |
| 38.213 | 0092 | 0 | Rel-16 | Corrections on Positioning | F | 16.0.0 | RAN1#100-e | R1-2001457 | Samsung | NR_pos-Core |
| 38.213 | 0093 | 0 | Rel-16 | Corrections on Sidelink | F | 16.0.0 | RAN1#100-e | R1-2001458 | Samsung | 5G_V2X_NRSL-Core |
| 38.213 | 0094 | 0 | Rel-16 | Corrections on UE power savings | F | 16.0.0 | RAN1#100-e | R1-2001459 | Samsung | NR_UE_pow_sav-Core |
| 38.213 | 0095 | 0 | Rel-16 | Corrections on Ultra Reliable Low Latency Communications Enhancements | F | 16.0.0 | RAN1#100-e | R1-2001460 | Samsung | NR_L1enh_URLLC-Core |
| 38.212 | 0032 | 0 | Rel-16 | Corrections for Rel-16 NR-U after RAN1#100-e | F | 16.0.0 | RAN1#100-e | R1-2001461 | Huawei | NR_unlic-Core |
| 38.212 | 0033 | 0 | Rel-16 | Corrections for NR MIMO after RAN1#100-e | F | 16.0.0 | RAN1#100-e | R1-2001462 | Huawei | NR_eMIMO-Core |
| 38.212 | 0034 | 0 | Rel-16 | Corrections for URLLC after RAN1#100-e | F | 16.0.0 | RAN1#100-e | R1-2001463 | Huawei | NR_L1enh_URLLC-Core |
| 38.212 | 0035 | 0 | Rel-16 | Corrections for power saving after RAN1#100-e | F | 16.0.0 | RAN1#100-e | R1-2001464 | Huawei | NR_UE_pow_sav-Core |
| 38.212 | 0036 | 0 | Rel-16 | Corrections on 5G V2X sidelink features after RAN1#100-e | F | 16.0.0 | RAN1#100-e | R1-2001465 | Huawei | 5G_V2X_NRSL-Core |
| 38.211 | 0033 | 0 | Rel-16 | Corrections to integrated access and backhaul for NR | F | 16.0.0 | RAN1#100-e | R1-2001466 | Ericsson | NR_IAB-Core |
| 38.211 | 0034 | 0 | Rel-16 | Corrections to NR positioning support | F | 16.0.0 | RAN1#100-e | R1-2001467 | Ericsson | NR_pos-Core |
| 38.211 | 0035 | 0 | Rel-16 | Corrections to two-step RACH | F | 16.0.0 | RAN1#100-e | R1-2001468 | Ericsson | NR_2step_RACH-Core |
| 38.211 | 0036 | 0 | Rel-16 | Corrections to cross-carrier scheduling with different numerologies | F | 16.0.0 | RAN1#100-e | R1-2001469 | Ericsson | LTE_NR_DC_CA_enh-Core |
| 38.211 | 0037 | 0 | Rel-16 | Corrections to NR-based access to unlicensed spectrum | F | 16.0.0 | RAN1#100-e | R1-2001470 | Ericsson | NR_unlic-Core |
| 38.211 | 0038 | 0 | Rel-16 | Corrections to V2X | F | 16.0.0 | RAN1#100-e | R1-2001471 | Ericsson | 5G_V2X_NRSL-Core |
| 38.211 | 0039 | 0 | Rel-16 | Corrections to MIMO enhancements | F | 16.0.0 | RAN1#100-e | R1-2001472 | Ericsson | NR_eMIMO-Core |
| 38.213 | 0096 | 0 | Rel-15 | Corrections to 38.213 including alignment of terminology across specifications in RAN1#100-e | F | 15.8.0 | RAN1#100-e | R1-2001474 | Samsung | NR_newRAT-Core |
| 38.213 | 0097 | 0 | Rel-16 | Introduction of half-duplex operation in CA with unpaired spectrum | B | 16.0.0 | RAN1#100-e | R1-2001475 | Samsung | TEI16 |
| 38.214 | 0077 | 0 | Rel-15 | CR on UL PTRS density selection | F | 15.8.0 | RAN1#100-e | R1-2001476 | Nokia | NR_newRAT-Core |
| 38.215 | 0020 | 0 | Rel-16 | Corrections to V2X measurement definitions | F | 16.0.1 | RAN1#100-e | R1-2001480 | Intel Corporation | 5G_V2X_NRSL-Core |

# Annex C-1:	List of Outgoing LSs from RAN1 #100-e

| TDoc # | Title | Source | Release | Related WIs | Reply to | To | Cc | Original LS | Reply in |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2001300 | LS to RAN2 on NR-U related changes for 38.300 running CR | RAN1, Qualcomm | Rel-16 | NR_unlic-Core |  | RAN2 |  |  |  |
| R1-2001304 | Reply LS on NR V2X resource pool configuration and selection | RAN1, vivo | Rel-16 | 5G_V2X_NRSL-Core | R2-1908486 | RAN2 |  |  |  |
| R1-2001306 | Reply LS for clarification of PUCCH configuration | RAN1, Huawei | Rel-15 | NR_newRAT-Core | R2-1916481 | RAN2 |  |  |  |
| R1-2001307 | Reply LS on default codebook parameters | RAN1, Nokia | Rel-15 | NR_newRAT-Core | R2-1916592 | RAN2 |  |  |  |
| R1-2001319 | Reply LS on clarification of CLI resource configuration | LG Electronics | Rel-16 | NR_CLI_RIM-Core | R2-1916645 | RAN2 | RAN4 |  |  |
| R1-2001320 | LS on CLI measurement and reporting | LG Electronics | Rel-16 | NR_CLI_RIM-Core |  | RAN4 | RAN2 |  |  |
| R1-2001356 | LS to RAN2 on Rel-16 WIs (except NR-U) related changes for 38.300 running CR | RAN1, Nokia | Rel-16 | 5G_V2X_NRSL-Core, NR_pos-Core, LTE_NR_DC_CA_enh-Core |  | RAN2 |  |  |  |
| R1-2001357 | LS on SSB index and candidate SSB index for NR-U | RAN1, Samsung, Charter Communications | Rel-16 | NR_unlic-Core |  | RAN2, RAN4 |  |  |  |
| R1-2001375 | LS on NR-U enhancements to initial access procedures | RAN1, Charter Communications | Rel-16 | NR_unlic-Core |  | RAN2 |  |  |  |
| R1-2001376 | Reply LS on UL skipping | RAN1, vivo | Rel-15 | NR_newRAT-Core | R2-1916572 | RAN2 |  |  |  |
| R1-2001483 | LS on outcome of email discussions on aperiodic SRS for positioning configuration from RAN1#100e | RAN1, Ericsson | Rel-16 | NR_pos-Core |  | RAN2 |  |  |  |
| R1-2001397 | Reply LS on consistent Uplink LBT failure detection mechanism | RAN1, Nokia | Rel-16 | NR_unlic-Core | R2-1916380 | RAN2 |  |  |  |
| R1-2001421 | LS on uplink power control for NR-NR Dual-Connectivity | RAN1, Apple | Rel-16 | LTE_NR_DC_CA_enh-Core |  | RAN2 |  |  |  |
| R1-2001426 | LS on sidelink HARQ | RAN1, LG Electronics | Rel-16 | 5G_V2X_NRSL-Core |  | RAN2 |  |  |  |
| R1-2001479 | LS on updated Rel-16 LTE and NR parameter lists | RAN1, Qualcomm | Rel-16 | LTE_eMTC5-Core, NB_IOTenh3-Core, LTE_DL_MIMO_EE-Core, LTE_terr_bcast-Core, NR_2step_RACH-Core, NR_unlic-Core, NR_IAB-Core, 5G_V2X_NRSL-Core, NR_L1enh_URLLC-Core, NR_IIOT-Core, NR_eMIMO-Core, NR_UE_pow_sav-Core, NR_pos-Core, NR_Mob_enh-Core, LTE_NR_DC_CA_enh-Core |  | RAN2 |  |  |  |
| R1-2001486 | LS on Rel-16 RAN1 UE features lists for LTE | RAN1, NTT DOCOMO | Rel-16 | LTE_eMTC5-Core, NB_IOTenh3-Core, LTE_DL_MIMO_EE-Core, LTE_terr_bcast-Core |  | RAN2 | RAN4 |  |  |

# Annex C-2:	List of Incoming LSs from RAN1 #100-e

| TDoc # | Title | Source | Release | Related WIs | Reply to | To | Cc | Original LS | Reply in |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2000152 | LS reply on PUR transmission for NB-IoT/eMTC | RAN2, Ericsson | Rel-16 | NB_IOTenh3-Core, LTE_eMTC5-Core | R1-1911399 | RAN1 | RAN4 | R2-1916435 |  |
| R1-2000153 | Feedback on RAN1 agreements on PUR | RAN2, Qualcomm | Rel-16 | LTE_eMTC5-Core, NB_IOTenh3-Core |  | RAN1 |  | R2-1916437 |  |
| R1-2000154 | Reply LS on support of high peak data rate for Cat-M1 Ues in HD-FDD | RAN2, Qualcomm | Rel-16 | TEI16 | R1-1911398 | RAN1, RAN |  | R2-1916327 |  |
| R1-2000155 | LS on CLI measurements UE capabilities | RAN2, Qualcomm | Rel-16 | NR_CLI_RIM-Core |  | RAN1 | RAN4 | R2-1916348 |  |
| R1-2000156 | LS on consistent Uplink LBT failure detection mechanism | RAN2, Qualcomm | Rel-16 | NR_unlic-Core |  | RAN1 |  | R2-1916380 |  |
| R1-2000157 | Reply LS on signalling of sidelink RSRP and CSI | RAN2, LG Electronics | Rel-16 | 5G_V2X_NRSL-Core | R1-1911698 | RAN1 |  | R2-1916457 |  |
| R1-2000158 | Response LS on additional high layer information for sidelink physical layer operations | RAN2, LG Electronics | Rel-16 | 5G_V2X_NRSL-Core | R1-1911746 | RAN1 |  | R2-1916458 |  |
| R1-2000159 | Response LS on TX resource (re-)selection and MAC related agreements | RAN2, LG Electronics | Rel-16 | 5G_V2X_NRSL-Core | R1-1911746 | RAN1 |  | R2-1916460 |  |
| R1-2000160 | Reply LS on sidelink synchronization under multiple synchronization sources with different timing | RAN2, Qualcomm | Rel-16 | 5G_V2X_NRSL-Core | R4-1912826 | RAN4, RAN1 |  | R2-1916465 |  |
| R1-2000161 | Reply LS on UL-SL prioritization | RAN2, OPPO | Rel-16 | 5G_V2X_NRSL-Core | R1-1911720 | RAN1, RAN4 |  | R2-1916468 |  |
| R1-2000162 | LS to RAN1 for clarification of PUCCH configuration | RAN2, Huawei | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1916481 |  |
| R1-2000163 | LS on Uplink skipping | RAN2, vivo | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1916572 |  |
| R1-2000164 | LS on default codebook parameters | RAN2, Nokia | Rel-15 | NR_newRAT-Core |  | RAN1 |  | R2-1916592 |  |
| R1-2000165 | LS on secondary DRX group | RAN2, Ericsson | Rel-16 | TEI16 |  | RAN1, RAN4 |  | R2-1916597 |  |
| R1-2000166 | LS on RAN2 progress on Scell uplink behaviour of the UE in dormancy | RAN2, Intel | Rel-16 | LTE_NR_DC_CA_enh-Core |  | RAN1 | RAN4 | R2-1916601 |  |
| R1-2000167 | Response LS on SL RLM/RLF | RAN2, InterDigital | Rel-16 | 5G_V2X_NRSL-Core | R1-1913464 | RAN1, SA2 |  | R2-1916621 |  |
| R1-2000168 | LS on clarification of CLI resource configuration | RAN2, LG Electronics | Rel-16 | NR_CLI_RIM-Core |  | RAN1 |  | R2-1916645 |  |
| R1-2000169 | LS on updates for TS 36.300 and TS 38.300 | RAN2, Huawei | Rel-16 | NB_IOTenh3-Core, LTE_eMTC5-Core |  | RAN3 | RAN1 | R2-1916563 |  |
| R1-2000170 | Reply LS on clarification of OAM requirements for RIM | RAN3, Nokia | Rel-16 | NR_CLI_RIM-Core | S5-195662 | SA5 | RAN1 | R3-197540 |  |
| R1-2000171 | Reply LS on Tx DC location | RAN4, Huawei | Rel-15 | NR_newRAT-Core | R1-1909849 | RAN1, RAN2 |  | R4-1915361 |  |
| R1-2000172 | Response LS on Reference Point for Timing Related Measurements in FR2 | RAN4, CATT | Rel-16 | NR_pos-Core | R1-1907869 | RAN1 | RAN2, RAN3 | R4-1915801 |  |
| R1-2000173 | Reply LS on applicable timing for pathloss RS activated/updated by MAC-CE | RAN4, Apple | Rel-16 | NR_eMIMO-Core | R1-1911616 | RAN1 |  | R4-1915929 |  |
| R1-2000174 | Reply LS to RAN2 on UL-SL Prioritization | RAN4, Futurewei | Rel-16 | 5G_V2X_NRSL-Core | R2-1911679 | RAN2, RAN1 |  | R4-1915985 |  |
| R1-2000175 | Reply to LS on UE capabilities on DAPS HO | RAN4, Qualcomm Incorporated | Rel-16 | NR_Mob_enh-Core, LTE_feMob-Core | R2-1913999 | RAN2 | RAN1 | R4-1915781 |  |
| R1-2000176 | LS on UE capabilities and RRC signalling on Tx switching period delay | RAN4, Apple | Rel-16 | NR_RF_FR1 |  | RAN2 | RAN1 | R4-1916083 |  |
| R1-2000177 | LS on channel raster for NR V2X UE | RAN4, CATT | Rel-16 | 5G_V2X_NRSL-Core |  | RAN2 | RAN1 | R4-1916146 |  |
| R1-2000178 | LS on clarifying NR PC5 priority level | SA2, LG Electronics | Rel-16 | eV2XARC |  | RAN1 | RAN2 | S2-1912003 |  |
| R1-2000179 | Reply LS on Response LS on SL RLM/RLF | SA2, Qualcomm |  | eV2XARC | R2-1916621 | RAN2, RAN1, CT1 |  | S2-2000973 |  |
| R1-2001236 | LS to RAN1 on Support of 2-step CFRA | RAN2, ZTE |  | NR_2step_RACH-Core |  | RAN1 |  | R2-2001929 |  |
| R1-2001237 | LS to RAN1 on NR-U PRACH root sequence for 2-step RA | RAN2, Ericsson |  | NR_unlic-Core, NR_2step_RACH-Core |  | RAN1 |  | R2-2002138 |  |

# Annex D:	List of Approved updated WIDs

None

# Annex E:	List of draft TSs/TRs agreed at RAN1 #100-e

| Tdoc Number | Title | Source | Conclusion/Decision |
| --- | --- | --- | --- |
| R1-2001230 | TR 36.976 Overall description of LTE-based 5G broadcast | Qualcomm Incorporated | Endorsed by RAN1 as version 2.0.0 for approval |
| R1-2001223 | TR 37.985, “Overall description of Radio Access Network (RAN) aspects for Vehicle-to-everything (V2X) based on LTE and NR”, v1.1.0 | Huawei | Endorsed version by RAN1 |

# Annex F:	List of actions

- Outgoing LS

Further to [100e-LTE-Rel-16-UEFeatures] email thread, the following LS to RAN2, Cc RAN4 is approved.

R1-2001486	LS on Rel-16 RAN1 UE features lists for LTE	RAN1, NTT DOCOMO

- CR approval

None

- TS/TR

None

- Miscellaneous

[100e-LTE-Rel-16-UEFeatures] – Hiroki (DCM)/Xiaoyi (ATT)

Email discussion/approval of Rel-16 LTE UE feature till next meeting (starting no earlier than 3/6)

Done: According to Chair's decision posted on April 10th, the LTE UE features (intermediate) list for Rel-16 is noted, to be used as the baseline/reference for the next update.

R1-2001485	RAN1 UE features list for Rel-16 LTE after RAN1#100-e	Moderator (AT&T, NTT DOCOMO, INC.)

[100e-NR-Rel-16-UEFeatures] – Hiroki (DCM)/Xiaoyi (ATT)

Email discussion/approval of Rel-16 NR UE feature till next meeting (starting no earlier than 3/6)

Done: According to Chair's decision posted on April 9th, the NR UE features (intermediate) list for Rel-16 is noted, to be used as the baseline/reference for the next update.

R1-2001484	RAN1 UE features list for Rel-16 NR after RAN1#100-e	Moderator (AT&T, NTT DOCOMO, INC.)

# Annex G:	List of participants at RAN1 #100-e

Please see excel file attached to this report

# Annex H:	TSG RAN WG1 meetings in 2020 – 2021

| TITLE | TYPE | DATES | LOCATION | CTRY |
| --- | --- | --- | --- | --- |
| 3GPPRAN1#100bis 3GPPRAN1#100bis-e | WG AH | 20 - 30 Apr 2020 | Busan e-meeting |  |
| 3GPPRAN1#101 3GPPRAN1#101-e | WG AH | 25 May – 05 Jun 2020 | Athens e-meeting |  |
| 3GPPRAN1#102 | WG | 24 - 28 Aug 2020 | Toulouse | France |
| 3GPPRAN1#102bis | WG | 12 - 16 Oct 2020 | TBD | China |
| 3GPPRAN1#103 | WG | 16 - 20 Nov 2020 | TBD | US |
| 3GPPRAN1#104 | WG | 01 - 05 Mar 2021 | TBD | EU |
| 3GPPRAN1#104bis | WG | 19 - 23 Apr 2021 | TBD | China |
| 3GPPRAN1#105 | WG | 24 - 28 May 2021 | TBD | US |
| 3GPPRAN1#106 | WG | 23 - 27 Aug 2021 | TBD | EU |
| 3GPPRAN1#106bis | WG | 11 - 15 Oct 2021 | TBD | China |
| 3GPPRAN1#107 | WG | 15 - 19 Nov 2021 | TBD | US |

| MEETING TYPES |  |  |
| --- | --- | --- |
| AH = Ad Hoc | ST = Startup Meeting | RG = Rapporteurs Group |
| JM = Joint | WG = Working Group | SG = Steering Group |
| PM = Preparatory Meeting | CM = Chairmen's meeting | TG = Task Group |
| RM = Resolution Meeting | OR = Ordinary | XO = Extraordinary |

End of document