3GPP TSG RAN WG1 Meeting #100bis-e	R1-2001619

e-Meeting, April 20th – 30th, 2020

Source: 	ZTE Corporation

Title: 	Remaining Issues on Single Tx UL Transmission

Agenda item:	7.2.10.2

Document for:	Discussion/Decision

# Introduction

During RAN1#100e meeting, most of the issues on single Tx UL transmission have been resolved except for the two candidate TPs in [100e-NR-LTE_NR_DC_CA_enh-SingleTx-02] [1]. In this contribution, we present discussion and analysis on the remaining issues on single Tx UL transmission.

# Discussion

## Issue 2 from RAN1#100e

The two candidate TPs for Issue 2 are summarized in [1]. The major difference between TP1 and TP2 is highlighted in yellow as shown below. Overall, the condition in TP1 and TP2 could be summarized as following.

TP1:

(1) UE indicates capability for dynamic power sharing between LTE and NR for EN-DC, and

(2) UE indicates using singleUL-Transmission that it does not support simultaneous UL transmissions, and

(3) UE is configured with tdm-PatternConfig-r16.

TP1: If the UE indicates capability for dynamic power sharing between E-UTRA and NR for EN-DC, and if the UE is configured with tdm-PatternConfig-r16, then for the band combinations for which the UE indicates using singleUL-Transmission that it does not support simultaneous UL transmissions (as defined in TS 38.101-3 [8-3]), the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission on a subframe on the MCG.

TP2:

(1) UE indicates capability for dynamic power sharing between LTE and NR for EN-DC, and

(2) UE does not indicate a capability tdm-Pattern-dualTx, and

(3) UE is configured with tdm-PatternConfig-r16.

TP2: If the UE indicates capability for dynamic power sharing between E-UTRA and NR for EN-DC and does not indicate a capability tdm-Pattern-dualTx in [16, TS 38.306], and is configured with tdm-PatternConfig-r16, the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission on a subframe on the MCG.

TP2 consists of the following two scenarios. Scenario#1 is covered by TP1 while Scenrio#2 is not. The key of determining whether TP1 or TP2 shall be adopted is to clarify the UE behaviour for Scenario#2. In fact, this issue could be boiled down to a more general question, i.e,, for a Rel-16 type1 UE not indicating either singleUL-Transmission or tdm-Pattern-dualTx but configured with tdm-PatternConfig-r16, whether it should be regarded as a singleTx UE or dualTx UE. The corresponding question for Rel-15 UE is, for a Rel-15 type1 UE not indicating singleUL-Transmission but configured with tdm-PatternConfig-r15, whether it should be regarded as a singleTx UE or dualTx UE.

Scenario#1: UE indicates singleUL-Transmission + DPS UE configured with tdm-PatternConfig-r16

Scenario#2: UE does not indicate singleUL-Transmission or tdm-Pattern-dualTx + DPS UE configured with tdm-PatternConfig-r16

The Rel-15 spec doesn’t specify the UE behaviour for type1 UE not indicating singleUL-Transmission but configured with tdm-PatternConfig-r15. That’s the reason causing different understandings on TP1 and TP2 in Rel-16.

Based on our understanding, for a Rel-15 type1 UE not indicating singleUL-Transmission but configured with tdm-PatternConfig-r15, it should be regarded as dualTx UE since the Rel-15 spec doesn’t specify any restriction on it. Following this philosophy, for a Rel-16 type1 UE not indicating either singleUL-Transmission or tdm-Pattern-dualTx but configured with tdm-PatternConfig-r16, it should be regarded as dualTx UE.

Proposal 1: For a Rel-16 type1 UE not indicating singleUL-Transmission or tdm-Pattern-dualTx but configured with tdm-PatternConfig-r16, it should be regarded as dualTx UE.

It is preferred to first clarify the corresponding UE behaviour before jumping into the detailed TP.

## UE behaviours of semi-staticULTransInAllSubframe

As introduced in [2], the UE behaviours of semi-staticULTransInAllSubframe (as the highlighted part below shown) has not been defined in the Rel-16 spec yet. Since it is related to the specific UE feature, the TP for introducing UE behaviour of semi-staticULTransInAllSubframe could be suspended after the outcome of UE feature discussion.

Agreements: For a UE configured with DL-reference DL/UL configuration in Rel-16 (including single Tx with LTE TDD PCell or LTE FDD PCell, and dual Tx cases): - For type 2 UE (i.e., UE without dynamic power sharing capability), any LTE UL transmissions should take place only in UL subframes designated for HARQ-ACK feedback. - For type 1 UE (i.e., UE with dynamic power sharing capability), - Confirm that any LTE UL transmissions scheduled/triggered by DCI can take place in UL subframes not designated for HARQ-ACK feedback. - FFS UE is not expected to transmit semi-statically configured LTE UL transmissions in the UL subframes other than those designated as UL in the DL-reference configuration if such transmission collide with NR UL transmissions. Agreements For the FFS part in the agreement above, - semi-statically configured LTE UL transmissions are allowed in all UL subframes. - Note: In case of collision, LTE transmission is prioritized - Note: this configuration is subject to UE capability

We provide the following TPs for introducing UE behaviour of semi-staticULTransInAllSubframe for further discussion. TP1 is to introduce the UE behaviour pertaining to UE capability semi-staticULTransInAllSubframe in TS38.213. TP2 is to introduce the UE behaviour pertaining to UE capability semi-staticULTransInAllSubframe in TS36.213. The same description exists in numerous paragraphs in TS36.213, thus the TP may also need to be copied to other corresponding paragraphs.

TP1: {38.213: 7.6.1	EN-DC}

If a UE is configured with , where is the linear value of , is the linear value of , and is the linear value of a configured maximum transmission power for EN-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the SCG as follows. - If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 or by tdm-PatternConfig-r16 in [13, TS 36.213]) - If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for EN-DC, the UE does not transmit in a slot on the SCG in FR1 when a corresponding subframe on the MCG is an UL subframe in the reference TDD configuration. - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC, and does not indicate a capability tdm-Pattern-dualTx in [16, TS 38.306], and is configured with tdm-PatternConfig-r16, the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission on a subframe on the MCG. - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC, and indicates a capability tdm-Pattern-dualTx and a capability semi-staticULTransInAllSubframe in [16, TS 38.306], and is configured with tdm-PatternConfig-r16, the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission that is not associated with a DCI on a subframe on the MCG.

TP2: {36.213: 5.1 Uplink power control}

For a UE configured with EN-DC, if the UE does not indicate a capability for dynamic power sharing (as specified in [17]) and if the UE is configured with subframeAssignment-r16 for a serving cell, the UE is not expected to transmit any uplink physical channel or signal in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r16 to the subframes denoted as uplink in the UL/DL configuration subframeAssignment-r16. For a UE configured with EN-DC, if the UE indicates a capability for dynamic power sharing and does not indicate a capability semi-staticULTransInAllSubframe (as specified in [17]) and if the UE is configured with subframeAssignment-r16 for a serving cell, the UE is not expected to transmit any uplink physical channel or signal without associated DCI in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r16 to the subframes denoted as uplink in the UL/DL configuration subframeAssignment-r16.

# Conclusion

In this contribution, we provide our analysis on Issue 2 from RAN1#100e and UE behaviour related to UE capability semi-staticULTransInAllSubframe.

Proposal 1: For a Rel-16 type1 UE not indicating singleUL-Transmission or tdm-Pattern-dualTx but configured with tdm-PatternConfig-r16, it should be regarded as dualTx UE.

TP1: {38.213: 7.6.1	EN-DC}

If a UE is configured with , where is the linear value of , is the linear value of , and is the linear value of a configured maximum transmission power for EN-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the SCG as follows. - If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 or by tdm-PatternConfig-r16 in [13, TS 36.213]) - If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for EN-DC, the UE does not transmit in a slot on the SCG in FR1 when a corresponding subframe on the MCG is an UL subframe in the reference TDD configuration. - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC, and does not indicate a capability tdm-Pattern-dualTx in [16, TS 38.306], and is configured with tdm-PatternConfig-r16, the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission on a subframe on the MCG. - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC, and indicates a capability tdm-Pattern-dualTx and a capability semi-staticULTransInAllSubframe in [16, TS 38.306], and is configured with tdm-PatternConfig-r16, the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission that is not associated with a DCI on a subframe on the MCG.

TP2: {36.213: 5.1 Uplink power control}

For a UE configured with EN-DC, if the UE does not indicate a capability for dynamic power sharing (as specified in [17]) and if the UE is configured with subframeAssignment-r16 for a serving cell, the UE is not expected to transmit any uplink physical channel or signal in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r16 to the subframes denoted as uplink in the UL/DL configuration subframeAssignment-r16. For a UE configured with EN-DC, if the UE indicates a capability for dynamic power sharing and does not indicate a capability semi-staticULTransInAllSubframe (as specified in [17]) and if the UE is configured with subframeAssignment-r16 for a serving cell, the UE is not expected to transmit any uplink physical channel or signal without associated DCI in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r16 to the subframes denoted as uplink in the UL/DL configuration subframeAssignment-r16.

# Reference

- R1-2001392, Outcome of email thread [100e-NR-LTE_NR_DC_CA_enh-SingleTx-02], Apple, RAN1#100e.

- R1-2000362, Remaining Issues on Single Tx Switched Uplink Solution for EN-DC, ZTE, RAN1#100e.