3GPP TSG RAN WG1 Meeting #100bis	R1-2001637

e-Meeting, April 20th – 30th, 2020

Title:	Draft reply LS on eMIMO RRC parameters

Response to:	R2-2001683

Release:	Rel-16

Work Item:	NR_eMIMO-Core

Source:	vivo [RAN WG1]

To:	RAN WG2

Cc:

# Contact Person:

# Name:

# E-mail Address:

# 1. Overall Description:

RAN1 thanks RAN2 for the LS R2-2001683 on eMIMO RRC parameters. On the five questions from RAN2, answers from RAN1 are as follows:

Question 1.

RAN2 would like to inform RAN1 about the above agreement and ask RAN1 to indicate if they find issues with the agreement.

Answer1:

From RAN1 point of view there is no issue on the following parameter ul-FullPowerTransmission in IE PUSCH-Config. RAN1 also understands that relevant changes to 38.213 is required.

ul-FullPowerTransmission-r16            ENUMERATED {fullpower, fullpowerMode1, fullpoweMode2}     OPTIONAL    -- Need R

Question 2. RAN2 would like to ask RAN1 to inform RAN2 whether the parameter BDFactor should be per cell (e.g. within IE ServingCellConfig) or per cell group (e.g. within IE PhysicalCellGroupConfig)?

Answer2: From RAN1 point of view the parameter BDFactor should be configured per cell.

Question 3. RAN2 asks RAN1 to inform RAN2 if repetition schemes 2a/2b/3 (fdmSchemeA, fdmSchemeB and tdmScheme) and scheme 4 (slotBased) are mutually exclusive in all UE configuration options, and to provide a reference which could be used in TS 38.331 for referencing about the configuration limitations for the repetition schemes.

Answer3:

RAN1 confirms currently endorsed RRC structure for repetition scheme 2a/2b/3 and scheme 4 which correctly reflects RAN1’s understanding. From RAN1 perspective, scheme 2a/2b/3 and scheme 4 are not necessarily mutually exclusive.

Question 4. RAN2 would like to ask RAN1 whether the intention is to activate per SRS resource set or per SRS resource. Further, if per SRS resource, whether RAN1 sees any issues in indicating spatial relation, potentially different, for more than one resource in one MAC CE in order to save overhead.

Answer4: RAN1 assumes the activation is per SRS resource, as for the activation of semi-persistent SRS resources. RAN1 sees no issue when multiple SRS resources with potentially different spatial relation are activated with one MAC CE.

Question 5. RAN2 asks RAN1 to provide values for maximum number of PUCCH resources in a PUCCH group and for maximum value of serving cells in per CC/BWP lists.

Answer5: RAN1 understands that the maximum number of PUCCH resources is 128 and the maximum value of serving cells in a CC list is 32.

# 2. Actions:

# To RAN WG2

# RAN1 respectfully requests RAN2 to take the above answers into account.

# 4. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting#101,	         25th - 29th May 2020 e-meeting

TSG-RAN WG1 Meeting#102,	         24h - 28th August 2020 Toulouse, France