3GPP TSG RAN WG1#100bis		R1-2001654

e-Meeting, April 20th – 30th, 2020

Source:	vivo

Title:	Remaining issues on HARQ operation for NR-U

Agenda Item:	7.2.2.2.3

Document for:	Discussion and Decision

- Introduction & Background

In RAN#82 meeting, the WID of the NR access to the unlicensed spectrum has been approved [1]. The following physical layer procedures have been identified as the objectives of the work item.

-	Physical layer procedure(s) including [RAN1, RAN2]:

- 	HARQ operation: NR HARQ feedback mechanisms are the baseline for NR-U operation with extensions in line with agreements during the study phase (NR-U TR section 7.2.1.3.3), including immediate transmission of HARQ A/N for the corresponding data in the same shared COT as well as transmission of HARQ A/N in a subsequent COT. Potentially support mechanisms to provide multiple and/or supplemental time and/or frequency domain transmission opportunities. (RAN1)

-	Scheduling multiple TTIs for PUSCH in-line with agreements from the study phase (TR 38.889, Section 7.2.1.3.3). (RAN1)

…

In RAN#82 meeting, the work item has been completed with some remaining issues. In this contribution we discuss the remaining issues on HARQ operation for NR-U Rel-16.

- Discussion

  - HARQ-ACK feedback

In NR-U Rel-16, enhancements for HARQ-ACK feedback have been introduced, mainly including enhancements for dynamic codebook, and introduction of a totally new codebook, i.e. one-shot feedback. In the following sections, we will discuss the remaining issues for these two items one by one.

- 2.1.1 Enhanced dynamic codebook

- Transmission of SPS PDSCH HARQ-ACK

In NR Rel-15, the network ensures SPS-Config is configured for at most one cell in a cell group. In NR-U Rel-16 this keeps valid when no enhancement about DL SPS is considered.

In TS38.213 v16.1.0 [3], there are some descriptions as follows:

9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

……

Generate first HARQ-ACK information for PUCCH transmission occasion i(g) in a slot, as described in Clause 9.1.3.1, where

……

If hg+1mod2g=∅ or hg+1mod2g=h(g+1mod2), generate second HARQ-ACK information for PUCCH transmission occasion i(g+1mod2) in a slot, as described in Clause 9.1.3.1, where

……

If hg+1mod2g≠∅ and hg+1mod2g≠h(g+1mod2), generate second HARQ-ACK information, as described in Clause 9.1.3.1, by setting VC-DAI,c,mDL=0 for all c and all m and, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, setting Vtemp2=VDAIg+1mod2.

……

In TS38.213 v16.1.0 [3] clause 9.1.3.1, there are some descriptions about SPS PDSCH HARQ-ACK as follows:

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

……

If the UE transmits HARQ-ACK information in a PUCCH in slot  and for any PUCCH format, the UE determines the , for a total number of  HARQ-ACK information bits, according to the following pseudo-code:

……

Set

while

if a single SPS PDSCH reception is activated for a UE and the UE is configured to receive SPS PDSCH in a slot  for serving cell , where  is the PDSCH-to-HARQ-feedback timing value for SPS PDSCH on serving cell

= HARQ-ACK information bit associated with the SPS PDSCH reception

end if

;

end while

……

From the aforementioned descriptions it can be seen that none, one or both of the first HARQ-ACK information and the second HARQ-ACK information can contain a HARQ-ACK bit for SPS PDSCH reception, depending on the relative positions between PUCCH transmission occasion i(g) or i(g+1mod2) and SPS PDSCH receptions. A typical case when both of the first HARQ-ACK information and the second HARQ-ACK information contain a HARQ-ACK bit for respective SPS PDSCH reception is shown in Figure 1(a), in which HARQ-ACK feedback for some SPS PDSCHs could also have retransmission opportunities, e.g. U1 in Figure 1(a). When PUCCH transmission occasion i(g) and i(g+1mod2) are just in the same slot, and the transmitted HARQ-ACK codebook contains both the first HARQ-ACK information and the second HARQ-ACK information, HARQ-ACK corresponding to SPS PDSCH in a slot ig-K1,c or i(g+1mod2)-K1,c will be contained in the first HARQ-ACK information and the second HARQ-ACK information at the same time, according to the pseudo-code listed above, resulting in redundant feedback or information confusion, which is illustrated in Figure 1(b).

Figure 1 Transmission of SPS PDSCH HARQ-ACK both in first and second HARQ-ACK information

To avoid information redundancy or confusion for SPS PDSCH HARQ-ACK, a straightforward way is that HARQ-ACK for SPS PDSCH, if any, is always placed at the end of transmitted HARQ-ACK codebook, no matter whether the transmitted HARQ-ACK codebook contains only the first HARQ-ACK information, or both the first HARQ-ACK information and the second HARQ-ACK information. When it is assumed that SPS-Config is configured for at most one cell in a cell group, at most a HARQ-ACK bit for SPS PDSCH is appended at the end of the transmitted HARQ-ACK codebook.

Proposal 1: HARQ-ACK for SPS PDSCH, if any, is always placed at the end of enhanced HARQ-ACK codebook to be transmitted, no matter whether the transmitted HARQ-ACK codebook contains only the first HARQ-ACK information, or both the first HARQ-ACK information and the second HARQ-ACK information.

Regarding number of PDSCH groups involved in the transmitted HARQ-ACK codebook, there are some corresponding descriptions in TS38.213 v16.1.0 [3] as follows:

9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

……

Set q to the value of a number of requested PDSCH group(s) field, if any

……

If q=0, the UE

includes only the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

elseif q=1

if g = 1

appends the first HARQ-ACK information to the second HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

else

append the second HARQ-ACK information to the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

end if

end if

……

It can be seen that whether the second HARQ-ACK information is triggered to be transmitted along with the first HARQ-ACK information is only based on q, i.e. the value of a number of requested PDSCH group(s) field in a DCI format. But there may be a case that the PUCCH transmission occasion i(g+1mod2) for the second HARQ-ACK information and the PUCCH transmission occasion i(g) for the first HARQ-ACK information are just in the same slot, which leads to the first HARQ-ACK information and the second HARQ-ACK information being multiplexed in a same PUCCH transmission occasion irrespective of the value of q, as illustrated in Figure 2. In this case, the first HARQ-ACK information and the second HARQ-ACK information should also be multiplexed in an ascending order of group index. So it should be captured in TS38.213 that in any case when the first HARQ-ACK information and the second HARQ-ACK information are multiplexed in a same PUCCH transmission occasion, they should be placed in an ascending order of group index.

Figure 2 HARQ-ACK information for both groups reported in a same PUCCH transmission

Proposal 2: It should be captured in TS38.213 that in any case when the first HARQ-ACK information and the second HARQ-ACK information are multiplexed in a same PUCCH transmission occasion, they should be placed in an ascending order of group index.

A corresponding TP can be seen in section 3.1.

- T-DAI related issues

In TS38.213 v16.1.0 [3], there are some descriptions as follows:

9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

……

If hg+1mod2g=∅ or hg+1mod2g=h(g+1mod2), generate second HARQ-ACK information for PUCCH transmission occasion i(g+1mod2) in a slot, as described in Clause 9.1.3.1, where

……

-	if VDAIg+1mod2≠∅, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, set Vtemp2=VDAIg+1mod2 for both sub-codebooks, if any.

……

According to these descriptions, when VDAIg+1mod2≠∅, if the second HARQ-ACK codebook does not contain two sub-codebooks, i.e., only TB-based HARQ-ACK should be fed back, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, set Vtemp2=VDAIg+1mod2 for this single codebook. Otherwise, if the second HARQ-ACK codebook contains two sub-codebooks, i.e., both TB-based HARQ-ACK and CBG-based HARQ-ACK should be fed back at the same time, set Vtemp2=VDAIg+1mod2 for both sub-codebooks after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1. However, it will bring severe limitation for PDSCH scheduling for each sub-codebook, since these two sub-codebooks should match a same total DAI. In addition, there has been no agreement achieved regarding this issue during NR-U Rel-16 WI. A straightforward way to avoid this limitation is that when the RRC parameter NFI-TotalDAI-Included-r16 = enable and two sub-codebooks may be applied, i.e., PDSCH-CodeBlockGroupTransmission is provided at least for a serving cell, two separate total DAI fields for PDSCH group g+1mod2 will be indicated in a non-fallback DCI format, one for the TB-based sub-codebook and the other for the CBG-based sub-codebook, when applicable. Another alternative way is that when two sub-codebooks may be applied, the RRC parameter NFI-TotalDAI-Included-r16 is always not configured to avoid above scheduling limitation or indication overhead.

Proposal 3: It should be determined how to indicate and apply VDAIg+1mod2 when the second HARQ-ACK codebook contains two sub-codebooks.

- NFI related issues

In TS38.213 v16.1.0 [3], there are some descriptions as follows:

9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

……

If hg+1mod2g≠∅ and hg+1mod2g≠h(g+1mod2), generate second HARQ-ACK information, as described in Clause 9.1.3.1, by setting VC-DAI,c,mDL=0 for all c and all m and, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, setting Vtemp2=VDAIg+1mod2.

……

It could be understood that when a DCI format scheduling PDSCH group g indicates an NFI value NFIx for PDSCH group g+1mod2 other than an NFI value NFIy indicated by a latest DCI format scheduling PDSCH group g+1mod2 for exactly the scheduled group, it is mostly probable that after the latest DCI format scheduling PDSCH group g+1mod2, all DCI formats scheduling PDSCH group g+1mod2 with an NFI value NFIx have been miss-detected by UE. So in the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, no VC-DAI,c,mDL needs to be actually used, since there is no PDCCH detected for any PDCCH monitoring occasion m and any serving cell c. Thus the operation that setting  VC-DAI,c,mDL=0 for all c and all m in aforementioned descriptions in TS38.213 is not needed. Considering that no agreement during NR-U Rel-16 WI has been achieved to deal with this case, it should be clarified that related operation captured in above descriptions is reasonable with common understanding.

Proposal 4: The corresponding operation when a DCI format scheduling PDSCH group g indicates an NFI value for PDSCH group g+1mod2 other than an NFI value indicated by a latest DCI format scheduling PDSCH group g+1mod2 for exactly the scheduled group should be clarified.

- 2.1.2 One-shot feedback

- Codebook construction for spatial bundling

In TS38.213 v16.1.0 [3], there are some descriptions about type-3 HARQ-ACK codebook determination as follows:

9.1.4	Type-3 HARQ-ACK codebook determination

……

Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, set NTB,cDL=1

Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in Clause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG-r16 is provided; else, set NHARQ-ACK,cCBG/TB,max=0

Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI-r16 is provided; else set NDIHARQ=1

……

if NDIHARQ=0

if NHARQ-ACK,cCBG/TB,max>0

while t<NTB,cDL

while g<NHARQ-ACK,cCBG/TB,max

= HARQ-ACK information bit for CBG g of TB t for HARQ process number h of serving cell c

j=j+1

g=g+1

end while

= NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit(s) for TB t for HARQ process number h on serving cell c, if any; else,

g=0

j=j+1

t=t+1

end while

else

while t<NTB,cDL

= HARQ-ACK information bit for TB t for HARQ process h of serving cell c

j=j+1

= NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit(s) for TB t for HARQ process number h on serving cell c, if any; else,

j=j+1

t=t+1

end while

end if

t=0

else

……

end if

……

From above part of pseudo-code it can be seen that when spatial bundling is configured, NTB,cDL=1, and only HARQ-ACK information for the first TB, i.e. TB 0 is reported, so HARQ-ACK information for the second TB will be lost totally, which is not exactly the intention of spatial bundling.

A feasible way is that when CBG-based HARQ-ACK reporting is enabled for a serving cell, i.e.NHARQ-ACK,cCBG/TB,max>0, spatial bundling is always disabled regardless of spatial bundling related configuration, which is aligned with NR Rel-15 behavior, since it is more important to report detailed HARQ-ACK information than to reduce signaling overhead. When CBG-based HARQ-ACK reporting is disabled for a serving cell, i.e.NHARQ-ACK,cCBG/TB,max=0, spatial bundling is enabled based on spatial bundling related configuration, i.e., whether harq-ACK-SpatialBundlingPUCCH is provided for the cell group of this serving cell. When spatial bundling is enabled, the single HARQ-ACK value actually reported for HARQ process h of serving cell c is set to binary AND operation of the HARQ-ACK information bits corresponding to first and second TBs for HARQ process h of serving cell c.

Proposal 5: When CBG-based HARQ-ACK reporting is enabled for a serving cell, spatial bundling is always disabled regardless of spatial bundling related configuration; otherwise spatial bundling is enabled based on whether harq-ACK-SpatialBundlingPUCCH is provided for the cell group of this serving cell.

Proposal 6: When spatial bundling is enabled, the single HARQ-ACK value actually reported for HARQ process h of serving cell c is set to binary AND operation of the HARQ-ACK information bits corresponding to first and second TBs for HARQ process h of serving cell c.

A corresponding TP can be seen in section 3.1.

- Codebook construction when reporting NDI

When NDI is configured to be part of one-shot feedback, i.e. NDIHARQ=0, if spatial bundling is not enabled actually, NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit or default value 0 for TB t for HARQ process number h on serving cell c will be reported right after HARQ-ACK information bit for TB t for HARQ process h of serving cell c. But if spatial bundling is enabled actually, how to place the result of binary AND operation of the HARQ-ACK information bits corresponding to two TBs and the NDI value for each TB has no corresponding agreement so far. A straightforward way is that the result of binary AND operation is followed by the NDI value for each TB in the order of increasing TB index.

Proposal 7: When NDI is configured to be part of one-shot feedback and spatial bundling is enabled actually, for a HARQ process of a serving cell, the result of binary AND operation of the HARQ-ACK information bits corresponding to two TBs is followed by the NDI value for each TB in the order of increasing TB index.

A corresponding TP can be seen in section 3.1.

- FDRA value for indicating no scheduled PDSCH

In RAN1#99 meeting [2] following agreement has been achieved with a working assumption:

Agreement:

If a UE is configured to monitor feedback request for one-shot HARQ-ACK codebook feedback and the feedback is requested in DL DCI 1_1

- This DL DCI can either schedule or not schedule a PDSCH

  - Working assumption: One value of the frequency domain resource assignment field indicates that this DCI does not schedule a PDSCH

  - If the DL DCI does not schedule a PDSCH, the HARQ process ID and NDI fields are ignored by the UE

- If UE is triggered to report both one-shot and other HARQ-ACK feedback in the same slot, the UE reports only the one-shot feedback.

With respect to the specific value of frequency domain resource assignment filed, the values adopted in downlink SPS release PDCCH validation could be considered, i.e. setting to all '0's for FDRA Type 0 and setting to all '1's for FDRA Type 1. To distinguish these usages, when the UE is provided pdsch-HARQ-ACK-OneShotFeedback-r16, and a downlink DCI including a one-shot HARQ-ACK request field with value 1 is detected with FDRA field set to aforementioned specific value, the DCI can be regarded as triggering one-shot HARQ-ACK feedback with no scheduled PDSCH.

Proposal 8: When UE is provided pdsch-HARQ-ACK-OneShotFeedback-r16, and a downlink DCI including a One-shot HARQ-ACK request field with value 1 is detected with FDRA field set to all '0's for FDRA Type 0 and all '1's for FDRA Type 1, the DCI is regarded as triggering one-shot HARQ-ACK feedback with no scheduled PDSCH.

- HARQ-ACK feedback for SPS PDSCH release

There is a case that one-shot feedback and HARQ-ACK feedback for a SPS PDSCH release may happen to exist in a same PUCCH slot. This case can be regarded as an error case, but it will introduce additional restrictions for scheduling.

To ensure the HARQ-ACK for SPS PDSCH release, multiplexing HARQ-ACK feedback for a SPS PDSCH release in one-shot feedback needs to be supported. The HARQ-ACK feedback for SPS PDSCH release can be appended to the one-shot feedback, which may increase the codebook size for one-shot feedback.

Alternatively, a SPS PDSCH release can be mapped to a HARQ process in the one-shot feedback, in which the HARQ-ACK bit(s) for the HARQ process corresponds to the SPS PDSCH release. The HARQ process corresponding to the SPS PDSCH release can be determined based on the HARQ process ID derivation equation in TS38.321, where the SPS PDSCH location for the derivation can be determined based on the following options:

- Option 1: A SPS PDSCH location in time domain explicitly indicated by the SPS release DCI, e.g. by TDRA field in the DCI, or as well as by the slot/symbol where the SPS release DCI is detected.

- Option 2: A SPS PDSCH location in time domain implicitly derived according to the SPS configuration to which the SPS release DCI corresponds, e.g. the next SPS PDSCH occasion corresponding to the SPS configuration after the reception of the SPS release DCI.

Figure 3 Multiplexing of one-shot feedback and HARQ-ACK for SPS PDSCH release

Proposal 9: Support HARQ-ACK feedback for SPS PDSCH release in one-shot HARQ-ACK codebook. HARQ-ACK corresponding to a SPS PDSCH release is mapped to a HARQ process in the one-shot feedback, where the HARQ process is determined based on the HARQ process ID derivation for SPS PDSCH in TS38.321 and the SPS PDSCH location for the derivation is determined based on the following options:

- Option 1: A SPS PDSCH location in time domain explicitly indicated by the SPS release DCI, e.g. by TDRA field in the DCI, or as well as by the slot/symbol where the SPS release DCI is detected.

- Option 2: A SPS PDSCH location in time domain implicitly derived according to the SPS configuration to which the SPS release DCI corresponds, e.g. the next SPS PDSCH occasion corresponding to the SPS configuration after the reception of the SPS release DCI.

- Others

During last RAN1#100 meeting, how to process HARQ-ACK in one-shot feedback for a PDSCH without sufficient processing time is extensively discussed without a common understanding at the end. There remains an FFS for cases where the UE has not yet obtained HARQ-ACK information for a TB corresponding to a scheduled PDSCH reception. To keep that the network is always in control of UE’s behavior, a simple way is that the UE is always report HARQ-ACK for PDSCHs with sufficient processing time. The UE is not expected to be required to report HARQ-ACK for a HARQ process which has been scheduled with a PDSCH without sufficient processing time when reporting one-shot HARQ-ACK codebook.

Proposal 10: UE is not expected to be required to report HARQ-ACK for a HARQ process which has been scheduled with a PDSCH without sufficient processing time when reporting one-shot HARQ-ACK codebook.

- Text proposals

  - Text proposals for TS38.213

For TS38.213 v16.1.0 [3], text proposals based on aforementioned proposals are collected as below.

The TP for proposal 2 is in the clause 9.1.3.3 in TS38.213, and the combined TP for proposal 5, 6 and 7 is in the clause 9.1.4 in TS38.213, with TP for proposal 5 in blue, TP for proposal 6 in green, and TP for proposal 7 in red. Other proposals have no corresponding TPs.

-------------------------------------------------------Start text proposal-----------------------------------------------------------

#### 9.1.3.3	Type-2 HARQ-ACK codebook grouping and HARQ-ACK retransmission

< Unchanged part omitted >

For any case that HARQ-ACK information for both PDSCH groups is multiplexed in a PUCCH transmission or PUSCH transmission, HARQ-ACK information for g=1 is appended to that for g=0.

If a UE detects DCI formats with respective PDSCH-to-HARQ_feedback timing field values indicating a same PUCCH transmission occasion and none of the DCI formats that the UE detects after a last PUCCH transmission occasion for g=0 includes a New_Feedback indicator field for g=0, and at least one of the DCI formats is DCI format 1_0, the UE generates HARQ-ACK information only for PDSCH receptions scheduled by detections of DCI format 1_0, as described in Clause 9.1.3.1 or 9.1.3.2 for multiplexing in the PUCCH transmission occasion.

For PUCCH transmission occasion i(g), the UE determines a PUCCH or a PUSCH transmission to multiplex the HARQ-ACK information according to the procedures in Clause 9.2.5.

### 9.1.4	Type-3 HARQ-ACK codebook determination

If a UE is provided pdsch-HARQ-ACK-OneShotFeedback-r16, the UE determines a Type-3 HARQ-ACK codebook according to the following procedure.

Set NcellsDL to the number of serving cells

Set NHARQ,cDL to the value of nrofHARQ-ProcessesForPDSCH for serving cell c, if provided; else, set NHARQ,cDL=8

Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, set NTB,cDL=1

Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in Clause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG-r16 is provided; else, set NHARQ-ACK,cCBG/TB,max=0

Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI-r16 is provided; else set NDIHARQ=1

Set c=0 – serving cell index

Set h=0 – HARQ process number

Set t=0 – TB index

Set g=0 – CBG index

Set j=0

while c<NcellsDL

while h<NHARQ,cDL

if NDIHARQ=0

if NHARQ-ACK,cCBG/TB,max>0

while t<NTB,cDL

while g<NHARQ-ACK,cCBG/TB,max

= HARQ-ACK information bit for CBG g of TB t for HARQ process number h of serving cell c

j=j+1

g=g+1

end while

= NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit(s) for TB t for HARQ process number h on serving cell c, if any; else,

g=0

j=j+1

t=t+1

end while

else

if harq-ACK-SpatialBundlingPUCCH is not provided

while t<NTB,cDL

= HARQ-ACK information bit for TB t for HARQ process h of serving cell c

j=j+1

= NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit(s) for TB t for HARQ process number h on serving cell c, if any; else,

j=j+1

t=t+1

end while

else

if NTB,cDL = 2

= binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks for HARQ process h of serving cell c - if the UE receives one transport block, the UE assumes ACK for the second transport block;

else

= HARQ-ACK information bit for TB 0 for HARQ process h of serving cell c

end if

j=j+1

while t<NTB,cDL

= NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit(s) for TB t for HARQ process number h on serving cell c, if any; else,

j=j+1

t=t+1

end while

end if

end if

t=0

else

if NHARQ-ACK,cCBG/TB,max>0

while t<NTB,cDL

if UE has reported HARQ-ACK information for TB t for HARQ process number h on serving cell c, and has not subsequently detected a DCI format scheduling a PDSCH reception, or received a SPS PDSCH, with TB t for HARQ process number h on serving cell c

while g<NHARQ-ACK,cCBG/TB,max

j=j+1

g=g+1

end while

end if

if UE has obtained HARQ-ACK information for TB t for HARQ process number h on serving cell c corresponding to a PDSCH reception and has not reported the HARQ-ACK information corresponding to the PDSCH reception

while g<NHARQ-ACK,cCBG/TB,max

= HARQ-ACK information bit for CBG g of TB t for HARQ process number h of serving cell c

j=j+1

g=g+1

end while

end if

g=0

t=t+1

end while

else

if harq-ACK-SpatialBundlingPUCCH is not provided

while t<NTB,cDL

if UE has reported HARQ-ACK information for TB t for HARQ process number h on serving cell c and has not subsequently detected a DCI format scheduling a PDSCH reception, or received a SPS PDSCH, with TB t for HARQ process number h on serving cell c

j=j+1

t=t+1

end if

if UE has obtained HARQ-ACK information for TB t for HARQ process number h on serving cell c corresponding to a PDSCH reception and has not reported the HARQ-ACK information corresponding to the PDSCH reception

= HARQ-ACK information bit for TB t for HARQ process h of serving cell c

j=j+1

t=t+1

end if

end while

else

if UE has reported HARQ-ACK information for TB t for HARQ process number h on serving cell c and has not subsequently detected a DCI format scheduling a PDSCH reception, or received a SPS PDSCH, with TB t for HARQ process number h on serving cell c

j=j+1

end if

if UE has obtained HARQ-ACK information for TB t for HARQ process number h on serving cell c corresponding to a PDSCH reception and has not reported the HARQ-ACK information corresponding to the PDSCH reception

if NTB,cDL = 2

= binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks for HARQ process h of serving cell c - if the UE receives one transport block, the UE assumes ACK for the second transport block;

else

= HARQ-ACK information bit for TB 0 for HARQ process h of serving cell c

end if

j=j+1

end if

end if

end if

t=0

end if

h=h+1

end while

h=0

c=c+1

end while

< Unchanged part omitted >

--------------------------------------------------------End text proposal----------------------------------------------------------

Proposal 11: Adopt the text proposals listed in section 3.1 for Rel-16 TS38.213.

- Conclusion

In this contribution, some remaining issues on NR-U Rel-16 HARQ operation have been discussed, and the proposals made are summarized below:

Proposal 1: HARQ-ACK for SPS PDSCH, if any, is always placed at the end of enhanced HARQ-ACK codebook to be transmitted, no matter whether the transmitted HARQ-ACK codebook contains only the first HARQ-ACK information, or both the first HARQ-ACK information and the second HARQ-ACK information.

Proposal 2: It should be captured in TS38.213 that in any case when the first HARQ-ACK information and the second HARQ-ACK information are multiplexed in a same PUCCH transmission occasion, they should be placed in an ascending order of group index.

Proposal 3: It should be determined how to indicate and apply VDAIg+1mod2 when the second HARQ-ACK codebook contains two sub-codebooks.

Proposal 4: The corresponding operation when a DCI format scheduling PDSCH group g indicates an NFI value for PDSCH group g+1mod2 other than an NFI value indicated by a latest DCI format scheduling PDSCH group g+1mod2 for exactly the scheduled group should be clarified.

Proposal 5: When CBG-based HARQ-ACK reporting is enabled for a serving cell, spatial bundling is always disabled regardless of spatial bundling related configuration; otherwise spatial bundling is enabled based on whether harq-ACK-SpatialBundlingPUCCH is provided for the cell group of this serving cell.

Proposal 6: When spatial bundling is enabled, the single HARQ-ACK value actually reported for HARQ process h of serving cell c is set to binary AND operation of the HARQ-ACK information bits corresponding to first and second TBs for HARQ process h of serving cell c.

Proposal 7: When NDI is configured to be part of one-shot feedback and spatial bundling is enabled actually, for a HARQ process of a serving cell, the result of binary AND operation of the HARQ-ACK information bits corresponding to two TBs is followed by the NDI value for each TB in the order of increasing TB index.

Proposal 8: When UE is provided pdsch-HARQ-ACK-OneShotFeedback-r16, and a downlink DCI including a One-shot HARQ-ACK request field with value 1 is detected with FDRA field set to all '0's for FDRA Type 0 and all '1's for FDRA Type 1, the DCI is regarded as triggering one-shot HARQ-ACK feedback with no scheduled PDSCH.

Proposal 9: Support HARQ-ACK feedback for SPS PDSCH release in one-shot HARQ-ACK codebook. HARQ-ACK corresponding to a SPS PDSCH release is mapped to a HARQ process in the one-shot feedback, where the HARQ process is determined based on the HARQ process ID derivation for SPS PDSCH in TS38.321 and the SPS PDSCH location for the derivation is determined based on the following options:

- Option 1: A SPS PDSCH location in time domain explicitly indicated by the SPS release DCI, e.g. by TDRA field in the DCI, or as well as by the slot/symbol where the SPS release DCI is detected.

- Option 2: A SPS PDSCH location in time domain implicitly derived according to the SPS configuration to which the SPS release DCI corresponds, e.g. the next SPS PDSCH occasion corresponding to the SPS configuration after the reception of the SPS release DCI.

Proposal 10: UE is not expected to be required to report HARQ-ACK for a HARQ process which has been scheduled with a PDSCH without sufficient processing time when reporting one-shot HARQ-ACK codebook.

Proposal 11: Adopt the text proposals listed in section 3.1 for Rel-16 TS38.213.

- References

- Chairman’s Notes, RAN#82, Sorrento, Italy, Dec. 10-13, 2018.

- Chairman's Notes, RAN1#99, Reno, USA, Nov. 18-22, 2019

- TS 38.213, Physical layer procedures for control, v16.1.0, Mar. 2020