3GPP TSG RAN WG1 Meeting #100bis	 R1-2001715

Electronic Meeting, April 20-30, 2020

Source: 	ZTE, Sanechips

Title: 	Discussion on UE Features for NR-U

Agenda item:	7.2.11.2

Document for:	Discussion/Decision

# Introduction

After RAN1#99 meeting, it has been declared all the Rel-16 RAN1-led WIs as finished in RAN1 perspective. Since then, there has been two rounds of email discussion for the L1 UE features [1]. In this contribution, we provide some further views based on the latest UE feature list for Rel-16 NR-U. The suggested revisions are listed in the Appendix.

# Views on the UE features for NR-U

## Structure of UE features for NR-U

In previous email discussion as well as the RAN Plenary discussion, there has been discussed about the structure of the feature groups.

  - There are at least two possible approaches below to define the set of feature groups for a purpose.

    - Approach 1: A basic feature group(s), which is a set of components that are viewed necessary to provide a minimum level of support for the feature. Defining a basic feature group(s) is not always possible or necessary for a given feature.

    - Approach 2: A set(s) of feature groups necessary to be supported for the purpose is defined somewhere in specification(s).

Approach 1 is the ordinary way as we adopted in Rel-15, which should be also applied for NR-U features. For NR-U, depending on different use scenarios, there could be multiple basic feature groups defined for UE to support LBE mode (DL+UL), FBE mode (DL+UL), LBE mode (DL only) and FBE mode (DL only), respectively. The different operation modes can be regarded as different sub-features for the NR-U feature.

Each basic feature group includes multiple components that are essential for the UE to support the sub-feature. Other optional feature groups can use one or multiple of the above basic feature groups as prerequisite. One thing could be further discussed is that whether or not to include more components to the basic feature group, for example the supported enhancements on HARQ, configured grant, SRS and CORESET/SS. Those enhancements are used to compensate the potential loss of LBT failure in terms of efficiency and reliability, it may be useful to make these function as mandatory, at least for LBE mode.

Proposal 1:

- Multiple basic feature groups can be defined for NR-U, corresponding to different operation modes.

  - Further discuss whether the enhancements on CORESET/SS, SRS, HARQ, CG can be merged into the basic feature group(s).

There has been various enhancements made in NR-U WI on different aspects based on Rel-15 NR design. For the enhancements on the same aspect, it may not be a good way to split the features into too many small pieces and make all of them as optional. It would be hard for network to implement and utilize a meaningful Rel-16 functionality if different UEs support drastically different combinations of feature groups for one aspect.

For example for the interlace structure, currently there are 4 optional feature groups defined for PUSCH and each of the PUCCH formats respectively, which implies one UE may support interlaced PUSCH but not support interlaced PUCCH, or vice versa. This is not necessary, as in the RRC signalling there is only one parameter to just indicate whether the interlaced structure is enabled or not. Therefore, it would be better to merge them into one feature group. Similar principle can be applied to the enhancements on HARQ and configured grant.

Proposal 2:

- To avoid implementation complexity, the enhancements on the same aspect should be combined into one feature group, including the following:

  - Interlaced structure: combine 10-3, 10-3a, 10-3b, and 10-3c

  - HARQ enhancement: combine 10-14, 10-15, 10-16, 10-16a, and 10-17

  - Configured grant: combine 10-18 and 10-28

## Applicability of some features for NR licensed spectrum

Another remaining issue to be discussed is about the application range of the enhancements made in NR-U WI, i.e. whether some of them can be used for NR licensed spectrum.

We think by default the enhancements made in NR-U WI shall be limited to the shared spectrum, as the WG discussions in terms of the selection of schemes are mainly based on the assumptions specific to NR-U use scenarios. On top of that, we can further consider whether some of them can be applied to licensed spectrum, with the following principles:

1) if it is indeed beneficial for NR licensed spectrum; and

2) if it is not conflict with the enhancement made in Rel-16 NR on the same aspect.

In particular, our preference on the application range for various enhanced aspects are as follows:

- Interlaced mapping including FG 10-3/3a/3b/3c.

The interlaced mapping is proposed to simplify the signalling of the FDRA of interlace in order to satisfy the OCB requirement from ESTI. There is no such OCB requirement for NR licensed spectrum, so we do not think there is a need to extend the application range for this feature.

- CG enhancement: FG 10-18 and 10-24.

The enhancement on the configured grant, for example the CG-UCI and retransmission on CG resources have been discussed in Rel-15, but not agreed at that moment. And then in Rel-16 URLLC WI, the configured grant has been enhanced with different approaches. If the enhancements in NR-U are applied to licensed spectrum, there will be two ways to do configured grant which are not compatible. It is not clear on the configurations and UE behavior for the operation of configured grant. So we think the CG enhancement shall not be applied to licensed spectrum at least in Rel-16. We can further discuss how to optimize the URLLC for NR-U in Rel-17.

- TypeB PDSCH length & SRS starting position: including FG 10-8 and FG 10-11

- CORESET/SS: including FG 10-9/9a/9b/9c, 10-20/20a.

- HARQ enhancements: including FG 10-14 ~ 10-17.

In general, the above enhancements on PDSCH, SRS, CORESET/SS, and HARQ could be beneficial to licensed spectrum in terms of enhanced flexibility and reliability. On the other hand, it may introduce implementation complexity for NR UEs. Probably they can be considered as optional features to be applied to NR licensed spectrum.

Proposal 3:

- The interlace structure and enhancement on configured grant shall not be applied to NR licensed spectrum.

- Enhancements on TypeB PDSCH length, SRS starting position, HARQ and CORESET/SS can be considered to be applied to NR licensed spectrum as optional features.

# Conclusion

In this contribution, the structure of NR-U UE features has been discussed with the following proposals.

Proposal 1:

- Multiple basic feature groups can be defined for NR-U, corresponding to different operation modes.

  - Further discuss whether the enhancements on CORESET/SS, SRS, HARQ, CG can be merged into the basic feature group(s).

Proposal 2:

- To avoid implementation complexity, the enhancements on the same aspect should be combined into one feature group, including the following:

  - Interlaced structure: combine 10-3, 10-3a, 10-3b, and 10-3c

  - HARQ enhancement: combine 10-14, 10-15, 10-16, 10-16a, and 10-17

  - Configured grant: combine 10-18 and 10-28

Proposal 3:

- The interlace structure and enhancement on configured grant shall not be applied to NR licensed spectrum.

- Enhancements on TypeB PDSCH length, SRS starting position, HARQ and CORESET/SS can be considered to be applied to NR licensed spectrum as optional features.

# Reference

R1-2001484, RAN1 UE features list for Rel-16 NR after RAN1#100-E

# Appendix

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-1 | UE stand-alone (DL and UL) operation in shared spectrum under dynamic channel access mode | 1. Type 1 channel access 2. Type 2A channel access 3. Type 2B channel access (FFS if move this to separate feature) 4. Type 2C channel access 5. 20MHz LBT bandwidth 6. Contention window adjustment 7. CP extension up to 1 symbol for PUSCH/PUCCH transmission 8. SSB/MIB/RMSI reception with Q 9. SSB RRM with Q in DMTC 10. SSB-RLM with Q in DMTC window 11. Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0 |  |  | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band with both DL and UL transmission support under dynamic channel access | Optional with capability signalling |
|  | 10-1a | UE DL only operation in shared spectrum under dynamic channel access mode | 1. SSB RRM with Q in DMTC | 6-5 |  | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band with DL only operation | Optional with capability signalling |
|  | 10-2 | UE stand-alone (DL and UL) operation in shared spectrum under semi-static channel access mode | 1. Type 2C channel access 2. Single sensing slot of 9us channel access 3. 20MHz LBT bandwidth 4. SSB/MIB/RMSI reception with Q 5. SSB RRM with Q in DMTC 6. SSB-RLM with Q in DMTC window 7. Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0 8. Support fixed frame period of 5ms and 10ms |  |  | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band. Support of channel access mechanism for FBE operation, including fixed frame period, Cat 2 LBT, Cat 1 LBT | Optional with capability signalling |
|  | 10-2a | UE DL only operation in shared spectrum under semi-static channel access mode | 1. SSB RRM with Q in DMTC 2. Support fixed frame period of 5ms and 10ms | 6-5 |  | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band with DL only operation | Optional with capability signalling |
|  | 10-2b | UE stand-alone (DL and UL) operation in shared spectrum under semi-static channel access mode | 1. Support fixed frame periods shorter than 5ms | 10-2 or 10-2a |  | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-3 | PRB interlace mapping for PUSCH | 1. PRB interlace frequency domain resource allocation for PUSCH 2. PRB interlace frequency domain resource allocation for PUCCH format 0 and format 1 3. PRB interlace frequency domain resource allocation for PUCCH format 2 4. PRB interlace frequency domain resource allocation for PUCCH format 3 | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | Support of PRB interlace PUSCH | Optional with capability signalling |
|  |  |  |  |  |  |  |  |  |  | N/A |  | Support of PRB interlace PUCCH format 0/1 | Optional with capability signalling |
|  |  |  |  |  |  |  |  |  |  | N/A |  | Support of PRB interlace PUCCH format 2 | Optional with capability signalling |
|  |  |  |  |  |  |  |  |  |  | N/A |  | Support of PRB interlace PUCCH format 3 | Optional with capability signalling |
|  | 10-7 | LBT bandwidth size of 10MHz | FFS the components | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-8 | Type B PDSCH length | Length 3, 5, 6, 8, 11, 12, 13 FFS the capability are separate for each length or some groups are formed to signal the capability together | Need discussion for licensed use | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A |  | Support of additional length (other than 2/4/7) for type B PDSCH. Note length 9/10 are already covered by 14-3 | Optional with capability signalling |
|  | 10-9 | Search space set group switching with explicit DCI 2_0 bit field trigger | 1. Two groups of search space sets 2. Monitor DCI 2_0 with a search space set switching field 3. Support a timer to switch back to original search space set group 4. Monitor DCI 2_0 for channel occupancy time and use the end of channel occupancy time to switch back to the original search space set group | 10-1, 10-1a, 10-2, or 10-2a Need discussion for licensed use | Yes | N/A |  | FFS: Per UE or per band or per BC | N/A | N/A |  | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
|  | 10-9a | Search space set group switching with implicit PDCCH decoding with DCI 2_0 monitoring | 1. Two groups of search space sets 2. Support switching the search space set group with PDCCH decoding in group 1 3. Support a timer to switch back to original search space set group 4. Monitor DCI 2_0 for channel occupancy time and use the end of channel occupancy time to switch back to the original search space set group | 10-1, 10-1a, 10-2, or 10-2a Need discussion for licensed use | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A |  | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
|  | 10-9b | Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring | 1. Two groups of search space sets 2. Support switching the search space set group with PDCCH decoding in group 1 3. Support a timer to switch back to original search space set group | 10-1, 10-1a, 10-2, or 10-2a Need discussion for licensed use | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A |  | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
|  | 10-9c | Joint search space group switching across multiple cells | 1. Configured with a group of cells and switch search space set group jointly over these cells | 10-9, 10-9a, or 10-9b | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A |  | Without this capability, the UE will switch search space set groups for different cells independently | Optional with capability signalling |
|  | 10-10 | RSSI and channel occupancy measurement and reporting | 1. RSSI measurement 2. Channel occupancy reporting | 10-1, 10-1a, 10-2, or 10-2a | Yes | N/A |  | per band | N/A | N/A |  | RSSI measurement for channel occupancy. | Optional with capability signalling |
|  | 10-11 | SRS starting position at any OFDM symbol in a slot |  | Need discussion for licensed use | Yes | N/A |  | Per band or per UE | N/A | N/A |  | Support transmitting SRS starting in all symbols (0,…,13) of a slot | Optional with capability signalling |
|  | 10-12 | OCC for PRB interlace mapping for PF2 and PF3 FFS if need this capability, or if we split this for PF2 and PF3 separately | 1. OCC2 2. OCC4 | 10-3b or 10-3c | Yes | N/A |  | Per band | N/A | N/A |  | UE OCC capability for EPF2/EFP3 | Optional with capability signalling |
|  | 10-13a | Extended CP range of more than one symbol for CG-PUSCH FFS if need this capability |  | 10-1, Do we need this for 10-2? 5-19 or 5-20 | Yes | N/A |  | Per band | N/A | N/A |  | How long a UE can generate the CP extension beyond 1 symbol for CG-PUSCH | Optional with capability signalling |
|  | 10-14 | Enhanced HARQ-ACK feedback | 1. Non-numerical PDSCH to HARQ-ACK timing 2. Enhanced dynamic HARQ codebook 3. One-shot HARQ ACK feedback 4. One-shot HARQ ACK feedback trigger with empty DCI 1_1 5. Multi-PUSCH UL grant | 10-1 or 10-2 Need discussion for licensed use | Yes | N/A |  | Per band or per UE | N/A | N/A |  | If non-numerical K1 value is supported | Optional with capability signalling |
|  |  |  |  |  |  |  |  |  |  | N/A |  | Enhanced dynamic HARQ codebook supporting grouping of HARQ ACK and triggering the retransmission of HARQ ACK in each groups | Optional with capability signalling |
|  |  |  |  |  |  |  |  |  |  | N/A |  | Upon triggering, UE reports A/N for all HARQ processes and all CCs in a PUCCH group. | Optional with capability signalling |
|  |  |  |  |  |  |  |  |  |  | N/A |  |  | Optional with capability signalling |
|  |  |  |  |  |  |  |  |  |  | N/A |  |  | Optional with capability signalling |
|  | 10-18 | Enhanced Configured grant | 1. Support configuration of resources with cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16 2. Support retransmission in CG resources 3. Support configured grant retransmission timer 4. Support DFI monitoring 5. Support CG-UCI in CG-PUSCH | 10-1 or 10-2, 5-19 or 5-20 | Yes | N/A |  | Per band | N/A | N/A |  | Support configured grant with retransmission in configured grant resource | Optional with capability signalling |
|  | 10-19 | Number of LBT bandwidth FFS if this is needed | Number of ED measurements the UE is able to perform simultaneously [Whether the UE in WB operating mode can support nx20MHz measurement | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | This is the number of LBT bandwidth a UE can perform separate ED check on simultaneously | Optional with capability signalling |
|  | 10-19a | Support DL reception with subset of RB sets | 1. When DL BWP has multiple RB sets, support using the available RB set bitmap in DCI 2_0 to validate the periodic CSI-RS transmission if the CSI-RS is over multiple RB-sets | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | Without this capability, UE will assume all RB sets in the DL BWP are all transmitted or none of them are transmitted | Optional with capability signalling |
|  | 10-19b | Support UL transmission with subset of RB sets passing LBT | 1. When UL BWP has multiple RB sets, support transmission of UL signal or channels when LBT passes for only the RB sets the UL signals or channels are located | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  | Without this capability, UE will transmit UL when all RB sets in the UL BWP pass LBT | Optional with capability signalling |
|  | 10-20 | Support search space set configuration with freqMonitorLocation-r16 | 1. Support search space set configuration with freqMonitorLocations-r16 | 10-1, 10-1a, 10-2, or 10-2a Need discussion for licensed use | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-20a | Support coreset configuration with rb-Offset | 1. Support coreset configuration with rb-Offset | 10-1, 10-1a, 10-2, or 10-2a Need discussion for licensed use | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-21 | Support using ED threshold for UL to DL COT sharing | 1. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for scheduled UL to share COT with gNB for DL 2. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for CG-PUSCH to share COT with gNB for DL 3. Indicate in CG-UCI the COT sharing information as configured in cg-COT-Sharing-r16 | 10-1 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-22 | No gap 2-step RACH msgA transmission | 1. Support transmitting PRACH and PUSCH of msgA without gap in between FFS if RAN1 can disallow this in NR-U. | 10-1 or 10-2 and 9-1 | Yes | N/A |  | Per band | N/A | N/A |  | For licensed case, a minimum gap between PRACH and PUSCH of msgA is introduced, but is not applicable to NR-U | Optional with capability signalling |
|  | 10-23 | CGI reading based on off-sync raster SSB for ANR functionality | 1. Support acquisition of relevant information from a neighbouring NR unlicensed cell in an unlicensed carrier by reading the RMSI of the neighbouring unlicensed cell and reporting the acquired information to the network | 10-1, 10-1a, 10-2, or 10-2a | Yes | N/A |  | Per band | N/A | N/A |  | Support reading RMSI from SCell from an off-sync raster SSB for ANR | Optional with capability signalling |
|  | 10-24 | CG-UCI multiplexing with HARQ ACK | 1. Support multiplexing CG-UCI with HARQ ACK | 10-18, 5-19 or 5-20 Need discussion for licensed use | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-25 | Enable configured UL transmission out of COT | 1. Support configuration of enableConfiguredUL-r16 and enable Cat 4 LBT based transmission of RRC configured UL *SRS, PUCCH, CG-PUSCH etc) out of COT when DCI 2_0is configured but not detected | 10-1 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-26 | CSI-RS based RLM outside of discovery burst transmission window | Support RLM measurements using CSI-RS resources that are outside of discovery burst transmission window | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-27 | Wideband PRACH | Enhanced PRACH design for NR-U by adopting a single long ZC sequence, with ZC sequence = 1151 for 15kHz and ZC sequence = 571 for 30kHz | 10-1 or 10-2 | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
|  | 10-29 | Support available RB set indicator field in DCI 2_0 | Support monitoring DCI 2_0 to read availableRB-Sets-r16 | 10-1, 10-1a, 10-2, or 10-2a | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-30 | Support channel occupancy duration indicator field in DCI 2_0 | Support monitoring DCI 2_0 to read COT duration | 10-1, 10-1a, 10-2, or 10-2a | Yes | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |