3GPP TSG RAN WG1 #100bis		R1-2001729

e-Meeting, April 20th – 30th, 2020

Source:	OPPO

Title:	Text proposals for low PAPR RS

Agenda Item:	7.2.6.5

Document for:	Discussion and Decision

Introduction

In this contribution, we present our views on the remaining issues of the new low PAPR DMRS introduced in Rel-16 and propose some text proposals to update the specification.

Discussion

DMRS for PUSCH with pi/2-BPSK

In the current version of TS 38.211, some higher layer parameters are not aligned with the ones in TS 38.331. Thus some alignments are needed.

Proposal 1: Align the following RRC parameters in Section 6.4.1.1.1.2 with the ones in TS 38.331

- dmrsUplinkTransformPrecoding-r16   -> dmrs-UplinkTransformPrecoding-r16

- pi2BPSKscramblingID0 -> pi2BPSK-ScramblingID0

- pi2BPSKscramblingID1 -> pi2BPSK-ScramblingID1

The following gives the corresponding text proposal for Proposal 1:

In TS 38.211 Section 6.4.1 <omitted text> 6.4.1.1.1.2 Sequence generation when transform precoding is enabled If transform precoding for PUSCH is enabled, the reference-signal sequence shall be generated according to where ru,vα,δ(n) with δ=1 and α=0 depends on the configuration: - if the higher-layer parameter dmrs-UplinkTransformPrecoding-r16 is configured, π/2-BPSK modulation is used for PUSCH, and the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space, ru,vα,δ(n) is given by clause 5.2.3 with cinit given by cinit=217Nsymbslotns,fμ+l+12NIDnSCID+1+2NIDnSCID+nSCIDmod 231 where nSCID=0 unless given by the DCI according to clause 7.3.1.1.2 in [4, TS38.212] for a transmission scheduled by DCI format 0_1 or given by the higher-layer parameter antennaPort for a PUSCH transmission scheduled by a type-1 configured grant; and - NID0,NID1∈0,1,…,65535 are given by the higher-layer parameters pi2BPSK-ScramblingID0 and pi2BPSK-ScramblingID1, respectively, in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_1 or by a PUSCH transmission with a configured grant; - - NID0∈0,1,…,65535 is given by the higher-layer parameter pi2BPSK-ScramblingID0 in the DMRS-UplinkConfig IE if provided and the PUSCH is scheduled by DCI format 0_0 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI; - - NIDnSCID=NIDcell otherwise; - otherwise, ru,vα,δ(n) is given by clause 5.2.2. The sequence group u=fgh+nIDRS mod 30, where nIDRS is given by - nIDRS=nIDPUSCH if nIDPUSCH is configured by the higher-layer parameter nPUSCH-Identity in the DMRS-UplinkConfig IE, the higher-layer parameter dmrs-UplinkTransformPrecoding-r16 is not configured, and the PUSCH is neither scheduled by RAR UL grant nor scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI according to clause 8.3 in [5, TS 38.213]; - nIDRS=NIDnSCID if the higher-layer parameter dmrs-UplinkTransformPrecoding-r16 is configured, π/2-BPSK modulation is used for PUSCH, the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space; - nIDRS=NIDcell otherwise where and the sequence number v are given by: <omitted text>

DMRS for PUCCH with pi/2-BPSK

For Rel-16 new DMRS sequence for PUSCH with pi/2-BPSK, there are two alternatives discussed during RAN1#100 e-meeting

- Alt.1: Rel-16 new DMRS for PUSCH with pi/2-BPSK does not depend on the value of the cyclic shift α. Thus the specification does not need to specify how to determine the value of α.

- Alt.2: Rel-16 new DMRS for PUSCH with pi/2-BPSK should also use the cyclic shift value α to further randomize the interference. Thus the specification should revise the formula

From our understanding, the advantage of Alt.2 is not well justified. Thus we prefer Alt.1, i.e., the specification does not need to specify how to determine its value.

Proposal 2: In TS 38.211, remove the description on how to determine the value of α for Rel-16 new DMRS for PUCCH format 3/4 with pi/2-BPSK

There is also some alignment of RRC parameter.

Proposal 3: Align the following RRC parameters in Section 6.4.1.3.3.1 with its counterpart in TS 38.331

- dmrsUplinkTransformPrecodingPUCCH   ->  dmrs-UplinkTransformPrecodingPUCCH-r16

The following gives the corresponding text proposal for Proposal 2 and Proposal 3:

In TS 38.211 Section 6.4.1 <omitted text> ##### 6.4.1.3.3 Demodulation reference signal for PUCCH formats 3 and 4 ###### 6.4.1.3.3.1 Sequence generation The reference-signal sequence rlm shall be generated according to where MscPUCCH,s is given by clause 6.3.2.6.3 and ru,vα,δ(m) depends on the configuration: - if the higher-layer parameter dmrs-UplinkTransformPrecodingPUCCH-r16 is configured, ru,vα,δ(m) is given by clause 5.2.3 with cinit given by clause 6.4.1.3.2.1. The sequence group u and the sequence number v depend on the sequence hopping in clause 6.3.2.2.1 and the cyclic shift α depends on the cyclic shift hopping in clause 6.3.2.2.2. - otherwise, ru,vα,δ(m) is given by clause 6.3.2.2. The cyclic shift α varies with the symbol number and slot number according to clause 6.3.2.2.2 with for PUCCH format 3 without interlaced mapping and obtained from Table 6.4.1.3.3.1-1 with the orthogonal sequence index n given by clause 6.3.2.6.3 for PUCCH format 3 with interlaced mapping and PUCCH format 4. <omitted text>

PDSCH DMRS

There is an alignment for RRC parameter.

Proposal 4: Align the following RRC parameters in Section 7.4.1.1.1 with its counterpart in TS 38.331

- dmrsDownlink-r16   ->  dmrs-Downlink-r16

The following gives the corresponding text proposal for Proposal 4:

In TS 38.211 Section 7.4.1 <omitted text> - 7.4.1.1 Demodulation reference signals for PDSCH ##### 7.4.1.1.1 Sequence generation The UE shall assume the sequence is defined by . where the pseudo-random sequence is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with cinit=217Nsymbslotns,fμ+l+12NIDnSCIDλ+1+217λ2+2NIDnSCIDλ+nSCIDλmod 231 where is the OFDM symbol number within the slot, ns,fμ is the slot number within a frame, and - NID0,NID1∈0,1,…,65535 are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-DownlinkConfig IE if provided and the PDSCH is scheduled by PDCCH using DCI format 1_1 or 1_2 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI - NID0∈0,1,…,65535 is given by the higher-layer parameter scramblingID0 in the DMRS-DownlinkConfig IE if provided and the PDSCH is scheduled by PDCCH using DCI format 1_0 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI; - NIDnSCIDλ=NIDcell otherwise; - nSCIDλ and λ are given by - if the higher-layer parameter dmrs-Downlink-r16 in the DMRS-DownlinkConfig IE is provided nSCIDλ=nSCIDλ=0 or λ=21-nSCIDλ=1 λ=λ where λ is the CDM group defined in clause 7.4.1.1.2. - otherwise by nSCIDλ=nSCID λ=0 The quantity nSCID∈0, 1 is given by the DM-RS sequence initialization field, if present, in the DCI associated with the PDSCH transmission if DCI format 1_1 or 1_2 in [4, TS 38.212] is used, otherwise nSCID=0. <omitted text>

Conclusion

In this contribution, we present our view on the current specification regarding Rel-16 new DMRS (low PAPR RS). The following proposals and the corresponding text proposals are provided.

Proposal 1: Align the following RRC parameters in Section 6.4.1.1.1.2 with those in TS 38.331

- dmrsUplinkTransformPrecoding-r16   -> dmrs-UplinkTransformPrecoding-r16

- pi2BPSKscramblingID0 -> pi2BPSK-ScramblingID0

- pi2BPSKscramblingID1 -> pi2BPSK-ScramblingID1

Proposal 2: In TS 38.211, remove the description on how to determine the value of α for Rel-16 new DMRS for PUCCH format 3/4 with pi/2-BPSK

Proposal 3: Align the following RRC parameters in Section 6.4.1.3.3.1 with its counterpart in TS 38.331

- dmrsUplinkTransformPrecodingPUCCH   ->  dmrs-UplinkTransformPrecodingPUCCH-r16

Proposal 4: Align the following RRC parameters in Section 7.4.1.1.1 with its counterpart in TS 38.331

- dmrsDownlink-r16   ->  dmrs-Downlink-r16

References

- TS 38.211, NR; Physical channels and modulation, v16.1.0

- TS 38.331, NR; Radio Resource Control (RRC) protocol specification, v16.0.0