3GPP TSG RAN WG1 #100bis		R1-2001732

e-Meeting, April 20th – 30th, 2020

Source:	OPPO

Title:	Remaining Issues on UL Positioning Reference Signal

Agenda Item:	7.2.8.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we present our views on various remaining issues of uplink positioning reference signals and provides text proposals for specification update.

- Discussion

  - TS 38.211

In TS 38.211, the differentiation of traditional SRS and SRS for positioning is based on whether or not the SRS resources are configured by SRS-Config. However, RAN2 has decided to configure both traditional SRS and SRS for positioning in the same SRS-Config. We need to differentiate these two types of SRS by using SRS-ResourceSet and SRS-PosResourceSet-r16.

Proposal 1: Use SRS-ResourceSet and SRS-PosResourceSet-r16 to differentiate the traditional SRS and SRS for positioning

The following give the corresponding text proposals for Proposal 1.

In TS 38.211 Section 6.4.1.4 <omitted text> ##### 6.4.1.4.2 Sequence generation The sounding reference signal sequence for an SRS resource shall be generated according to r(pi)n,l'=ru,v(αi,δ)n 0≤n≤Msc,bSRS-1 l'∈0,1,…,NsymbSRS-1 where Msc,bSRS is given by clause 6.4.1.4.3, ru,vα,δ(n) is given by clause 5.2.2 with δ=log2KTC and the transmission comb number KTC∈2,4,8 is contained in the higher-layer parameter transmissionComb. The cyclic shift for antenna port is given as , where is contained in the higher layer parameter transmissionComb. The maximum number of cyclic shifts nSRScs,max are given by Table 6.4.1.4.2-1. The sequence group u=fghns,fμ,l'+nIDSRS mod 30 and the sequence number in clause 5.2.2 depends on the higher-layer parameter groupOrSequenceHopping in the SRS-Config SRS-ResourceSet IE or the [SRS-for-positioning] SRS-PosResourceSet-r16 IE. The SRS sequence identity is given by the higher layer parameter sequenceId in the SRS-Config SRS-ResourceSet IE, in which case nIDSRS∈0, 1, …, 1023, or the [SRS-for-positioning] SRS-PosResourceSet-r16 IE, in which case nIDSRS∈0, 1, …, 65535. The quantity l'∈0,1,…,NsymbSRS-1 is the OFDM symbol number within the SRS resource. <omitted text>

In TS 38.211 Section 6.4.1.4 ##### 6.4.1.4.3 Mapping to physical resources <omitted text> The frequency-domain starting position k0(pi) is defined by k0(pi)=k0(pi)+b=0BSRSKTCMsc,bSRSnb where k0(pi)=nshiftNscRB+kTC(pi)+koffsetl' mod KTC kTC(pi)=kTC+KTC2 mod KTC if nSRScs∈nSRScs,max2, …, nSRScs,max-1 and NapSRS=4 and pi∈1001, 1003kTCotherwise If NBWPstart≤nshift the reference point for k0(pi)=0 is subcarrier 0 in common resource block 0, otherwise the reference point is the lowest subcarrier of the BWP. If the SRS is configured by the IE [SRS-for-positioning] SRS-PosResourceSet-r16, the quantity koffsetl' is given by Table 6.4.1.4.3-2, otherwise koffsetl'=0. The frequency domain shift value adjusts the SRS allocation with respect to the reference point grid and is contained in the higher-layer parameter freqDomainShift in the SRS-Config SRS-ResourceSet IE or the [SRS-for-positioning] SRS-PosResourceSet-r16 IE. The transmission comb offset kTC∈0,1,…,KTC-1 is contained in the higher-layer parameter transmissionComb in the SRS-Config SRS-ResourceSet IE or the [SRS-for-positioning] SRS-PosResourceSet-r16 IE and is a frequency position index. Frequency hopping of the sounding reference signal is configured by the parameter bhop∈0,1,2,3, given by the field b-hop contained in the higher-layer parameter freqHopping if configured, otherwise bhop=0. <omitted text>

  - TS 38.213

Proposal 1 is applicable for TS 38.213 as well. Moreover, some RRC parameters in current version of TS 38.213 are not aligned with the ones in TS 38.331. Thus we have the following proposal

Proposal 2: Align the following RRC parameters in TS 38.213 with those in TS 38.331

- SRS-Positioning-Config   ->  SRS-PosResourceSet-r16

- SRS-ResourceSetId -> srs-PosResourceSetId-r16

- SRS-ResourceSet ->  SRS-PosResourceSet-r16

- pathlossReferenceRS -> pathlossReferenceRS-Pos-r16

The following gives the corresponding text proposal for Proposal 1 and proposal 2.

In TS 38.213 Section 7 # 7 Uplink Power control Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions. A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in Clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by IE SRS-Positioning-Config SRS-PosResourceSet-r16 as described in Clause 7.3.1. A PUSCH/PUCCH/SRS/PRACH transmission occasion is defined by a slot index within a frame with system frame number , a first symbol within the slot, and a number of consecutive symbols . <omitted text>

In TS 38.213 Section 7 - 7.3.1 UE behaviour If a UE transmits SRS based on a configuration by IE SRS-Config SRS-ResourceSet on active UL BWP of carrier of serving cell using SRS power control adjustment state with index , the UE determines the SRS transmission power in SRS transmission occasion as [dBm] <omitted text>

In TS 38.213 Section 7 ##### Section 7.3.1 UE behaviour <omitted text> If a UE transmits SRS based on a configuration by IE SRS-Positioning-Config SRS-PosResourceSet-r16 on active UL BWP b of carrier f of serving cell c, the UE determines the SRS transmission power PSRS,b,f,ci,qs in SRS transmission occasion i as [dBm] where, - PO,SRS,b,f,cqs and αO,SRS,b,f,cqs are provided by p0 and alpha respectively, for active UL BWP b of carrier f of serving cell c, and SRS resource set qs is indicated by SRS-ResourceSetId srs-PosResourceSetId-r16 from SRS-ResourceSet SRS-PosResourceSet-r16, and - PLb,f,cqd is a downlink pathloss estimate in dB calculated by the UE, as described in Clause 7.1.1 in case of an active DL BWP of a serving cell c, using RS resource indexed qd in a serving or non-serving cell for SRS resource set qs [6, TS 38.214]. A configuration for RS resource index qd associated with SRS resource set qs is provided by pathlossReferenceRS pathlossReferenceRS-Pos-r16 - if a ssb-Index is provided, referenceSignalPower is provided by ss-PBCH-BlockPower - if a dl-PRS-ResourceId is provided, referenceSignalPower is provided by dl-PRS-ResourcePower If the UE determines that the UE is not able to accurately measure PLb,f,cqd, the UE calculates PLb,f,cqd using a RS resource obtained from the SS/PBCH block of the serving cell that the UE uses to obtain MIB The UE indicates a capability for a number of pathloss estimates that the UE can simultaneously maintain. <omitted text>

There are two typos in description of SRS for positioning in TS 38.213 and suggest to make the following changes to correct typos:

In TS 38.213 ##### Section 7.3.1 UE behaviour <omitted text> If a UE transmits SRS based on a configuration by IE SRS-Positioning-Config on active UL BWP b of carrier f of serving cell c, the UE determines the SRS transmission power PSRS,b,f,ci,qs in SRS transmission occasion i as [dBm] where, - PO_SRS,b,f,cqsPO,SRS,b,f,cqs and αSRS,b,f,cqs αO,SRS,b,f,cqs are provided by p0 and alpha respectively, for active UL BWP b of carrier f of serving cell c, and SRS resource set qs is indicated by SRS-ResourceSetId from SRS-ResourceSet, and - PLb,f,cqd is a downlink pathloss estimate in dB calculated by the UE, as described in Clause 7.1.1 in case of an active DL BWP of a serving cell c, using RS resource indexed qd in a serving or non-serving cell for SRS resource set qs [6, TS 38.214]. A configuration for RS resource index qd associated with SRS resource set qs is provided by pathlossReferenceRS - if a ssb-Index is provided, referenceSignalPower is provided by ss-PBCH-BlockPower - if a dl-PRS-ResourceId is provided, referenceSignalPower is provided by dl-PRS-ResourcePower If the UE determines that the UE is not able to accurately measure PLb,f,cqd, the UE calculates PLb,f,cqd using a RS resource obtained from the SS/PBCH block that the UE uses to obtain MIB The UE indicates a capability for a number of pathloss estimates that the UE can simultaneously maintain. <omitted text>

- Conclusion

In this contribution, we presented our views on remaining issues of SRS for NR positioning and corresponding text proposals are provided. Based on the discussion, the following proposals are provided:

Proposal 1: Use SRS-ResourceSet and SRS-PosResourceSet-r16 to differentiate the traditional SRS and SRS for positioning

Proposal 2: Align the following RRC parameters in TS 38.213 with those in TS 38.331

- SRS-Positioning-Config   ->  SRS-PosResourceSet-r16

- SRS-ResourceSetId -> srs-PosResourceSetId-r16

- SRS-ResourceSet ->  SRS-PosResourceSet-r16

- pathlossReferenceRS -> pathlossReferenceRS-Pos-r16

- Reference

- TS 38.211, NR; Physical channels and modulation, v16.1.0

- TS 38.213, NR; Physical layer procedures for control, V16.1.0

- TS 38.331, NR; Radio Resource Control (RRC) protocol specification, v16.0.0