3GPP TSG RAN WG1 #100bis		R1-2001740

e-Meeting, April 20th – 30th, 2020

Source:	OPPO

Title:	Discussion on UE features for CLI/RIM

Agenda Item:	7.2.11.11

Document for:	Discussion and Decision

Introduction

In this contribution, we present our views on UE features for CLI/RIM and  corresponding proposals will be provided.

Discussions

Regarding the measurement of CLI-RSSI, we made the following agreements:

Agreement In information element of resource configuration for CLI-RSSI measurement, reference subcarrier spacing for CLI-RSSI measurement is included. - All subcarrier spacings supported in Rel-15 NR for FR1, FR2 - Subcarrier spacing is a reference unit of time/frequency resource configuration. - UE operates CLI measurement within the active BWP. - The subcarrier spacing for CLI-measurement resource configuration can be same or different from the SCS of the active BWP. Agreement One or multiple resources for CLI-RSSI measurement can be configured - Maximum number of measurement resource for CLI-RSSI measurement is 64.

Considering various UE capabilities, the maximum number of measurement resources for CLI-RSSI measurement shall be part of UE capability and shall be reported by the UE.  Furthermore, the maximum number of measurement resources for CLI-RSSI measurement within one slot shall be UE capability too.

As we agreed, the subcarrier spacing for CLI measurement resource can be same or different from the subcarrier spacing of the active BWP. However, measuring CLI measurement resource with same or different subcarrier spacing would require totally different UE capability. It is preferred to support UE reporting whether support measuring CLI measurement with different subcarrier spacings.

Proposal 1: For FG17-1, CLI-RSSI measurement, support the UE reports the followings:

- maximum number of measurement resources configured for CLI-RSSI measurement.

  - The candidate value can be {8, 16, 32, 64}

- maximum number of measurement resources configured for CLI-RSSI measurement within one slot

  - The candidate value can be {1, 2, 4, 8}

Regarding the SRS-RSRP measurement, we made the following agreements:

Agreement For SRS-RSRP: - The number of SRS to be monitored by the UE should not exceed 8 within a slot - Network may configure more than 8 SRSs over different slots - The total number of SRSs to be monitored by a UE should not exceed [32] Agreement For SRS-RSRP, the total number of SRS resources to be monitored by a UE should not exceed 32.

Similar to the CLI-RSSI measurement resources, the maximum number of SRS resources that can be configured for SRS-RSRP measurement shall be a UE capability and the maximum number of SRS resources that can be configured within one slot for SRS-RSRP measurement shall be a UE capability too.

Proposal 2: For FG17-2, SRS-RSRP measurement, support the UE reports the followings:

- maximum number of SRS resources configured for SRS-RSRP measurement per UE. The candidate value can be {4, 8, 16, 32}

- maximum number of SRS resources configured for SRS-RSRP measurement within one slot. The candidate value can be {1, 2, 4, 8}

With the proposed Proposals 1 and 2, the FG 17-1 and 17-2 shall be updated as follows:

| 17-1 | CLI-RSSI measurement | - Support CLI-RSSI measurement, - The maximum number of resources across all CCs - The maximum number of resources within one slot - Subcarrier spacing for CLI-RSSI measurement is same as subcarrier spacing for active BWP |  |  | N/A |  | Per UE | TDD only | Yes |  | UE operates CLI-RSSI measurement. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17-2 | SRS-RSRP measurement | - Support SRS-RSRP measurement - The maximum number of SRS resources across all CCs - The maximum number of SRS resources within one slot |  |  | N/A |  | Per UE | TDD only | Yes |  | UE operates SRS-RSRP measurement. | Optional with capability signalling |

Conclusion

In this contribution, we discussed a few remaining issues on multi-beam operation enhancement. Based on the discussion, the following proposals and TPs are provided:

Proposal 1: For FG17-1, CLI-RSSI measurement, support the UE reports the followings:

- maximum number of measurement resources configured for CLI-RSSI measurement.

  - The candidate value can be {8, 16, 32, 64}

- maximum number of measurement resources configured for CLI-RSSI measurement within one slot

  - The candidate value can be {1, 2, 4, 8}

Proposal 2: For FG17-2, SRS-RSRP measurement, support the UE reports the followings:

- maximum number of SRS resources configured for SRS-RSRP measurement per UE. The candidate value can be {4, 8, 16, 32}

- maximum number of SRS resources configured for SRS-RSRP measurement within one slot. The candidate value can be {1, 2, 4, 8}

References

- Chairman’s Note RAN1 Ad-Hoc Meeting 1901

- Chairman’s Note RAN1 #96