3GPP TSG RAN WG1 #100bis		R1-2001742

e-Meeting, April 20th – 30th, 2020

Source:	OPPO

Title:	Discussion on the support of SRS transmission in all symbols of a slot

Agenda Item:	7.2.11.13

Document for:	Discussion and Decision

Introduction

In this contribution, we present our view on the support of SRS transmission on any symbol of a slot.

Discussion

In the NR unlicensed session, it was agreed to support SRS transmission whose starting position can be at any symbol of a slot, and the UE feature is captured as follows

| 10-11 | SRS starting position at any OFDM symbol in a slot |  |  | Yes | N/A |  | Per band or per UE | N/A | N/A |  | Support transmitting SRS starting in all symbols (0,…,13) of a slot | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

This feature is useful not only for unlicensed bands, but also for licensed bands. Rel-16 LTE has supported such feature. Moreover, during the discussion of Rel-17 further enhancement for MIMO, it was identified the capacity of SRS resources should be enhanced and [2] captured that “due to the use of SRS for various scenarios, SRS can and should be further enhanced at least for capacity and coverage”. Thus we can have the following observation

Observation 1: Support of SRS starting position at any symbol in a slot will alleviate the shortage of SRS capacity and is beneficial for licensed bands as well.

In the current version of TS 38.211, the description on the starting position of SRS resource doesn’t differentiate licensed and unlicensed bands. TS 38.331 also doesn’t differentiate the configurations of SRS resource for licensed and unlicensed bands.

6.4.1.4.1 SRS resource An SRS resource is configured by the SRS-Resource IE or the [SRS-for-positioning] IE and consists of - NapSRS∈1,2,4 antenna ports pii=0NapSRS-1, where the number of antenna ports is given by the higher layer parameter nrofSRS-Ports if configured, otherwise NapSRS=1, and pi=1000+i when the SRS resource is in a SRS resource set with higher-layer parameter usage in SRS-ResourceSet not set to 'nonCodebook', or determined according to [6, TS 38.214] when the SRS resource is in a SRS resource set with higher-layer parameter usage in SRS-ResourceSet set to 'nonCodebook' - NsymbSRS∈1,2,4,8,12 consecutive OFDM symbols given by the field nrofSymbols contained in the higher layer parameter resourceMapping - , the starting position in the time domain given by l0=Nsymbslot-1-loffset where the offset loffset∈0,1,…,13 counts symbols backwards from the end of the slot and is given by the field startPosition contained in the higher layer parameter resourceMapping and loffset≥NsymbSRS-1 - , the frequency-domain starting position of the sounding reference signal

SRS-Resource ::=                        SEQUENCE {

srs-ResourceId                          SRS-ResourceId,

nrofSRS-Ports                           ENUMERATED {port1, ports2, ports4},

ptrs-PortIndex                          ENUMERATED {n0, n1 }                                       OPTIONAL,   -- Need R

transmissionComb                        CHOICE {

n2                                      SEQUENCE {

combOffset-n2                           INTEGER (0..1),

cyclicShift-n2                          INTEGER (0..7)

},

n4                                      SEQUENCE {

combOffset-n4                           INTEGER (0..3),

cyclicShift-n4                          INTEGER (0..11)

}

},

resourceMapping                         SEQUENCE {

startPosition                           INTEGER (0..5),

nrofSymbols                             ENUMERATED {n1, n2, n4},

repetitionFactor                        ENUMERATED {n1, n2, n4}

},

freqDomainPosition                      INTEGER (0..67),

freqDomainShift                         INTEGER (0..268),

freqHopping                             SEQUENCE {

c-SRS                                   INTEGER (0..63),

b-SRS                                   INTEGER (0..3),

b-hop                                   INTEGER (0..3)

},

groupOrSequenceHopping                  ENUMERATED { neither, groupHopping, sequenceHopping },

resourceType                            CHOICE {

aperiodic                               SEQUENCE {

...

},

semi-persistent                         SEQUENCE {

periodicityAndOffset-sp                     SRS-PeriodicityAndOffset,

...

},

periodic                                SEQUENCE {

periodicityAndOffset-p                      SRS-PeriodicityAndOffset,

...

}

},

sequenceId                              INTEGER (0..1023),

spatialRelationInfo                     SRS-SpatialRelationInfo                                 OPTIONAL,   -- Need R

...,

[[

resourceMapping-r16                     SEQUENCE {

startPosition-r16                       INTEGER (0..13),

nrofSymbols-r16                         ENUMERATED {n1, n2, n4},

repetitionFactor-r16                    ENUMERATED {n1, n2, n4}

}                                                                                               OPTIONAL    -- Need R

]]

}

Thus, we have the following observation:

Observation 2: Support of SRS starting position at any symbol in a slot for licensed bands will lead to NO additional standardization efforts.

Based on the above discussions and observations, we have the following proposal

Proposal 1: Support SRS resource starting at any symbol in a slot for licensed bands in Rel-16, i.e., UE feature 10-11 is also applicable to licensed bands.

Conclusion

In this contribution, we present our view on on the support of SRS transmission on any symbol of a slot, and we have the following observations and proposal:

Observation 1: Support of SRS starting position at any symbol in a slot will alleviate the shortage of SRS capacity and is beneficial for licensed bands as well.

Observation 2: Support of SRS starting position at any symbol in a slot for licensed bands will lead to NO additional standardization efforts.

Proposal 1: Support SRS resource starting at any symbol in a slot for licensed bands in Rel-16, i.e., UE feature 10-11 is also applicable to licensed bands.

References

- R1-2001484, RAN1 UE features list for Rel-16 NR after RAN1#100-E, AT&T, DOCOMO

- RP-193133, New WID: Further enhancements on MIMO for NR, Samsung

- TS 38.211, NR; Physical channels and modulation, V16.1.0

- TS 38.331, NR; Radio Resource Control (RRC) protocol specification, v16.0.0