3GPP TSG RAN WG1 #100bis-e	R1-2001815

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.4.1

Source: 	Sony

Title:	Remaining issues on physical layer structure for NR sidelink

Document for:	Discussion

# Introduction

In the email discussion at RAN1 #100e, we discussed resource pool configuration and made a working assumption as follows [1]:

Working assumption - For the number of PRBs for resource pool, allow configuration of all number of PRBs in a SL BWP. - FFS until RAN1#100bis-e whether/how to deal with remaining PRBs if the configured PRBs for resource pool is not a multiple of subchannel size.

In this contribution, we give our views related to the remaining PRBs.

# Discussion

In the email discussion of [100e-NR-5G_V2X_NRSL-PHYstructure-03], RAN1 discussed the number of PRBs for resource pool and made a working assumption to allow configuration of all number of PRBs in a SL BWP. In addition, it was FFS whether/how to deal with remaining PRBs if the configured PRBs for resource pool is not a multiple of subchannel size.

The configurable values for the sub-channel size are {10, 15, 20, 25, 50, 75, 100}. In our understanding, the contention point was whether/how to utilise the full frequency resources available in the ITS band. The remaining PRBs arise based on the configured sub-channel size. The remaining PRBs can be minimised by choosing a suitable sub-channel size. For example, there are in total 106 PRBs in a 40MHz bandwidth when 30kHz SCS is used. The least number of remaining PRBs could be 1 PRB if the sub-channel size is 15. But in the email discussion, it was mentioned from a company that the sub-channel size would be determined from several aspects, not just based on the issue of the number of remaining PRBs. If the assumption is correct, the number of remaining PRBs may not be able to be minimised. In the case of the above example, the number of the remaining PRBs is 6 PRBs when the sub-channel size is 10. Resource utilisation may be diminished when there are remaining PRBs.

Therefore, some companies proposed special handling for the remaining PRBs, e.g. the last sub-channel consists of the remaining PRBs, or the remaining PRBs are evenly distributed to multiple sub-channels as much as possible. We could support such special solutions to avoid the waste of resources due to the remaining PRBs. However, we may need to carefully investigate any impact on other aspects arising from such solutions, e.g. PSCCH size / PSFCH resource / TBS determination. During current the CR phase, these kind of changes should be avoided. Therefore, we propose no special solution for the remaining PRBs in Rel-16, i.e. if the configured number of PRBs is not a multiple of sub-channel size, the remaining PRBs are not used in Rel-16.

Proposal 1: confirm the working assumption of “For the number of PRBs for resource pool, allow configuration of all  number of PRBs in a SL BWP.”

Proposal 2: if the configured number of PRBs is not a multiple of sub-channel size, the remaining PRBs are not used in Rel-16.

# Summary

In this contribution, the following proposals are made:

Proposal 1: confirm the working assumption of “For the number of PRBs for resource pool, allow configuration of all  number of PRBs in a SL BWP.”

Proposal 2: if the configured number of PRBs is not a multiple of sub-channel size, the remaining PRBs are not used in Rel-16.

# References

[1] List of email approvals for RAN1 #100e, RAN1 chairman