3GPP TSG RAN WG1 #100bis			        R1-2001905

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.2.2.5

Source: 	MediaTek Inc.

Title:	Remaining issues on wideband operation for NR-U

Document for: 	Discussion and Decision

Introduction

In this contribution, we discuss the remaining issues on wideband operation for NR-U and provide text proposals to the latest version of the R16 NR-U specifications.

Remaining issues on wideband operation

Issue 1: CORESET associated with SS sets configured with and without freqMonitorLocation-r16

Extract from clause 10.1 in the endorsed CR of TS 38.213 for R16 NR-U [1]

## *** Unchanged text is omitted ***

For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap.

-	if a CORESET is not associated with any search space set configured with freqMonitorLocation-r16, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of  NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-offset is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-offset.

-	if a CORESET is associated with at least one search space set configured with freqMonitorLocation-r16, the first NRBG,set0size  bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of  NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset. NRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP, and NRBoffset is provided by rb-offset or NRBoffset=0 if rb-offset is not provided.

## *** Unchanged text is omitted ***

For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with  search space sets where, for each search space set from the  search space sets, the UE is provided the following by SearchSpace:

## *** Unchanged text is omitted ***

-	a bitmap by freqMonitorLocation-r16, if provided, to indicate one or more RB sets for the search space set s, where the MSB k in the bitmap corresponds to RB set k-1 in the DL BWP. For RB set k indicated in the bitmap, the first PRB of the frequency domain monitoring location confined within the RB set is given by NRB,  set kstart+NRBoffset, where NRB,set kstart is the index of first PRB of the RB set k, and NRBoffset is provided by rb-offset or NRBoffset=0 if rb-offset is not provided. The frequency domain resource allocation pattern for each monitoring location is determined based on the first NRBG, set 0size bits in frequencyDomainResources provided by the associated CORESET configuration.

After RAN1#100 meeting, the above specifications were made in TS38.213 clause 10.1 to clarify frequency domain resource allocation for a CORESET. However, it is not quite clear from the current specification that how to determine frequency domain resource for a CORESET associated with SS sets configured with and without freqMonitorLocation-r16. Apparently, the current specification may lead to a confusion. In our understanding, frequency domain resource for the CORESET should be determined as follows:

- For a SS set configured without freqMonitorLocation-r16, the CORESET is located in the RB set 0 for the DL BWP with a starting CRB index  NBWPstart+NRBoffset

- For a SS set configured with a bitmap by freqMonitorLocation-r16, the CORESET is located in the RB set(s) for the DL BWP indicated by the bitmap with a starting CRB index NRB,  set kstart+NRBoffset

The above two scenarios should be clearly clarified in the specification to avoid the confusion.

Proposal 1: Text Proposal 1 is adopted in TS38.213 clause 10.1 to clarify how to determine frequency domain resource for a CORESET associated with SS sets configured with and without freqMonitorLocation-r16.

==================================Text Proposal 1 Starts======================================

10.1 UE procedure for determining physical downlink control channel assignment

*** Unchanged text is omitted ***

For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap.

-	if a CORESET is not associated with any search space set configured with freqMonitorLocation-r16, the bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of  NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index 6⋅NBWPstart/6 if rb-offset is not provided, or the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset where NRBoffset is provided by rb-offset.

-	if a CORESET is associated with at least one search space set configured with freqMonitorLocation-r16, the first NRBG,set0size  bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of  NRBBWP PRBs with starting common RB position NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset. NRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP, and NRBoffset is provided by rb-offset or NRBoffset=0 if rb-offset is not provided.

*** Unchanged text is omitted ***

For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with  search space sets where, for each search space set from the  search space sets, the UE is provided the following by SearchSpace:

-	a search space set index , , by searchSpaceId

*** Unchanged text is omitted ***

-	a bitmap by freqMonitorLocation-r16, if provided, to indicate one or more RB sets for the search space set s, where the MSB k in the bitmap corresponds to RB set k-1 in the DL BWP. For RB set k indicated in the bitmap, the first PRB of the frequency domain monitoring location confined within the RB set is given by NRB,  set kstart+NRBoffset, where NRB,set kstart is the index of first PRB of the RB set k, and NRBoffset is provided by rb-offset or NRBoffset=0 if rb-offset is not provided. The frequency domain resource allocation pattern for each monitoring location is determined based on the first NRBG, set 0size bits in frequencyDomainResources provided by the associated CORESET configuration, whereNRBG,set0size=(NRB,set0size-NRBoffset)/6, NRB,set0size is a number of available PRBs in the RB set 0 for the DL BWP. If search-space s is not provided with freqMonitorLocation-r16 and the associated CORESET is associated with at least one another search space set configured with freqMonitorLocation-r16, the first NRBG, set 0size bits in frequencyDomainResources provided by the associated CORESET configuration have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs in the RB set 0 for the DL BWP, in ascending order of the PRB index in the DL BWP bandwidth of  NRBBWP PRBs with starting common RB index NBWPstart, where the first common RB of the first group of 6 PRBs has common RB index NBWPstart+NRBoffset.

*** Unchanged text is omitted ***

================================== Text Proposal 1 Ends======================================

Issue 2: Definition of single RB set for a carrier with one single LBT bandwidth

In RAN1#97 meeting, RAN1 has an agreement to introduce available LBT bandwidth indicator in GC-PDCCH to inform UE one or more carriers and/or LBT bandwidths are not available or available for DL reception. In RAN1#98 meeting, RAN1 agrees to use a bitmap to indicate available LBT bandwidths for a serving cell. More specifically, even a serving cell has only one LBT, gNB still can configure one bit in DCI format 2_0 for indicating the availability of the LBT bandwidth in the serving cell. However, current specification doesn't exactly reflect these agreements.

Agreement from RAN1#97:

When GC-PDCCH is configured, explicit indication via GC-PDCCH is supported as a mechanism to inform the UE that one or more carriers and/or LBT bandwidths are not available or available for DL reception, at least for slot(s) that are not at the beginning of DL transmission burst.

- FFS: Signalling details of the indication, including e.g., the time domain validity of the indication

Agreement from RAN1#98:

Support bit field corresponding to available LBT bandwidths in GC-PDCCH (add a bitmap in the GC-PDCCH DCI)

In TS38.214 clause 7 [2], RB sets in a serving cell are determined only when intra-cell GBs are provided. However, for a serving cell including only one LBT bandwidth, it is clear that intra-cell GBs are not necessary. Since RAN1 already has the agreement that “no intra-cell GBs are configured“ can be indicated for a serving cell and how to indicate it will be decided by RAN2, a single RB set could be defined for this scenario.

Proposal 2: Text proposal 2 is adopted in TS38.214 clause 7 to define a single RB set for a serving indicated with “no intra-cell guard-bands are configured” by intraCellGuardBandDL-r16 or intraCellGuardBandUL-r16.

==================================Text Proposal 2 Starts=====================================

7. UE procedures for transmitting and receiving on a carrier with intra-cell guard bands

For operation with shared spectrum channel access, when the UE is configured with any of intraCellGuardBandUL-r16 for UL carrier and intraCellGuardBandDL-r16 for DL carrier, the UE is provided with  NRB-set-1  intra-cell guard bands on a carrier, each defined by start and end CRB, GB sstart,μ  and  GB send,μ , respectively. The intra-cell guard bands separate  NRB-set RB-sets, each defined by start and end CRB, RB sstart,μ and  RB send,μ, respectively.  UE determines RB 0start,μ=Ngridstart,μ, RBNRB-set-1end,μ=Ngridstart,μ+Ngridsize,μ  , and the remaining start and end CRBs as RB send,μ=GB sstart,μ-1 and RB s+1start,μ=GB send,μ+1. When the UE is not configured with intraCellGuardBandUL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ]. When the UE is not configured with intraCellGuardBandDL-r16, the UE determines intra-cell guard band and corresponding RB-set according to the [default intra-cell GB pattern from 38.101 corresponding to μ and carrier size Ngridsize,μ].

For a carrier with intra-carrier guard bands, the UE does not expect to receive a BWP configuration by BWP-Downlink or BWP-Uplink partially overlapping with a RB-set. RB-sets within BWP form a set SRB-sets of cardinality NRB-setBWP.

[The configuration of intraCellGuardBandDL-r16 and intraCellGuardBandUL-r16 can indicate to the UE that no intra-cell guard-bands are configured.] If a UE is provided with intraCellGuardBandDL-r16 or intraCellGuardBandUL-r16 to indicate no intra-cell guard-bands are configured for a DL or UL carrier, the UE determines a single RB-set with RB 0start,μ=Ngridstart,μ and RB0end,μ=Ngridstart,μ+Ngridsize,μ for the carrier and assumes NRB-set=1.

================================== Text Proposal 2 Ends======================================

Conclusion

In this contribution, the remaining issues on wideband operation for NR-U were discussed. Based on the discussion in the previous sections, we made the following proposals:

Proposal 1: Text Proposal 1 is adopted in TS38.213 clause 10.1 to clarify how to determine frequency domain resource for a CORESET associated with SS sets configured with and without freqMonitorLocation-r16.

Proposal 2: Text proposal 2 is adopted in TS38.214 clause 7 to define a single RB set for a serving indicated with “no intra-cell guard-bands are configured” by intraCellGuardBandDL-r16 or intraCellGuardBandUL-r16.

- References

- 3GPP RAN1#100e R1-2001456, “Corrections on shared spectrum channel access”

- 3GPP RAN1#100e R1-2001441, “Corrections on NR-U”