3GPP TSG RAN WG1 #100bis                             R1-2001937

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.2.2.3

Source: 	LG Electronics

Title: 	Remaining issues of HARQ procedure for NR-U

Document for:	Discussion and decision

- Introduction

In this contribution, we discuss the remaining issues on HARQ procedure for NR-U operation, and also provide the corresponding (text) proposal.

- Enhanced dynamic HARQ-ACK codebook

- Issue 1-1: HARQ-ACK codebook with CBG configuration

In case with CBG configuration for PDSCH transmission, the corresponding HARQ-ACK codebook procedure is still incomplete in terms of T-DAI signalling via DL DCI, especially when T-DAI (and NFI) indication for the non-scheduled PDSCH group is configured for DL DCI. On this issue, one of the following three alternatives needs to be adopted.

- Alt 1: Two T-DAI values are indicated for the non-scheduled PDSCH group.

  - One value corresponds to TB-based PDSCH.

  - The other value corresponds to CBG-based PDSCH.

- Alt 2: Only one T-DAI value is indicated for the non-scheduled PDSCH group.

  - The value corresponds to TB-based PDSCH.

  - For CBG-based PDSCH, the last received T-DAI value indicated by other DCI (scheduling the CBG-based PDSCH) is applied.

- Alt 3: Only one T-DAI value is indicated for the non-scheduled PDSCH group.

  - If both or none of TB-based PDSCH and CBG-based PDSCH is scheduled, the value corresponds to TB-based PDSCH.

  - If only one PDSCH type of TB-based and CBG-based PDSCHs is scheduled, the value corresponds to the scheduled PDSCH type.

  - For other PDSCH type, the last received T-DAI value indicated by other DCI (scheduling the PDSCH type) is applied.

Furthermore, for UL DAI (i.e., T-DAI) signalling/indication via UL DCI, similar alternatives can be considered, for example, according to whether the T-DAI value is indicated for both or one of TB-based PDSCH and CBG-based PDSCH per each of PDSCH groups.

Among the above three alternatives, Alt 1 is preferred to guarantee the robustness/reliability of HARQ-ACK feedback as well as to avoid PDSCH scheduling restriction in case with CBG configuration.

Proposal #1: For the case when CBG based PDSCH transmission is configured and T-DAI indication for the non-scheduled PDSCH group is configured for DL DCI, the following is adopted.

- Two T-DAI values are indicated for the non-scheduled PDSCH group.

  - One value corresponds to TB-based PDSCH.

  - The other value corresponds to CBG-based PDSCH.

Proposal #2: For the case when CBG based PDSCH transmission is configured and T-DAI indication for both or one of two PDSCH groups is configured for UL DCI, the following is adopted.

- Two T-DAI values are indicated per PDSCH group.

  - One value corresponds to TB-based PDSCH.

  - The other value corresponds to CBG-based PDSCH.

- Issue 1-2: NFI value applied for HARQ-ACK on PUSCH

For HARQ-ACK piggyback on PUSCH, the followings were agreed in RAN1#100 in terms of UL DAI (i.e., T-DAI) indication via UL DCI.

- In case when T-DAI is configured for both two PDSCH groups,

  - First T-DAI corresponds to the first group 0.

  - Second T-DAI corresponds to the second group 1.

- In case when T-DAI is configured for only one PDSCH group.

  - If both or none of two groups is scheduled, the T-DAI corresponds to the first group 0.

  - If only one of two groups is scheduled, the T-DAI corresponds to the scheduled group.

Regarding the above, one thing required to be clarified is the UE assumption on the NFI value for the PDSCH group corresponding to the T-DAI in UL DCI, in case when the PDSCH group is actually not scheduled in the UE side. For an example, in case configured with T-DAI for both two PDSCH groups, if the first group 0 is not scheduled (while the first T-DAI indicated by UL DCI is not equal to 4), the UE assumption is needed for the NFI value (e.g., whether it is toggled) corresponding to the first group 0, in order to generate the HARQ-ACK payload corresponding to the first group 0. For another example, in case configured with T-DAI for only one PDSCH group, if no PDSCH is scheduled in the UE side (while the T-DAI in UL DCI is not equal to 4), the UE assumption is needed for the NFI value corresponding to the first group 0.

To handle this issue, one of the following two alternatives needs to be adopted.

- Alt 1: NFI value for the PDSCH group corresponding to the T-DAI in UL DCI is assumed to be toggled from the latest value, in case when the PDSCH group is not scheduled.

- Alt 2: NFI (for the PDSCH group corresponding to the T-DAI in UL DCI) is signaled via the UL DCI (as for DL DCI).

Proposal #3: For the case when a PDSCH group is not scheduled in UE side and the PDSCH group corresponds to the T-DAI in UL DCI, one of the following alternatives is adopted.

- Alt 1: NFI value for the PDSCH group is assumed to be toggled from the latest value.

- Alt 2: NFI (for the PDSCH group) is signaled via the UL DCI (as for DL DCI).

- One-shot HARQ-ACK codebook

- Issue 2-1: HARQ-ACK codebook with NDI feedback

On the one-shot HARQ-ACK codebook configured with NDI inclusion, one remaining issue is the HARQ-ACK feedback composition in case when spatial HARQ-ACK bundling is configured. On this issue, following four alternatives can be considered where Alt 1/2/3 were the alternatives captured in FL’s summary document (R1-2001191) during RAN1#100.

- Alt 1: Spatial bundling is not applied to Type-3 HARQ-ACK codebook even if spatial HARQ-ACK bundling is configured.

- Alt 2: Spatial bundling is applied to Type-3 HARQ-ACK codebook, with AND operation for 2 HARQ-ACKs corresponding to 2 TBs and with 2 NDIs for 2 TBs.

- Alt 3: Spatial bundling is applied to Type-3 HARQ-ACK codebook, with AND operation for 2 HARQ-ACKs corresponding to 2 TBs and with AND operation for 2 NDIs corresponding to 2 TBs.

- Alt 4: Spatial bundling is applied to Type-3 HARQ-ACK codebook, with AND operation for 2 HARQ-ACKs corresponding to 2 TBs and with 1 NDI for the first TB.

In case of Alt 1, PUCCH overhead would increase and PUCCH coverage would decrease due to large UCI payload size (consisting of HARQ-ACK + NDI per each TB) on the PUCCH. In case of Alt 2, PUCCH overhead and coverage might be better than Alt 1, but NDI payload size would be (unnecessarily) larger rather than HARQ-ACK payload size. In case of Alt 3, there would be potential misalignment on the NDI value (and the corresponding HARQ-ACK) between UE and gNB in case when some DL scheduling DCI is missed by the UE.

On the other hand, with Alt 4, there would be no ambiguity on the NDI (and HARQ-ACK) between UE and gNB even in case of DCI missing by the UE, and PUCCH overhead/coverage would be improved compared to the above Alt 1/2. For these reasons, Alt 4 is preferred as the HARQ-ACK feedback composition in case configured with spatial bundling.

Proposal #4: For one-shot HARQ-ACK codebook with NDI inclusion, the following is adopted for the HARQ-ACK feedback composition in case configured with spatial bundling.

- 1-bit bundled HARQ-ACK (by AND operation for 2 HARQ-ACKs corresponding to 2 TBs) and 1-bit NDI for the first TB are mapped per PDSCH (HARQ process).

- Issue 2-2: HARQ-ACK codebook without NDI feedback

On the one-shot HARQ-ACK codebook not configured with NDI inclusion, one remaining FFS point is whether/how to specify UE behaviour for the cases where the UE has not yet obtained HARQ-ACK information for a TB corresponding to a scheduled PDSCH reception. On this issue, one of the following two alternatives could be adopted for the finalization.

- Alt 1: UE behavior is not specially/strictly specified for the above cases in the specification (then, it would be up to the UE).

- Alt 2: UE behavior is specified according to whether the NDI value for the TB is toggled (HARQ-ACK is reset to NACK in this case) or not toggled (HARQ-ACK is kept as previous report in this case).

Between the above two alternatives, Alt 2 is preferred in terms of reducing system resource overhead by avoiding unnecessary PDSCH retransmission in gNB side (especially, in case when ACK has been reported previously by UE for a TB, then after that, the NDI value corresponding to the same TB is received by the UE and is not toggled).

Proposal #5: For one-shot HARQ-ACK codebook without NDI inclusion, following UE behaviour is to be specified for the cases where the UE has not yet obtained HARQ-ACK information for a TB corresponding to a scheduled PDSCH reception.

- HARQ-ACK is reset to NACK if the NDI value for the TB is toggled.

- HARQ-ACK is kept as previous report if the NDI value is not toggled.

[TP for TS38.213 corresponding to Proposal #4 and Proposal #5]

### 9.1.4 Type-3 HARQ-ACK codebook determination If a UE is provided pdsch-HARQ-ACK-OneShotFeedback-r16, the UE determines a Type-3 HARQ-ACK codebook according to the following procedure. Set NcellsDL to the number of serving cells Set NHARQ,cDL to the value of nrofHARQ-ProcessesForPDSCH for serving cell c, if provided; else, set NHARQ,cDL=8 Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is not provided; else, set NTB,cDL=1 Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in Clause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG-r16 is provided; else, set NHARQ-ACK,cCBG/TB,max=0 Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI-r16 is provided; else set NDIHARQ=1 Set c=0 – serving cell index Set h=0 – HARQ process number Set t=0 – TB index Set g=0 – CBG index Set j=0 while c<NcellsDL while h<NHARQ,cDL if NDIHARQ=0 if NHARQ-ACK,cCBG/TB,max>0 while t<NTB,cDL while g<NHARQ-ACK,cCBG/TB,max = HARQ-ACK information bit for CBG g of TB t for HARQ process number h of serving cell c j=j+1 g=g+1 end while = NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit(s) for TB t for HARQ process number h on serving cell c, if any; else, g=0 j=j+1 t=t+1 end while else while t<NTB,cDL if harq-ACK-SpatialBundlingPUCCH is provided = a bundled HARQ-ACK information bit for the TBs for HARQ process h of serving cell c j=j+1 = NDI value for the first TB index 0 indicated in the DCI format corresponding to the HARQ-ACK information bit for HARQ process number h on serving cell c, if any; else, j=j+1 else = HARQ-ACK information bit for TB t for HARQ process h of serving cell c j=j+1 = NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit(s) for TB t for HARQ process number h on serving cell c, if any; else, j=j+1 t=t+1 end while end if t=0 else if NHARQ-ACK,cCBG/TB,max>0 while t<NTB,cDL if UE has reported HARQ-ACK information for TB t for HARQ process number h on serving cell c, and has not subsequently detected a DCI format scheduling a PDSCH reception, or received a SPS PDSCH, with TB t for HARQ process number h on serving cell c while g<NHARQ-ACK,cCBG/TB,max j=j+1 g=g+1 end while end if if UE has obtained HARQ-ACK information for TB t for HARQ process number h on serving cell c corresponding to a PDSCH reception and has not reported the HARQ-ACK information corresponding to the PDSCH reception while g<NHARQ-ACK,cCBG/TB,max = HARQ-ACK information bit for CBG g of TB t for HARQ process number h of serving cell c j=j+1 g=g+1 end while end if g=0 t=t+1 end while else while t<NTB,cDL if UE has reported HARQ-ACK information for TB t for HARQ process number h on serving cell c and has not subsequently detected a DCI format scheduling a PDSCH reception, or received a SPS PDSCH, with TB t for HARQ process number h on serving cell c = NACK j=j+1 t=t+1 end if if UE has obtained HARQ-ACK information for TB t for HARQ process number h on serving cell c corresponding to a PDSCH reception and has not reported the HARQ-ACK information corresponding to the PDSCH reception = HARQ-ACK information bit for TB t for HARQ process h of serving cell c j=j+1 t=t+1 end if if UE has not yet obtained HARQ-ACK information for TB t for HARQ process number h on serving cell c corresponding to a PDSCH reception if the NDI value for the TB indicated in the DCI format is toggled = NACK else = HARQ-ACK information bit for the TB reported previously end if end while end if t=0 end if h=h+1 end while h=0 c=c+1 end while […]

- Issue 2-3: HARQ-ACK triggering by DCI without PDSCH

According to the following agreements made in RAN1#99, one-shot HARQ-ACK codebook can be triggered by either the DL DCI with PDSCH scheduling or the DL DCI without PDSCH scheduling, based on the indicated FDRA value.

Agreement: If a UE is configured to monitor feedback request for one-shot HARQ-ACK codebook feedback: - The feedback can be requested in DL DCI 1_1 with the introduction of 1 bit (present if one-shot HARQ-ACK feedback is configured to the UE) for triggering the feedback - UE determines the PUCCH for one-shot HARQ-ACK feedback from K1, PRI, TPC - FFS: Use of NFI for determining PUCCH - Note: as per Rel-15 behaviour, the feedback can be piggybacked on PUSCH Agreement: If a UE is configured to monitor feedback request for one-shot HARQ-ACK codebook feedback and the feedback is requested in DL DCI 1_1 - This DL DCI can either schedule or not schedule a PDSCH - Working assumption: One value of the frequency domain resource assignment field indicates that this DCI does not schedule a PDSCH - If the DL DCI does not schedule a PDSCH, the HARQ process ID and NDI fields are ignored by the UE - If UE is triggered to report both one-shot and other HARQ-ACK feedback in the same slot, the UE reports only the one-shot feedback.

Regarding the one-shot HARQ-ACK triggering by the DCI without PDSCH, there are three remaining issues to be decided as below.

- Issue #1

  - How to indicate no PDSCH scheduling by FRDA field in the DL DCI triggering one-shot HARQ-ACK feedback transmission

    - For example, decide to use the FRDA value consisting of all ‘0’ for RA type 0 and the FDRA value consisting of all ‘1’ for RA type 1

- Issue #2

  - How to apply the K1 value indicated in the DCI in terms of the reference slot corresponding to K1=0 for determination of the slot with HARQ-ACK feedback

    - For example, decide to set the reference slot as the slot with the DCI transmission or the slot corresponding to the TDRA value indicated in the DCI

- Issue #3

  - How to define the (minimum) UE processing time between the trigging DCI without PDSCH scheduling and the corresponding one-shot HARQ-ACK feedback

    - For example, decide which of the existing UE processing time (e.g., N1 for PDSCH, N2 for PUSCH, N for SPS release) is used as the reference for this case

Proposal #6: For one-shot HARQ-ACK triggering by DL DCI without PDSCH scheduling, following three issues need to be decided.

- Issue 1: How to indicate no PDSCH scheduling by FRDA field

- Issue 2: How to apply the K1 value indicated in the DCI

- Issue 3: How to define the (minimum) UE processing time

- Conclusions

In this contribution, the remaining issues on HARQ procedure for the NR-U operation were discussed, and the followings are proposed.

Proposal #1: For the case when CBG based PDSCH transmission is configured and T-DAI indication for the non-scheduled PDSCH group is configured for DL DCI, the following is adopted.

- Two T-DAI values are indicated for the non-scheduled PDSCH group.

  - One value corresponds to TB-based PDSCH.

  - The other value corresponds to CBG-based PDSCH.

Proposal #2: For the case when CBG based PDSCH transmission is configured and T-DAI indication for both or one of two PDSCH groups is configured for UL DCI, the following is adopted.

- Two T-DAI values are indicated per PDSCH group.

  - One value corresponds to TB-based PDSCH.

  - The other value corresponds to CBG-based PDSCH.

Proposal #3: For the case when a PDSCH group is not scheduled in UE side and the PDSCH group corresponds to the T-DAI in UL DCI, one of the following alternatives is adopted.

- Alt 1: NFI value for the PDSCH group is assumed to be toggled from the latest value.

- Alt 2: NFI (for the PDSCH group) is signaled via the UL DCI (as for DL DCI).

Proposal #4: For one-shot HARQ-ACK codebook with NDI inclusion, the following is adopted for the HARQ-ACK feedback composition in case configured with spatial bundling.

- 1-bit bundled HARQ-ACK (by AND operation for 2 HARQ-ACKs corresponding to 2 TBs) and 1-bit NDI for the first TB are mapped per PDSCH (HARQ process).

Proposal #5: For one-shot HARQ-ACK codebook without NDI inclusion, following UE behaviour is to be specified for the cases where the UE has not yet obtained HARQ-ACK information for a TB corresponding to a scheduled PDSCH reception.

- HARQ-ACK is reset to NACK if the NDI value for the TB is toggled.

- HARQ-ACK is kept as previous report if the NDI value is not toggled.

Proposal #6: For one-shot HARQ-ACK triggering by DL DCI without PDSCH scheduling, following three issues need to be decided.

- Issue 1: How to indicate no PDSCH scheduling by FRDA field

- Issue 2: How to apply the K1 value indicated in the DCI

- Issue 3: How to define the (minimum) UE processing time