3GPP TSG RAN WG1 #100-bis								R1-2002097

E-meeting, April 20th – 30th, 2020

Source:	CATT

Title:	Remaining issues on NR Positioning Measurements

Agenda item:	7.2.8.3

Document for:	Discussion and Decision

Introduction

With the progress in RAN1#99 and RAN1-#100e, most of the issues for Rel-16 NR UE/gNB positioning measurements were resolved [1]. The definitions of the Rel-16 positioning measurements are captured in TS 38.215 [2]. The parameters for supporting measurement configurations are summarized in [3]. In this contribution, we discuss some remaining issues from RAN1#100-e.

# Discussion of UE measurement capability

Following agreements were made on Rel-16 UE/gNB positioning measurements in RAN1#99[1]:

Agreement: The prior agreement is updated as given below: UE can be configured to measure and report up to N=8 (> 1) DL PRS RSRP measurements on different DL PRS resources from the same TRP - N=[3] FFS: N is a UE capability Agreement: UE can be configured for DL PRS processing according to the following table: Description; Maximum numbers for DL PRS resources; Values that can be signaled as part of UE Capability. Max number of frequency layers (X1); X1=4; Values = {1,4} FFS: other values. Max number of TRPs per frequency layer (X2); X2=64. Max number of PRS resource sets per TRP (X3) per frequency layer; X3=2; Values = {1,2}. Max number of Resources per PRS resource set (X4); X4=64; FFS: values. Max number of DL PRS Resources per UE (X5); NA; FFS: values. Max number of TRPs for all frequency layers (X6) per UE; 256; FFS: values. Max number of Resources per frequency layer (X7); NA; FFS: values. Note: This does not constrain in any way how features and feature sets are defined. The values in the table above may or may not be signalled to be different for different features or feature sets.

During the email discussion of Rel-16 NR UE featureError! Reference source not found., a list of feature groups and components related to NR positioning are proposed, including UE DL PRS processing and measurement capability, as shown in left column of the Table 1. Most of the FFSs remaining in above agreements are also discussed there.

At the moment, it is unclear whether the discussion of the remaining issues related to UE measurement and reporting capability will be fully covered in AI 7.2.11.8 for Rel-16 NR UE feature of NR positioning. Thus, we include our views on the DL PRS measurement and reporting capability in Table 1 of this document, just in case the discussion will take place in AI 7.2.8.3. In the meanwhile, we also include the discussion in our companion paper for AI 7.2.11.8, which includes our complete views on the parameters related to UE DL PRS processing and measurement capability, as well as SRS  of NR positioning [4].

Table 1.  UE DL PRS Measurement and Reporting Capability

| UE DL PRS Processing Capability proposed in Error! Reference source not found. | CATT’s Comments |
| --- | --- |
| ------- UE DL PRS RSRP Measurement Reporting Capability for DL-AoD ------- - Max number of DL PRS RSRP measurements on different PRS resources from the same TRP supported by the UE Values = {1, 2, 3, 4, 5, 6, 7, 8} - Support of inter-frequency DL PRS RSRP measurement report in RRC_CONNECTED state - [Support of SSB from serving cell as QCL Type C source of a DL PRS resource from serving cell] - [Support of SSB from neighbor cells as QCL Type C source of a DL PRS resource from neighbor cells] - [Support of SSB from serving cell as QCL Type D source of a DL PRS resource from serving cell] - [Support of SSB from neighbor cells as QCL Type D source of a DL PRS resource from neighbour cells] - [Support of a DL PRS resource from serving cell as QCL Type D source of another DL PRS resource from serving cell] - [Support of a DL PRS resource from neighbour cells as QCL Type D source of another DL PRS resource from neighbour cells] | Component 1. In RAN#99, the following agreement was made on DL PRS RSRP measurements. Agreement: The prior agreement is updated as given below: UE can be configured to measure and report up to N=8 (> 1) DL PRS RSRP measurements on different DL PRS resources from the same TRP - N=[3] FFS: N is a UE capability It is so far undecided whether N is a UE capability. In our view, there is no need to set N as an UE capability, since the UE will at least try to measure all DL PRS from one TRP, and can report N=8 DL PRS RSRP measurements if the UE detects 8 or more DL PRS. Component 2 - 8: Support. However, we should make it clear that QCLed signals should come from the same neighbour cell. Thus, we suggest add the following clarification: “from the same serving cell” “from the same neighbour cells” |
| ----------UE DL RSTD Measurement Reporting Capability for DL-TDOA ---------- - Max number of DL RSTD measurements per pair of TRPs. Values = {1, 2, 3, 4}. Note: This is a max number of DL RSTD measurements per pair of TRPs with each measurement between a different pair of DL PRS resources or DL PRS resource sets. All the RSTD measurements in a single report should have a single reference timing. - Support of inter-frequency DL RSTD measurement report in RRC_CONNECTED state - [Support of SSB from serving cell as QCL Type C source of a DL PRS resource] - [Support of SSB from neighbor cells as QCL Type C source of a DL PRS resource] - [Support of SSB from serving cell as QCL Type D source of a DL PRS resource] - [Support of SSB from neighbor cells as QCL Type D source of a DL PRS resource] - [Support of a DL PRS resource from serving cell as QCL Type D source of another DL PRS resource from serving cell] - [Support of a DL PRS resource from neighbor cells as QCL Type D source of another DL PRS resource from neighbor cells] - [Support of DL RSTD measurement quality metric] | Component 1, 2: Support Components 3 - 8: Support. However, we should make it clear that QCLed signals should come from the same neighbour cell. Thus, we suggest add the following clarification: “from the same serving cell” “from the same neighbour cells” Component 9: Support |
| ------------UE Rx-Tx Measurement Reporting Capability for Multi-RTT------------ - Support of UE Rx-Tx time difference measurement with serving cell - Support of UE Rx-Tx time difference measurement with neighbouring cells - Support of UE Rx-Tx time difference measurements across different positioning frequency layers for DL PRS processing Note: Covers scenario when DL PRS are processed across different DL PRS frequency layers associated with a given component carrier used for SRS for positioning - Max number of UE Rx – Tx time difference measurements per TRP DL PRS Resource Set/Resource Note: A UE can be configured to report multiple Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL PRS resource/resource set. The DL PRS resource/resource sets can be in different positioning frequency layers - [Support of UE Rx-Tx time difference measurement quality metric] - [Support of UE Rx-Tx time difference measurements across different component carriers for SRS for positioning. Note: Covers scenario when SRS for positioning is transmitted in different component carriers than the component carrier to which DL PRS is configured] | Components 1 – 5: Support Component 6: Not support. We assume Component 6 has two meanings: 1) Support UE to report one or more UE Rx–Tx time difference measurement where UE receives PRS from one carrier (or more carriers) and UE transmits SRS for positioning in a different carrier; 2) Support UE to report one or more UE Rx–Tx time difference measurement where UE receives PRS from one carrier (or more carriers) and UE transmits SRS for positioning in more than one different carrier. In our view, the first case is already covered by Component 3. For the second scenario, it was discussed in RAN1, but there is no agreement. Therefore, we proposed to delete Component 6 for now. |

Conclusions

At the moment, it is unclear whether all o the details related to UE measurement and reporting capability parameters will be fully covered in AI 7.2.11.8 for Rel-16 NR UE feature of NR positioning. Thus, in this document, we presented our views on DL PRS measurement and reporting capability in Table 1. In the meanwhile, we also include the similar discussion in our companion paper for AI 7.2.11.8, where we present our complete views related to the UE features for NR positioning, including DL PRS measurement capability as well as other capability issues, e.g., UE DL PRS processing capability and  SRS of NR positioning [4].

References

- 3GPP RAN1 Chairman’s Notes of TSG RAN WG1 Meeting #99.

- 3GPP TS 38.215, “Physical layer measurements”, v16.0.0

- R1-2001484, “RAN1 UE features list for Rel-16 NR after RAN1#100-E” , AT&T, NTT DOCOMO, INC. (Email discussion: [100e-LTE-Rel-16-UEFeatures] email discussion/approval of Rel-16 LTE UE feature)

- R1-2002073, “Discussion of Rel-16 UE features for NR positioning”, CATT

Appendix Proposed UE features for NR Positioning[3]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13a. NR E-CID | 13a-1 | NR E-CID DL SSB RRM measurements for NR Positioning | Prerequisite – support of SSB measurements for RRM (SS-RSRP, SS-RSRQ) based on Rel.15 for NR Positioning in Rel.16 Configuration and reporting of SSB RRM measurements (SS-RSRP, SS-RSRQ) based on Rel.15 for NR Positioning in Rel.16 Note: No new UE capability is assumed on top of the R15 capabilities. | [FG 1-1] | [No] | NA | UE signalling to facilitate E-CID NR Positining is not supported | TBD | [No] |  |  |  | Optional with capability signaling |
|  | 13a-2 | NR E-CID DL CSI-RS RRM measurements for NR Positioning | Prerequisite – support CSI-RS for RRM (CSI-RSRP, CSI-RSRQ) based on Rel.15 for NR Positioning in Rel.16 Configuration and reporting of CSI-RS for RRM (CSI-RSRP, CSI-RSRQ) for NR Positioning in Rel.16 Note: No new UE capability is assumed on top of the R15 capabilities. | [FG 1-4 FG1-5] | [No] | NA | UE CSI-RS for RRM measurements and signalling to facilitate E-CID NR Positining are not supported | TBD | [No] |  |  |  | Optional with capability signaling |
| 13b. NR DL AoD | 13b-1 | Reception of DL PRS and UE measurement reporting to facilitate NR DL AoD support | -------------- UE DL PRS Processing Capability of RSRP for DL AoD ------------- - Maximum DL PRS bandwidth in MHz, supported by UE. Values = [20, 50, 100, 200, 400] in MHz - Duration of DL PRS symbol in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. Values for T = [0.125, 0.25, 0.5, 1, 40, 80, 160, 320, 640, 1280] ms Notes: - UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value - UE DL PRS processing capability is defined for a single positioning frequency layer - UE DL PRS processing capability is agnostic to DL PRS comb factor configuration - FFS if UE DL PRS processing capability is agnostic to the configured SCS settings of DL PRS - FFS if reported values of T are the same across bands within a FR or across FRs - FFS cases w/ and w/o configuration of measurement gap - Max number of positioning frequency layers supported by UE for DL PRS RSRP measurement report. Values = {1, [2, 3], 4} - Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE for DL PRS RSRP measurement report. Values = {1,2} - Max number of DL PRS Resources per DL PRS Resource Set a UE can be configured for DL PRS RSRP measurement report Values = [1, 4, 8, 16, 32, 64] - Max number of DL PRS Resources supported by UE for DL PRS RSRP measurement report across all frequency layers, TRPs and DL PRS Resource Sets. Values = [64, 128, 192, 256, 512, 1024, 2048] - Max number of TRPs across all positioning frequency layers per UE for DL PRS RSRP measurement report. Values = [16, 32, 64, 128, 256] - Max number of DL PRS Resources per positioning frequency layer for DL PRS RSRP measurement report. Values = [32, 64, 128, 256, 512, 1024] - [Max number of DL PRS resources per TRP across all frequency layers. Value set: {4,8,16,32,64,128}] ------- UE DL PRS RSRP Measurement Reporting Capability for DL-AoD ------- - Max number of DL PRS RSRP measurements on different PRS resources from the same TRP supported by the UE Values = {1, 2, 3, 4, 5, 6, 7, 8} - Support of inter-frequency DL PRS RSRP measurement report in RRC_CONNECTED state - [Support of SSB from serving cell as QCL Type C source of a DL PRS resource from serving cell] - [Support of SSB from neighbor cells as QCL Type C source of a DL PRS resource from neighbor cells] - [Support of SSB from serving cell as QCL Type D source of a DL PRS resource from serving cell] - [Support of SSB from neighbor cells as QCL Type D source of a DL PRS resource from neighbour cells] - [Support of a DL PRS resource from serving cell as QCL Type D source of another DL PRS resource from serving cell] - [Support of a DL PRS resource from neighbour cells as QCL Type D source of another DL PRS resource from neighbour cells] |  | [No] | NA | UE measurements and signalling to facilitate DL-AoD NR Positining are not supported | TBD | [No] |  |  |  | Optional with capability signaling |
| 13c. NR DL-TDOA | 13c-1 | Reception of DL PRS and UE measurement reporting to facilitate NR DL-TDOA support | -------------- UE DL PRS Processing Capability of RSTD for DL-TDOA ---------- - Maximum DL PRS bandwidth in MHz, supported by UE. Values = [20, 50, 100, 200, 400] in MHz - Duration of DL PRS symbol in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. Values for T = [0.125, 0.25, 0.5, 1, 40, 80, 160, 320, 640, 1280] ms Notes: - UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value - UE DL PRS processing capability is defined for a single positioning frequency layer - UE DL PRS processing capability is agnostic to DL PRS comb factor configuration - FFS if UE DL PRS processing capability is agnostic to the configured SCS settings of DL PRS - FFS if reported values of T are the same across bands within a FR or across FRs - FFS cases w/ and w/o configuration of measurement gap - Max number of positioning frequency layers supported by UE for DL RSTD measurement report Values = {1, [2, 3], 4} - Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE for DL RSTD measurement report. Values = {1, 2} - Max number of DL PRS Resources per DL PRS Resource Set a UE can be configured for DL RSTD measurement report. Values = [1, 4, 8, 16, 32, 64] - Max number of DL PRS Resources supported by UE for DL RSTD measurement report across all frequency layers, TRPs and DL PRS Resource Sets. Values = [64, 128, 192, 256, 512, 1024, 2048] - Max number of TRPs across all positioning frequency layers per UE for DL RSTD measurement report. Values = [16, 32, 64, 96, 128, 256] - Max number of DL PRS Resources per positioning frequency layer for DL RSTD measurement report. Values = [32, 64, 128, 256, 512, 1024] - [Max number of DL PRS resources per TRP across all frequency layers. Value set: {4,8,16,32,64,128}] ----------UE DL RSTD Measurement Reporting Capability for DL-TDOA ---------- - Max number of DL RSTD measurements per pair of TRPs. Values = {1, 2, 3, 4}. Note: This is a max number of DL RSTD measurements per pair of TRPs with each measurement between a different pair of DL PRS resources or DL PRS resource sets. All the RSTD measurements in a single report should have a single reference timing. - Support of inter-frequency DL RSTD measurement report in RRC_CONNECTED state - [Support of SSB from serving cell as QCL Type C source of a DL PRS resource] - [Support of SSB from neighbor cells as QCL Type C source of a DL PRS resource] - [Support of SSB from serving cell as QCL Type D source of a DL PRS resource] - [Support of SSB from neighbor cells as QCL Type D source of a DL PRS resource] - [Support of a DL PRS resource from serving cell as QCL Type D source of another DL PRS resource from serving cell] - [Support of a DL PRS resource from neighbor cells as QCL Type D source of another DL PRS resource from neighbor cells] - [Support of DL RSTD measurement quality metric] | [13b] | [No] | NA | UE measurements and signalling to facilitate DL-TDOA NR Positionign are not supported | TBD | [No] |  |  |  | Optional with capability signaling |
| 13d. NR SRS for positioning | 13d-1 | Transmission of SRS for positioning | - Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 16}. Other values FFS - Max number of aperiodic SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Values = {64}, Other values FFS. - Max number of semi-persistent SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Values = {64}, Other values FFS. - Max number of periodic SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Values = {64}, Other values FFS. - Support of SSB for serving cell DL pathloss estimation and OLPC for SRS for positioning. - Support of SSB for neighbour cell(s) DL pathloss estimation and OLPC for SRS for positioning. - Support of DL PRS for serving cell DL pathloss estimation and OLPC for SRS for positioning. - Support of DL PRS for neighbour cell(s) DL pathloss estimation and OLPC for SRS for positioning. - Max number of pathloss estimates across all SRS resource sets for positioning in addition to the up to four pathloss estimates per serving cell specified for PUSCH/PUCCH/SRS transmission. It is indicated from the following set of values {0, [1], 4, 8, 16} - Spatial relation of SRS Resources for positioning with DL PRS Resources from serving cell - Spatial relation of SRS Resources for positioning with DL PRS Resources from neighbour cells - Spatial relation of SRS Resources for positioning with SSB from serving cell - Spatial relation of SRS Resources for positioning with SSB from neighbour cells - Spatial Relation of SRS Resources for positioning with other SRS resources - [Max number of active spatial relations including both DL PRS Resources and SSB. Values = [FFS]] |  | [Yes] | NA | UE signaling to facilitate UL-TDOA and multi_RTT NR positioning is not supported | TBD | [No] |  |  |  | Optional with capability signaling |
| 13e. NR Multi-RTT | 13e-1 | Reception of DL PRS and transmission of SRS for positioning to facilitate NR Multi-RTT support | ------------UE Rx-Tx Measurement Reporting Capability for Multi-RTT------------ - Support of UE Rx-Tx time difference measurement with serving cell - Support of UE Rx-Tx time difference measurement with neighbouring cells - Support of UE Rx-Tx time difference measurements across different positioning frequency layers for DL PRS processing Note: Covers scenario when DL PRS are processed across different DL PRS frequency layers associated with a given component carrier used for SRS for positioning - Max number of UE Rx – Tx time difference measurements per TRP DL PRS Resource Set/Resource Note: A UE can be configured to report multiple Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL PRS resource/resource set. The DL PRS resource/resource sets can be in different positioning frequency layers - [Support of UE Rx-Tx time difference measurement quality metric] - [Support of UE Rx-Tx time difference measurements across different component carriers for SRS for positioning. Note: Covers scenario when SRS for positioning is transmitted in different component carriers than the component carrier to which DL PRS is configured] | [13c, 13d] | [Yes] | NA | UE measurements and signalling to facilitate Multi-RTT NR Positioning are not supported | TBD | [No] |  |  |  | Optional with capability signaling |