3GPP TSG RAN WG1 #100bis-e	  R1-2002103

# e-Meeting, April 20th-30th, 2020

Title:	Draft reply LS on the starting point of MSGB window

Response to:	R1-2001511 (R2-2002298)

Release:	Rel-16

Work Item:	NR_2step_RACH-Core

Source:	Samsung [RAN1]

To:	RAN2

Contact Person:

#### Name:	Qi Xiong

E-mail Address:	q1005.xiong (at) samsung (dot) com

1. Overall Description:

RAN1 would like to thank RAN2 for the requests to verify whether the starting point for MSGB window is clear from RAN1 specs when the UE transmits only PRACH in a valid PRACH occasion if the PRACH occasion is not mapped to a valid PUSCH occasion. RAN1 would like to provide responses as below.

RAN1 finds there could be more cases causing UE to transmit PRACH only, and RAN1 concludes that:

- For a transmission of both PRACH and the corresponding PUSCH, UE determines the msgB window starting position by using end of corresponding PUSCH as reference point;

- For a transmission of only PRACH with a valid corresponding PUSCH but not transmitted, UE determines the msgB window starting position by using end of corresponding PUSCH as reference point;

- For a transmission of only PRACH without a valid corresponding PUSCH, UE determines the msgB window starting position by using end of the PRACH as reference point.

## 2. Actions:

To RAN3:

ACTION: RAN1 respectfully ask RAN2 to take the above information into account.

3. Date of Next TSG-RAN1 Meetings:

TSG-RAN1 Meeting #101-e       25th May  – 05th June  2020	Online

TSG-RAN1 Meeting #102	24th  – 28th     August 2020	Toulouse, FR