3GPP TSG RAN WG1 #100bis																																											R1-2002154

e-Meeting, April 20th – 30th, 2020

Agenda Item:		7.2.11.5

Source:		Samsung

Title:		UE features for URLLC/IIoT

Document for:		Discussion and decision

Introduction

In this contribution, it discusses about UE features for URLLC/IIoT.

Discussion

eURLLC

| 11-1a | Monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space | 1) Supports monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space | 11-1 | Yes | N/A |  | Per UE | [No] | [No] | support mixture of FDD/TDD and/or FR1/FR2 | FFS: Whether to split 11-1a into two rows as below: 11-1a: DCI format 1_2 with DCI format 1_1 in the same search space 11-1b: DCI format 0_2 with DCI format 0_1 in the same search space | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Regarding FFS, motivation is not clear. Given that DCI formats 0_2 and 1_2 can have a same size, a split of the DCI formats into two search space sets as in the FFS is actually counterproductive.

| 11-2 | Rel-16 PDCCH monitoring capability | 1) Supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span for combination (X, Y, ) 2) If UE reports the support of more than one combination of C(X, Y) for a given SCS, and if multiple combinations of C(X, Y) are valid for the span pattern, the maximum value of C of the valid combinations is applied 3) Supports the limit M on the maximum number of monitored PDCCH candidates per PDCCH monitoring span for combination (X, Y, ) 4) If UE reports the support of more than one combination of M(X, Y) for a given SCS, and if multiple combinations of M(X, Y) are valid for the span pattern, the maximum value of M of the valid combinations is applied 5) Capability on the number of CCs with Rel-16 PDCCH monitoring capability on all the serving cells. | 3-5b | Yes | N/A |  | [FSPC] FFS: Compoent 5) reported per UE | [N/A] | [N/A] |  | This capability is necessary for SCS 15 kHz and 30 kHz. FFS: Adding a component for “supported combination(s) (X, Y, ), which may depend on how to report C, M and (X, Y, ) A list of separate UE capabilities C(X, Y, ), M(X, Y, ) for processing capability #1; A list of separate UE capabilities C(X, Y, ), M(X, Y, ) for processing capability #2; For component 5), if UE supports carrier aggregation with more than [x] DL carriers with Rel-16 PDCCH monitoring capability on all the carriers, UE should report this capability. Value of x (can be < 4) is TBD. FFS: Whether to merge component 1) and 3), and accordingly merge component 2) and 4) FFS：Whether to add a capability for supporting 3 unicast PDSCH/PUSCH per slot separately for each minimum processing capability to match the number of spans for (4,3) pair | Optional with capability signalling Candidate value set for (X, Y): {(7, 3), (4, 3), (2, 2)} The value of C for combination (7, 3) for 15 kHz and 30 kHz is 56 FFS the value of C for combination (4, 3) and (2, 2) FFS the value of M for combination (7, 3), (4, 3) and (2, 2) Candidate value for component 5): { x, x+1, …, 16} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- For 3 unicast PDSCH/PUSCH per slot capability, it is not clear motivation in which (4,3) pair will provide 4 monitoring occasion in slot. It is sufficient to have 1/2/4/7 unicast PDSCH/PUSCH capability in current UE capability.

- For component 5), it will be per UE or per BC.

- No need to merge 1) and 3). There may be a scenario where C needs to increase while M still is same or smaller

| 11-3 | More than one PUCCH for HARQ-ACK transmission within a slot | 1) Supports sub-slot based HARQ-ACK feedback procedure. • A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot. • At least one sub-slot configuration for PUCCH can be UE specifically configured to a UE. • Supports a single configuration for PUCCH resource for all sub-slots in a slot. The starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot. Any sub-slot PUCCH resource is not across sub-slot boundaries. 2) Supported sub-slot configuration [3) Supported combinations of (A, B), where A is the minimum gap between sub-slots containing actual PUCCH transmissions measured from beginning to beginning of the sub-slots, including across slots, and B is the sub-slot duration, with both A and B in units of symbols] |  | Yes | N/A |  | [Per UE] | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | Candidate value set for component 2): { 7-symbol*2, 2-symbol*7 and 7-symbol*2} [Candidate value set for component 3): (A, B) = {(7, 7), (4, 2) and (7, 7), (2, 2) and (7, 7)}] FFS: Whether to keep component 3) and accordingly the above note for component 3) FFS: Any relationship between FG 11-3 and CBG-based PUSCH with minimum processing time capability #2? | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- No need for component 3 – a UE should be able to transmit PUCCH at least as often as receive PDSCH/transmit PUSCH. For 120 kHz, it is similar to transmitting PUCCH every 2 symbols for 15 kHz/30 kHz.

- It is preferable to have “Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot” instead of component 3).

| 11-4 | Two HARQ-ACK codebooks [with up to one sub-slot based HARQ-ACK codebook] simultaneously constructed for supporting PDSCH reception with different priorities at a UE | 1) Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed [with the restriction up to one sub-slot based HARQ-ACK codebook]. 2) Supports separate PUCCH configuration for different HARQ-ACK codebooks 3) Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. [4) Supports a DCI format (from the formats 0_1/1_1/0_2/1_2) scheduling PDSCH with different HARQ-ACK priorities or PUSCH with different priorities when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured per BWP] 5) Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and ‘codeBlockGroupTransmission” for different HARQ-ACK codebooks. [6) Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot] |  | Yes | N/A |  | [Per UE] FFS: FS | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | FFS: Whether and how to combine FG 11-4 and FG 12-1 FFS: For component 4), whether to separate DL priority and UL priority, and whether to separate DCI format 0_1/1_1 and DCI format 0_2/1_2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- It is not necessary to have components related to sub-slot configuration since 11-3 provide the related to capability. Two HARQ-ACK codebooks can be operated even in slot based PUCCH transmission.

- In this sense, component 6) should be removed here and can be moved into 11-3.

- For the first FFS, no need to combine. For the second FFS, OK to separate DCI format 0_1/1_1 and DCI format 0_2/1_2.

| [11-4x] | [Two sub-slot based HARQ-ACK codebooks simultaneously constructed for supporting PDSCH reception with different priorities at a UE]. | 1) Supports two sub-slot based HARQ-ACK codebooks with different priorities to be simultaneously constructed. 2) Supports separate PUCCH configuration for different HARQ-ACK codebooks 3) Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. 4) Supports a DCI format (from the formats 0_1/1_1/0_2/1_2) scheduling PDSCH with different HARQ-ACK priorities or PUSCH with different priorities when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured in USS per BWP 5) Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and ‘codeBlockGroupTransmission” for different HARQ-ACK codebooks. | 11-3 | Yes | N/A |  | [Per UE] | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | FFS: whether to add this FG and the contents of this FG | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- It is not necessary to have this feature since the combination of 11-3 and 11-4 can support this operation without introducing additional signaling.

| 11-5 | PUSCH repetition type B | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots. 2) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. [3) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field.] 4) PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 5) S and L are separately indicated (4-bit for S and 4-bit for L). L <= 14. [6) TBS is determined based on L indicated in TDRA table entry reusing Rel-15 mechanism.] [7) Handling of interaction with DL/UL directions depending on whether dynamic SFI is configured or not, including both cases with and without higher layer parameter InvalidSymbolPattern configured] [8) Supported maximum number of actual repetitions within a slot] [9) Supported PUSCH hopping scheme] |  | Yes | N/A |  | [Per UE] FFS: Per band | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | Candidate value for component 8): {2, 3, 4, 7, [8], [12]} FFS: Whether to add new feature groups for the total number of unicast PUSCHs for different TBs per slot per CC, or just add some note here with an example below: [The total number of unicast PUSCHs for different TBs per slot per CC is subjected to the capability reported by FG 5-12, 5-12a, 5-12b, 5-13d, 5-13e and 5-13f] FFS: Whether to set separate UE capabilities for dynamic grant and configured grant. Can we just add some note here with an example below for compromise? [PUSCH repetition type B with configured grant is applied only if UE reports the support of FG 5-19 or FG 5-20, and subjected to the capability of FG 5-19 and FG 5-20]. FFS: Whether to set separate UE capabilities for the case that dynamic SFI is configured and InvalidSymbolPattern is configured. Can we just add some note here with an example below for compromise? [The case that both dynamic SFI and InvalidSymbolPattern are configured is applied only if UE reports the support of FG3-6.] FFS: Whether to set separate UE capabilities for DCI format 0_1 and DCI format 0_2 for PUSCH repetition type B. Can we go majority view that no separate UE capability? | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- There is no need to explicitly list 3) and 6)

- It is preferable to have [8) Supported maximum number of actual repetitions within a slot].

| 11-9 | Multiple active configured grant configurations for a BWP of a serving cell | 1) Supports up to 12 configured/active configured grant configurations in a BWP of a serving cell. • Separate RRC parameters for different configured grant configurations • Separate activation for different configured grant Type 2 configurations • Separate release for different configured grant Type 2 configurations [2) Supported maximum number of configured grant configurations in a BWP of a serving cell] [3) Supported maximum number of configured grant configurations across all serving cells] |  | Yes | N/A |  | [Per UE] FFS: FSPC | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- It is not clear to make a restriction to the maximum number of configured grant per BWP per serving cell group because UE can choose one of multiple configured grants to send PUSCH if UE has limited PUSCH transmission capability.

IIoT

| 12-3 | SPS release by DCI format 1_1 and 1_2 | - Support of SPS release by DCI format 1_1 - Support of SPS release by DCI format 1_2 | downlinkSPS | Yes | N/A |  | Per UE | No | No | support mixture of FDD/TDD and/or FR1/FR2 | A UE supporting component 1 and 11-1 (DCI format 0_2/1_2) shall also support component 2 (SPS release by DCI format 1_2). | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- It should be separated into 1_1 and 1_2 because some UE may not support to monitor 1_2 depending on 11-1.