3GPP TSG RAN WG1 #100-bis		R1-2002179

e-Meeting, April 20th – April 30th, 2020

Agenda item:	6.2.4.1

Source: 	Qualcomm Incorporated

Title: 	Support of longer numerologies for rooftop reception

Document for:	Discussion and Decision

- Issue #1: MBSFN RS mapping to resource elements

The equation for mapping the MBSFN reference signal type 1 for 0.37 kHz subcarrier spacing doesn’t lead to the RS mapping pattern (for reference signal pattern type 1) that has been agreed in RAN199 (and reproduced below) for system bandwidths of 5 MHz and 15 MHz:

Agreement:

The working assumption on D_t = 4 and D_f = 3 is confirmed with the following clarifications:

- The pattern is per the figure below:

To illustrate the issues with the current text in the specifications in subclause 6.10.2.2.4, consider a 5 MHz system, where  NRBDL=25, NSCRB=486, ∆=NRBmax,DL-NRBDL2=42.5 (where NRBmax, DL=110). With the current resource element mapping per subclause 6.10.2.2.4 for MBSFN reference signal type 1:

- For ns=0 (first slot in a stagger of 4 consecutive slots):

  - k=12m'-NscRB12Δ=12(m'-1700),  m'=0,1,…,NscRB12NRBmax,DL-1

  - Hence, the values of k in the desired range are 0,12,24,36,…

- For ns=1 (second slot in a stagger of 4 consecutive slots):

  - k=12m'-NscRB12Δ+3=12m'-1700+3,  m'=0,1,…,NscRB12NRBmax,DL-1

  - Hence, the values of k in the desired range are 3,15,27,39,…

- For ns=2 (third slot in a stagger of 4 consecutive slots):

  - k=12m'-NscRB12Δ+6=12m'-1742.5+6,  m'=0,1,…,NscRB12NRBmax,DL-1

  - Hence, the values of k in the desired range are 0,12,24,36,…

- For ns=3 (fourth slot in a stagger of 4 consecutive slots):

  - k=12m'-NscRB12Δ+9=12m'-1742.5+9,  m'=0,1,…,NscRB12NRBmax,DL-1

  - Hence, the values of k in the desired range are 3,15,27,39,…

This leads to the following RS mapping to resource elements (in Fig. 1 below), which is not in line with the agreements.

Figure 1: Illustration of RS pattern for MBSFN reference signal pattern type 1 Δf=0.37 kHz according to current specifications (for 5 MHz system bandwidth).

Similar erroneous mapping occurs for a 15 MHz system, owing to the use of ⌊.⌋ and ⌈.⌉ operations for different slots within a stagger of 4 slots.

Towards fixing this, note that the complications in the RE mapping occurs because NscRB (=486) is not divisible by the RS periodicity in frequency (12 subcarriers) in a given PMCH symbol, for the numerology with Δf=0.37 kHz with reference signal pattern type 1 (Td=4)—i.e., the mapping is not identical from one RB to the next. However, also note that 2NscRB (=972) is exactly divisible by the RS periodicity in frequency, for this case. As a result, there may be instances where (specifically, for odd values of NRBDL, e.g., for 5 MHz, 15 MHz system bandwidths), the number of RS tones in the first two slots ns mod 4∈0,1 of a four-slot stagger may have one more RS tone than the last two slots ns mod 4∈0,1  in the stagger. To capture this fact, and choose the tones (corresponding to m') from the centre of the underlying RS sequence generated in subclause 6.10.2.1.4  (over NRBmax,DL RBs), is indeed the intention of the present text; yet, as we demonstrated above, the depiction is not correct.

Observe that for even values of NRBDL,  the number of RS tones in all slots of a four-slot stagger is given by NscRB12NRBDL.

Hence, for even values of NRBDL, the equation for the subcarrier index k corresponding to the RS tones can be written as

kalleven= 12m'-12NscRB12NRBmax,DL-NRBDL+3ns mod 4

= 12m'-NscRB12Δ+3ns mod 4

Note that the use of the ⌊.⌋ operation takes care of the fact that there may be an odd number of “remaining” tones in the underlying sequence, when selecting the ones from the middle, to form the RS tones for a given system. Note also, that there is no occurrence of the different cases based on the slot number, as in the current text.

Turning our attention now to the more involved case for odd values of NRBDL, we observe that the number of RS tones in the first two slots of a four-slot stagger ns mod 4∈0,1 is given by NscRB12NRBDL, while for the second two slots, the number of RS tones is given by  NscRB12NRBDL-1.

We can straightaway see that the above is leading towards a more involved expression—one that is not straightforwardly expressible in terms of Δ, for starters.

For the first two slots of the stagger (ns  mod  4∈{0,1}), the subcarrier index k0,1odd may be expressed as

k0,1odd= 12m'-12NscRB12 NRBmax,DL- NscRB12NRBDL+3(ns mod 4)

For the last two slots of the stagger (ns  mod 4∈{2,3}), we similarly have

k2,3odd= 12m'-12NscRB12NRBmax,DL- NscRB12NRBDL-1+3(ns mod 4)

As a result, the mapping equation should have three distinct cases—first, where NRBDL is even; second, where NRBDL is odd and the slot numbers correspond to  ns mod 4∈0,1; third, where NRBDL is odd and the slot numbers correspond to ns mod 4∈2,3.

With the discussion for reference signal pattern type 1 above, one can also observe that a similar change as for the even NRBDL case needs to be made for the reference signal pattern type 2 RE mapping (note that for type-2, the more involved descriptions for odd NRBDL do not apply, since NscRB is divisible by the frequency periodicity of the RS in a PMCH symbol). As per the current equation, the 0th slot’s RE mapping doesn’t start from k=0 when NRB is odd. Hence, for pattern type 2, the value(s) of subcarrier index k should be given by

ktype-2=6m'-NscRB6Δ+3ns mod 2

Furthermore, there is—what we believe—a typo, as it relates to the “range” of values k can take. In the current text, it appears as if the range of values for k is the same for both reference signal pattern type 1 and type 2, which is not the case. In each PMCH symbol, there will be twice as many RS tones for pattern type 2 than for pattern type 1.

In TP1 below, we fix the discussed issues, as it relates to the MBSFN reference signals.

<TP1, 6.10.2.2.4>

The reference-signal sequence rlm' in OFDM symbol l shall be mapped to complex-valued modulation symbols ak,l(p) with p=4 according to

ak,l(p)=rl,nsm'

0≤k<NscRB12NRBDL

where  ns is the 3ms absolute slot number, defined as ns= ns+13nf/4, ns is the 3 ms slot number as defined in clause 4.1 and

-	for MBSFN reference signal pattern type 1

0≤k<NscRB12NRBDL

l=0

m'=0,1,…,NscRB12NRBmax,DL-1

  - For NRBDL odd

k=12m'-12NscRB12 NRBmax,DL- NscRB12NRBDLNscRB12Δ+3ns mod 4if ns mod 4∈0,112m'-12NscRB12NRBmax,DL- NscRB12NRBDL-1NscRB12Δ+3ns mod 4if ns mod 4∈2,3
l=0
Δ=NRBmax,DL-NRBDL2
m'=0,1,…,NscRB12NRBmax,DL-1

  - For NRBDL even

k= 12m'-12NscRB12NRBmax,DL-NRBDL+3ns mod 4

-	for MBSFN reference signal pattern type 2

0≤k<NscRB6NRBDL

k=6m'-NscRB6Δ+3ns mod 2
l=0
Δ=NRBmax,DL-NRBDL2
m'=0,1,…,NscRB6NRBmax,DL-1

</TP1>

Proposal 1: Endorse TP1

# Issue#2: Warm-up subframes and MCCH decoding

The structure for signalling the MBSFN subframes for a given MBSFN area is as follows:

- SIB13 includes in mcch-Config the subframes in which the UE decodes MCCH for a given MBSFN area.

- Within MCCH, the IE commonSF-Alloc includes the subframes allocated to that MBSFN area.

Thus, before decoding MCCH, the UE is not aware of whether the subframes/slots before MCCH belong to the same MBSFN area. For the 0.370kHz numerology, this means that the UE cannot estimate the full channel, since that requires the UE to get MBSFN-RS from previous subframes, but current specification does not allow the UE to assume that those subframes belong to the same MBSFN subframe.

In the general case, if a UE is configured to decode PMCH with 0.370kHz numerology in absolute slot ns, the UE should be able to obtain the full channel by causal channel estimation, i.e., for Td=2 the UE may assume that MBSFN-RS for the MBSFN area are present in ns-1, and for Td=4 the UE may assume that MBSFN-RS for the MBSFN area are present in ns-{1,2,3}.

This issue was brought up in the previous meeting, and some concerns were raised. We try to address them below:

Concern 1: Maybe only a couple of very first symbols which has no preceding RS has performance degraded but the overall performance may not be so bad

Answer: The observed degradation is close to ~7dB from system level evaluation. Also, when decoding MCCH, the UE does not know what is around it, so if this is not decodable the UE cannot bootstrap the system and move to cross-slot channel estimation.

Concern 2: eNB would not schedule MCCH in the first 3 slots where only MBSFN-RS is transmitted instead. This is where NW performance loss come from in terms of spectral efficiency. The three slots are always vacant for MBSFN-RS transmission

Answer: Actually, this is not the case. Those first 3 slots will have MTCH information, but the UE doesn’t know whether those subframes belong to the same MBSFN area before decoding MCCH. Note that the signalling structure is as follows:

- SIB13 includes the following information, which indicates the subframe/slots in which MCCH is received. At this point, the UE doesn’t know what are the other slots/subframes belonging to the same MBSFN area as the MCCH (i.e., the UE only “sees” the MCCH subframes/slots):

MBSFN-AreaInfoList-r16 ::=		SEQUENCE (SIZE(1..maxMBSFN-Area)) OF MBSFN-AreaInfo-r16

MBSFN-AreaInfo-r16 ::=				SEQUENCE {

mbsfn-AreaId-r16					MBSFN-AreaId-r12,

notificationIndicator-r16			INTEGER (0..7),

mcch-Config-r16						SEQUENCE {

mcch-RepetitionPeriod-r16			ENUMERATED {rf1, rf2, rf4, rf8, rf16, rf32, rf64,

rf128, rf256, spare7},

mcch-ModificationPeriod-r16			ENUMERATED {rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128,

rf256, rf512, rf1024, spare5},

mcch-Offset-r16					INTEGER (0..10),

sf-AllocInfo-r16				BIT STRING (SIZE(10)),

signallingMCS-r16				ENUMERATED {n2, n7, n13, n19}

},

subcarrierSpacingMBMS-r16		ENUMERATED {kHz7dot5, kHz2dot5, kHz1dot25, kHz0dot37, spare4},

timeSeparation-r16				ENUMERATED {sl2, sl4} OPTIONAL,	-- Need OR

...

}

- MCCH includes which other subframes/slots are in the same MBSFN area:

MBSFNAreaConfiguration-r9 ::=		SEQUENCE {

commonSF-Alloc-r9					CommonSF-AllocPatternList-r9,

commonSF-AllocPeriod-r9				ENUMERATED {

rf4, rf8, rf16, rf32, rf64, rf128, rf256},

pmch-InfoList-r9					PMCH-InfoList-r9,

nonCriticalExtension				MBSFNAreaConfiguration-v930-IEs	OPTIONAL

}

Concern 3: It means impliedly that NW may use the subframe not configured for MBSFN for other purpose, e.g., unicast transmission.

Answer: In our view, the usefulness of transmitting unicast in a carrier that supports this numerology is very reduced. But if this is a concern, we can limit the applicability to dedicated MBMS (not included in the TP).

<TP2, 36.211 6.10.2>

### 6.10.2	MBSFN reference signals

MBSFN reference signals shall be transmitted in the MBSFN region of MBSFN subframes only when the PMCH is transmitted. MBSFN reference signals are transmitted on antenna port 4.

For a PMCH transmitted with 0.37 kHz subcarrier spacing in slot n,

-	for MBSFN reference signal pattern type 1, the UE may assume that MBSFN reference signals associated with the same NIDMBSFN are present in the three preceding slots to slot n.

-	for MBSFN reference signal pattern type 2, the UE may assume that MBSFN reference signals associated with the same NIDMBSFN are present in the preceding slot to slot n.

MBSFN reference signals are defined for extended cyclic prefix only.

</TP2>

Proposal 2: Endorse TP2

# Issue#3: Language on TBS scaling

In RAN1#98b, the following agreement was made regarding the Transport Block Size (TBS) determination for the MBSFN numerology with Δf=0.37 kHz, wherein, we draw special emphasis to the highlighted part below:

Agreement:

For the TBS determination for the 300us CP numerology, the procedure for calculating the TBS is as follows:

- Reuse the modulation and TBS index table in TS 36.213 and according to the signalled IMCS, obtain ITBS

  - FFS whether modifications to the legacy modulation and TBS index table are needed (to be checked once the RS pattern is agreed)

- Reuse the transport block size table in TS 36.213 and according to the obtained ITBS, obtain the corresponding TBS

- Calculate the used TBS’ as TBS'=round{TBS ×α}, where the round operation maps TBS ×α to the closest TBS in the legacy transport block size tables

  - In case two TBSs are at the same distance of TBS ×α, the larger TBS is chosen

  - The value of α is fixed in the specification to α=3

Selecting the scaled TBS in case of a “tie” between the smaller and larger TBS in terms of distance from the scaled TBS is implemented in the present text of the specifications in subclause 11.1 of TS 36.213 as follows:

“For PMCH with  the UE shall scale the derived transport block size by , then round half-way towards positive infinity to the closest valid transport block size…”

In our view, the language “round half-way towards positive infinity” is a rather convoluted, and potentially confusing way of implementing the highlighted part of the agreement. We propose to simplify this language; TP3 below captures our proposed re-wording. We also propose to parse the very long sentence for TBS determination corresponding to whether the 256QAM table is used or not.

<TP3, 36.213. 11.1>

The UE shall then follow the procedure in Subclause 7.1.7.2.1 to determine the transport block size, assuming is equal to. For PMCH with  the UE shall scale the derived transport block size by , then round to the closest valid transport block size in the union of Table 7.1.7.2.1-1 and Table 7.1.7.2.4-1 by including, in the rounding procedure, only the TBS entries in Table 7.1.7.2.1-1 with ITBS≤33B and the entries in 7.1.7.2.4-1 for which the TBS_L1 is present in Table 7.1.7.2.1-1 with ITBS≤33B if the UE is configured by higher layers to decode the PMCH based on QPSK, 16QAM, 64QAM, and 256QAM; and only the TBS entries in Table 7.1.7.2.1-1 with ITBS≤26A and the entries in 7.1.7.2.4-1 for which the TBS_L1 is present in Table 7.1.7.2.1-1 with ITBS≤26A otherwise. In case the scaled TBS has the same distance to two valid transport block sizes, it is rounded to the larger transport block size. The UE shall set the redundancy version to 0 for the PMCH.

</TP3>

Proposal 3: Endorse TP3