3GPP TSG RAN WG1 #100bis-e	         	 R1- 2002192

e-Meeting, April 20th – 30th, 2020

Agenda item:		7.2.2.1.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining Issues on UL Signals and Channels for NR-U

Document for:		Discussion and Decision

# 1	Introduction

NR Work Item Description [1] on the NR-based access to unlicensed spectrum was approved in RAN plenary #84. The related work was concluded for the most part in RAN1#99. In this contribution we discuss the few remaining open issues related to UL signals and channels for NR Unlicensed, and point out some errors and discrepancies in the newly approved Rel-16 specifications.

# 2. Corrections

TS 38.213

Issue #1 (Issue #11 in FL Summary R1-2001203):

Section 9.2.2 defines the PUCCH formats for UCI transmission. There are some discrepancies though, as pointed out below in the corresponding text proposal:

---------- Start of Text Proposal -------

### 9.2.2	PUCCH Formats for UCI transmission

If a UE is not transmitting PUSCH, and the UE is transmitting UCI, the UE transmits UCI in a PUCCH using

-	PUCCH format 0 if

-	the transmission is over 1 symbol or 2 symbols,

-	the number of HARQ-ACK information bits with positive or negative SR (HARQ-ACK/SR bits) is 1 or 2

-	PUCCH format 1 if

-	the transmission is over 4 or more symbols,

-	the number of HARQ-ACK/SR bits is 1 or 2

-	PUCCH format 2 if

-	the transmission is over 1 symbol or 2 symbols,

-	the number of UCI bits is more than 2

-	PUCCH format 3 if

-	the transmission is over 4 or more symbols,

-	the number of UCI bits is more than 2,

-	the PUCCH resource does not include an orthogonal cover code or the UE is provided useInterlacePUCCH-Dedicated-r16

-	PUCCH format 4 if

-	the transmission is over 4 or more symbols,

-	the number of UCI bits is more than 2,

-	the PUCCH resource includes an orthogonal cover code and the UE is not provided useInterlacePUCCH-Dedicated-r16.

---------- End of Text Proposal -------

Issue #2: CP extension for SRS (Issue #13 in FL Summary R1-2001203):

As discussed in a companion contribution R1-2002193, in RAN1#99, the details of using CP extension prior to UL transmission were agreed, e.g.:

Agreement:

For fallback DL assignments scheduling UL transmissions (e.g. PUCCH):

- LBT type, and the length of the CP extension are jointly encoded in the DL assignment and signaled with 2 bits.

- The supported combinations of LBT type and length of the CP extension are hardcoded in RAN1 specs

Related clarification to 38.211 were agreed in RAN1#100-e. However, it was left open whether to support CP extension also with SRS.

SRS transmissions in NR can be triggered dynamically (i.e. aperiodic SRS), using a DL assignment (DCI 1_1, 1_2), a UL grant (0_1, 0_2), or DCI format 2_3. Furthermore, periodic SRS configuration is also supported (bot (periodic or semi-persistent SRS).

We note that the in the case of Aperiodic SRS, the triggering DCIs 1_1 and 0_1 already include the “ChannelAccess-CPext-CAPC”-field, and hence it becomes straight forward to apply CP extension to SRS as well. The other DCIs capable of triggering SRS do not presently support indication of CP extension. The latest version of 38.212 and 38.214 seem to already support this behavior, and hence the only necessary specification change is in 38.211.

Proposal 2 When Aperiodic SRS is triggered with a DCI (0_1, 1_1) that also includes indication of CP extension, the CP extension applies to SRS as well.

A corresponding TP is embedded below.

-------- Beginning of Text Proposal ------------

In case of cyclic prefix extension of the first OFDM symbol l allocated for PUSCH, SRS, or PUCCH transmission, the time-continuous signal sext(p,μ)t for the interval tstart,lμ-Text≤t<tstart,lμ preceding the first OFDM symbol for PUSCH, SRS, or PUCCH is given by

sext(p,μ)t=sl(p,μ)t

where t<0 refers to the signal in the previous subframe and

-	for dynamically scheduled PUSCH, SRS, and PUCCH transmissions

Text=minmaxText',0, Tsymb,(l-1)mod7∙2μμ

Text'=k=1CiTsymb,  l-kmod 7∙2μ μ-Δi

-	where Δi is given by Table 5.3.1-1 with C1=1 for μ∈0,1, C1=2 for μ=2, and C2 and C3 given by the higher-layer parameters cp-ExtensionC2-r16 and cp-ExtensionC3-r16, respectively, and TTA given by clause 4.3.1. For contention-based random access, or in absence of higher-layer configuration of C2 and C3, the value of Cishall be set to the largest integer fulfilling Text'<Tsymb,  (l-1)mod7∙2μ μ for each of the values of i∈2,3.

-	for a PUSCH transmission using configured grant, Text is given by the procedure in [6, TS 38.214].

The starting position of OFDM symbol  for subcarrier spacing configuration in a subframe is given by

Table 5.3.1-1: The variables Ci and Δi for cyclic prefix extension

| Text index i | Ci | Δi |
| --- | --- | --- |
| 0 | - | - |
| 1 | C1 | 25∙10-6 |
| 2 | C2 | 16∙10-6+TTA |
| 3 | C3 | 25∙10-6+TTA |

-------- End of Text Proposal ------------

# 3. Conclusions

In this contribution we point out issues related to UL signals and channels:

Issue #1: There are some issues related to how PUCCH format selection works in TS 38.213, Section 9.2.2

- This corresponds to the issue #11 in FL Summary R1-2001203

Issue #2: Use of PC extension for SRS: When Aperiodic SRS is triggered with a DCI (0_1, 1_1) that also includes indication of CP extension, the CP extension applies to SRS as well.

- This relates to the issue #13 in FL Summary R1-2001203

# References

- TR 38.899 Study on NR-based access to unlicensed spectrum