3GPP TSG-RAN WG1 Meeting #100bis-e	R1- 2002237

e-Meeting, April 20th - April 30th, 2020

Agenda Item:	7.2.4.4

Source:	Ericsson

Title:	In-device coexistence between LTE and NR sidelinks

Document for:	Discussion, Decision

# 1	Introduction

So far RAN1 has made the following agreements on in-device coexistence of LTE SL and NR SL:

RAN1#96bis Conclusion: - RAN1 does not see any specification impact for support of Long Term Time-Scale TDM for coexistence of NR and LTE sidelinks Working assumption: - For Tx/Tx overlap, - If packet priorities of both LTE and NR sidelink transmissions are known to both RATs prior to time of transmission subject to processing time restriction, then the packet with a higher relative priority is transmitted - In case the priorities of LTE and NR SL transmissions are the same, then it is up to UE implementation as to which transmission is chosen (e.g., taking into account congestion, etc.) - If packet priorities of both LTE and NR sidelink transmissions are not known to both RATs prior to time of transmission subject to processing time restriction, then it is up to UE implementation to manage Tx/Tx overlaps (e.g., LTE transmissions are always prioritized, etc.) - RAN1 does not assume any impact to LTE physical layer specifications RAN1#97 Agreements: - For Tx/Tx overlap, - Confirm the working assumption made in RAN1#96bis - UE capability is defined for short-term time-scale TDM for in-device coexistence Agreements: - For Rx/Rx overlap, - Up to UE implementation to manage receptions of LTE and NR sidelinks. RAN1#98 Agreements: Unless packet priorities of both LTE and NR sidelink are known to both RATs prior to time of collision (subject to processing time restriction), then - It is up to UE implementation to handle LTE Tx/NR Rx overlap. - It is up to UE implementation to handle NR Tx and LTE Rx overlap. Agreements: - RAN1 understand that NR V2X priority field and PPPP are directly comparable i.e. the same numerical value has the same meaning in both the RATs. - Ask SA2 to confirm the understanding. If understanding is incorrect, please provide solution. RAN1#98bis Agreements: - For Tx/Rx overlap, - If packet priorities of both LTE and NR sidelinks are known to both RATs prior to time of transmission/reception (subject to processing time restrictions), then the packet with a higher relative priority is transmitted/received - In case the priorities of LTE and NR sidelink packets are the same, then it is up to UE implementation as to which packet is transmitted/received Agreements: - For sidelink synchronization signal/channel (including S-SSB and LTE SLSS/PSBCH) priority for a UE is (pre)-configured per UE - The (pre)-configured priority is used in the same way as the priority for other channel/signals w.r.t. prioritization for handling in-device co-existence - Note: it is understood that the same priority (pre)-configuration is intended for all the related UEs - The priority of PSFCH is set as the priority of the corresponding PSSCH. Agreements [98b-NR-18] - UE reports its capability to the network of whether it supports short-term time scale TDM solutions. - Resource allocation related information is not reported to other RAT. RAN1#99 (Reno, US) Agreements: - When NR multiple transmissions (if supported) are overlapped with LTE SL TX/RX and if these NR multiple transmissions have different priorities (which are known in advance to the UE), the highest priority value of NR multiple transmissions is used for comparing that of LTE SL TX/RX and then SL operation with a higher relative priority is performed. Agreements: - In-device coexistence conflicts for network-controlled modes are addressed in the same way as for UE-autonomous modes - No addition spec is expected

We believe that the only thing left that needs to be clarified is what is referred to by the priorities mentioned in the above agreements. To this end, it is most straightforward to use the priority carried by the SCI for this purpose.

Priority indicated by the SCI is used in the prioritization rules for in-device coexistence.

# 2	Conclusion

In the previous sections we proposed the following:

Proposal 1	Priority indicated by the SCI is used in the prioritization rules for in-device coexistence.