3GPP TSG RAN WG1 #100bis			                 R1-2002246

e-Meeting, April 20th – 30th, 2020

Source: 	ETRI

Title: 	UL signals and channels

Agenda Item:	7.2.2.1.3	UL signals and channels

Document for:	Discussion

- Introduction

In this contribution, we address our views about remaining issues focusing on SRS transmissions.

- Discussion

  - Time resource for SRS transmission

Agreement: - For the CP extension prior to at least a dynamically scheduled PUSCH transmission, the CP extension is located in the symbol(s) immediately preceding the PUSCH allocation indicated by SLIV. The supported durations for CP extension at the UE are: - 0 (i.e. no CP extension) - C1*symbol length – 25 us - C2*symbol length – 16 us - TA - C3*symbol length – 25 us – TA - C1=1 for 15 and 30 kHz SCS, C1=2 for 60 kHz SCS - FFS: Whether C2/C3 is fixed or implicitly derived based on TA for each subcarrier spacing - The N2 timeline (UL grant to PUSCH delay) needs to be relaxed to take the CP extension into account - FFS: Whether the limit as per the previous agreement bounding the resulting CP extension to be less than or equal to one symbol for the given subcarrier spacing should be relaxed - FFS: Applicability of this to other UL transmissions - FFS: Whether the number of durations for CP extension that the UE can be signalled dynamically can be configured

We would propose to give more flexibility, when SRS request is triggered by UE-specific DCI, the field ChannelAccess-CPext-CAPC can be also applicable to the SRS transmission. The previous meeting (#99) has agreements applicable to PUSCH and PUCCH, i.e., the field ChannelAccess-CPext-CAPC is applied to the SRS transmission. According to the agreement, C1 is a fixed value and C2/C3 can be configured.

In our understanding, the TDM of SRS resource and other UL transmissions is not clearly stated in the current specification. The paragraph in TS 37.213-g10 below says about the Type 1 channel access for SRS, and nothing is mentioned in the Type 2 channel access. In our understanding, we need some additional discussion to support TDM of SRS resource and other UL transmissions.

TS 37.213-g10 section 4.2.1 … A UE shall use Type 1 channel access procedures for transmitting SRS transmissions not including a PUSCH transmission. UL channel access priority class p=1 in Table 4.2.1-1 is used for SRS transmissions not including a PUSCH. If a UE is scheduled by an eNB/gNB to transmit PUSCH and SRS in contiguous transmissions without any gaps in between, and if the UE cannot access the channel for PUSCH transmission, the UE shall attempt to make SRS transmission according to uplink channel access procedures specified for SRS transmission. …

The CP extension can also be adopted to multiplex PUSCH by TDM, which is illustrated in the Figure 1. Since SRS symbols can be allocated in any symbol in a slot, this CP extension will help using consecutive transmission after some transmission. In Rel-14 eLAA, the SRS is assigns at the end of a subframe, this discussion may not be needed, however in NR the SRS resource can be used for many purposes such as UL CSI or UL beam management and may start with any symbol in a slot. We believe that this can happen in the deployment, and need to discuss further.

Figure 1.  (a) CP extension of a SRS resource		(b) CP extension of the PUSCH/PUCCH

A scheduling DCI can indicate which CP extension to be used, and it is desirable that the CP extension can be applied to the SRS as well as the scheduled PUSCH or PUCCH. Especially, SRS can be transmitted immediately before/after the PUSCH/PUCCH depending on scheduling DCI or other DCI. However, for SRS transmission based on group common DCI (i.e., DCI format 2_3), any symbol extension may not be allowed because additional signalling should be necessary and no such signalling is discussed in Rel-16.

Proposal 1: The CP extension is additionally supported for SRS transmission.

  - Starting position of an SRS resource

According to the previous agreement, an SRS resource can be configured to have startPosition any value.

Agreement: Support RRC configuration of an SRS resource to start at any OFDM symbol within a slot by extending the RRC parameter startPosition of resourceMapping of SRS-Config for Rel-16 to have a value range 0..13.

We found that the above agreement is not yet captured in TS 38.331 as well, and we propose to extend the value range. In addition, since startPosition is read by the end of the slot according to TS 38.211, it is desirable to add a note to address this, which is left to the specification editor.

Proposal 2: Modify the startPosition, and put a note in SRS-Config field descriptions if necessary.

TS 38.331-g00 section 6.3.2, in SRS-Config IE SRS-Resource ::= SEQUENCE { … resourceMapping SEQUENCE { startPosition INTEGER (0..13), nrofSymbols ENUMERATED {n1, n2, n4}, repetitionFactor ENUMERATED {n1, n2, n4} }, freqDomainPosition INTEGER (0..67), freqDomainShift INTEGER (0..268), freqHopping SEQUENCE { c-SRS INTEGER (0..63), b-SRS INTEGER (0..3), b-hop INTEGER (0..3) }, … }

  - BWP interpretation

In the unlicensed band, BWP configuration in the TS 38.213 can follow the framework for the unpaired spectrum operation or paired spectrum operation. In our understanding, both are possible depending on the implementation, but it seems that an unpaired spectrum operation is generally assumed during the work item. It is good to clarify and explicitly state in the specification.

Proposal 3: In BWP section, it is clarified that unpaired spectrum framework is applied for the unlicensed band operation.

- Conclusion

In this contribution, we discuss some remaining issues.

Proposal 1: The CP extension is additionally supported for SRS transmission.

Proposal 2: Modify the startPosition, and put a note in SRS-Config field descriptions if necessary.

Proposal 3: In BWP section, it is clarified that unpaired spectrum framework is applied for the unlicensed band operation.