3GPP TSG RAN WG1 #100bis                               	                  R1-2002263

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.2.2.2

Source:	Spreadtrum Communications

Title:	Remaining issues on initial access procedure

Document for:	Discussion and decision

Introduction

This contribution discusses the remaining issues on initial access procedure in NR-U. This contribution is revision of our contribution [R1-2000419] in RAN1#100e.

SSB transmission

Interpretation of ssb-PositionsInBurst

In RAN1#100-e meeting, interpretation of ssb-PositionsInBurst for reception of PDCCH was clarified. However, some other wordings in 38.213 [1] may need clarification too.

RO validation

For RO validation, currently the wording uses “the index of the SS/PBCH block is provided by ssb-PositionsInBurst”. In our view, it may mean the candidate SS/PBCH block, because it is related to resource mapping.

UL validation in SFI

For UL validation in SFI, currently the wording uses “SS/PBCH blocks with indexes indicated to a UE by ssb-PositionsInBurst”. In our view, it may mean the candidate SS/PBCH block, because it is related to resource mapping.

Therefore, we have the following proposal.

Proposal 1: For RO validation and UL validation in SFI, the candidate SS/PBCH block index can be used, and consider to adopt TP in Appendix 5.1.

Terminology of “discovery burst transmission window”

As mentioned in our companion contribution [2], there was no intention to define “DRS” in WI stage except for channel access topic. For each agreement related to “discovery burst transmission window”, there seems nothing mentioned about Type0-PDCCH or corresponding SIB1 PDSCH. So, when we are talking about “discovery burst transmission window”, indeed we are talking about “SS/PBCH block transmission window”. Especially when M=1 or 2 in table of Type0-PDCCH monitoring occasion, the time span of “DRS” or discovery burst (including Type0-PDCCH and corresponding SIB1 PDSCH) will be too large, so “discovery burst transmission window” seem much larger than that of “SS/PBCH block transmission window”. Hence, at least in case of M=1 or 2, “discovery burst transmission window” in 38.213 only means the SS/PBCH block transmission window. If we keep “discovery burst transmission window” in 38.213, there could be a misunderstanding that Type0-PDCCH and corresponding SIB1 PDSCH should be confined in the window as well in the future discussion.

Therefore, we suggest replacing terminology “discovery burst transmission window” with “SS/PBCH block transmission window”.

Proposal 2: Replacing terminology “discovery burst transmission window” with “SS/PBCH block transmission window” in spec 38.213, and consider to adopt TP in Appendix 5.2.

Conclusion

We have the following proposals.

Reference

- R1-1913538, “Summary of discussion on SS/PBCH block indexing”, Samsung, November 18th – 22nd, 2019.

- R1-2000418, “Remaining issues on initial access signals/channels”, Spreadtrum, RAN1#100-e, February 24th – March 6th, 2020.

Appendix

TP for TS 38.213

------------------------------------------------------ Start of Text Proposal ----------------------------------------------------------

## 8.1	Random access preamble

------------------------------------------------------ Unchanged parts omitted ------------------------------------------------------

For unpaired spectrum,

-	if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if it does not precede a SS/PBCH block in the PRACH slot and starts at least  symbols after a last SS/PBCH block reception symbol, where  is provided in Table 8.1-2.

-	the SS/PBCH block is provided according to ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon

-	If a UE is provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if

-	it is within UL symbols, or

-	it does not precede a SS/PBCH block in the PRACH slot and starts at least  symbols after a last downlink symbol and at least  symbols after a last SS/PBCH block symbol, where  is provided in Table 8.1-2, and if ChannelAccessType-r16 = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213]

-	the SS/PBCH block is provided according to ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon.

-------------------------------------------------------- End of Text Proposal --------------------------------------------------------

------------------------------------------------------ Start of Text Proposal ----------------------------------------------------------

A SS/PBCH block symbol is a symbol of an SS/PBCH block with candidate SS/PBCH block index indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon.

-------------------------------------------------------- End of Text Proposal --------------------------------------------------------

------------------------------------------------------ Start of Text Proposal ----------------------------------------------------------

- 11.1.1	UE procedure for determining slot format

------------------------------------------------------ Unchanged parts omitted ------------------------------------------------------

For a set of symbols of a slot corresponding to SS/PBCH blocks with candidate SS/PBCH block indexes indicated to a UE by ssb-PositionsInBurst in SIB1, or by ssb-PositionsInBurst in ServingCellConfigCommon, the UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols of the slot as uplink.

-------------------------------------------------------- End of Text Proposal --------------------------------------------------------

TP for TS 38.213

------------------------------------------------------ Start of Text Proposal ----------------------------------------------------------

# 13	UE procedure for monitoring Type0-PDCCH CSS sets

------------------------------------------------------ Unchanged parts omitted ------------------------------------------------------

For operation with shared spectrum channel access, a UE assumes that transmission of SS/PBCH blocks in a half frame is within a SS/PBCH block transmission window that starts from the first symbol of the first slot in a half-frame. The UE can be provided per serving cell by DiscoveryBurst-WindowLength-r16 a duration of the SS/PBCH block transmission window. If DiscoveryBurst-WindowLength-r16 is not provided, the UE assumes that the duration of the SS/PBCH block transmission window is a half frame. For a serving cell, the UE assumes that a periodicity of the SS/PBCH block transmission window is same as a periodicity of half frames for receptions of SS/PBCH blocks in the serving cell. The UE assumes that one or more SS/PBCH blocks indicated by ssb-PositionsInBurst may be transmitted within the SS/PBCH block transmission window and have candidate SS/PBCH blocks indexes corresponding to SS/PBCH block indexes provided by ssb-PositionsInBurst. If MSB k, k≥1, of ssb-PositionsInBurst is set to 1, the UE assumes that one or more SS/PBCH blocks within the SS/PBCH block transmission window with candidate SS/PBCH block indexes corresponding to SS/PBCH block index equal to k-1 may be transmitted; if MSB k is set to 0, the UE assumes that the SS/PBCH block(s) are not transmitted.

For operation with shared spectrum channel access, a UE assumes that SS/PBCH blocks in a serving cell that are within a same SS/PBCH block transmission window or across SS/PBCH block transmission windows are quasi co-located with respect to average gain, QCL-TypeA, and QCL-TypeD properties, when applicable [6, TS 38.214], if a value of NDM-RSPBCHmodNSSBQCL is same among the SS/PBCH blocks. NDM-RSPBCH is an index of a DM-RS sequence transmitted in a PBCH of a corresponding SS/PBCH block, and NSSBQCL is either provided by ssbPositionQCL-Relationship-r16 or, if ssbPositionQCL-Relationship-r16 is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4-1. ssbSubcarrierSpacingCommon indicates SCS of RMSI only for the case of "operation without shared spectrum".The UE assumes that within a SS/PBCH block transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL. The UE can determine an SS/PBCH block index according to NDM-RSPBCHmodNSSBQCL, or according to i modNSSBQCL where i is the candidate SS/PBCH block index.

-------------------------------------------------------- End of Text Proposal --------------------------------------------------------