3GPP TSG-RAN WG1 Meeting #100bis	R1-2002296

e-Meeting, April 20th – 30th, 2020

Agenda item:		7.2.6.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on Full Tx power UL transmission

Document for:		Discussion and Decision

# 1	Introduction

There is one email discussion in RAN1 #100 e-meeting under this agenda, as [1]

[100e-NR-eMIMO-ULFPTx-01] Email discussion/approval regarding codebook configuration in Mode2 with multiple SRS, one with 4 another with 2 SRS ports by 2/28

Under this email discussion, three issues were discussed [1]:

Issue 1: Which codebook subset is used for the indication of 2-port SRS resource if the codebooksubset is configured to partialAndNonCoherent? - Alt. 1-1: The codebook subset for 2-port SRS resource is determined based on UE separate capacity reporting - Alt.1-2: The codebook subset for 2-port SRS resource is the one of noncoherent Issue 2: What value of maxRank used for the indication of 2-port SRS resource? - Alt.2-1: maxRank = 2 - Alt.2-2: If maxRank>2, maxRank=2, else use the configured value of maxRank Issue 3: Correction to 38.214

These agreements were reached in RAN1 #100-e meeting, including related TPs to TS 38.214 and TS 38.212 [1]:

Agreement - In Mode 2, when at least one SRS resource in a SRS resource set is configured with 4 ports, for a 2 port SRS resource (indicated via SRI) in the same SRS resource set, If maxRank>2, maxRank=2, else use the configured value of maxRank. - The following text proposal for TS38.214 subclause 6.1.1.1 is endorsed for the editor CR on TS38.214. - UE shall not expect to be configured with the higher layer parametercodebookSubset or the higher layer parameterorcodebookSubset-ForDCIFormat0_2 set to'partialAndNonCoherent' when higher layer parameternrofSRS-Ports in an SRS-ResourceSet with usage set to 'codebook' indicates thattwo SRS antenna ports are configuredthe maximum number of the configured SRS antenna ports in theSRS-ResourceSet is two. - The following text proposal for TS38.212 is provided as a starting point for implementing this agreement. It is up to TS38.212 editor how to exactly capture this. 7.3.1.1.2 Format 0_1 … For the higher layer parameter txConfig = codebook, if ULFPTxModes is configured to Mode2, if maxRank is configured to be larger than 2, and if SRS resources with 4 ports and 2 ports are configured within an SRS resource set with usage set to ‘codebook’, when the SRS with 2 ports is indicated at least one SRS resource with 4 SRS antenna ports is configured in a SRS resource set with usage set to ‘codebook’ and an SRS resource with 2 SRS antenna ports is indicated via SRI in the same SRS resource set, the field of Precoding information and number of layers is according to Table 7.3.1.1.2-4. For the higher layer parameter txConfig = codebook, if different SRS resources with different number of antenna ports are configured, the bitwidth is determined according to the maximum number of ports in a SRS resource among the configured SRS resources. If the number of ports for a configured SRS resource is less than the maximum number of ports in a SRS resource among the configured SRS resources, a number of most significant bits with value set to '0' are inserted to the field. …

Other discussion issues were raised before the RAN1 #100 e-meeting phase but there were no agreements even before the RAN1 #100 e-meeting. We shall continue to discuss other issues to complete this Rel-16 feature of uplink full power transmission.

One important issue is left unfinished in RAN1 #99 meeting [2]. This issue is related to the details of UE capability signaling for 4Tx ports. This contribution provides further discussion on this issue with our view to complete this Rel-16 issue.

The Rel-16 feature to support full Tx power transmission includes two UE modes, mode-1 and mode-2, as solutions for uplink full power transmission. The definition of mode-1 and mode-2 are listed here based on a RAN1 #97 agreement [2], for the purpose of our discussion:

Working Assumption Support following scheme for UL full power Tx for UE capability 2 and 3: - A UE can be configured for one of two modes of full power operation to support ‘Capability 2’ and ‘Capability 3’ subject to UE capability - A UE can be configured by the network to support full power transmission - Mode 1: The UE can be configured with one or more SRS resources with same number of SRS ports (according to Rel-15) within an SRS resource set which usage is set to ‘codebook’ - gNB can configure the UE to use a subset of TPMIs that combine ports in a layer to produce full power transmission. - A new codebookSubset is introduced only for the rank value(s) where full power transmission in UL is not achievable includes the TPMI precoders in fullyAndPartialAndNonCoherent defined in Rel-15 - FFS: At least a subset of the non-antenna selection TPMI precoder(s) is(are) supported - FFS: Additional support of antenna selection TPMI precoders - Note: as non-coherent UE, it is not capable of maintaining relative phase of antenna ports according to TPMI - Mode 2: The UE can be configured with one SRS resource or multiple SRS resources with different number of SRS ports within a SRS resource set which usage is set to ‘codebook’ - UE transmits SRS and PUSCH in same manner, whether antenna virtualization is used or not - Rel-15 codebooks and codebook subsets are used - Note: Antenna selection precoder can be used to enable full power related PA(s) to produce full power transmission for Capability-3 UE. - UL full power Tx is achieved for PUSCH transmission according to indicated SRI and/or TPMI - A set of TPMIs that deliver full power can be signalled by the UE in order to support at least UEcap3, for SRS resource with more than 1 ports, - e.g. For SRI indicating SRS resource with 1 port then single layer PUSCH is transmitted with full power in same manner as single port SRS, if SRI indicating SRS resource with multiple ports is signalled based on Rel-15 MIMO behaviour (transmission rank indicator, TPMI indicator, etc) except the power scaling aspects - The following cases are not precluded - For example, for 4TX on UE side (with 20+20+17+17dBm) virtualized as 2 SRS ports, full uplink power transmission can be enabled by precoder [1 0] or [0 1] - FFS: number of SRS resources supported - 2 - 3 - FFS: for 4 Tx, how many different TPMIs/TPMI groups support full power - FFS: any rules for spatial filter update for the SRS resources with different number ports Note: How to capture the behaviour for ‘Mode 1’ and ‘Mode 2’ in specifications is TBD Note: For single port, there is no SRI and TPMI Note: Support of Mode 1, Mode 2 have separate UE capability

These TPMIs for Mode 1 have been agreed to support full power transmission:

Table 1    Agreed TPMIs for full power Tx

| UE mode | Tx | rank | Supported TPMI for full power Tx |
| --- | --- | --- | --- |
| UE mode 1 | 2Tx | 1 | 0, 1, 2 |
|  |  | 2 | 0 |
|  | 4Tx | 1 | 12, 13, 14, 15 |
|  |  | 2 | 6 |
|  |  | 3 | 1 |
| UE mode 2 | 2Tx | 1 | 0, 1 |

RAN1 #99 meeting reached this agreement on UE capability signaling for 2Tx ports [1], where 2-bit bitmap shall be used to indicate 2Tx UE capability to support full Tx power:

Agreement For 2 ports, number of bits to indicate TPMI(s) which can deliver UL full power: - 2 bits (bitmap) - Whether is this capability reporting is optional or not will be discussed as part of UE capability discussions

The UE capability signaling for 4Tx ports has this agreement in [1]:

Agreement For 4 ports, number of bits to indicate TPMI(s) which can deliver UL full power: - Non Coherent 2 bits - Partial coherent 4 bits - Additional entries on top of existing entries may be added to table 1 and table 2 - Whether is this capability reporting is optional or not will be discussed as part of UE capability discussions Table 1. 4Tx, nonCoherent; 4Tx, partial coherent (4bit). G0; G0. G1; G1. G2; G2. G3; G3. G4. G5. G6. Definition of G0~G6 can be found in the table below. Table 2.

The table 1 and table 2 for the 4Tx capacity signaling is not finalized, based on the agreement in the last meeting (RAN1 #99). This contribution addresses this issue and provide a related proposal.

# 2	Discussion

Two UE modes are supported as UE solutions for full power transmission. These two UE modes, Mode-1 and Mode-2, are parts of UE capability. For full Tx power uplink transmission, a UE may claim its capability as

- Full Tx power capability, for UE with full coherent Tx ports;

- Mode-1 UE, where only a subset of TPMI will support full power transmission;

- Mode-2 UE, where UE may use virtualized port to deliver full Tx power.

From the network side, gNB can use RRC parameters ULFPTx, and ULFPTxModes to configure UE, within UE’s capability.

For 4Tx UE, Table 1 and Table 2 in RAN1 #99 agreement defines UE capability signalling, with 2-bit for non-coherent 4Tx, and 4-bit for partial-coherent 4Tx. One of the TPMI groups, G0, G1, G2, and G3, can be indicated by a 2-bit capacity signalling to indicate UE’s capability for 4Tx non-coherent ports.

For non-coherent 4Tx ports, G0 indicates rank-1, G1 indicates rank-1 and rank-2, G2 indicates rank-1, rank-2, and rank-3, and G3 indicates rank-2 and rank-3 transmission. Not all combination for 4Tx non-coherent ports are listed here; however, these G0~G3 are considered general cases that UE could support.

For partial-coherent 4Tx ports, a total of 7 TPMI groups are defined in Table 2 for full-power TPMI signalling. G0~G3 groups are included for non-coherent ports; while G4~G6 groups are considering the 2 coherent ports out of total 4Tx ports. G4 is covering rank-1 and rank-2 case; G5 covers rank-1, rank-2 and rank-3 case; and G6 covers another case of rank-1, rank-2, and rank-3 transmission.

Given that only 7 TPMI groups are defined in Table 2 to support UE capability signaling, 3-bits of signaling should be sufficient. However, it is agreed to use 4-bit for partial-coherent 4Tx signaling, as agreed in RAN1 #99. Extra TPMI groups (up to total of 16) can be added to support more flexible UE implementations.

One alternative is to have TPMI groups with limited ranks for full power transmission. We list the new TPMI groups in the table for rank-1 in G7 and G9, rank-1/2 in G8 and G10. A UE can use these TPMI groups to indicate their limited rank capability for full power transmission, with 4Tx partial-coherent ports.

| G7 | 121010,1210-10,1210j0, 1210-j0 |
| --- | --- |
| G8 | 121010,1210-10,1210j0, 1210-j0,1210000010, 1210010000, 1201000010 |
| G9 | 121010,1210-10,1210j0, 1210-j0, 120101,12010-1,12010j, 12010-j |
| G10 | 121010,1210-10,1210j0, 1210-j0, 120101,12010-1,12010j, 12010-j, 1210000010, 1210010000, 1201000010, 1201000001, 1210000001, 1200001001 |

Proposal 1: Include these new TPMI groups, G7, G8, G9 and G10 for 4Tx partial-coherent full-Tx power capability signaling.

# 3	Conclusions

This contribution addresses one remaining issue for Rel-16 full power uplink transmission feature. Given that it was agreed to use 4-bit to indicate capability for 4Tx partial-coherent UE, we shall add a few more TPMI groups for further implementation flexibility.

Proposal 1: Include these new TPMI groups, G7, G8, G9 and G10 for 4Tx partial-coherent full-Tx power capability signaling.

| G7 | 121010,1210-10,1210j0, 1210-j0 |
| --- | --- |
| G8 | 121010,1210-10,1210j0, 1210-j0,1210000010, 1210010000, 1201000010 |
| G9 | 121010,1210-10,1210j0, 1210-j0, 120101,12010-1,12010j, 12010-j |
| G10 | 121010,1210-10,1210j0, 1210-j0, 120101,12010-1,12010j, 12010-j, 1210000010, 1210010000, 1201000010, 1201000001, 1210000001, 1200001001 |

# References

[1]	RAN1 #100 e-meeting, emails under [100e-NR-eMIMO-ULFPTx-01] email chain.

[2]	RAN1 #99 meeting, Chair’s notes, November 2019

[3]	RAN1 #97 meeting, Chair’s notes, May 2019