3GPP TSG RAN WG1 #100bis	R1-2002352

e-Meeting, April 20th - April 30th, 2020

Agenda Item:	7.2.11.5

Source:	Apple Inc.

Title:	Discussions on UE Features on URLLC/IIoT

Document for:	Discussion/Decision

# 1.	Introduction

A preliminary draft on UE feature list was provided in [1], which serves as the starting point for discussion in RAN1#100bis-e. In this contribution, we discuss the UE features for URLLC and IIoT WIs, and provide our views.

# 2.	FG 11-2/2b (PDCCH monitoring capability)

| 11-2 | Rel-16 PDCCH monitoring capability | 1) Supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span for combination (X, Y, ) 2) If UE reports the support of more than one combination of C(X, Y) for a given SCS, and if multiple combinations of C(X, Y) are valid for the span pattern, the maximum value of C of the valid combinations is applied 3) Supports the limit M on the maximum number of monitored PDCCH candidates per PDCCH monitoring span for combination (X, Y, ) 4) If UE reports the support of more than one combination of M(X, Y) for a given SCS, and if multiple combinations of M(X, Y) are valid for the span pattern, the maximum value of M of the valid combinations is applied 5) Capability on the number of CCs with Rel-16 PDCCH monitoring capability on all the serving cells. | 3-5b | Yes | N/A |  | [FSPC] FFS: Component 5) reported per UE | [N/A] | [N/A] |  | This capability is necessary for SCS 15 kHz and 30 kHz. FFS: Adding a component for “supported combination(s) (X, Y, ), which may depend on how to report C, M and (X, Y, ) A list of separate UE capabilities C(X, Y, ), M(X, Y, ) for processing capability #1; A list of separate UE capabilities C(X, Y, ), M(X, Y, ) for processing capability #2; For component 5), if UE supports carrier aggregation with more than [x] DL carriers with Rel-16 PDCCH monitoring capability on all the carriers, UE should report this capability. Value of x (can be < 4) is TBD. FFS: Whether to merge component 1) and 3), and accordingly merge component 2) and 4) FFS：Whether to add a capability for supporting 3 unicast PDSCH/PUSCH per slot separately for each minimum processing capability to match the number of spans for (4,3) pair | Optional with capability signalling Candidate value set for (X, Y): {(7, 3), (4, 3), (2, 2)} The value of C for combination (7, 3) for 15 kHz and 30 kHz is 56 FFS the value of C for combination (4, 3) and (2, 2) FFS the value of M for combination (7, 3), (4, 3) and (2, 2) Candidate value for component 5): { x, x+1, …, 16} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11-2b | Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells | [Support Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells] - Capability on the number of CCs with Rel-15 PDCCH monitoring capability - Capability on the number of CCs with Rel-16 PDCCH monitoring capability | 11-2 | Yes | N/A |  | [Per UE] | [No] | TBD |  | Capability on the number of CCs with Rel-15 PDCCH monitoring capability can be smaller than 4 CCs; Capability on the number of CCs with Rel-16 PDCCH monitoring capability can be smaller than 4 CCs; The summation of the minimum of the capability on the number of CCs with Rel-15 PDCCH monitoring capability and the minimum of the capability on the number of CCs with Rel-16 PDCCH monitoring capability is not larger than 4 [Rel-15 monitoring capability here is subjected to the capability of FG 3-1, FG 3-2 and FG 3-5b.] | Optional with capability signalling |

Dependency of 11-2 on 3-5b

Currently FG 3-5b is listed as a pre-requisite for 11-2. However, this is not necessary in our view. Features 11-2 and 3-5b share the same concept of span pattern (span duration and span gap), but the handling is quite different. For 3-5b, the overbooking/dropping is performed on a per-slot basis, while for 11-2, it is performed on a per-span basis. Logically speaking, there is no reason why a UE has to support 3-5b to be able to support 11-2. 3-5b and 11-2 can be considered as two different approaches to support Case 2 (multiple monitoring occasions within a slot) of PDCCH monitoring, and there should not be dependency between the two (of course a UE can still claim to support both).

Proposal 1: Remove the dependency of FG 11-2 on FG 3-5b.

FFS: Whether to merge component 1) and 3), and accordingly merge component 2) and 4)

It makes sense to merge component 1) and 3) in 11-2, because C and M need to report together for each supported combination (X, Y, ). Combining them avoid the possibility that a UE report C but not M (or vice versa) for one (X, Y, ) combination.

Whether to merge 2) and 4) is less critical because there is no separate signaling for these two, and both will be supported if a UE reports 11-2. Therefore, it does not matter much whether to merge 2) and 4) or not.

Proposal 2: Merge component 1) and 3) in FG 11-2.

Whether to split 11-2b into 3 FGs, corresponding to 3-1, 3-2, and 3-5b in Rel-15, respectively?

There have been some discussions from the companies on whether to have a separate FG corresponding to each of Rel-15 PDCCH FGs in case of mixed Rel-15 and Rel-16 PDCCH monitoring capability. We see some benefit of defining separate features.

- Even though the number of CCs is not separated reported for 3-1, 3-2 and 3-5b in Rel-15, it is generally acknowledged that the three features do not have the same complexity. In particular, 3-5b is more complicated than 3-1/3-2 and requires more UE processing power. By splitting 11-2b into 3 features, it allows the UE to report different capabilities corresponding to different Rel-15 features. For example, the UE may potentially report larger number of CCs for the combination of 3-1 and 11-2 than for the combination of 3-5b and 11-2. Otherwise, the UE would have to report conservatively, i.e., report the number corresponding to the most complicated one among 3-1, 3-2 and 3-5b (if all are supported), which is most likely to be 3-5b.

- There is some similarity between 3-5b and 11-2, and they are both used to support multiple monitoring occasions in a slot. It may not even be so necessary for the UE to handle a mix of 3-5b and 11-2.

Proposal 3: Split FG 11-2b into 3 FGs, corresponding to 3-1, 3-2 and 3-5b for Rel-15 PDCCH monitoring capability in case of mixed Rel-15 and Rel-16 PDCCH monitoring capability.

# 3.	FG 11-3 (sub-slot based HARQ-ACK feedback)

| 11-3 | More than one PUCCH for HARQ-ACK transmission within a slot | 1) Supports sub-slot based HARQ-ACK feedback procedure. • A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot.• At least one sub-slot configuration for PUCCH can be UE specifically configured to a UE. • Supports a single configuration for PUCCH resource for all sub-slots in a slot. The starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot. Any sub-slot PUCCH resource is not across sub-slot boundaries. 2) Supported sub-slot configuration [3) Supported combinations of (A, B), where A is the minimum gap between sub-slots containing actual PUCCH transmissions measured from beginning to beginning of the sub-slots, including across slots, and B is the sub-slot duration, with both A and B in units of symbols] |  | Yes | N/A |  | [Per UE] | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | Candidate value set for component 2): { 7-symbol*2, 2-symbol*7 and 7-symbol*2} [Candidate value set for component 3): (A, B) = {(7, 7), (4, 2) and (7, 7), (2, 2) and (7, 7)}] FFS: Whether to keep component 3) and accordingly the above note for component 3) FFS: Any relationship between FG 11-3 and CBG-based PUSCH with minimum processing time capability #2? | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

One open issue for FG 11-3 is whether to introduce component 3), which defines certain minimum gap between PUCCHs which mainly helps with the 2-symbol sub-slot implementation. In principle we support introducing this component because it would allow UEs to implement the feature with reduced complexity, similar to the span pattern that has been introduced for PDCCH. However, it should be clarified whether PUCCH transmissions here include the ones that include UCI other than HARQ-ACK. E.g. PUCCH for CSI or with different UCI types multiplexed may be a longer PUCCH that span across multiple sub-slots. From UE complexity point of view, it makes sense to include all PUCCH transmissions in the definition.

Proposal 4: Include component 3) in FG 11-3 by modifying it to the following: “Supported combinations of (A, B), where A is the minimum gap between sub-slots containing within which the actual PUCCH transmissions start, measured from beginning to beginning of the sub-slots, including across slots, and B is the sub-slot duration, with both A and B in units of symbols”.

# 4.	FGs 11-4/4a (two HARQ-ACK codebooks) and 12-1 (intra-UE prioritization/mulplexing)

| 11-4 | Two HARQ-ACK codebooks [with up to one sub-slot based HARQ-ACK codebook] simultaneously constructed for supporting PDSCH reception with different priorities at a UE | 1) Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed [with the restriction up to one sub-slot based HARQ-ACK codebook]. 2) Supports separate PUCCH configuration for different HARQ-ACK codebooks 3) Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. [4) Supports a DCI format (from the formats 0_1/1_1/0_2/1_2) scheduling PDSCH with different HARQ-ACK priorities or PUSCH with different priorities when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured per BWP] 5) Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and ‘codeBlockGroupTransmission” for different HARQ-ACK codebooks. [6) Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot] |  | Yes | N/A |  | [Per UE] FFS: FS | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | FFS: Whether and how to combine FG 11-4 and FG 12-1 FFS: For component 4), whether to separate DL priority and UL priority, and whether to separate DCI format 0_1/1_1 and DCI format 0_2/1_2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [11-4x] | [Two sub-slot based HARQ-ACK codebooks simultaneously constructed for supporting PDSCH reception with different priorities at a UE]. | 1) Supports two sub-slot based HARQ-ACK codebooks with different priorities to be simultaneously constructed. 2) Supports separate PUCCH configuration for different HARQ-ACK codebooks 3) Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. 4) Supports a DCI format (from the formats 0_1/1_1/0_2/1_2) scheduling PDSCH with different HARQ-ACK priorities or PUSCH with different priorities when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured in USS per BWP 5) Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and ‘codeBlockGroupTransmission” for different HARQ-ACK codebooks. | 11-3 | Yes | N/A |  | [Per UE] | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | FFS: whether to add this FG and the contents of this FG | Optional with capability signalling |
| 11-4a | Monitoring a DCI format (from the formats 0_1/1_1/0_2/1_2) scheduling PDSCH with different HARQ-ACK priorities or PUSCH with different priorities when both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP |  | 11-1a, 11-4 | Yes | N/A | FFS | Per UE | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | FFS: Whether to split 11-4a into two rows as below: 11-4x: DL priority indication in DCI with mixed DCI formats 11-4y: UL priority indication in DCI with mixed DCI formats | Optional with capability signalling |

| 12-1 | UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer | Support intra-UE multiplexing/prioritization of UL overlapping channels/signals with two priority levels in physical layer (PHY) - Configuration of PHY priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH - Multiplexing/prioritization between UL channels/signals with the same PHY priority level - Prioritization between UL channels/signals with different PHY priority levels - Additional number of symbols (d1) needed beyond the PUSCH preparation time for cancelling a low priority UL transmission. - Additional number of symbols (d2) needed beyond the PUSCH preparation time for scheduling a high priority UL transmission that cancels a low priority UL transmission |  | Yes | N/A |  | Per UE | No | No | [support mixture of FDD/TDD and/or FR1/FR2 ] | A UE supporting this feature shall also support the LCP restriction based on DCI priority indication ([lch-ToGrantPriorityRestriction-r16]) and intra-UE prioritization in MAC ([lch-PriorityBasedPrioritization-r16]). The relationship between this feature and the feature of up to two HARQ-ACK codebooks of 11-4[x] including merging these features should be further discussed. | Optional with capability signaling Candidate value set for component 4: {0, 1, 2} Candidate value set for component 5: {0, 1, 2} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Both 11-4 and 12-1 are part of the two priority level support for intra-UE multiplexing/multiplexing, therefore, it makes sense to discuss them together. 12-1 has a more general description without including HARQ-ACK priority, and 11-4 is for HARQ-ACK priority only, even though the current description also includes the priority indication for PUSCH with DCI format 0_1/0_2.

Whether to define separate FGs for HARQ-ACK priority and PUSCH/SR priority?

We currently support two priority levels for HARQ-ACK, PUSCH and SR, within which HARQ-ACK is to support DL transmissions, while PUSCH and SR are for UL transmissions. The first question is whether there is a need to separate the support of the HARQ-ACK priority and the support of PUSCH/SR priority, or they should be merged together as suggested in the notes for 11-4 and 12-1. We think it provides more flexibility to have separate features defined for HARQ-ACK priority and PUSCH/SR priority, to adapt to different requirements for different use cases. (An alternative approach is to combine 11-4 and 12-1, while adding an additional component to allow the UE to report one of the {“two-level HARQ-ACK priority only”, “two-level PUSCH/SR priority only”, “two-level HARQ-ACK priority and PUSCH/SR priority”}.)

Proposal 5: Keep separate UE FGs for the support of two-level HARQ-ACK priority and the support of two-level PUSCH/SR priority. Update FG 11-4 to include the multiplexing/prioritization between UL channels so that it becomes a complete and independent FG.

For intra-UE prioritization/multiplexing, the cancellation of an ongoing transmission and the initiation of another transmission (especially PUSCH canceling PUSCH) have significant impact on the UE implementation, so this capability may have dependency on the number of CCs being configured. In addition, it may not be so necessary practically speaking to support the intra-UE prioritization/multiplexing behavior on many CCs at the same time. Therefore, we propose the corresponding FG to be defined as per FSPC.

Proposal 6: Define FG 12-1 to be per FSPC.

Whether to define separate FGs for the support of dynamic priority indication for the existing DCI format 0_1 (or 1_1) and the new DCI format 0_2 (or 1_2)?

We support this split because it allows the UE to implement the dynamic priority indication for the new DCI formats only without impacting the existing DCI formats.

Proposal 7: Define separate UE FGs for DCI format 0_1 and DCI format 0_2 for the support of dynamic PUSCH priority indication.

Proposal 8: Define separate UE FGs for DCI format 1_1 and DCI format 1_2 for the support of dynamic HARQ-ACK priority indication.

CBG retransmission handling for PUSCH cancelation

Per specification, the TB CRC is generated as part of L1 processing. If CBG-based operation is configured and the initial transmission is cancelled, it may be possible that a full TB CRC may not be available for CBG-based retransmission. Therefore, we would like to propose an additional UE feature to add the restriction that the UE does not expect to be scheduled with a partial TB retransmission (without including all CBGs) in a HARQ retransmission in case the initial HARQ transmission is cancelled.

Proposal 9: Introduce a FG (e.g. 12-1a) that a UE is not expected to be scheduled with a CBG-based HARQ retransmission that does not include the full TB if the initial HARQ transmission was cancelled in case of intra-UE prioritization.

Whether to have separate FGs for up to one sub-slot-based HARQ-ACK codebook and two sub-slot-based HARQ-ACK codebooks?

More typical use cases for two HARQ-ACK codebooks would be two slot-based codebooks, or one slot-based plus one sub-slot-based codebook, while supporting two sub-slot-based HARQ-ACK codebooks does not seem to have any obvious use cases. Therefore, it would make sense to define two separate UE features for them. In addition, it would be good to clarify the relationship between 11-3 and 11-4. If a UE support 11-3, supporting 11-4 means that it can support either two slot-based codebooks, or one slot-based plus one sub-slot-based codebook. If a UE does not support 11-3, supporting 11-4 means that it can only support two sub-slot-based codebooks.

Proposal 10: Introduce FG 11-4x for two sub-slot-based HARQ-ACK codebooks, and update FG 11-4 to be up to one sub-slot-based HARQ-ACK codebook. Clarify for FG 11-4 that if a UE does not support 11-3 but supports 11-4, it means the UE can only support two slot-based HARQ-ACK codebooks.

Whether to split FG 11-4a into two FGs for DL and UL DCI formats?

Following the same spirit as proposal 5, we support the split of FG 11-4a into two FGs, one for DL DCI formats, and another one for UL DCI formats.

Proposal 11: Split FG 11-4a into two FGs, one for HARQ-ACK priority indication in DCI formats 1_1/1_2, and another one for PUSCH priority indication in DCI formats 0_1/0_2.

# 5.	FG 11-5 (PUSCH enhancements)

| 11-5 | PUSCH repetition type B | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots. 2) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. [3) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field.] 4) PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 5) S and L are separately indicated (4-bit for S and 4-bit for L). L <= 14. [6) TBS is determined based on L indicated in TDRA table entry reusing Rel-15 mechanism.] [7) Handling of interaction with DL/UL directions depending on whether dynamic SFI is configured or not, including both cases with and without higher layer parameter InvalidSymbolPattern configured] [8) Supported maximum number of actual repetitions within a slot] [9) Supported PUSCH hopping scheme] |  | Yes | N/A |  | [Per UE] FFS: Per band | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | Candidate value for component 8): {2, 3, 4, 7, [8], [12]} FFS: Whether to add new feature groups for the total number of unicast PUSCHs for different TBs per slot per CC, or just add some note here with an example below: [The total number of unicast PUSCHs for different TBs per slot per CC is subjected to the capability reported by FG 5-12, 5-12a, 5-12b, 5-13d, 5-13e and 5-13f] FFS: Whether to set separate UE capabilities for dynamic grant and configured grant. Can we just add some note here with an example below for compromise? [PUSCH repetition type B with configured grant is applied only if UE reports the support of FG 5-19 or FG 5-20, and subjected to the capability of FG 5-19 and FG 5-20]. FFS: Whether to set separate UE capabilities for the case that dynamic SFI is configured and InvalidSymbolPattern is configured. Can we just add some note here with an example below for compromise? [The case that both dynamic SFI and InvalidSymbolPattern are configured is applied only if UE reports the support of FG3-6.] FFS: Whether to set separate UE capabilities for DCI format 0_1 and DCI format 0_2 for PUSCH repetition type B. Can we go majority view that no separate UE capability? | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

There are many open points regarding whether to split this FG further based on different aspects. We think it makes sense to separate the support of InvalidSymbolPattern, because this is not considered as an essential part of PUSCH repetition Type B (i.e. can still work without it). For the other aspects, we do not see the need to split the FG further, either because the existing FGs can be reused, or because we do not see much additional complexity.

Proposal 11: Split FG 11-4a into two FGs, one for HARQ-ACK priority indication in DCI formats 1_1/1_2, and another one for PUSCH priority indication in DCI formats 0_1/0_2.

# 6.	FG 11-7 (UL cancelation)

| 11-7 | UL cancelation scheme | 1) Supports group common DCI (i.e. DCI format 2_4) for cancelation indication 2) UL cancelation for PUSCH • Cancellation is applied to each PUSCH repetition individually in case of PUSCH repetitions 3) UL cancelation for SRS symbols that overlap with the cancelled symbols [4) For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by higher layer.] |  | Yes | N/A |  | [Per UE] FFS: FS | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | FFS: Whether to split this FG 11-7 into one feature group for the case of UL CI on the same CC and another feature group for the case of UL CI on another CC FFS: Whether to add new FG with FG11-7 as prerequisite for the support of more than one monitoring occasion for DCI 2_4 per slot? Can we just add the following note to address the concern? [More than one monitoring occasion for DCI format 2_4 per slot is applied only if the UE reports to support FG 3-5 or FG 3-5a or FG 3-5b] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [11-7a] | Cancellation of the overlapping PUSCHs in an intra-band UL CA without indication in the DCI format 2-4 | 1) For a UE indicating the capability of pa-PhaseDiscontinuityImpacts, and if the PUSCH on at least one serving cell is cancelled, the UE may cancel the (repetition of the) PUSCHs transmission on all other intra-band serving cell(s). The cancellation of the (repetition of the) PUSCH transmission on a the set of intra-band serving cell(s) includes all symbols from the earliest symbol that is overlapping with the first cancelled symbol of the PUSCH on the serving cell for which the DCI format 2_4 is applicable to. | 6-23, 11-7 | Yes | N/A |  | [PerBand] | [N/A] | [N/A] | TBD | FFS: Whether to add this FG and the content for each column if added | Optional with capability signaling |

We do not see the need to have a separate feature defined for the case with more than one monitoring occasions within a slot for DCI format 2_4. The configuration should be allowed as long as it is supported by the UE PDCCH monitoring capability (e.g. the UE supports FG 3-5/3-5a/3-5b/11-2).

Proposal 13: Do not define a separate feature for the case with more than one monitoring occasions within a slot for DCI format 2_4.

On the same-CC and cross-CC monitoring of DCI format 2_4, we would prefer to separate them. The same-CC and cross-CC handling may be very different in UE implementation, and the timeline consideration can also be different.

Proposal 14: Split FG 11-7 into two FGs, one for same-CC monitoring and one for cross-CC monitoring of DCI format 2_4.

For FG 11-7a, we support adding this FG to capture what was concluded in RAN1#100-e email discussions.

Proposal 15: Introduce FG 11-7a.

On the handling of CBG-based transmission, there is the same issue on PUSCH cancelation as in intra-UE prioritization. Similarly, we propose:

Proposal 16: Introduce a FG (e.g. 11-7b) that a UE is not expected to be scheduled with a CBG-based HARQ retransmission that does not include the full TB if the initial HARQ transmission was cancelled in case of inter-UE cancelation.

# 3. Conclusions

In this contribution we share our views on UE features for URLLC/IIoT, and propose the following:

Proposal 1: Remove the dependency of FG 11-2 on FG 3-5b.

Proposal 2: Merge component 1) and 3) in FG 11-2.

Proposal 3: Split FG 11-2b into 3 FGs, corresponding to 3-1, 3-2 and 3-5b for Rel-15 PDCCH monitoring capability in case of mixed Rel-15 and Rel-16 PDCCH monitoring capability.

Proposal 4: Include component 3) in FG 11-3 by modifying it to the following: “Supported combinations of (A, B), where A is the minimum gap between sub-slots containing within which the actual PUCCH transmissions start, measured from beginning to beginning of the sub-slots, including across slots, and B is the sub-slot duration, with both A and B in units of symbols”.

Proposal 5: Keep separate UE FGs for the support of two-level HARQ-ACK priority and the support of two-level PUSCH/SR priority. Update FG 11-4 to include the multiplexing/prioritization between UL channels so that it becomes a complete and independent FG.

Proposal 6: Define FG 12-1 to be per FSPC.

Proposal 7: Define separate UE FGs for DCI format 0_1 and DCI format 0_2 for the support of dynamic PUSCH priority indication.

Proposal 8: Define separate UE FGs for DCI format 1_1 and DCI format 1_2 for the support of dynamic HARQ-ACK priority indication.

Proposal 9: Introduce a FG (e.g. 12-1a) that a UE is not expected to be scheduled with a CBG-based HARQ retransmission that does not include the full TB if the initial HARQ transmission was cancelled in case of intra-UE prioritization.

Proposal 10: Introduce FG 11-4x for two sub-slot-based HARQ-ACK codebooks, and update FG 11-4 to be up to one sub-slot-based HARQ-ACK codebook. Clarify for FG 11-4 that if a UE does not support 11-3 but supports 11-4, it means the UE can only support two slot-based HARQ-ACK codebooks.

Proposal 11: Split FG 11-4a into two FGs, one for HARQ-ACK priority indication in DCI formats 1_1/1_2, and another one for PUSCH priority indication in DCI formats 0_1/0_2.

Proposal 11: Split FG 11-4a into two FGs, one for HARQ-ACK priority indication in DCI formats 1_1/1_2, and another one for PUSCH priority indication in DCI formats 0_1/0_2.

Proposal 13: Do not define a separate feature for the case with more than one monitoring occasions within a slot for DCI format 2_4.

Proposal 14: Split FG 11-7 into two FGs, one for same-CC monitoring and one for cross-CC monitoring of DCI format 2_4.

Proposal 15: Introduce FG 11-7a.

Proposal 16: Introduce a FG (e.g. 11-7b) that a UE is not expected to be scheduled with a CBG-based HARQ retransmission that does not include the full TB if the initial HARQ transmission was cancelled in case of inter-UE cancelation.

# References

- R1-2001484, “RAN1 UE features list for Rel-16 NR after RAN1#100-E”, AT&T, NTT DOCOMO, INC., RAN1#100-e, February 2020.