3GPP TSG RAN WG1 #100bis                                                      	R1- 2002353

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.11.6

Source:	Apple Inc.

Title:	Views on Rel-16 eMIMO UE feature list

Document for:	Discussion/Decision

# Introduction

Based on the email discussion on RAN1 reflector, the latest UE feature list for Rel-16 eMIMO is quoted in the Appendix. In this contribution, we provide our views based on the latest UE feature list for Rel-16 eMIMO.

# Views on Rel-16 eMIMO UE feature list

The current Rel-16 eMIMO UE feature list is not exactly agreeable to us, due to the following reason

- For FG16-2a, Multi-DCI based Multi-TRP, component 4/5/6, i.e., OOO PDSCH/HARQ-ACK/PUSCH cannot be the basic components which is violating the existing agreement. It was already agreed that OOO PDSCH/HARQ-ACK/PUSCH is optional feature for UE that supports Multi-DCI based Multi-TRP operation. Therefore, component 4/5/6 has to be moved to optional components

Agreement For multi-DCI based multi-TRP, when PDCCHs schedule two PDSCHs/PUSCHs across TRPs, i.e. PDCCHs are associated with different values of CORESETpoolIndex, following operations are allowed: - PDCCH to PDSCH: For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH associated with a value of CORESETpoolIndex ending in symbol i, the UE can be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH associated with a different value of CORESETpoolIndex that ends later than symbol i. - PDCCH to PUSCH: For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH associated with a value of CORESETpoolIndex ending in symbol i, the UE can be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH associated with a different value of CORESETpoolIndex that ends later than symbol i. - Note that from the UE perspective, it does not imply overlapped PUSCHs at the time. - PDSCH to HARQ-Ack: In a given scheduled cell, the UE can receive a first PDSCH in slot i, with the corresponding HARQ-ACK assigned to be transmitted in slot j, and a second PDSCH associated with a CORESETpoolindex different from the first PDSCH starting later than the first PDSCH with its corresponding HARQ-ACK assigned to be transmitted in a slot before slot j. Above features are optional for a UE that supports multi-DCI based multi-TRP

# Views on Rel-16 eMIMO UE FGs

FG 16-1a: L1-SINR reporting

To support L1-SINR based beam management, the basic feature should be CMR based only, UE should be able to indicated whether it supports ZP-IMR (CSI-IM) based, and, NZP-IMR (NZP-CSI-RS) based interference measurement separately. Therefore, we suggest to adopt similar approach as Rel-15 for L1-RSRP, i.e., FG 2-24, by adding components as the following

- The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot

- The max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR

- The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot

- Supported density of CSI-RS for CMR to measure L1-SINR

- The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as ZP IMR to measure L1-SINR within a slot

- The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as ZP IMR to measure L1-SINR

- The max number of CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as NZP IMR to measure L1-SINR within a slot

- The max number of CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as NZP IMR to measure L1-SINR

FG 16-1g:

For the first component, “The maximum number of SSB/CSI-RS resources across all CCs for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, and new beam identification”. It is agreed and captured in 38.213 that UE maintains at most 4 PL estimate per serving cell, and, at most 2 BFD resources for BFR per BWP. Therefore, we suggest to remove the PL and BFD related resources, i.e., “The maximum number of SSB/CSI-RS resources across all CCs for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, and new beam identification”

FG 16-2a: Multi-DCI based multi-TRP

Basic components

- Component 4/5/6, i.e., OOO PDSCH/HARQ-ACK/PUSCH cannot be the basic components which is violating the existing agreement. It was already agreed that OOO PDSCH/HARQ-ACK/PUSCH is optional feature for UE that supports Multi-DCI based Multi-TRP operation. Therefore, component 4/5/6 has to be moved to optional components

Optional components

- Component 1: We first need a top feature which is whether UE supports CRS rate matching for Multi-DCI based Multi-TRP operation. Then it was agreed that supporting the union of CRS rate matching patterns from both TRPs is mandatory, but supporting individual CRS rate matching per TRP is optional. Therefore, we suggest the change

  - Support of CRS rate matching for Multi-DCI based Multi-TRP operation. “Joint” is mandatory for UE that supports Multi-DCI based Multi-TRP CRS rate matching, but “Separate” is optional

    - Joint: UE rate match around the union of CRS from both TRPs

    - Separate: UE rate match around configured CRS patterns which is associated with CORESETPoolIndex (if not configured, CORESETPoolIndex=0) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex

- Component 3: Some format issue here, “Support of fully/partially time/frequency overlapped PDSCH reception (PDSCHs overlapping types in time and frequency domain)” should be a separate component

- Component 6, we can split into the following 3 cases

  - Two TDMed long PUCCHs within a slot

  - TDMed short PUCCH and long PUCCH within a slot

  - TDMed short PUCCH and short PUCCH within a slot

- We propose to add one component to report whether UE can support a single closed-loop power control process for PUCCH/PUSCH associated with different CORESETPoolIndex, i.e. “Support of common closed-loop power control process for PUCCH/PUSCH associated with different CORESETPoolIndex”

  - We think one possible implementation for multi-DCI based operation is to use CA-like architecture. So, the simple approach is to configure separate closed-loop power control processes to PUCCH/PUSCH associated with different CORESETPoolIndex.

FG 16-3a: Regular eType-II

Basic components

- We can add another component “Support of rank 1,2”

- Component 5, “UCI omission”, we prefer this to be moved to “Optional components”

Optional components

- Component 3, “CBSR”. There are 2 mode of CBSR, one is just on and off, i.e. only 2 amplitude restrictions, the other is choice from 4 amplitude restrictions. The second mode is agreed to be optional even for UE supports CBSR. Therefore, we suggest the change "(1) Whether UE supports CBSR (2) If UE supports CBSR, whether UE supports 4 values of restriction, i.e. amplitudeSubsetRestriction as in 38.214"

FG 16-3b: Port selection eType-II

Basic components

- We can add another component “Support of rank 1,2”

- Component 5, “UCI omission”, we prefer this to be moved to “Optional components”

FG 16-5a: UL full power transmission mode 0

- We prefer this to be at least per band per BC

FG 16-5b: UL full power transmission mode 1

- 2-bit bitmap is more flexible {2Tx, 4Tx}. For UE capable of 4 ports UL, UE can indicate whether UE supports mode 1 when UE is configured with 2 port SRS or 4 port SRS for codebook based PUSCH operation, independently

- We prefer this to be at least per band per BC

FG 16-5c: UL full power transmission mode 2

- Component 2: similar comment, 2-bit bitmap is more flexible {2Tx, 4Tx}

- We prefer this to be at least per band per BC

# Appendix

# Latest UE feature list for Rel-16 eMIMO

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16. NR_eMIMO | 16-1a | L1-SINR reporting | - The maximum number of L1-SINR based beam measurement and reporting based on ZP IMR and/or NZP IMR (FFS details on the sub-components, e.g., FG 2-24) FFS: Support of group-based reporting for L1-SINR | TBD |  | N/A |  | TBD [Per band] | N/A | N/A |  |  | TBD |
|  | 16-1b | TCI state activation and spatial relation update | - [Support of / maximum number of lists for] Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH - [Support of / maximum number of lists for] Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - [Support of / The maximum number of] PUCCH resource groups per BWP for simultaneous spatial relation update - FFS: details on whether/how to indicate band pairs which can share the same DL TCI state - FFS: details on whether/how to indicate band pairs which can share the same UL spatial relation info | Component 1: 2-1, 2-4 Component 2: 2-59, 2-60 Component 3: 2-53, 2-59, 4-24 |  | N/A |  | TBD [Per BC or per band] | N/A | N/A |  |  | TBD |
|  | 16-1c | Default spatial relation | - Support of default spatial relation and pathloss reference RS for dedicated-PUCCH/SRS and PUSCH scheduled by DCI format 0_0 | 2-53, 2-59 |  | N/A |  | TBD [Per band] | N/A | N/A |  |  | TBD |
|  | 16-1d | MAC CE spatial relation update for AP-SRS | [Support of / The maximum number of] spatial relation update for AP-SRS via MAC CE | 2-53, 2-59 |  | N/A |  | TBD [Per UE] | No | No |  |  | TBD |
|  | 16-1e | Pathloss reference RS activation via MAC CE | - The maximum number of configured pathloss reference RSs for PUSCH/SRS by RRC for MAC-CE based pathloss reference RS update - FFS: The maximum number of activated pathloss reference RS update for PUSCH/SRS/PUCCH [across CCs / within a slot across all CCs / per CC] FFS: Number of measurement samples N to apply newly activated pathloss reference RS | 8-2, 8-3 |  | N/A |  | TBD [Per UE] | No | No |  |  | TBD |
|  | 16-1f | SCell beam failure recovery | - The maximum number of SCells for SCell beam failure recovery - FFS: Support of PUCCH-BFR - FFS: The maximum number of CSI-RS and/or SSB resources for new beam identification of SCell BFR [across all CCs / within a slot across all CCs / per CC] - FFS: Density of CSI-RS for new beam identification for SCell BFR | 2-31 |  | N/A |  | TBD | N |  |  |  | TBD |
|  | 16-1g | FFS: Resources for beam management, pathloss measurement, and BFR | - FFS: The maximum number of SSB/CSI-RS resources across all CCs for any of L1-RSRP measurement, L1-SINR measurement, pathloss measurement, BFD, and new beam identification. - FFS: The maximum number of SSB/CSI-RS resources across all CCs for pathloss measurement - FFS: The maximum number of SSB/CSI-RS resources across all CCs for BFD - FFS: The maximum number of SSB/CSI-RS resources across all CCs for new beam identification |  |  | N/A |  | TBD | N |  |  |  | TBD |
|  | 16-2a | Multi-DCI based multi-TRP | Basic components: - The maximum number of CORESETs configured per “PDCCH-Config” - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” - The value of R=[1,2] for BD/CCESupport of fully/partially time/frequency overlapped PDSCH reception (PDSCHs overlapping types in time and frequency domain) - Support of out-of-order operation for PDCCH to PDSCH (FFS whether to be a basic component) - Support of out-of-order operation for PDSCH to HARQ-ACK (FFS whether to be a basic component) - Support of out-of-order operation for PDCCH to PUSCH (FFS whether to be a basic component) - FFS: The maximum number of activated TCI states - FFS: The maximum number of MIMO layers of scheduled PDSCHs - FFS: the maximum number of CCs supporting multi-DCI based multi-TRP Optional components: - Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex - FFS: Support of two PDSCH scrambling sequences per serving cell - Support of default QCL assumption per CORESETPoolIndex - Support of separate HARQ-ACK - Support of joint HARQ-ACK - Support of two TDMed long PUCCHs in a slot - Support of common closed-loop power control process for PUCCH/PUSCH associated with different CORESETPoolIndex | TBD |  | N/A |  | TBD [per band / per FSPC] | N/A | N/A |  |  | TBD |
|  | 16-2b | Single-DCI based multi-TRP | Basic component(s): - FFS: Support of MAC CE to activate multiple TCI states for a TCI codepoint - FFS: Number of CCs supporting single-DCI based multi-TRP operation Optional components: Support of default QCL assumption with two TCI states | TBD |  | N/A |  | TBD | N | TBD |  |  | TBD |
|  | 16-2b-1 | Single-DCI based SDM scheme | - FFS: Support of DCI indication of of 2 TCI states by a codepoint and DMRS ports within two CDM groups - Whether supporting two PTRS ports - FFS Support of DMRS entry {0, 2, 3} | 16-2b, TBD |  | N/A |  | TBD | N | TBD |  |  | TBD |
|  | 16-2b-2 | Single-DCI based FDMSchemeA | Support of FDMSchemeA | 16-2b, TBD |  | N/A |  | TBD | N | TBD |  |  | TBD |
|  | 16-2b-3 | Single-DCI based FDMSchemeB | - Support of FDMSchemeB - For FDMSchemeB, whether the UE can support CW soft combining | 16-2b, TBD |  | N/A |  | TBD [per FSPC] | N | TBD |  |  | TBD |
|  | 16-2b-4 | Single-DCI based TDMSchemeA | - Support of TDMSchemeA - Supported maximum TBS size for TDMSchemeA | 16-2b, TBD |  | N/A |  | TBD | N | TBD |  |  | TBD |
|  | 16-2b-5 | Single-DCI based inter-slot TDM | - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size according to RepNumR16 in PDSCH-TimeDomainResourceAllocation FFS: TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping) | 16-2b, TBD |  | N/A |  | TBD | N | TBD |  |  | TBD |
|  | 16-3a | Regular eType-II | Basic components: - FFS: {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II - 8 parameter combinations (FFS: Value of L per the number of antenna ports) - Support of PMI sub-bands with value R=1 - Rank restriction - UCI omission Optional components - Support of PMI sub-bands with R=2 - Support of rank 3,4 - CBSR - FFS: The maximum number of configured aperiodic CSI Report Settings - FFS: Support of mixed codebook types | TBD |  | N/A |  | FFS: Per band or Per band per BC | N/A | N/A |  |  | Optional |
|  | 16-3b | Port selection eType-II | Basic components: - FFS: {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II - 6 parameter combinations (combos with L=6 don’t apply) (FFS: Value of L per the number of antenna ports) - Support of PMI sub-bands with value R=1 - Rank restriction - UCI omission Optional components: - Support of PMI sub-bands with R=2 - Support of rank 3,4 - FFS: The maximum number of configured aperiodic CSI Report Settings - FFS: Support of mixed codebook types | TBD |  | N/A |  | FFS: Per band or Per band per BC | N/A | N/A |  |  | Optional |
|  | 16-4 | Low PAPR DMRS for DL | Low PAPR DMRS for PDSCH | TBD |  | N/A |  | Per UE | No | No |  |  | [Optional] |
|  | 16-5a | UL full power transmission mode 0 | - Supported UL full power transmission [mode 0] Number of Tx to support mode 0: {2Tx, 4Tx, 2Tx_4Tx} | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | N/A | N/A |  |  | TBD |
|  | 16-5b | UL full power transmission mode 1 | - Supported UL full power transmission mode 1 - Number of Tx to support mode 1: {2Tx, 4Tx, 2Tx_4Tx} FFS: New UL codebook set(s) per supported Tx | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | N/A | N/A |  |  | TBD |
|  | 16-5c | UL full power transmission mode 2 | - Supported UL full power transmission mode 2 - Number of Tx to support mode 2: {2Tx, 4Tx, 2Tx_4Tx} - The maximum number of SRS resources in set with different number of ports [for usage set to ‘codebook’]. FFS on details for supported number of Tx. - FFS: Number of ports per SRS resource - FFS: Maximum number of different spatial relation info for all SRS resources for usage set to ‘codebook’ in a resource set - TPMI group which delivers full power. FFS on details for supported number of Tx. Note: UE indicating mode 2 shall support full power transmission for 1 antenna port | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | N/A | N/A |  |  | TBD |
|  | 16-6a | Low PAPR DMRS for PUSCH | - For PUSCH without transform precoding - For PUSCH with transform precoding and with pi/2 BPSK modulation | TBD |  | N/A |  | FFS: Per band | N/A | N/A |  |  | FFS: Optional with capability signalling |
|  | 16-6b | Low PAPR DMRS for PUCCH | - For PUCCH format 3 and/or PUCCH format 4 with transform precoding and with pi/2 BPSK modulation | TBD |  | N/A |  | FFS: Per band | N/A | N/A |  |  | FFS: Optional with capability signalling |