3GPP TSG RAN WG1 #100bis	R1-2002393

e-Meeting, April 20th – 30th, 2020

Source:	Sharp

Title:	Discussion on UE feature for NR-U

Agenda Item:	7.2.11.2

Document for:	Discussion and Decision

Background

In [1], the outcome of the E-mail discussion of UE feature list are included for further discussion. We provide views on some of the NR-UE features.

Discussion

  - [10-19] Number of LBT bandwidth

FG 10-19 specifies the number of ED measurements the UE is able to perform simultaneously. We see that it is not necessary since the number of ED measurements should be implicitly determined by the supported channel bandwidth. If the UE supports 80 MHz channel bandwidth, the UE is required to support 4 ED measurements.

Proposal 1: FG 10-19 is not necessary since the number of ED measurements is determined by the supported channel bandwidth.

  - [10-19a] Support DL reception with subset of RB sets

The functionality of FG 10-19a coincides with FG 10-29. Both FG indicates RB-set indicator interpretation in DCI format 2_0. Thus, we propose to delete FG 10-19a from the list.

Proposal 2: FG 10-19a is deleted from the list since the functionality coincides with FG 10-29.

  - [10-29] Support available RB set indicator field in DCI 2_0

FG 10-29 requires Support of COT indication since the availability of the RB-sets continues until the end of the COT. Thus, this feature requires COT indication reception or dynamic SFI in DCI format 2_0.

Proposal 3: Set “10-30 or 3-6” as pre-requisite feature for FG 10-29.

Conclusion

In this contribution, we have the following proposals:

Proposal 1: FG 10-19 is not necessary since the number of ED measurements is determined by the supported channel bandwidth.

Proposal 2: FG 10-19a is deleted from the list since the functionality coincides with FG 10-29.

Proposal 3: Set “10-30 or 3-6” as pre-requisite feature for FG 10-29.

References

- R1-2001484, “RAN1 UE features list for Rel-16 NR after RAN1#100-E”, RAN1#100 e-Meeting, February 2020