3GPP TSG RAN WG1 Meeting RAN1#100bis	R1-2002410

e-Meeting, April 20th – 30th, 2020

Source:	Motorola Mobility, Lenovo

Title:	Remaining issues on MU-CSI Enhancements

Agenda Item:	7.2.6.1

Document for:	Discussion and Decision

# Introduction

At RAN1#98bis an agreement was reached for Rel-16 Type II CBSR process. In this contribution, we address a few issues related to the current agreement, and propose a solution that addresses these issues.

# CBSR Related Issues

## Current Agreement

In TS 38.214 [1], CBSR for NR eType-II is specified in Section 5.2.2.2.5, as follows

----------------------------Beginning of text for TS38.214 -----------------------------------------------------------

The bitmap parameter n1-n2-codebookSubsetRestriction-r16 forms the bit sequence B=B1B2 and configures the vector group indices g(k) as in clause 5.2.2.2.3. Bits b2k,2(N1x2+x1+1)b2k,2(N1x2+x1) indicate the maximum allowed average amplitude, γi+pL (p=0,1), with i∈{0,1,…,L-1}, of the coefficients associated with the vector in group g(k) indexed by x1,x2, where the maximum amplitudes are given in Table 5.2.2.2.5-6 and the average coefficient amplitude is restricted as follows

1f=0Mυ-1kl,i+pL,f(3)f=0Mυ-1kl,i+pL,f(3)pl,p1pl,i+pL,f22≤γi+pL

for l=1,…,υ, and p=0,1. A UE that does not report the parameter amplitudeSubsetRestriction='supported' in its capability signaling is not expected to be configured with b2k,2(N1x2+x1+1)b2k,2(N1x2+x1)=01 or 10.

Table 5.2.2.2.5-6: Maximum allowed average coefficient amplitudes for restricted vectors

| Bit b2k,2(N1x2+x1+1)b2k,2(N1x2+x1) | Maximum Average Coefficient Amplitude γi+pL |
| --- | --- |
| 00 | 0 |
| 01 | 1/4 |
| 10 | 1/2 |
| 11 | 1 |

----------------------------End of text for TS38.214 -----------------------------------------------------------

## Discussion on CBSR for NR eType-II

In RAN1#98bis [2], an agreement was reached to support soft CBSR with sum-power constraint on the delay-domain coefficients, as follows

1f=0Mυ-1kl,i+pL,f(3)f=0Mυ-1kl,i+pL,f(3)pl,p1pl,i+pL,f22≤γi+pL                (1)

Here, l, p, i and f indicate the layer, polarization, beam and delay indices, respectively. Mv is the total number of delay indices given rank v. pl,p1 is the reference amplitude value for layer l, polarization p. kl,i+pL,f(3) is a binary indicator of a non-zero coefficient in layer l, polarization p, beam i, delay index f, and pl,i+pL,f2 is the corresponding differential amplitude value. Finally, γi+pL is the amplitude restriction for beam i in polarization p, which is RRC configured with values from Table 5.2.2.2.5-6 above. To the best of our understanding, supporting a sum-power constraint for CBSR was theoretically motivated by Parseval’s theorem for Energy conservation, which states that the total Energy of a time-domain function is conserved after being transformed to the frequency domain via Fourier transform, i.e.,

n=0N-1x(n)2= k=0N-1X(k)2                               (2)

where n, k are time and frequency domain indices, respectively. Thereby, the wideband power of a restricted beam in eType-II codebook can be tied to the sum-power of the coefficients across the delay domain (prior to applying the IDFT to the precoder), up to a constant. One can rewrite the CBSR check equation in (1) as follows

f=0Mυ-1kl,i+pL,f(3)pl,p1pl,i+pL,f22≤γi+pL.f=0Mυ-1kl,i+pL,f3              (3)

Thereby, (3) imposes a restriction on the sum power of the coefficients over the delay indices f= 0, …, Mv-1. In that sense, the left-hand side of (3) resembles that in Parseval’s equation in (2), and hence one can regard the right-hand side of (3) as a threshold on the wideband power per polarization of beam i. However, one issue arises with the CBSR check equation in (3): the right-hand side representing the threshold varies with the square root of the number of non-zero coefficients in beam i,  f=0Mυ-1kl,i+pL,f3, leading the threshold to be scalable from 1 up to Mv, which is highly undesirable. Since the UE has control on the number of non-zero coefficients within the restricted beam, it can alter the codebook design so that a soft-restricted beam would pass the CBSR check, without actually reducing the average power of the restricted beam. The following example illustrates the underlying issue with the current CBSR check equation in (1), where two different parameter designs of a restricted beam are provided, where both designs have negligible difference in total power, yet showing substantial irregularity in the behavior of the CBSR check equation.

Example 1:

Setup. v=1 Mv=4, and beam i* is restricted with γi*=1/2

Design 1., p1,p1=1,  k1,i*,03=1,  p1,i*,02=1 and  k1,i*,13=k1,i*,23=k1,i*,33=0

i.e., W2(i*,:)=1   0   0   0.

Obviously, this beam violates the CBSR check equation in (1), since 1>12. However, the UE can alter the codebook design to pass the CBSR check, as follows

Design 2. p1,p1=1,  k1,i*,03=1,  p1,i*,02=1 and  k1,i*,13=k1,i*,23=1,  p1,i*,12=p1,i*,22=182, and  k1,i*,33=0

i.e., W2'(i*,:)=1  182  182   0.

Here, Design 2 passes the CBSR check in equation (1), since 130384<12, although the change in the power of beam i* is negligible. In general, the CBSR check equation in (1) may allow the UE to manipulate the CBSR process via converting one or more zero-valued coefficients within a restricted beam to non-zero coefficients with the minimum amplitude value to satisfy the CBSR threshold, without actually reducing the restricted beam power, as intended by the CBSR process.  Figure 1 supports the previous observation, in which system simulation results are used to compute the power of a beam whenever there is only one non-zero coefficient with amplitude larger than 1/2, similar to Design 1, and compare it with the beam power under Design 2, where two zero-valued coefficients are transformed to non-zero coefficients with amplitude 1/82 each. Assuming a CBSR threshold value of 1/2, the beam under Design 2 would pass the CBSR check in (1), despite having larger average power at each sub-band compared with Design 1, which would not pass the CBSR check. This implies the current CBSR equation in (1) is not robust. Note that such issue may require an additional CBSR check to be pursued at the network side, where the power of restricted beams could be further reduced to conform with CBSR requirements, leading to irregularities in the codebook and CQI mismatches. More details on the simulation assumptions can be found in the Appendix.

The current CBSR check equation is not robust and can be bypassed via manipulating the codebook design at the UE.

## Proposal for CBSR Equation

As shown in Example 1, the underlying issue with the CBSR check in (1) for eType-II is the normalization with the number of non-zero coefficients in the restricted beam, f=0Mυ-1kl,i+pL,f3, leading to scaling inconsistencies with particular amplitude values within the restricted beam. Instead, the beam power restriction should be tied to the strongest unrestricted beam in the codebook, i.e., the unrestricted beam with the maximum wideband power. For instance, the following CBSR equation can be used

1maxp∈0,1maxi'ϵLf=0Mυ-1kl,i'+pL,f(3)pl,p1pl,i'+pL,f22f=0Mυ-1kl,i+pL,f(3)pl,p1pl,i+pL,f22≤γi+pL        (4)

where L is the set of indices of the selected L beams that do not include beams from the restricted beam set g(k), which is defined in TS 38.214, Sec. 5.2.2.2.3 [1].

Figure 1: Beam i* power per sub-band using both designs in Example 1

The proposed CBSR check equation has the following characteristics:

- It resembles the CBSR behavior in Rel. 15 Type-II codebook, in the sense that the amplitude restriction is with respect to the largest wideband amplitude value of an unrestricted beam, which has value one.

- It avoids the issue encountered in Example 1 with the current CBSR check equation, where the UE could bypass reducing the restricted beam power via adding dummy coefficients with low amplitude. The proposed equation in (4) provides an intuitive and accurate behavior for CBSR checking.

- It is flexible, in the sense that it can be generalized to a frequency domain codebook, wherein the index f∈{0,1,…,Mv } can be replaced with f'∈0,1,…,Nsb-1 . Thereby, the proposed CBSR equation is expected to be forward-compatible with codebook designs in future releases.

Modify the CBSR check equation to be normalized with the power of the strongest selected beam that does not belong to the restricted set.

Consider the following text proposal for TS 38.214

---------------------------- Start of proposed TP for TS38.214 -----------------------------------------------------------

5.2.2.2.5	Enhanced Type II Codebook

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031}, and UE configured with higher layer parameter codebookType set to ' typeII-r16'

--- Unchanged text omitted ---------

The bitmap parameter n1-n2-codebookSubsetRestriction-r16 forms the bit sequence B=B1B2 and configures the vector group indices g(k) as in clause 5.2.2.2.3. Bits b2k,2(N1x2+x1+1)b2k,2(N1x2+x1) indicate the maximum allowed average amplitude, γi+pL (p=0,1), with i∈{0,1,…,L-1}, of the coefficients associated with the vector in group g(k) indexed by x1,x2, where the maximum amplitudes are given in Table 5.2.2.2.5-6 and the average coefficient amplitude is restricted as follows

1f=0Mυ-1kl,i+pL,f(3)f=0Mυ-1kl,i+pL,f(3)pl,p1pl,i+pL,f22≤γi+pL

1maxp∈0,1maxi'ϵLf=0Mυ-1kl,i'+pL,f(3)pl,p1pl,i'+pL,f22f=0Mυ-1kl,i+pL,f(3)pl,p1pl,i+pL,f22≤γi+pL

for l=1,…,υ, and p=0,1. L⊆0,1,…, L-1 is the set of indices of the selected beams that are not associated with any of the sets of group indices g(k) for k=0,1,2,3 described in 5.2.2.3. A UE that does not report the parameter amplitudeSubsetRestriction='supported' in its capability signaling is not expected to be configured with b2k,2(N1x2+x1+1)b2k,2(N1x2+x1)=01 or 10.

Table 5.2.2.2.5-6: Maximum allowed average coefficient amplitudes for restricted vectors

| Bit b2k,2(N1x2+x1+1)b2k,2(N1x2+x1) | Maximum Average Coefficient Amplitude γi+pL |
| --- | --- |
| 00 | 0 |
| 01 | 1/4 |
| 10 | 1/2 |
| 11 | 1 |

--- Unchanged text omitted ---------

----------------------------End of proposed TP for TS38.214 -----------------------------------------------------------

# Conclusion

Based on the discussion in this contribution we make the following observations:

- The current CBSR check equation is not robust and can be bypassed via manipulating the codebook design at the UE.

- Modify the CBSR check equation to be normalized with the power of the strongest selected beam that does not belong to the restricted set.

Our proposal is as follows:

- Consider the text proposal for TS38.214.

# References

| [1] | 3GPP NR TS 38.214, “Physical layer procedures for data,” Mar. 2020. |
| --- | --- |
| [2] | “RAN1#98bis Chairman's Notes,” Chongqing, CN, Oct. 14-20, 2019. |

# Appendix

| Modulation | Up to 256 QAM |
| --- | --- |
| Coding on PDSCH | LDPC |
| Numerology | 15KHz 14 OFDM symbol slot and 52 PRBs |
| Frequency band | 4 GHz |
| Simulation bandwidth | 10 MHz |
| Transmission scheme | Closed SU/MU-MIMO adaptation |
| Scenario | Dense Urban (Micro layer only) |
| UE antenna height and gain | TR36.873 |
| Channel model | 38.901 UMa channel model B |
| Inter-site distance | 200 m. |
| Traffic model | FTP model 1 with packet size 0.5 Mbytes |
| Type II coedebook parameters | (L, p0, β) = (4, 1/4, 1/4) |
| PRBs bundling per SB | 1 PRB |
| MU dimension | Up to 12 layers |
| SU dimension | 1/2 layers |
| Codeword (CW)-to-layer mapping | Single codeword |
| CSI feedback | PMI, CQI: every 5 slot; 4 slot delay, RI: every 5 slot; Sub-band based |
| Interference measurement | SU-CQI; CSI-IM for inter-cell interference measurement |
| ACK/NACK delay | The next available UL slot |
| Re-transmission delay | The next available DL slot after receiving NACK |
| Antenna configuration at TRxP | (M,N,P,Mg,Ng) =(8,1,2,1,1) (dH, dV)=(0.5, 0.8)λ |
| Antenna configuration at UE | (M,N,P,Mg,Ng) = (1,1,2,1,1) (dH, dV)=(0.5, N/A)λ |
| Scheduling | PF |
| Receiver | MMSE-IRC |
| Channel estimation | Non-ideal |
| UE Noise Figure | 9 dB |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |
| Handover margin (dB) | 1 dB |
| TRxP total transmit power | 41 dBm |