3GPP TSG-RAN WG1 Meeting #100bis	R1-2002412

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.5.6

Source:	ITRI

Title:	Maintenance of UL grant Type 2 scheduling activation

Document for:	Discussion and Decision

1	Introduction

In this contribution, we propose a text correction for the table specifying special fields for single UL grant Type 2 scheduling activation PDCCH validation.

2	Discussion and text proposal

When a UE is provided a single UL grant Type 2 configuration, HARQ process number and redundancy version with predetermined values (i.e., set to all ‘0’s) in a UL grant DCI can represent an activation for the UL grant type 2 transmission. This function can be supported by all DCI formats for UL grant scheduling (i.e., DCI format0_0/0_1/0_2). However, there is a non-necessary text in the corresponding specification. Therefore, we have the following text proposal for Table 10.2-1 in Section 10.2 of TS 38.213 V16.1.0 to remove the non-necessary text:

< Start of the text proposal > 10.2 PDCCH validation for DL SPS and UL grant Type 2 < Unchanged parts are omitted > Table 10.2-1: Special fields for single DL SPS or single UL grant Type 2 scheduling activation PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration DCI format 0_0/0_1/0_2; DCI format 1_0/1_2; DCI format 1_1. HARQ process number; set to all '0's; set to all '0's; set to all '0's. Redundancy version; set to all '0's; set to all '0's; For the enabled transport block: set to all '0's. < End of the text proposal >

References

TS38.213, Physical layer procedures for control (Release 16), v16.1.0.