3GPP TSG RAN WG1 #100b eMeeting	R1- 2002566

April 20th – 30th, 2020

Agenda item:	7.2.11.5

Source: 	Qualcomm

Title: 	Discussion on eURLLC and IIOT features

Document for:	Discussion and Decision

# 1	Introduction

The most updated feature list is available in [1]. Based on this document, our proposed changes and additions for eURLLC and IIOT features are presented in the subsequent section.

# 2         Proposed Changes and Additions for NR eURLLC

In the table below, our proposed changes/additions are captured with track changes.

| 11. NR_L1enh_URLLC | 11-1 | Monitoring DCI format 1_2 and DCI format 0_2 | 1) Supports monitoring DCI format 1_2 for DL scheduling 2) Supports monitoring DCI format 0_2 for UL scheduling |  | Yes | N/A |  | Per UE | Yes | Yes | The differentiation is from the perspective of the scheduling cell |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11-1a | Monitoring both DCI format 0_1/0_2 in the same search space | Supports monitoring both DCI format 0_1/0_2in the same search space | 11-1 | Yes | N/A |  | Per UE | Yes | Yes | The differentiation is from the perspective of the scheduling cell |  | Optional with capability signalling |
|  | 11-1b | Monitoring both DCI format 1_2 and 1_1 in the same search space | Supports monitoring both DCI format 1_2/1_1 in the same search space | 11-1 | Yes | N/A |  | Per UE | Yes | Yes | differentiation is from the perspective of the scheduling cell |  | Optional with capability signaling |
|  | 11-2 | Rel-16 PDCCH monitoring capability | - Supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span and the limit M on the maximum number of BDs per PDCCH monitoring span for combination (X, Y, ) 2) Supported combinations of (X,Y,u) 3) If UE reports the support of more than one combination of (X, Y) for a given SCS, and if multiple combinations of (X, Y) are valid for the span pattern, the span pattern with the maximum value of C and M from the valid combinations is applied. 4) Capability on the number of CCs with Rel-16 PDCCH monitoring capability on all the serving cells. |  | Yes | N/A |  | FSPC | N/A | N/A |  | This capability is necessary for SCS 15 kHz and 30 kHz. A list of separate UE capabilities C(X, Y, ), M(X, Y, ) for processing capability #1; A list of separate UE capabilities C(X, Y, ), M(X, Y, ) for processing capability #2; For component 4), if UE supports carrier aggregation with more than [x] DL carriers with Rel-16 PDCCH monitoring capability on all the carriers, UE should report this capability. Value of x is 2. | Optional with capability signalling Candidate value set for (X, Y): {(7, 3), (4, 3), (3,2) (2, 2)} The value of C for combination (7, 3) for 15 kHz and 30 kHz is 56 FFS the value of C for combination (4, 3) and (3,2) and (2, 2) FFS the value of M for combination (7, 3), (4, 3) and (3,2) and (2, 2) Candidate value for component 5): { 2, 3, …, 16} |
|  | 11-2a | Mix of Rel. 16 PDCCH monitoring capability and FG3-1 PDCCH monitoring capability in the same slot in the same CC | 1) Supports PDCCH monitoring operation according to FG3-1 2) In addition to 1), supports PDCCH monitoring with limit C on the maximum number of additional non-overlapped CCEs for channel estimation per PDCCH monitoring span and with limit M on the maximum number of additional BDs, for a combination (X, Y, ) 3) Supported combinations of (X, Y, ) 4) If UE reports the support of more than one combination of (X, Y) for a given SCS, and if multiple combinations of (X, Y) are valid for the span pattern, the span pattern with the maximum value of C and M from the valid combinations is applied | 11-2 | Yes | N/A |  | FSPC | N/A | N/A |  | This capability is necessary for SCS 15kHz and 30 kHz. Component-3 candidate value set: (X, Y) = {(7, 3), (4, 3), (3,2) (2, 2)} The candidate values for capability on the number of CCs with FG3-1 PDCCH monitoring capability are {2,3,…,16} The candidate values for capability on the number of CCs with Rel-16 PDCCH monitoring capability are {1,2,…,16} | Optional with capability signalling |
|  | 11-2b | Mix of Rel. 16 PDCCH monitoring capability and FG3-2 PDCCH monitoring capability in the same slot in the same CC | 1) Supports PDCCH monitoring operation according to FG3-2 2) In addition to 1), supports PDCCH monitoring with limit C on the maximum number of additional non-overlapped CCEs for channel estimation per PDCCH monitoring span and with limit M on the maximum number of additional BDs, for a combination (X, Y, ) 3) Supported combinations of (X, Y, ) 4) If UE reports the support of more than one combination of (X, Y) for a given SCS, and if multiple combinations of (X, Y) are valid for the span pattern, the span pattern with the maximum value of C and M from the valid combinations is applied | 11-2, 3-2 | Yes | N/A |  | FSPC | N/A | N/A |  | This capability is necessary for SCS 15kHz and 30 kHz. Component-3 candidate value set: (X, Y) = {(7, 3), (4, 3), (3,2), (2, 2)} The candidate values for capability on the number of CCs with FG3-2 PDCCH monitoring capability are {2,3,…,16} The candidate values for capability on the number of CCs with Rel-16 PDCCH monitoring capability are {1,2,…,16} | Optional with capability signaling |
|  | 11-2c | Mix of Rel. 16 PDCCH monitoring capability and FG3-5b PDCCH monitoring capability in the same slot in the same CC | 1) Supports PDCCH monitoring operation according to FG3-5b for combination (X1, Y1, ) 2) In addition to 1), supports PDCCH monitoring with limit C on the maximum number of additional non-overlapped CCEs for channel estimation per PDCCH monitoring span and with limit M on the maximum number of additional BDs, for a combination (X, Y, ) 3) Supported combinations of (X1, Y1, ) 4) Supported combinations of (X2, Y2, ) 5) If UE reports the support of more than one combination of (X2, Y2) for a given SCS, and if multiple combinations of (X2, Y2) are valid for the span pattern, the span pattern with the maximum value of C and M from the valid combinations is applied | 11-2 | Yes | N/A |  | FSPC | N/A | N/A |  | This capability is necessary for SCS 15kHz and 30 kHz. Component-3 candidate value set: (X1, Y1) = {(7, 3), (4, 3) and (7, 3), (2, 2) and (4, 3) and (7, 3)} Component-4 candidate value set: (X2, Y2) = {(7, 3), (4, 3), (3,2), (2, 2)} The candidate values for capability on the number of CCs with FG3-5b PDCCH monitoring capability are {2,3,…,16} The candidate values for capability on the number of CCs with Rel-16 PDCCH monitoring capability are {1,2,…,16} | Optional with capability signaling |
|  | 11-2d | Relative TDRA for DL | The time domain resource allocation for PDSCH is a function of the PDCCH symbol index within the slot | 11-2 | Yes | N/A |  | FS | N/A | N/A |  |  | Optional with capability signaling |
|  | 11-3 | More than one PUCCH for HARQ-ACK transmission within a slot | 1) Supports sub-slot based HARQ-ACK feedback procedure. • A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot. • At least one sub-slot configuration for PUCCH can be UE specifically configured to a UE. • Supports a single configuration for PUCCH resource for all sub-slots in a slot. The starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot. Any sub-slot PUCCH resource is not across sub-slot boundaries. 2) Supported sub-slot configuration 3) Supported combinations of (A, B), where A is the minimum gap between sub-slots containing actual PUCCH transmissions measured from beginning to beginning of the sub-slots, including across slots, and B is the sub-slot duration, with both A and B in units of symbols |  | Yes | N/A |  | PerBand | N/A | N/A |  | Candidate value set for component 2): { 7-symbol*2, 2-symbol*3 and 7-symbol*2, 7-symbol*2 and 2-symbol*3 and 2-symbol*4, 7-symbol*2 and 2-symbol*3 and 2-symbol*4 and 2-symbol*7} Candidate value set for component 3): (A, B) = {(7, 7), (4, 2) and (7, 7), (3,2) and (4,2) and (7,7), (2, 2) and (3,2) and (4,2) and (7, 7)} | Optional with capability signalling |
|  | 11-4 | Two HARQ-ACK codebooks simultaneously constructed for supporting PDSCH reception with different priorities at a UE with restriction | 1) Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed 2) Supports separate PUCCH configuration for different HARQ-ACK codebooks 3) Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. 4) Only one of the HARQ-ACK codebooks can have a sub-slot based PUCCH configuration 5) Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and ‘codeBlockGroupTransmission” for different HARQ-ACK codebooks. [6) Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot] 7) If both processing time capability 1 CC(s) and processing time capability 2 CC(s) are configured, and both slot-based and sub-slot based PUCCH are configured, then HARQ-ACK feedback for a processing time capability 1 CC can only take place in the slot-based PUCCH and HARQ-ACK feedback for a processing time capability 2 CC can only take place in the sub-slot based PUCCH | 11-3 | Yes | N/A |  | FS | N/A | N/A |  |  | Optional with capability signalling |
|  | 11-4x | Two HARQ-ACK codebooks simultaneously constructed for supporting PDSCH reception with different priorities at a UE without restriction | 1) Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed. 2) Supports separate PUCCH configuration for different HARQ-ACK codebooks 3) Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. 5) Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and ‘codeBlockGroupTransmission” for different HARQ-ACK codebooks. | 11-3, 11-4 | Yes | N/A |  | FS | N/A | N/A |  |  | Optional with capability signalling |
|  | 11-4a | DL priority indication in DCI with mixed DCI formats | When both DCI format 1_1 and DCI format 1_2 are configured to be monitored per BWP, only one DCI format (from the formats 1_1/1_2) can be used to schedule PDSCH with low priority HARQ-ACK and only one can be used to schedule PDSCH with high priority HARQ-ACK. | 11-1, 11-4 | Yes | N/A |  | Per UE | Yes | Yes | The differentiation is from the perspective of the scheduling cell. |  | Optional with capability signalling |
|  | 11-4b | UL priority indication in DCI with mixed DCI formats | When both DCI format 0_1 and DCI format 0_2 are configured to be monitored per BWP, only one DCI format (from the formats 0_1/0_2) can be used to schedule PUSCH with low priority and only one can be used to schedule PUSCH with high priority. | 11-1, 11-4 | Yes | N/A |  | Per UE | Yes | Yes | The differentiation is from the perspective of the scheduling cell. |  | Optional with capability signaling |
|  | 11-5a | PUSCH repetition type B with 1 unicast PUSCHs per slot per CC with UE processing time capability 1 | - For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with up to 1 unicast PUSCHs per slot per CC with UE processing time capability 1. - Supported scheduling mode 3 Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. [4) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field.] 5) PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 6) S and L are separately indicated (4-bit for S and 4-bit for L). L <= 14. 7) TBS is determined based on L indicated in TDRA table entry reusing Rel-15 mechanism. 8) Handling of interaction with DL/UL directions depending on whether dynamic SFI is configured or not, including both cases with and without higher layer parameter InvalidSymbolPattern configured 9) Supported maximum number of actual repetitions within a slot [10) Supported PUSCH hopping scheme] |  | Yes | N/A |  | PerBand | N/A | N/A |  | Candidate value for component 9): {1, 2, 3, 4, 7, 8, 12, 16} Candidate value for component 2: {self-carrier scheduling, cross-carrier scheduling, none) FFS: Whether to set separate UE capabilities for dynamic grant and configured grant. Can we just add some note here with an example below for compromise? PUSCH repetition type B with configured grant is applied only if UE reports the support of FG 5-19 or FG 5-20, and subjected to the capability of FG 5-19 and FG 5-20. FFS: Whether to set separate UE capabilities for the case that dynamic SFI is configured and InvalidSymbolPattern is configured. Can we just add some note here with an example below for compromise? [The case that both dynamic SFI and InvalidSymbolPattern are configured is applied only if UE reports the support of FG3-6.] FFS: Whether to set separate UE capabilities for DCI format 0_1 and DCI format 0_2 for PUSCH repetition type B. Can we go majority view that no separate UE capability? | Optional with capability signalling |
|  | 11-5b | PUSCH repetition type B with up to 2 unicast PUSCHs per slot per CC with UE processing time capability 1 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with up to 2 unicast PUSCHs per slot per CC with UE processing time capability 1. 2)Supported scheduling mode 3) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. [4) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field.] 5) PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 6) S and L are separately indicated (4-bit for S and 4-bit for L). L <= 14. 7) TBS is determined based on L indicated in TDRA table entry reusing Rel-15 mechanism. 8) Handling of interaction with DL/UL directions depending on whether dynamic SFI is configured or not, including both cases with and without higher layer parameter InvalidSymbolPattern configured 9) Supported maximum number of actual repetitions within a slot [10) Supported PUSCH hopping scheme] | 5-12 | Yes | N/A |  | PerBand | N/A | N/A |  | Candidate value for component 9): {1, 2, 3, 4, 7, 8, 12, 16} Candidate value for component 2: {self-carrier scheduling, cross-carrier scheduling, none) FFS: Whether to set separate UE capabilities for dynamic grant and configured grant. Can we just add some note here with an example below for compromise? PUSCH repetition type B with configured grant is applied only if UE reports the support of FG 5-19 or FG 5-20, and subjected to the capability of FG 5-19 and FG 5-20. FFS: Whether to set separate UE capabilities for the case that dynamic SFI is configured and InvalidSymbolPattern is configured. Can we just add some note here with an example below for compromise? [The case that both dynamic SFI and InvalidSymbolPattern are configured is applied only if UE reports the support of FG3-6.] FFS: Whether to set separate UE capabilities for DCI format 0_1 and DCI format 0_2 for PUSCH repetition type B. Can we go majority view that no separate UE capability? | Optional with capability signalling |
|  | 11-5c | PUSCH repetition type B with up to 7 unicast PUSCHs per slot per CC with UE processing time capability 1 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with up to 7 unicast PUSCHs per slot per CC with UE processing time capability 1. 2)Supported scheduling mode 3) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. [4) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field.] 5) PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 6) S and L are separately indicated (4-bit for S and 4-bit for L). L <= 14. 7) TBS is determined based on L indicated in TDRA table entry reusing Rel-15 mechanism. 8) Handling of interaction with DL/UL directions depending on whether dynamic SFI is configured or not, including both cases with and without higher layer parameter InvalidSymbolPattern configured 9) Supported maximum number of actual repetitions within a slot [10) Supported PUSCH hopping scheme] | 5-12a | Yes | N/A |  | PerBand | N/A | N/A |  | Candidate value for component 9): {1, 2, 3, 4, 7, 8, 12, 16} Candidate value for component 2: {self-carrier scheduling, cross-carrier scheduling, none) FFS: Whether to set separate UE capabilities for dynamic grant and configured grant. Can we just add some note here with an example below for compromise? PUSCH repetition type B with configured grant is applied only if UE reports the support of FG 5-19 or FG 5-20, and subjected to the capability of FG 5-19 and FG 5-20. FFS: Whether to set separate UE capabilities for the case that dynamic SFI is configured and InvalidSymbolPattern is configured. Can we just add some note here with an example below for compromise? [The case that both dynamic SFI and InvalidSymbolPattern are configured is applied only if UE reports the support of FG3-6.] FFS: Whether to set separate UE capabilities for DCI format 0_1 and DCI format 0_2 for PUSCH repetition type B. Can we go majority view that no separate UE capability? | Optional with capability signalling |
|  | 11-5d | PUSCH repetition type B with up to 4 unicast PUSCHs per slot per CC with UE processing time capability 1 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with up to 4 unicast PUSCHs per slot per CC with UE processing time capability 1. 2)Supported scheduling mode 3) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. [4) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field.] 5) PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 6) S and L are separately indicated (4-bit for S and 4-bit for L). L <= 14. 7) TBS is determined based on L indicated in TDRA table entry reusing Rel-15 mechanism. 8) Handling of interaction with DL/UL directions depending on whether dynamic SFI is configured or not, including both cases with and without higher layer parameter InvalidSymbolPattern configured 9) Supported maximum number of actual repetitions within a slot [10) Supported PUSCH hopping scheme] | 5-12b | Yes | N/A |  | PerBand | N/A | N/A |  | Candidate value for component 9): {1, 2, 3, 4, 7, 8, 12, 16} Candidate value for component 2: {self-carrier scheduling, cross-carrier scheduling, none) FFS: Whether to set separate UE capabilities for dynamic grant and configured grant. Can we just add some note here with an example below for compromise? PUSCH repetition type B with configured grant is applied only if UE reports the support of FG 5-19 or FG 5-20, and subjected to the capability of FG 5-19 and FG 5-20. FFS: Whether to set separate UE capabilities for the case that dynamic SFI is configured and InvalidSymbolPattern is configured. Can we just add some note here with an example below for compromise? [The case that both dynamic SFI and InvalidSymbolPattern are configured is applied only if UE reports the support of FG3-6.] FFS: Whether to set separate UE capabilities for DCI format 0_1 and DCI format 0_2 for PUSCH repetition type B. Can we go majority view that no separate UE capability? | Optional with capability signalling |
|  | 11-5e | PUSCH repetition type B with up to 3 unicast PUSCHs per slot per CC with UE processing time capability 1 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with up to 3 unicast PUSCHs per slot per CC with UE processing time capability 1. 2)Supported scheduling mode 3) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. [4) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field.] 5) PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 6) S and L are separately indicated (4-bit for S and 4-bit for L). L <= 14. 7) TBS is determined based on L indicated in TDRA table entry reusing Rel-15 mechanism. 8) Handling of interaction with DL/UL directions depending on whether dynamic SFI is configured or not, including both cases with and without higher layer parameter InvalidSymbolPattern configured 9) Supported maximum number of actual repetitions within a slot [10) Supported PUSCH hopping scheme] | 5-12c | Yes | N/A |  | PerBand | N/A | N/A |  | Candidate value for component 9): {1, 2, 3, 4, 7, 8, 12, 16} Candidate value for component 2: {self-carrier scheduling, cross-carrier scheduling, none) FFS: Whether to set separate UE capabilities for dynamic grant and configured grant. Can we just add some note here with an example below for compromise? PUSCH repetition type B with configured grant is applied only if UE reports the support of FG 5-19 or FG 5-20, and subjected to the capability of FG 5-19 and FG 5-20. FFS: Whether to set separate UE capabilities for the case that dynamic SFI is configured and InvalidSymbolPattern is configured. Can we just add some note here with an example below for compromise? [The case that both dynamic SFI and InvalidSymbolPattern are configured is applied only if UE reports the support of FG3-6.] FFS: Whether to set separate UE capabilities for DCI format 0_1 and DCI format 0_2 for PUSCH repetition type B. Can we go majority view that no separate UE capability? | Optional with capability signalling |
|  | 11-5f | PUSCH repetition type B with 1 unicast PUSCHs per slot per CC with UE processing time capability 2 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with 1 unicast PUSCHs per slot per CC with UE processing time capability 2. 2)Supported scheduling mode 3) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. [4) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field.] 5) PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 6) S and L are separately indicated (4-bit for S and 4-bit for L). L <= 14. 7) TBS is determined based on L indicated in TDRA table entry reusing Rel-15 mechanism. 8) Handling of interaction with DL/UL directions depending on whether dynamic SFI is configured or not, including both cases with and without higher layer parameter InvalidSymbolPattern configured 9) Supported maximum number of actual repetitions within a slot [10) Supported PUSCH hopping scheme] | 5-5c | Yes | N/A |  | PerBand | N/A | N/A |  | Candidate value for component 9): {1, 2, 3, 4, 7, 8, 12, 16} Candidate value for component 2: {self-carrier scheduling, cross-carrier scheduling, none) FFS: Whether to set separate UE capabilities for dynamic grant and configured grant. Can we just add some note here with an example below for compromise? PUSCH repetition type B with configured grant is applied only if UE reports the support of FG 5-19 or FG 5-20, and subjected to the capability of FG 5-19 and FG 5-20. FFS: Whether to set separate UE capabilities for the case that dynamic SFI is configured and InvalidSymbolPattern is configured. Can we just add some note here with an example below for compromise? [The case that both dynamic SFI and InvalidSymbolPattern are configured is applied only if UE reports the support of FG3-6.] FFS: Whether to set separate UE capabilities for DCI format 0_1 and DCI format 0_2 for PUSCH repetition type B. Can we go majority view that no separate UE capability? | Optional with capability signalling |
|  | 11-5g | PUSCH repetition type B with up to 2 unicast PUSCHs per slot per CC with UE processing time capability 2 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with up to 2 unicast PUSCHs per slot per CC with UE processing time capability 2. 2)Supported scheduling mode 3) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. [4) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field.] 5) PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 6) S and L are separately indicated (4-bit for S and 4-bit for L). L <= 14. 7) TBS is determined based on L indicated in TDRA table entry reusing Rel-15 mechanism. 8) Handling of interaction with DL/UL directions depending on whether dynamic SFI is configured or not, including both cases with and without higher layer parameter InvalidSymbolPattern configured 9) Supported maximum number of actual repetitions within a slot [10) Supported PUSCH hopping scheme] | 5-13d | Yes | N/A |  | PerBand | N/A | N/A |  | Candidate value for component 9): {1, 2, 3, 4, 7, 8, 12, 16} Candidate value for component 2: {self-carrier scheduling, cross-carrier scheduling, none) FFS: Whether to set separate UE capabilities for dynamic grant and configured grant. Can we just add some note here with an example below for compromise? PUSCH repetition type B with configured grant is applied only if UE reports the support of FG 5-19 or FG 5-20, and subjected to the capability of FG 5-19 and FG 5-20. FFS: Whether to set separate UE capabilities for the case that dynamic SFI is configured and InvalidSymbolPattern is configured. Can we just add some note here with an example below for compromise? [The case that both dynamic SFI and InvalidSymbolPattern are configured is applied only if UE reports the support of FG3-6.] FFS: Whether to set separate UE capabilities for DCI format 0_1 and DCI format 0_2 for PUSCH repetition type B. Can we go majority view that no separate UE capability? | Optional with capability signalling |
|  | 11-5h | PUSCH repetition type B with up to 7 unicast PUSCHs per slot per CC with UE processing time capability 2 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with up to 7 unicast PUSCHs per slot per CC with UE processing time capability 2. 2)Supported scheduling mode 3) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. [4) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field.] 5) PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 6) S and L are separately indicated (4-bit for S and 4-bit for L). L <= 14. 7) TBS is determined based on L indicated in TDRA table entry reusing Rel-15 mechanism. 8) Handling of interaction with DL/UL directions depending on whether dynamic SFI is configured or not, including both cases with and without higher layer parameter InvalidSymbolPattern configured 9) Supported maximum number of actual repetitions within a slot [10) Supported PUSCH hopping scheme] | 5-13e | Yes | N/A |  | PerBand | N/A | N/A |  | Candidate value for component 9): {1, 2, 3, 4, 7, 8, 12, 16} Candidate value for component 2: {self-carrier scheduling, cross-carrier scheduling, none) FFS: Whether to set separate UE capabilities for dynamic grant and configured grant. Can we just add some note here with an example below for compromise? PUSCH repetition type B with configured grant is applied only if UE reports the support of FG 5-19 or FG 5-20, and subjected to the capability of FG 5-19 and FG 5-20. FFS: Whether to set separate UE capabilities for the case that dynamic SFI is configured and InvalidSymbolPattern is configured. Can we just add some note here with an example below for compromise? [The case that both dynamic SFI and InvalidSymbolPattern are configured is applied only if UE reports the support of FG3-6.] FFS: Whether to set separate UE capabilities for DCI format 0_1 and DCI format 0_2 for PUSCH repetition type B. Can we go majority view that no separate UE capability? | Optional with capability signalling |
|  | 11-5i | PUSCH repetition type B with up to 4 unicast PUSCHs per slot per CC with UE processing time capability 2 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with up to 4 unicast PUSCHs per slot per with UE processing time capability 2. 2)Supported scheduling mode 3) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. [4) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field.] 5) PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 6) S and L are separately indicated (4-bit for S and 4-bit for L). L <= 14. 7) TBS is determined based on L indicated in TDRA table entry reusing Rel-15 mechanism. 8) Handling of interaction with DL/UL directions depending on whether dynamic SFI is configured or not, including both cases with and without higher layer parameter InvalidSymbolPattern configured 9) Supported maximum number of actual repetitions within a slot [10) Supported PUSCH hopping scheme] | 5-13f | Yes | N/A |  | PerBand | N/A | N/A |  | Candidate value for component 9): {1, 2, 3, 4, 7, 8, 12, 16} Candidate value for component 2: {self-carrier scheduling, cross-carrier scheduling, none) FFS: Whether to set separate UE capabilities for dynamic grant and configured grant. Can we just add some note here with an example below for compromise? PUSCH repetition type B with configured grant is applied only if UE reports the support of FG 5-19 or FG 5-20, and subjected to the capability of FG 5-19 and FG 5-20. FFS: Whether to set separate UE capabilities for the case that dynamic SFI is configured and InvalidSymbolPattern is configured. Can we just add some note here with an example below for compromise? [The case that both dynamic SFI and InvalidSymbolPattern are configured is applied only if UE reports the support of FG3-6.] FFS: Whether to set separate UE capabilities for DCI format 0_1 and DCI format 0_2 for PUSCH repetition type B. Can we go majority view that no separate UE capability? | Optional with capability signalling |
|  | 11-5j | PUSCH repetition type B with up to 3 unicast PUSCHs per slot per CC with UE processing time capability 2 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with up to 3 unicast PUSCHs per slot per with UE processing time capability 2. 2)Supported scheduling mode 3) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. [4) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field.] 5) PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 6) S and L are separately indicated (4-bit for S and 4-bit for L). L <= 14. 7) TBS is determined based on L indicated in TDRA table entry reusing Rel-15 mechanism. 8) Handling of interaction with DL/UL directions depending on whether dynamic SFI is configured or not, including both cases with and without higher layer parameter InvalidSymbolPattern configured 9) Supported maximum number of actual repetitions within a slot [10) Supported PUSCH hopping scheme] |  | Yes | N/A |  | PerBand | N/A | N/A |  | Candidate value for component 9): {1, 2, 3, 4, 7, 8, 12, 16} Candidate value for component 2: {self-carrier scheduling, cross-carrier scheduling, none) FFS: Whether to set separate UE capabilities for dynamic grant and configured grant. Can we just add some note here with an example below for compromise? PUSCH repetition type B with configured grant is applied only if UE reports the support of FG 5-19 or FG 5-20, and subjected to the capability of FG 5-19 and FG 5-20. FFS: Whether to set separate UE capabilities for the case that dynamic SFI is configured and InvalidSymbolPattern is configured. Can we just add some note here with an example below for compromise? [The case that both dynamic SFI and InvalidSymbolPattern are configured is applied only if UE reports the support of FG3-6.] FFS: Whether to set separate UE capabilities for DCI format 0_1 and DCI format 0_2 for PUSCH repetition type B. Can we go majority view that no separate UE capability? | Optional with capability signaling |
|  | 11-6 | PUSCH repetition Type A | 1) PUSCH transmission with Rel-15 behavior with or without slot aggregation. • With slot aggregation, the number of repetitions can be [either semi-statically configured (as in Rel-15) or] dynamically indicated (as agreed for Rel-16). • When dynamically indicated, the number of repetitions is jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. - Maximum number of PUSCH repetitions |  | Yes | N/A |  | PerBand | N/A | N/A |  | Componenet-1 candidate value set: {‘semi-static only’, ‘both semi-static and dynamic’} Componenet-2 candidate value set: {1,2,3,4,7,8,12,16} | Optional with capability signalling |
|  | 11-7a | UL cancelation scheme on same CC | 1) Supports group common DCI (i.e. DCI format 2_4) for cancelation indication on the same CC as PUSCH or SRS 2) UL cancelation for PUSCH • Cancellation is applied to each PUSCH repetition individually in case of PUSCH repetitions 3) UL cancelation for SRS symbols that overlap with the cancelled symbols [4) For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by higher layer.] |  | Yes | N/A |  | FS | N/A | N/A |  | [More than one monitoring occasion for DCI format 2_4 per slot is applied only if the UE reports to support FG 3-5 or FG 3-5a or FG 3-5b] | Optional with capability signalling |
|  | 11-7b | More than one monitoring occasion for DCI 2_4 per slot on same CC | Monitoring occasions per slot for DCI 2_4 reception on the same CC as PUSCH or SRS Supported combinations of (X, Y), where X is the minimum gap between monitoring occasions measured from beginning to beginning of the monitoring occasions, including across slots, and Y is the duration of the monitoring occasion, with both X and Y in units of symbols | 11-7a | Yes | N/A |  | FS | N/A | N/A |  | Candidate value set: (X, Y) = {(7, 3), (7, 3) and (4, 3), (7, 3) and (4, 3) and (3,2), (7, 3) and (4,3) and (3,2) and (2,2), (7,3) and (4,3) and (2,2)} | Optional with capability signaling |
|  | 11-7c | UL cancellation scheme on another CC | 1) Supports group common DCI (i.e. DCI format 2_4) for cancelation indication on a different CC as PUSCH or SRS 2) UL cancelation for PUSCH • Cancellation is applied to each PUSCH repetition individually in case of PUSCH repetitions 3) UL cancelation for SRS symbols that overlap with the cancelled symbols [4) For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by higher layer.] |  | Yes | N/A |  | FS | N/A | N/A |  |  | Optional with capability signaling |
|  | 11-7d | More than one monitoring occasion for DCI 2_4 per slot on another CC | Monitoring occasions per slot for DCI 2_4 reception on a different CC from PUSCH or SRS Supported combinations of (X, Y), where X is the minimum gap between monitoring occasions measured from beginning to beginning of the monitoring occasions, including across slots, and Y is the duration of the monitoring occasion, with both X and Y in units of symbols | 11-7c | Yes | N/A |  | FS | N/A | N/A |  | Candidate value set: (X, Y) = {(7, 3), (7, 3) and (4, 3), (7, 3) and (4, 3) and (3,2), (7, 3) and (4,3) and (3,2) and (2,2), (7,3) and (4,3) and (2,2)} | Optional with capability signaling |
|  | 11-7e | Cancellation of the overlapping PUSCHs in an intra-band UL CA without indication in the DCI format 2-4 | 1) For a UE indicating the capability of pa-PhaseDiscontinuityImpacts, and if the PUSCH on at least one serving cell is cancelled, the UE may cancel the (repetition of the) PUSCHs transmission on all other intra-band serving cell(s). The cancellation of the (repetition of the) PUSCH transmission on a the set of intra-band serving cell(s) includes all symbols from the earliest symbol that is overlapping with the first cancelled symbol of the PUSCH on the serving cell for which the DCI format 2_4 is applicable to. | 6-23, 11-7 | Yes | N/A |  | PerBand | N/A | N/A |  |  | Optional with capability signaling |
|  | 11-8 | Enhanced UL power control scheme | For DG-PUSCH, one bit (separately from SRI) in UL grant is used to indicate the open loop power control parameter set if SRI is present in the UL grant, and 1 or 2 bits is used to indicate the P0 value if SRI is not present in the UL grant |  | Yes | N/A |  | Per UE | Yes | Yes | If UE supports 11-8 and also supports cross-carrier scheduling, then the differentiation is from the perspective of the scheduled cell |  | Optional with capability signalling |
|  | 11-9 | Multiple active configured grant configurations for a BWP of a serving cell | 1) Supports up to 12 configured/active configured grant configurations in a BWP of a serving cell. • Separate RRC parameters for different configured grant configurations • Separate activation for different configured grant Type 2 configurations • Separate release for different configured grant Type 2 configurations 2) Supported maximum number of configured grant configurations in a BWP of a serving cell 3) Supported maximum number of configured grant configurations across all serving cells] |  | Yes | N/A |  | Per UE | Yes | Yes |  | Component-2, candidate value set is {1, 2, …, 12} Component-3, candidate value set is {1, 2, …, [24]} Total number in FR1 is not greater than X value reported for FR1. Total number in FR2 is not greater than X value reported for FR2. Total number across FR1 and FR2 is not greater than the larger of the FR1 and FR2 values | Optional with capability signalling |
|  | 11-9a | Joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell | M<=4 bits indication in the Release DCI is used for indicating which CG configuration(s) is/are released, where the association between each state indicated by the indication and the CG configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple CG configurations to be released • In case of no higher layer configured state(s), separate release is used where the release corresponds to the CG configuration index indicated by the indication | 11-9 | Yes | N/A |  | Per UE | Yes | Yes | The differentiation is from the perspective of the cell with release DCI | FFS: A UE supporting this feature shall also support 11-10 (Type 2 configured grant release by DCI format 0_1). A UE supporting this feature and 11-1 (DCI format 0_2/1_2) shall also support 11-11 (Type 2 configured grant release by DCI format 0_2). | Optional with capability signalling |
|  | 11-10 | Type 2 configured grant release by DCI format 0_1 and 0_2 | - Support of type 2 configured grant release by DCI format 0_1 - Support of type 2 configured grant release by DCI format 0_2 |  | Yes | N/A |  | Per UE | Yes | Yes | The capability interpretation is from the perspective of a carrier on which the release DCI is received | [A UE supporting component 1 and 11-1 (DCI format 0_2/1_2) shall also support component 2 (Type 2 configured grant release by DCI format 0_2).] | Optional with capability signalling |

We further propose the following FGs. The proposed changes in 5-11c, 5-12c, 5-13d and 5-13g are to allow for 3 TB scheduling in both UL and DL. Considering the (4,3) span pattern of FG 3-5b, these additions would allow for matching the number of TBs and the spans in each slot. In addition, the proposed FG 11-3a-e would allow for capability signalling for the simultaneous use of CBG-based UL transmission and minimum processing capability 2.

| 5-11c | Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 PDSCH(s) for Msg. 4 is included |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5-12c | Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1 |  | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
| 5-13d | Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 3 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5a or 5-5b | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS More than one set of per SCS per band reports can be signaled for a given band combination | Optional with capability signalling |
| 5-13g | Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology • When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 3 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5c | Yes | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS More than one set of per SCS per band reports can be signaled for a given band combination | Optional with capability signalling |

| 11-3a | CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-5a or 5-5b | Yes | N/A |  | Per UE | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11-3b | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13 | Yes | N/A |  | Per UE | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |
| 11-3c | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13a | Yes | N/A |  | Per UE | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |
| 11-3d | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13c | Yes | N/A |  | Per UE | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |
| 11-3e | CBG based transmission for UL with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13d | Yes | N/A |  | Per UE | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |

# 2         Proposed Changes and Additions for NR IIOT

In the table below, our proposed changes/additions are captured with track changes.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12. NR_IIOT | 12-1 | UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer | Support intra-UE multiplexing/prioritization of UL overlapping channels/signals with two priority levels in physical layer (PHY) - Configuration of PHY priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH - Multiplexing/prioritization between UL channels/signals with the same PHY priority level - Prioritization between UL channels/signals with different PHY priority levels - Additional number of symbols (d1) needed beyond the PUSCH preparation time for cancelling a low priority UL transmission. - Additional number of symbols (d2) needed beyond the PUSCH preparation time for scheduling a high priority UL transmission that cancels a low priority UL transmission |  | Yes | N/A |  | FSPC | N/A | N/A |  | [A UE supporting this feature shall also support the LCP restriction based on DCI priority indication ([lch-ToGrantPriorityRestriction-r16]) and intra-UE prioritization in MAC ([lch-PriorityBasedPrioritization-r16])]. The relationship between this feature and the feature of up to two HARQ-ACK codebooks of 11-4[x] including merging these features should be further discussed. | Optional with capability signaling Candidate value set for component 4: {0, 1, 2} Candidate value set for component 5: {0, 1, 2} |
|  | 12-x | Fixed TB CRC for interrupted initial PUSCH | TB CRC set to all zeros for a re-transmission of a TB in case the initial transmission was cancelled | 5-25 | Yes | N/A |  | PerBand | N/A | N/A |  | The cancellation could be due to support of ULCI and/or intra-UE prioritization | Optional with capability signaling |
|  | 12-2 | Multiple SPS configurations | - Support of up to 8 configured SPS configurations in a BWP of a serving cell, including separate RRC parameters and separate activation/release for different SPS configurations - The max number of active SPS configurations in a BWP of a serving cell - The max number of active SPS configurations across all serving cells - The related HARQ-ACK enhancements to support multiple active SPS configurations | 5-18 | Yes | N/A |  | Per UE | Yes | Yes | differentiation is from the perspective of cell applying activation/release |  | Optional with capability signaling Component-2, candidate value set is {1, 2, …, 8} Component-3, candidate value set is {1, …, 16} The total number in FR1 is not greater than X value reported for FR1. Total number in FR2 is not greater than X value reported for FR2. Total number across FR1 and FR2 is not greater than the larger of the FR1 and FR2 values |
|  | 12-2a | Joint release in a DCI for two or more SPS configurations for a given BWP of a serving cell | - M<=4 bits indication in the Release DCI is used for indicating which SPS configuration(s) is/are released, where the association between each state indicated by the indication and the SPS configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple SPS configurations to be released • In case of no higher layer configured state(s), separate release is used where the release corresponds to the SPS configuration index indicated by the indication - The related HARQ-ACK enhancements to support joint release | 12-2 | Yes | N/A |  | Per UE | Yes | Yes | differentiation is from the perspective of cell applying release |  | Optional with capability signaling |
|  | 12-3 | SPS release by DCI format 1_1 and 1_2 | - Support of SPS release by DCI format 1_1 - Support of SPS release by DCI format 1_2 | 5-18 | Yes | N/A |  | Per UE | Yes | Yes | differentiation is from the perspective of cell applying release | A UE supporting component 1 and 11-1 (DCI format 0_2/1_2) shall also support component 2 (SPS release by DCI format 1_2). | Optional with capability signaling |
|  | 12-5 | Configuration of aggregation factor per SPS configuration | Support of configurable PDSCH aggregation factor ({1, 2, 4, 8}) per DL SPS configuration | 5-18 | Yes | N/A |  | Per UE | Yes | Yes | differentiation is from the perspective of cell applying activation/release |  | Optional with capability signaling |

# 3         References

[1] R1-2001484, “RAN1 UE features list for Re-16 NR after RAN1 #100-E,” ATT, NTT DOCOMO.