3GPP TSG RAN WG1 Meeting #100bis-e	R1-2002586

E-meeting, April 20 – April 30, 2020

Agenda Item:	7.2.4.8

Source:	Huawei, HiSilicon

Title:	             Remaining details of sidelink RRC Parameter List

Document for:	Discussion and Decision

Introduction

In this contribution, we provide some Text Proposals to rectify some TBD, missing or incorrectly captured agreements in “R1-2001478 Updated consolidated parameter list for Rel-16 NR” and provide our views on some remaining details of sidelink RRC parameter list.

Discussions

SL-BWP

For per SL-BWP defined parameters locationAndBandwidth-SL, subcarrierSpacing-SL and cyclicPrefix-SL, it is TBD whether those parameters are UE specific or cell-specific. Since a SL-BWP contains resource pools which are UE specifically or cell-specifically (pre-)configured for sidelink communication, a SL-BWP can also be UE specifically or cell-specifically (pre-)configured at least for shared carrier, where NR Uu and sidelink configurations can be aligned.

Proposal 1: Confirm that the parameter locationAndBandwidth-SL, subcarrierSpacing-SL and cyclicPrefix-SL are UE-specific and cell-specific.

Similarly, offsetToCarrier-SL can also reuse configuration of NR Uu, i.e. a per-cell level parameter.

Proposal 2: The parameter offsetToCarrier-SL is defined at per cell level and cell-specific.

We provide changes for Row #3, #4, #5 and #7 in the R1-1913674 Updated consolidated parameter list for Rel-16 NR:

| Row # | Sub-feature group | Parameter name in the spec | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | SL BWP | locationAndBandwidth-SL | Frequency domain location and bandwidth of this bandwidth part. The value of the field shall be interpreted as resource indicator value (RIV) as defined TS 38.214 with assumptions as described in TS 38.213, section 12, i.e. setting N_{BWP}^{size}=275. The first PRB is a PRB determined by subcarrierSpacing-SL of this sidelink BWP and offsetToCarrier-SL. | 0..37949 | Per SL BWP | UE specific, Cell specific | 36.331, 38.331 |
| 4 | SL BWP | subcarrierSpacing-SL | Subcarrier spacing to be used in this sidelink BWP for all channels and reference signals. Corresponds to subcarrier spacing according to 38.211, Table 4.2-1. The value kHz15 corresponds to µ=0, kHz30 to µ=1, and so on. Only the values 15, 30, or 60 kHz (<6GHz), and 60 or 120 kHz (>6GHz) are applicable. | kHz15, kHz30, kHz60, kHz120 | Per SL BWP | UE specific, Cell specific | 36.331, 38.331 |
| 5 | SL BWP | cyclicPrefix-SL | Indicates whether to use the extended cyclic prefix for this bandwidth part. If not set, the UE uses the normal cyclic prefix. Extended CP is supported only for 60 kHz subcarrier spacing. | enabled | Per SL BWP | UE specific, Cell specific | 36.331, 38.331 |
| 7 | SL carrier | offsetToCarrier-SL | Offset in frequency domain between Point A (lowest subcarrier of common RB 0) and the lowest usable subcarrier on this carrier in number of PRBs. | 0..2199 | Per cell | Cell specific | 36.331, 38.331 |

Without proposed changes, the SL-BWP cannot be configured.

SSB

Confirm that SL-SSB related parameters, e.g. numSSBwithinPeriod-SL, timeOffsetSSB-SL, timeIntervalSSB-SL, SSID-SL and synchPriority-SL should be defined in SL-BWP level, i.e. SL-SSB is fixed per SL-BWP.

Proposal 3: Confirm that SL-SSB is defined as per SL-BWP.

We provide changes for Row #9, #10, #11, #12 and #13 in the R1-2001478 Updated consolidated parameter list for Rel-16 NR:

| Row # | Sub-feature group | Parameter name in the spec | Parameter name in the text | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9 | S-SSB | numSSBwithinPeriod-SL | numSSBwithinPeriod-SL | Indicates the number of sidelink SSB transmissions within one sidelink SSB period. In FR1, 1 for 15 kHz SCS, 1 or 2 for 30 kHz SCS, 1, 2, or 4 for 60 kHz SCS are applicable. In FR2, 1, 2, 4, 8, 16 or 32 for 60 kHz SCS, 1, 2, 4, 8, 16, 32 or 64 for 120 kHz SCS are applicable. | 1, 2, 4, 8, 16, 32, 64 | Per SL BWP | UE specific, Cell specific | 36.331, 38.331 |
| 10 | S-SSB | timeOffsetSSB-SL | timeOffsetSSB-SL | Indicate the slot offset from the start of sidelink SSB period to the first sidelink SSB. | 0 .. 1279 | Per SL BWP | UE specific, Cell specific | 36.331, 38.331 |
| 11 | S-SSB | timeIntervalSSB-SL | timeIntervalSSB-SL | Indicate the slot interval between neighboring sidelink SSBs. This value is applicable when there are more than one sidelink SSBs within one sidelink SSB period. | 0 .. 639 | Per SL BWP | UE specific, Cell specific | 36.331, 38.331 |
| 12 | S-SSB | SSID-SL | SSID-SL | Indicate the ID of sidelink synchronization signal assoicated with different synchronization priorities. | 0..671 | Per SL BWP | UE specific, Cell specific | 36.331, 38.331 |
| 13 | S-SSB | synchPriority-SL | synchPriority-SL | Indicates the number of sidelink SSB transmissions within one sidelink SSB period. In FR1, 1 for 15 kHz SCS, 1 or 2 for 30 kHz SCS, 1, 2, or 4 for 60 kHz SCS are applicable. In FR2, 1, 2, 4, 8, 16 or 32 for 60 kHz SCS, 1, 2, 4, 8, 16, 32 or 64 for 120 kHz SCS are applicable. | 1, 2, 4, 8, 16, 32, 64 | Per SL BWP | UE specific, Cell specific | 36.331, 38.331 |

Without proposed changes, the SL-SSB cannot be configured to transmit or receive.

Resource pool

PSSCH DMRS time domain pattern indication

Since the PSSCH DRMS time pattern is designed to cover all supported symbols of a slot for sidelink, including the case that PSFCH symbols and PSSCH symbols are multiplexed in a slot, the DMRS pattern is common in slots with PSFCH and slots without PSFCH

Proposal 4: The configuration set for PSSCH DRMS time pattern is common in slots with PSFCH and slots without PSFCH.

Subchannel size

According to RAN1#100-e working assumption:

- For the number of PRBs for resource pool, allow configuration of all number of PRBs in a SL BWP.

As discussion in our companion paper [2], in order for NR-V2X to support utilization of all frequencies, e.g. in the allocated ITS band, and avoid to have impact on neither TBS determination nor resource allocation.  New subchannel sizes can be introduced for configuration for 10 MHz, 20 MHz, 30 MHz and 40 MHz, which are defined for both n36 band and n47 band for all SCS in FR1, according to TR 38.886.

Note that, based on current configuration, the minimum gap between subchannel size configuration ({10, 15, 20, 25, 50, 75, 100}) and number of RBs for PSCCH configuration ({10, 12, 15, 20}) is 3 PRBs. The new introduced subchannel size should avoid the minimum gap being less than 3 PRBs, which may pose additional performance impact.

Proposal 5: Introduce subchannel size {11, 12, 13, 17, 19, 27, 53} PRBs for configuration in a resource pool so that all number of PRBs in a SL BWP (if is configured to utilize the whole frequency band ) can be used, subject to at least 3 PRBs for PSSCH DMRS in all PRBs.

In summary, we provide changes regarding resource pool configuration for Row #14 and #20 in the R1-2001478 Updated consolidated parameter list for Rel-16 NR:

| Row # | Sub-feature group | Parameter name in the spec | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | Resource pool | subchannelsize | Indicates the minimum granularity in frequency domain for the sensing for PSSCH resource selection in the unit of PRB. | 10, 11, 12, 13, 15, 17, 19, 20, 25, 27, 50, 53, 75, 100 | Per resource pool | UE specific, Cell specific | 36.331, 38.331 | Note: It is not expected a configuration that the difference between the subchannel size and PRBs for PSCCH (frequencyResourcePSCCH) in a resource pool is less than 3 PRBs. |
| 20 | Resource pool | TimePatternPsschDmrs | Indicates the set of PSSCH DMRS time domain patters in terms of PSSCH DMRS symbols in a slot that can be used in the resource pool. | CHOICE { {2}, {3}, {4}, {2, 3}, {2, 4}, {3, 4}, {2, 3, 4} } | Per resource pool | UE specific, Cell specific | 36.331, 38.331 |  |

Without proposed changes, not all PRBs in a given band can be used for NR-V and DMRS location is ambiguous for the case with or without PSFCH in a slot.

Power control

Similar to LTE-V, power-control related parameters for NR-V2X can be (pre-)configured per resource pool so that a UE, no matter it is RRC_CONNECTED or RRC_IDLE or OoC, is able to know these power control configurations when it is communicating with another UE utilizing that resource pool. However, For SSB, since it is configured per SL-BWP, power-control related parameters for PSBCH can be configured per SL-BWP.

Proposal 6: Power control for PSCCH/PSSCH/PSFCH related parameters are (pre-)configured per resource pool. Power control for PSBCH related parameters are (pre-)configured per SL-BWP.

We provide the changes for Row #36 to #45 in the R1-2001478 Updated consolidated parameter list for Rel-16 NR:

| Row # | Sub-feature group | Parameter name in the spec | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 36 | Power control | maximumtransmitPower-SL | Indicates the maximum value of the UE’s sidelink transmission power on this resource pool in the unit of dBm. | -30..33 | Per resource pool | UE specific, Cell specific | 36.331, 38.331 |
| 37 | Power control | p0-DL-PSCCHPSSCH | P0 value for DL pathloss based power control for PSCCH/PSSCH. If not configured, DL pathloss based power control is disabled for PSCCH/PSSCH. | -16..15 | Per resource pool | UE specific, Cell specific | 36.331, 38.331 |
| 38 | Power control | p0-DL-PSFCH | P0 value for DL pathloss based power control for PSFCH. If not configured, DL pathloss based power control is disabled for PSFCH. | -16..15 | Per resource pool | UE specific, Cell specific | 36.331, 38.331 |
| 39 | Power control | p0-SL-PSCCHPSSCH | P0 value for SL pathloss based power control for PSCCH/PSSCH. If not configured, SL pathloss based power control is disabled for PSCCH/PSSCH. | -16..15 | Per resource pool | UE specific, Cell specific | 36.331, 38.331 |
| 40 | Power control | alpha-DL-PSCCHPSSCH | alpha value for DL pathloss based power control for PSCCH/PSSCH. When the field is absent the UE applies the value 1 | {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} | Per resource pool | UE specific, Cell specific | 36.331, 38.331 |
| 41 | Power control | alpha-DL-PSFCH | alpha value for DL pathloss based power control for PSFCH. When the field is absent the UE applies the value 1 | {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} | Per resource pool | UE specific, Cell specific | 36.331, 38.331 |
| 42 | Power control | alpha-SL-PSCCHPSSCH | alpha value for SL pathloss based power control for PSCCH/PSSCH when sl-pathloss is enabled. When the field is absent the UE applies the value 1 | {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} | Per resource pool | UE specific, Cell specific | 36.331, 38.331 |
| 43 | Power control | filterCoefficient-SL | Indicates the measurement filtering coefficient for long-term measurement used for sidelink open-loop power control | "{fc0, fc1, fc2, fc3, fc4, fc5, fc6, fc7, fc8, fc9, fc11, fc13, | Per resource pool | UE specific, Cell specific | 36.331, 38.331 |
| 44 | Power control | p0-DL-PSBCH | P0 value for DL pathloss based power control for PSBCH. If not configured, DL pathloss based power control is disabled for PSBCH. | -16..15 | Per SL BWP | UE specific, Cell specific | 36.331, 38.331 |
| 45 | Power control | alpha-DL-PSBCH | alpha value for DL pathloss based power control for PSBCH. When the field is absent the UE applies the value 1 | {alpha0, alpha04, alpha05, alpha06, alpha07, alpha08, alpha09, alpha1} | Per SL BWP | UE specific, Cell specific | 36.331, 38.331 |

Without proposed changes, a UE cannot be (pre-)configured to perform power control.

Resource allocation mode 1

periodSlCG

Values of period can be reused from reservationPeriodAllowed to align between mode 1 and mode 2, since the periodicity used for either mode 1 or mode 2 is mainly set according to services.

Proposal 7: Values of periodicity for CG can be same as that for reservation in mode 2.

timeGapFirstSidelinkTransmission

In LTE-V, 2 bits are used in LTE DCI format 5A for 15 kHz-only SCS, indicating a value from {0, 1, 2, 3}. For NR, multiple SCS are supported. For SCS larger than 15 kHz, more bits are necessary in NR DCI format 3_0. However, taken into account DCI overhead, 3 bits can be considered for DCI format 3-0 (also discussed in our companion paper [1]), indicating a value from {0, 1, 2, 3, 4, 5, 6, 7,}, i.e. a simple extension of LTE-V DCI format 5A.

Proposal 8: {0, 1, 2, 3, 4, 5, 6, 7} are used for indicating the time gap between DCI format 3_0 and scheduled PSSCH.

HARQ process number

Reuse of NR Uu on number of HARQ processes is a straightforward way for NR sidelink, given that multiple HARQ processes enable efficient NR V2X transition/reception to meet advanced NR V2X service.

Proposal 9: Confirm that maximum number of HARQ process is 16.

In summary, we provide changes regarding resource pool configuration for Row #49, #457 and #62 in the R1-2001478 Updated consolidated parameter list for Rel-16 NR:

| Row # | Sub-feature group | Parameter name in the spec | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 49 | Resource allocation Mode 1 | Indicates the period of sidelink configured grant | Indicates the period of sidelink configured grant | 0, 1..99, 100, 200, 300, 400, 500, 600, 700, 800, 900, 1000 | Per configured grant | UE specific | 36.331, 38.331 |
| 57 | Resource allocation Mode 1 | timeGapFirstSidelinkTransmission | A table of values which indicates the possible time gap values in terms of slots between DCI reception and the first sidelink transmission scheduled by the DCI. | {0,1,2,3,4,5,6,7} | Per configured grant | UE specific | 36.331, 38.331 |
| 62 | Resource allocation Mode 1 | nrofHARQ-Processes-SL | The number of HARQ processes configured. It applies for both Type 1 and Type 2. | 1..16 | Per configured grant | UE specific | 36.331, 38.331 |

Without proposed changes, a UE does not know when to start PSSCH transmission upon reception of a sidelink grant, i.e. DCI format 3_0, and the periodic resource of configured grant cannot used for mode 1 CG type-1.

CSI acquisition

CSIsiReporting

Similar to parameter reserveResourceDifferentTB, CSIsiReporting can be (pre-)configured per resource pool level, so that a UE, no matter it is RRC_CONNECTED or RRC_IDLE or OoC, is able to know if the SL CSI reporting is enabled or not. CSI reporting is only used for unicast transmission, and PC5-RRC signaling can be used for configuration. However, CSI report is carried in a MAC CE on PSSCH which is scheduled by the network in mode 1, thus resource pool level configuration is necessary to enable network control of resource management.

Proposal 10: CSI reporting is enabled or disabled per resource pool.

Since the first symbol and the last symbol in a slot is used for AGC and gap purpose, all the remaining symbols are available for SL CSI-RS transmission. For NCP and ECP, there will in total 12 symbols and 10 symbols, respectively.

Proposal 11: The first OFDM symbol in a PRB used for SL CSI-RS should avoid AGC and gap symbols, i.e. the first symbol and last symbol in a slot.

We provide the changes for Row #72 in the R1-1913674 Updated consolidated parameter list for Rel-16 NR:

| Row # | Sub-feature group | Parameter name in the spec | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 72 | CSI acquisition | CSIsiReporting | Indicates whether CSI reporting is enabled in sidelink unicast. If not set, SL CSI reporting is disabled. | enable | Per resource pool | UE specific, Cell specific | 38.331 |

Without proposed changes, SL CSI reporting configuration is incomplete

CSI RS

firstSymbolInTimeDomainCSIRS-SL

Since the number of symbols for a sidelink slot is (pre-)configured per SL-BWP, it is known prior to  transmission and reception. Similar to DMRS pattern design, which is a common design for any number of sidelink symbols in a slot without dependence on presence of PSFCH at the end of the slot. The first OFDM symbol for CSI-RS is also common in a slot no matter if there is PSFCH in a slot.

Proposal 12: CSI RS indication is common in a slot without dependence on presence of PSFCH in the slot. Confirm the value range is [1, 12].

We provide text proposal for Row #80 in the R1-1913674 Updated consolidated parameter list for Rel-16 NR:

| Row # | Sub-feature group | Parameter name in the spec | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 80 | CSI acquisition | firstSymbolInTimeDomainCSIRS-SL | "Indicate the first OFDM symbol location in a PRB used for SL CSI-RS. | 1…12 | Per UE | UE specific | 38.331 |

Without proposed changes, the position of SL CSI RS transmission cannot be known to the RX UE.

Configuration for CG transmission

In RAN#99, the following agreements were made:

- For configured grant, the maximum number of times that a TB can be retransmitted using the resources provided by the configured grant is configured per priority per configured grant.

- At least the following parameters are part of a SL configured grant configuration:

  - Configuration index of the CG

  - Time offset (for type-1 only)

  - Time-frequency allocation (for type-1 only)

    - Using the same format as in DCI.

  - Periodicity

  - The configured grant is associated with a single transmit resource pool.

  - RAN2 can add other parameters if deemed necessary by RAN2

To align with the PSSCH resource indication in DCI for dynamic grant and configured grant type -2, the current higher layer parameters rrc-ConfiguredSidelinkGrant for configured grant type -1 can only provide maximum 3 resources. When the maximum number of transmission of a TB provided by the higher layer parameter sl-MaxTransNum (with the range of [1, 32]) is larger than 3, rrc-ConfiguredSidelinkGrant alone cannot work.

To solve the problem, a new higher layer parameter, denoted as sl-IntervalCG, which indicates the interval between adjacent a group of resources configured by rrc-ConfiguredSidelinkGrant should be added in the SL-ConfiguredGrantConfig information element in TS 38.331. An illustration for SL-configuredGrantConfig to configure maximum number of 9 (re)transmission of a TB is shown in Figure 1.

Figure 1 SL-configuredGrantConfig to configure maximum number of 9 (re)transmission of a TB

Proposal 13: The higher layer parameter, sl-IntervalCG, is need to indicate the interval between a groups of resources configured by rrc-ConfiguredSidelinkGrant in SL-ConfiguredGrantConfig.

We propose to add a new parameter for indicating the interval between a groups of resources configured by rrc-ConfiguredSidelinkGrant in SL-ConfiguredGrantConfig:

| Sub-feature group | Parameter name in the spec | Description | Value range | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification |
| --- | --- | --- | --- | --- | --- | --- |
| Resource allocation mode 1 | sl-IntervalCG | Indicate the interval in terms of slots between a groups of resources configured by CG resources assignment | Integer (0…31) | Per configured grant | UE specific | 36.331, 38.331 |

Without proposed changes, the mode 1 CG type-1 cannot indicate more than 3 resources in each period of grant.

Conclusions

In this contribution, we propose following:

Proposal 1: Confirm that the parameter locationAndBandwidth-SL, subcarrierSpacing-SL and cyclicPrefix-SL are UE-specific and cell-specific.

Proposal 2: The parameter offsetToCarrier-SL is defined at per cell level and cell-specific.

Proposal 3: Confirm that SL-SSB is defined as per SL-BWP.

Proposal 4: The configuration set for PSSCH DRMS time pattern is common in slots with PSFCH and slots without PSFCH.

Proposal 5: Introduce subchannel size {11, 12, 13, 17, 19, 27, 53} PRBs for configuration in a resource pool so that all number of PRBs in a SL BWP (if is configured to utilize the whole frequency band ) can be used, subject to at least 3 PRBs for PSSCH DMRS in all PRBs.

Proposal 6: Power control for PSCCH/PSSCH/PSFCH related parameters are (pre-)configured per resource pool. Power control for PSBCH related parameters are (pre-)configured per SL-BWP.

Proposal 7: Values of periodicity for CG can be same as that for reservation in mode 2.

Proposal 8: {0, 1, 2, 3, 4, 5, 6, 7} are used for indicating the time gap between DCI format 3_0 and scheduled PSSCH.

Proposal 9: Confirm that maximum number of HARQ process is 16.

Proposal 10: CSI reporting is enabled or disabled per resource pool.

Proposal 11: The first OFDM symbol in a PRB used for SL CSI-RS should avoid AGC and gap symbols, i.e. the first symbol and last symbol in a slot.

Proposal 12: CSI RS indication is common in a slot without dependence on presence of PSFCH in the slot. Confirm the value range is [1, 12].

Proposal 13: The higher layer parameter, sl-IntervalCG, is need to indicate the interval between a groups of resources configured by rrc-ConfiguredSidelinkGrant in SL-ConfiguredGrantConfig.

- References

R1-2001551, “Remaining details of sidelink resource allocation mode 1”, Huawei, HiSilicon, RAN1 #100-e-b, April, 2020.

R1-2001550, “Remaining details of sidelink physical layer structure”, Huawei, HiSilicon, RAN1 #100-e-b, April, 2020.