3GPP TSG RAN WG1 Meeting #100bis-e                                                                  		R1-2002596

E-meeting, April 20 - April 30, 2020

Agenda Item:	7.2.11.11

Source:	Huawei, HiSilicon

Title:	Rel-16 UE features for CLI/RIM

Document for:	Discussion and Decision

Introduction

In this contribution, we will provide some views towards the CLI/RIM related part.

Discussion

- FG 17-1

The wording in component 2) is not clear enough and probably misleading because it was agreed that configured reference SCS can be different from the SCS of active BWP. Therefore, suggest to replace it with “The configured SCS for the CLI-RSSI resource can be different with that of the active BWP, and the UE performs the CLI-RSSI measurement based on the SCS of the active BWP.” As agreed in R1-2001320.

Proposal 1: Replace component 2) in FG 17-1 with “The configured SCS for the CLI-RSSI resource can be different with that of the active BWP, and the UE performs the CLI-RSSI measurement based on the SCS of the active BWP.”

- FG 17-3

Please put the sentence in brackets “if not supported that UE only receives CLI-RSSI resources”, because it is not aligned with current RAN2 agreement and RAN4 has provided such a view in R1-2001523 to RAN2. It is not good for RAN1 to draw a decision now for such discrepancy, which may need more discussions between RAN2 and RAN4, or more in RAN1.

Proposal 2: Put the sentence in brackets “if not supported that UE only receives CLI-RSSI resources” in FG 17-3

- FG 17-4

Similar to the comment above for 17-3, the sentence “if not supported that UE only receives SRS-RSRP resources” should be put within brackets.

Proposal 3: Put the sentence in brackets “if not supported that UE only receives CLI-RSSI resources” in FG 17-4

In summary, all proposed changes above are incorporated into Table 1.

Proposal 4: In summary, adopt all changes in Table 1.

| Index | Feature group | Components | Prerequisite feature groups | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17-1 | CLI-RSSI measurement | 1) Support CLI-RSSI measurement, The max number of resources across all CCs configured to measure RSSI simultaneously shall not exceed 64 2) Up to 64 of measurement resource for CLI-RSSI measurement 2) The configured SCS for the CLI-RSSI resource can be different with that of the active BWP, and the UE performs the CLI-RSSI measurement based on the SCS of the active BWP. |  | [Per UE] | TDD only | Yes | UE operates CLI-RSSI measurement. | Optional with capability signalling |
| 17-2 | SRS-RSRP measurement | 1) Support SRS-RSRP measurement, The max number of SRS resources across all CCs configured to measure SRS-RSRP simultaneously shall not exceed 32 2) Up to 32 of SRS resources to be monitored by a UE |  | [Per UE] | TDD only | Yes | UE operates SRS-RSRP measurement. | Optional with capability signalling |
| 17-3 | Simultaneous reception of PDSCH and CLI-RSSI measurement resource | Support simultaneousRxPDSCHCLIRSSIMeasResource [if not supported that UE only receives CLI-RSSI resources] | 17-1 | Per UE | TDD only | Yes | UE can assume that PDSCH is FDMed with CLI-RSSI measurement resource(s) | Optional with capability signalling |
| 17-4 | Simultaneous reception of PDSCH and SRS-RSRP measurement resource | Support simultaneousRxPDSCHSRSRSRPMeasResource [if not supported that UE only receives SRS-RSRP resources] | 17-2 | Per UE | TDD only | Yes | UE can assume that PDSCH is FDMed with SRS RSRP measurement resource(s). | Optional with capability signalling |

Conclusions

In this contribution, we discuss the issues for Rel-16 UE feature groups corresponding to CLI/RIM, with the following proposals.

Proposal 1: Replace component 2) in FG 17-1 with “The configured SCS for the CLI-RSSI resource can be different with that of the active BWP, and the UE performs the CLI-RSSI measurement based on the SCS of the active BWP.”

Proposal 2: Put the sentence in brackets “if not supported that UE only receives CLI-RSSI resources” in FG 17-3

Proposal 3: Put the sentence in brackets “if not supported that UE only receives CLI-RSSI resources” in FG 17-4

Proposal 4: In summary, adopt all changes in Table 1.

- References

R1-2001484, “RAN1 UE features list for Rel-16 NR after RAN1#100-E ”, 3GPP, RAN1#100-e