3GPP TSG-RAN WG1 Meeting #100b-e	R1-2002620

e-Meeting, April 20th – April 30th , 2020

Agenda Item:	7.2.8.1

Source:	Ericsson

Title:	maintenance of Rel-16 DL reference signals for NR positioning

Document for:	Discussion, Decision

# Introduction

This contribution address  maintenance issues pertaining to DL reference signals for NR positioning.

# Maintenance for the DL PRS

## Muting pattern for DL PRS:

In 38.211, the PRS muting pattern is defined using two bitmaps b1 and b2  which, when both configured, are used with a logical AND to create the final muting pattern.

For a downlink PRS resource in a downlink PRS resource set, the UE shall assume the downlink PRS resource being transmitted when the slot and frame numbers fulfil Nslotframe,μnf+ns,fμ-ToffsetPRS-Toffset,resPRS mod 2μTperPRS∈iTgapPRSi=0TrepPRS-1 and one of the following conditions are fulfilled: - the higher-layer parameter DL-PRS-MutingPattern is not provided; - the higher-layer parameter DL-PRS-MutingPattern is provided and bitmap b1 but not bitmap b2 is provided, and bit bi1 is set; - the higher-layer parameter DL-PRS-MutingPattern is provided and bitmap b2 but not bitmap b1 is provided, and bit bi2 is set; - the higher-layer parameter DL-PRS-MutingPattern is provided and both bitmaps b1 and b2 are provided, and both bit bi1 and bi2 are set.

In 38.214, the muting pattern is described as follow:

- DL-PRS-MutingPattern defines a bitmap of the time locations where the DL PRS resource is expected to not be transmitted for a DL PRS resource set. The bitmap size can be {2, 4, 8, 16, 32} bits long. The bitmap has two options for applicability. In the first option each bit in the bitmap corresponds to a configurable number of consecutive instances of a DL-PRS-ResourceSet where all the DL-PRS-Resources within the set are muted for the instance that is indicated to be muted. In the second option each bit in the bitmap corresponds to a single repetition index for each of the DL-PRS-Resources within each instance of a DL-PRS-ResourceSet and the length of the bitmap is equal to DL-PRS-ResourceRepetitionFactor. Both options may be configured at the same time in which case the logical AND operation is applied to the bit maps as described in clause 7.4.1.7.4 of [4, TS 38.211].

In 38.214, the bitmap size is specified as {2, 4, 8, 16, 32} bits long. However, this is only true for option 1. Option 2 can also take a 1-bit bitmap (no repetitions) if the parameter DL-PRS-ResourceRepetitionFactor  is set to the value 1. Clearly a 1-bit muting pattern is not of practical value, so we propose to clarify that option 2 should not be used if no repetition is present.

The UE is not expected to be configured with DL PRS muting option 2 for a PRS resource with DL-PRS-ResourceRepetitionFactor configured  with value 1.

To correct the muting description in 38.214, we propose the changes shown in text proposal 2.

Endorse text proposal 1 for inclusion in 38.214.

## Collision handling for PRS and SS/PBCH

The agreement on collision for SSB and PRS from RAN1#99 states

Agreement: - For serving TRP, UE assumes that DL PRS is not mapped to any symbol that contains SS/PBCH - Note: In a slot in which SS/PBCH is transmitted on some symbols, DL PRS can be transmitted on other symbols - For neighbor TRPs, when time-frequency location for SSB transmissions on the neighbour TRP is provided, UE assumes that the DL-PRS is not mapped on symbols occupied by SSB transmissions of the neighbour TRP (i.e. DL PRS is not transmitted on these symbols)

The agreement does not differentiate between FR1 and FR2. In FR1, digital beamforming will allow to multiplex multiple beams so that SSB REs and non-overlapping PRS REs could coexist in the same transmitting symbol, with PRS REs punctured only at the SSB location. In FR2, in general only the SSB or the PRS will be possible to be transmitted at the same time.It is proposed to allow for different dropping behaviour between FR1 and FR2, to increase efficiency.

For Frequency range 1, for serving TRP, UE assumes that DL PRS is not transmitted in any resource element that contains SS/PBCH. For neighbor TRPs, when time-frequency location for SSB transmissions on the neighbour TRP is provided, UE assumes that the DL-PRS is not transmitted on resource elements occupied by SSB transmissions of the neighbour TRP

For frequency range 2, For serving TRP, UE assumes that DL PRS is not transmitted in any symbol that contains SS/PBCH. For neighbor TRPs, when time-frequency location for SSB transmissions on the neighbour TRP is provided, UE assumes that the DL-PRS is not transmitted on symbols occupied by SSB transmissions of the neighbour TRP (i.e. DL PRS is not transmitted on these symbols)

Text proposal 2 reflects the proposal for changes  in the specification 38.214.

Endorse text proposal 2 for inclusion in 38.214.

## PRS resource spatial transmission filter

The specification 38.214 mentions that a PRS resource has an associated spatial transmission filter. We don’t think this sentence is needed in the text as the specification does not define the term spatial transmission filter. Instead, QCL relation are defined. We propose to follow the same way for  describing the DL PRS and remove the sentence.

Text proposal 3 reflects the proposal for changes  in the specification 38.214.

Endorse text proposal 3 for inclusion in 38.214.

## DL PRS Sequence ID

During the RAN1#100e, it was proposed to support a fallback to physical cell ID in case no DL PRS sequence ID was configured.  This can be useful in particular when a single PRS is configured per cell at a given time.

For the value of the DL PRS sequence ID, Support fallback to cell ID in case the PRS sequence ID is not configured.

A text proposal is given in text proposal 4.

Endorse text proposal 4 for inclusion in 38.214.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The UE is not expected to be configured with DL PRS muting option 2 for a PRS resource with DL-PRS-ResourceRepetitionFactor configured  with value 1.

Proposal 2	Endorse text proposal 1 for inclusion in 38.214.

Proposal 3	For Frequency range 1, for serving TRP, UE assumes that DL PRS is not transmitted in any resource element that contains SS/PBCH. For neighbor TRPs, when time-frequency location for SSB transmissions on the neighbour TRP is provided, UE assumes that the DL-PRS is not transmitted on resource elements occupied by SSB transmissions of the neighbour TRP

Proposal 4	For frequency range 2, For serving TRP, UE assumes that DL PRS is not transmitted in any symbol that contains SS/PBCH. For neighbor TRPs, when time-frequency location for SSB transmissions on the neighbour TRP is provided, UE assumes that the DL-PRS is not transmitted on symbols occupied by SSB transmissions of the neighbour TRP (i.e. DL PRS is not transmitted on these symbols)

Proposal 5	Endorse text proposal 2 for inclusion in 38.214.

Proposal 6	Endorse text proposal 3 for inclusion in 38.214.

Proposal 7	For the value of the DL PRS sequence ID, Support fallback to cell ID in case the PRS sequence ID is not configured.

Proposal 8	Endorse text proposal 4 for inclusion in 38.214.

# References

RP-190752, “New WID: NR Positioning Support”, Intel Corporation, Ericsson, Shenzhen, China, March, 2019

# Text Proposals for Specification CRs

Text proposal 1 (38.214):

### 5.1.6.5 PRS reception procedure ----unchange text ommitted -------- - DL-PRS-MutingPattern defines a bitmap of the time locations where the DL PRS resource is expected to not be transmitted for a DL PRS resource set. The bitmap size can be {2, 4, 6, 8, 16, 32} bits long. The bitmap has two options for applicability. In the first option each bit in the bitmap corresponds to a configurable number provided by higher layer parameter DL-PRS.MutingBitRepetitionFactor of consecutive instances of a DL-PRS-ResourceSet where all the DL-PRS-Resources within the set are muted for the instance that is indicated to be muted. In the second option each bit in the bitmap corresponds to a single repetition index for each of the DL-PRS-Resources within each instance of a DL-PRS-ResourceSet and the length of the bitmap is equal to DL-PRS-ResourceRepetitionFactor. Both options may be configured at the same time in which case the logical AND operation is applied to the bit maps as described in clause 7.4.1.7.4 of [4, TS 38.211]. The UE is not expected to be configured with DL PRS muting option 2 for a PRS resource with DL-PRS-ResourceRepetitionFactor configured with value 1. ----unchange text ommitted --------

Text proposal 2 (38.214):

### 5.1.6.5 PRS reception procedure ----unchange text ommitted -------- For frequency range 1, the UE assumes that for the serving cell the DL PRS is not transmitted in any resource element that contains SS/PBCH. If the time frequency location of the SS/PBCH block transmissions from non-serving cells are provided to the UE then the UE also assumes that for the non-serving cell the DL PRS is not transmitted to any resource element that contains the SS/PBCH block of the non-serving cell. For frequency range 2, Tthe UE assumes that for the serving cell the DL PRS is not mapped totransmitted in any symbol that contains SS/PBCH. If the time frequency location of the SS/PBCH block transmissions from non-serving cells are provided to the UE then the UE also assumes that for the non serving cell the DL PRS is not mapped totransmitted in any symbol that contains the SS/PBCH block of the non-serving cell. For the serving cell, if the DL PRS resource collides with SS/PBCH, the UE assumes that the PRS resource is transmitted if the PRS resource is muted in the symbols where the SS/PBCH is transmitted, otherwise the PRS resource colliding with the SS/PBCH is dropped. For the non-serving cell, when time-frequency location for SSB transmissions on the neighbour TRP is provided, if the DL PRS resource collides with SS/PBCH, the UE assumes that the PRS resource is transmitted if the PRS resource is muted in the symbols where the SS/PBCH is transmitted, otherwise the PRS resource colliding with the SS/PBCH is dropped. ----unchange text ommitted --------

Text proposal 3 (38.214):

### 5.1.6.5 PRS reception procedure The UE can be configured with one or more DL PRS resource set configuration(s) as indicated by the higher layer parameters DL-PRS-ResourceSet and DL-PRS-Resource. Each DL PRS resource set consists of K≥1 DL PRS resource(s) where each has an associated spatial transmission filter.

Text proposal 4 (38.211):

The pseudo-random sequence generator shall be initialised with cinit=222nID,seqPRS1024+210Nsymbslotns,fμ+l+12nID,seqPRS mod 1024+1+nID,seqPRS mod 1024 mod 231 where ns,fμ is the slot number, the downlink PRS sequence ID nID,seqPRS∈0,1,…,4095 is given by the higher-layer parameter DL-PRS-SequenceId, if present, otherwise nID,seqPRS is equal to physical cell ID, and l is the OFDM symbol within the slot to which the sequence is mapped.