3GPP TSG RAN WG1 #100bis		R1- 2002706

e-Meeting, April 20th – 30th, 2020

Agenda item:	6.2.4

Source: 	Moderator (Qualcomm Incorporated)

Title: 	Summary of issues for LTE-based 5G terrestrial broadcast

Document for:	Discussion and Decision

# Summary of issues

The following corrections have been submitted to this meeting:

Issue #1: Corrections to TS 36.214

Issue #2: Antenna port definition

Issue #3: Presence of RS

Issue #4: Correction to MBSFN-RS generation

Issue #5: TP for 36.201

Issue #6: Assumption on CFI

Issue #7: Text for TBS scaling

# Issue #1: Corrections to TS 36.214

In x1636, it is proposed to modify the text in 36.214 to clarify that measurements are made in “slots” in which the UE receives the new numerology with 0.37kHz SCS. The change is as follows (in different subclauses of the specification):

NOTE 3:	The measurement is made only in subframes or slots corresponding to 0.37 kHz subcarrier spacing and on carriers where the UE is decoding PMCH.

# Issue #2: Antenna port definition

In x1635, it is proposed to modify the text in TS 36.211 on antenna port definition as follows:

-	MBSFN reference signals are transmitted on antenna port. The channel over which a symbol on antenna portis conveyed can be inferred from the channel over which another symbol on the same antenna port is conveyed only if the two symbols correspond to subframes of the same MBSFN area. For MBSFN reference signal pattern type 1 of 0.37 kHz subcarrier spacing, the channel over which a symbol on antenna port  is conveyed can be inferred from the channel over which another three symbols on the same port is conveyed only if the four symbols correspond to slots of the same MBSFN area. For MBSFN reference signal pattern type 2 of 0.37 kHz subcarrier spacing, the channel over which a symbol on antenna port  is conveyed can be inferred from the channel over which another symbol on the same port is conveyed only if the two symbols correspond to slots of the same MBSFN area.

This introduces the following changes:

- Use “slots” instead of subframes.

- Limit the coherency for four symbols / two symbols depending on the RS pattern.

# Issue #3: Presence of RS

In x2626 and x2179, it is recommended to discuss the assumptions the UE can make regarding the presence of reference symbols outside a given PMCH slot. More precisely, the following observations are made:

- In x2626: A UE implementing frequency-only channel interpolation for the PMCH numerology with f = 0.37 kHz and RS pattern with Dt = 4/2 only achieves a theoretical equalisation interval of 225/450µs.

- In x2179: The following TP is proposed, along with some additional observations on the proposals:

<TP2, 36.211 6.10.2>

### 6.10.2	MBSFN reference signals

MBSFN reference signals shall be transmitted in the MBSFN region of MBSFN subframes only when the PMCH is transmitted. MBSFN reference signals are transmitted on antenna port 4.

For a PMCH transmitted with 0.37 kHz subcarrier spacing in slot n,

-	for MBSFN reference signal pattern type 1, the UE may assume that MBSFN reference signals associated with the same NIDMBSFN are present in the three preceding slots to slot n.

-	for MBSFN reference signal pattern type 2, the UE may assume that MBSFN reference signals associated with the same NIDMBSFN are present in the preceding slot to slot n.

MBSFN reference signals are defined for extended cyclic prefix only.

</TP2>

# Issue #4: Correction to MBSFN-RS generation

In x2179, a potential issue with the MBSFN-RS generation for 0.37kHz SCS is presented. The current equation gives the following pattern for Td=4:

A TP is presented in x2179 as follows:

<TP1, 6.10.2.2.4>

The reference-signal sequence rlm' in OFDM symbol l shall be mapped to complex-valued modulation symbols ak,l(p) with p=4 according to

ak,l(p)=rl,nsm'

0≤k<NscRB12NRBDL

where  ns is the 3ms absolute slot number, defined as ns= ns+13nf/4, ns is the 3 ms slot number as defined in clause 4.1 and

-	for MBSFN reference signal pattern type 1

0≤k<NscRB12NRBDL

l=0

m'=0,1,…,NscRB12NRBmax,DL-1

  - For NRBDL odd

k=12m'-12NscRB12 NRBmax,DL- NscRB12NRBDLNscRB12Δ+3ns mod 4if ns mod 4∈0,112m'-12NscRB12NRBmax,DL- NscRB12NRBDL-1NscRB12Δ+3ns mod 4if ns mod 4∈2,3
l=0
Δ=NRBmax,DL-NRBDL2
m'=0,1,…,NscRB12NRBmax,DL-1

  - For NRBDL even

k= 12m'-12NscRB12NRBmax,DL-NRBDL+3ns mod 4

-	for MBSFN reference signal pattern type 2

0≤k<NscRB6NRBDL

k=6m'-NscRB6Δ+3ns mod 2
l=0
Δ=NRBmax,DL-NRBDL2
m'=0,1,…,NscRB6NRBmax,DL-1

</TP1>

# Issue #5: TP for 36.201

In x2601, a TP for introducing the new numerologies to 36.201 is presented as follows:

### < Unchanged parts are omitted >

## 4.2	General description of Layer 1

### 4.2.1	Multiple access

The multiple access scheme for the LTE physical layer is based on Orthogonal Frequency Division Multiplexing (OFDM) with a cyclic prefix (CP) in the downlink, and on Single-Carrier Frequency Division Multiple Access (SC-FDMA) with a cyclic prefix in the uplink and sidelink. To support transmission in paired and unpaired spectrum, two duplex modes are supported: Frequency Division Duplex (FDD), supporting full duplex and half duplex operation, and Time Division Duplex (TDD).

The Layer 1 is defined in a bandwidth agnostic way based on resource blocks, allowing the LTE Layer 1 to adapt to various spectrum allocations. A resource block spans either 12 sub-carriers with a sub-carrier bandwidth of 15kHz or 24 sub-carriers with a sub-carrier bandwidth of 7.5kHz each over a slot duration of 0.5ms, 72 sub-carriers with a sub-carrier bandwidth of 2.5kHz each over a slot duration of 0.5ms, 144 sub-carriers with a sub-carrier bandwidth of 1.25kHz over a slot duration of 1ms, or 486 sub-carriers with a sub-carrier bandwidth of approximately 0.37kHz over a slot duration of 3ms. Narrowband operation is also defined, whereby certain UEs may operate using a maximum transmission and reception bandwidth of 6 contiguous resource blocks within the total system bandwidth; for narrowband operation, sub-resource-block operation may also be used in the uplink, using 2, 3 or 6 sub-carriers.

For Narrowband Internet of Things (NB-IoT) operation, a UE operates in the downlink using 12 sub-carriers with a sub-carrier bandwidth of 15kHz, and in the uplink using a single sub-carrier with a sub-carrier bandwidth of either 3.75kHz or 15kHz or alternatively 3, 6 or 12 sub-carriers with a sub-carrier bandwidth of 15kHz.

The radio frame structure type 1 is only applicable to FDD (for both full duplex and half duplex operation) and has a duration of 10ms and consists of 20 slots with a slot duration of 0.5ms. Two adjacent slots form one sub-frame of length 1ms, except when the sub-carrier bandwidth is 1.25kHz or approximately 0.37kHz, in which case one slot forms one sub-frame or a time duration of 3ms, respectively. When the sub-carrier bandwidth is 15kHz, a slot can be further subdivided into three subslots of length 2 or 3 OFDM or SC-FDMA symbols for reduced latency operation.

The radio frame structure type 2 is only applicable to TDD and consists of two half-frames with a duration of 5ms each and containing each either 10 slots of length 0.5ms, or 8 slots of length 0.5ms and three special fields (DwPTS, GP and UpPTS) which have configurable individual lengths and a total length of 1ms. A subframe consists of two adjacent slots, except for subframes which consist of DwPTS, GP and UpPTS, namely subframe 1 and, in some configurations, subframe 6. Both 5ms and 10ms downlink-to-uplink switch-point periodicity are supported. Further details on the LTE frame structure are specified in [2]. Adaptation of the uplink-downlink subframe configuration via Layer 1 signalling is supported.

The radio frame structure type 3 is only applicable to LAA secondary cell operation. It has a duration of 10ms and consists of 20 slots with a slot duration of 0.5ms. Two adjacent slots form one subframe of length 1ms. Any subframe may be available for downlink or uplink transmission. For downlink transmission, the eNB shall perform the channel access procedures as specified in [4] prior to transmitting. A downlink or uplink transmission may start at the subframe boundary or later, and may end at the subframe boundary or earlier. For uplink transmission, the UE shall perform the channel access procedures as specified in [4] prior to transmitting.

To support a Multimedia Broadcast and Multicast Service (MBMS), LTE offers the possibility to transmit Multicast/Broadcast over a Single Frequency Network (MBSFN), where a time-synchronized common waveform is transmitted from multiple cells for a given duration. MBSFN transmission enables highly efficient MBMS, allowing for over-the-air combining of multi-cell transmissions in the UE, where the cyclic prefix is utilized to cover the difference in the propagation delays, which makes the MBSFN transmission appear to the UE as a transmission from a single large cell. Transmission on a dedicated carrier for MBSFN is supported, as well as transmission of MBSFN on a carrier with both MBMS transmissions and point-to-point transmissions using time division multiplexing. In addition to the 15kHz sub-carrier bandwidth, the sub-carrier bandwidth of 7.5kHz with a longer CP, the sub-carrier bandwidth of 2.5kHz with a long CP (100µs), the sub-carrier bandwidth of 1.25kHz with very long CP (200µs), and the sub-carrier bandwidth of approximately 0.37kHz with a long CP (300µs) are all supported on dedicated MBSFN carriers, whereas MBSFN subframes that are time-multiplexed on the same carrier with non-MBSFN subframes may be configured with the 2.5kHz, 1.25kHz, or approximately 0.37kHz sub-carrier bandwidth. Transmission of PDSCH also in MBSFN subframes that are not used for MCH is supported.

Transmission with multiple input and multiple output antennas (MIMO) are supported with configurations in the downlink with up to 32 transmit antenna ports and eight receive antennas, which allow for multi-layer downlink transmissions with up to eight streams and beamforming in both horizontal and vertical dimensions. Multi-layer uplink transmissions with up to four streams are supported with configurations in the uplink with up to four transmit antenna ports and four receive antennas. Multi-user MIMO, i.e. allocation of different streams to different users is supported in both UL and DL.

Coordinated Multi-Point (CoMP) transmission and reception are supported, including the possibility to configure a UE with multiple Channel State Information (CSI) feedback processes.

Aggregation of multiple cells is supported in the uplink and downlink with up to 32 serving cells, where each serving cell can use a transmission bandwidth of up to 110 resource blocks and can operate with either frame structure type 1 or frame structure type 2. Dual connectivity to groups of serving cells that belong to two different eNode-Bs is also supported.

Sidelink transmissions are defined for ProSe Direct Discovery and ProSe Direct Communication between UEs. The sidelink transmissions use the same frame structure as uplink and downlink when the UEs are in network coverage; however, the sidelink transmissions are restricted to a sub-set of the uplink resources. V2X communication between UEs is supported via sidelink transmissions or via the eNB.

### < Unchanged parts are omitted >

# Issue #6: Assumption on CFI

In x2180 it is proposed to change the “shall” to “may” in the following subclause regarding use of semistatic CFI:

<TP1 36.213>

For a MBMS-dedicated cell, if a UE is configured with higher layer parameter semistaticCFI included in MasterInformationBlock-MBMS, the UE may assume the CFI is equal to the value of the higher layer parameter semistaticCFI for non-MBSFN subframes.

</TP1 36.213>

# Issue #7: Text for TBS scaling

In x2179, it is proposed to modify the text for TBS scaling, since the current text may not be clear:

<TP3, 36.213. 11.1>

The UE shall then follow the procedure in Subclause 7.1.7.2.1 to determine the transport block size, assuming is equal to. For PMCH with  the UE shall scale the derived transport block size by , then round to the closest valid transport block size in the union of Table 7.1.7.2.1-1 and Table 7.1.7.2.4-1 by including, in the rounding procedure, only the TBS entries in Table 7.1.7.2.1-1 with ITBS≤33B and the entries in 7.1.7.2.4-1 for which the TBS_L1 is present in Table 7.1.7.2.1-1 with ITBS≤33B if the UE is configured by higher layers to decode the PMCH based on QPSK, 16QAM, 64QAM, and 256QAM; and only the TBS entries in Table 7.1.7.2.1-1 with ITBS≤26A and the entries in 7.1.7.2.4-1 for which the TBS_L1 is present in Table 7.1.7.2.1-1 with ITBS≤26A otherwise. In case the scaled TBS has the same distance to two valid transport block sizes, it is rounded to the larger transport block size. The UE shall set the redundancy version to 0 for the PMCH.

</TP3>

# Proposed scope for email discussions

In view of the submissions to this meeting, the moderator makes the following proposal for setting up the email discussions:

- Email discussion #1: Aspects on reference signals for 0.37kHz numerology (Issues #2, #3, #4).

- Email discussion #2: Other aspects of new numerologies (Issues #1, #5, #7)

Issue #6 is deemed to be lower priority and thus is proposed not to be discussed during this meeting.

# References

| R1-2001635 | Corrections to TS 36.211 for the 0.37kHz subcarrier spacing MBSFN | ZTE |
| --- | --- | --- |
| R1-2001636 | Text proposal for new numerology in TS 36.214 | ZTE |
| R1-2002179 | Support of longer numerologies for rooftop reception | Qualcomm Incorporated |
| R1-2002180 | Corrections for CAS reception | Qualcomm Incorporated |
| R1-2002601 | Text proposal for new numerology in TS 36.201 | Huawei, HiSilicon |
| R1-2002626 | UE assumptions of MBSFN-RS for new PMCH numerology for support of rooftop reception | EBU, BBC, IRT |